Readme File for Recommendation P.1511 Data Files

The TOPO.dat file contains the height above mean sea level.

The TOPO.dat data file has 2164 rows and 4324 columns, where each row corresponds to a geodetic latitude, each column corresponds to a geodetic longitude, and the latitude and longitude row and column increments are 1/12° ~0.08333°. The file contains two additional rows at the top and bottom and two additional columns at the left and right which wrap around the Earth to facilitate the bi-cubic interpolation procedure. For example, the (Latitude, Longitude) in the five rows and five columns in the upper left corner of the data file are as follows.

| (+89.875°, -0.125°) | (+89.875°, -0.041666° | (+89.875°, +0.041666°) | (+89.875°, +0.125°) | (+89.875°, +0.208333 |
| --- | --- | --- | --- | --- |
| (+89.958333°, -0.125°) | (+89.958333° ,-0.041666° | (+89.958333°, +0.041666°) | (+89.958333°, +0.125°) | (+89.958333°, +0.208333 |
| (+89.958333°, +179.875°) | (+89.958333°, +179.958333°) | (+89.958333°, -179.958333°) | (+89.958333°, -179.875°) | (+89.958333°, -179.791667°) |
| (+89.875°, +179.875°) | (+89.875°, +179.958333° | (+89.875°, -179.958333°) | (+89.875°, -179.875°) | (+89.875°, -179.791667 |
| (+89.791666°, +179.875°) | (+89.791666°, +179.958333° | (+89.791666°, -179.958333°) | (+89.791666°, -179.875°) | (+89.791666°, -179.791667 |

Depending on the interpolation method (e.g. the Matlab bi-cubic interpolation function interp2(..) which requires strictly monotonic latitude and longitude values), it may be convenient to assume the upper left corner corresponds to a “virtual” latitude of +90.125° and “virtual” longitude of -180.125°, and the row and column increments are 1/12° ~0.08333°

The height above mean sea level at any location on the surface of the Earth can be obtained using the bi-cubic interpolation procedure provided in Annex 1 of Recommendation ITU-R P.1144.

Matlab users should note that the Matlab bi-cubic interpolation function interp2(..) is identical to the bi-cubic interpolation procedure provided in Annex 1 of Recommendation ITU-R P.1144; however, Octave users should note that the Octave bi-cubic interpolations function interp2(..) is different than the bi-cubic interpolation procedure in Annex 1 of Recommendation ITU-R P.1144.

Input data and model development

The topographic height data are based on the Earth2014 model with a resolution of 5 arcminutes in longitude and latitude, which was developed by the TUM School of Engineering and Design, Technische Universität München.