RECOMMENDATION  ITU-R  PN.311-7

ACQUISITION,  PRESENTATION  AND  ANALYSIS  OF  DATA
IN  STUDIES  OF  TROPOSPHERIC  PROPAGATION

(1953-1956-1959-1970-1974-1978-1982-1990-1992-1994)

Rec. ITU-R PN.311-7

The ITU Radiocommunication Assembly,

considering

a)	that for the design of communication systems, propagation prediction models with global validity are necessary;

b)	that propagation and radiometeorological data are of fundamental importance for the development and testing of such prediction models;

c)	that, to facilitate the comparison of data and results, it is desirable to acquire and present propagation and radiometeorological data in a uniform manner,

recommends

1.	that data on tropospheric propagation, submitted to Radiocommunication Study Group 3, follow the principles and formats contained in Annex 1.

ANNEX  1

Data banks to support evaluation of prediction methods

1.	Introduction

2.	Responsibilities, updates and distributions

3.	Acceptance criteria

4.	Testing criteria for comparing prediction methods

5.	Description of the data banks of Radiocommunication Study Group 3 concerning tropospheric propagation

5.1	Part I:	Terrestrial line-of-sight path data

5.2	Part II:	Earth-space path data

5.3	Part III:	Terrestrial trans-horizon path and rain scatter data

5.4	Part IV:	Radiometeorological data

5.5	Part V:	Terrestrial land mobile data

5.6	Part VI:	Terrestrial broadcasting data

5.7	Part VII:	Data for mobile-satellite services

6.	Auxiliary information for submission of data

6.1	Country codes used

6.2	Definition of format for submitting path profile information

6.3	Terrain cover types

# 1.	Introduction

One of the essential requirements for the provision of reliable methods for prediction of radio propagation effects is the establishment of suitable computer data banks. Such data banks must:

–	contain all data available that are of an adequate standard,

–	be widely accepted as the source material on which to conduct testing,

–	be readily available.

It is a principle of the data banks that they shall contain only such data as may be used for:

–	testing prediction methods recommended by Radiocommunication Study Group 3 (and may of course be used to test other methods); and

–	for the creation and updating of radiometeorological maps relevant to the prediction of radio propagation effects.

In special cases in studies on tropospheric propagation, where no prediction method has been adopted by Radiocommunication Study Group 3, tabular data remain in the Annex of the relevant Recommendation to give guidance to the reader from the best measured data currently available.

The current data banks relate to:

–	evaluation of prediction methods for terrestrial line-of-sight propagation,

–	evaluation of prediction methods for Earth-space propagation,

–	evaluation of prediction methods for interference or reliability over trans-horizon paths,

–	radiometeorological data,

–	evaluation of prediction methods for terrestrial land mobile services,

–	evaluation of prediction methods for terrestrial broadcasting services,

–	evaluation of prediction methods for mobile-satellite services.

Administrations are urged to submit their data to Radiocommunication Study Group 3 and/or the relevant Working Party in accordance with the requirements given in this Annex. Section 2 outlines the more administrative aspects related to the data banks, the procedure for input of the new data for inclusion in the data banks and the procedure for obtaining copies of the data banks. Section 3 gives the criteria with which the data submitted must comply to being accept. Section 4 lists the testing criteria used. Section 5 lists in detail all the tables of the data banks and the specific definitions used. Section 6 lists the names, abbreviations and units used for the individual data elements contained in the data bank.

# 2.	Responsibilities, updates and distributions

The responsibility for the data banks lies with Radiocommunication Study Group 3, taking full advantage of the Working Parties for technical input and management, and the services of the Radiocommunication Bureau for publication and distribution. Responsibility as to the accuracy and significance of the data remains with the authors given in the references and/or with the administrations that have submitted the data. However, in order to facilitate the transformation of the given data to computer-data, and to ensure the quality of the data banks, the data shall first be reviewed by the relevant Working Party according to the set of criteria explained in § 3. Non-compliant data may still be accepted after additional information and/or adequate explanations are sought and received from the relevant administration.

Ensuring adequate procedures for technical maintenance and production of the data banks will be a continuing requirement to keep under review. It is proposed that each table of the data bank be allocated to a Working Party to consider, and that the relevant WPs nominate, for each table for which they are responsible, an individual to coordinate updates.

Copies of the data banks, on floppy disk (DOS format ASCII files on 3.5 or 5.25 in media), may be obtained from the Radiocommunication Bureau. Only in special cases will the Bureau provide printed copies of the data banks. Participants in Radiocommunication Study Group 3 involved in database management and model testing receive a copy of DB SG5, a database management system allowing the data bank to be queried in order to carry out comparative tests of prediction methods.

Table III-1a “Clear air spot measurement data” is currently available as a separate database. The table currently includes approximately 100 000 measurements recorded over 1 326 paths. They were obtained from measurements lasting from 10 min to 1 h. The database is available from the Radiocommunication Bureau on 3.5 in floppy disks (DOS format dBASEIII files). A routine that allows the user to read, print and edit the data is provided on the disk.

# 3.	Acceptance criteria

Data given for inclusion in the data banks will be reviewed as to their suitability according to the following criteria:

–	Compliance of the given information with the formats described in § 5. In particular, the units of measure should consistently adhere to the ones listed in the table description sheets. With few exceptions they are based on the International System of Units (SI system). For the definitions of terms see Recommendation ITU-R PN.310. It is recommended to use copies of the tables in § 5 for submission of the data and to enter additional important information under “Remarks”.

–	For tables requesting both cumulative statistics of propagation phenomena and precipitation, concurrent data are required. This means that the two statistics have to be derived from the same period of observation. If the data taken were matched by eliminating the data of the other measurement for this time interval, this should be noted under “Remarks”.

–	For long-term and yearly cumulative statistics the observation period shall be an integer multiple of 12 months and the equipment up-time should be at least 90% of the total time reported.

–	The worst-month cumulative statistics (see Recommendation ITU-R PN.581) must have been derived from all the 12 monthly statistics of the relevant year. The equipment up-time must be at least 75% of each month.

–	Accuracy of interpolation: when converting the measured cumulative statistics into the format requested (for several fixed percentages), interpolation may be required. For this, a sufficiently large number of reference levels must be chosen such that for successive reference levels the ratio of the probabilities is greater than 0.8 and less than 1.25. Extrapolated values should not be submitted.

–	For terrestrial wideband data the receiver dynamic range should be at least 18 dB to provide for a minimum peak-to-noise ratio of 15 dB.

The above criteria shall be applied by the reviewers of the given data. In special cases, however, some of the criteria may be relaxed (e.g. in multipath phenomena the fade statistics show a definite linear trend in the tail of the distribution when plotted on a logarithmic-linear graph, so that interpolation becomes less of a problem). It is also appropriate to apply less stringent acceptance criteria in cases where the statistical data come from a region that is hardly represented in the respective data-table. Data which are accepted in spite of not meeting the acceptance criteria (due to reasons given above) will be marked with a particular flag by the responsible table-coordinator and are subject to removal from the data bank once a sufficient number of fully compliant data have been entered.

# 4.	Testing criteria for comparing prediction methods

## 4.1	General considerations

In order to judge the relative merits of a prediction method, an objective set of criteria needs to be defined. In general, the data used for the comparison have to be suitable for the purpose of the application (see data acceptance criteria, § 3). While the database generally lists all data that are suitable for at least one type of test, some data entries may not be suitable for certain types of predictions and need therefore to be excluded from such tests. (Example: some of the data in Table III-1 are not suitable for testing trans-horizon reliability – they are therefore marked in the flag-field.) It is also important to exclude any dependent data (where one entry is a sub-set of another entry). However, data from measurements made at the same station during the same period with different elevation angle or polarization can be treated as independent data.

In addition, in most cases, the duration of the measurement (in multiples of years) is to be used as a weighting function. (Note that the duration is defined as the actual number of days made up by the complete set of valid data which is normally less than the time from start date to end date; the difference is the “down-time” of the experiment.)

The general requirements for models are (in decreasing order of importance).

### 4.1.1	Best performance in terms of testing variable

This testing variable (i.e. minimum mean of difference between predicted and measured value or minimum standard deviation of the difference) has to be agreed upon within the responsible Working Party. It should be noted that the test needs to be carried out on the full data set currently applicable and on agreed-upon sub-sets of the data.

### 4.1.2	“Physical basis” of the method chosen

Most propagation prediction methods used are semi-empirical in nature because the details of the physical process are either not precisely known or because the number of input parameters would simply be impractical to provide. The better the underlying physical principles are represented in a model, the better the chances that the model can also be applied in hitherto unexplored domains (new frequencies, new climatic zones, etc.). A purely empirical method that is simply derived from curve-fitting to measured data is normally not suitable for being applied outside the domain of the measurements made and should therefore be avoided.

### 4.1.3	“Simplicity”

This criterion, which to some degree may be interpreted as contradictory to the “physical basis” requirement should only be applied to keep the number of required input parameters to a minimum and to assure that the description of the algorithm lends itself to a clear and unambiguous implementation in a computer program. Nomographs can be highly useful simplified representations of a prediction method but they cannot be accepted as the method per se.

## 4.2	Testing variable for comparing rain attenuation predictions

Attenuation predictions are generally made for a number of transmission paths at a fixed set of probability levels. Data for comparison of prediction methods are to be tabulated at fixed probability levels, e.g. 0.001, 0.01 and 0.1% of the year. The ratio of predicted to measured attenuation is calculated for each path. The natural logarithm of the ratios is used as a test variable. To compensate for the effects of both, contributions from attenuation sources other than rain and measurement inaccuracies, which predominantly affect the lower attenuation values, the logarithm is to be multiplied by a scaling factor that is a power function of the measured attenuation. The thus modified test variable closely follows a normal distribution. The mean and standard deviations of the (modified) test variable are then calculated to provide the statistics for prediction method comparison.

### 4.2.1	Procedure

–	For each percentage of time, calculate the ratio of predicted attenuation, Ap (dB), to measured attenuation, Am (dB), for each radio link:

Si  =  Ap,i / Am,i

where Si is the above ratio calculated for the ith radio link.

–	Calculate the test variable:

Vi  =  ln Si  ´  (Am,i / 10)0.2

–	Calculate the mean mV, standard deviation sV, and r.m.s. value rV, of the Vi values:

–	Repeat the procedure for each percentage of time.

In the comparison of prediction methods, the best prediction method produces the smallest values of the statistical parameters. It is to be noted that the logarithmic parameters can afterwards be converted to equivalent percentage parameters. The standard deviation, for example, leads to equivalent upper and lower percentage deviations:

which are measures of the spread of the predicted values with respect to the measured ones, normalized to an attenuation value of 10 dB.

The procedure provides a tool not only to assess the performance of the various prediction methods but also to give indications for their improvement. Graphical inspection of the spreads of Ap and Am values may also give useful information about the relative merits of both experimental data and prediction methods.

Moreover, these statistical parameters may provide some information on the expected spread of actual attenuation values around a predicted one. For this purpose, the above scaling procedure can be used in the reverse direction, i.e. the normalized standard deviation for an attenuation level of 10 dB can be scaled to the standard deviation to be expected at another predicted attenuation level, Ap (dB), by the factor (10/Ap)0.2.

It has to be noted that the ultimate limit for the accuracy of any prediction method is the accuracy with which the rain climatic conditions for a given location can be characterized by an assumed point rainfall intensity cumulative distribution.

# 5.	Description of the data banks of Radiocommunication Study Group 3 concerning tropospheric propagation

## 5.1	Part I:  Terrestrial line-of-sight path data

Table I-1:	Line-of-sight rain attenuation statistics

Table I-2:	Line-of-sight average worst-month multipath fading and enhancement in narrow bandwidths

Table I-3:	Line-of-sight diversity data

Table I-4:	Line-of-sight clear sky XPD and CPA statistics

Table I-5:	Line-of-sight XPD and CPA statistics due to precipitation

Table I-6:	Line-of-sight worst month multipath channel characteristics and outage times

## 5.2	Part II:  Earth-space path data

Table II-1:	Slant path annual rain attenuation and rain rate statistics

Table II-2:	Slant path worst-month rain attenuation statistics

Table II-3:	Slant path fade duration statistics

Table II-4:	Slant path site diversity statistics

Table II-5a:	Slant path annual XPD statistics

Table II-5b:	Slant path annual XPD statistics conditioned to CPA

Table II-6:	Slant path statistics of amplitude scintillations

Table II-7:	Slant path standard deviations of scintillations

## 5.3	Part III:  Terrestrial trans-horizon path and rain scatter data

Table III-1:	Clear-air trans-horizon basic transmission loss statistics

Table III-1a:	Clear-air spot measurement data. (This table is a separate data bank (see § 2).)

Table III-2:	Rain scatter on terrestrial paths

## 5.4	Part IV:  Radiometeorological data

Table IV-1:	Statistics of rain intensity

Table IV-2:	Rain integration time conversion factor

Table IV-3:	Annual statistics of sky noise temperature

Table IV-4:	Statistics of mean surface refractivity

## 5.5	Part V:  Terrestrial land mobile data

Table V-1:	Terrestrial land mobile wideband statistics

Table V-2: 	Terrestrial land mobile narrow-band statistics

## 5.6	Part VI:  Terrestrial broadcasting data

Table VI-1:	Terrestrial broadcasting signal level variation with time

Table VI-2:	Terrestrial broadcasting signal level variation with location

## 5.7	Part VII:  Data for mobile-satellite services

Table VII-1:	Wideband statistics for mobile-satellite links

Table VII-2:	Narrow-band statistics of maritime mobile-satellite links

Table VII-3:	Narrow-band statistics of land mobile-satellite links

Table VII-4:	Narrow-band statistics of aeronautical mobile-satellite links

Part I:  Terrestrial line-of-sight path data

TABLE  I-1 – Line-of-sight rain attenuation statistics

Path number 	_ _ _ _ _ _

Transmit station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1) 	_ _

TX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

TX longitude (0..360) (degrees) E 	_ _ _  × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _

TX antenna height ag ht (m)	_ _ _

TX 3 dB beamwidth qt (degrees)	_ _ _ × _ _

Receive station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No. 	_ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver 	_ _ _ _ _ _ _ _ _ _

Path length l (km)	_ _ _ _ × _ _

Terrain type (land/sea/mixed)	_ _ _ _ _ _ _ _ _ _

Path profile (file name)	_ _ _ _ _ _ _ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Attenuation presented as excess or total (E/T)	_

Table:

Attenuation (dB) exceeded for percentage of time (average year)

Rain rate (mm/h) exceeded for percentage of time (average year)

| Percentage of time | 0.001 | 0.002 | 0.003 | 0.006 |  |
| --- | --- | --- | --- | --- | --- |
| Rain rate (mm/h) | – | – | – | – |  |
| Attenuation (dB) | – | – | – | – |  |
| Percentage of time | 0.01 | 0.02 | 0.03 | 0.06 | 0.1 |
| Rain rate (mm/h) | – | – | – | – | – |
| Attenuation (dB) | – | – | – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

TABLE  I-2 – Line-of-sight average worst-month multipath fading and enhancement in narrow bandwidths

Path number	_ _ _ _ _ _

Transmit station

TX site name 	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1)	_ _

TX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

TX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _

TX antenna height ag ht (m)	_ _ _

TX 3 dB beamwidth qt (degrees)	_ _ _ × _ _

TX antenna type	_ _ _ _ _ _ _ _ _ _

TX antenna diameter Dt (m)	_ _ × _ _

TX antenna feed type	_ _ _ _ _ _ _ _ _ _

Receive station

RX site name 	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _

RX antenna diameter Dr (m)	_ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX dynamic range  (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ _ × _ _

Data resolution (dB)	_ _ × _ _ _

Path length l (km)	_ _ _ _ _ × _ _

Terrain type(2)	_ _ _ _ _ _ _ _ _ _

Path profile (file name)	_ _ _ _ _ _ _ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)(3)	_

Polarization tilt angle jp (degrees)(3)	_ _ _ × _

Grazing angle F (mrad)(4)	_ _ _ × _ _ _

Average path clearance (k = ¥)  h¥ (m)(5)	_ _ _ × _ _ _

Path clearance from obstacle hob (m)	_ _ _ × _ _ _

Distance TX to biggest obstacle dob (km)	_ _ _ × _ _ _

Average terrain altitude amsl at (m)	_ _ _ × _ _ _

TX height above terrain he (m)	_ _ _ × _ _ _

RX height above terrain hr (m)	_ _ _ × _ _ _

Path profile roughness (PPR)(6)

PPR, r.m.s height, d = 1.0 km s1_1 (m)	_ _ _ × _ _ _

PPR, r.m.s height, d = 0.5 km s1_0.5 (m)	_ _ _ × _ _ _

PPR, r.m.s slope, d = 1.0 km s2_1 (mrad)	_ _ _ × _ _ _

PPR, r.m.s slope, d = 0.5 km s2_0.5 (mrad)	_ _ _ × _ _ _

Path inclination ep (mrad)(7)	_ _ _ × _ _ _

Average terrain slope et (mrad)(8)	_ _ _ × _ _ _

Tail points(9):

Tail point A1 (dB)	_ _ _ × _ _

Probability of A1 p1 (%)	_ _ × _ _ _ _ _

Tail point at probability p1/10 A2 (dB)	_ _ _ × _ _

Probability of refractivity p1 (%)	_ _ × _ _ _ _ _

Worst month to annual conversion factor	_ _ _ × _ _ _

Number of worst months for average	_ _

Table:

Fade depth and signal enhancement in (dB) percentage of average worst month (dB)

| Percentage of worst month | 0.001 | 0.005 | 0.01 | 0.1 | 1.0 | 10.0 |
| --- | --- | --- | --- | --- | --- | --- |
| Fade depth (dB) | – | – | – | – | – | – |
| Enhancement (dB) | – | – | – | – | – | – |

References:

Comments:

Notes to Table I-2:

(1)	See § 6.1 for list of country codes.

(2)	Terrain type (see § 6.3).

(3)	Polarization linear (L) or Circular (C). Polarization tilt angle is 0 for horizontal and 90 for vertical. For RHCP enter +45 and for LHCP enter –45°.

(4)	The grazing angle F is defined for specular reflection from the “average terrain profile” under normal atmospheric conditions (k = 4/3).

(5)	The average path clearance is defined at path height for k = ¥ above the “average terrain profile” at mid-path h¥  (k¥). hob is the path clearance of the most dominant obstacle for (k = 4/3) and dob is the distance from the transmitter to the most dominant obstacle.

(6)	Path profile roughness s1 is defined as the r.ms. value of the path profile heights referred to the path mean height, using for both, mean and r.ms. calculations sample distances of 1 and 0.5 km, excluding 1 km at the terminals. s2 is defined similar to s1, but using path profile slopes instead of heights.

(7)	The path inclination ep is defined as the angle between the path and a flat Earth.

(8)	The average terrain slope et is defined as the angle between the regression line through the path profile samples at 1 km intervals (“average terrain profile”), with 1 km intervals excluded at terminals (k = ¥).

(9)	Tail points (A1 (dB), p1; A2 (dB), p1/10): The “tail” is the relatively straight portion of the worst-month cumulative distribution for large fade depths, as plotted on a semi-logarithmic scale. Two points are defined on the tail. The first point (A1 p1) corresponds to the highest possible value of probability within the tail; the second point (A2 p1/10) corresponds to a point on the tail at a probability one decade lower than p1.

TABLE  I-3 – Line-of-sight diversity data

Path number	_ _ _ _ _ _

Transmit station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1)	_ _

TX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

TX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _

TX antenna height ag ht (m)	_ _ _

TX 3 dB beamwidth qt (degrees)	_ _ _ × _ _

TX antenna type	_ _ _ _ _ _ _ _ _ _

TX antenna diameter Dt (m)	_ _ × _ _

TX antenna feed type	_ _ _ _ _ _ _ _ _ _

Receive station A

RXa site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RXa country(1)	_ _

RXa latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RXa longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RXa altitude amsl hgr (m)	_ _ _ _

RXa antenna height ag hr (m)	_ _ _ _

RXa 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RXa antenna type	_ _ _ _ _ _ _ _ _ _

RXa antenna diameter D (m)	_ _ × _ _

RXa antenna feed type	_ _ _ _ _ _ _ _ _ _

RXa dynamic range (dB)	_ _ _ × _ _

RXa integration time (s)	_ _ _ _

Receive station B

RXb site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RXb country(1) 	_ _

RXb latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RXb longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RXb altitude amsl hgt (m)	_ _ _ _

RXb antenna height ag hgr (m)	_ _ _ _

RXb 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RXb antenna type	_ _ _ _ _ _ _ _ _ _

RXb antenna diameter D (m)	_ _ _ × _ _ _

RXb antenna feed type	_ _ _ _ _ _ _ _ _ _

RXb dynamic range (dB)	_ _ _ × _ _

RXb integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Path profile

Path length l (km)	_ _ _ _ × _ _

Terrain type (land/sea/mixed)	_ _ _ _ _ _ _ _ _ _

Path profile (file name)	_ _ _ _ _ _ _ _ _ _

Environment classification
(land, sea, coast)	_ _ _ _ _ _ _ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Diversity type	_ _ _ _ _

Frequency separation (MHz)	_ _ _ × _ _

Receive antenna angle separation (degrees)	_ _ _ × _ _

Receive antenna vertical spacing (m)	_ _ _ × _ _

Receive antenna horizontal spacing (m)	_ _ _ × _ _

Receive antenna gain difference (dB)	_ _ _ × _ _

Type of combiner	_ _ _ _ _ _

Improvement at 20 dB	_ _ _ _ × _ _ _

Improvement at 30 dB	_ _ _ _ × _ _ _

Measured fading occurrence factor P0, meas	_ _ _ × _ _ _

Predicted fading occurrence factor P0	_ _ _ × _ _ _

Space correlation coefficient K	_ × _ _ _

K measured directly? (Y/N)	_

Table:

Fade depth (dB) for reference receiver (FDR) and diversity protected receiver (combined signal, FDD) (dB) for percentage of average worst month

| Percentage of worst month | 0.001 | 0.01 | 0.1 | 1.0 | 10.0 |
| --- | --- | --- | --- | --- | --- |
| Fade depth reference (dB) | – | – | – | – | – |
| Fade depth diversity (dB) | – | – | – | – | – |

References:

Comments:

(1)	See § 6.1 for list country codes.

TABLE  I-4 – Line-of-sight clear sky XPD and CPA statistics

Path number	_ _ _ _ _ _

Transmit station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1)	_ _

TX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

TX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _

TX antenna height ag ht (m)	_ _ _ _

TX 3 dB beamwidth qt (degrees)	_ _ _ × _ _

TX antenna type	_ _ _ _ _ _ _ _ _ _

TX antenna diameter D (m)	_ _ _ × _ _ _

TX antenna feed type 	_ _ _ _ _ _ _ _ _ _

TX clear-sky level XPD0 (dB)	_ _ _ × _ _

TX maximum XPD level XPDmax (dB)	_ _ _ × _ _

TX maximum side-lobe angle (degrees)	_ _ _ × _ _

TX relative level of maximum side lobe (dB)	_ _ _ × _ _

Receive station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum XPD level XPDmax (dB)	_ _ _ × _ _

RX maximum side-lobe angle (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Path length l (km)	_ _ _ _ × _ _

Terrain type (land/sea/mixed)	_ _ _ _ _ _ _ _ _ _

Path profile (file name)	_ _ _ _ _ _ _ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Table:

Cross-polar discrimination exceeded for percentage of time, XPD (dB)

Co-polar attenuation exceeded for percentage of time, CPA (dB)

| Percentage of time | 0.001 | 0.01 | 0.1 |
| --- | --- | --- | --- |
| XPD (dB) | – | – | – |
| CPA (dB) | – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

TABLE  I-5 – Line-of-sight XPD and CPA statistics due to rain

Path number	_ _ _ _ _ _

Transmit station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1)	_ _

TX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

TX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _

TX antenna height ag ht (m)	_ _ _ _

TX 3 dB beamwidth qt (degrees)	_ _ _ × _ _

TX antenna type	_ _ _ _ _ _ _ _ _ _

TX antenna diameter D (m)	_ _ _ × _ _ _

TX antenna feed type 	_ _ _ _ _ _ _ _ _ _

TX clear-sky level XPD0 (dB)	_ _ _ × _ _

TX maximum XPD level XPDmax (dB)	_ _ _ × _ _

TX maximum side-lobe angle (degrees)	_ _ _ × _ _

TX relative level of maximum side lobe (dB)	_ _ _ × _ _

Receive station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

Path length l (km)	_ _ _ _ × _ _

Terrain type (land/sea/mixed)	_ _ _ _ _ _ _ _ _ _

Path profile (file name)	_ _ _ _ _ _ _ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _ _

Table:

Cross-polar discrimination exceeded for percentage of time, XPD (dB)

Co-polar attenuation exceeded for percentage of time, CPA (dB)

Rain rate exceeded for percentage of time, R (mm/h)

| Percentage of time | 0.001 | 0.01 | 0.1 |
| --- | --- | --- | --- |
| XPD (dB) | – | – | – |
| CPA (dB) | – | – | – |
| R (mm/h) | – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

TABLE  I-6 – Line-of-sight worst month multipath channel characteristics and outage times

Path number	_ _ _ _ _ _

Transmit station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1)	_ _

TX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

TX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _

TX antenna height ag ht (m)	_ _ _ _

TX 3 dB beamwidth qt (degrees)	_ _ _ × _ _

TX antenna diameter D (m)	_ _ _ × _ _ _

TX antenna tilt angle yt (degrees)	_ _ _ × _

TX antenna first side-lobe level (dB)	_ _ _ × _

TX antenna first side-lobe angle (degrees)	_ _ × _

Receive station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna diameter D (m)	_ _ _ × _ _ _

RX antenna tilt angle yr (degrees)	_ _ _ × _

RX antenna first side-lobe level (dB)	_ _ _ × _

RX antenna first side-lobe angle (degrees)	_ _ × _

Only for diversity: diversity type 	_ _ _ _ _

RXd diversity antenna height (m)	_ _ _ _

RXd 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RXd diversity antenna diameter (m)	_ _ _ × _ _ _

RXd antenna tilt angle yrd (degrees)	_ _ _ × _

RXd antenna first side-lobe level (dB)	_ _ _ × _

RXd antenna first side-lobe angle (degrees)	_ _ × _

Frequency separation (MHz)	_ _ _ × _ _

RX antenna angle separation (degrees)(2)	_ _ _ × _ _

RX antenna vertical spacing (m)	_ _ _ × _ _

RX antenna horizontal spacing (m)	_ _ _ × _ _

RX antenna gain difference(3) (dB)	_ _ _ × _ _

Type of combiner	_ _ _ _ _ _

Path length l (km)	_ _ _ × _ _

Terrain type (land/sea/mixed)	_ _ _ _ _ _ _ _ _ _

Path profile (file name)	_ _ _ _ _ _ _ _ _ _

Classification (land, sea, coast)	_ _ _ _ _ _ _ _ _ _

Measurement: Experiment No.	_ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Data resolution (dB)	_ _ _ × _ _ _

IBAD frequency separation(4) (MHz)	_ _ _ × _ _

Measurement bandwidth Bm (MHz)	_ _ _ × _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Modulation 	_ _ _ _ _ _ _ _

Data rate (Mbit/s)	_ _ _ × _ _

Equalizer used (Analogue/Digital/None)(5)	_

Flat fade margin (dB)	_ _ × _ _ _

Signature width W (GHz)	_ _ × _ _ _

Normalized signature depth la (GHz)	_ _ × _ _ _

Reference delay for la , ta (ns)	_ _ _ × _

Fading occurrence factor P0	_ _ _ × _ _ _ _

Time percentage for	| without diversity	| with diversity

BER = 10–3 (%)	| _ _ × _ _ _ _	| 	_ _ × _ _ _ _
   BER = 10–4 (%)	| _ _ × _ _ _ _	| 	_ _ × _ _ _ _
   BER = 10–5 (%)	| _ _ × _ _ _ _	| 	_ _ × _ _ _ _

IBAD or IBPD measured (A/P)	_

Table:

In-band amplitude dispersion (IBAD) (or, if IBAD not available, in-band power difference (IBPD) – indicate above) and simultaneously measured fade depth (dB) exceeded for percentage of worst month

| Percentage of worst month | 0.001 0.1 | 0.002 0.2 | 0.005 0.5 | 0.01 1 | 0.02 2 | 0.05 5 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IBAD without diversity (dB) | – – | – – | – – | – – | – – | – – | – |
| Fading without diversity (dB) | – – | – – | – – | – – | – – | – – | – |
| IBAD with diversity (dB) | – – | – – | – – | – – | – – | – – | – |
| Fading with diversity (dB) | – – | – – | – – | – – | – – | – – | – |

References:

Comments:

Notes to Table I-6:

(1)	See § 6.1 for list of country codes.

(2)	In case of sum and difference pattern antenna give angle between the two maxima of the difference pattern.

(3)	Two beam antenna: enter difference between maximum gains. Sum and difference pattern: give difference between maximum gain in the sum-pattern and average gain for the two maxima of the difference pattern.

(4)	IBAD frequency separation is the difference in MHz between two filters used for measuring the amplitude difference in the radio channel (preferably at equal distance from the centre of the radio channel). In case of IBPD the difference between the maximum and minimum frequency used should be given.

(5)	If an equalizer is used, the “time percentage of BER=...” should be reported after removal of the equalizer improvement. Note that no FEC should be applied.

Part II:  Earth-space path data

TABLE  II-1 – Slant path annual rain attenuation and rain rate statistics

Station number	_ _ _ _ _ _

Receive station
RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX rain zone(2)	_

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) E 	_ _ _ _ × _ _ _

Type of experiment	_ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Elevation angle (degrees)	_ _ × _ _

RG data concurrent? (Y/N)	_

Medium temperature (if radiometer) Tmed (K)	_ _ _ × _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

Attenuation presented as excess or total (E/T) 	_

Table(3):

Rain rate exceeded for percentage of the year, R (mm/h)

Attenuation exceeded for percentage of the year, A (dB)

| Percentage of year | 0.001 | 0.002 | 0.003 | 0.005 | 0.01 | 0.02 | 0.03 | 0.05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R (mm/h) A (dB) | – – | – – | – – | – – | – – | – – | – – | – – |
| Percentage of year | 0.1 | 0.2 | 0.3 | 0.5 | 1 | 2 | 3 | 5 |
| R (mm/h) A (dB) | – – | – – | – – | – – | – – | – – | – – | – – |
| Percentage of year | 10 | 20 | 30 | 50 |  |  |  |  |
| R (mm/h) A (dB) | – – | – – | – – | – – |  |  |  |  |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

(3)	It is preferred that cumulative distributions for individual years (= 12 consecutive months) be submitted rather than distributions averaged over several years. For beacon measurements an indication should be given, how the 0 dB reference level was established in the comments (i.e. by radiometer, by pre- and post event level linking, etc.).

TABLE  II-2 – Slant path worst-month* rain attenuation statistics

Station number	_ _ _ _ _ _

Receive station
RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX rain zone(2)	_

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) E 	_ _ _ _ × _ _ _

Type of experiment	_ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Elevation angle (degrees)	_ _ × _ _

RG data concurrent? (Y/N)	_

Medium temperature (if radiometer) Tmed (K)	_ _ _ × _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

Attenuation presented as excess or total (E/T) 	_

Table:

Rain rate exceeded for percentage of worst month, R (mm/h)

Attenuation exceeded for percentage of worst month, A (dB)

| Percentage of worst month | 0.001 | 0.002 | 0.003 | 0.005 | 0.01 | 0.02 | 0.03 | 0.05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R (mm/h) A (dB) | – – | – – | – – | – – | – – | – – | – – | – – |
| Percentage of worst month | 0.1 | 0.2 | 0.3 | 0.5 | 1 | 2 | 3 | 5 |
| R (mm/h) A (dB) | – – | – – | – – | – – | – – | – – | – – | – – |
| Percentage of worst month | 10 | 20 | 30 | 50 |  |  |  |  |
| R (mm/h) A (dB) | – – | – – | – – | – – |  |  |  |  |

References:

Comments:

*	For the definition of the worst month refer to Recommendation ITU-R  PN.581.

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R  PN.837.

TABLE  II-3 – Slant path fade duration statistics

Station number	_ _ _ _ _ _

Receive station
RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX rain zone(2)	_

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) E 	_ _ _ × _ _ _

Type of experiment	_ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Elevation angle (degrees)	_ _ × _ _

RG data concurrent? (Y/N)	_

Medium temperature (if radiometer) Tmed (K)	_ _ _ × _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ _ × _ _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

Attenuation presented as excess or total (E/T) 	_

Table a:

Fraction of total fading time (0-1) for a fade duration (s) at a given level (dB)

| Fade duration (s) | 1 | 6 | 10 | 18 | 60 | 180 | 600 | 1 800 | 3 600 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Level: 3 dB 5 dB 6 dB 10 dB 15 dB 20 dB 25 dB | – – – – – – – | – – – – – – – | – – – – – – – | – – – – – – – | – – – – – – – | – – – – – – – | – – – – – – – | – – – – – – – | – – – – – – – |

Table b:

Total fading time (s) and number of fades for a given level (dB)

| Level | T total (s) | No. of fades |
| --- | --- | --- |
| 3 dB 5 dB 6 dB 10 dB 15 dB 20 dB 25 dB |  |  |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

TABLE  II-4 – Slant path site diversity statistics

Station number	_ _ _ _ _ _

Receive station A	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RXa country(1)	_ _

RXa latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RXa longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RXa altitude amsl hgr (m)	_ _ _ _

RXa antenna height ag hr (m)	_ _ _ _

RXa 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RXa rain zone(2)	_

RXa antenna type	_ _ _ _ _ _ _ _ _ _

RXa antenna diameter D (m)	_ _ _ × _ _

RXa antenna feed type	_ _ _ _ _ _ _ _ _ _

RXa radome? (Y/N)	_ _ _

RXa figure of merit (dB(K–1))	_ _ _ × _ _

RXa clear-sky level XPD0 (dB)	_ _ _ × _ _

RXa dynamic range (dB)	_ _ _ × _ _

RXa integration time (s)	_ _ _ _

Receive station B	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RXb country(1) 	_ _

RXb latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RXb longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RXb altitude amsl hgd (m)	_ _ _ _

RXb antenna height ag hd (m)	_ _ _ _

RXb 3 dB beamwidth qd (degrees)	_ _ _ × _ _

RXb rain zone(2)	_

RXb antenna type	_ _ _ _ _ _ _ _ _ _

RXb antenna diameter D (m)	_ _ _ × _ _

RXb antenna feed type	_ _ _ _ _ _ _ _ _ _

RXb radome? (Y/N)	_ _ _

RXb figure of merit (dB(K–1))	_ _ _ × _ _

RXb clear-sky level XPD0 (dB)	_ _ _ × _ _

RXb dynamic range (dB)	_ _ _ × _ _

RXb integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) 	_ _ _ × _ _ _

Type of experiment	_ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Elevation angle (degrees)	_ _ × _ _

RG data concurrent? (Y/N)	_

Baseline length	_ _ _ × _ _ _

Baseline orientation (AZ) (degrees)	_ _ _ × _ _

Medium temperature (if radiometer) Tmed (K)	_ _ _ × _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

Attenuation presented as excess or total (E/T)	_

Table:

Site A, Site B and joint attenuation exceeded for percentage of time (dB)

Site A, Site B and joint rain rate exceeded for percentage of time (mm/h)

| Percentage of year | 0.001 0.1 10 | 0.002 0.2 20 | 0.003 0.3 30 | 0.005 0.5 50 | 0.01 1 | 0.02 2 | 0.03 3 | 0.05 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attenuation A (dB) Site A | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Attenuation B (dB) Site B | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Joint attenuation (dB) (diversity) | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Rain rate A (mm/h) Site A | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Rain rate B (mm/h) Site B | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Joint rain rate (mm/h) (diversity) | – – – | – – – | – – – | – – – | – – | – – | – – | – – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

TABLE  II-5a – Slant path annual XPD statistics

Station number	_ _ _ _ _ _

Receive station
RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) E 	_ _ _ × _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _ _

Elevation angle (degrees)	_ _ × _ _

Rain zone(2)	_

RG data concurrent? (Y/N)	_

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

Attenuation presented as excess or total (E/T) 	_

Cancellation method(3)	_ _ _ _ _ _ _ _ _ _

Table:

XPD not exceeded for percentage of time (dB)

Attenuation (dB) exceeded for percentage of time

Rain rate (mm/h) exceeded for percentage of time

| Percentage of year | 0.001 0.1 10 | 0.002 0.2 20 | 0.003 0.3 30 | 0.005 0.5 50 | 0.01 1 | 0.02 2 | 0.03 3 | 0.05 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XPD not exceeded (dB) | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Attenuation exceeded (dB) | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Rain rate exceeded (mm/h) | – – – | – – – | – – – | – – – | – – | – – | – – | – – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

(3)	For XPD cancellation, the following options apply: 1: none, 2: static, 3: software.

TABLE  II-5b – Slant path annual XPD statistics conditioned to CPA

Station number	_ _ _ _ _ _

Receive station
RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) E 	_ _ _ × _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Elevation angle (degrees)	_ _ × _ _

Rain zone(2)	_

RG data concurrent? (Y/N)	_

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

Attenuation presented as excess or total (E/T) 	_

Cancellation method(3)	_ _ _ _ _ _ _ _ _ _

Table:

Mean and standard deviation (dB) of XPD for given CPA intervals

XPD not exceeded at given percentage of measurement time (90% means low depolarization, 10% means strong depolarization), conditioned to CPA mean values (dB) (CPA interval given as: greater than lower bound and less or equal to upper bound)

| XPD and CPA between (dB) | 0-2 20-25 | 2-4 25-30 | 4-6 30-35 | 6-8 35-40 | 8-10 40-45 | 10-12 > 45 | 12-16 | 16-20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean | – – | – – | – – | – – | – – | – – | – | – |
| Standard deviation | – – | – – | – – | – – | – – | – – | – | – |
| 90% | – – | – – | – – | – – | – – | – – | – | – |
| 70% | – – | – – | – – | – – | – – | – – | – | – |
| 50% | – – | – – | – – | – – | – – | – – | – | – |
| 30% | – – | – – | – – | – – | – – | – – | – | – |
| 10% | – – | – – | – – | – – | – – | – – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

(3)	For XPD cancellation, the following options apply: 1: none, 2: static, 3: software.

TABLE  II-6 – Slant path statistics of amplitude scintillations

Station number	_ _ _ _ _ _

Receive station
RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB)	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) E 	_ _ _ × _ _ _

Type of experiment	_ _ _

Type of filtering	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Elevation angle (degrees)	_ _ × _ _

Type of data (SD/PP/FD) 	_ _

Rain zone(2)	_

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Water vapour concentration measurements

Type of instrument	_ _ _ _ _ _ _ _ _ _

Resolution (g/m3)	_ _ _ × _ _ _

Location	_ _ _ _ _ _ _ _ _ _

Average surface temperature (degrees C)	_ _ _ × _ _ _

Surface relative humidity (%)	_ _ _ × _ _ _

Table:

Scintillation fading exceeded for percentage of year (dB)

Scintillation fading exceeded for percentage of worst month (dB)

Scintillation fading not exceeded for percentage of year (dB)

Scintillation fading not exceeded for percentage of worst month (dB)

| Percentage of year or worst month |  | 0.001 0.1 10 | 0.002 0.2 20 | 0.003 0.3 30 | 0.005 0.5 50 | 0.01 1 | 0.02 2 | 0.03 3 | 0.05 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Year | – | – | – | – | – | – | – | – |
| Scintillation |  | – – | – – | – – | – – | – | – | – | – |
| exceeded (dB) | Worst | – | – | – | – | – | – | – | – |
|  | month | – – | – – | – – | – – | – | – | – | – |
|  | Year | – | – | – | – | – | – | – | – |
| Scintillation NOT |  | – – | – – | – – | – – | – | – | – | – |
| exceeded (dB) | Worst | – – | – – | – – | – – | – – | – – | – – | – – |
|  | month | – | – | – | – |  |  |  |  |

References:

Comments

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

TABLE  II-7 – Slant path standard deviations of scintillations

Station number	_ _ _ _ _ _

Receive station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1) 	_ _

RX latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX antenna diameter D (m)	_ _ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX clear-sky level XPD0 (dB)	_ _ _ × _ _

RX maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RX relative level of maximum side lobe (dB) 	_ _ _ × _ _

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Measurement: Experiment No.	_ _ _

Satellite name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Satellite orbital position (degrees) E 	_ _ _ × _ _ _

Type of experiment	_ _ _

Type of filtering	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Elevation angle (degrees)	_ _ × _ _

Type of data (SD/PP/FD)	_ _

Rain zone(2)	_

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Water vapour concentration measurements

Type of instrument	_ _ _ _ _ _ _ _ _ _

Resolution (g/m3)	_ _ _ × _ _ _

Location	_ _ _ _ _ _ _ _ _ _

Table:

Average surface temperature (degrees C), relative humidity (%) and
standard deviation of scintillation as function of season

|  | Season(3) | Average surface temperature (degrees C) | Average relative humidity (%) | Standard deviation of scintillations (dB) |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

(3)	Seasons in this context are to be defined as: WI (winter), SP (spring), SU (summer), AU (autumn), RA (rainy), DR (dry).

Part III:  Terrestrial trans-horizon path and rain scatter data

TABLE  III-1 – Clear-air trans-horizon basic transmission loss statistics

Station number	_ _ _ _ _ _

Frequency f (GHz) 	_ _ _ × _ _ _

Path length d (km)	_ _ _ _ × _ _

Scatter angle qs (mrad)	_ _ _ × _ _ _

Transmit station

TX site name and code	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1)	_ _

TX latitude LAT1 (–90..+90) (degrees) 	_ _ _ × _ _ _ _ _

TX longitude LON1 (0..360) (degrees) E 	_ _ _ × _ _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _ × _

TX antenna height ag ht (m)	_ _ _ _ × _

TX antenna horizon gain gt (dBi)	_ _ _ _ × _

Receive station

RX site name and code	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX latitude LAT2 (–90..+90) (degrees)	_ _ _ × _ _ _ _ _

RX longitude LON2 (0..360) (degrees) E 	_ _ _ × _ _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _ × _

RX antenna height ag hr (m)	_ _ _ _ × _

RX antenna horizon gain gr (dBi)	_ _ _ _ × _

Effective earth radius k	_ _ _ × _ _ _

CCIR climate (2)	_ _

TX rain zone(3)	_ _

RX rain zone(3)	_ _

Measurement: Experiment No.	_ _ _

Minimum monthly mean surface 
   refractivity Nsmin(4)	_ _ _ × _

Average annual sea-level refractivity N0(4)	_ _ _ × _

Maximum monthly mean sea-level 
   refractivity N0max(4)	_ _ _ × _

Average annual –100 N/km time percentage, b0 (%)	_ _ _ × _

Maximum monthly –100 N/km time 
   percentage b0max (%)	_ _ _ × _

Average annual refractivity gradient DN (N/km)	_ _ _ × _

Maximum monthly refractivity gradient DNmax (N/km)	_ _ _ × _

Aggregate zone A1 distance da1 (km)	_ _ _ _ × _

Aggregate zone A2 distance da2 (km)	_ _ _ _ × _

Aggregate zone B distance db (km)	_ _ _ _ × _

Aggregate zone C distance dc (km)	_ _ _ _ × _

Longest land section (A1 and/or A2) dtm (km)	_ _ _ _ × _

Longest inland section (A2) dlm (km) 	_ _ _ _ × _

Delta-h Dh (m)	_ _ _ _ × _

Minimum clearance height hm (m)	_ _ _ _ × _

TX effective antenna height hte (m)	_ _ _ _ × _

TX station horizon distance  dlt (km)	_ _ _ _ × _

TX station horizon elevation  qt (mrad)	_ _ _ _ × _

TX distance to coast (along path) dct  (km)	_ _ _ _ × _ _

RX effective antenna height hre (m)(5)	_ _ _ _ × _

RX station horizon distance dlr (km)	_ _ _ _ _ × _ _

RX station horizon elevation qr (mrad)	_ _ _ _ _ × _

RX distance to coast (along path) dcr (km)	_ _ _ _ _ × _

Data format (Code No.)(6)	_ _ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)  	_ _ _ _ × _ _ × _ _

Duration of valid data (hours)	_ _ _ _ _ × _ _

Data source	_ _ _ _

Path profile file name	_ _ _ _ _ _ _ _ _ _

Table(7):

Basic transmission loss NOT exceeded for percentage of total time TLt (dB)

Basic transmission loss NOT exceeded for percentage of year TLa (dB)

Basic transmission loss NOT exceeded for percentage of worst month TLwm (dB)

| Percentage of time | 0.001 0.1 10 | 0.002 0.2 20 | 0.003 0.3 30 | 0.005 0.5 50 | 0.01 1 99 | 0.02 2 100 | 0.03 3 | 0.05 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total period TLt (dB) | – – – | – – – | – – – | – – – | – – – | – – – | – – | – – |
| Single year TLa (dB) | – – – | – – – | – – – | – – – | – – – | – – – | – – | – – |
| Worst month TLwm (dB) | – – – | – – – | – – – | – – – | – – – | – – – | – – | – – |

References:

Comments:

Notes to Table III-1:

(1)	See § 6.1 for list of country codes.

(2)	The CCIR climate is defined as per Annex 1 of Recommendation ITU-R PN.617.

(3)	See Recommendation ITU-R PN.837.

(4)	All refractivity values are to be given in N-units.

(5)	For definition of minimum clearance height and effective antenna height see Recommendation ITU-R PN.452.

(6)	Code: 1: £ 1 s sampling; 2: 10 s–1 min sampling; 3: 1 min median; 4: hourly median; 5: specify in Comments.

(7)	This Table includes data of earlier Tables III-1, III-2 and III-3 of ex-CCIR Report 1144 as indicated in the data-source field. For data from the ITS report, the values of DSA1, DSA2, DSB and DSC should be treated as approximate.

TABLE  III-1a – Clear-air spot measurement data*

Station number	_ _ _ _ _ _

Environment	_ _ _ _ _

Path profile file name	_ _ _ _ _ _ _ _ _ _

Path length d (km)	_ _ _ _ × _ _

Path ground conductivity 
   (see Rec. ITU-R  PN.527) (S/m)	_ _ _ × _

Path ground permittivity 
   (see Rec. ITU-R  PN.527)	_ _ _ × _

Path average annual sea level 
   refractivity N0 (N-units)	_ _ _ × _

Transmit station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country	_ _

TX latitude LAT1 (–90..+90) (degrees) 	_ _ _ × _ _ _ _ _

TX longitude LON1  (0..360) (degrees) E 	_ _ _ × _ _ _ _ _

TX altitude amsl hgt (m)	_ _ _ _ × _

TX antenna height ag ht (m)	_ _ _ _ × _

TX antenna gain gt (dBi)	_ _ _ _ × _

Receive station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country	_ _

RX latitude LAT2  (–90..+90) (degrees) 	_ _ _ × _ _ _ _ _

RX longitude LON2 (0..360) (degrees) E 	_ _ _ × _ _ _ _ _

RX altitude amsl hgr (m)	_ _ _ _ × _

RX antenna height ag  hr (m)	_ _ _ _ × _

RX antenna gain gr (dBi)	_ _ _ _ × _

Climate (see Recommendation ITU-R PN.617)	_ _

Table:

Basic transmission loss relative to free space (dB) for different receive antenna heights and different frequencies (f1 ... f6 in MHz)

| RX antenna height ag | Type | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| hr | (open/concealed) | – | – | – | – | – | – |
|  |  | – | – | – | – | – | – |
|  |  | – | – | – | – | – | – |
|  |  | – | – | – | – | – | – |
|  |  | – | – | – | – | – | – |

*	This table forms a separate data base and is not included in the standard distribution format. For this table data are to be reported as the median value of a short (< 1 h) recording.

TABLE  III-2 – Rain scatter on terrestrial paths

Station number	_ _ _ _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Site separation d (km)	_ _ _ _ × _ _

Scatter geometry	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Scatter angle qs (mrad)	_ _ _ × _ _ _

Transmit station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX country(1)	_ _

TX latitude LAT1 (–90..+90) (degrees) 	_ _ _ × _ _ _ _

TX longitude  LON1 (0..360) (degrees) E 	_ _ _ × _  _ _ _

TX altitude amsl hg1 (m)	_ _ _ × _

TX antenna height ag h1 (m)	_ _ _ × _

TX antenna gain g1 (dBi)	_ _ _ _ × _ _

TX antenna efficiency h1  	_ × _ _

TX antenna 3 dB beamwidth q3 dB1 (degrees)	_ _ × _ _

TX highest side-lobe gain gj1 (dB)	_ _ × _

TX off-axis angle of highest side lobe dj1 (degrees)	_ _ × _

TX elevation q1 (degrees)	_ _ _ × _ _

TX distance to common volume (km)	_ _ _ _ _ × _

TX polarization (L/C)	_

TX polarization tilt angle jp1 (degrees)	_ _ _ × _

TX deviation of azimuth from direction 
TX to RX da1 (degrees)	_ _ _ × _ _

TX local horizon eh1 (degrees)	_ _ × _ _

Measurement: Experiment No.	_ _ _

Receive station

RX site name and code	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX latitude LAT2 (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX longitude LON2 (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX altitude amsl hg2 (m)	_ _ _ × _

RX antenna height ag h2 (m)	_ _ _ × _

RX antenna gain g2 (dBi)	_ _ _ _  × _ _

RX antenna efficiency h2 	_ × _ _

RX antenna 3 dB beamwidth q3 dB2 (degrees)	_ _ × _ _

RX highest side-lobe gain gj2 (dB)	_ _ × _

RX off-axis angle of highest side lobe dj2 (degrees)	_ _ × _

RX elevation q2 (degrees)	_ _ _ × _ _

RX distance to common volume (km)	_ _ _ _ _ × _

RX polarization (L/C)	_

RX polarization tilt angle jp2 (degrees)	_ _ _ × _

RX deviation of azimuth from direction 
RX to RX da2 (degrees)	_ _ _ × _ _

RX local horizon eh2 (degrees)	_ _ × _ _

Common volume height hcv (km)	_ _ × _

Rain height hFR (km)	_ _ × _

Rain climate zone(2)	_

Start date (yyyy.mm.dd) 	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd) 	_ _ _ _ × _ _ × _ _

Duration of valid data (days)	_ _ _ _ _ × _ _

Duration of scatter events (days)	_ _ _ _ _ × _ _

Data source	_ _ _ _

Table:

Transmission loss for percentage of total time (dB)

| Percentage of time | 0.001 | 0.003 0,01 | 0.01 | 0.03 | 0.1 | 0.3 |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission loss (dB) | – | – | – | – | – | – |
| Percentage of time | 1 | 3 | 10 | 30 | 90 |  |
| Transmission loss (dB) | – | – | – | – | – |  |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

Part IV:  Radiometeorological data

TABLE  IV-1 – Statistics of rain intensity

Station number	_ _ _ _ _ _

RG site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RG country(1)	_ _

RG latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RG longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RG altitude amsl hg (m)	_ _ _ _

Rain zone(2)	_

Measurement: Experiment No.	_ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG accumulation per tip (mm/tip)	_ × _ _ _ _

RG aperture (cm2)	_ _ _ _ × _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ _ × _ _ _

Percentage of year where R > 0.5 mm/h p0 (%)	_ × _ _

Table a:

Rain rate exceeded for percentage of year Ra (mm/h)

Rain rate exceeded for percentage of worst month Rwm (mm/h)

| Percentage of time | 0.001 0.1 | 0.002 0.2 | 0.003 0.3 | 0.005 0.5 | 0.01 1 | 0.02 2 | 0.03 3 | 0.05 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rain rate of year Ra (mm/h) | – – | – – | – – | – – | – – | – – | – – | – – |
| Rain rate of worst month Rwm (mm/h) | – – | – – | – – | – – | – – | – – | – – | – – |

Table b(3):

Monthly variation of rain

Rain rate exceeded for 0.01%, 0.1% and 1% of the month

Total accumulation of rain during one month

| Month of year | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R 0.01 (mm/h) |  |  |  |  |  |  |  |  |  |  |  |  |
| R 0.1 (mm/h) |  |  |  |  |  |  |  |  |  |  |  |  |
| R 1 (mm/h) |  |  |  |  |  |  |  |  |  |  |  |  |
| Total rain (mm) |  |  |  |  |  |  |  |  |  |  |  |  |
| Snow included(4) |  |  |  |  |  |  |  |  |  |  |  |  |
| RG up-time(5) (%) |  |  |  |  |  |  |  |  |  |  |  |  |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

(3)	Wherever possible, cumulative distributions for individual years (= 12 consecutive months) are preferred. For multi-year data, one form per year should be used. However, where long term climatic data have been analysed and it is impractical to submit individual yearly distributions, multi-year averages should be submitted with detailed description of the data and analysis parameters.

(4)	In geographic locations where precipitation in the form of ice or snow may occur, an explanation should be given as to when and how long these conditions prevail and how the rain gauge data is treated during these intervals. For Table b, the occurrence of snow is to be indicated as follows: 0: No snow, 1: Snow eliminated from statistics, 2: Equivalent water content of snow was included in the measurements, 3: Other (explain under Comments).

(5)	If outages occurred in the measurement of rain, explain under Comments whether rainfall occurred during the outage or not.

TABLE  IV-2 – Rain integration time conversion factor

Station number	_ _ _ _ _ _

RG site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RG country(1)	_ _

RG latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RG longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RG altitude amsl hg (m)	_ _ _ _

Rain zone(2)	_

Measurement: Experiment No.	_ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG accumulation per tip (mm/tip)	_ × _ _ _ _

RG aperture (cm2)	_ _ _ _ × _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Conversion factor time Tcf (min)	_ _ _ × _ _

Table(3):

Conversion factor found for percentage of year

| Percentage of year | 0.001 0.1 | 0.002 0.2 | 0.003 0.3 | 0.005 0.5 | 0.01 1 | 0.02 2 | 0.03 3 | 0.05 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conversion factor | – – | – – | – – | – – | – – | – – | – – | – – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

(3)	The conversion factor is the ratio of the 1 min rainfall rate of the rainfall rate measured with a different integration time Tcf. The conversion factors should be primarily calculated for the typical time interval used by the local Met office for reporting rain rates. For reporting different conversion factors, use a separate sheet for each one.

TABLE  IV-3 – Annual statistics of sky noise temperature

Station number	_ _ _ _ _ _

Rain zone(1)	_

Radiometer station

Type of RM	_ _ _ _ _ _ _ _ _ _

RM site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RM country(2)	_ _

RM latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RM longitude (0..360) (degrees) E	_ _ _ × _ _ _ _

RM altitude amsl hgr (m)	_ _ _ _

RM antenna height ag hr (m)	_ _ _ _

RM 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RM antenna type	_ _ _ _ _ _ _ _ _ _

RM antenna diameter D (m)	_ _ _ × _ _ _

RM antenna feed type	_ _ _ _ _ _ _ _ _ _

RM radome? (Y/N)	_ _ _

RM maximum side-lobe angle < 4 (degrees)	_ _ _ × _ _

RM relative level of maximum side lobe (dB)	_ _ _ × _ _

RM dynamic range (dB)	_ _ _ × _ _

RM integration time (s)	_ _ _ _

RM bandwidth (MHz)	_ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ _ × _ _ _

Data resolution (K)	_ _ _ _ × _ _ _

Measurement: Experiment No.	_ _ _

Rain gauge type	_ _ _ _ _ _ _ _ _ _

RG resolution (mm/h)	_ _ _ _ × _ _ _

RG integration time (s)	_ _ _ _

RG location relative to receiver	_ _ _ _ _ _ _ _ _ _

RG data concurrent? (Y/N)	_

RG accumulation per tip (mm/tip)	_ × _ _ _ _

RG aperture (cm2)	_ _ _ _ × _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _ _

Elevation angle (degrees)	_ _ × _ _

Radiosonde data collected? (Y/N)	_

Temperature recorded? (Y/N)	_

Humidity measured? (Y/N)	_

Pressure measured? (Y/N)	_

Cloud cover recorded? (Y/N)	_

Table:

Sky noise temperature Tsky exceeded for percentage of time (K)

Rain rate R  exceeded for percentage of time (mm/h)

| Percentage of year | 0.001 0.1 10 | 0.002 0.2 20 | 0.003 0.3 30 | 0.005 0.5 50 | 0.01 1 | 0.02 2 | 0.03 3 | 0.05 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tsky exceeded (K) | – – – | – – – | – – – | – – – | – – | – – | – – | – – |
| Rain rate exceeded (mm/h) | – – – | – – – | – – – | – – – | – – | – – | – – | – – |

References:

Comments:

(1)	See Recommendation ITU-R PN.837.

(2)	See § 6.1 for list of country codes.

TABLE  IV-4 – Statistics of mean surface refractivity

Station number	_ _ _ _ _ _

Site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Country(1)	_ _

Latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

Longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

Altitude amsl hg (m)	_ _ _ _

Rain zone(2)	_

Distance from the sea (km)	_ _ _ _ × _

Measurement: Experiment No.	_ _ _

Type of measuring equipment(3)	_

Resolution (%)	_ _ × _ _ _

For refractivity difference:

TX height of lower measurement point a.g. (m)	_ _ _ × _

TX height difference between measurement 
TX   points (m)	_ _ _ × _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Time interval between two measurements (min)	_ _ _ × _

Table:

Ground refractivity Ns (N-units) and refractivity gradient DN (N-units/km) for percentage of time

| Percentage of time |  | 0.01 90 | 0.03 97 | 0.1 99 | 0.3 99.7 | 1 99.9 | 3 99.97 | 10 99.99 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ground refractivity | Year | – – | – – | – – | – – | – – | – – | – – | – |
| (N-units) | Worst month | – – | – – | – – | – – | – – | – – | – – | – |
| Refractivity gradient | Year | – – | – – | – – | – – | – – | – – | – – | – |
| (N-units/km) | Worst month | – – | – – | – – | – – | – – | – – | – – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See Recommendation ITU-R PN.837.

(3)	Type of equipment: 1: Refractometer, 2: Psychrometer + Barometer, 3: Thermometer + Hygrometer + Barometer, 4: Other (explain under Comments).

Part V:  Terrestrial land mobile data

Table V-1 – Terrestrial land mobile wideband statistics

Station number	_ _ _ _ _ _

Frequency f (MHz)	_ _ _ × _ _ _

Base station

BS site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

BS country(1)	_ _

BS latitude LATb (–90..+90) (degrees) 	_ _ _ × _ _ _ _

BS longitude LONb (0..360) (degrees) E 	_ _ _ × _ _ _ _

BS altitude amsl hgb (m)	_ _ _ × _

Maximum building height around BS (m)	_ _ _

BS antenna height ag hb (m)	_ _ _ × _

BS antenna gain gb (dBi)	_ _ _ _ × _ _

BS antenna horizontal beamwidth qbh (degrees)	_ _ _ _ × _ _

BS antenna vertical beamwidth qbv (degrees)	_ _ _ _ × _ _

BS e.i.r.p. (W)	_ _ _ _ × _ _

BS polarization (L/C)	_

BS polarization tilt angle jp (degrees)	_ _ × _

Mobile station

MS site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

MS altitude amsl hmg (m)	_ _ _ × _

MS antenna height ag hma (m)	_ _ _ × _

MS antenna gain gm (dBi)	_ _ _ _ × _ _

Path

Minimum radio path length lmin  (km)	_ _ _ × _

Maximum radio path length lmax  (km)	_ _ _ × _

Average radio path length lavg  (km)	_ _ _ × _

Environment type(2)	_ _

Terrain characteristics(3)	_ _

Mean orientation of street (degrees)	_ _ _

Average width of street (m)	_ _

Mean building height (m)	_ _ _

Maximum building height (m)	_ _ _

Packing density of buildings (0..1)	_ × _ _

Mean tree height (m)	_ _ _

Maximum tree height (m)	_ _ _

Packing density of trees	_ × _ _

Measurement: Experiment No.	_ _ _

Year	_ _ _ _

Season	_ _

Duration d (days)	_ _ _ _

80% CDF of basic transmission loss, L80 (dB)	_ _ × _

90% CDF of basic transmission loss, L90 (dB)	_ _ × _

90% mean delay – 10% mean delay CDF (ms)	_ _ × _

80% value of delay spread CDF, S80 (ms)	_ _ × _

90% value of delay spread CDF, S90 (ms)	_ _ × _

Table:

Delay window (ms) and delay interval (ms) at percentage of Cumulative Distribution Function (CDF)

Correlation bandwidth (MHz) for 90%, 50% and 37% correlation at given percentage of CDF

|  | Delay window for percentage of impulse energy (ms) |  |  | Delay interval for x dB below peak (ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 90% | 75% | 50% | 9 dB | 12 dB | 15 dB |
| At 80% of CDF | – | – | – | – | – | – |
| At 90% of CDF | – | – | – | – | – | – |
| At 95% of CDF | – | – | – | – | – | – |
| At 99% of CDF | – | – | – | – | – | – |

|  | Correlation bandwidth (MHz) for percentage correlation |  |  |
| --- | --- | --- | --- |
|  | 90% | 50% | 37% |
| At 20% of CDF | – | – | – |
| At 10% of CDF | – | – | – |
| At 5% of CDF | – | – | – |
| At 1% of CDF | – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See § 6.3 for environment types. Measurements should only cover one type of environment.

(3)	Terrain characteristics can be given as: FL: flat, HI: hilly, MO: mountainous.

TABLE  V-2 – Terrestrial land mobile narrow-band statistics

Station number	_ _ _ _ _ _

Frequency f (MHz)	_ _ _ × _ _ _

Base station

BS site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

BS country(1) 	_ _

BS latitude LATb (–90..+90) (degrees) 	_ _ _ × _ _ _ _

BS longitude LONb (0..360) (degrees) E 	_ _ _ × _ _ _ _

BS altitude amsl hgb (m)	_ _ _ × _

BS antenna height ag hb (m)	_ _ _ × _

BS antenna gain gb (dBi)	_ _ _ _ × _ _

BS antenna horizontal beamwidth qbh (degrees)	_ _ _ _ × _ _

BS antenna vertical beamwidth qbv (degrees)	_ _ _ _ × _ _

BS e.i.r.p. (W)

BS polarization (L/C)	_

BS polarization tilt angle jp (degrees)	_ _ × _

Maximum height of buildings around BS (m)	_ _ _

Mobile station

MS site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

MS altitude amsl hgm (m)	_ _ _ × _

MS antenna height ag hm (m)	_ _ _ × _

MS antenna gain gm (dBi)	_ _ _ _ × _ _

Path

Minimum radio path length lmin (km)	_ _ _ × _

Maximum radio path length lmax (km)	_ _ _ × _

Average radio path length lavg (km)	_ _ _ × _

Environment type(2)	_ _

Terrain characteristics(3)	_ _

Mean orientation of street (degrees)	_ _ _

Average width of street (m)	_ _

Mean building height (m)	_ _ _

Maximum building height (m)	_ _ _

Packing density of buildings (0...1)	_ × _ _

Mean tree height (m)	_ _

Maximum tree height (m)	_ _

Measurement: Experiment No.	_ _ _

Year	_ _ _ _

Season	_ _

Duration d (days)	_ _ _ _

Table:

Fade depth not exceeded for percentage of locations (dB)

| Percentage of locations |  | 0.1 95 |  | 1 99 |  | 10 99.9 |  | 50 |  | 70 |  | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fade depth A (dB) | – – |  | – – |  | – – |  | – |  | – |  | – |  |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See § 6.3 for environment types. Measurements should only cover one type of environment.

(3)	Terrain characteristics can be given as: FL: flat, HI: hilly, MO: mountainous.

Part VI:  Terrestrial broadcasting data

TABLE  VI-1 – Terrestrial broadcasting signal level variation with time

Station number	_ _ _ _ _ _

Broadcast TX station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX station code	_ _ _ _ _

TX country(1)	_ _

TX latitude  LAT1 (–90..+90) (degrees) 	_ _ _ × _ _ _ _

TX longitude  LON1  (0..360) (degrees) E 	_ _ _ × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ × _

TX antenna height amt ht (m)	_ _ _ × _

TX antenna polarization (H, V, C)	_

TX e.r.p. of horizontal polarization 
TX component (dBW)	_ _ _ × _

TX  e.r.p. of vertical polarization 
TX component (dBW)	_ _ _ × _

TX frequency f (MHz)	_ _ _ × _ _ _

Measurement: Experiment No.	_ _ _

Broadcast RX station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX latitude LAT2 (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX longitude LON2 (0..360) (degrees) E 	_ _ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ × _

RX antenna height ag hr (m)	_ _ _ × _

RX antenna directional/omni (D/O)	_

RX environment(2)	_ _

Field strength in the plane of polarization 
RX of RX antenna (dB(mV/m))	_ _ _ × _ _

Meteorology

TX rain zone(3)	_ _ _ _

RX rain zone(3)	_ _ _ _

Minimum monthly mean surface 
RX refractivity Nsmin (N-units)	_ _ _ _ _ _ _ _

Average annual sea-level 
RX refractivity N0 (N-units)	_ _ _ _ _ _ _ _

Other meteorological data	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Path length l (km)	_ _ _ _ × _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Start time daily recordings (hh:mm)	_ _ × _ _

End time daily recordings (hh:mm)	_ _ × _ _

Total measurement time d (days)	_ _ _ _ × _ _ _

Sampling continuous/intermittent (C/I)	_

Sampling interval (s)	_ _ _ _ × _

Table:

Field strength exceeded for percentage of time (dB(mV/m))

| Percentage of time | 50 | 10 | 5 | 1 | 0.5 | 0.1 |
| --- | --- | --- | --- | --- | --- | --- |
| E – total time (dB(mV/m)) | – | – | – | – | – | – |
| E – worst month (dB(mV/m)) | – | – | – | – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See § 6.3 for environment types.

(3)	See Recommendation ITU-R PN.837.

TABLE  VI-2 – Terrestrial broadcasting signal level variation with location

Station number	_ _ _ _ _ _

Broadcast TX station

TX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

TX station code	_ _ _ _ _

TX country(1)	_ _

TX latitude  LAT1 (–90..+90) (degrees) 	_ _ _ × _ _ _ _

TX longitude  LON1 (0..360) (degrees) E 	_ _ _ × _ _ _ _

TX altitude amsl hgt (m)	_ _ _ × _

TX antenna height amt ht (m)	_ _ _ × _

TX antenna polarization (H, V, C)	_

TX e.r.p. of horizontal polarization 
TX component (dBW)	_ _ _ × _

TX e.r.p. of horizontal polarization 
TX component (dBW)	_ _ _ × _

TX frequency f (MHz)	_ _ _ × _ _ _

Measurement: Experiment No.	_ _ _

Broadcast RX station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX latitude LAT2 (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX longitude LON2 (0..360) (degrees) E 	_ _ _ _ × _ _ _ _

RX altitude amsl hgr (m)	_ _ _ × _

RX antenna height ag hr (m)	_ _ _ × _

RX antenna directional/omni (D/O)	_

RX environment(2)	_ _

Meteorology

TX rain zone(3)	_ _ _ _

RX rain zone(3)	_ _ _ _

Minimum monthly mean surface
RX refractivity Nsmin (N-units)	_ _ _ _ _ _ _ _

Average annual sea-level
RX refractivity N0 (N-units)	_ _ _ _ _ _ _ _

Other meteorological data	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

Path length l (km)	_ _ _ _ × _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Start time daily recordings (hh:mm)	_ _ × _ _

End time daily recordings (hh:mm)	_ _ × _ _

Total measurement time d (days)	_ _ _ _ × _ _ _

Type of measurement(4)	_

For group measurements: number of measurements	_ _ _

Horizontal distance covered (m)(5)	_ _ _ _ _

Size of area (km2)	_ _ _ _ _ × _

Number of individual measurements made	_ _ _

Height variation across measured area (m)	_ _ _ _

Median value of field strength (dB(mV/m))	_ _ _ _ × _ _

Minimum value of field strength (dB(mV/m))	_ _ _ _ × _ _

Maximum value of field strength (dB(mV/m))	_ _ _ _ × _ _

Standard deviation of field strength	_ _ _ _ × _ _

Table:

Measured field strength at given percentage of locations E (dB(mV/m))

| Percentage of locations | 10 | 50 | 90 |
| --- | --- | --- | --- |
| Measured E (dB(mV/m)) | – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	Indicate the predominant environment at the receive sites. Use codes from § 6.3.

(3)	See Recommendation ITU-R PN.837.

(4)	The following categories of measurement are distinguished:
1: Group (cluster of individual spot measurements)
2: Continuous measurement while antenna is moved
3: Sampling (as continuous but sampled at intervals).

(5)	It is preferred that measurements extend over a distance of 10 to 20 wavelengths.

Part VII:  Data for mobile-satellite services

TABLE  VII-1 – Wideband statistics for mobile-satellite links

Station number	_ _ _ _ _ _

Type of experiment (Land/Sea/Aeronautical)	_

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees)	_ _ _ × _

Average elevation angle (degrees)	_ _ × _ _ _

Signal source (transmitter on i.e.: satellite, 
   helicopter, aeroplane, balloon, tower)	_ _ _ _ _ _ _ _ _ _

If satellite: satellite name 	_ _ _ _ _ _ _ _ _ _

orbital position (degrees) E 	_ _ _ × _ _ _

Mobile station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX average latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX average longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX average altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)     	_ _ × _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX 3 dB beamwidth Az qrAz (degrees)	_ _ _ × _ _

RX 3 dB beamwidth El qrEl (degrees)	_ _ _ × _ _

RX gain (dBi)	_ _ × _

RX antenna diameter D (m)	_ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX multipath reduction? (Y/N)	_

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Measurement: Experiment No.	_ _ _

Year	_ _ _ _

Season	_ _

Duration (h)	_ _ _ _

Environment (only fill applicable part)

Land mobile terrain type(2)	_ _ _ _ _ _ _ _ _ _

Land mobile building type(2)	_ _ _ _ _ _ _ _ _ _

Land mobile vegetation type(2)	_ _ _ _ _ _ _ _ _ _

Land mobile surface type(3)	_ _ _ _ _ _ _ _ _ _

Sea state	_ _ _ _ _ _ _ _ _ _

Aeronautical ground type(4)	_ _ _ _ _ _ _ _ _ _

80% CDF of basic transmission loss, L80 (dB)	_ _ × _

90% CDF of basic transmission loss, L90 (dB)	_ _ × _

90% mean delay CDF (ms)	_ _ × _

10% mean delay CDF (ms)	_ _ × _

80% value of delay spread CDF, S80 (ms)	_ _ × _

90% value of delay spread CDF, S90 (ms)	_ _ × _

Table:

Delay window (ms) and delay interval (ms) at percentage of CDF

Correlation bandwidth (MHz) for 90%, 50% and 37% correlation at given percentage of CDF

|  | Delay window for percentage of impulse energy (ms) |  |  | Delay interval for x dB below peak (ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 90% | 75% | 50% | 9 dB | 12 dB | 15 dB |
| At 80% of CDF | – | – | – | – | – | – |
| At 90% of CDF | – | – | – | – | – | – |
| At 95% of CDF | – | – | – | – | – | – |
| At 99% of CDF | – | – | – | – | – | – |

|  | Correlation bandwidth (MHz) for percentage correlation |  |  |
| --- | --- | --- | --- |
|  | 90% | 50% | 37% |
| At 20% of CDF | – | – | – |
| At 10% of CDF | – | – | – |
| At 5% of CDF | – | – | – |
| At 1% of CDF | – | – | – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	See § 6.3 for environment types. Measurements should only cover one type of environment.

(3)	Terrain characteristics can be given as: FL: flat, HI: hilly, MO: mountainous.

(4)	Aeronautical ground types: see § 6.3.

TABLE  VII-2 – Narrow-band statistics of maritime mobile-satellite links

Station number	_ _ _ _ _ _

TX frequency f (GHz)	_ _ _ × _ _ _

TX polarization (L/C)	_

TX polarization tilt angle jp (degrees)	_ _ _ × _

RX average elevation angle (degrees)	_ _ × _ _ _

RX rain zone(1)	_

Signal source (transmitter on i.e: satellite, 
   helicopter, aeroplane, balloon, tower)	_ _ _ _ _ _ _ _ _ _

If satellite: satellite name 	_ _ _ _ _ _ _ _ _ _

orbital position (degrees) E 	_ _ _ × _ _ _

TX antenna gain towards mobile (dBi)	_ _ × _

Mobile station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(2)	_ _

RX average latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX average longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX average altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ × _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna gain (dBi)	_ _ × _

RX antenna diameter D (m)	_ _ × _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX multipath reduction? (Y/N)	_

RX dynamic range (dB)	_ _ _ × _ _

RX integration distance (wavelength)	_ _ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Measurement: Experiment No.	_ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Environment

Sea state	_ _ _ _ _ _ _ _ _ _

Wave height (m)	_ _

Table a:

Fade depth (dB relative to LOS) exceeded and XPD (dB) NOT exceeded for percentage of time

| Percentage of time | 0.1 | 1 | 5 | 10 | 30 | 50 | 90 | 99 | 99.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fade depth A (dB) | – | – | – | – | – | – | – | – | – |
| XPD (dB) | – | – | – | – | – | – | – | – | – |

Table b:

Fade duration (s) NOT exceeded for percentage of locations at given fade levels

| Percentage of locations | 0.1 | 1 | 5 | 10 | 30 | 50 | 90 | 99 | 99.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 dB 02 dB 05 dB 10 dB | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – |

References:

Comments:

(1)	See Recommendation ITU-R PN.837.

(2)	See § 6.1 for list of country codes.

TABLE  VII-3 – Narrow-band statistics of land mobile-satellite links

Station number	_ _ _ _ _ _

TX frequency f (GHz)	_ _ _ × _ _ _

TX polarization (L/C)	_

TX polarization tilt angle jp (degrees)	_ _ _ × _

RX average elevation angle (degrees)	_ _ × _ _ _

RX rain zone(1)	_

Signal source (transmitter on i.e.: satellite, 
   helicopter, aeroplane, balloon, tower)	_ _ _ _ _ _ _ _ _ _

If satellite: satellite name 	_ _ _ _ _ _ _ _ _ _

orbital position (degrees) E 	_ _ _ × _ _ _

TX antenna gain towards mobile (dBi)	_ _ × _

Mobile station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(2)	_ _

RX average latitude (–90..+90) (degrees) 	_ _ _ × _ _ _ _

RX average longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

Average velocity of vehicle (km/h)	_ _ _

RX average altitude amsl hgr (m)	_ _ _ _

RX antenna height ag hr (m)	_ _ × _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX 3 dB beamwidth in Az qrAz (degrees)	_ _ _ × _ _

RX 3 dB beamwidth in El qrEl (degrees)	_ _ _ × _ _

RX antenna gain (dBi)	_ _ × _

RX antenna diameter D (m)	_ _ × _ _

RX radome? (Y/N)	_ _ _

RX polarization (L/C)	_

RX polarization tilt angle jp (degrees)	_ _ _ × _

RX multipath reduction? (Y/N)	_

RX dynamic range (dB)	_ _ _ × _ _

RX minimum signal-to-noise ratio (dB)	_ _ × _

RX integration time (s)	_ _ _ _

RX bandwidth (Hz)	_ _ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Measurement: Experiment No.	_ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration of measurement in 
   this environment (h)	_ _ _ _ × _ _ _

Season	_ _

Environment

Land mobile terrain type(3)	_ _ _ _ _ _ _ _ _ _

Building type	_ _ _ _ _ _ _ _ _ _

Vegetation type	_ _ _

Surface condition (wet, dry, snow)	_ _ _ _

Table a:

Fade depth (dB relative to LOS) exceeded and XPD (dB) NOT exceeded for percentage of time

| Percentage of time | 0.1 | 1 | 5 | 10 | 30 | 50 | 90 | 99 | 99.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fade depth A (dB) | – | – | – | – | – | – | – | – | – |
| XDP (dB) | – | – | – | – | – | – | – | – | – |

Table b:

Fade duration reported in seconds or metres (s/m)

Fade duration (s) or (m) NOT exceeded for percentage of locations at given fade levels

| Percentage of locations | 0.1 | 1 | 5 | 10 | 30 | 50 | 90 | 99 | 99.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 dB 02 dB 05 dB 10 dB | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – |

References:

Comments:

(1)	See Recommendation ITU-R PN.837.

(2)	See § 6.1 for list of country codes.

(3)	See § 6.3 for environment types. Measurements should preferably only cover one type of environment. Also enter the terrain characteristics: FL: flat, HI: hilly, MO: mountainous (Example: 33, HI means suburban, hilly).

TABLE  VII-4 – Narrow-band statistics of aeronautical mobile-satellite links

Station number	_ _ _ _ _ _

Frequency f (GHz)	_ _ _ × _ _ _

Polarization (L/C)	_

Polarization tilt angle jp (degrees) 	_ _ _ × _

Signal source

Satellite name 	_ _ _ _ _ _ _ _ _ _

Orbital position (degrees) E	_ _ _ × _ _ _

TX antenna gain towards mobile (dBi)	_ _ × _

Aeronautical mobile station

RX site name	_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

RX country(1)	_ _

RX start latitude (–90..+90) (degrees)  	_ _ _ × _ _ _ _

RX start longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX end latitude (–90..+90) (degrees)	_ _ _ × _ _ _ _

RX end longitude (0..360) (degrees) E 	_ _ _ × _ _ _ _

RX average altitude amsl hgr (m)	_ _ _ _ _

RX antenna type	_ _ _ _ _ _ _ _ _ _

RX 3 dB beamwidth qr (degrees)	_ _ _ × _ _

RX antenna gain (dBi)	_ _ × _

RX antenna diameter D (m)	_ _ _ × _ _ _

RX antenna feed type	_ _ _ _ _ _ _ _ _ _

RX radome? (Y/N)	_ _ _

RX figure of merit (dB(K–1))	_ _ _ × _ _

RX multipath reduction? (Y/N)	_

RX dynamic range (dB)	_ _ _ × _ _

RX integration time (s)	_ _ _ _

Data sampling interval (s)	_ _ _ _ × _ _ _

Calibration interval (days)	_ _ _ × _ _

Data resolution (dB)	_ _ _ _ × _ _ _

Measurement: Experiment No.	_ _ _

Start date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

End date (yyyy.mm.dd)	_ _ _ _ × _ _ × _ _

Duration d (days)	_ _ _ _ × _ _ _

Average elevation angle (degrees)	_ _ × _

Average velocity of aircraft (knots)	_ _ _

Environment

Ground types(2)	_ _ _ _ _ _ _ _ _ _

Sea state	_ _ _ _ _ _ _ _ _ _

Table a:

Fade depth relative to LOS (dB) exceeded and XPD (dB) NOT exceeded for percentage of time

| Percentage of time | 0.1 | 1 | 5 | 10 | 30 | 50 | 90 | 99 | 99.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fade depth A (dB) | – | – | – | – | – | – | – | – | – |
| XPD (dB) | – | – | – | – | – | – | – | – | – |

Table b:

Fade duration (s) NOT exceeded for percentage of locations at given fade levels

| Percentage of locations | 0.1 | 1 | 5 | 10 | 30 | 50 | 90 | 99 | 99.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 dB 02 dB 05 dB 10 dB | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – | – – – – |

References:

Comments:

(1)	See § 6.1 for list of country codes.

(2)	Aeronautical ground types: sea water, fresh water, marsh, moist soil, dry soil, sea ice, glacier ice, wet snow, dry snow.

# 6.	Auxiliary information for submission of data

### 6.1	Country codes used

To facilitate geographical region assignment of the data, 2-character country codes as per International Organization for Standardization (ISO) Standard 3166:1988, are used. The list is based on the standard of 31 December 1992 and is sorted in alphabetical order. No political meaning whatsoever is associated with this list.

AFGHANISTAN	AF

ALBANIA	AL

ALGERIA	DZ

AMERICAN SAMOA	AS

ANDORRA	AD

ANGOLA	AO

ANGUILLA	AI

ANTARCTICA	AQ

ANTIGUA AND BARBUDA	AG

ARGENTINA	AR

ARMENIA	AM

ARUBA	AW

AUSTRALIA	AU

AUSTRIA	AT

AZERBAIJAN	AZ

BAHAMAS	BS

BAHRAIN	BH

BANGLADESH	BD

BARBADOS	BB

BELARUS	BY

BELGIUM	BE

BELIZE	BZ

BENIN	BJ

BERMUDA	BM

BHUTAN	BT

BOLIVIA	BO

BOSNIA AND HERZEGOVINA	BA

BOTSWANA	BW

BOUVET ISLAND	BV

BRAZIL	BR

BRITISH INDIAN OCEAN TERRIT.	IO

BRUNEI DARUSSALAM	BN

BULGARIA	BG

BURKINA FASO	BF

BURUNDI	BI

CAMBODIA	KH

CAMEROON	CM

CANADA	CA

CAPE VERDE	CV

CAYMAN ISLANDS	KY

CENTRAL AFRICAN REPUBLIC	CF

CHAD	TD

CHILE	CL

CHINA	CN

CHRISTMAS ISLAND	CX

COCOS (KEELING) ISLANDS	CC

COLOMBIA	CO

COMOROS	KM

CONGO	CG

COOK ISLANDS	CK

COSTA RICA	CR

CÔTE D'IVOIRE 	CI

CROATIA	HR

CUBA	CU

CYPRUS	CY

CZECH REPUBLIC	CZ

DENMARK	DK

DJIBOUTI	DJ

DOMINICA	DM

DOMINICAN REPUBLIC	DO

EAST TIMOR	TP

ECUADOR	EC

EGYPT	EG

EL SALVADOR	SV

EQUATORIAL GUINEA	GQ

ESTONIA	EE

ETHIOPIA	ET

FALKLAND ISLANDS (MALVINAS)	FK

FAROE ISLANDS	FO

FIJI	FJ

FINLAND	FI

FRANCE	FR

FRENCH GUIANA	GF

FRENCH POLYNESIA	PF

FRENCH SOUTHERN TERRITORIES	TF

GABON	GA

GAMBIA	GM

GEORGIA	GE

GERMANY	DE

GHANA	GH

GIBRALTAR	GI

GREECE	GR

GREENLAND	GL

GRENADA	GD

GUADELOUPE	GP

GUAM	GU

GUATEMALA	GT

GUINEA	GN

GUINEA-BISSAU	GW

GUYANA	GY

HAITI	HT

HEARD AND MC DONALD ISLANDS	HM

HONDURAS	HN

HONG KONG	HK

HUNGARY	HU

ICELAND	IS

INDIA	IN

INDONESIA	ID

IRAN (ISLAMIC REPUBLIC OF)	IR

IRAQ	IQ

IRELAND	IE

ISRAEL	IL

ITALY	IT

JAMAICA	JM

JAPAN	JP

JORDAN	JO

KAZAKHSTAN	KZ

KENYA	KE

KIRIBATI	KI

KOREA, DEM. PEOPLE’S REPUBL. OF	KP

KOREA, REPUBLIC OF	KR

KUWAIT	KW

KYRGYSTAN	KG

LAO PEOPLE’S DEMOCRATIC REPUBLIC	LA

LATVIA	LV

LEBANON	LB

LESOTHO	LS

LIBERIA	LR

LIBYAN ARAB JAMAHIRIYA	LY

LIECHTENSTEIN	LI

LITHUANIA	LT

LUXEMBOURG	LU

MACAU	MO

MADAGASCAR	MG

MALAWI 	MW

MALAYSIA	MY

MALDIVES	MV

MALI	ML

MALTA	MT

MARSHALL ISLANDS	MH

MARTINIQUE	MQ

MAURITANIA	MR

MAURITIUS	MU

MEXICO	MX

MICRONESIA (FEDERATE STATES OF)	FM

MOLDOVA, REPUBLIC OF	MD

MONACO 	MC

MONGOLIA	MN

MONTSERRAT	MS

MOROCCO	MA

MOZAMBIQUE	MZ

MYANMAR	MM

NAMIBIA	NA

NAURU	NR

NEPAL	NP

NETHERLANDS	NL

NETHERLANDS ANTILLES	AN

NEUTRAL ZONE	NT

NEW CALEDONIA	NC

NEW ZEALAND	NZ

NICARAGUA 	NI

NIGER	NE

NIGERIA	NG

NIUE	NU

NORFOLK ISLAND	NF

NORTHERN MARIANA ISLANDS	MP

NORWAY	NO

OMAN	OM

PAKISTAN	PK

PALAU	PW

PANAMA	PA

PAPUA NEW GUINEA	PG

PARAGUAY	PY

PERU	PE

PHILIPPINES	PH

PITCAIRN	PN

POLAND	PL

PORTUGAL	PT

PUERTO RICO	PR

QATAR	QA

REUNION	RE

ROMANIA	RO

RUSSIAN FEDERATION	RU

RWANDA 	RW

SAINT HELENA	SH

SAINT KITTS AND NEVIS	KN

SAINT LUCIA	LC

SAINT PIERRE AND MIQUELON	PM

SAINT-VINCENT AND THE GRENADINES	VC

SAMOA	WS

SAN MARINO	SM

SAO TOME AND PRINCIPE	ST

SAUDI ARABIA	SA

SENEGAL	SN

SEYCHELLES	SC

SIERRA LEONE	SL

SINGAPORE	SG

SLOVAC REPUBLIC	CS

SLOVENIA	SI

SOLOMON ISLANDS	SB

SOMALIA	SO

SOUTH AFRICA	ZA

SPAIN	ES

SRI LANKA 	LK

SUDAN	SD

SURINAME	SR

SVALBARD AND JAN MAYEN ISLANDS	SJ

SWAZILAND 	SZ

SWEDEN	SE

SWITZERLAND	CH

SYRIAN ARAB REPUBLIC	SY

TAIWAN, PROVINCE OF CHINA	TW

TAJIKISTAN	TJ

TANZANIA, UNITED REPUBLIC OF	TZ

THAILAND	TH

THE FORMER YUGOSLAV REPUBLIC
OF MACEDONIA	YM

TOGO	TG

TOKELAU	TK

TONGA	TO

TRINIDAD AND TOBAGO	TT

TUNISIA	TN

TURKEY	TR

TURKMENISTAN	TM

TURKS AND CAICOS ISLANDS	TC

TUVALU	TV

UGANDA	UG

UKRAINE	UA

UNITED ARAB EMIRATES	AE

UNITED KINGDOM	GB

UNITED STATES OF AMERICA	US

URUGUAY	UY

UZBEKISTAN	UZ

VANUATU	VU

VATICAN CITY STATE	VA

VENEZUELA	VE

VIET NAM	VN

VIRGIN ISLANDS (BRITISH)	VG

VIRGIN ISLANDS (U. S.)	VI

WALLIS AND FUTUNA ISLANDS	WF

WESTERN SAHARA	EH

YEMEN, REPUBLIC OF	YE

YUGOSLAVIA	YU

ZAIRE	ZR

ZAMBIA	ZM

ZIMBABWE	ZW

## 6.2	Definition of format for submitting path profile information

The path profile to be submitted with data relating to terrestrial links (Tables I and III) are specified in the following way: (preferred: DOS format ASCII File)

| ____________× PFL LAT |  | LON | ...file name, containing path code |
| --- | --- | --- | --- |
| TX CNTRY ___ ___×___ |  | ___×___ | ...country code, latitude, longitude |
| RX CNTRY ___ ___×___ |  | ___×___ | ...country code, latitude, longitude |
| DATE PROFILE TAKEN: |  | _____×___ | ...when was source map/DTBS created |
| SOURCE MAP- SCALE: 1: |  | _______ | ...if profile was retrieved from map |
| SOURCE DTBS-RES. (km): |  | ______ | ...if dig. terrain database was used |
| First point TX or RX: __ |  |  | ...pfl starts at trans or rcv site |
| Total path length (km): |  | _____×__ | ...distance TX to RX in km |
| Number of points: |  | ______ | ...number of points reported |

--------------------------------------------------------------------------

| =Distance= from first point (km) | Ground height amsl (m) | Coverage code (0-99) | Height of cover (m) |  |
| --- | --- | --- | --- | --- |

--------------------------------------------------------------------------

| 0.000 | ____._ | __ | ___ | ...actual profile starts here |
| --- | --- | --- | --- | --- |
| ___.___ | ____._ | __ | ___ |  |
| ___.___ | ____._ | __ | ___ |  |
| ___.___ | ____._ | __ | ___ |  |
| ___.___ | ____._ | __ | ___ |  |

Only the first two columns (distance and ground height) are mandatory. The coverage code and the height of cover (trees, buildings) are optional (see § 6.3).

The name of the ASCII file reflects the path code. This is a 4-character table name followed by the four digit path number. For example, for Table III-1, this is a four-digit number preceded by "3-1-". (Exception: Table III-1a, where the file name contains the seven character long path number preceded by "3".)

All path profile files have the extension ".PFL".

The file consists of 15 lines of header information followed by the profile information (one line per data point). The file can be created easily by setting up a spreadsheet with a column width of 12 for the first (A) column and 10 for the second (B), third column (C) and fourth column (D).

The following fields are used:

Profile file name (8 characters + ".PFL")

| Transmitter country code: | – – | (see § 6.1) |
| --- | --- | --- |
| Transmitter latitude: | – – – × – – – – | (decimal degrees; North = positive) |
| Transmitter longitude: | – – – × – – – – | (decimal degrees; 0-360, East = positive) |
| Receiver country code: | – – | (see § 6.1) |
| Receiver latitude: | – – – × – – – – | (decimal degrees; North = positive) |
| Receiver longitude: | – – – × – – – – | (decimal degrees; 0-360, East = positive) |
| Date profile taken: | YYYY.MM.DD | (this relates to the date the map or the digital terrain database was created – if not known, fill in the date when the profile has been extracted) |
| First point TX or RX: | – | (enter T if 0 km means transmit site, enter R if 0 km means receive site) |
| Total path length (km): | – – – – × – | (distance TX to RX in km) |
| Number of points: | – – – – – – | (number of points in the profile) |

-----------------------------------

| =Distance= from first point (km) | Ground height amsl (m) | Coverage code (0-99) |
| --- | --- | --- |
| ------- × ----------- × ------------- |  |  |
| XXXX.XXX XXXX.XX |  |  |

The lines from 15 onwards should only contain numeric information, namely:

1.	the distance from the origin in kilometres with 3 decimals; the decimal point should be located in the 8th character position from the left margin;

2.	the height of the ground (not tree canopy or top of buildings) above mean sea level in metres with 2 decimals; the decimal point should be located in the 20th character position from the left margin;

3.	(optional) the terrain cover classification with a two-digit integer number at positions 30 and 31 from the left margin;

4.	(optional) the height of the terrain cover.

## 6.3	Terrain cover types

| 00 |  | Unknown |  |
| --- | --- | --- | --- |
| 10 | RURAL OPEN |  |  |
|  | 11 | pastures, grassland |  |
|  | 12 | low crop fields |  |
|  | 13 | high crop fields (wine, hops, ...) |  |
|  | 19 | park land |  |
| 20 | TREE COVERED |  |  |
|  | 21 | irregularly spaced sparse trees |  |
|  | 22 | orchard (regularly spaced) |  |
|  | 23 | deciduous trees (irregularly spaced) |  |
|  | 24 | deciduous trees (regularly spaced) |  |
|  | 25 | coniferous trees (irregularly spaced) |  |
|  | 26 | coniferous trees (regularly spaced) |  |
|  | 27 | mixed tree forest |  |
|  | 28 | tropical rain forest |  |

| 30 | BUILT-UP AREA |  |
| --- | --- | --- |
|  | 31 | sparse houses |
|  | 32 | village centre |
|  | 33 | suburban |
|  | 34 | dense suburban |
|  | 35 | urban |
|  | 36 | dense urban |
|  | 37 | industrial zone |
| 40 | DRY GROUND |  |
|  | 42 | sand dunes |
|  | 43 | desert |
| 50 | WET TERRAIN (no trees) |  |
|  | 52 | marshlands |
|  | 54 | mud flats |
| 60 | FRESH WATER |  |
| 70 | SEA WATER |  |
| 80 | CRYOSPHERE |  |
|  | 82 | sea ice |
|  | 83 | freshwater ice |
|  | 84 | glacier |
|  | 86 | dry snow |
|  | 88 | wet snow |
| 90 | OTHER | (specify) |

___________________