RECOMMENDATION  ITU-R  P.369-6

REFERENCE  ATMOSPHERE  FOR  REFRACTION

(Question ITU-R 201/3)

(1959-1963-1966-1978-1982-1990-1992-1994)

Rec. ITU-R P.369-6

The ITU Radiocommunication Assembly,

considering

a)	that the long-term mean dependence of the refractive index n of the atmosphere at radio frequencies upon the height h is well expressed by an exponential law:

n(h)    1    a    exp (– b h)	(1)

where:

a and b : parameters which can be determined statistically for different climates;

b)	that there is a need for a common reference to facilitate comparisons of calculations,

recommends

1.	that the reference atmosphere for refraction be computed by means of the following formula:

n(h)    1    N0    10–6    exp (–h / h0)	(2)

where:

N0    315:	average value of atmospheric refractivity extrapolated to sea level (N-units) (see Recommenda-
tion ITU-R P.453)

h0    7.35 km, and

h :	height above sea level (km).

_________________