SECTION  6E:	IONOSPHERIC  PROPAGATION  PREDICTION  AT  FREQUENCIES  BETWEEN
ABOUT  1.6  AND  30  MHz

RECOMMENDATION  371-6

CHOICE  OF  INDICES  FOR  LONG-TERM  IONOSPHERIC  PREDICTIONS

(Question 34/6)

(1963-1970-1974-1978-1982-1986-1990)

Rec. 371-6

The CCIR,

UNANIMOUSLY  RECOMMENDS

1.	that the 12-month running mean sunspot number R12 be adopted as the index to be used for all ionospheric predictions for dates more than twelve months ahead of the date of the last observed value of R12;

2.	that R12, or the 12-month running mean value of the indices IG, or F, be adopted as the index to be used for predicting monthly median values of foF2 and M(3000)F2 for dates certainly up to six, and perhaps up to twelve months ahead of the date of the last observed values: substantially equivalent results should be obtainable by the use of any of these indices;

3.	that F12 be adopted as the index to be used for predicting monthly median values of foE and foF1 for dates certainly up to six, and perhaps up to twelve months ahead of the date of the last observed value;

4.	that caution be shown in the use of the recommended indices at high magnetic latitudes, where the resulting ionospheric predictions may not be sufficiently accurate.

ANNEX  I

1.	Introduction

The concept of indices for long-term ionospheric predictions relies on the assumption that the important characteristics of the ionosphere, such as the critical frequencies of the various layers and the MUF factor M(3000)F2, depend in a systematic way on certain measurable quantities concerned with solar radiation. It should however be noted that the correlation between these indices and the actual ionospheric characteristics does not necessarily imply a causal relationship, but rather an indication of associated phenomena. Changes of solar activity in general contain three components:

–	a fairly regular component with a period of about 11 years, which represents the well known cycle of solar activity;

–	a component that has a quasi-period of about a year or a little less; and

–	erratic fluctuations with periods of less than a month.

2.	Sunspot numbers

For studies of the main component of the solar cycle the 12-month running mean sunspot number R12 is used because the resultant smoothing considerably reduces complicated rapidly-varying components but does not obscure the slowly varying component.

The definition of R12 is:

(1)

in which Rk is the mean of the daily sunspot numbers for a single month k, and R12 is the smoothed index for the month represented by k = n.

The two main disadvantages in the use of R12 are:

–	the most recent available value is necessarily centred on a month at least 6 months earlier than the present time;

–	it cannot be used to predict the shorter-term variation in solar activity.

Nonetheless, R12 appears to be the most useful parameter for long-term studies and predictions concerning the F2 layer.

3.	Index F

Consistent and reasonably long series of observations of the solar radio noise flux at about 10 cm wavelength have been made by Canadian, Japanese and other laboratories. The monthly mean, F, of the daily values at Ottawa, expressed in units of 10–22 Wm–2 Hz–1, should be regarded as the reference data for this index. F is more closely correlated with E-layer critical frequency than are noise flux values at other wavelengths [Kundu, 1960]. As the solar flux observations are only available from 1947, the sunspot numbers remain one of the longest series of observations of a natural phenomenon. Therefore, the continued collection and recording of sunspot observations is encouraged.

4.	Ionospherically derived solar activity index, IG

An index IG, an ionospherically derived solar activity index for prediction of foF2 has been developed 
[Liu et al., 1983] especially for use with the Oslo version of the CCIR maps (see also Opinion 82).

5.	Other indices

An index IF 2 has been developed, based on measurement of foF2 at noon at long-established ionospheric observatories. Minnis [1964] showed that IF 2 was more closely correlated with foF2 than any other index available at that time. A further index, T, developed in Australia [IPSD, 1968] uses measured data from 30 stations and is based on the mean of the monthly median values of foF2 at each hour. Apart from the use of data from all hours, the derivation of the index is similar to that for IF 2.

6.	Correlation between various indices

The following figures and equations show relationships between observed values of R12, F, IG12 and IF 2.

Figure 1 shows the mean relationship between R12 and F as determined from data for the years 1947-1966 [Joachim, 1966a, 1966b]. This corresponds to the expression:

F  =  R12  +  46  +  23 e–0.05 R12	(2)

Stewart and Leftin [1972] have examined the relationship between R12 and F12. They propose the equation, also evaluated in Fig. 1, of:

(3)

On the basis of data for the years 1943-1981 [Liu et al., 1983], the relationship between R12 and IG12 is given by:

(4)

Several workers have considered the relationship between R12 and IF 2. Using data for the years 1938-1960, Barclay [1962] concluded that values are consistent with:

(5)

This expression is shown evaluated in Fig. 2.

Joachim [1966a] has examined the relationship between R12 and IF 2 separately for increasing and for decreasing solar activity. His overall mean relationship (see Fig. 2) is given by:

IF 2  =  (2.05  +  0.001  R12 ) R12(0.946  –  0.00047 R12 )  –  20	(6)

Muggleton and Kouris, [1968] have produced separate expressions for the relationship between R12 and IF 2 for each half solar cycles between 1938 and 1958.

Table I gives relationships for different months between the index T and R12 [IPSD, 1968].

It should be noted that slightly different forms of the above relationships will apply to different sunspot cycles.

TABLE  I  –  Relationship between T,
Ionospheric index used by Australian Administration and R12

T  =  c  +  d  ×  R12

| Month | Ordinates at origin (c) | Slope (d) |
| --- | --- | --- |
| January February March April May June July August September October November December | –1.58 0.81 –0.65 –0.54 –0.06 0.34 –0.18 1.19 2.70 1.87 2.45 –0.04 | 0.96 0.94 0.97 0.99 1.00 1.01 1.00 0.97 0.93 0.93 0.92 0.96 |

FIGURE  1.....[D01]  =  13.5 cm

FIGURE  2.....[D02]  =  15 cm

7.	The prediction of indices

There is as yet no method whereby it is possible to predict accurately indices for the next sunspot cycle, or, more generally, for a cycle which has not yet begun. Indices that have been calculated by using harmonic analysis or by using empirical and statistical laws applied to observations over some earlier and even recent cycles, have not proved useful in predicting those for a new cycle. After a sunspot minimum has been observed, future development of the cycle can be extrapolated to a certain extent, although the deviations have been observed to be rather extreme.

A study undertaken by the Secretariat of the CCIR in 1956 of several different kinds of prediction methods concluded that it was not possible to improve on an error of approximately 10 in the prediction of the solar index, R12. Only the statistical methods (comparison of cycles or autocorrelation) are considered useful, the methods based on harmonic analysis having never produced satisfactory results.

In the United States of America R12 is predicted using an improvement of the McNish-Lincoln objective method by Stewart and Ostrow [1970]. First a mean cycle is computed from all past values of R12 starting from the sunspot minimum of each cycle through eleven years thereafter. For prediction of a value in the current cycle the first approximation is the value of the mean cycle at the stated time after minimum (Fig. 3). This estimate is improved by adding a correction proportional to the departure of the last observed value for the current cycle from the mean cycle. With the current computer programmes a new prediction for each month of the remainder of the cycle can be made as soon as a new observed value becomes available. The statistical uncertainty of the prediction is fairly small for the first few months after the last observed value but becomes large for predictions 12 months or more in advance. As soon as a minimum is identified, new correction factors can be computed by including the observed values for the preceding cycle, for application to the new cycle.

FIGURE  3.....[D03]  =  16 cm

In the USSR the prediction of relative sunspot numbers is based upon one of the different prediction methods given by Vitinski [1973].

Predictions of IG12 are also prepared regularly in the United Kingdom, using a modified version of the McNish-Lincoln method. Details are given by Smith [1986], who has shown that the resulting predictions of IG12 are more accurate than those of R12, especially for prediction periods more than one year ahead.

Predictions of the index IF 2, six months into the future, are prepared regularly in the United Kingdom by extrapolating the 12-monthly running mean value. The procedure used to determine the future trend is to match values for the current solar cycle of activity with those for earlier cycles; extension is by reference to the magnitude and rate of change at the same phase in the former cycles [Smith, 1968]. Assessments made subjectively by this method are normalized to be consistent with values indicated by alternative means at times near solar maximum and minimum, where the cycle has turning points. Suitable statistical relationships, such as the variation of R12 for different times after sunspot maximum and minimum in terms of its maximum value for the cycle, serve to provide the required information on cycle behaviour, with sunspot numbers so derived being converted to values of IF 2 using the formulae given in § 6 above.

Predictions of F12, based on the McNish-Lincoln method, are carried out by the CCIR Secretariat.

Measured and predicted values of R, F, IG, IF 2 and their 12-month running means are published in the monthly circulars of the CCIR as well as in the ITU Telecommunication Journal.

8.	Comparison of indices

Studies of the prediction errors in foF2 resulting from the use of IG12 and R12 have been reported in [CCIR, 1982-1986a, b, c and d]. These comparisons were made using data obtained at many stations, covering a range of latitudes, longitudes and periods of comparison. Similar studies have been reported [Wilkinson, 1982; CCIR, 1985]. These studies were undertaken using at least ten years of observations at 13 vertical incidence stations. Comparisons were made using IG, R12, IF 2 and T indices as well as others.

It is clear from these studies that there is little advantage to be gained in using one index over another. However, on balance there is a slight degree of favourability for ionospherically derived indices, for example IG12. While the difference in accuracy shown by the use of this index is small, it is in the direction that would be expected from the use of a purely ionospheric index.

REFERENCES

BARCLAY, L. W. [1962] Variations in the relation between sunspot number and IF 2. J. Atmos. Terr. Phys., 24, 547-549.

IPSD [1968] The development of the ionospheric index T. Report IPS-R11. Ionospheric Prediction Service, Sydney, Australia.

JOACHIM, M. [1966a]. Study of correlation of the three basic indices of ionospheric propagation: R12, IF 2 and F. Nature, Vol. 210, 289-290.

JOACHIM, M. [1966b] Un effet d'hysteresis ionospherique. C.R. Acad. Sci. (Paris), B, 263, 92-94.

KUNDU, M. R. J. [1960] Solar radio emission on centimeter waves and ionization of the E-layer of the ionosphere. J. Geophys. Res., 65, 3903-3907.

LIU, R. Y., SMITH, P. A. and KING, J. W. [1983] A new solar index which leads to improved foF2 predictions using the CCIR Atlas. Telecomm. J., Vol. 50, VIII, 408-414.

MINNIS, C. M. [1964] Ionospheric indices. Advances in Radio Research, Vol. II, Ed. J. A. Saxton. Academic Press, London and New York.

MUGGLETON, L. M. and KOURIS, S. S. [1968] Relation between sunspot number and the ionospheric index IF 2. Radio Sci., 3, 1109-1110.

SMITH, P. A. [1968] An ionospheric prediction system based on the index IF 2. J. Atmos. Terr. Phys., 30, 177-185.

SMITH, P. A. [1986] Some techniques used to predict solar activity through the 11-year cycle. Solar-Terrestrial Predictions: Proceedings of a Workshop at Meudon, France, 18-22 June, 1984, p. 8, Ed. P. A. Simon, G. Heckman and M. A. Shea, published by National Oceanic and Atmospheric Administration, 325 Broadway, Boulder, Colorado 80303, USA, and Air Force Geophysics Laboratory, Hanscom AFB, Bedford, Mass. 01731, USA.

STEWART, F. G. and LEFTIN, M. [1972] Relationship between Ottawa 10.7 cm solar radio noise flux and Zurich sunspot number. Telecomm. J., 39, 159-169.

STEWART, F. G. and OSTROW, S. M. [1970] Improved version of the McNish-Lincoln method for prediction of solar activity. Telecomm. J., 37, 228-232.

VITINSKI, J. I. [1973] Cycles and Predictions of Solar Activity (in Russian). Akademiia Nauk, Leningrad, USSR.

WILKINSON, P. J. [1982] A comparison of monthly indices of the ionospheric F region. IPS Tech. Rep. R-41. Ionospheric Prediction Service, Sydney, Australia.

CCIR Documents

[1982-86]: a. 6/181 (France); b. 6/188 (German Democratic Republic); c. 6/204 (Japan); d. 6/207 (People’s Republic of China).

[1985]: IWP 6/1 Doc. 256 (USA).

_________________