SECTION  P-C:	GROUND  AND  OBSTACLE  EFFECTS

RECOMMENDATION  ITU-R  P.526-4

PROPAGATION  BY  DIFFRACTION

(Question ITU-R 202/3)

(1978-1982-1992-1994-1995)

Rec. ITU-R P.526-4

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to provide engineering information for the calculation of field strengths over diffraction paths,

recommends

1	that the methods described in Annex 1 be used for the calculation of field strengths over diffraction paths, which may include a spherical Earth surface, or irregular terrain with different kinds of obstacles.

ANNEX  1

# 1	Introduction

Although diffraction is produced only by the surface of the ground or other obstacles, account must be taken of the mean atmospheric refraction on the transmission path to evaluate the geometrical parameters situated in the vertical plane of the path (angle of diffraction, radius of curvature, height of obstacle). For this purpose, the path profile has to be traced with the appropriate equivalent Earth radius (Recommendation ITU-R P.834). If no other information is available, an equivalent Earth radius of 8 500 km may be taken as a basis.

# 2	Fresnel ellipsoids and Fresnel zones

In studying radiowave propagation between two points A and B, the intervening space can be subdivided by a family of ellipsoids, known as Fresnel ellipsoids, all having their focal points at A and B such that any point M on one ellipsoid satisfies the relation:

(1)

where n is a whole number characterizing the ellipsoid and n = 1 corresponds to the first Fresnel ellipsoid, etc., and l is the wavelength.

As a practical rule, propagation is assumed to occur in line-of-sight, i.e. with negligible diffraction phenomena if there is no obstacle within the first Fresnel ellipsoid.

The radius of an ellipsoid at a point between the transmitter and the receiver is given by the following formula:

(2)

or, in practical units:

(3)

where f is the frequency (MHz) and d1 and d2 are the distances (km) between transmitter and receiver at the point where the ellipsoid radius (m) is calculated.

Some problems require consideration of Fresnel zones which are the zones obtained by taking the intersection of a family of ellipsoids by a plane. The zone of order n is the part between the curves obtained from ellipsoids n and n – 1, respectively.

# 3	Diffraction over a spherical Earth

The additional transmission loss due to diffraction over a spherical Earth can be computed by the classical residue series formula (Recommendation ITU-R P.368). At long distances over the horizon, only the first term of this series is important. This first term can be written as the product of a distance term, F, and two height gain terms, GT and GR. These terms can be obtained either from simple formulae or from nomograms.

## 3.1	Numerical calculation

### 3.1.1	Influence of the electrical characteristics of the surface of the Earth

The extent to which the electrical characteristics of the surface of the Earth influence the diffraction loss can be determined by calculating a normalized factor for surface admittance, K, given by the formulae:

in self-consistent units:

(4)

for horizontal polarization and:

(5)

for vertical polarization;

or, in practical units:

(4a)

(5a)

where:

ae :	effective radius of the Earth (km)

e :	effective relative permittivity

s :	effective conductivity (S/m) and

f :	frequency (MHz).

Typical values of K are shown in Fig. 1.

If K is less than 0.001, the electrical characteristics of the Earth are not important. For values of K greater than 0.001, the appropriate formulae given below should be used.

FIGURE  1/PN.526...[D01] = 3 CM

### 3.1.2	Diffraction field strength formulae

The diffraction field strength, E, relative to the free-space field strength, E0, is given by the formula:

(6)

where X is the normalized length of the path between the antennas at normalized heights Y1 and Y2 (and where  is generally negative).

In self-consistent units:

(7)

(8)

or, in practical units:

(7a)

(8a)

where:

d :	path length (km)

ae :	equivalent Earth’s radius (km)

h :	antenna height (m)

f :	frequency (MHz).

b is a parameter allowing for the type of ground and for polarization. It is related to K by the following semi-empirical formula:

(9)

For horizontal polarization at all frequencies, and for vertical polarization above 20 MHz over land or 300 MHz over sea, b may be taken as equal to 1.

For vertical polarization below 20 MHz over land or 300 MHz over sea, b must be calculated as a function of K. However, it is then possible to disregard e and write:

(9a)

where s is expressed in S/m, f (MHz) and k is the multiplying factor of the Earth’s radius.

The distance term is given by the formula:

(10)

The height gain term, G(Y) is given by the following formulae:

for		Y  >  2	(11)

For Y < 2 the value of G(Y) is a function of the value of K computed in § 3.1.1:

for  10 K	<	Y  <  2	(11a)

for  K / 10	<	Y  <  10 K	(11b)

for		Y  <  K / 10	(11c)

## 3.2	Calculation by nomograms

Under the same approximation condition (the first term of the residue series is dominant), the calculation may also be made using the following formula:

(12)

where:

E :	received field strength

E0 :	field strength in free space at the same distance

d :	distance between the extremities of the path

h1 and h2 :	heights of the antennas above the spherical Earth.

The function F (influence of the distance) and H (height-gain) are given by the nomograms in Figs. 2, 3, 4 and 5.

These nomograms (Figs. 2 to 5) give directly the received level relative to free space, for k = 1 and k = 4/3, and for frequencies greater than approximately 30 MHz. k is the effective Earth radius factor, defined in Recommendation ITU-R P.310. However, the received level for other values of k may be calculated by using the frequency scale for k = 1, but replacing the frequency in question by a hypothetical frequency equal to f / k2 for Figs. 2 and 4 and f / µ § for Figs. 3 and 5.

Very close to the ground the field strength is practically independent of the height. This phenomenon is particularly important for vertical polarization over the sea. For this reason Fig. 5 includes a heavy black vertical line AB. If the straight line should intersect this heavy line AB, the real height should be replaced by a larger value, so that the straight line just touches the top of the limit line at A.

NOTE 1 – Attenuation relative to free space is given by the negative of the values given by equation (12). If equation (12) gives a value above the free-space field, the method is invalid.

# 4	Diffraction over obstacles and irregular terrain

Many propagation paths encounter one obstacle or several separate obstacles and it is useful to estimate the losses caused by such obstacles. To make such calculations it is necessary to idealize the form of the obstacles, either assuming a knife-edge of negligible thickness or a thick smooth obstacle with a well-defined radius of curvature at the top. Real obstacles have, of course, more complex forms, so that the indications provided in this Recommendation should be regarded only as an approximation.

In those cases where the direct path between the terminals is much shorter than the diffraction path, it is necessary to calculate the additional transmission loss due to the longer path.

The data given below apply when the wavelength is fairly small in relation to the size of the obstacles, i.e., mainly to VHF and shorter waves (f > 30 MHz).

FIGURE  2/PN.526...[D02] = 21 CM

FIGURE  3/PN.526...[D03] = 21 CM

FIGURE  4/PN.526...[D04] = 21 CM

FIGURE  5/PN.526...[D05] = 21 CM

## 4.1	Single knife-edge obstacle

In this extremely idealized case (Figs. 6a) and 6b)), all the geometrical parameters are combined together in a single dimensionless parameter normally denoted by n which may assume a variety of equivalent forms according to the geometrical parameters selected:

(13)

(14)

(15)

(16)

where:

h :	height of the top of the obstacle above the straight line joining the two ends of the path. If the height is below this line, h is negative

d1 and d2 :	distances of the two ends of the path from the top of the obstacle

d :	length of the path

q :	angle of diffraction (rad); its sign is the same as that of h. The angle q is assumed to be less than about 0.2 rad, or roughly 12°

a1 and a2 :	angles between the top of the obstacle and one end as seen from the other end. a1 and a2 are of the sign of h in the above equations.

NOTE 1 – In equations (13) to (16) inclusive h, d, d1, d2 and l should be in self-consistent units.

Figure 7 gives, as a function of n, the loss (dB) caused by the presence of the obstacle. For n greater than –0.7 an approximate value can be obtained from the expression:

(17)

## 4.2	Finite-width screen

Interference suppression for a receiving site (e.g. a small earth station) may be obtained by an artificial screen of finite width transverse to the direction of propagation. For this case the field in the shadow of the screen may be calculated by considering three knife-edges, i.e. the top and the two sides of the screen. Constructive and destructive interference of the three independent contributions will result in rapid fluctuations of the field strength over distances of the order of a wavelength. The following simplified model provides estimates for the average and minimum diffraction loss as a function of location. It consists of adding the amplitudes of the individual contributions for an estimate of the minimum diffraction loss and a power addition to obtain an estimate of the average diffraction loss. The model has been tested against accurate calculations using the Uniform Theory of Diffraction (UTD) and high-precision measurements.

FIGURE  6/PN.526...[D06] = 21 CM

FIGURE  7/PN.526...[D07] = 16 CM

Step 1:  Calculate the geometrical parameter n for each of the three knife-edges (top, left side and right side) using any of equations (13) to (16);

Step 2:  Calculate the loss factor j (n) = 10 J (n)/20 associated with each edge from equation (17);

Step 3:  Calculate minimum diffraction loss Jmin from:

Jmin (n)  =  –20 log                dB	(18)

or, alternatively,

Step 4:  Calculate average diffraction loss Jav from:

Jav (n)  =  –10 log                dB	(19)

## 4.3	Single rounded obstacle

The geometry of a rounded obstacle of radius R is illustrated in Fig. 6 c). Note that the distances d1 and d2, and the height h above the baseline, are all measured to the vertex where the projected rays intersect above the obstacle. The diffraction loss for this geometry may be calculated as:

A  =  J(n)  +  T(m,n)               dB	(20)

where:

a)	J (n) is the Fresnel-Kirchoff loss due to an equivalent knife-edge placed with its peak at the vertex point. The dimensionless parameter n may be evaluated from any of equations (13) to (16) inclusive. For example, in practical units equation (13) may be written:

n  =  0.0316 h 	(21)

where h and l are in metres, and d1 and d2 are in km.

J (n) may be obtained from Fig. 7 or from equation (17). Note that for an obstruction to line-of-sight propagation, n is positive and equation (17) is valid.

b)	T(m,n) is the additional attenuation due to the curvature of the obstacle:

T(m,n)  =  k  mb	(22a)

where:

k  =  8.2  +  12.0 n	(22b)

b  =  0.73  +  0.27 [1  –  exp ( – 1.43 n)]	(22c)

and:

m  =  R §	(23)

n  =  h §	(24)

and R, d1, d2, h and l are in self-consistent units.

T(m,n) can also be derived from Fig. 8.

Note that as R tends to zero, m, and hence T(m,n), also tend to zero. Thus equation (20) reduces to knife-edge diffraction for a cylinder of zero radius.

It should be noted that the cylinder model is intended for typical terrain obstructions. It is not suitable for trans-horizon paths over water, or over very flat terrain, when the method of § 3 should be used.

## 4.4	Double isolated edges

This method consists of applying single knife-edge diffraction theory successively to the two obstacles, with the top of the first obstacle acting as a source for diffraction over the second obstacle (see Fig. 9). The first diffraction path, defined by the distances a and b and the height h'1, gives a loss L1 (dB). The second diffraction path, defined by the

distances b and c and the height h'2, gives a loss L2 (dB). L1 and L2 are calculated using formulae of § 4.1. A correction term Lc (dB) must be added to take into account the separation b between the edges. Lc may be estimated by the following formula:

(25)

which is valid when each of L1 and L2 exceeds about 15 dB. The total diffraction loss is then given by:

L  =  L1  +  L2  +  Lc	(26)

FIGURE  8/PN.526...[D08] = 13.5 CM

The above method is particularly useful when the two edges give similar losses.

FIGURE  9/PN.526...[D09] = 3 CM

If one edge is predominant (see Fig. 10), the first diffraction path is defined by the distances a and b + c and the height h1. The second diffraction path is defined by the distances b and c and the height h'2. The losses corresponding to these two paths are added, without addition of a third term.

FIGURE  10/PN.526...[D10] = 5.5 CM

The same method may be applied to the case of rounded obstacles using § 4.3.

In cases where the diffracting obstacle may be clearly identified as a flat-roofed building a single knife-edge approximation is not sufficient. It is necessary to calculate the phasor sum of two components: one undergoing a double knife-edge diffraction and the other subject to an additional reflection from the roof surface. It has been shown that, where the reflectivity of the roof surface and any difference in height between the roof surface and the side walls are not accurately known, then a double knife-edge model produces a good prediction of the diffracted field strength, ignoring the reflected component.

## 4.5	General method for one or more obstacles

The following method is recommended for the diffraction loss over irregular terrain which forms one or more obstacles to line-of-sight propagation. The calculation takes Earth curvature into account.

The terrain height profile should be available as a set of samples of ground height above sea level, the first and last being the heights of the transmitter and receiver above sea level. Distance and height values are described as though stored in arrays indexed from 1 to N, where N equals the number of profile samples.

In the following a systematic use of suffices is made:

hi :	height above sea level of the i-th point

di :	distance from the transmitter to the i-th point

dij :	distance from the i-th to the j-th points.

The first step is to perform a “stretched string” analysis of the profile. This identifies the sample points which would be touched by a string stretched over the profile from transmitter to receiver. This may be done by the following procedure, in which all values of height and distance are in self-consistent units, and all angles are in radians. The method includes approximations which are valid for radio paths making small angles to the horizontal. If a path has ray gradients exceeding about 5° more exact geometry may be justified.

Each string point is identified as the profile point with the highest angular elevation above the local horizontal as viewed from the previous string point, starting at one end of the profile and finishing at the other. Viewed from point s, the elevation of the i-th profile sample (i > s) is given by:

e  =  [(hi  –  hs) / dsi ]  –  [dsi / 2ae ]	(27)

where:

ae :	effective Earth radius, given by:

=	k ´ 6 371 (km)

and:

k :	effective Earth-radius factor.

The evaluation of k for interference calculations is described in Recommendation ITU-R P.452, § 3.2.4.

A test is now applied to determine whether any group of two or more string points should represent the same terrain obstruction. For samples at spacings of 250 m or less any group of string points which are consecutive profile samples, other than the transmitter or receiver, should be treated as one obstruction.

Each obstruction is now modelled as a cylinder, as illustrated in Fig. 11. The geometry of each individual cylinder corresponds with Fig. 6c). Note that in Fig. 11 the distances s1, s2 for each cylinder are shown as measured horizontally between the vertex points, and that for near-horizontal rays these distances approximate to the slope distances d1 and d2 in Fig. 6c). For ray angles to the horizontal greater than about 5° it may be necessary to set s1 and s2 to the inter-vertex slope distances d1 and d2.

FIGURE  11/PN.526...[D11] = 18.5 CM

Similarly in Fig. 11, the height h of each cylinder is shown as measured vertically from its vertex down to the straight line joining the adjacent vertex or terminal points. The value of h for each cylinder corresponds to h in Fig. 6c). Again, for near-horizontal rays the cylinder heights may be computed as though vertical, but for steeper ray angles it may be necessary to compute h at right angles to the baseline of its cylinder.

Figure 12 illustrates the geometry for an obstruction consisting of more than one string point. The following points are indicated by:

w :	closest string point or terminal on the transmitter side of the obstruction which is not part of the obstruction

x :	string point forming part of the obstruction which is closest to the transmitter

y :	string point forming part of the obstruction which is closest to the receiver

z :	closest string point or terminal on the receiver side of the obstruction which is not part of the obstruction

v :	vertex point made by the intersection of incident rays above the obstruction.

FIGURE  12/PN.526...[D12] = 3 CM

The letters w, x, y and z will also be indices to the arrays of profile distance and height samples. For an obstruction consisting of an isolated string point, x and y will have the same value, and will refer to a profile point which coincides with the vertex. Note that for cascaded cylinders, points y and z for one cylinder are points w and x for the next, etc.

A step-by-step method for fitting cylinders to a general terrain profile is described in Appendix 1. Each obstruction is characterized by w, x, y and z. The method of Appendix 1 is then used to obtain the cylinder parameters s1, s2, h and R. Having modelled the profile in this way, the diffraction loss for the path is computed as the sum of three terms:

–	the sum of diffraction losses over the cylinders;

–	the sum of sub-path diffraction between cylinders (and between cylinders and adjacent terminals);

–	a correction term.

The total diffraction loss, in dB relative to free-space loss, may be written:

dB	(28)

where:

L'i :	diffraction loss over the i-th cylinder (dB) calculated by the method of § 4.3

L"(w x)1 :	sub-path diffraction loss for the section of the path between points w and x for the first cylinder

L"(y z)i :	sub-path diffraction loss for the section of the path between points y and z for all cylinders

CN :	correction factor to account for spreading loss due to diffraction over successive cylinders.

Appendix 2 gives a method for calculating L" for each line-of-sight section of the path between obstructions.

The correction factor, CN, is calculated using:

CN  =  (Pa / Pb)0.5	(29)

where:

(30)

(31)

and the suffices to round brackets indicate individual cylinders.

APPENDIX  1

TO  ANNEX  1

Calculation of cylinder parameters

# 1	Introduction

The following procedure can be used to calculate the cylinder parameters illustrated in Figs. 6c) and 12 for each of the terrain obstructions. Self-consistent units are used, and all angles are in radians. The approximations used are valid for radio paths which are within about 5° of horizontal.

# 2	Diffraction angle and position of vertex

Although not used directly as cylinder parameters, both the diffraction angle over the cylinder and the position of the vertex are required.

The diffraction angle over the obstacle is given by:

q  =  aw  +  az  +  ae	(32)

where aw and az are the angular elevations of points x and y above the local horizontal as viewed from points w and z respectively, given by:

aw  =  (hx  –  hw) / dwx  –  dwx / 2ae	(33)

az  =  (hy  –  hz) / dyz  –  dyz / 2ae	(34)

and ae is the angle subtended by the great-circle distance between points w and z given by:

ae  =  dwz / ae	(35)

The distance of the vertex from point w is calculated according to whether the obstruction is represented by a single profile sample or by more than one:

For a single-point obstruction:

dwv  =  dwx	(36)

For a multipoint obstruction it is necessary to protect against very small values of diffraction:

dwv  =  [(az  +  ae / 2) dwz  +  hz  –  hw ] / q	for q · ae  ³  dxy	(37a)

dwv  =  (dx  +  dy) / 2		for q · ae  <  dxy	(37b)

The distance of point z from the vertex point is given by:

dvz  =  dwz  –  dwv	(38)

The height of the vertex point above sea level is calculated according to whether the obstruction is represented by a single profile sample or by more than one.

For a single point obstruction:

hv  =  hx	(39)

For a multipoint obstruction:

hv  =  dwv aw  +  hw +  / 2ae	(40)

# 3	Cylinder parameters

The cylinder parameters illustrated in Fig. 6c) can now be calculated for each of the terrain obstacles defined by the string analysis:

d1 and d2 are the positive inter-vertex distances to the obstacles (or terminals) on the transmitter and receiver sides of the obstacle respectively,

and:

h  =  hv  +  dwv dvz / 2ae  –  (hw dvz  +  hz dwv) / dwz	(41)

To calculate the cylinder radius use is made of two further profile samples:

p :	the point adjacent to x on the transmitter side,

and:

q :	the point adjacent to y on the receiver side.

Thus the profile indices p and q are given by:

p  =  x  –  1	(42)

and:

q  =  y  +  1	(43)

If a point given by p or q is a terminal, then the corresponding value of h should be the terrain height at that point, not the height above sea level of the antenna.

The cylinder radius is calculated as the difference in slope between the profile section p-x and y-q, allowing for Earth curvature, divided by the distance between p and q.

The distances between profile samples needed for this calculation are:

dpx  =  dx  –  dp	(44)

dyq  =  dq  –  dy	(45)

dpq  =  dq  –  dp	(46)

The difference in slope between the p-x and y-q profile sections is given in radians by:

t  =  (hx  –  hp) / dpx  +  (hy  –  hq) / dyq  –  dpq / ae	(47)

where ae is the effective Earth radius.

The cylinder radius is now given by:

(48)

where n is the dimensionless knife-edge parameter in equation (20).

In equation (48), the second factor is an empirical smoothing function applied to the cylinder radius to avoid discontinuities for marginally line-of-sight obstructions.

APPENDIX  2

TO  ANNEX  1

Sub-path diffraction losses

# 1	Introduction

This Appendix provides a method for computing the sub-path diffraction loss for a line-of-sight subsection of a diffraction path. The path has been modelled by cascaded cylinders each characterized by profile points w, x, y and z as illustrated in Figs. 11 and 12. The sub-path diffraction is to be calculated for each subsection of the overall path between points represented by w and x, or by y and z. These are the line-of-sight sections of the path between obstructions, or between a terminal and an obstruction.

The method can also be used for a line-of-sight path with sub-path diffraction, in which case it is applied to the entire path.

# 2	Method

For a line-of-sight section of the profile between profile samples indexed by u and v, the first task is to identify the profile sample between but excluding u and v which obstructs the largest fraction of the first Fresnel zone for a ray travelling from u to v.

To avoid selecting a point which is essentially part of one of the terrain obstacles already modelled as a cylinder, the profile between u and v is restricted to a section between two additional indices p and q, which are set as follows:

–	Set p = u + 1.

–	If both p < v and hp > hp+1, then increase p by 1 and repeat.

–	Set q = v – 1.

–	If both q > u and hq > hq–1, then decrease q by 1 and repeat.

If p = q then the sub-path obstruction loss is set to 0. Otherwise the calculation proceeds as follows.

It is now necessary to find the minimum value of normalized clearance, CF, given by hz / F1, where in self-consistent units:

hz :	height of ray above profile point

F1 :	radius of first Fresnel zone.

The minimum normalized clearance may be written:

(49)

where:

(hz)i  =  (hr)i  –  (ht)i	(50)

(F1)i  =  	(51)

(hr)i, the height of the ray above a straight line joining sea level at u and v at the i-th profile point is given by:

(hr)i  =  (hu · div  +  hv · dui) / duv	(52)

(ht)i, the height of the terrain above a straight line joining sea level at u and v at the i-th profile point is given by:

(ht)i  =  hi  +  dui · div / 2ae	(53)

The minimum value of normalized clearance is used to compute the knife-edge diffraction geometrical parameter for the most significant sub-path obstruction:

n  =  – CF · 	(54)

The sub-path diffraction loss L" is now obtained from equation (17) or Fig. 7.

For some applications it may be undesirable to include sub-path diffraction enhancements. In this case a value of L" should be set to zero when it would otherwise be negative.

_________________