Recommendation ITU-R P.527-4 (06/2017)

Electrical characteristics of the surface of the Earth

P Series Radiowave propagation

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2017

ITU 2017

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  P.527-4

Electrical characteristics of the surface of the Earth

(1978-1982-1992-2017)

Scope

This Recommendation gives methods to model the electrical characteristics of the surface of the Earth, including pure water, sea water, ice, soil and vegetation cover, for frequencies up to 1 000 GHz, in a systematic manner based on the evaluation of complex relative permittivity. In all cases conductivity can be calculated as a function of frequency and temperature from these evaluations. Previous information on electrical characteristics below 30 MHz in terms of permittivity and conductivity is retained in Appendix in view of its association with Recommendations ITU-R P.368 and ITU-R P.832. The new modelling method is fully compatible with this earlier information.

Keywords

Complex permittivity, conductivity, penetration depth, Earth’s surface, water, vegetation, soil, ice

The ITU Radiocommunication Assembly,

considering

a)	that the electrical characteristics may be expressed by three parameters: magnetic permeability,  electrical permittivity, , and electrical conductivity, σ;

b)	that the permeability of the Earth’s surface, , can normally be regarded as equal to the permeability in a vacuum;

c)	that the electrical properties of the Earth’s surface can be expressed by the complex permittivity or, equivalently, by the real part and imaginary part of the complex permittivity;

d)	that information on the variation of the penetration depth with frequency is needed;

e)	that knowledge of the electrical characteristics of the Earthʼs surface is needed for several purposes in propagation modelling, including ground-wave signal strength, ground reflection at a terrestrial terminal, interference between aeronautical and/or space borne stations due to reflections or scattering from the Earth's surface, and for Earth science applications;

f)	that Recommendation ITU-R P.368 contains ground-wave propagation curves from 1 MHz to 30 MHz for different ground conditions characterised by permittivity and electrical conductivity;

g)	that Recommendation ITU-R P.832 contains a world atlas of ground electrical conductivity for frequencies below 1 MHz,

recommends

that the information in Annex 1 be used to model the electrical characteristics of the surface of the Earth.

Annex 1

# 1	Introduction

This Annex provides prediction methods that predict the electrical characteristics of the following Earth’s surfaces for frequencies up to 1 000 GHz:

–	Water

–	Sea (i.e. Saline) Water

–	Dry and Wet Ice

–	Dry and Wet Soil (combination of sand, clay, and silt)

–	Vegetation (above and below freezing)

# 2	Complex permittivity

The characteristics of the Earth’s surface can be characterized by three parameters:

–	the magnetic permeability, μ,

–	the electrical permittivity, ε, and

–	the electrical conductivity, σ.

Magnetic permeability is a measure of a material’s ability to support the formation of a magnetic field within itself in response to an applied magnetic field; i.e. the magnetic flux density B divided by the magnetic field strength H. Electrical permittivity is a measure of a materialʼs ability to oppose an electric field; i.e. the electrical flux density D divided by the electrical field strength E. Electrical conductivity is a measure of a materialʼs ability to conduct an electric current; i.e. the ratio of the current density in the material to the electric field that causes the current flow.

Given an incident plane wave E(r,t)=E0ej(ωt-k∙r), with radial frequency ω, time t, magnetic permeability μ, electrical permittivity ε, and electrical conductivity σ, the propagation wave number vector k, has a magnitude k given by

k=-jωμσ+jωε	(1a)

The vacuum values of permittivity, permeability, and conductivity are:

–	Vacuum Permittivity		ε0=8.854 187 817×10-12 	(F/m)

–	Vacuum Permeability	μ0=4π×10-7	(N/A2)

–	Vacuum Conductivity	σ0=0.0  	(S/m)

It is convenient to define the relative permittivity, εr, and the relative permeability, μr, relative to their vacuum values as follows:

–	Relative Permittivity		εr=εε0

–	Relative Permeability	μr=μμ0

where ε and  are the associated permittivity and permeability of the medium. This Recommendation assumes μ = μ0, in which case μr = 1.

As shown in equation (1a) the wavenumber depends on both σ and ε, not either separately. Also formulations of other physical parameters describing various radio wave propagation mechanisms such as scattering cross section, reflection coefficients, and refraction angles, depend on values of this combination. Furthermore, the square root of this combination is equivalent to the refractive index formulation used in characterizing the troposphere and the ionosphere. The refractive index is also used in characterizing different materials at the millimetre wave and optical frequency bands. Accordingly, to simplify the formulations describing various propagation mechanisms and to standardize terminologies of electrical characteristics at different frequency bands, the combination ε- jσω is defined as the complex permittivity and used to describe the electrical characteristics of substances.

While permittivity refers to ε, relative permittivity refers to εr, and complex relative permittivity, defined as εr'-j εr'', refers to:

εr'-jεr''= εε0- jσωε0	(1b)

where ε may be complex.

In equation (1b), εr' is the real part of the complex permittivity, and εr'' is the imaginary part of the complex permittivity. The real part of the complex relative permittivity, εr', is associated with the stored energy when the substance is exposed to an electromagnetic field. The imaginary part of the complex relative permittivity, εr'', influences energy absorption and is known as the loss factor. The minus sign in equation (1b) is associated with an electromagnetic field having time dependence of e2jπft (f is frequency in Hz, and t is time in seconds). If the time dependence is e-2jπft, the minus (–) sign in equation (1b) is replaced by a plus (+) sign.

At frequencies up to 1 000 GHz, dissipation within the Earth’s surface is attributed to either translational (conduction current) charge motion or vibrational (dipole vibration) charge motion, and the imaginary part of the complex relative permittivity, εr'' can be decomposed into two terms:

εr''= εd''+σ2πfε0	(2)

where εd'' represents the dissipation due to displacement current associated with dipole vibration, and σ2πfε0 represents the dissipation due to conduction current.

Conduction current consists of the bulk translation movement of free charges and is the only current at zero (i.e. dc) frequency. Conduction current is greater than displacement current at frequencies below the transition frequency, ft, and the displacement current is greater than the conduction current at frequencies above the transition frequency, ft. The transition frequency, ft, defined as the frequency where the conduction and displacement currents are equal, is:

ft= σ2πε0εd''	(3)

For non-conducting (lossless) dielectric substances σ=0, and hence εr''= εd''. For some of those substances, such as dry soil and dry vegetation, εd''=0, and hence εr''=0 irrespective of the frequency, which is the case considered in § 2.1.2.3 of Recommendation ITU-R P.2040. On the other hand, for some other non-conducting substances, such as pure water and dry snow, εd,'', and hence εr'', equal zero only at zero frequency. Accordingly, § 2.1.2.3 of Recommendation ITU-R P.2040 cannot be applied to those substances.

For conducting (lossy) dielectric substances, such as sea water and wet soil, the electrical conductivity σ has finite values different than zero. Accordingly, as the frequency tends to zero, the imaginary part of the complex relative permittivity of those substances tends to ∞ as it can be inferred from equation (3). In this case, it is easier to work with the conductivity σ instead of the imaginary part of the complex relative permittivity which can be written from equation (2) after setting εd''= 0 as follows:

σ=2πε0f εr''=0.05563fGHzεr''	(3a)

with fGHz is the frequency in GHz. Generalizing the above formulation to other frequencies, as done by equation (12) of Recommendation ITU-R P.2040, yields the sum of two terms: one term gives the electrical conductivity and the other term accounts for the power dissipation associated with the displacement current.

This Recommendation provides prediction methods for the real and imaginary parts of the complex relative permittivity, εr' and εr''; and the accompanying example figures show trends of the real and imaginary parts of the complex relative permittivity with frequency under different environmental conditions.

## 2.1	Layered ground

The models in § 5 apply to homogeneous sub-surface soil; however, the sub-surface is rarely homogeneous. Rather, it consists of multiple layers of different thicknesses and different electrical characteristics that must be taken into account by introducing the concept of effective parameters to represent the homogeneous soil. Effective parameters can be used with the homogeneous smooth Earth ground-wave propagation curves of Recommendation ITU-R P.368.

# 3	Penetration depth

The extent to which the lower strata influence the effective electrical characteristics of the Earth’s surface depends upon the penetration depth of the radio energy, δ, which is defined as the depth at which the amplitude of the field strength of electromagnetic radiation inside a material falls to 1/e (about 37%) of its original value at (or more properly, just beneath) the surface. The penetration depth, δ, in a homogeneous medium of complex relative permittivity εr (εr=εr'-jεr'') is given by:

δ=λ2π2εr'2+ εr''2- εr'      (m)	(4)

where λ is the wavelength in metres. Note that as the imaginary part of the complex relative permittivity in equation (4) tends to zero, the penetration depth tends to infinity.

Figure 1 depicts typical values of penetration depth as a function of frequency for different types of Earth’s surface components including pure water, sea water, dry soil, wet soil, and dry ice. The penetration depths for pure water and sea water are calculated at 20 oC, and the salinity of sea water is 35 g/kg. The penetration depths for dry soil and wet soil assume the volumetric water content is 0.07 and 0.5, respectively. Other soil parameters are the same as in Fig. 7. The penetration depth of dry ice is calculated at 0 oC.

FIGURE 1

Penetration depth of surface types as a function of frequency

# 4	Factors determining the effective electrical characteristics of soil

The effective values of the electrical characteristics of the soil are determined by the nature of the soil, its moisture content, temperature, general geological structure, and the frequency of the incident electromagnetic radiation.

## 4.1	Nature of the soil

Although it has been established by numerous measurements that values of the electrical characteristics of soil vary with the nature of the soil, this variation may be due to its ability to absorb and retain moisture rather than the chemical composition of the soil. It has been shown that loam, which normally has a conductivity on the order of 10−2 S/m can, when dried, have a conductivity as low as 10−4 S/m, which is the same order as granite.

## 4.2	Moisture content

The moisture content of the ground is the major factor determining the permittivity and conductivity of the soil. Laboratory measurements have shown that as the moisture content of the ground increases from a low value, the permittivity and conductivity of the ground increase and reach their maximum values as the moisture content approaches the values normally found in such soils. At depths of one metre or more, the wetness of the soil at a particular site is typically constant. Although the wetness may increase during periods of rain, the wetness returns to its typical value after the rain has stopped due to drainage and surface evaporation.

The typical moisture content of a particular soil may vary considerably from one site to another due to differences in the general geological structure which provides different drainage.

## 4.3	Temperature

Laboratory measurements of the electrical characteristics of soil have shown that, at low frequencies conductivity increases by approximately 3% per degree Celsius, while permittivity is approximately constant over temperature. At the freezing point, there is generally a large decrease in both conductivity and permittivity.

## 4.4	Seasonal variation

The effects of seasonal variation on the electrical characteristics of the soil surface are due mainly to changes in water content and temperature of the top layer of the soil.

# 5	Complex relative permittivity prediction methods

The models described in the following sub-sections provide prediction methods for the complex relative permittivity of the following Earth’s surfaces:

–	Pure Water

–	Sea (i.e. Saline) Water

–	Ice

–	Dry Soil (combination of sand, clay, and silt)

–	Wet Soil (dry soil plus water)

–	Vegetation (above and below freezing)

In this section, subscripts of the complex relative permittivity and hence the subscripts of its real and imaginary parts are chosen to denote the relative permittivity specialised for specific cases; e.g. the subscript “pw” for pure water, the subscript “sw” for sea water, etc.

## 5.1	Water

This sub-section provides prediction methods for the complex relative permittivity of pure water, sea water, and ice.

### 5.1.1	Pure water

The complex relative permittivity of pure water, εpw, is a function of frequency, fGHz (GHz), and temperature, T (oC):

εpw= εpw'-j εpw''	(5)

εpw'= εs-ε11+fGHz/f12+ ε1-ε∞1+fGHz/f22+ε∞	(6)

εpw''=fGHz/f1εs-ε11+fGHz/f12 +fGHz/f2ε1-ε∞1+fGHz/f22 	(7)

where:

εs=77.66+103.3Θ   	(8)

ε1=0.0671εs  	(9)

ε∞=3.52-7.52Θ	(10)

Θ= 300T+273.15-1	(11)

and f1 and f2 are the Debye relaxation frequencies:

f1=20.20-146.4 Θ+316Θ2               (GHz)	(12)

f2=39.8 f1                                                    (GHz)	(13)

### 5.1.2	Sea water

The complex relative permittivity of sea (saline) water, εsw, is a function of frequency, fGHz (GHz), temperature, T (oC), and salinity S (g/kg or ppt).

εsw= εsw'-j εsw''	(14)

εsw'= εss-ε1s1+fGHz/f1s2+ ε1s-ε∞s1+fGHz/f2s2+ε∞s	(15)

εsw''=fGHz/f1sεss-ε1s1+fGHz/f1s2 +fGHz/f2sε1s-ε∞s1+fGHz/f2s2+18σswfGHz	(16)

where

εss=εs exp-3.56417×10-3S+ 4.74868×10-6S2+1.15574×10-5TS	(17)

f1s= f1 1+S2.39357×10-3-3.13530×10-5T+ 2.52477×10-7T2 (GHz)	(18)

ε1s=ε1 exp-6.28908×10-3S+ 1.76032×10-4S2-9.22144×10-5TS	(19)

f2s= f2 1+S-1.99723×10-2+ 1.81176×10-4T                                          (GHz)	(20)

ε∞s= ε∞ 1+S-2.04265×10-3+ 1.57883×10-4T	(21)

Values of εs, ε1, ε∞, f1and f2 are obtained from equations (8), (9), (10), (12), and (13). Furthermore, σsw is given by

σsw= σ35 R15RT15   (S/m)	(22)

σ35=2.903602+8.607×10-2 T+4.738817×10-4 T2

-2.991×10-6 T3+4.3047×10-9 T4	(23)

R15=S 37.5109+5.45216S+1.4409 ×10-2 S21004.75+182.283 S + S2	(24)

RT15=1+ α0T-15α1+T 	(25)

α0= 6.9431+3.2841S-9.9486 ×10-2S284.850+69.024 S+ S2	(26)

α1=49.843-0.2276 S+0.198 × 10-2 S2 	(27)

The complex relative permittivity of pure water given in equations (5) – (7) is a special case of equation (14) – (16) where S = 0. The complex relative permittivity of pure water (S = 0 g/kg) and sea water (S = 35 g/kg) vs. frequency are shown in Fig. 2 for T=20 oC and in Figure 3 for T=0 oC.

FIGURE 2

Complex relative permittivity of pure and sea water as a function of frequency 
(T=20 oC)

FIGURE 3

Complex relative permittivity of pure and sea water as a function of frequency
(T=0 oC)

### 5.1.3	Ice

This sub-section provides prediction methods for the complex relative permittivity of dry ice and wet ice.

#### 5.1.3.1	Dry ice

Dry ice is composed of frozen pure water (i.e. T ≤ 0 oC) . The complex relative permittivity of dry ice, εice, is

εice= εice'-j εice''	(28)

The real part of the complex relative permittivity, εice', is a function of temperature, T (oC), and is independent of frequency:

εice'=3.1884+0.00091 T	(29)

and the imaginary part of the complex relative permittivity, εice'', is a function of temperature T (oC) and frequency, fGHz (GHz):

εice''= AfGHz+ BfGHz	(30)

where

A= 0.00504+0.0062Θexp-22.1Θ	(31)

B= 0.0207T+273.15 exp⁡(-τ)exp⁡(-τ)-12+1.16×10-11fGHz2+ exp(-9.963+0.0372T)	(32)

τ= 335T+273.15		(33)

Θ= 300T+273.15-1		(34)

The real and imaginary parts of complex relative permittivity of dry ice are shown in Fig. 4 for
T=-10 oC.

#### 5.1.3.2 	Wet ice

When the ice is wet (at 0 oC), its grains are surrounded by liquid water. Considering ice grains as spherical inclusions within a liquid water background, the Maxwell Garnett dielectric mixing formula is applied to express the complex relative permittivity of wet ice, εwet ice, as a combination of the complex relative permittivity of dry ice, εice, and the complex relative permittivity of pure water, εpw

εwet ice= εice+2εpw+2 εice-εpw 1- Fwcεice+2εpw- εice-εpw 1 –Fwcεpw	(35)

Fwc is the liquid water volume fraction (m3/m3). Equation (35) is complex, and it can be split into the real part and the imaginary part. Each part is a function of the real and imaginary parts of the complex relative permittivity of dry ice and corresponding parts of water. The real and imaginary parts of wet ice at fGHz=60 GHz and T=0 oC are depicted in Fig. 5 as a function of liquid water content.

FIGURE 4

Complex relative permittivity of dry ice as a function of frequency
(T=-10 oC)

FIGURE 5

Complex relative permittivity of wet ice as a function of liquid water content
(f=60 GHz and T=0 oC)

## 5.2	Soil

The complex relative permittivity of soil, εsoil, is a function of frequency, fGHz (GHz), temperature, T (oC), soil composition, and volumetric water content.

The soil composition is characterized by the percentages by volume of the following dry soil constituents which are available from field surveys and laboratory analysis: a) Psand = % sand, b) Pclay = % clay, and c) Psilt = % silt as well as d) the specific gravity (i.e. the mass density of soil divided by the mass density of water) of the dry mixture of soil constituents, ρs , and e) the volumetric water content, mv , equal to the water volume divided by total soil volume for a given soil sample.

The bulk density (i.e. the mass of soil in a given volume (g cm−3) of the soil, ρb, is also required as an input. While it is not easily measured directly, it can be derived from the percentages of the dry constituents. If a local pseudo-transfer function is not available, the following empirical pseudo-transfer function can be used

ρb=1.07256+0.078886lnPsand+0.038753lnPclay+0.032732 lnPsilt 	(36)

Equation (36) is not reliable for less than 1% of any constituent. If the percentages of a constituent are less than 1%, the corresponding term in equation (36) should be omitted. The constituent percentages of the included terms should sum to 100%.

Table 1 shows typical constituent percentages, specific gravities, and bulk densities for four representative soil types.

TABLE 1

Physical parameters of various soil types

| Soil Designation Textural Class | 1 Sandy Loam | 2 Loam | 3 Silty Loam | 4 Silty Clay |
| --- | --- | --- | --- | --- |
| % Sand | 51.52 | 41.96 | 30.63 | 5.02 |
| % Clay | 13.42 | 8.53 | 13.48 | 47.38 |
| % Silt | 35.06 | 49.51 | 55.89 | 47.60 |
| ρs | 2.66 | 2.70 | 2.59 | 2.56 |
| ρb (g cm−3) | 1.6006 | 1.5781 | 1.5750 | 1.4758 |

FIGURE 6

Soil texture triangle

The soil designation textural class reported in the first row of Table 1 is based on the soil texture triangle depicted in Fig. 6.

This prediction method considers soil as a mixture of four components: a) soil particles composed of a combination of clay, sand, and silt, b) air, c) bound water (water attached to soil particles by forces such as surface tension, where the thickness of the water layer and its dielectric constant and relaxation frequencies are unknown), and d) free water (also known as bulk water that flows freely within soil bores). The complex relative permittivity of soil, εsoil, of this four component mixture is

εsoil= εsoil'-jεsoil         ''	(37)

where:

εsoil'= 1+ ρbρs εsm'α-1+mvβ'εfw'α-mv1/α	(38)

εsoil''= mvβ''εfw''α1/α		(39)

εsm'= 1.01+0.44 ρs2-0.062	(40)

β'=1.2748-0.00519 Psand-0.00152 Pclay	(41)

β''=1.33797-0.00603 Psand-0.00166 Pclay	(42)

and

α = 0.65		(43)

εfw' and εfw'' are the real and the imaginary parts of the complex relative permittivity of free water:

εfw'= εs-ε11+fGHz/f12+ ε1-ε∞1+fGHz/f22+ε∞+ 18 σeff'fGHzρs- ρbρsmv	(44)

εfw''=fGHz/f1εs-ε11+fGHz/f12 +fGHz/f2ε1-ε∞1+fGHz/f22+  18 σeff''fGHzρs- ρbρsmv	(45)

where εs, ε1, ε∞, f1 and f2 are obtained from equations (8), (9), (10), (12), and (13), and σeff' and σeff'' are:

σeff'= fGHz/1.35σ1-σ21+ fGHz/1.352	(46)

σeff''= σ2+ σ1-σ21+ fGHz/1.352	(47)

and

σ1=0.0467 +0.2204 ρb-0.004111Psand-0.006614Pclay 	(48)

σ2= -1.645+1.939 ρb-0.0225622Psand+0.01594Pclay	(49)

The complex relative permittivity of two examples of soil types are shown in Figs 7, 8 and 9. The soil composition in Figs 7 and 9 are identical except for the volumetric water content, indicating that both the real part and imaginary part of the complex relative permittivity are directly related to the volumetric water content.

FIGURE 7

Complex relative permittivity of a silty loam soil as a function of frequency 
(mv = 0.5,  T =23 oC, ρs=2.59 ,   ρb=1.5750 g cm-3)

FIGURE 8

Complex relative permittivity of a silty clay soil as a function of frequency
(mv= 0.5, T =23 oC, ρs=2.56 ,  ρb=1.4758 g cm-3)

FIGURE 9

Complex relative permittivity of a silty loam soil as a function of frequency
(mv= 0.07, T =23 oC, ρs=2.59 ,  ρb=1.5750 g cm-3)

## 5.3	Vegetation

The complex relative permittivity of vegetation is a function of frequency fGHz (GHz), temperature T (oC), and vegetation gravimetric water content, Mg, which is defined as

Mg= Mmv-MdvMmv	(50)

Mmv is the weight of the moist vegetation, and Mdv is the weight of the dry vegetation. Mg is between 0.0 and 0.7.

This prediction method considers the vegetation as a mixture of bulk vegetation, saline free water, bounded water, and ice (if applicable). The complex relative permittivity of this mixture is given by

εv= εv'-j εv''	(51)

The real part, εv', and the imaginary part, εv'', of the complex relative permittivity of vegetation are given in § 5.3.1 for above freezing temperatures, and in § 5.3.2 for below freezing temperatures.

### 5.3.1	Above freezing temperatures

At temperatures above freezing (T > 0 C), the real, and imaginary parts of the complex relative permittivity of vegetation are:

εv'= εdv+vfwε∞+εs-ε11+ fGHz/f12+ε1-ε∞1+ fGHz/f22+ vbw2.9+551+ fGHz/0.02f11+2 fGHz/0.02f1+fGHz/0.01f1		(52)

εv''=  vfwfGHz/f1εs-ε11+ fGHz/f12+fGHz/f2ε1-ε∞1+ fGHz/f22+18 σswfGHz+vbw 55fGHz/0.02f11+2 fGHz/0.02f1+fGHz/0.01f1	(53)

where εdv is the real part of the relative permittivity of bulk vegetation, vfw is the free water volume fraction, and vbw is the bound water volume fraction with:

εdv=1.7-0.74 Mg+6.16Mg2	(54)

vfw= Mg(0.55 Mg-0.076)	(55)

vbw=4.64Mg2/1+7.36Mg2	(56)

σsw is the electrical conductivity of saline water given in equations (22) to (27), where the salinity, S, is

S= -28.7Mg+34.83              gkg	(57)

and εs, ε1, ε∞, f1 and f2 are obtained from equations (8), (9), (10), (12) and (13) respectively.

For a temperature of 22 °C and a frequency range up to 40 GHz, equations (52) and (53) are modified as follows:

εv'= εdv+vfw4.9+751+ fGHz/182+ vbw2.9+ 551+ fGHz/0.361+2 fGHz/0.36+fGHz/0.18	(58)

εv''=  vfw75fGHz/181+ fGHz/182+22.86fGHz+ vbw 55fGHz/0.361+2 fGHz/0.36+fGHz/0.18	(59)

Equations (58) and (59) are more general than equation (16) of Recommendation ITU-R P.833 since they account for both free and bound water and include the temperature dependence.

The real and imaginary parts of the complex relative permittivity of vegetation vs. frequency at two different values of gravimetric water content are shown in Figs 10 and 11 demonstrating that both the real part and the imaginary part of the complex relative permittivity of vegetation increase as the gravimetric water content increases.

FIGURE 10

Complex relative permittivity of vegetation as a function of frequency
(Mg=0.68, T=22 oC)

FIGURE 11

Complex relative permittivity of vegetation as a function of frequency
(Mg=0.26, T=22 oC)

### 5.3.2	Below freezing temperatures

For below freezing temperatures between (−20 C ≤ T < 0 C) the real and imaginary parts of the complex relative permittivity are:

εv'= εdv+ vfw4.9+ 82.21+ fGHz/92+ vbw8.092+14.2067 X1+3.15 vice	(60)

εv''= vfw82.2fGHz/91+ fGHz/92+ 11.394fGHz+ 14.2067 vbw Y1	(61)

where:

εdv=6.76-10.24 Mg+6.19Mg2	(62)

vfw= -0.106+0.6591Mg-0.610Mg2 exp0.06+0.6883Mg+0.0001Mg2Δ	(63)

vbw= -0.16+1.1876Mg-0.387Mg2 exp0.721-1.2733Mg+0.8139Mg2Δ	(64)

vice= Aice Δ2+ BiceΔ+ Cice	(65)

Aice= 0.001-0.012Mg+0.0082Mg2	(66)

Bice= 0.036-0.2389Mg+0.1435Mg2	(67)

Cice= -0.0538+0.4616Mg-0.3398Mg2	(68)

X1= 1+ fGHz/1.25820.2054cos0.2054π/21+2 fGHz/1.25820.2054cos0.2054π/2+fGHz/1.25820.4108 	(69)

Y1= fGHz/1.25820.2054sin0.2054π/21+2 fGHz/1.25820.2054cos0.2054π/2+fGHz/1.25820.4108 	(70)

Δ=T-Tf	(71)

and the vegetation freezing temperature, Tf, is −6.5 oC.

The real and the imaginary parts of the complex relative permittivity vs. frequency and temperature are shown in Figs 12 and 13. These Figures show that reducing the temperature below freezing decreases both the real and imaginary parts of the vegetation complex relative permittivity, and decreases the dependence of those parameters on frequency. For frequencies above 20 GHz, the complex relative permittivity of vegetation becomes less dependent on temperature.

FIGURE 12

Complex relative permittivity of vegetation as a function of frequency
(Mg=0.68, T=-7 oC)

FIGURE 13

Complex relative permittivity of vegetation as a function of frequency
(Mg=0.68, T=-10 oC)

Appendix 

to Annex 1

Electrical properties expressed as permittivity and conductivity 
as used in Recommendations ITU-R P.368 and ITU-R P.832

# 1	Introduction

Figure 14 below is reproduced from Fig. 1 showing typical values of conductivity and permittivity for different types of ground, as a function of frequency. These graphs are retained from earlier revisions of this Recommendation as a convenience to users of Recommendation ITU-R P.386 and ITU-R P.832.

FIGURE 14

Relative permittivity εr, and conductivity σ, as a function of frequency

It is called electrical conductivity to differentiate it from other conductivities such as thermal conductivity and hydraulic conductivity. It is called hereinafter as conductivity.

The term “ppt” stands for “parts per thousand”.