Recommendation ITU-R P.528-4 (08/2019)

A propagation prediction method for aeronautical mobile and radionavigation services using the VHF, UHF and SHF bands

P Series Radiowave propagation

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2019

ITU 2019

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  P.528-4,

A propagation prediction method for aeronautical mobile and radionavigation services using the VHF, UHF and SHF bands

(Question ITU-R 203/3)

(1978-1982-1986-2012-2019)

Scope

This Recommendation contains a method for predicting basic transmission loss in the frequency range 125 MHz to 15.5 GHz for aeronautical and satellite services. It provides a step-by-step method to compute the basic transmission loss. The only data needed for this method are the distance between antennas, the heights of the antennas above mean sea level, the frequency, and the time percentage.

This Recommendation, also, gives the calculations for the expected protection ratio or wanted-to-unwanted signal ratio exceeded at the receiver for at least 95% of the time, R (0.95). This calculation requires the following additional data for both the wanted and unwanted signals: the transmitted power, the gain of transmitting antenna, and the gain of receiving antenna.

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to give guidance to engineers in the planning of radio services in the VHF, UHF, and SHF bands;

b)	that the propagation model given in Annex 2 is based on a considerable amount of experimental data (see Annex 1);

c)	that the aeronautical service often provides a safety of life function and therefore requires a higher standard of availability than many other services;

d)	that a time availability of 0.95 should be used to obtain more reliable service,

recommends

1	that the integral software in this Recommendation should be used to generate basic transmission loss values and curves for terminal heights, frequencies, and time percentages likely to be encountered in the aeronautical services;

2	that the following Notes should be regarded as part of this Recommendation.

NOTE 1 – It should be emphasized that the generated values are based on data obtained mainly for a continental temperate climate.

NOTE 2 – The method gives basic transmission loss, that is, the loss between ideal loss-free isotropic antennas. Where surface reflection multipath at the ground station or the facility has been mitigated using counterpoises or a directional vertical radiation pattern suitable antenna radiation patterns should be included within the analysis.

Annex 1

Development and application of the model

Transmission loss prediction methods have been developed that determine the basic transmission losses for time percentages from 1% to 99% of the time for antenna heights applicable to the aeronautical services. These methods are based on a considerable amount of experimental data, and extensive comparisons of predictions with data have been made. In performing these calculations, a smooth (terrain parameter h  0) Earth with an effective earth radius factor k of 4/3 (surface refractivity Ns  301) was used along with compensation for the excessive ray bending associated with the k  4/3 model at high altitudes. Constants for average ground horizontal polarization, isotropic antennas, and long-term power fading statistics for a continental temperate climate were also used. Although these parameters may be considered either reasonable or worst-case for many applications, the computed values should be used with caution if conditions differ drastically from those assumed.

With the exception of a region ‘near’ the radio horizon, values of median basic transmission loss for ‘within-the-horizon’ paths were obtained by adding the attenuation due to atmospheric absorption (in decibels) to the transmission loss corresponding to free-space conditions. Within the region ‘near’ the radio horizon, values of the transmission loss were calculated using geometric optics, to account for interference between the direct ray and a ray reflected from the surface of the Earth.

The two-ray interference model was not used exclusively for within-the-horizon calculations, because the lobing structure obtained from it for short paths is highly dependent on surface characteristics (roughness as well as electrical constants), atmospheric conditions (the effective earth radius is variable in time), and antenna characteristics (polarization, orientation and gain pattern). Such curves would often be more misleading than useful, i.e. the detailed structure of the lobing is highly dependent on parameters that are difficult to determine with sufficient precision. However, the lobing structure is given statistical consideration in the calculation of variability.

For time availabilities other than 0.50, the basic transmission loss, Lb, user-generated values do not always increase monotonically with distance. This occurs because variability changes with distance can sometimes overcome the median level changes. Variability includes contributions from both hourly-median or long-term power fading and within-the-hour or short-term phase interference fading. Both surface reflection and tropospheric multipath are included in the short-term fading.

The basic transmission loss, Lb(0.05) values may be used to estimate Lb values for an unwanted interfering signal that is exceeded during 95% (100% – 5%) of the time. Median (50%) propagation conditions may be estimated with the Lb(0.50) values. The Lb(0.95) values may be used to estimate the service range for a wanted signal at which service would be available for 95% of the time in the absence of interference.

The expected protection ratio or wanted-to-unwanted signal ratio exceeded at the receiver for at least 95% of the time, R(0.95), can be estimated as follows:

R(0.95)    R(0.50)    YR(0.95)	(1)

R(0.50)    [Pt    Gt    Gr  –  Lb(0.50)]Wanted  –  [Pt    Gt    Gr  –  Lb(0.50)]Unwanted	(2)

and

YR    –  	Lb0.95-Lb0.50Wanted2+ Lb0.05-Lb0.50Unwanted2	(3)

In equation (2), Pt is the transmitted power, and Gt and Gr are the isotropic gains of the transmitting and receiving antennas expressed in dB.

Additional variabilities could easily be included in equation (3) for such things as antenna gain when variabilities for them can be determined. Continuous (100%) or simultaneous channel utilization is implicit in the R(0.95) formulation provided above so that the impact of intermittent transmitter operation should be considered separately.

The integral software to generate basic transmission loss values and curves is made available in the file R-REC-P.528-4-201908-P1!ZIP-E.zip from the supplement file R-REC-P.528-4-201908-I!!ZIP-E.zip, with documentation. In addition, there are select tabulated basic transmission loss values also available in the file R-REC-P.528-4-201908-P2!ZIP-E.zip from the supplement file.

Annex 2

Step by step method

This Annex uses the convention that variables describing the low terminal will be represented with the subscript ‘1’ (i.e. the low terminal height hr1) while variables for the high terminal will be represented with the subscript ‘2’ (i.e. the high terminal height hr2).

# 1	Introduction

This Annex describes a step-by-step method for calculating the basic transmission loss for user-specified path, defined by:

–	terminal heights, hr1 and hr2, in km above mean sea level, with 0.0015≤hr1,2≤20 (1.5 metres to 20 000 metres)

–	frequency, f, in MHz, with 125≤f≤15 500 MHz

–	time percentage, q, with 0.01≤q≤0.99

–	path distance, d, in km.

# 2	Assumptions, definitions and conventions

Recommendation ITU-R P.528 assumes the following values:

Ns :	surface refractivity, in N-Units. Set to 301 N-Units

a0 :	actual radius of the Earth. Set to 6 370 km

ae :	effective radius of the Earth. Set to 8 493 km (corresponding to a surface refractivity of 301 N-Units)

ϵr :	relative dielectric constant. Set to 15 (corresponding to average ground)

σ :	conductivity. Set to 0.005 S/m (corresponding to average ground).

Additionally, terminal antennas are assumed to be horizontally polarized.

# 3	Step-by-step method

Step 1: Compute the geometric parameters associated with each terminal. This requires using the steps presented in § 4 for both the low terminal and the high terminal. Once completed, proceed to Step 2. Use § 4 as follows:

given:

hr1,2 :	real height of terminal above mean sea level (user input), in km;

find:

d1,2 :	arc length to the smooth Earth horizon distance, in km

θ1,2 :	incident angle of the ray from the terminal to the smooth Earth horizon distance, in radians

h1,2 :	adjusted height of the terminal above mean sea level used in subsequent calculations, in km

Δh1,2 :	terminal height correction term, in km.

Step 2: Determine the maximum line-of-sight distance, dML, between the two terminals.

dML=d1+d2     (km)	(4)

Step 3: Smooth Earth diffraction is modelled linearly in Recommendation ITU-R P.528. This is done by selecting two distances well beyond dML, computing the smooth Earth diffraction loss at these distances, and constructing a smooth Earth diffraction line that passes through both of these points.

Step 3.1: Compute two distances, d3 and d4, that are well beyond the maximum line-of-sight distance dML from the above equation (4).

d3=dML+0.5ae2f13     (km)	(5)

d4=dML+1.5ae2f13     (km)	(6)

Step 3.2: Compute the diffraction losses Ad3 and Ad4 at the corresponding distances d3 and d4. This will require using § 6 twice – once for each path distance, d3,4. Once completed proceed to Step 3.3. Use the method in § 6 as follows:

given:

d3,4 :	the path distance of interest, d0, as required for § 10, in km

d1,2 :	arc lengths to the smooth Earth horizon distance of the terminals, h1 and h2, in km, as determined in Step 1 above

f :	frequency, in MHz

find:

Ad3,4 :	smooth Earth diffraction loss, Ad, in dB, corresponding to the distance d3,4.

Step 3.3: Create the smooth Earth diffraction line from the two distances, d3 and d4, and their respective losses, Ad3 and Ad4, by computing the slope Md and intercept Ad0.

Md=Ad4-Ad3d4-d3     (dB/km)	(7)

Ad0=Ad4-Mdd4     (dB)	(8)

Step 3.4: Compute the diffraction loss at the distance dML and the distance dd, in km, at which the diffraction line predicts 0 dB loss.

AdML=MddML+Ad0     (dB/km)	(9)

dd=-Ad0Md     (km)	(10)

Step 4: Determine if the propagation path is in the line-of-sight region or transhorizon for the desired distance d. If d<dML the path is line-of-sight and go to Step 5. Else, the path is transhorizon and go to Step 6.

Step 5: Refer to § 5 for the line-of-sight region calculations.

Step 6: In the transhorizon region (d ≥dML), as distance increases, the propagation path will begin with smooth Earth diffraction and transition into troposcatter. Physically, the models for smooth Earth diffraction and troposcatter need to be consistent at the transition point. Physical consistency implies there is no discontinuity at the transition point. The following iterative process ensures that the transition between the two models occurs without discontinuity.

Step 6.1: Let d' and d'' be the iterative test distances and be initialized to:

d'=dML+3     (km)	(11)

d''=dML+2     (km)	(12)

Step 6.2: Compute the troposcatter losses Asd' and Asd'' at distances d' and d'' respectively. Use § 7 as follows:

given:

d :	representing the path distance of interest d' and d'', in km

d1,2 :	arc length to the smooth Earth horizon distance of the terminals, in km

f :	frequency, in MHz

h1,2 :	adjusted height of the terminal above mean sea level used in subsequent calculations, in km

find:

Asd',d'' :	troposcatter loss, As, in dB.

Step 6.3: Compute the slope, Ms, of the line containing the two troposcatter points (d',Asd') and (d'', Asd'') from Step 6.2. This line is approximately tangent to the troposcatter loss at distance d'.

Ms=Asd'-Asd''d'-d''     (dB/km)	(13)

Step 6.4: Compare the slope Ms to the slope of the diffraction line, Md, from equation (7). If Ms>Md, then increase d'and d'' by 1 km and return to Step 6.2 to continue iterating. Else, proceed to Step 6.5.

Step 6.5: With Ms≤Md, the distance d' represents the approximate distance where either:

Case 1: Smooth Earth diffraction losses are predicted to be less than the losses due to troposcatter, and the diffraction model is guaranteed to intersect the troposcatter model at some distance ≥d'. The transhorizon region propagation loss is physically consistent.

Case 2: The diffraction line is parallel to the tangent of the troposcatter model. However, the transhorizon region propagation loss could be not physically consistent, i.e., there is a potential discontinuity.

To determine which of the above cases are true, find the diffraction loss at distance d''.

Add''=Mdd''+Ad0     (dB)	(14)

If Asd''≥Add'' , then Case 1 in Step 6.5 is true and calculation proceeds to Step 7. Else, the slope of the diffraction line should be adjusted to the tangent point d', ensuring physical consistency. The adjustment is done by pinning one end of the diffraction line at (dML, AdML) and the other end at (d'', Asd''), then re-compute the new smooth Earth diffraction line.

Md=Asd''-AdMLd''-dML     (dB/km)	(15)

Ad0=Asd'-Mdd'     (dB)	(16)

At this point the transhorizon region is physically consistent. Proceed to Step 7.

Step 7: Compute, AT, the loss not represented by free space loss and atmospheric absorption. This is determined based on the diffraction and troposcatter models, including any adjustments that were performed in Step 6.

Step 7.1: Compute the predicted smooth Earth diffraction loss, Ad, for the path distance d.

Ad=Mdd+Ad0     (dB)	(17)

Step 7.2: Compute the troposcatter loss, As, for the path distance d. Use § 7 as follows:

given:

d: 	path distance of interest, in km

d1,2: 	arc length to the smooth Earth horizon distance of the terminals, in km

f: 	frequency, in MHz

h1,2: 	adjusted height of the terminal above mean sea level used in subsequent calculations, in km

find:

As: 	troposcatter loss, in dB

hv:	height to the common volume, in km

ds: 	the scattering distance, in km

dz: 	half the scattering distance, in km

θA:	cross-over angle

Step 7.3: Select the loss value based on the following:

If d<d' (with d' originating from the final iteration in Step 6), then:

AT=Ad     (dB)	(18)

Else, depending on whether Case 1 or Case 2 was true in Step 6.5:

AT=Min(Ad, As),  Case 1 is TRUEAs,  Case 2 is TRUE     (dB)	(19)

Step 8: Compute the free space loss, Afs, in dB, for the path:

r1,2=hr1,22+4a0+hr1,2*a0sin20.5d1,2a0 0.5     (km)	(20)

rfs=r1+r2+ds     (km)	(21)

Afs=-32.45-20log10f-20log10rfs     (dB)	(22)

Step 9: Compute the atmospheric absorption loss, Aa, for a transhorizon path, using § 13. Then proceed to Step 10. Use the method presented in § 13 as follows:

given:

h1,2:	terminal heights, in km

d1,2:	terminal horizon distances, in km

θ1,2:	terminal grazing ray take off angle, in radians

f: 	frequency, in MHz

hv:	height to the common volume, in km, from Step 7.2

θA:	cross-over angle, from Step 7.2

dz: 	half the scattering distance, in km, from Step 7.2

find:

Aa: 	atmospheric absorption loss, in dB

Step 10: Compute the long-term variability loss, Ytotal(q), for the time quantile q. Use § 10. Then proceed to Step 11.

Given:

hr1,2:	actual terminal heights, in km

d:	path distance of interest, in km

f: 	frequency, in MHz

q:	time percentage

find:

Yq:	Long-term variability loss, in dB.

Step 11: Compute the basic transmission loss, A, in dB.

A=Afs+Aa+AT+Y(q)      (dB)	(23)

This completes the step by step procedure for the given user defined input parameters.

# 4	Terminal geometry

This section computes the following geometric parameters associated with a terminal.

Given:

hr:	real terminal height above mean sea level, in km

find:

d:	arc length to the smooth Earth horizon distance, in km

θ:	incident angle of the ray from the terminal to the smooth Earth horizon, in radians

h:	adjusted height of the terminal above mean sea level used in subsequent calculations, in km

Δh:	terminal height correction term, in km

As previously defined, the effective radius of the Earth, ae, is 8 493 km.

Step 1: Use ray tracing, as defined in § 5, to determine the following:

given:

hr:	real height of terminal above mean sea level (user input), in km

Ns:	surface refractivity of 301 N-Units

find:

dr:	real arc distance (real smooth Earth horizon), in km

θr:	ray incident angle on the terminal, in radians

Step 2: Compute the effective height of the terminal, he, in km, using the small angle approximation if required.

ϕ=drae     (rad)	(24)

he=dr22ae,  &ϕ≤ 0.1aecosϕ-ae,  &ϕ> 0.1     (km)	(25)

Step 3: When the effective height, he, is greater than the real height, hr, the effect of ray bending can be overestimated. Therefore, compare he with hr to determine the values of h, d, and θ to be used to define the terminal’s geometric parameters.

h=he,  &he≤hrhr,  &he>hr     (km)	(26)

d=dr,  &he≤hr2aehr,  &he>hr     (km)	(27)

θ=θr     (rad)	(28)

Step 4: Compute the terminal height correction term, Δh.

Δh=hr-h     (km)	(29)

Step 5: If Δh=0 km, then make the following adjustments to θ and d:

θ=2hrae     (rad)	(30)

d=2hrae     (km)	(31)

This concludes the section to compute the terminal geometry.

# 5	Ray tracing

Radio waves traveling through the atmosphere bend due to changes in the atmospheric refractivity. In traditional terrestrial models, this is normally accounted for using the standard “4/3 Earth” method, which models a linear atmospheric refractivity gradient and is a valid approximation for paths near the surface. However, the actual atmospheric gradient is exponential in nature and in air-to-ground propagation paths, the use of a linear model can incur substantial errors.

Recommendation ITU-R P.528 utilizes ray tracing techniques to compute the path of the ray through the atmosphere. The atmosphere is modelled as a set of concentric atmospheric shells, with exponentially decreasing refractivity. Applications of Snell’s Law in a spherical environment, shown in equation (32), are then applied to trace the ray.

niricosθi=ni+1ri+1cosθi+1	(32)

Figure 1 presents a generalized geometry of a ray through a single layer of the atmosphere.

Figure 1

Geometry of tracing a ray through a layer of atmosphere

For the atmospheric model, Recommendation ITU-R P.528 uses the 25-layer reference atmosphere shown in Table 1. Rays above 475 km are assumed to travel in a straight line.

TABLE 1

Details of the 25-layer reference atmosphere

| Layer, i | Height (AGL), hi | Layer, i | Height (AGL), hi | Layer, i | Height (AGL), hi |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 km | 9 | 1.00 km | 18 | 50.00 km |
| 1 | 0.01 km | 10 | 1.524 km | 19 | 70.00 km |
| 2 | 0.02 km | 11 | 2.00 km | 20 | 90.00 km |
| 3 | 0.05 km | 12 | 3.048 km | 21 | 110.00 km |
| 4 | 0.10 km | 13 | 5.00 km | 22 | 225.00 km |
| 5 | 0.20 km | 14 | 7.00 km | 23 | 350.00 km |
| 6 | 0.305 km | 15 | 10.00 km | 24 | 475.00 km |
| 7 | 0.50 km | 16 | 20.00 km |  |  |
| 8 | 0.70 km | 17 | 30.48 km |  |  |

Given:

hr:	the real terminal height above ground, in km

Ns:	the surface refractivity, in N-Units

find:

dr:	arc length to the smooth Earth horizon distance, in km

θr:	the incident angle of the grazing ray at the terminal, in radians

As previously defined, the actual radius of the Earth, a0, is 6 370 km.

Step 1: Compute the scale factor, ΔN:

ΔN=-7.32 e0.005577 Ns	(33)

Step 2: Compute the constant, Ce:

Ce=logNsNs+ΔN	(34)

Step 3: Ray tracing through the atmosphere is an iterative process, starting at the surface of the Earth and tracing up through each atmospheric layer until the height of the terminal is reached. Repeat the following sub-steps until the ray has been traced to hr. The use of subscripts i and i+1 designate the lower and upper limits of the current iteration’s atmospheric layer, respectively, as labelled in Fig. 1. For the first iteration (i = 0), let θ0=0 radians (representing a grazing ray).

Step 3.1: Compute the refractivity, Ni,i+1, the refractive index, ni,i+1, and the radial from the center of the Earth, ri,i+1 for the current atmospheric layer:

ri,i+1=a0+hi,i+1     (km)	(35)

Ni,i+1=Ns*exp-Cehi,i+1     (N-Units)	(36)

ni,i+1=1+(Ni,i+1*10-6)	(37)

Step 3.2: If hi+1>hr, corresponding to the terminal existing within the current atmospheric layer, then clamp the current layer iteration parameters to the terminal height and recompute the refractivity and refractive index:

ri+1=a0+hr     (km)	(38)

Ni+1=Ns*exp-Cehr     (N-Units)	(39)

ni+1=1+(Ni+1*10-6)	(40)

Step 3.3: Compute the ray exit angle, θi+1:

θi+1=cos-1riniri+1ni+1cosθi     (rad)	(41)

Step 3.4: Compute the atmospheric layer’s bending contribution, τi:

Ai=logni+1-lognilogri+1-logri	(42)

τi=θi+1-θi-AiAi+1     (rad)	(43)

Step 3.5: Repeat Step 3 for the next atmospheric layer until either (a) the height of the terminal has been reached or (b) the ray has escaped the atmosphere, i.e. reached a height of 475 km.

Step 4: If the ray has reached a height of 475 km and has still not reached the terminal height, hr, compute the incident angle, by applying a final iteration of Snell’s Law with ni+1=1, ri=a0+475 km, and ri+1=a0+hr km. Else, proceed to Step 5.

θi+1=cos-1(a0+ 475) nia0+hrcosθi     (rad)	(44)

Step 5: With the ray now traced from the surface of the Earth to the terminal, the incident angle, θr, is:

θr=θi+1     (rad)	(45)

Step 6: The total bending angle, τ, is the sum of the bending contributions of each layer traced:

τ=iτi     (rad)	(46)

Step 7: Compute the arc distance along the surface of the Earth that the ray travelled by using the central angle ϕ.

ϕ=θr+τ     (rad)	(47)

dr=ϕa0     (km)	(48)

This completes the section on ray tracing.

# 6	Line-of-sight region

This section describes the step stake to compute the propagation loss for a line-of-sight path.

Given:

dML:	maximum line-of-sight distance, in km

dd:	distance, in km

h1,2:	terminal heights, in km

d1,2:	terminal horizon distances, in km

f:	frequency, in MHz

AdML:	diffraction loss at distance dML, in dB

q: 	time percentage of interest

d: 	path distance of interest

find:

A: 	the basic transmission loss, in dB

K: 	value used in later variability calculations

Step 1: Compute the wavelength, λ.

λ=0.2997925f	(49)

Step 2: The calculations to compute the loss in the line-of-sight region do not have a closed-form solution and thus require multiple iterations in order to converge on the correct result. To assist this process, it is useful to build a table of tuples ψ, Δr, d, which can be reference throughout this section as a means to interpolate. In this table, ψ is the reflection angle of the indirect ray in radians, Δr is the difference in ray length between the direct ray and the indirect ray, and d is the path distance between the two terminals. The following sub-steps provide useful points to populate this reference table with.

Step 2.1: Add the tuple (0, 0, dML) to the table, representing the maximum extent of the line-of-sight region.

Step 2.2: Add a set of tuples which are based on fractions of λ. Let R be the set of constants 0.06, 0.1,19,18,17,16,15,14,13,12. For each constant r∈R, compute the angle ψ:

ψ=sin-1λr2he1      (rad)	(50)

Then use the ray optics method described in Section 7 to determine the values Δr and d for the reflection angle ψ. Add this tuple ψ, Δr, d to the table. Proceed to Step 2.3 after all 10 tuples have been computed and added to the table. Use Section 7 as follows:

given:

ψ :	the ray reflection angle, in radians;

hr1,2 :	actual terminal heights, in km;

Δh1,2:	terminal height correction terms, in km;

find:

Δr:	ray length distance between the direct ray and the indirect ray, in km;

d:	distance between the terminals corresponding to a reflection angle of ψ, in km.

Step 2.3: Add another set of tuples which again are based on fractions of λ. Using the same R defined in Step 2.2, compute the angle ψ for each constant r∈R:

ψ=sin-1λr2d1     (rad)	(51)

Then use the ray optics method described in § 7 to determine the values Δr and d for the reflection angle ψ. Add this tuple ψ, Δr, d to the table. Proceed to Step 2.4 after all 10 tuples have been computed and added to the table. Use § 7 as follows:

given:

ψ:	the ray reflection angle, in radians

hr1,2:	actual terminal heights, in km

Δh1,2:	terminal height correction terms, in km

find:

Δr:	ray length distance between the direct ray and the indirect ray, in km

d:	distance between the terminals corresponding to a reflection angle of ψ, in km

Step 2.4: Generate a set of tuples based on the following set, S, of reflection angles ψ. Let S= .2, .5, .7, 1, 1.2, 1.5, 1.7, 2, 2.5, 3, 3.5, 4, 5, 6, 7, 8, 10, 20, 45, 70, 80, 85, 88, 89 degrees. For each s∈S, compute the angle ψ, in radians,

ψ=sπ180     (rad)	(52)

Then use the ray optics method described in § 7 to determine the values Δr and d for the reflection angle ψ. Add this tuple ψ, Δr, d to the table. Proceed to Step 2.5 after all 24 tuples have been computed and added to the table. Use § 7 as follows:

given:

ψ: 	the ray reflection angle, in radians

hr1,2: 	actual terminal heights, in km

Δh1,2: 	terminal height correction terms, in km

find:

Δr:	ray length distance between the direct ray and the indirect ray, in km

d:	distance between the terminals corresponding to a reflection angle of ψ, in km

Step 2.5: Add the final tuple (π2, 2h1,0) to the table.

Step 3: Use the generated table to interpolate and determine the distance dλ/2, the distance corresponding to the distance at which Δr= λ2.  This is the minimum distance at which Recommendation ITU-R P.528 considers the effects of destructive interference through a two-ray model.

Step 4: Determine ψlimit, the reflecting angle corresponding to the distance dλ/2 by again using the generated table and interpolating.

Step 5: Use the generated table to determine the distance dλ6, the distance at which the difference in path lengths between the direct and reflected wave is λ/6 metres.

Step 6: Determine the distance d0 in km.

If d≥dd or dd≥dML,

do=d1,  &d>dλ/6 or dλ/6>dMLdλ/6,  &else     (km)	(53)

Else if dd<dλ/6 and dλ/6<dML,

do=dλ/6,  &dd<dλ/6 and dλ/6<dMLdd,  &else     (km)	(54)

Step 7: The current value of d0 can be a course approximation for certain paths. In order to tune it, iteratively convert the distance into a reflection angle ψ using the table and compute the ray optics as defined in § 7. If the resulting d from § 7 is greater than or equal to the original d0, or if increasing the distance 1 metre causes it to be beyond dML, use the ray optics output distance as the value for d0. Else, increase the distance by 1 meter and re-compute the ray optics method.

Step 8: Compute the line-of-sight loss at the distance d0. Use the table to generate the corresponding reflection angle ψd0. Then use § 7 to compute the ray optics for ψd0. Lastly, use § 8 to determine the loss, Ad0.

Step 9: Convert the desired distance d into its corresponding reflection angle ψ using the table. Then apply the ray optics calculations in § 7. In most cases, the resulting distance from the ray optics calculations will be different from the desired distance d. This is a source of error in the final result. To reduce this error, apply a small modification to ψ. An increase in ψ causes a decrease in the resulting ray optics distance. Iterate until the difference between the ray optics resulting distance and the desired distance is within the acceptable range of error. In general, ϵ≈1 metre is sufficient for all cases. Let the final resulting ray optics distance be called dro.

Step 10: Use § 8 to compute the line-of-sight loss. Then proceed to Step 11. Use § 8 as follows:

given:

ψ:	the ray reflection angle, in radians

hr1,2:	actual terminal heights, in km

Δh1,2:	terminal height correction terms, in km

find:

Δr:	ray length distance between the direct ray and the indirect ray, in km

d:	distance between the terminals corresponding to a reflection angle of ψ, in km

Step 11: Compute the atmospheric absorption loss for the path. The effective thicknesses of the absorbing layer for oxygen and water vapour are different. For oxygen, the effective thickness of the absorbing layer is Teo=3.25 km. For water vapour, the effective thickness of the absorbing layer is Tow=1.36 km.

Step 11.1: Compute the effective ray lengths through the oxygen and water vapour absorbing layers for the path using the steps described in § 8. This requires applying Section 8 twice: once for the oxygen absorbing layer and once for the water vapour absorbing layer. Then proceed to Step 11.2. Use § 8 as follows:

given:

z1:	the radial of the low point, in km

z2:	the radial of the high point, in km

aa:	the effective radius of the Earth, in km. Set to 8 493 km

r0:	the arc distance between the two points, in km, from equation (74)

θh1:	the ray take off angle, in radians, from equation (77)

Teo,ew:	the thickness of the absorbing layer, Te, in km, where Te=Teo=3.25 km for the oxygen absorbing layer and Te=Tew=1.36 km for the water vapour absorbing layer

find:

reo,ew:	the effective ray length, re, in km. The ray length through the oxygen absorbing layer, reo, corresponds to Te=Teo=3.25 km. The ray length through the water vapour absorbing layer, rew, corresponds to Te=Tew=1.36 km.

Step 11.2: Determine the atmospheric absorption rates for oxygen, γoo, and water vapour, γow, in dB/km using § 14. Then proceed to Step 11.3. Use § 14 as follows:

given:

f:	frequency, in MHz

find:

γoo:	oxygen absorption rate, in dB/km

γow:	water vapour absorption rate, in dB/km

Step 11.3: Compute the total atmospheric absorption loss, Aa, using the absorption rates γoo and γow from Step 11.2 and the effective ray lengths reo and rew from equation (153).

Aa=γooreo+γowrew     (dB)	(55)

Step 12: Compute the free space loss, Afs, in dB.

Step 12.1: Compute θfs.

θfs=aaθ1+θ2a0      (rad)	(56)

Step 12.2: Compute the radials, z1,2, in km.

z1,2=a0+h1,2     (km)	(57)

Step 12.3: Compute the ray length, rFS, in km.

rfs=maxz2-z12+4z1z2sin20.5 θfs0.5,z2-z1     (km)	(58)

Step 12.4: Compute the total free space loss, Afs, in dB.

Lbf=-32.45-20log10f     (dB)	(59)

Afs=Lbf-20log10rfs     (dB)	(60)

Step 13: Compute the contribution of variability to the total loss. Use § 16 to compute Ytotal. Then proceed to Step 14. Use § 16 as follows:

given:

hr1,2:	actual terminal heights, in km

d:	path distance of interest, in km

f: 	frequency, in MHz

q:	time percentage

find:

Yq:	long-term variability loss, in dB.

Step 14: Compute the basic transmission loss.

A=Afs+Aa+ALOS+Yq     (dB)	(61)

This concludes this section.

# 7	Line-of-sight ray optics

This section describes how to perform the geometric path parameters for two terminals within line-of-sight distance of each other using ray optics.

The inputs for this section are:

ψ: 	the ray reflection angle, in radians

hr1,2: 	actual terminal heights, in km

Δh1,2: 	terminal height correction terms, in km.

The output for this section is:

Δr: 	ray length distance between the direct ray and the indirect ray, in km

d: 	distance between the terminals corresponding to a reflection angle of ψ, in km.

Step 1: Compute the adjusted earth radius, aa:

z=a0ae-1	(62)

ka=11+cosψ	(63)

aa=a0ka     (km)	(64)

Step 2: Compute the adjusted earth terminal height correction terms, Δha1,2.

Δha1,2=Δh1,2aa-a0ae-ao     (km)	(65)

Step 3: Compute the heights, H1,2.

H1,2=hr1,2-Δha1,2     (km)	(66)

Step 4: Compute the terminal geometric parameters z1,2, θ1,2, D1,2, and H1,2'.

z1,2=aa+H1,2     (km)	(67)

θ1,2= cos-1aacosψz1,2-ψ     (rad)	(68)

D1,2=z1,2sinθ1,2     (km)	(69)

H1,2'=H1,2,  &ψ>1.56D1,2tanψ,  &ψ ≤ 1.56     (km)	(70)

Step 5: Compute Δz, the difference in the terminal radials.

Δz=z1-z2     (km)	(71)

Step 6: Compute the path distance between the two terminals corresponding to a reflection angle of ψ.

d=maxaaθ1+ θ2, 0     (km)	(72)

Step 7: With the geometric parameters computed, determine the length of the direct ray, r0, and the indirect ray, r12.

α=tan-1H2'-H1'D1+D2     (rad)	(73)

r0=D1+D2cosα     (km)	(74)

r12=D1+D2cosψ     (km)	(75)

Step 8: Compute the difference in the length between the two rays.

Δr=4 H1'H2'r0+r12     (km)	(76)

Step 9: Compute the angles θh1,2.

θh1=α- θ1     (rad)	(77)

θh2=-α+ θ2     (rad)	(78)

This completes this section.

# 8	Line-of-sight loss calculations

This section describes how to compute the loss calculations for a line-of-sight path.

Given:

d: 	path distance of interest, in km

ψ: 	the ray reflection angle, in radians

d0: 	path distance at which diffraction begins to affect the line-of-sight region, in km

f: 	frequency, in MHz

find:

ALOS: 	line-of-sight loss, in dB.

Step 1: If the path distance, d, is greater than d0, then the path is within the LOS-Diffraction blending region and the loss ALOS is determine using equation (79). Else, d≤d0 and skip to Step 2.

ALOS=d-d0AdML-Ad0dML-d0+Ad0     (dB)	(79)

Step 2: If the reflection angle ψ<ψlimit, then set ALOS=0 dB and return, as Recommendation ITU-R P.528 does not consider a 2-ray model within this region. Else, ψ≥ψlimit and skip to Step 3.

Step 3: The path length d, is such that Recommendation ITU-R P.528 uses a 2-ray model. Compute the wavelength, λ.

λ=0.2997925f	(80)

Step 4: Compute the complex reflection coefficients, Rg and ϕg using § 9.

Step 5: The divergence factor, Dv, takes into account that a reflection from a smooth curved Earth surface is less efficient than from a flat Earth. Compute the divergence factor for the path.

Dv=1+2Rr1+sin2ψaasinψ+2Rraa2-12	(81)

Step 6: The ray length factor, Fr, takes into account geometries in which the direct ray is significantly larger (and shorter) than the indirect ray, such as when both terminals are high and close in the case of two aircraft. Compute Fr:

Fr=minr0r12,  1	(82)

Step 7: Compute the effective reflection coefficients RTg and ϕTg.

RTg=RgDvFr	(83)

ϕTg=2 πΔrλ+ϕg	(84)

Step 8: Compute the loss, ALOS.

R=RTgcosϕTg-RTgsinϕTg	(85)

WRL=min1+R, 1	(86)

WR0=WRL2	(87)

ALOS=10log10WR0	(88)

This completes this section.

# 9	Ground reflection coefficients

This section describes the steps taken to compute the ground reflection coefficients.

Given:

ψ:	reflection angle, in radians

f:	frequency, in MHz

find:

Rg:	the real part of the reflection coefficient

ϕg:	the imaginary part of the reflection coefficient.

With the previously stated assumptions for the electrical properties of the ground of σ=0.005 S/m and ϵr=15 (corresponding to average ground).

Step 1: Compute the following values,

X=18 000 σf	(89)

Y=ϵr-cos2ψ	(90)

T=Y2+X20.5+Y	(91)

P=0.5T0.5	(92)

Q=X2P	(93)

B=1(P2+Q2)	(94)

A=2PP2+Q2	(95)

Step 2: Compute the real and imaginary parts of the reflection coefficient.

Rg=1+Bsin2ψ-Asinψ 1+Bsin2ψ+Asinψ0.5	(96)

ϕg=tan2-1-Q,sinψ-P-tan2-1Q,sinψ+P	(97)

This concludes this section.

# 10	Smooth Earth diffraction

This section describes the steps taken to compute the smooth Earth diffraction loss at a specified distance within the diffraction zone. The model assumes an “average ground” with a conductivity of 0.005 S/m and a relative dielectric constant of 15. The effective earth radius factor, k, of 4/3 is assumed (corresponding to a surface refractivity Ns=300 N-Units). Horizontal polarization is assumed.

Given:

d0: 	path distance of interest, in km

d1,2: 	horizon distance for the terminals, in km

f: 	frequency, in MHz

find:

Ad: 	smooth Earth diffraction loss, in dB.

Smooth Earth diffraction is computed using equation (98):

Ad=Gx0-Fx1-Fx2-20     (dB)	(98)

Step 1: Compute the normalized distances.

x0,1,2=1.607f13d0,1,2     (km)	(99)

Step 2: Compute the distance dependent term for all three normalized distances.

Gx0,1,2=0.05751x0,1,2-10log10x0,1,2     (dB)	(100)

Step 3: Compute the term y1,2

y1,2=40log10x1,2-117     (dB)	(101)

Step 4: Compute the height functions.

If x1,2≥2 000 km:

Fx1,2=Gx1,2     (dB)	(102)

Else if 200 <x1,2<2 000 km:

W1,2=0.0134x1,2e-0.005x1,2	(103)

Fx1,2=W1,2y1,2+1-W1,2Gx1,2     (dB)	(104)

Else, x≤200.

Fx1,2=y1,2     (dB)	(105)

Step 5: With G(x0) and F(x1,2) computed, use equation (98) above to compute the smooth Earth diffraction loss.

This completes this section.

# 11	Troposcatter

This section describes the steps taken to compute the troposcatter for a given distance. The computation of troposcatter loss is performed using mathematical techniques that consider the curved ray paths bounding the common volume of the two terminals. The earth radius factor k of 4/3 is assumed (surface refractivity Ns=301 N-Units). Troposcatter is computed using equation (106) (described below).

As=Se+SV+10 log10κθs3l     (dB)	(106)

given:

d1,2:	arc length along the surface of the smooth Earth, in km

h1,2:	terminal heights, in km

f:	frequency, in MHz

d:	path distance of interest, in km

find:

As: 	troposcatter loss, in dB.

Step 1: Compute the scatter distance, ds, in km.

ds=d-d1-d2     (km)	(107)

Step 2: If ds=0, there is no common volume in the path geometry and thus propagation via troposcatter is not supported. Set the below results and return from § 7. Else ds>0 and continue to Step 3.

As=0     (dB)	(108)

hv=0     (km)	(109)

dz=0     (km)	(110)

θs=0     (rad)	(111)

θA=0     (rad)	(112)

Step 3: The smooth Earth arc distance from each terminal’s grazing ray to the centre of the common volume, dz, is:

dz=0.5ds     (km)	(113)

Step 4: Compute the atmospheric gradient parameters:

Am=1a0	(114)

dN=Am-1ae	(115)

γe=Ns×10-6/ dN	(116)

Step 5: The following equations are used to determine the geometric parameters associated with troposcatter, including the height of the common volume, hv, in km, and the ray slopes at the cross-over angle, θA, in radians.

za=12aedz22     (km)	(117)

zb=12aedz2     (km)	(118)

Qo=Am-dN	(119)

Qa,b=Am-dN*e-za,bγe	(120)

Za=7Qo+6Qa-Qbdz296      (km)	(121)

Zb=Qo+2Qadz26     (km)	(122)

QA,B=Am-dN*e-Za,bγe	(123)

hv=Qo+2QAdz26     (km)	(124)

θA=Qo+4QA+QBdz6     (rad)	(125)

θs=2 θA     (rad)	(126)

Step 6:	Compute the scattering efficiency term, Se.

ϵ1=5.67×10-6Ns2-0.00232Ns+0.031 	(127)

ϵ2=0.0002Ns2-0.06Ns+6.6	(128)

γ=0.14241+ϵ1exphv46	(129)

Se=83.1- ϵ21+0.07716hv2+20log100.1424γ2eγhv     (dB)	(130)

Step 7: Compute the scattering volume term, SV.

XA1,2=h1,22+4ae+h1,2aesin2d1,22ae	(131)

l1,2=XA1,2+dz     (km)	(132)

l=l1+ l2     (km)	(133)

s=l1- l2l	(134)

η=γθsl2	(135)

κ=f0.0477	(136)

ρ1,2=2 κ θsh1,2     (km)	(137)

SV=10 log10Aη2+BSηq1q2ρ12ρ22+CS     (dB)	(138)

where:

Xv1=1+s2η	(139)

Xv2=1-s2η	(140)

q1= Xv12+ ρ12	(141)

q2=Xv22+ρ22	(142)

A=1-s22	(143)

BS=6+8s2+81-sXv12ρ12q12+81+sXv22ρ22q22+ 21-s21+2Xv12/q11+2Xv22/q2	(144)

CS=12 ρ1+2ρ12ρ2+ 2ρ22ρ1+ ρ2ρ1+ ρ2+22	(145)

Step 8: Compute the troposcatter loss using equation (106).

This completes this section.

# 12	Effective ray length

This section describes the steps to determine the effective ray length for use in the calculations of atmospheric absorption. This section uses a geometric model in which an absorbing layer sits upon the surface of the Earth and is defined by an effective thickness. Geometric calculations are then performed to determine the effective ray length, the portion of the ray path that is contained within the absorbing layer.

Three possible geometries exist, as showing in Fig. 2:

1	Scenario 1: Both endpoints are within the absorbing layer

2	Scenario 2: One endpoint is within the absorbing layer and one endpoint is above the absorbing layer

3	Scenario 3: Both endpoints are above the absorbing layer, although the ray path could still intersect the absorbing layer

Figure 2

The three possible geometries for determining the effective ray length relative to an absorbing layer

Given:

z1,2:	radials, in km

a:	radius of the Earth, in km. The radius is dependent on the type of path

darc:	arc length between the two radials along the surface of the Earth, in km

β:	ray take off angle, relative to the tangent of the Earth surface, in radians

Te:	effective thickness of the absorbing layer, in km

find:

re: 	Effective ray length, in km.

Step 1: Compute the angle α and the radial zT.

α=(π/ 2) +β     (rad)	(146)

zT=a+Te     (km)	(147)

Step 2: It is necessary to determine the correct geometry based on the parameters provided. If ze≤zT, then both endpoints are within the absorbing layer resulting in Scenario 1 in Fig. 2. Compute re using equation (148) and return. Else continue to Step 3.

re=darc     (km)	(148)

Step 3: If zT<z1, then both terminals are above the absorbing layer resulting in Scenario 3 in Fig. 2 and continue with this step. Else, proceed to Step 4.

To determine if the ray path intersects with the absorbing layer, compute they radial zc, the lowest point along the ray path. Then use equation (150) to determine the correct value for re.

zc=z1sinα     (km)	(149)

re=0,  &zT≤zc2 zTsincos-1zczT,  &else     (km)	(150)

Step 4: The geometry is determined to be that of Scenario 2 in Fig. 2, with the low terminal being within the absorbing layer and the high terminal being above it. The following equations are used to determine the portion of the ray path that is within the absorbing layer.

Aq=sin-1z1sinαzT     (rad)	(151)

Ae=π-α-Aq     (rad)	(152)

re=zT-z1,  &Ae=0z1sinAesinAq,  &Ae≠0     (km)	(153)

This completes this section.

# 13	Atmospheric absorption loss for transhorizon paths

This section describes the steps to compute the loss due to atmospheric absorption for transhorizon paths.

Given:

h1,2:	terminal heights, in km

d1,2:	horizon distances, in km

f:	frequency, in MHz

hv:	height to the common volume, in km

θA: 	cross-over angle at the common volume, in radians

find:

Aa: 	atmospheric absorption loss, in dB.

Step 1: Compute the radials z1, z2, and zv from the centre of the Earth to the low terminal, the high terminal, and the common volume, respectively.

z1=h1+ae     (km)	(154)

z2=h2+ae     (km)	(155)

zv=hv+ae     (km)	(156)

Step 2: Focus on the portion of the path from the low terminal to the common volume, noting that the height of the common volume can be above or below that of the low terminal. To compute the effective ray length for this portion of the path, set the parameters zlow1, zhigh1, and β1 so that the geometric parameters are consistent with how the calculations for effective ray length is presented.

zlow1=zv,  &zv<z1z1,  &zv≥z1     (km)	(157)

zhigh1=z1,  &zv<z1zv,  &zv≥z1     (km)	(158)

β1=-tan-1θA,  &zv<z1-θ1,  &zv≥z1     (rad)	(159)

Step 3: Now focus on the portion of the path from the common volume to the high terminal, noting that the height of the common volume can be above or below that of the high terminal. To compute the effective ray length for this portion of the path, set the parameters zlow2, zhigh2, and β2 so that the geometric parameters are consistent with how the calculations for effective ray length is presented.

zlow2=zv,  &zv<z2z2,  &zv≥z2     (km)	(160)

zhigh2=z2,  &zv<z2zv,  &zv≥z2     (km)	(161)

β2=-tan-1θA,  &zv<z2-θ2,  &zv≥z2     (rad)	(162)

Step 4: Compute the arc distances for each of the two above portions of the path.

darc1=d1+dz     (km)	(163a)

darc2=d2+dz     (km)	(163b)

Step 5: The effective thicknesses of the absorbing layer for oxygen and water vapour are different. For oxygen, the effective thickness of the absorbing layer is Teo=3.25 km. For water vapour, the effective thickness of the absorbing layer is Tow=1.36 km. Compute the effective ray lengths through the oxygen and water vapour absorbing layers for the first portion of the path (from the low terminal to the common volume) using the steps described in § 8. This requires applying § 8 twice: once for the oxygen absorbing layer and once for the water vapour absorbing layer. Then proceed to Step 6. Use § 8 as follows:

given:

zlow1: 	the radial of the low point, in km, from equation (157)

zhigh1: 	the radial of the high point, in km, from equation (158)

ae: 	the effective radius of the Earth, in km. Set to 8 493 km

darc1: 	the arc distance between the two points, in km, from equation (163a)

β1:	the ray take off angle, in radians, from equation (159)

Teo,ew:	the thickness of the absorbing layer, Te, in km, where Te=Teo=3.25 km for the oxygen absorbing layer and Te=Tew=1.36 km for the water vapour absorbing layer

find:

reo1,ew1: 	the effective ray length, re, in km. The ray length through the oxygen absorbing layer, reo, corresponds to Te=Teo=3.25 km. The ray length through the water vapour absorbing layer, rew, corresponds to Te=Tew=1.36 km.

Step 6: Compute the effective ray lengths through the oxygen and water vapour absorbing layers for the second portion of the path (from the height of the common volume to the high terminal), in the same manner as was done in Step 5 but using the geometric parameters associated with the path segment from the common volume to the high terminal. Then proceed to Step 7. Use § 8 as follows:

given:

zlow2: 	the radial of the low point, in km, from equation (160)

zhigh2: 	the radial of the high point, in km, from equation (161)

ae: 	the effective radius of the Earth, in km. Set to 8 493 km

darc2: 	the arc distance between the two points, in km, from equation (163b)

β2:	the ray take off angle, in radians, from equation (162)

Teo,ew:	the thickness of the absorbing layer, Te, in km, where Te=Teo=3.25 km for the oxygen absorbing layer and Te=Tew=1.36 km for the water vapour absorbing layer

find:

reo2,ew2: 	the effective ray length, re, in km. The ray length through the oxygen absorbing layer, reo, corresponds to Te=Teo=3.25 km. The ray length through the water vapour absorbing layer, rew, corresponds to Te=Tew=1.36 km.

Step 7: Compute the total effective ray lengths for the oxygen absorbing layer, reo, and for the water vapour absorbing layer, rew, in km.

reo=reo1+reo2     (km)	(164a)

rew=rew1+rew2     (km)	(164b)

Step 8: Determine the atmospheric absorption rates for oxygen, γoo, and water vapour, γow, in dB/km using § 14. Then proceed to Step 9. Use § 14 as follows:

given:

f :	frequency, in MHz

find:

γoo: 	oxygen absorption rate, in dB/km

γow: 	water vapour absorption rate, in dB/km.

Step 9: Compute the total atmospheric absorption loss, Aa, using the absorption rates γoo and γow from Step 8 and the effective ray lengths reo and row from equation (164).

Aa=γooreo+γowrew     (dB)	(165)

This concludes the calculation of atmospheric absorption for a transhorizon path.

# 14	Atmospheric absorption rates

This section describes the steps taken to determine the absorption rate of oxygen, γoo, and water vapour, γow, in dB/km.

Given:

f :	frequency, in MHz

find:

γoo: 	oxygen absorption rate, in dB/km

γow: 	water vapour absorption rate, in dB/km.

TABLE 2

Data on absorption rates vs frequency

| f (MHz) | γoo (dB/km) | γow (dB/km) | f (MHz) | γoo (dB/km) | γow (dB/km) | f (MHz) | γoo (dB/km) | γow (dB/km) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 0.000 19 | 0 | 550 | 0.002 5 | 0 | 4 000 | 0.010 | 0.0001 7 |
| 150 | 0.000 42 | 0 | 700 | 0.003 | 0 | 4 900 | 0.011 | 0.003 4 |
| 205 | 0.000 70 | 0 | 1 000 | 0.004 2 | 0 | 8 300 | 0.014 | 0.002 1 |
| 300 | 0.000 96 | 0 | 1 520 | 0.005 | 0 | 10 200 | 0.015 | 0.009 |
| 325 | 0.001 3 | 0 | 2 000 | 0.007 | 0 | 15 000 | 0.017 | 0.025 |
| 350 | 0.001 5 | 0 | 3 000 | 0.008 8 | 0 | 17 000 | 0.018 | 0.045 |
| 400 | 0.001 8 | 0 | 3 400 | 0.009 2 | 0.000 1 |  |  |  |

Step 1: Use the values in Table 2 to interpolate for γoo and γow. Let frequencies f' and f'' be selected from the table such that f'<f<f''. Likewise for γoo'< γoo< γoo'' and γow'< γow< γow''.

Step 2: Compute the interpolation scale factor, R.

R=log10f-log10f'log10f''-log10(f')	(166)

Step 3: Interpolate the value γoo.

X=Rlog10γoo''-log10γoo'+log10(γoo')	(167)

γoo=10X     (dB/km)	(168)

Step 4: Interpolate the value γow. Note that the first 13 values of γow in Table 2 are 0 dB/km. Thus care should be taken when interpolating. If f<3400, then γow=0 dB/km. Else:

Y=Rlog10γow''-log10γow'+log10(γow')	(169)

γow=10Y     (dB/km)	(170)

This completes this section.

# 15	Total variability for transhorizon paths

This section defines how to compute the total contribution of variability to the median basic transmission loss for a transhorizon path.

Given:

hr1,2:	real terminal heights, in km

q:	time percentage of interest. Model input variable

f:	frequency, MHz

d:	path distance of interest, in km

AT:	loss predicted by either diffraction or troposcatter, in dB

θs:	the scattering angle, in radians

find:

Ytotalq: 	total variability loss, in dB.

Step 1: Compute the contribution of long term variability for the time percentage q by using § 17. Then proceed to Step 2. Use § 17 as follows:

given:

hr1,2:	real terminal heights, in km

d:	path distance of interest, in km

f:	frequency, MHz

q:	time percentage of interest

fθh:	set to the value fθh=1

AT:	loss predicted by either diffraction or troposcatter, in dB

find:

Yeq: 	long term variability loss, in dB.

Step 2: In order to correctly combine the effects of long term variability and tropospheric multipath, both of which are distributions, the mean value of the long term variability distribution is required. Compute the contribution of long term variability for time percentage 0.50 by using § 17. Then proceed to Step 3. Use § 17 as follows:

given:

hr1,2:	real terminal heights, in km

d:	path distance of interest, in km

f:	frequency, MHz

0.50:	the mean time percentage (q=0.50)

fθh:	set to the value fθh=1

AT:	loss predicted by either diffraction or troposcatter, in dB

find:

Ye0.50:	long term variability loss, in dB.

Step 3: In order to smoothly transition the effects of tropospheric multipath from the line-of-sight region to the transhorizon region, the value of K, from which tropospheric multipath is determined, should be determined at transition point between line-of-sight and non-line-of-sight. Compute the line-of-sight loss calculations, as described in § 6. Then proceed to Step 3. Use § 6 as follows:

given:

dML:	maximum line-of-sight distance, in km

dd:	distance, in km

h1,2:	terminal heights, in km

d1,2:	horizon distances, in km

f:	frequency, in MHz

AdML:	diffraction loss at distance dML, in dB

q:	time percentage of interest

d:	path distance of interest

find:

A:	the basic transmission loss, in dB

K:	value used in later variability calculations.

Step 4: Compute the value Kt which is used to determine the effects of tropospheric multipath. Let θ1.5=0.02617993878 radians (1.5 degrees).

Kt=                                     20, θs≥θ1.5                             KLOS, θs≤0θs20-KLOS/θ1.5+KLOS, 0< θs< θ1.5 	(171)

Step 5: Compute the contribution of tropospheric multipath for time percentage q using § 18. Then proceed to Step 6. Use § 18 as follows:

given:

Kt:	the value K

q: 	time percentage of interest

find:

Yπq: 	contribution of tropospheric multipath at time percentage q, in dB.

Step 6: Combine the effects long term variability with tropospheric multipath to get the total variability contribution, Ytotalq, using the previously computed values Yeq, Ye0.50, and Yπq. The mean of tropospheric multipath is Yπ0.50 = 0.

Ytotal0.50=Ye0.50+Yπ(0.50)	(172)

Y=Yeq-Ye(0.50)2+Yπq-Yπ(0.50)20.5	(173)

Ytotal=Ytotal0.50+Y,  &q<0.50Ytotal0.50-Y,  &q≥0.50     (dB)	(174)

This completes this section.

# 16	Total variability for line-of-sight paths

This section defines how to compute the contribution of variability to the median basic transmission loss.

Given:

hr1,2:	real terminal heights, in km

q:	time percentage of interest. Model input variable

f:	frequency, MHz

d:	path distance of interest, in km

AT:	loss predicted, in dB

θs:	the scattering angle, in radians

fθh:	input value

find:

Ytotalq: 		Total variability loss, in dB.

Step 1: Compute the value fθh using the value of θh1 from the ray optics calculations previously.

fθh=1 , θh1≤00 , θh1≥0max0.5-0.31831tan-120log1032 θh1, 0 , else	(175)

Step 2: Compute the contribution of long term variability for the time percentage q by using § 17. Then proceed to Step 3. Use § 17 as follows:

given:

hr1,2:	real terminal heights, in km

d:	path distance of interest, in km

f:	frequency, MHz

q:	time percentage of interest

fθh:	input value to this section

AT:	loss predicted, in dB

find:

Yeq: 	long term variability loss, in dB.

Step 3: In order to correctly combine the effects of long term variability and tropospheric multipath, both of which are distributions, the mean value of the long term variability distribution is required. Compute the contribution of long term variability for time percentage 0.50 by using § 17. Then proceed to Step 3. Use § 17 as follows:

given:

hr1,2:	real terminal heights, in km

d:	path distance of interest, in km

f:	frequency, MHz

0.50:	the mean time percentage (q=0.50)

fθh:	input value to this section

AT:	loss predicted, in dB

find:

Ye0.50: 	Long term variability loss, in dB.

Step 4: Compute the following value of KLOS, used for determining the effects of tropospheric multipath, as follows:

FAY=1, AY≤00.1, AY≥91.1+0.9cosπAY/ 92, else	(176)

FΔr=1, Δr≥λ/20.1, Δr≤λ/60.51.1-0.9cos((3π/λ) (Δr-λ/6)) , else	(177)

Rs=RTsFΔrFAY	(178)

If rew, the effective ray path through the water vapour absorbing layer is 0 km, then Wa=0.0001. Else, compute the value of Yπ0.99, as:

Yπ0.99=10log10f rew3-84.26     (dB)	(179)

Then use Table 7 to interpolate for the value of K corresponding to Yπ0.99, and use that value K to compute Wa as:

Wa=100.1K	(180)

With Wa computed, complete the computation of KLOS as:

WR=Rs2+0.012	(181)

W=WR+Wa	(182)

KLOS=0,  &W≤010log10W,  &W>0	(183)

Step 5: Compute the contribution of tropospheric multipath for time percentage q using § 18. Then proceed to Step 6. Use § 18 as follows:

given:

KLOS:	value set as KT

q: 	time percentage of interest

find:

Yπq: 	contribution of tropospheric multipath at time percentage q, in dB.

Step 6: Combine the effects long term variability with tropospheric multipath to get the total variability contribution, Ytotalq, using the previously computed values Yeq, Ye0.50, and Yπq. The mean of tropospheric multipath is Yπ0.50 = 0.

Ytotal0.50=Ye0.50+Yπ(0.50)	(184)

Y=Yeq-Ye(0.50)2+Yπq-Yπ(0.50)20.5	(185)

Ytotal=Ytotal0.50+Y,  &q<0.50Ytotal0.50-Y,  &q≥0.50     (dB)	(186)

This completes this section.

# 17	Long term variability

This section describes the steps taken to compute the statistical distribution of long-term variability for the desired time percentage, q. Long term variability utilizes a normalized effective distance, de, on an effective earth radius of 9 000 km (corresponding to Ns=329 N-Units). This section relies on statistical parameters that are based on long term empirical measurement data.

Given:

h1,2:	terminal heights, in km

q:	time percentage of interest. Model input variable

f:	frequency, MHz

d:	path distance of interest, in km

fθh:	previously calculated parameter

AT:	loss predicted by either LOS, Diffraction, or Troposcatter models (previously calculated), in dB

find:

Yeq: 	long term variability loss, in dB.

Note: The inverse complementary cumulative normal distribution function, Q-1(q), is used in various places in this section. A technique to approximate its value for this step-by-step procedure is included in Recommendation ITU-R P.1057.

Step 1: Compute the smooth Earth horizon distances via ray tracing for each terminal. Let each terminals horizon be defined as dLq1,2. Use § 4 as follows:

given:

h1,2:	terminal heights, in km

Ns:	surface refractivity of 329 N-Units

find:

dLq1,2: 	arc distance (smooth Earth horizon), in km.

Step 2: Compute de, the effective distance in kilometres between the two terminals.

dqs=60100f13     (km)	(187)

dLq=dLq1+dLq2     (km)	(188)

dq=dLq+dqs     (km)	(189)

de=130 ddq,  &d≤dq130+d-dq,  &d>dq     (km)	(190)

Step 3: Compute g0.1 and g0.9.

g0.1=0.21sin5.22log10f200+1.28,  &f≤16001.05,  &f>1600	(191)

g0.9=fx=0.18sin5.22log10f200+1.23,  &f≤16001.05,  &f>1600	(192)

Step 4: Compute V0.5, Y00.1, and Y00.9 using the below equations and the values from Table 3.

TABLE 3

Values to compute long term variability equations

|  | V0.5 | Y00.1 | Y00.9 |
| --- | --- | --- | --- |
| c1 | 1.59e-5 | 5.25e-4 | 2.93e-4 |
| c2 | 1.56e-11 | 1.57e-6 | 3.75e-8 |
| c3 | 2.77e-8 | 4.70e-7 | 1.02e-7 |
| n1 | 2.32 | 1.97 | 2.00 |
| n2 | 4.08 | 2.31 | 2.88 |
| n3 | 3.25 | 2.90 | 3.15 |
| f∞ | 0.0 | 5.4 | 3.2 |
| fm | 3.9 | 10.0 | 8.2 |

f2=f∞+fm-f∞exp-c2den2	(193)

V(0.5)Y0(0.1)Y0(0.9)=c1den1-f2exp-c3den3+f2     (dB)	(194)

Step 5: Compute Yeq, the variability associated with long-term (hour-to-hour) power fading, based on the desired time percentage, q.

If q=0.50, then:

Yq=V0.5     (dB)	(195)

If q>0.50, then:

z0.9=Q-1(0.9)	(196)

zq=Q-1q	(197)

cq=zqz0.9	(198)

Y=cq-Y00.9*g0.9     (dB)	(199)

Yq=Y+V(0.5)     (dB)	(200)

If q<0.50, then additional steps should be taken. If q≥0.10, then:

z0.1=Q-1(0.1)	(201)

zq=Q-1(q)	(202)

cq=zqz0.1	(203)

Y=cqY00.1*g0.1     (dB)	(204)

Yq=Y+V(0.5)     (dB)	(205)

Else, 0.01≤q<0.10. Use Table 4 of values to linearly interpolate cq from q. Then apply equations (204) and (205) to get Yq.

TABLE 4

Low probability values for cq

| q | cq |
| --- | --- |
| 0.10 | 1.000 0 |
| 0.05 | 1.326 5 |
| 0.02 | 1.716 6 |
| 0.01 | 1.950 7 |

Step 6: Compute Y0.10, the variability associated with long-term (hourly) power fading for 
q=0.10.

Y0.10=Y00.1*g0.1+V(0.5)     (dB)	(206)

Step 7: Compute YeIq and YeI0.1.

YeIq=fθh Yq     (dB)	(207)

YeI0.1=fθhY0.10     (dB)	(208)

Step 8: Compute AY, which is used to prevent available signal powers from exceeding levels expected for free-space propagation by an unrealistic amount when the variability around the median is large and near its free-space level.

AYI=YeI0.1-AT-3     (dB)	(209)

AY=maxAYI, 0     (dB)	(210)

Step 9: If q≥0.10, compute the total variability loss, completing this section. Else, proceed to Step 10 and continue.

Yeq=YeIq-AY     (dB)	(211)

Step 10: For time percentages less than 10% q<0.10, an additional correction may be required. Compute the value Ytemp.

Ytemp=YeIq-AY-AT     (dB)	(212)

Step 11: Use Table 5 to linearly interpolate cYq from q.

TABLE 5

Low probability correction values

| q | cYq |
| --- | --- |
| 0.10 | 0.00 |
| 0.05 | –3.70 |
| 0.02 | –4.50 |
| 0.01 | –5.00 |

Step 12: Compute the total variability loss.

Yeq=-cYq+AT,  &Ytemp> -cYqYtemp+AT,  &else	(213)

This concludes the long term variability section.

# 18	Tropospheric multipath

This section describes how to compute the contribution to the total variability due to tropospheric multipath.

Given:

K:	input parameter

q:	time percentage of interest

find:

Yπ(q): 	variability for q, in dB.

This section utilizes tabular data of the Nakagami-Rice distribution. Table 6 presents the data for q<0.50 and Table 7 presents the data for q>0.50. For all values of q= 0.50, Yπq=0 dB.

TABLE 6

Low time percentage values for the Nakagami-Rice distribution

| K | Yπ0.01 | Yπ0.02 | Yπ0.05 | Yπ0.10 | Yπ0.15 | Yπ0.20 | Yπ0.30 | Yπ0.40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| −40 | -0.1417 | -0.1252 | -0.1004 | -0.0784 | -0.0634 | -0.0515 | -0.0321 | -0.0155 |
| −25 | -0.7676 | -0.6811 | -0.5497 | -0.4312 | -0.3504 | -0.2856 | -0.1790 | -0.0870 |
| −20 | -1.3183 | -1.1738 | -0.9524 | -0.7508 | -0.6121 | -0.5003 | -0.3151 | -0.1537 |
| −18 | -1.6263 | -1.4507 | -1.1805 | -0.9332 | -0.7623 | -0.6240 | -0.3940 | -0.1926 |
| −16 | -1.9963 | -1.7847 | -1.4573 | -1.1557 | -0.9462 | -0.7760 | -0.4916 | -0.2410 |
| −14 | -2.4355 | -2.1829 | -1.7896 | -1.4247 | -1.1695 | -0.9613 | -0.6113 | -0.3007 |
| −12 | -2.9491 | -2.6507 | -2.1831 | -1.7455 | -1.4375 | -1.1846 | -0.7567 | -0.3737 |
| −10 | -3.5384 | -3.1902 | -2.6407 | -2.1218 | -1.7535 | -1.4495 | -0.9307 | -0.4619 |
| −8 | -4.1980 | -3.7974 | -3.1602 | -2.5528 | -2.1180 | -1.7565 | -1.1345 | -0.5662 |
| −6 | -4.9132 | -4.4591 | -3.7313 | -3.0306 | -2.5247 | -2.1011 | -1.3655 | -0.6855 |
| −4 | -5.6559 | -5.1494 | -4.3315 | -3.5366 | -2.9578 | -2.4699 | -1.6150 | -0.8154 |
| −2 | -6.3810 | -5.8252 | -4.9219 | -4.0366 | -3.3871 | -2.8364 | -1.8638 | -0.9455 |
| 0 | -7.0247 | -6.4249 | -5.4449 | -4.4782 | -3.7652 | -3.1580 | -2.0804 | -1.0574 |
| 2 | -7.5229 | -6.8862 | -5.8424 | -4.8090 | -4.0446 | -3.3927 | -2.2344 | -1.1347 |
| 4 | -7.8532 | -7.1880 | -6.0963 | -5.0145 | -4.2145 | -3.5325 | -2.3227 | -1.1774 |
| 6 | -8.0435 | -7.3588 | -6.2354 | -5.1234 | -4.3022 | -3.6032 | -2.3656 | -1.1975 |
| 20 | -8.2238 | -7.5154 | -6.3565 | -5.2137 | -4.3726 | -3.6584 | -2.3979 | -1.2121 |

TABLE 7

High time percentage values for the Nakagami-Rice distribution

| K | Yπ0.60 | Yπ0.70 | Yπ0.80 | Yπ0.85 | Yπ0.90 | Yπ0.95 | Yπ0.98 | Yπ0.99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| −40 | 0.0156 | 0.0323 | 0.0518 | 0.0639 | 0.0791 | 0.1016 | 0.1271 | 0.1441 |
| −25 | 0.0878 | 0.1828 | 0.2953 | 0.3651 | 0.4537 | 0.5868 | 0.7390 | 0.8420 |
| −20 | 0.1564 | 0.3269 | 0.5308 | 0.6585 | 0.8218 | 1.0696 | 1.3572 | 1.5544 |
| −18 | 0.1969 | 0.4127 | 0.6722 | 0.8355 | 1.0453 | 1.3660 | 1.7417 | 2.0014 |
| −16 | 0.2478 | 0.5209 | 0.8519 | 1.0615 | 1.3326 | 1.7506 | 2.2463 | 2.5931 |
| −14 | 0.3114 | 0.6573 | 1.0802 | 1.3505 | 1.7028 | 2.2526 | 2.9156 | 3.3872 |
| −12 | 0.3903 | 0.8281 | 1.3698 | 1.7198 | 2.1808 | 2.9119 | 3.8143 | 4.4714 |
| −10 | 0.4874 | 1.0404 | 1.7348 | 2.1898 | 2.7975 | 3.7820 | 5.0373 | 5.9833 |
| −8 | 0.6045 | 1.2999 | 2.1887 | 2.7814 | 3.5868 | 4.9288 | 6.7171 | 8.1319 |
| −6 | 0.7415 | 1.6078 | 2.7374 | 3.5059 | 4.5714 | 6.4060 | 8.9732 | 11.0973 |
| −4 | 0.8935 | 1.9530 | 3.3611 | 4.3363 | 5.7101 | 8.1216 | 11.5185 | 14.2546 |
| −2 | 1.0458 | 2.2979 | 3.9771 | 5.1450 | 6.7874 | 9.6276 | 13.4690 | 16.4251 |
| 0 | 1.1723 | 2.5755 | 4.4471 | 5.7363 | 7.5266 | 10.5553 | 14.5401 | 17.5511 |
| 2 | 1.2535 | 2.7446 | 4.7144 | 6.0581 | 7.9073 | 11.0003 | 15.0270 | 18.0526 |
| 4 | 1.2948 | 2.8268 | 4.8377 | 6.2021 | 8.0724 | 11.1869 | 15.2265 | 18.2566 |
| 6 | 1.3130 | 2.8619 | 4.8888 | 6.2610 | 8.1388 | 11.2607 | 15.3047 | 18.3361 |
| 20 | 1.3255 | 2.8855 | 4.9224 | 6.2992 | 8.1814 | 11.3076 | 15.3541 | 18.3864 |

Step 1: Using Tables 6 and 7, linearly interpolate to determine Yπq for the desired values K and q. Remember that Yπ0=0.

This concludes this section.

Annex 3

Experimental results

Propagation tests at 930 MHz were conducted on air-to-ground paths in Japan in November 1982 and April and June 1983. According to the test results, propagation losses within line-of-sight agreed well with free space values. The line-of-sight distance as calculated from the measured data at an altitude of 10 000 m was shorter than the distance implied by the curves in Annex 3 (on hold).

This Recommendation is brought to the attention of Study Group 5.

Radiocommunication Study Group 3 made editorial amendments to this Recommendation in the year 2020 in accordance with Resolution ITU-R 1.