SECTION  5E:	ASPECTS  RELATIVE  TO  THE  TERRESTRIAL  FIXED  SERVICE

RECOMMENDATION  ITU-R  PN.530-5

PROPAGATION  DATA  AND  PREDICTION  METHODS  REQUIRED  FOR
THE  DESIGN  OF  TERRESTRIAL  LINE-OF-SIGHT  SYSTEMS

(Question ITU-R 204/3)

(1978-1982-1986-1990-1992-1994)

Rec. ITU-R PN.530-5

The ITU Radiocommunication Assembly,

considering

a)	that for the proper planning of terrestrial line-of-sight systems it is necessary to have appropriate propagation prediction methods and data;

b)	that methods have been developed that allow the prediction of some of the most important propagation parameters affecting the planning of terrestrial line-of-sight systems;

c)	that as far as possible these methods have been tested against available measured data and have been shown to yield an accuracy that is both compatible with the natural variability of propagation phenomena and adequate for most present applications in system planning,

recommends

1.	that the prediction methods and other techniques set out in Annex 1 be adopted for planning terrestrial line-of-sight systems in the respective ranges of parameters indicated.

ANNEX  1

# 1.	Introduction

Several propagation effects must be considered in the design of line-of-sight radio-relay systems. These include:

–	diffraction fading due to obstruction of the path by terrain obstacles under adverse propagation conditions;

–	attenuation due to atmospheric gases;

–	fading due to atmospheric multipath or beam spreading (commonly referred to as defocusing in the English technical literature) associated with abnormal refractive layers;

–	fading due to multipath arising from surface reflection;

–	attenuation due to precipitation or solid particles in the atmosphere;

–	variation of the angle-of-arrival at the receiver terminal and angle-of-launch at the transmitter terminal due to refraction;

–	reduction in cross-polarization discrimination in multipath or precipitation conditions;

–	signal distortion due to frequency selective fading and delay during multipath propagation.

One purpose of this Annex is to present in concise step-by-step form simple prediction methods for the propagation effects that must be taken into account in the majority of fixed line-of-sight links, together with information on their ranges of validity. Another purpose of this Annex is to present other information and techniques that can be recommended in the planning of terrestrial line-of-sight systems.

Prediction methods based on specific climate and topographical conditions within an administration’s territory may be found to have advantages over those contained in this Annex.

With the exception of the interference resulting from reduction in cross-polarization discrimination, the Annex deals only with effects on the wanted signal. Other interference aspects are treated in separate Recommendations, namely:

–	interference involving other terrestrial links and earth stations in Recommendation ITU-R PN.452,

–	interference involving space stations in Recommendation ITU-R PN.619.

To optimize the usability of this Annex in system planning and design, the information is arranged according to the propagation effects that must be considered, rather than to the physical mechanisms causing the different effects.

It should be noted that the term “worst month” used in this Recommendation is equivalent to the term “any month” (see Recommendation ITU-R PN.581).

# 2.	Propagation loss

The propagation loss on a terrestrial line-of-sight path relative to the free-space loss (see Recommendation ITU-R PN.525) is the sum of different contributions as follows:

–	attenuation due to atmospheric gases,

–	diffraction fading due to obstruction or partial obstruction of the path,

–	fading due to multipath, beam spreading and scintillation,

–	attenuation due to variation of the angle-of-arrival/launch,

–	attenuation due to precipitation,

–	attenuation due to sand and dust storms.

Each of these contributions has its own characteristics as a function of frequency, path length and geographic location. These are described in the subsections that follow.

Sometimes propagation enhancement is of interest. In such cases it is considered following the associated propagation loss.

## 2.1	Attenuation due to atmospheric gases

Some attenuation due to absorption by oxygen and water vapour is always present, and should be included in the calculation of total propagation loss at frequencies above about 10 GHz. The attenuation on a path of length d km is given by:

(1)

The specific attenuation ga (dB/km) should be obtained using Recommendation ITU-R PN.676.

Note 1 – On long paths at frequencies above about 20 GHz, it may be desirable to take into account known statistics of water vapour density and temperature in the vicinity of the path. Some information on water vapour density is given in Recommendation ITU-R PN.836.

## 2.2	Diffraction fading

Variations in atmospheric refractive conditions cause changes in the effective Earth’s radius or k-factor from its median value of approximately 4/3 for a standard atmosphere (see Recommendation ITU-R PN.310). When the atmosphere is sufficiently sub-refractive (large positive values of the gradient of refractive index, low k-factor values),

the ray paths will be bent in such a way that the Earth appears to obstruct the direct path between transmitter and receiver, giving rise to the kind of fading called diffraction fading. This fading is the factor that determines the antenna heights.

k-factor statistics for a single point can be determined from measurements or predictions of the refractive index gradient in the first 100 m of the atmosphere (see Recommendation ITU-R PN.453 on effects of refraction). These gradients need to be averaged in order to obtain the effective value of k for the path length in question (ke). Values of ke exceeded for 99.9% of the time are discussed in terms of path clearance criteria in the following section.

### 2.2.1	Diffraction loss dependence on path clearance

Diffraction loss will depend on the type of terrain and the vegetation. For a given path ray clearance, the diffraction loss will vary from a minimum value for a single knife-edge obstruction to a maximum for smooth spherical Earth. Methods for calculating diffraction loss for these two cases and also for paths with irregular terrain are discussed in Recommendation ITU-R PN.526. These upper and lower limits for the diffraction loss are shown in Fig. 1.

FIGURE 1  [D01]  =  3  cm

The diffraction loss over average terrain can be approximated for losses greater than about 15 dB by the formula:

(2)

where h is the height difference (m) between most significant path blockage and the path trajectory (h is negative if the top of the obstruction of interest is above the virtual line-of-sight) and F1 is the radius of the first Fresnel ellipsoid given by:

(3)

with:

f :	frequency (GHz)

d 	path length (km) and

d1 and d2 :	distances (km) from the terminals to the path obstruction.

A curve, referred to as Ad, based on equation (2) is also shown in Fig. 1. This curve, strictly valid for losses larger than 15 dB, has been extrapolated up to 6 dB loss to fulfil the need of link designers.

### 2.2.2	Planning criteria for path clearance

At frequencies above about 2 GHz, diffraction fading of this type has in the past been alleviated by installing antennas that are sufficiently high, so that the most severe ray bending would not place the receiver in the diffraction region when the effective Earth radius is reduced below its normal value. Diffraction theory indicates that the direct path between the transmitter and the receiver needs a clearance above ground of at least 60% of the radius of the first Fresnel zone to achieve free-space propagation conditions. Recently, with more information on this mechanism and the statistics of ke that are required to make statistical predictions, some administrations are installing antennas at heights that will produce some small known outage.

In the absence of a general procedure that would allow a predictable amount of diffraction loss for various small percentages of time and therefore a statistical path clearance criterion, the following procedure is advised for temperate and tropical climates:

#### 2.2.2.1	Non-diversity antenna configurations

a)	determine the antenna heights required for the appropriate median value of the point k-factor (see § 2.2 above; in the absence of any data, use k = 4/3) and 1.0 F1 clearance over the highest obstacle (temperate and tropical climates);

b)	obtain the value of ke (99.9%) from Fig. 2 for the path length in question;

c)	calculate the antenna heights required for the value of ke obtained from step b) and the following Fresnel zone clearance radii:

| Temperate climate | Tropical climate |
| --- | --- |
| 0.0 F1 (i.e. grazing) if there is a single isolated path obstruction | 0.6 F1 for path lengths greater than about 30 km |
| 0.3 F1 if the path obstruction is extended along a portion of the path |  |

d)	use the larger of the antenna heights obtained by steps a) and c).

In cases of uncertainty as to the type of climate, the more conservative clearance rule for tropical climates may be followed or at least a rule based on an average of the clearances for temperate and tropical climates. Smaller fractions of F1 may be necessary in steps a) and c) above for frequencies less than about 2 GHz in order to avoid unacceptably large antenna heights.

Higher fractions of F1 may be necessary in step c) for frequencies greater than about 10 GHz in order to reduce the risk of diffraction in sub-refractive conditions.

FIGURE 2  [D02]  =  3 cm

#### 2.2.2.2	Two antenna space-diversity configurations

a)	Calculate the height of the lower antenna for the appropriate median value of the point k-factor (in the absence of any data use k = 4/3) and the following Fresnel zone clearances:

0.6 F1 to 0.3 F1 if the path obstruction is extended along a portion of the path,

0.3 F1 to 0.0 F1 if there are one or two isolated obstacles on the path profile.

One of the lower values in the two ranges noted above may be chosen if necessary to avoid increasing heights of existing towers or if the frequency is less than 2 GHz.

Alternatively, the clearance of the lower antenna may be chosen to give about 6 dB of diffraction loss during normal refractivity conditions (i.e. during the middle of the day), or some other loss appropriate to the fade margin of the system, as determined by test measurements. Measurements should be carried out on several different days to avoid anomalous refractivity conditions.

In this alternative case the diffraction loss can also be estimated using Fig. 1 or equation (2).

b)	Calculate the height of the upper antenna using the procedure for single antenna configurations noted above.

c)	Verify that the spacing of the two antennas satisfies the requirements for diversity under multipath fading conditions. If not, increase the height of the upper antenna accordingly.

This fading, which results when the path is obstructed or partially obstructed by the terrain during sub-refractive conditions, is the factor that governs antenna heights.

### 2.3	Fading and enhancement due to multipath and related mechanisms

Three clear-air fading mechanisms caused by extremely refractive layers in the atmosphere must be taken into account in the planning of links of more than a few kilometres in length; beam spreading (commonly referred to as defocusing in the English technical literature), antenna decoupling (see Note 1), surface multipath, and atmospheric multipath. Most of these mechanisms can occur by themselves or in combination with each other. A particularly severe form of frequency selective fading occurs when beam spreading of the direct signal combines with a surface reflected

signal to produce multipath fading. Scintillation fading due to smaller scale turbulent irregularities in the atmosphere is always present with these mechanisms but at frequencies below about 40 GHz its effect on the overall fading distribution is not significant.

Note 1 – Antenna decoupling governs the minimum beamwidth of the antennas that should be chosen.

Two methods for predicting the single-frequency (or narrow-band) fading distribution at large fade depths in any part of the world are given below. Method 1 in § 2.3.1 does not make use of the path profile and can be used for initial planning, licensing, or even design purposes. Method 2 in § 2.3.2, which does employ the path profile, should be used for critical link designs, particularly if geoclimatic factors K are available from measured data. Appropriate equations are given in these methods for overland and overwater paths. A third method in § 2.3.3 that is suitable for all fade depths employs Methods 1 or 2 for large fade depths as appropriate and an interpolation procedure for small fade depths. All methods take into account an average mixture of the clear-air fading mechanisms noted above.

A method for predicting enhancement is given in § 2.3.4. The method uses the fade depth predicted by the method in § 2.3.1 or § 2.3.2 as the only input parameter.

### 2.3.1	Method for initial planning purposes at small percentages of time (Method 1)

a)	For the path location in question, estimate the geoclimatic factor K for the average worst month from fading data for the area if these are available (see Annex 2). If such data are not available, K can be estimated from the contour maps of Figs. 7-10 of Recommendation ITU-R PN.453 for the percentage of time pL that the average refractivity gradient in the lowest 100 m of the atmosphere is less than –100 N units/km, and the following empirical relations:

| K = | overland links for which the lower of the transmitting and receiving antennas is less than 700 m above mean sea level (see Note 2) | (4) |
| --- | --- | --- |
| K = | overland links for which the lower of the transmitting and receiving antennas is higher than 700 m above mean sea level (see Notes 1 and 2) | (5) |
| K = | links over medium-sized bodies of water (see Note 3), coastal areas beside such bodies of water (see Note 4), or regions of many lakes (see Note 5) | (6) |
| K = | links over large bodies of water (see Note 3), or coastal areas beside such bodies of water (see Note 4) | (7) |

where the coefficient CLat of latitude x is given by:

CLat  =  0	for		53° S  ³	x  £  53° N	(8)

CLat  =  –5.3  +  x /10	for	53° N	or °S  <	x  <  60° N or °S	(9)

CLat  =  0.7	for			x  ³  60° N or °S	(10)

and the longitude coefficient CLon, by:

CLon  =  0.3	for longitudes of Europe and Africa	(11)

CLon  =  –0.3	for longitudes of North and South America	(12)

CLon  =  0	for all other longitudes	(13)

The month that has the highest value of pL should be chosen from the four seasonally representative months of February, May, August and November for which maps are given in Figs. 7-10 of Recommendation ITU-R PN.453. An exception to this is that only the maps for May and August should be used for latitudes greater than 60° N or 60° S.

Note 1 – In mountainous areas for which the data used to prepare the maps in Figs. 7-10 of Recommendation ITU-R PN.453 were non-existent or very sparse, these maps have insufficient detail and the value of K estimated from

equation (4) tends to be an upper bound. Such areas include the mountainous regions of Western Canada, the European Alps and Japan. The adjustment contained in equation (5) can be used until a more detailed correction is available.

Note 2 – Links passing over a small lake or river should normally be classed as overland links. In cases of uncertainty, the first coefficient in the exponent of equation (4) should be replaced by 6.2 (see also Note 4); for Method 2 in § 2.3.2, replace exponent coefficient 5.4 by 5.1 in equation (16).

Note 3 – Medium-sized bodies of water include the Bay of Fundy (east coast of Canada) and the Strait of Georgia (west coast of Canada), the Gulf of Finland, and other bodies of water of similar size. Large bodies of water include the English Channel, the North Sea, the larger reaches of the Baltic and Mediterranean Seas, Hudson Strait, and other bodies of water of similar size or larger. In cases of uncertainty as to whether the size of the body of water in question should be classed as medium or large, the first coefficient in the exponent of equations (6) or (7) should be replaced by 5.7; for Method 2 in § 2.3.2, replace exponent coefficient in equations (18) or (19) by 4.6.

Note 4 – The link may be considered to be crossing a coastal area if a section of the path profile is less than 100 m above mean sea level and within 50 km of the coastline of a medium or large body of water, and if there is no height of land above 100 m altitude between the link and the coast. If the entire path profile is less than 100 m above mean sea level, then K should be obtained from equations (6) or (7) as appropriate. If only a fraction, rc, of the path profile is below 100 m altitude and within 50 km of the coastline, then the coefficient 5.9 in the exponent of equation (6) can be replaced by 6.5 – 0.6rc, and the exponent 5.5 in equation (7) by 6.5 – rc; for Method 2 in § 2.3.2, replace exponent coefficient 4.8 in equation (18) by 5.4 – 0.6rc, and 4.4 in equation (19) by 5.4 – rc.

Note 5 – Regions (not otherwise in coastal areas) in which there are many lakes over a fairly large area are known to behave like coastal areas. The region of lakes in southern Finland provides the best known example. Until such regions can be better defined, K can be obtained from equation (6). In cases of uncertainty, the coefficient 5.9 in the exponent of equation (6) can be replaced by 6.2; for Method 2 in § 2.3.2, replace 4.8 in equation (18) by 5.1.

b)	If the antenna heights he and hr (m above sea level or some other reference height) are known, calculate the magnitude of the path inclination ½ ep ½ in milliradians from:

(14)

where the path length d is in km.

c)	Calculate the percentage of time pw that fade depth A (dB) is exceeded in the average worst month from:

(15)

where the frequency f is in GHz.

For prediction of exceedance percentages for the average year instead of the average worst month, see § 2.3.5.

### 2.3.2	Method for detailed link design at small percentages of time (Method 2)

a)	Obtain the geoclimatic factor K as indicated for Method 1, but replacing equations (4) to (7) by:

| K = | overland links for which the lower of the transmitting and receiving antennas is less than 700 m above mean sea level (see Note 2 in § 2.3.1) | (16) |
| --- | --- | --- |
| K = | overland links for which the lower of the transmitting and receiving antennas is higher than 700 m above mean sea level (see Note 1 below; see Notes 1 and 2 in § 2.3.1 for links crossing small lakes and rivers) | (17) |
| K = | links over medium-sized bodies of water (see Note 3 in § 2.3.1), coastal areas beside such bodies of water (see Note 4 in § 2.3.1), or regions of many lakes (see Note 5 in § 2.3.1) | (18) |

| K = | links over large bodies of water (see Note 3 in § 2.3.1), or coastal areas beside such bodies of water (see Note 4 in § 2.3.1) | (19) |
| --- | --- | --- |

b)	Obtain the magnitude of the path inclination ½ ep ½ as in Method 1.

c)	From the profile of the terrain along the path, obtain the terrain heights h at intervals of 1.0 km, beginning 1.0 km from one terminal and ending 1-2 km from the other. Using these heights, carry out a linear regression with the “method of least squares” to obtain the linear equation of the “average” profile

(20)

where x is the distance along the path. The regression coefficients can be calculated from the relations:

(21)

(22)

where n is the number of profile height samples. From equation (20) calculate h(0) and h(d), the heights of the average profile at the ends of the path, and the heights of the antennas above the average path profile:

(23)

where he and hr are as in equation (14).

d)	Calculate the “average” grazing angle j corresponding to a 4/3 earth radius model (i.e. ae  =  8 500 km) (see Recommendation ITU-R PN.834):

(24)

where:

(25)

(26)

(27)

In calculation of the coefficients m and c, the variables ae, d, h1 and h2 must be in the same units. The grazing angle j will be in the desired units of milliradians if h1 and h2 are in metres and d in kilometres.

e)	Calculate the percentage of time pw that fade depth A (dB) is exceeded in the average worst month from:

(28)

Equations (15) and (28) apply only to narrow-band systems. They are considered valid for fade depths greater than about 15 dB or the value exceeded for 0.1% of the worst month, whichever is greater. However, for fade depths approaching these limits, the method of § 2.3.3 should be applied to avoid potential errors.

For prediction of exceedance percentages for the average year instead of the average worst month, see § 2.3.5.

Note 1 – Equations (15) and (28) were derived from fading data paths with lengths in the range 7-9 km, frequencies in the range 2-37 GHz, path inclinations for the range 0-24 mrad, and grazing angles in the range 1-12 mrad. Checks using several other sets of data for paths up to 189 km in length and frequencies as low as 500 MHz suggest, however, that they are valid for larger ranges of path length and frequency. The results of a semi-empirical analysis indicate that the lower frequency limit of validity is inversely proportional to path length. A rough estimate of this lower frequency limit fmin can be obtained from:

(29)

### 2.3.3	Method for various percentages of time

The method given below for predicting fade depths at various percentages of time combines an empirical interpolation procedure between the deep fading region of the distribution and 0 dB, with Method 1 or 2 given in the preceding section.

a)	Using Method 1 or 2 as desired, calculate the percentage of time pw that a fade depth of 35 dB is exceeded in the tail of the distribution (i.e., equations (15) or (28)).

b)	Calculate the value of  for the fade depth A = 35 dB with the corresponding value of pw from step a):

(30)

c)	Calculate the value of the parameter qt from:

(31)

d)	If qt  >  0, repeat steps a) to c) for A  =  25 dB.

e)	For A  >  25 dB or A  >  35 dB, as appropriate, calculate the percentage of time pw that the fade depth A is exceeded using Method 1 or Method 2 as desired. For A  <  25 dB, or A  <  35 dB, as appropriate, calculate the percentage of time pw that A is exceeded from:

(32)

where qa is also a function of A given by:

(33)

Using qt as a parameter, Fig. 3 represents the method by a set of curves.

For prediction of exceedance percentages for the average year instead of the average worst month, see § 2.3.5.

### 2.3.4	Prediction method for enhancement

Large enhancements are observed during the same general conditions of frequent ducts that result in multipath fading. Average worst month enhancement above 10 dB should be predicted using:

(34)

where E is the enhancement (dB) not exceeded for p% of the time and A0.01 is the predicted deep fade depth using equations (15) or (28) exceeded for pw  =  0.01% of the time.

For the enhancement between 10 and 0 dB use the following step-by-step procedure:

a)	Calculate the percentage of time  with enhancement less or equal to 10 dB (E¢  =  10) using equation (34).

b)	Calculate  using:

(35)

c)	Calculate the parameter qs from:

(36)

d)	Calculate qe for the desired E using:

(37)

e)	The percentage of time that the enhancement E (dB) is not exceeded is found from:

(38)

FIGURE 3 [D03]  =  3cm

The set of curves in Fig. 4 presents the method.

FIGURE 4  [D04]  =  3 cm

For prediction of exceedance percentages for the average year instead of the average worst month, see § 2.3.5.

### 2.3.5	Conversion from average worst month to average annual distributions

The fading and enhancement distributions for the average worst month obtained from the methods of § 2.3.1-2.3.4 can be converted to distributions for the average year by employing the following procedure:

a)	Calculate the percentage of time pw fade depth A is exceeded in the large tail of the distribution for the average worst month from equations (15) or (28).

b)	Calculate the logarithmic geoclimatic conversion factor DG from

mmmmmmdB	(39)

where DG  £  10.8 dB and the positive sign in equation (39) is employed for x  £  45° and the negative sign for x  >  45°, and where:

x:	latitude (°N or °S)

d:	path length (km)

| ep |:	magnitude of path inclination (obtained from equation (14)).

c)	Calculate the percentage of time p fade depth A is exceeded in the large fade depth tail of the distribution for the average year from

mmmmmm%	(40)

d)	If the shallow fading range of the distribution is required (i.e. A  <  25 dB or A  <  35 dB, as appropriate), follow the method of § 2.3.3, replacing pw by p.

e)	If it is required to predict the distribution of enhancement for the average year, follow the method of § 2.3.4, where A0.01 is now the fade depth exceeded for 0.01% of the time in the average year. Obtain first pw by inverting equation (40) and using p = 0.01%. Then obtain fade depth A0.01 exceeded for 0.01% of the time in the average year by inverting equation (15) or (28) and using p in place of pw.

## 2.4	Attenuation due to hydrometeors

Attenuation can also occur as a result of absorption and scattering by such hydrometeors as rain, snow, hail and fog. Although rain attenuation can be ignored at frequencies below about 5 GHz, it must be included in design calculations at higher frequencies, where its importance increases rapidly. A technique for estimating long-term statistics of rain attenuation is given in § 2.4.1. On paths at high latitudes or high altitude paths at lower latitudes, wet snow can cause significant attenuation over an even larger range of frequencies. More detailed information on attenuation due to hydrometeors other than rain is given in Recommendation ITU-R PN.840.

At frequencies where both rain attenuation and multipath fading must be taken into account, the exceedance percentages for a given fade depth corresponding to each of these mechanisms can be added.

### 2.4.1	Long-term statistics of rain attenuation

The following simple technique may be used for estimating the long-term statistics of rain attenuation:

Step 1:	Obtain the rain rate R0.01 exceeded for 0.01% of the time (with an integration time of 1 min). If this information is not available from local sources of long-term measurements, an estimate can be obtained from the information given in Recommendation ITU-R PN.837.

Step 2:	Compute the specific attenuation, gR (dB/km) for the frequency, polarization and rain rate of interest using Recommendation ITU-R PN.838.

Step 3:	Compute the effective path length deff of the link by multiplying the actual path length d by a distance factor r. An estimate of this factor is given by:

(41)

where, for R0.01  £  100 mm/h:

(42)

For R0.01  >  100 mm/h, use the value 100 mm/h in place of R0.01.

Step 4:	An estimate of the path attenuation exceeded for 0.01% of the time is given by:

(43)

Step 5:	Attenuation exceeded for other percentages of time p in the range 0.001% to 1% may be deduced from the following power law:

(44)

This formula has been determined to give factors of 0.12, 0.39, 1 and 2.14 for 1%, 0.1%, 0.01% and 0.001% respectively, and must be used only within this range.

Step 6:	If worst-month statistics are desired, calculate the annual time percentages p corresponding to the worst-month time percentages pw using climate information specified in Recommendation ITU-R PN.841. The values of A exceeded for percentages of the time p on an annual basis will be exceeded for the corresponding percentages of time pw on a worst-month basis.

The prediction procedure outlined above is considered to be valid in all parts of the world at least for frequencies up to 40 GHz and path lengths up to 60 km.

### 2.4.2	Frequency scaling of long-term statistics of rain attenuation

When reliable long-term attenuation statistics are available at one frequency the following empirical expression may be used to obtain a rough estimate of the attenuation statistics for other frequencies in the range 7 to 50 GHz, for the same hop length and in the same climatic region:

(45)

where:

(46)

(47)

Here, A1 and A2 are the equiprobable values of the excess rain attenuation at frequencies f1 and f2 (GHz), respectively.

### 2.4.3	Polarization scaling of long-term statistics of rain attenuation

Where long-term attenuation statistics exist at one polarization (either vertical or horizontal) on a given link, the attenuation for the other polarization over the same link may be estimated through the following simple formulae:

(48)

or:

(49)

These expressions are considered to be valid in the range of path length and frequency for the prediction method of § 2.4.1.

### 2.4.4	Statistics of duration and fading

There is some evidence that the rate of fading due to rain is much less than that due to multipath. On the other hand, the average and median values of duration differ, indicating skewness of the distribution of fading duration.

### 2.4.5	Tandem and convergent paths, and path diversity

#### 2.4.5.1	Length of individual hops

The overall transmission performance of a tandem system is largely influenced by the propagation characteristics of the individual links. It is sometimes possible to achieve the same overall physical connection by different combinations of hop lengths. Increasing the length of individual hops inevitably results in an increase in the probability of outage for those hops. On the other hand, such a move could mean that fewer hops might be required and the overall performance of the tandem system might not be impaired.

#### 2.4.5.2	Correlated fading on tandem paths

If the occurrence of rainfall were statistically independent of location, then the overall probability of fading for a linear series of links in tandem would be given to a good approximation by:

(50)

where Pi is the ith of the total n links.

On the other hand, if precipitation events are correlated over a finite area, then the attenuation on two or more links of a multi-hop relay system will also be correlated, in which case the combined fading probability may be written as:

(51)

where K is a modification factor that includes the overall effect of rainfall correlation.

Few studies have been conducted with regard to this question. One such study examined the instantaneous correlation of rainfall at locations along an East-West route, roughly parallel to the prevailing direction of storm movement. Another monitored attenuation on a series of short hops oriented North-South, or roughly perpendicular to the prevailing storm track during the season of maximum rainfall.

For the case of links parallel to the direction of storm motion, the effects of correlation for a series of links each more than 40 km in length, l, were slight. The modification factor, K, in this case exceeded 0.9 for rain induced outage of 0.03% and may reasonably be ignored (see Fig. 5). For shorter hops, however, the effects become more significant: the overall outage probability for 10 links of 20, 10 and 5 km each is approximately 80%, 65% and 40% of the uncorrelated expectation, respectively (modification factors 0.8, 0.65, 0.4). The influence of rainfall correlation is seen to be somewhat greater for the first few hops and then decreases as the overall length of the chain increases.

The modification factors for the case of propagation in a direction perpendicular to the prevailing direction of storm motion are shown in Fig. 6 for several probability levels. In this situation, the modification factors fall more rapidly for the first few hops (indicating a stronger short-range correlation than for propagation parallel to storm motion) and maintain relatively steady values thereafter (indicating a weaker long-range correlation).

#### 2.4.5.3	Convergent paths

Where two or more radio paths converge to one radio-relay station, the correlation coefficient of attenuation between pairs of paths is dependent on the angle between the paths. This dependence, together with the differential attenuation on the converging paths and the interference between paths, was studied for the case when the path length is smaller or comparable in size with the rain cell. As an example it was found theoretically that for a path length of 4 km, the correlation coefficient increased from 0.8 to 0.97 when the angle between the paths decreased from 180° to 20°.

#### 2.4.5.4	Path diversity

Whereas fading due to multipath propagation can be overcome by a vertical separation of several metres between the antennas, a choice of paths with a separation of several kilometres may reduce fading due to precipitation.

Experimental data obtained in the United Kingdom in the 20-40 GHz range give an indication of the improvement in link reliability which can be obtained by the use of switched-path route diversity. The diversity gain (i.e. the difference between the attenuation (in dB) exceeded for a specific percentage of time on a single link and that simultaneously on two parallel links):

–	tends to decrease as the path length increases from 12 km for a given percentage of time, and for a given lateral path separation,

–	is generally greater for a spacing of 8 km than for 4 km, though an increase to 12 km does not provide further improvement,

–	is not significantly dependent on frequency in the range 20-40 GHz, for a given geometry, and

–	ranges from about 2.8 dB at 0.1% of the time to 4.0 dB at 0.001% of the time, for a spacing of 8 km, and path lengths of about the same value. Values for a 4 km spacing are about 1.8 to 2.0 dB.

FIGURE 5  [D05]  =  3 cm

# 3.	Variation in angle-of-arrival/launch

Abnormal gradients of the clear-air refractive index along a path can cause considerable variation in the angles of launch and arrival of the transmitted and received waves. This variation is substantially frequency independent and primarily in the vertical plane of the antennas. The range of angles is greater in humid coastal regions than in dry inland areas. No significant variations have been observed during precipitation conditions.

The effect can be important on long paths in which high gain/narrow beam antennas are employed. If the antenna beamwidths are too narrow, the direct outgoing/incoming wave can be sufficiently far off axis that a significant fade can occur (see § 2.3). Furthermore, if antennas are aligned during periods of very abnormal angles-of-arrival, the alignment may not be optimum. Thus, in aligning antennas on critical paths (e.g. long paths in coastal area), it may be desirable to check the alignment several times over a period of a few days.

FIGURE 6  [D06]  =  3 cm

# 4.	Reduction of cross-polarization discrimination

The cross-polarization discrimination (XPD) can deteriorate sufficiently to cause co-channel interference and, to a lesser extent, adjacent channel interference. The reduction in XPD that occurs during both clear-air and precipitation conditions must be taken into account.

## 4.1	XPD statistics during clear-air conditions

The combined effect of multipath propagation and the cross-polarization patterns of the antennas governs the reductions in XPD occurring for small percentages of time. For paths on which more detailed predictions or measurements are not available, a rough estimate of the unconditional distribution of XPD can be obtained from the cumulative distribution of the fade depth (CPA) (see § 2.3) using the equi-probability relation:

(52)

(53)

In this relation, XPD0 is the static XPD during unfaded conditions and Q is an improvement factor that shows strong dependence on the slope of the cross-polarized antenna patterns in the vertical plane.

Experimental data relevant to equation (52), valid in the deep fade region, are shown in Table 1. In addition to the equi-probability relation, data are also given for the related empirical equation for the conditional median value of XPD:

(54)

and the related empirical equation for the conditional r.m.s. value of XPD:

(55)

## 4.2	XPD statistics during precipitation conditions

Intense rain governs the reductions in XPD observed for small percentages of time. For paths on which more detailed predictions or measurements are not available, a rough estimate of the unconditional distribution of XPD can be obtained from a cumulative distribution of the co-polarized rain attenuation CPA (see § 2.4) using the equi-probability relation:

(56)

that applies for both linear and circular polarizations. The coefficients U and V( f ) are in general dependent on a number of variables and empirical parameters, including frequency,  f . For line-of-sight paths with small elevation angles and horizontal or vertical polarization, these coefficients may be approximated by:

(57)

(58)

An average value of U0 of about 15 dB, with a lower bound of 9 dB for all measurements, has been obtained for attenuations greater than 15 dB.

The variability in the values of U and V( f ) is such that the difference between the CPA values for vertical and horizontal polarizations is not significant when evaluating XPD. The user is advised to use the value of CPA for circular polarization when working with equation (56).

Long-term XPD statistics obtained at one frequency can be scaled to another frequency using the semi-empirical formula:

(59)

where XPD1 and XPD2 are the XPD values not exceeded for the same percentage of time at frequencies f1 and f2.

The relationship between XPD and CPA is influenced by many factors, including the residual antenna XPD, that has not been taken into account. Equation (59) is least accurate for large differences between the respective frequencies. It is most accurate if XPD1 and XPD2 correspond to the same polarization (horizontal or vertical).

TABLE  1

Values of empirical parameters C1, C2 and C3 and associated experimental information

| Frequency (GHz) | Path length (km) | Transmitted polarization(s) | Period | C1, C2, C3 | XPD0 (dB) | S (1) (dB/degrees) | Relative amplitude of ground-reflected wave | Angle differences (2) (degrees) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.15 | 37.2 | Horizontal | Winter 1971 (130 h) | 35 | > 30 | – | – | – | C3 estimated from XPD/CPA equi-probability curve |
| 6.72 | 37.2 | Circular | Autumn 1971 (130 h) | 30 | > 30 | – | – | – | C3 estimated from XPD/CPA equi-probability curve |
| 6.0 | 42.5 | Vertical | August 1974 to February 1975 | 47.6 | 25.9 | – | – | – | C2 calculated from XPDr.m.s. data |
| 11.6 | 42.5 | Vertical | August 1974 to February 1975 | 36.3 | 19.5 | – | – | – | C2 calculated from XPDr.m.s. data |
| 11.5 | 25.6 | Vertical | August 1974 to February 1975 | 45.3 | 42.7 | – | – | – |  |
| 7 | 50 | Vertical and horizontal | 5 August to 5 September 1973 | 52.5 | 40 | 50 | – | – | Receiving antenna with relatively sharp cross-polar pattern. C2 estimated from XPDmedian data |
| 7 | 50 | Vertical and horizontal | 5 August to 5 September 1973 | 57 | 41 | 6 | – | – | Receiving antenna with relatively flat cross-polar pattern. C2 estimated from XPDmedian data |
| 4.15 | 44.1 | Horizontal | April 1977 to December 1978 | 45 | 45 | 110 | – | – | Receiving antenna with sharp cross-polar amplitude pattern. C3 estimated from XPD/CPA equi-probability curve |
| 4.15 | 44.1 | Horizontal | April 1977 to December 1978 | 54 | 42 | 17.5 | – | – | Receiving antenna with relatively flat cross-polar amplitude patterns. C3 estimated from XPD/CPA equi-probability curve |
| 4.15 | 44.1 | Horizontal | April 1977 to March 1978 | 54 | 37 | 7.5 | – | – | Receiving antenna with very flat cross-polar amplitude and phase patterns. C3 estimated from XPD/CPA equi-probability curve |

TABLE  1  (continued)

| Frequency (GHz) | Path length (km) | Transmitted polarization(s) | Period | C1, C2, C3 | XPD0 (dB) | S (1) (dB/degrees) | Relative amplitude of ground-reflected wave | Angle differences (2) (degrees) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.80 | 16 | Vertical | May to September 1979 | 39 | 35 | – | – | – | C2 calculated from XPDr.m.s. data |
| 11.08 | 16 | Horizontal | May to September 1979 | 39 | 33 | – | – | – | C2 calculated from XPDr.m.s. data |
| 16.53 | 16 | Vertical | May to September 1979 | 38 | 33 | – | – | – | C2 calculated from XPDr.m.s. data |
| 16.81 | 16 | Horizontal | May to September 1979 | 37 | 40 | – | – | – | C2 calculated from XPDr.m.s. data |
| 7 | 59.8 | Horizontal | April to May 1987 | 55 | 37 | < 3 | – | – | C2 calculated from XPDr.m.s. data |
|  |  |  |  | 52 |  |  |  |  | C3 estimated from XPD/CPA equi-probability curve |
| 7 | 59.8 | Vertical | August to September 1987 | 55 | 42 | – | – | – | C2 calculated from XPDr.m.s. data |
|  |  |  |  | 53 |  |  |  |  | C3 estimated from XPD/CPA equi-probability curve |
| 4.61 | 116.6 | Horizontal | April 1985 to November 1986 | 50.1 | 42 |  | 0.03 | 0.37; 0.61 | C3 calculated |
| 7.6 | 29 | Vertical | September 1984 | 16.6 | 27 |  |  |  | C3 estimated from XPD/CPA equi-probability curve with a slope of –0.69 instead of –1 |
| 7.6 | 29 | Vertical | October 1984 | 19.6 | 22 |  |  |  | C3 estimated from XPD/CPA equi-probability curve with a slope of –0.67 instead of –1 |

TABLE  1  (end)

| Frequency (GHz) | Path length (km) | Transmitted polarization(s) | Period | C1, C2, C3 | XPD0 (dB) | S (1) (dB/degrees) | Relative amplitude of ground-reflected wave | Angle differences (2) (degrees) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 55.7 | Vertical | July to August 1979 | – | 27 | – | – | – | Value of C3 = 27 obtained with slope of –0.6 instead of –1. Path clearance of 13 m for k = 1 |
| 7.1 | 47.15 | Horizontal | July to September 1982 | 45 | 40 | – | – | – | C3 estimated from XPD/CPA equi-probability curve |
| 4.61 | 93.8 | Horizontal | August 1982 to July 1984 | 42 40 | 41 38 | – – | 0.50 | 0.76; 0.78 | C3 for two diversity antennas estimated from XPD/CPA equi-probability curve |
| 4.70 | 64.3 | Vertical | November 1980 to August 1981 | 50 | 40 | – | 0.60 | 0.21; 0.12 | C3 estimated from XPD/CPA equi-probability curve |
| 4.40 | 52.6 | Horizontal | May to August 1980 | 43 | 43 | – | 0.50 | 0.85; 0.19 | C3 estimated from XPD/CPA equi-probability curve |
| 4.67 | 63.4 | Vertical | May 1979 to June 1980 | 45 46 | 45 37 | – – | 0.12 | 0.20; 0.90 | C3 for two diversity antennas estimated from XPD/CPA equi-probability curve |
| 4.61 | 59.5 | Horizontal | June to September 1981 | 54 47 | 40 47 | – – | 0.05 | 1.80; 0.20 | C3 for two diversity antennas estimated from XPD/CPA equi-probability curve |
| 5.87 | 74.5 | Horizontal | July 1961 | 48 | 40 | – | 0.03 | 0.72; 0.15 | C3 estimated from XPD/CPA equi-probability curve |
| 2.12 | 74.5 | Vertical | September to October 1979 | 46 | 38 | – | 0.20 | 0.28; 0.10 | C3 estimated from XPD/CPA equi-probability curve |

(1)	S : slope of antenna cross-polarization pattern in the vertical plane near the bore sight (e.g. ± 0.1°).

(2)	Angle differences (at transmitter, receiver) between direct and ground reflected wave during normal refractive conditions.

# 5.	Distortion due to propagation effects

The primary cause of distortion on line-of-sight links in the UHF and SHF bands is the frequency dependence of amplitude and group delay during clear-air multipath conditions. Such distortion increases with increasing system bandwidth and depends on several link variables in a way that it is not yet well understood. Consequently, it is not yet possible to give a universal technique for predicting its effects.

Distortion resulting from precipitation is believed to be negligible, and in any case a much less significant problem than precipitation attenuation itself. Distortion is known to occur in millimetre and sub-millimetre wave absorption bands, but its effect on operational systems is not yet clear.

# 6.	Techniques for alleviating the effects of multipath propagation

The effects of slow relatively non-frequency selective fading (i.e. “flat fading”) due to beam spreading, and faster frequency-selective fading due to multipath propagation can be reduced by both non-diversity and diversity techniques.

## 6.1	Techniques without diversity

Links should be sited to take advantage of terrain in ways that will increase the path inclination, since increasing path inclination is known to reduce the effects of beam spreading, surface multipath fading, and atmospheric multipath fading. Links should also be sited where possible to reduce the level of surface reflections thus reducing the occurrence of multipath fading and distortion. Techniques include the siting of overwater links to place surface reflections on land rather than water and the siting of overland and overwater links to similarly avoid large flat highly reflecting surfaces on land. Another technique known to reduce the level of surface reflections is to tilt the antennas slightly upwards. Detailed information on appropriate tilt angles is not yet available. A trade-off must be made between the resultant loss in antenna directivity in normal refractive conditions that this technique entails, and the improvement in multipath fading conditions.

Another technique that is less well understood involves the reduction of path clearance. A trade-off must be made between the reduction of the effects of multipath fading and distortion and the increased fading due to subrefraction.

## 6.2	Diversity techniques

Diversity techniques include space, angle and frequency diversity. Frequency diversity should be avoided whenever possible so as to conserve spectrum. Whenever space diversity is used, angle diversity should also be employed by tilting the antennas at different upward angles. Angle diversity can be used in situations in which adequate space diversity is not possible or to reduce tower heights.

The degree of improvement afforded by all of these techniques depends on the extent to which the signals in the diversity branches of the system are uncorrelated. For narrow-band analogue systems, it is sufficient to determine the improvement in the statistics of fade depth at a single frequency. For wideband digital systems, the diversity improvement also depends on the statistics of in-band distortion.

The diversity improvement factor I for fade depth is defined by:

(60)

where pd(A) is the percentage of time in the combined diversity signal branch with fade depth larger than A and p(A) is the percentage for the unprotected path. The diversity improvement factor for digital systems is defined by the ratio of the exceedance times for a given bit-error ratio (BER) with and without diversity. A prediction procedure for the diversity improvement factor can be currently recommended only for narrow-band space-diversity systems.

The vertical space diversity improvement factor for narrow-band signals on an overland path can be estimated from:

(61)

where:

(62)

and:

(63)

with:

A :	fade depth (dB) for the unprotected path

pw :	percentage of time fade depth A is exceeded in the average worst month

P0 :	fading occurrence factor

S :	vertical separation (centre-to-centre) of receiving antennas (m)

f :	frequency (GHz)

d :	path length (km)

G1, G2 :	gains of the two antennas (dBi).

This equation was based on data in the data banks of Radiocommunication Study Group 3 for the following ranges of variables: 43  £  d  £  240 km, 2  £  f  £  11 GHz, and 3  £  S  £  23 m. There is some reason to believe that it may remain reasonably valid for path lengths as small as 25 km. The exceedance percentage pw can be calculated from equations (15) or (28) as appropriate. Equation (61) is valid in the deep-fading range for which equations (15) and (28) are valid.

ANNEX  2

Method for determining the geoclimatic factor K from
measured overland fading data

Step 1:	Obtain the worst calendar month envelope fading distribution for each year of operation, using the long-term median value as a reference. Average these to obtain the cumulative fading distribution for the average worst month and plot this on a semi-logarithmic graph.

Step 2:	From the graph note the fade depth A1, beyond which the cumulative distribution is approximately linear and obtain the corresponding percentage of time p1. This linear portion constitutes the large fade depth tail which can vary by up to about 3 or 4 dB/decade in slope about the average “Rayleigh” value of 10 dB/decade, the amount of this variation depending on the number of years of data contained in the average distribution.

Step 3:	Calculate the path inclination ½ ep ½ from equation (14) and the grazing angle j from equations (20) to (27).

Step 4:	Substitute the coordinates (p1, A1) of the “first tail point” into equation (28) along with the values d, f, ½ ep ½ and j and calculate the geoclimatic factor K.

Step 5:	If data are available for several paths in a region of similar climate and terrain, or several frequencies, etc., on a single path, an average geoclimatic factor should be obtained by averaging the values of log K.

_________________