RECOMMENDATION  ITU-R  PI.533-4

HF  PROPAGATION  PREDICTION  METHOD*

(Question 223/3)

(1978-1982-1990-1992-1994)

Rec. ITU-R PI.533-4

The ITU Radiocommunication Assembly,

considering

a)	that the ex-CCIR developed and evaluated three separate computer-based methods of field-strength estimation as presented in Report ITU-R PI.252, Supplement to Report ITU-R PI.252, Report ITU-R PI.894 and Annex 1 of this Recommendation which has been derived from Report ITU-R PI.894;

b)	that tests against ITU-R Data Bank D1 show that the method of Annex 1 has comparable accuracy to the other more complex methods;

c)	that associated computer codes have been formulated and made available to the Radiocommunication Bureau (see Resolution ITU-R 63),

recommends

1.	that the information contained in Annex 1 should be used in computerized prediction of sky-wave propagation at frequencies between 2-30 MHz;

2.	that administrations and ITU-R should endeavour to improve prediction methods to enhance operational facilities and to improve accuracy.

ANNEX  1

CONTENTS

1.	Introduction

2.	Location of control points

3.	Basic and operational maximum usable frequencies

3.1	Basic maximum usable frequencies

3.2	E-layer critical frequency (foE)

3.3	E-layer basic MUF

3.4	F2-layer characteristics

3.5	F2-layer basic MUF

3.5.1	Lowest-order mode

3.5.1.1	Paths up to dmax (km) (dmax : maximum hop length for F2 mode)

3.5.1.2	Paths longer than dmax (km)

3.5.2	Higher-order modes (paths up to 9 000 km)

3.5.2.1	Paths up to dmax (km)

3.5.2.2	Paths longer than dmax (km)

3.6	The path operational MUF

4.	E-layer maximum screening frequency (fs)

5.	Median sky-wave field strength

5.1	Paths up to 7 000 km

5.1.1	Modes considered

5.1.2	Elevation angle

5.1.3	Field-strength determination

5.2	Paths longer than 9 000 km

5.3	Paths between 7 000 and 9 000 km

6.	Median available receiver power

7.	Monthly median signal-to-noise ratio

8.	Signal-to-noise ratios for other percentages of time

9.	Lowest usable frequency (LUF)

10.	Basic circuit reliability (BCR)

# 1.	Introduction

This propagation prediction method for use in the estimation of reliability and compatibility between frequencies of about 3 MHz and 30 MHz derives from a method first proposed in 1983 by CCIR Interim Working Party 6/12 with later refinements following considerations by the WARC for HF broadcasting, the CCIR, ITU-R, broadcasting and other organizations. The procedure applies a ray-path analysis for path lengths up to 7 000 km, composite mode empirical formulations from the fit to measured data beyond 9 000 km and a smooth transition between these two approaches over the 7 000-9 000 km distance range.

Monthly median basic MUF, incident sky-wave field strength and available receiver power from a lossless receiving antenna of given gain are determined. Signal strengths are standardized against an ITU-R measurement data bank. The method requires the determination of a number of ionospheric characteristics and propagation parameters at specified “control points”.

A computer program in both mainframe and microcomputer versions has been developed for this prediction method. Information on the availability of the program is found in Resolution ITU-R 63.

# 2.	Location of control points

Propagation is assumed to be along the great-circle path between the transmitter and receiver locations via E modes (up to 4 000 km range) and F2 modes (for all distances). Depending on path length and reflecting layer, control points are selected as indicated in Table 1.

# 3.	Basic and operational maximum usable frequencies

The estimation of operational MUF, the highest frequency that would permit acceptable operation of a radio service, is in two stages: first, the estimation of basic MUF from a consideration of ionospheric parameters and second, the determination of a correction factor to allow for propagation mechanisms at frequencies above the basic MUF.

TABLE  1

Locations of control points for the determination of basic MUF, E-layer screening,
ray-path mirror-reflection heights and ionospheric absorption

a) Basic MUF and associated electron gyrofrequency

| Path length D (km) | E modes | F2 modes |
| --- | --- | --- |

| 0 < D £ 2 000 | M | M |
| --- | --- | --- |

| 2 000 < D £ 4 000 | T + 1 000, R -–– 1 000 | - |
| --- | --- | --- |

| 2 000 < D £ dmax | - | M |
| --- | --- | --- |

| D > dmax | - | T + d0 / 2, R - d0 / 2 |
| --- | --- | --- |

| b) E-layer screening |  |
| --- | --- |
| Path length D (km) | F2 modes |
| 0 < D £ 2 000 | M |
| 2 000 < D < 9 000 | T + 1 000, R – 1000 |

| c) Ray-path mirror-reflection heights |  |
| --- | --- |
| Path length D (km) | F2 modes |
| 0 < D £ dmax | M |
| dmax < D < 9 000 | T + d0 / 2, M, R – d0 / 2 |

| d) Ionospheric absorption and associated electron gyrofrequency |  |  |
| --- | --- | --- |
| Path length D (km) | E modes | F2 modes |
| 0 < D £ 2 000 | M | M |
| 2 000 < D £ 4 000 | T + 1 000, M, R – 1 000 | – |
| 2 000 < D £ dmax | – | T + 1 000, M, R – 1 000 |
| dmax < D < 9 000 | – | T + 1 000, T + d0 / 2, M, R – d0 / 2, R – 1 000 |

M : path mid-point. T : transmitter location. R : receiver location. dmax : maximum F2 hop length. d0 hop length of lowest-order mode.

Distances are quoted in kilometres.

## 3.1	Basic maximum usable frequencies

The basic MUFs of the various propagation modes are evaluated in terms of the corresponding ionospheric layer critical frequencies and a factor related to hop length. Where both E and F2 modes are considered the higher of the two basic MUFs of the lowest-order E and F2 modes gives the basic MUF for the path.

## 3.2	E-layer critical frequency (foE)

foE is determined as defined in Recommendation ITU-R PI.434.

## 3.3	E-layer basic MUF

foE is evaluated at the control points noted in Table 1a) and for path lengths of 2 000-4 000 km the lower value is selected. The basic MUF of an n-hop E mode over a path of length D is given by:

(1)

where i110  is the angle of incidence at a mid-hop mirror-reflection height of 110 km for a hop of length d = D/n.

The E-layer basic MUF for the path is the value of E(D)MUF for the lowest-order E-mode.

## 3.4	F2-layer characteristics

Numerical representations of the ionospheric characteristics foF2 and M(3000)F2, for solar-index values R12 = 0 and 100, and for each month are taken from Recommendation ITU-R PI.434 where the magnetic field is evaluated at a height of 300 km. The Oslo coefficients are used to determine these values for the required times and for the control points given in Table 1a). Linear interpolation or extrapolation is applied for the prevailing index values between R12 = 0 and 150 (see Recommendation ITU-R PI.371). For higher sunspot activity, R12 is set equal to 150 in the case of foF2 only.

## 3.5	F2-layer basic MUF

### 3.5.1	Lowest-order mode

#### 3.5.1.1	Paths up to dmax (km)

The order, n0, of the lowest-order mode is determined by geometrical considerations, using the mirror reflection height hr derived at the mid-path control point from the equation:

(2)

For this mode, the F2-layer basic MUF, which is also the F2-layer basic MUF for the path, is calculated as:

(3)

where:

fH : 		value of electron gyrofrequency, for a height of 300 km, determined at each of the appropriate control points given in Table 1a)

(4)

with Z = 1 – 2d/dmax

(5)

(6)

where:

d = D/n0 and dmax are in kilometres

C3000 : value of Cd for D = 3 000 km

x = foF2/foE, 	or 2, whichever is the larger

foE is calculated as in § 3.3.

#### 3.5.1.2	Paths longer than dmax (km)

The basic MUF of the lowest-order mode n0F2(D)MUF for path length D is taken equal to the lower of the F2(dmax)MUF values determined from equation (3) for the two control points given in Table 1a). This is also the basic MUF for the path.

### 3.5.2	Higher-order modes (paths up to 9 000 km)

#### 3.5.2.1	Paths up to dmax (km)

The F2-layer basic MUF for an n-hop mode is calculated using equations (3) to (6) at the midpath control point given in Table 1a) for hop length d = D/n.

#### 3.5.2.2	Paths longer than dmax (km)

The F2-layer basic MUF for an n-hop mode is calculated in terms of F2(dmax)MUF and a distance scaling factor dependent on the respective hop lengths of the mode in question and the lowest possible order mode.

(7)

where Mn/Mn0 is derived using equation (3) as follows:

(8)

The lower of the values calculated at the two control points of Table 1a) is selected.

## 3.6	The path operational MUF

The path operational MUF is the greater of the operational MUF for F2 modes and the operational MUF for E modes. For F2 modes, the operational MUF = basic MUF.Rop where Rop is given in Table 2 to Recommendation ITU-R PI.434. For E modes the operational MUF is equal to the basic MUF.

An estimate of the operational MUF exceeded for 10% and 90% of the days is determined by multiplying the median operational MUF by the factors 1.15 and 0.85 respectively in the case of the F modes and 1.05 and 0.95 respectively in the case of E modes.

# 4.	E-layer maximum screening frequency (fs)

E-layer screening of F2 modes is considered for paths up to 9 000 km. The foE value at the mid-point of the path (for paths up to 2 000 km), or the higher one of the foE values at the two control points 1 000 km from each end of the path (for paths longer than 2 000 km), is taken for the calculation of the maximum screening frequency (see Table 1b)).

(9)

with:

(10)

where:

i:	angle of incidence at height hr = 110 km

R0 :	radius of the Earth, 6 371 km

DF :		elevation angle for the F2-layer mode (determined from equation (11)).

# 5.	Median sky-wave field strength

The predicted field strength is the monthly median over all days of the month.

## 5.1	Paths up to 7 000 km

### 5.1.1	Modes considered

Up to three E modes (for paths up to 4 000 km only) and up to six F2 modes are selected, each of which meets all of the following separate criteria:

E modes	–	being the lowest-order mode with hop length up to 2 000 km, or one of the next two higher-order modes;

–	having an elevation angle ³ 3° as given from equation (11) for mirror-reflection from a height hr = 110 km.

F2 modes	–	being the lowest-order mode with a hop length up to d0 km or one of the next five higher-order modes;

–	having an elevation angle ³ 3° as given from equation (11) for mirror-reflection from a height hr determined from equation (2) where M(3000)F2 is evaluated at the mid-path (paths up to dmax (km)) or at the control point given in Table 1c) for which foF2 has the lower value (paths dmax to 9 000 km);

–	having an E-layer maximum screening frequency evaluated as described in § 4 which is less than the operating frequency.

### 5.1.2	Elevation angle

The elevation angle which applies for all frequencies, including those above the basic MUF, is given by:

(11)

where:

d  :	hop length of an n-hop mode given by d = D/n

hr :	equivalent plane-mirror reflection height

for E modes hr = 110 km

for F2 modes hr is taken as a function of time, location and hop length.

The mirror reflection height for F2 modes, hr, is calculated as follows, where:

with:

and	y = x or 1.8, whichever is the larger.

a)	For x > 3.33 and xr = f/foF2 ³ 1, where f is the wave frequency

hr  =  h or 800 km, whichever is the smaller	(12)

where:	h  =  A1  +  B1 2.4–a for B1 and  a  ³  0
	h  =  A1  +  B1  otherwise

with	A1  =  140  +  ( H  -  47) E1

B1  =  150  +  ( H  -  17) F1  -  A1

E1  =  -  0.09707 xr 3  +  0.6870  xr 2  -  0.7506 xr  +  0.6

F1  is such that:

F1  =  -  1.862 xr 4  +  12.95 xr 3  -  32.03 xr 2  +  33.50 xr  -  10.91	for  xr  £  1.71

F1  =  1.21  +  0.2 xr		for  xr  >  1.71

and	a varies with distance d and skip distance ds as:
	a  =  (d  -  ds) / ( H  +  140)

where:	ds  =  160  +  (H  +  43) G
	G  =  –  2.102 xr 4  +  19.50 xr 3  -  63.15 xr 2  +  90.47 xr  -  44.73	for  xr  £  3.7
	G  =  19.25		for  xr  >  3.7

b)	For x > 3.33 and xr < 1

hr  =  h or 800 km, whichever is the smaller	(13)

where:	h  =  A2  +  B2 b for B2  ³  0

h  =  A2  +  B2 otherwise

with	A2  =  151  +  ( H  -  47) E2

B2  =  141  +  ( H  -  24)  F2  -  A2

E2  =  0.1906 Z 2  +  0.00583 Z  +  0.1936

F2  =  0.645 Z 2  +  0.883 Z  + 0.162

where:	Z  =  xr or 0.1, whichever is the larger and b varies with normalized distance df, Z and H as follows:

b  =  -  7.535 df 4  +  15.75 df 3  -  8.834 df 2  -  0.378 df  +  1

where:

c)	For x £ 3.33

hr  =  115  +  H J  +  U d  or 800 km, whichever is the smaller	(14)

with	J  =  - 0.7126 y3  +  5.863 y2  -  16.13 y  +  16.07

and	U  =  8  ´  10–5 ( H  -  80) (1  +  11 y–2.2)  +  1.2  ´  10–3 H y–3.6

In the case of paths up to dmax (km) hr is evaluated at the mid-point of the path: for longer paths it is determined for all the control points given in Table 1c) and the mean value is used.

### 5.1.3	Field strength determination

For each mode w selected in § 5.1.1, the median field strength is given by:

(15)

where:

f :		transmitting frequency (MHz)

Pt :		transmitter power (dB(1 kW))

Gt :		transmitting antenna gain at the required azimuth angle and elevation angle (D) relative to an isotropic antenna (dB)

Lbf :		basic free-space transmission loss (dB) given by:

(16)

with:

p¢ :		virtual slant range (km)

(17)

Li :		absorption loss (dB) given by:

(18)

with		Ij  =  (1  +  0.0037 R12)  ×  (cos 0.881 cj)1.3	(19)

The minimum value of    å Ij is set to 0.1, and:

i :		angle of incidence at 110 km

k :		number of control points (from Table 1d))

fH :		mean of the values of electron gyrofrequency, for a height of 100 km, determined at the control points given in Table 1d)

cj :		solar zenith angle at the jth control point or 102.15° whichever is the smaller. The equation-of-time, for the middle of the month in question, is incorporated in the calculation of this parameter.

For frequencies above the basic MUF, the absorption continues to vary with frequency and is calculated assuming the same ray-paths as those at the basic MUF.

Lc :		correction term to the absorption loss for E modes which do not penetrate the whole of the absorbing layer:

for f < E(d) MUF:

(20)

if Lc > 0, Lc = 0 dB

if Lc < –9, Lc = –9 dB

for f ³ E (d)MUF and for F2 modes then

Lc = 0 dB

Lm :		“above-the-MUF” loss.

For frequency f equal to or less than the basic MUF (fb) of the given mode,

(21)

For E modes for f > fb:

(22)

or 81 dB whichever is the smaller.

For F2 modes for f > fb:

(23)

or 62 dB whichever is the smaller.

Lg :		summed ground-reflection loss at intermediate reflection points:

for an n-hop mode

Lg  =  2(n  -  1)               dB	(24)

Lh :		factor to allow for auroral and other signal losses, given in Table 2. Each value is evaluated in terms of the geomagnetic latitude Gn (N or S of equator) and local time t for an earth-centred dipole with pole at 78.5° N, 68.2° W: mean values for the control points of Table 1d) are taken.

In the Northern Hemisphere, winter is taken as December-February, equinox as March-May and September-November and summer as June-August. In the Southern Hemisphere, the months for winter and summer are interchanged.

For Gn < 42.5°, Lh = 0 dB;

Lz :		term containing those effects in sky-wave propagation not otherwise included in this method. The present recommended value is 8.0 dB (see also, definition of Ly given in § 5.2).

Discounting modes screened by the E layer, the resultant equivalent median sky-wave field strength, Ets, is taken as the root-sum-squared field strength for N modes where N is chosen to encompass up to the three selected strongest F2 modes and also, in the case of path lengths up to 4 000 km, the two strongest E modes i.e.:

(25)

TABLE  2

Values of Lh giving auroral and other signal losses (dB)

|  | a) Transmission ranges less than or equal to 2 500 km |  |
| --- | --- | --- |

|  | Mid-path local time, t |  |
| --- | --- | --- |

|  | 01 £ t < 04 | 04 £ t < 07 | 07 £ t < 10 | 10 £ t < 13 | 13 £ t < 16 | 16 £ t < 19 | 19 £ t < 22 | 22 £ t < 01 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Gn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 77.5° £ Gn | 2.0 | 16.6 | 16.2 | 1.5 | 0.5 | 1.4 | 11.5 | 11.0 |  |
| 72.5° £ Gn < 77.5° | 3.4 | 18.3 | 18.6 | 0.9 | 0.5 | 2.5 | 13.0 | 13.0 | W |
| 67.5° £ Gn < 72.5° | 6.2 | 15.6 | 12.8 | 2.3 | 1.5 | 4.6 | 17.0 | 15.0 | i |
| 62.5° £ Gn < 67.5° | 7.0 | 16.0 | 14.0 | 3.6 | 2.0 | 6.8 | 19.8 | 16.6 | n |
| 57.5° £ Gn < 62.5° | 2.0 | 14.5 | 16.6 | 1.4 | 0.8 | 2.7 | 13.0 | 12.0 | t |
| 52.5° £ Gn < 57.5° | 1.3 | 11.0 | 13.2 | 0.3 | 0.4 | 1.8 | 12.3 | 10.9 | e |
| 47.5° £ Gn < 52.5° | 0.9 | 10.6 | 12.2 | 0.2 | 0.2 | 1.2 | 11.5 | 10.6 | r |
| 42.5° £ Gn < 47.5° | 0.4 | 10.3 | 11.1 | 0.1 | 0.1 | 0.6 | 10.7 | 10.3 |  |
| 77.5° £ Gn | 1.4 | 12.5 | 17.4 | 3.8 | 1.0 | 2.4 | 12.4 | 13.3 | E |
| 72.5° £ Gn < 77.5° | 3.3 | 11.0 | 11.6 | 5.1 | 2.6 | 4.0 | 16.0 | 17.0 | q |
| 67.5° £ Gn < 72.5° | 6.5 | 12.0 | 21.4 | 8.5 | 4.8 | 6.0 | 10.0 | 13.7 | u |
| 62.5° £ Gn < 67.5° | 6.7 | 11.2 | 17.0 | 9.0 | 7.2 | 9.0 | 10.9 | 15.0 | i |
| 57.5° £ Gn < 62.5° | 2.4 | 14.4 | 17.5 | 5.0 | 2.6 | 4.8 | 15.5 | 16.1 | n |
| 52.5° £ Gn < 57.5° | 1.7 | 12.0 | 15.0 | 3.0 | 2.2 | 4.0 | 13.0 | 14.0 | o |
| 47.5° £ Gn < 52.5° | 1.1 | 11.3 | 13.3 | 2.0 | 1.4 | 2.6 | 12.0 | 12.6 | x |
| 42.5° £ Gn < 47.5° | 0.5 | 10.6 | 11.6 | 1.0 | 0.7 | 1.3 | 11.0 | 11.3 |  |
| 77.5° £ Gn | 2.2 | 12.7 | 11.2 | 2.3 | 2.2 | 3.8 | 14.2 | 13.8 |  |
| 72.5° £ Gn < 77.5° | 2.4 | 13.0 | 12.8 | 3.0 | 2.7 | 4.2 | 14.8 | 14.5 | S |
| 67.5° £ Gn < 72.5° | 4.9 | 14.2 | 16.2 | 4.5 | 3.8 | 5.4 | 17.7 | 17.2 | u |
| 62.5° £ Gn < 67.5° | 6.5 | 14.8 | 19.0 | 6.0 | 4.8 | 9.1 | 19.5 | 18.9 | m |
| 57.5° £ Gn < 62.5° | 3.2 | 12.7 | 14.0 | 3.0 | 3.0 | 6.5 | 16.7 | 15.0 | m |
| 52.5° £ Gn < 57.5° | 2.5 | 11.8 | 12.4 | 2.3 | 2.6 | 5.0 | 14.6 | 14.0 | e |
| 47.5° £ Gn < 52.5° | 1.6 | 11.2 | 11.6 | 1.5 | 1.7 | 3.3 | 13.1 | 12.6 | r |
| 42.5° £ Gn < 47.5° | 0.8 | 10.6 | 10.8 | 0.7 | 0.8 | 1.6 | 11.5 | 11.3 |  |

TABLE  2 (continued)

|  | b) Transmission ranges greater than 2 500 km |  |
| --- | --- | --- |

|  | Mid-path local time, t |  |
| --- | --- | --- |

|  | 01 £ t < 04 | 04 £ t < 07 | 07 £ t < 10 | 10 £ t < 13 | 13 £ t < 16 | 16 £ t < 19 | 19 £ t < 22 | 22 £ t < 01 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Gn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 77.5° £ Gn | 1.5 | 2.7 | 2.5 | 0.8 | 0.0 | 0.9 | 10.8 | 11.6 |  |
| 72.5° £ Gn < 77.5° | 2.5 | 4.5 | 4.3 | 0.8 | 0.3 | 1.6 | 12.0 | 14.8 | W |
| 67.5° £ Gn < 72.5° | 5.5 | 5.0 | 7.0 | 1.9 | 0.5 | 3.0 | 14.5 | 19.6 | i |
| 62.5° £ Gn < 67.5° | 5.3 | 7.0 | 5.9 | 2.0 | 0.7 | 4.0 | 14.5 | 10.0 | n |
| 57.5° £ Gn < 62.5° | 1.6 | 2.4 | 2.7 | 0.6 | 0.4 | 1.7 | 11.8 | 13.5 | t |
| 52.5° £ Gn < 57.5° | 0.9 | 1.0 | 1.3 | 0.1 | 0.1 | 1.0 | 11.5 | 11.4 | e |
| 47.5° £ Gn < 52.5° | 0.6 | 0.6 | 0.8 | 0.1 | 0.1 | 0.6 | 11.0 | 10.5 | r |
| 42.5° £ Gn < 47.5° | 0.3 | 0.3 | 0.4 | 0.0 | 0.0 | 0.3 | 10.5 | 10.4 |  |
| 77.5° £ Gn | 1.0 | 1.2 | 2.7 | 3.0 | 0.6 | 2.0 | 12.3 | 11.6 | E |
| 72.5° £ Gn < 77.5° | 1.8 | 2.9 | 4.1 | 5.7 | 1.5 | 3.2 | 15.6 | 13.6 | q |
| 67.5° £ Gn < 72.5° | 3.7 | 5.6 | 7.7 | 8.1 | 3.5 | 5.0 | 19.5 | 17.3 | u |
| 62.5° £ Gn < 67.5° | 3.9 | 5.2 | 7.6 | 9.0 | 5.0 | 7.5 | 10.0 | 17.9 | i |
| 57.5° £ Gn < 62.5° | 1.4 | 2.0 | 3.2 | 3.8 | 1.8 | 4.0 | 15.4 | 13.4 | n |
| 52.5° £ Gn < 57.5° | 0.9 | 0.9 | 1.8 | 2.0 | 1.3 | 3.1 | 12.7 | 12.0 | o |
| 47.5° £ Gn < 52.5° | 0.6 | 0.6 | 1.2 | 1.3 | 0.8 | 2.0 | 11.8 | 11.3 | x |
| 42.5° £ Gn < 47.5° | 0.3 | 0.3 | 0.6 | 0.6 | 0.4 | 1.0 | 10.9 | 10.6 |  |
| 77.5° £ Gn | 1.9 | 3.8 | 2.2 | 1.1 | 2.1 | 1.2 | 12.3 | 12.4 |  |
| 72.5° £ Gn < 77.5° | 1.9 | 4.6 | 2.9 | 1.3 | 2.2 | 1.3 | 12.8 | 12.7 | S |
| 67.5 °£ Gn < 72.5° | 4.4 | 6.3 | 5.9 | 1.9 | 3.3 | 1.7 | 14.4 | 14.5 | u |
| 62.5° £ Gn < 67.5° | 5.5 | 8.5 | 7.6 | 2.6 | 4.2 | 3.2 | 15.5 | 15.7 | m |
| 57.5° £ Gn < 62.5° | 2.8 | 3.8 | 3.7 | 1.4 | 2.7 | 1.6 | 14.5 | 13.2 | m |
| 52.5° £ Gn < 57.5° | 2.2 | 2.4 | 2.2 | 1.0 | 2.2 | 1.2 | 14.4 | 12.5 | e |
| 47.5° £ Gn < 52.5° | 1.4 | 1.6 | 1.4 | 0.6 | 1.4 | 0.8 | 12.9 | 11.6 | r |
| 42.5° £ Gn < 47.5° | 0.7 | 0.8 | 0.7 | 0.3 | 0.7 | 0.4 | 11.4 | 10.8 |  |

## 5.2	Paths longer than 9 000 km

In this method, predictions are made by dividing the path into the minimum number, n, of equal length hops, none of which exceeds 4 000 km.

The resultant median field strength Etl  is given by:

(26)

E0 is the free-space field strength for 3 MW e.i.r.p. In this case:

(27)

where p¢ is calculated using equations (17) and (11) with hr = 300 km.

Gtl :		largest value of transmitting antenna gain at the required azimuth in the elevation range 0 to 8° (dB)

Gap 	:	increase in field strength due to focusing at long distances given as:

(28)

As Gap from the above formula tends to infinity when D is a multiple of p R0, it is limited to the value of 15 dB.

Ly :		a term similar in concept to Lz. The present recommended value is –3.7 dB.

Note 1 – It should be noted that the values of Ly and Lz are dependent on the elements of the prediction method, so that any changes in those elements should be accompanied by revision of the Ly and Lz values.

fH :		mean of the values of electron gyrofrequency determined at the control points given in Table 1a)

fM :		upper reference frequency. It is determined separately for the two control points indicated in Table 1a) and the lower value is taken:

(29)

(30)

fg :	F2(4000)MUF = 1.1 F2(3000)MUF

fg, noon :	value of fg for a time corresponding to local noon

fg, min :	lowest value of fg which occurs during the 24 hours.

W, X and Y are given in Table 3. The azimuth of the great-circle path is determined at the centre of the whole path and this angle is used for linear interpolation in angle between the East-West and North-South values.

TABLE  3

Values of W, X and Y used for the determination
of the correction factor K

|  | W | X | Y |
| --- | --- | --- | --- |

| East-West | 0.1 | 1.2 | 0.6 |
| --- | --- | --- | --- |

| North-South | 0.2 | 0.2 | 0.4 |
| --- | --- | --- | --- |

fL :	lower reference frequency

(31)

where R12 does not saturate for high values.

In the summation, c is determined for each traverse of the ray-path through the height of 90 km. When c > 90°, cos1/2c is taken as zero.

i90 :		angle of incidence at a height of 90 km

I :		given in Table 4.

TABLE  4

Values of I used in the equation for fL

| Geographic latitudes | Month |
| --- | --- |

| One terminal | Other terminal | J | F | M | A | M | J | J | A | S | O | N | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| > 35° N | > 35° N | 1.1 | 1.05 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1.05 | - 1.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| > 35° N | 35° N-35° S | 1.05 | 1.02 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1.02 | 1.05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| > 35° N | > 35° S | 1.05 | 1.02 | 1 | 1 | 1.02 | 1.05 | 1.05 | 1.02 | 1 | 1 | 1.02 | 1.05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 35° N-35° S | 35° N-35° S | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35° N-35° S | > 35° S | 1 | 1 | 1 | 1 | 1.02 | 1.05 | 1.05 | 1.02 | 1 | 1 | 1 | 1 |
| > 35° S | > 35° S | 1 | 1 | 1 | 1 | 1.05 | 1.1 | 1.1 | 1.05 | 1 | 1 | 1 | 1 |

Aw :		winter-anomaly factor determined at the path mid-point which is unity for geographic latitudes 0 to 30° and at 90° and reaches the maximum values given in Table 5 at 60°. The values at intermediate latitudes are found by linear interpolation.

TABLE   5

Values of the winter-anomaly factor Aw. at 60° geographic
latitude used in the equation for fL

| Hemisphere | Month |
| --- | --- |

|  | J | F | M | A | M | J | J | A | S | O | N | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Northern | 1.30 | 1.15 | 1.03 | 1 | 1 | 1 | 1 | 1 | 1 | 1.03 | 1.15 | 1.30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Southern | 1 | 1 | 1 | 1.03 | 1.15 | 1.30 | 1.30 | 1.15 | 1.03 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The values of fL are calculated at each hour until the local time tr when fL £ 2fLN

where:

(32)

During the next three hours fL is calculated from:

(33)

where t is the time in hours after tr. For subsequent hours fL = fLN until the time when equation (31) gives a higher value.

## 5.3	Paths between 7 000 and 9 000 km

In this distance range the median sky-wave field strength Eti is determined by interpolation between values Ets and Etl. Ets is the root-sum-squared field strength given by equation (25) for up to the three strongest of the possible six F2 modes meeting the three criteria given in § 5.1.1. Etl refers to a composite mode as given by equation (26).

(34)

with

where:	Xs = 100.01 Ets

and	Xl = 100.01 Etl

The basic MUF for the path is equal to the lower of the F2 (dmax)MUF values given from equation (3) for the two control points noted in Table 1a).

# 6.	Median available receiver power

For distance ranges up to 7 000 km, where field strength is calculated by the method of § 5.1, for a given mode w having sky-wave field strength Etw (dB(1 mV/m)) at frequency f (MHz), the corresponding available signal power Prw (dBW) from a lossless receiving antenna of gain Grw (dB relative to an isotropic radiator) in the direction of signal incidence is:

(35)

The resultant median available signal power Pr (dBW) is given by summing the powers arising from the different modes, each mode contribution depending on the receiving antenna gain in the direction of incidence of that mode. For N modes contributing to the summation:

(36)

For distance ranges beyond 9 000 km, where field strength is calculated by the method of § 5.2, the field strength Etl is for the resultant of the composite modes. In this case Pr is determined using equation (35), where Grw is the largest value of receiving antenna gain at the required azimuth in the elevation range 0 to 8°.

In the intermediate range 7 000 to 9 000 km, the power is determined from equation (34) using the powers corresponding to Ets and Etl.

# 7.	Monthly median signal-to-noise ratio

Recommendation ITU-R PI.372 provides values of median atmospheric noise power for reception on a short vertical lossless monopole antenna above perfect ground and also gives corresponding man-made noise and cosmic noise intensities. Let the resultant external noise factor be Fa (dB(kTb)) at frequency f (MHz) where reception is on such an ideal short lossless vertical monopole over a perfectly conducting ground plane with k the Boltzmann constant and T a reference temperature of 288 K. Then, in general, when using some other practical reception antenna the resultant noise factor may differ from this value of Fa (see Recommendation ITU-R PI.372). However, in the absence of complete noise measurement data for different antennas, as a first approximation, it is appropriate to assume that the same Fa applies. Hence the monthly median signal-to-noise ratio S/N (dB) achieved within a bandwidth b (Hz) is:

(37)

# 8.	Signal-to-noise ratios for other percentages of time

Use is made of equations (11) and (12) of Report ITU-R PI.266 to determine the signal-to-noise ratio for a specified percentage of time in terms of the within-an-hour and day-to-day decile deviations of the signals and the noise. Signal fading allowances are those adopted by the Second Session of the World Administrative Radio Conference for the Planning of HF Bands Allocated to the Broadcasting Service (Geneva, 1987) (WARC HFBC-87) with a short-term upper decile deviation of 5 dB and a lower decile deviation of 8 dB. For long-term signal fading the decile deviations are taken as a function of the ratio of operating frequency to the path basic MUF as given in Table 2 of Recommendation ITU-R PI.842.

In the case of atmospheric noise, the decile deviations of noise power arising from day-to-day variability are taken from Recommendation ITU-R PI.372. No allowance for within-an-hour variability is currently applied. For man-made noise, in the absence of direct information on temporal variability, the decile deviations are taken as those given in Recommendation ITU-R PI.372 which strictly relate to a combination of temporal and spatial variability. The combined within-an-hour and day-to-day decile variability of galactic noise is taken as ± 2 dB.

# 9.	Lowest usable frequency (LUF)

The LUF is defined in Recommendation ITU-R PI.373. Consistent with this definition, this is evaluated as the lowest frequency, expressed to the nearest 0.1 MHz, at which a required signal-to-noise ratio is achieved by the monthly median signal-to-noise.

# 10.	Basic circuit reliability (BCR)

The BCR is defined in Recommendation ITU-R PI.842. It is evaluated on the basis of signal/noise ratios incorporating within-an-hour and day-to-day decile variations of both signal field strength and noise background. Distribution about the median uses a distribution formulation from Recommendation ITU-R PI.842. Caution is given that this approach makes no allowance for system degradation due to signal dispersion such as can be particularly important when dealing with digital transmissions.

_________________

*	 	A computer program associated with prediction procedures described in this Recommendation is available from the Radiocommunication Bureau; for details see Resolution ITU-R 63.