Readme File for Recommendation P.534 foEs Data Files

The accompanying foEs50.txt, foEs10.txt, foEs10.txt, and foEs0.1.txt files contain digital maps of the sporadic E critical frequency, foEs, exceeded for average annual average time percentages of 50%, 10%, 1% and 0.1%, respectively.

Each foEsXX.txt file has 121 rows and 241 columns, where each row corresponds to a geodetic latitude, each column corresponds to a geodetic longitude, and the latitude and longitude row and column increments are 1.5°. The upper left corner of each file corresponds to latitude 90° N and longitude 0° E and the lower right corner of the file corresponds to latitude 90° S and longitude 360° E

The accompanying foEs50.png, foEs10.png, foEs10.png, and foEs0.1.png are the corresponding contour files for convenient reference.

The value of foEsXX at any location can be obtained from the values at the four closest grid points using the bi-linear interpolation procedure provided in Annex 1 of Recommendation ITU-R P.1144.

The value of foEs at any location and at any desired exceedance probability level between 0.1% and 50% can be determined by linear interpolation between values of bracketed f0EsXX at the desired location on a linear foEs vs. exceedance probability scale.