Climatic zone map

The file TropoClim.txt is an ASCII file that provides the identification of the different climatic zones in terms of an integer value.  The data is represented as gridded latitude/longitude values in the “standard” ITU-R format. However, there is some variation in the grid spacing of the files, so rather than provide multiple latitude/longitude files and the data format is defined as:

| Source | Latitude (rows) |  |  | Longitude (columns) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | First row (°N) | Spacing (deg) | Number of rows | First col (°E) | Spacing (deg) | Number of cols |
| P.2001 | 89.75 | 0.5 | 360 | -179.75 | 0.5 | 720 |

Notes:

1.	The “First row” value is the latitude of the first row.

2.	The “First col” value is the longitude of the first column.  The columns represent longitudes increasing from 179.75°W to 179.75°E inclusive. That is, the longitude increases eastward. The last column is the same as the first column (360° = 0°) and is provided to simplify interpolation.

3.	“Spacing” gives the latitude/longitude increment between rows/columns.

5.	The file TropoClim.txt contains integer zone identifiers rather than continuous meteorological variables. Consequently the values should not be interpolated to obtain a value at a particular latitude/longitude. Instead the value at the closest grid point should be taken. For this file note that the values in the last column is not a duplicate of the first column. Consequently the latitudes of the rows range from 89.75°N to 89.75°S, and the longitudes of the columns range from 179.75°W to 179.75°E.