SECTION  5F:	ASPECTS  RELATIVE  TO  SPACE  TELECOMMUNICATION  SYSTEMS

RECOMMENDATION  ITU-R PN.618-3

PROPAGATION  DATA  AND  PREDICTION  METHODS  REQUIRED  FOR  THE  DESIGN  OF
EARTH-SPACE  TELECOMMUNICATIONS  SYSTEMS

(Question ITU-R 206/3)

(1986-1990-1992-1994)

Rec. ITU-R PN.618-3

The ITU Radiocommunication Assembly,

considering

a)	that for the proper planning of Earth-space systems it is necessary to have appropriate propagation data and prediction techniques;

b)	that methods have been developed that allow the prediction of the most important propagation parameters needed in planning Earth-space systems;

c)	that as far as possible, these methods have been tested against available data and have been shown to yield an accuracy that is both compatible with the natural variability of propagation phenomena and adequate for most present applications in system planning,

recommends

1.	that the methods for predicting the propagation parameters set out in Annex 1 be adopted for planning Earth-space radiocommunication systems, in the respective ranges of validity indicated in the Annex.

Note 1 – Supplementary information related to the planning of broadcasting-satellite systems as well as maritime, land, and aeronautical mobile-satellite systems, may be found in Recommendations ITU-R PN.679, ITU-R PN.680, ITU-R PN.681 and ITU-R PN.682, respectively.

ANNEX  1

# 1.	Introduction

In the design of Earth-space links for communication systems, several effects must be considered. Effects of the non-ionized atmosphere need to be considered at all frequencies, but become critical above about 1 GHz and for low elevation angles. These effects include:

a)	absorption in atmospheric gases; absorption, scattering and depolarization by hydrometeors (water and ice droplets in precipitation, clouds, etc.); and emission noise from absorbing media; all of which are especially important at frequencies above about 10 GHz;

b)	loss of signal due to beam-divergence of the earth-station antenna, due to the normal refraction in the atmosphere;

c)	a decrease in effective antenna gain, due to phase decorrelation across the antenna aperture, caused by irregularities in the refractive-index structure;

d)	relatively slow fading due to beam-bending caused by large-scale changes in refractive index; more rapid fading (scintillation) and variations in angle of arrival, due to small-scale variations in refractive index;

e)	possible limitations in bandwidth due to multiple scattering or multipath effects, especially in high-capacity digital systems;

f)	attenuation by the local environment of the ground terminal (buildings, trees, etc.);

g)	short-term variations of the ratio of attenuations at the up- and down-link frequencies, which may affect the accuracy of adaptive fade countermeasures.

This Annex deals with the propagation data required in system planning.

Effects of the Earth’s ionosphere and of extra-terrestrial ionized media which may need to be considered for systems operation are treated in Recommendation ITU-R PI.531 and include:

h)	rotation of the plane of polarization (Faraday rotation), particularly if frequency re-use and linear polarization are employed;

j)	dispersion, which results in a differential time delay across the bandwidth of the transmitted signal;

k)	excess time delay;

l)	scintillation, which affects amplitude, phase and angle-of-arrival of the received signal.

This Annex deals only with the effects of the troposphere on the wanted signal. Interference aspects are treated in separate Recommendations and Report:

–	interference between earth stations and terrestrial stations (Recommendation ITU-R PN.452);

–	interference from and to space stations (Recommendation ITU-R PN.619);

–	bidirectional coordination of earth stations (Report ITU-R PN.1010).

An apparent exception is path depolarization which, although of concern only from the standpoint of interference (e.g. between orthogonally-polarized signal transmissions), is directly related to the propagation impairments of the co-polarized direct signal.

The information is arranged according to the link parameters to be considered in actual system planning, rather than according to the physical phenomena causing the different effects. As far as possible, simple prediction methods covering practical applications are provided, along with indications of their range of validity. These relatively simple methods yield satisfactory results in most practical applications, despite the large variability (from year to year and from location to location) of propagation conditions.

As far as possible, the prediction methods in this Annex have been tested against measured data from the data banks of Radiocommunication Study Group 3 (see Recommendation ITU-R PN.311).

# 2.	Propagation loss

The propagation loss on an Earth-space path, relative to the free-space loss, is the sum of different contributions as follows:

–	attenuation by atmospheric gases;

–	attenuation by rain, other precipitation and clouds;

–	focusing and defocusing;

–	decrease in antenna gain due to wave-front incoherence;

–	scintillation and multipath effects;

–	attenuation by sand and dust storms.

Each of these contributions has its own characteristics as a function of frequency, geographic location and elevation angle. As a rule, at elevation angles above 10°, only gaseous attenuation, rain attenuation and possibly scintillation will exceed a few tenths of a decibel, depending on propagation conditions.

(In certain climatic zones, snow and ice accumulations on the surfaces of antenna reflectors and feeds can produce prolonged periods with severe attenuation, which might dominate even the annual cumulative distribution of attenuation.)

## 2.1	Attenuation due to atmospheric gases

Attenuation by atmospheric gases which is entirely caused by absorption depends mainly on frequency, elevation angle, altitude above sea level and water vapour density (absolute humidity). At frequencies below 10 GHz, it may normally be neglected. Its importance increases with frequency above 10 GHz, especially for low elevation angles. This effect is discussed in detail in Recommendation ITU-R PN.676.

### 2.1.1	Procedure for calculating gaseous attenuation

The method described below should be used to calculate the median gaseous absorption loss expected for a given value of surface water vapour density, rw, for frequencies up to 350 GHz (excluding the 57-63 GHz band for which information may be obtained from Recommendation ITU-R PN.676).

Parameters required for the method include:

¦ :	frequency (GHz)

q :	path elevation angle

hs :	height (km) above mean sea level of the Earth terminal; if unknown, a value of hs = 0 will give somewhat conservative results

rw :	water vapour density (g/m3) at the surface (i.e. at height hs) for the location of interest.

In general, the mean or median value of rw for a month or year is input to the model. Representative median values can be obtained from Recommendation ITU-R PN.836. Data are also available from some national weather services. Since the model assumes an averaged height profile for water vapour density, application of the calculation procedure to periods of less than one month may introduce inaccuracies and is not recommended.

Step 1:	Calculate the specific attenuations at the surface for dry air go, and water vapour, gw, for the frequency, ¦, and the water vapour density, rw, as specified in Recommendation ITU-R PN.676.

Step 2:	Compute the equivalent heights for dry air ho, and water vapour, hw, as specified in Recommendation ITU-R PN.676.

Step 3:	Calculate the total slant path gaseous attenuation, Ag, through the atmosphere.

–	For q > 10°:

dB	(1)

–	For q £ 10°:

dB	(2)

with:

(3a)

(3b)

where h is to be replaced by ho or hw as appropriate.

In this prediction method, Re is the effective earth radius after accounting for refraction (see Recommendation ITU-R PN.834). Typically, a value of Re = 8 500 km is appropriate for hs £ 1 km. (For hs > 1 km, see Recommendation ITU-R PN.676.)

Equations (2) to (3b) are engineering formulae derived from equations (10) to (13b) of Recommendation ITU-R PN.676, based on the following approximations:

Note that x = sin q for hs = 0.

### 2.1.2	Variability of gaseous attenuation

At a given frequency the oxygen contribution to atmospheric absorption is relatively constant. However, both water vapour density and its vertical profile are quite variable, which makes computation of accurate cumulative statistics of gaseous attenuation difficult. Approximate distributions can be obtained from the method of § 2.1.1, if surface water vapour density statistics and concurrent surface temperature information are used. Typically, the maximum gaseous attenuation occurs during the season of maximum rainfall (see Recommendation ITU-R PN.836).

Maps illustrating the seasonal variation of absolute humidity at ground level are provided in Recommendation ITU-R PN.836. These can be used in the method to estimate the seasonal variation in clear-air attenuation.

For some systems the variations in atmospheric attenuation exceeded for large percentages of the time (when no rain is present) are important. A study of 11.4 GHz sky noise level variations at several locations in Europe showed that seasonal variations in the monthly median level of total attenuation did not exceed 0.1 dB, and that the total attenuation exceeded for 20% of the worst month was 0.05 to 0.15 dB above the monthly median value, depending on location. These attenuations are thought to be caused mainly by water vapour absorption.

## 2.2	Attenuation by precipitation and clouds

### 2.2.1	Prediction of attenuation statistics for an average year

The general method to predict attenuation due to precipitation and clouds along a slant propagation path is presented in § 2.2.1.1.

If reliable long-term statistical attenuation data are available that were measured at an elevation angle and a frequency (or frequencies) different from those for which a prediction is needed, it is often preferable to scale these data to the elevation angle and frequency in question rather than using the general method. The recommended frequency-scaling method is found in § 2.2.1.2.

Site diversity effects may be estimated with the method of § 2.2.4.

#### 2.2.1.1	Calculation of long-term rain attenuation statistics from point rainfall rate

The following procedure provides estimates of the long-term statistics of the slant-path rain attenuation at a given location for frequencies up to 30 GHz. The following parameters are required:

R0.01 :	point rainfall rate for the location for 0.01% of an average year (mm/h)

hs :		height above mean sea level of the earth station (km)

q :		elevation angle

j :		absolute value of latitude of the earth station (degrees)

¦ :		frequency (GHz).

The geometry is illustrated in Fig. 1.

FIGURE 1/PN.618-3...[D01] = 3 CM

Step 1:	Calculate the effective rain height, hR, for the latitude of the station j:

(4)

Step 2:	For q ³ 5° compute the slant-path length, Ls, below the rain height from:

km	(5)

For q < 5°, the following formula is used:

km	(6)

Step 3:	Calculate the horizontal projection, LG, of the slant-path length from:

km	(7)

Step 4:	Obtain the rain intensity, R0.01, exceeded for 0.01% of an average year (with an integration time of 1 min). If this information cannot be obtained from local data sources, an estimate can be obtained from the maps of rain climates given in Recommendation ITU-R PN.837.

Step 5:	Calculate the reduction factor, r0.01, for 0.01% of the time for R0.01 £ 100 mm/h

(8)

where:	L0  =  35 exp (–0.015 R0.01)

For R0.01 > 100 mm/h, use the value 100 mm/h in place of R0.01.

Step 6:	Obtain the specific attenuation, gR, using the frequency-dependent coefficients given in Recommendation ITU-R PN.838 and the rainfall rate, R0.01, determined from Step 4, by using:

dB/km	(9)

Step 7:	The predicted attenuation exceeded for 0.01% of an average year is obtained from:

dB	(10)

Step 8:	The estimated attenuation to be exceeded for other percentages of an average year, in the range 0.001% to 1%, is determined from the attenuation to be exceeded for 0.01% for an average year by using:

(11)

This interpolation formula gives factors of 0.12, 0.38, 1 and 2.14 for 1%, 0.1%, 0.01% and 0.001%, respectively.

Step 9:	If desired, the value of p corresponding to a given value of Ap may be computed from the inverted form of equation (11):

(12)

with the constraint that:

(13)

When the complete prediction method above was tested using the procedure set out in Annex 1 to Recommendation ITU-R PN.311, the results differed between high and low latitudes. For latitudes above 30°, the prediction was found to be in good agreement with available measurement data in the range 0.001% to 0.1%, with a standard deviation of some 35%, when used with concurrent rain rate measurements.

This method provides an estimate of the long-term statistics of attenuation due to rain. When comparing measured statistics with the prediction, allowance should be given for the rather large year-to-year variability in rainfall rate statistics (see Recommendation ITU-R PN.678).

Attenuation due to water cloud or fog of known liquid water content can be calculated from Recommendation ITU-R PN.840. Except for clouds of high water content, total attenuations due to cloud will not be great at frequencies below 30 GHz and are likely in any case to be included in the measured statistics. The effects of ice cloud, dry hail and dry snow can be generally neglected for these frequencies.

As the frequency increases, the effect of clouds becomes steadily more important. However, recent measurements show that in some types of cloud (e.g. strato-cumulus) the additional attenuation produced at vertical incidence is no more than 0.5 to 1 dB, even at 150 GHz. On the other hand, clouds with a high liquid-water content, such as cumulo-nimbus, can cause an additional attenuation of about 4 to 5 dB at 100 GHz and up to 8 dB at 150 GHz.

#### 2.2.1.2	Long-term frequency and polarization scaling of rain attenuation statistics

The method of § 2.2.1.1 may be used to investigate the dependence of attenuation statistics on elevation angle, polarization and frequency, and is therefore a useful general tool for scaling of attenuation according to these parameters.

If reliable attenuation data measured at one frequency are available, the following empirical formula giving an attenuation ratio directly as a function of frequency and attenuation may be applied for frequency scaling on the same path in the frequency range 7 to 50 GHz:

(14)

where:

(15a)

(15b)

A1 and A2 are the equiprobable values of the excess rain attenuation at frequencies ¦1 and ¦2 (GHz), respectively.

Frequency scaling from reliable attenuation data is preferred, when applicable, rather than the prediction methods starting from rain data.

When polarization scaling is required, it is more appropriate to use directly the parameters k and a as given in Recommendation ITU-R PN.838. These parameters also provide a radiometeorological basis for frequency scaling.

### 2.2.2	Seasonal variations – worst-month

System planning often requires the attenuation value exceeded for a time percentage, pw, of the worst month. The following procedure is used to estimate the attenuation exceeded for a specified percentage of the worst month.

Step 1:	Obtain the annual time percentage, p, corresponding to the desired worst-month time percentage, pw, by using the equation specified in Recommendation ITU-R PN.581 and by applying any adjustments to p as prescribed therein.

Step 2:	For the path in question obtain the attenuation, A (dB), exceeded for the resulting annual time percentage, p, from the method of § 2.2.1.1, or from measured or frequency-scaled attenuation statistics. This value of A is the estimated attenuation for pw per cent of the worst month.

Curves giving the variation of worst-month values from their mean are provided in Recommendation ITU-R PN.581.

### 2.2.3	Variability in space and time of statistics

Precipitation attenuation distributions measured on the same path at the same frequency and polarization may show marked year-to-year variations. In the range 0.001% to 0.1% of the year, the attenuation values at a fixed probability level are observed to vary by more than 20% r.m.s. When the models for attenuation prediction or scaling in § 2.2.1 are used to scale observations at a location to estimate for another path at the same location, the variations increase to more than 25% r.m.s.

### 2.2.4	Site diversity

Intense rain cells that cause large attenuation values on an Earth-space link often have horizontal dimensions of no more than a few kilometres. Diversity systems able to re-route traffic to alternate earth stations, or with access to a satellite with extra on-board resources available for temporary allocation, can improve the system reliability considerably.

Two concepts exist for characterizing diversity performance: the “diversity improvement factor” is defined as the ratio of the single-site time percentage and the diversity time percentage, at the same attenuation level. “Diversity gain” is the difference (dB) between the single-site and diversity attenuation values for the same time percentage. Both parameters are important, depending on the system design approach, and prediction procedures for both are given below.

The procedures have been tested at frequencies between 10 and 30 GHz, which is the recommended frequency range of applicability. The diversity prediction procedures are only recommended for time percentages less than 0.1%. At time percentages above 0.1%, the rainfall rate is generally small and the corresponding site diversity improvement is not significant.

#### 2.2.4.1	Diversity improvement factor

The diversity improvement factor, I, is given by:

(16)

where p1 and p2 are the respective single-site and diversity time percentages, and b is a parameter depending on link characteristics. The approximation on the right-hand side of equation (16) is acceptable since b2 is generally small.

From a large number of measurements carried out in the 10-20 GHz band, and mainly between 11 and 13.6 GHz, it has been found that the value of b2 depends basically on the distance, d, between the stations, and only slightly on the angle of elevation and the frequency. It is found that b2 can be expressed by the following empirical relationship:

(17)

Figure 2 shows p2 versus p1 on the basis of equations (16) and (17).

#### 2.2.4.2	Diversity gain

The diversity gain, G (dB), between pairs of sites is calculated with the empirical expression given below. Parameters required for the calculation of diversity gain are:

d :	separation (km) between the two sites

A :	path rain attenuation (dB) for a single site

¦ :	frequency (GHz)

q :	path elevation angle (degrees)

y :	angle (degrees) made by the azimuth of the propagation path with respect to the baseline between sites, chosen such that y £ 90°.

Step 1:	Calculate the gain contributed by the spatial separation from:

(18)

where:

Step 2:	Calculate the frequency-dependent gain from:

(19)

Step 3:	Calculate the gain term dependent on elevation angle from:

(20)

Step 4:	Calculate the baseline-dependent term from the expression:

(21)

Step 5:	Compute the net diversity gain as the product:

dB	(22)

When the above method was tested against the Radiocommunication Study Group 3 site diversity data bank, the arithmetic mean and standard deviation were found to be 0.14 dB and 0.96 dB, respectively, with an r.m.s. error of 0.97 dB.

FIGURE 2/PN.618...[D02] = 3 CM

### 2.2.5	Characteristics of precipitation events

#### 2.2.5.1	Durations of individual fades

The durations of rain fades that exceed a specified attenuation level are approximately log-normally distributed. Median durations are of the order of several minutes. No significant dependence of these distributions on fade depth is evident in most measurements for fades of less than 20 dB, implying that the larger total time percentage of fades observed at lower fade levels or at higher frequencies is composed of a larger number of individual fades having more or less the same distribution of durations. Significant departures from log-normal seem to occur for fade durations of less than about half a minute. Fade durations at a specified fade level tend to increase with decreasing elevation angle.

For the planning of ISDN connections via satellite, data are needed on the contribution of attenuation events shorter than 10 s to the total fading time. This information is especially relevant for the attenuation level corresponding to the outage threshold, where events longer than 10 s contribute to system unavailable time, while shorter events affect system performance during available time (see Recommendation ITU-R S.579). Existing data indicate that in the majority of cases, the exceedance time during available time is 2% to 10% of the net exceedance time. However, at low elevation angles where the short period signal fluctuations due to tropospheric scintillation become statistically significant, there are some cases for which the exceedance time during available time is far larger than in the case at higher elevation Earth-space paths.

#### 2.2.5.2	Rates of change of attenuation (fading rate)

There is broad agreement that the distributions of positive and negative fade rates are log-normally distributed and very similar to each other. The dependence of fade rate on fade depth has not been established.

#### 2.2.5.3	Correlation of instantaneous values of attenuation at different frequencies

Data on the instantaneous ratio of rain attenuation values at different frequencies are of interest for a variety of adaptive fade techniques. The frequency-scaling ratio has been found to be log-normally distributed, and is influenced by rain type and rain temperature. Data reveal that the short-term variations in the attenuation ratio can be significant, and are expected to increase with decreasing path elevation angle.

## 2.3	Clear air effects

### 2.3.1	Defocusing loss

The regular decrease of refractive index with height causes ray-bending and hence a defocusing effect at low angles of elevation (Recommendation ITU-R PN.834). The magnitude of the defocusing loss of the antenna beam is independent of frequency, and is less than 0.4 dB at 3° elevation, even for very high values of refractivity at ground level. It is therefore seldom important in system design in comparison with the attenuation due to other causes.

### 2.3.2	Decrease in antenna gain due to wave-front incoherence

Incoherence of the wave-front of a wave incident on a receiving antenna is caused by small-scale irregularities in the refractive index structure of the atmosphere (see Recommendation ITU-R PN.834). Apart from the rapid signal fluctuations discussed in § 2.4, they cause an antenna-to-medium coupling loss that can be described as a decrease of the antenna gain.

This effect increases both with increasing frequency and decreasing elevation angle, and is a function of antenna diameter. Measurements with a 22 m antenna at a 5° elevation angle showed losses of about 0.2 to 0.4 dB at 4 and 6 GHz. Measurements with a 7 m antenna at 5° elevation at 15.5 and 31.6 GHz gave losses of 0.3 and 0.6 dB, respectively. In practice, the effect is only likely to be significant for narrow-beamwidth antennas, high frequencies, and elevation angles below 5°.

## 2.4	Scintillation and multipath effects

Small-scale irregularities in the atmospheric refractive index cause rapid signal variations. Tropospheric effects in the absence of precipitation are unlikely to produce serious fading in space telecommunication systems operating at frequencies below about 10 GHz and at elevation angles above 10°. At low elevation angles and at frequencies above about 10 GHz, tropospheric scintillations can on occasion cause serious degradations in performance. At some locations, ionospheric scintillations may be important at frequencies below about 6 GHz (see Recommendation ITU-R PI.531).

The magnitude of tropospheric scintillations depends on the magnitude and structure of the refractive index variations, increasing with frequency and with the path length through the medium, and decreasing as the antenna beamwidth decreases because of aperture averaging (see Recommendation ITU-R PI.531). Monthly-averaged r.m.s. fluctuations are well-correlated with the wet term of the radio refractivity, Nwet, which depends on the water vapour content of the atmosphere. Nwet may be estimated for periods of a month or longer from meteorological data obtained at the surface.

### 2.4.1	Calculation of monthly and long-term statistics of amplitude scintillations at elevation angles greater than 4°

A general technique for predicting the cumulative distribution of tropospheric scintillation at elevation angles greater than 4° is given below. It is based on monthly or longer averages of temperature t (°C) and relative humidity H, and reflects the specific climatic conditions of the site. As the averages of t and H vary with season, distributions of scintillation fade depth exhibit seasonal variations, which may also be predicted by using seasonal averages of t and H in the method. Values of t and H may be obtained from weather information for the site(s) in question.

The procedure has been tested at frequencies between 7 and 14 GHz, but is recommended for applications up to at least 20 GHz.

Parameters required for the method include:

t :	average surface ambient temperature (°C) at the site for a period of one month or longer

H :	average surface relative humidity (%) at the site for a period of one month or longer

¦ :	frequency (GHz), where 4 GHz £ ¦ £ 20 GHz

q :	path elevation angle, where q ³ 4°

D :	physical diameter (m) of the earth-station antenna

h :	antenna efficiency; if unknown, h = 0.5 is a conservative estimate.

Step 1:	For the value of t, calculate the saturation water vapour pressure, es, (kPa), as specified in Recommendation ITU-R PN.453.

Step 2:	Compute the wet term of the radio refractivity, Nwet, corresponding to es, t and H as given in Recommendation ITU-R PN.453.

Step 3:	Calculate the standard deviation of the signal amplitude, sre¦, used as reference:

dB	(23)

Step 4:	Calculate the effective path length L according to:

m	(24)

where hL is the height of the turbulent layer; the value to be used is hL = 1 000 m.

Step 5:	Estimate the effective antenna diameter, De¦¦, from the geometrical diameter D, and the antenna efficiency h:

m	(25)

Step 6:	Calculate the antenna averaging factor from:

(26)

with:

where ¦ is the carrier frequency (GHz).

Step 7:	Calculate the standard deviation of the signal for the considered period and propagation path:

(27)

Step 8:	Calculate the time percentage factor a(p) for the time percentage, p, of concern in the range 0.01 < p £ 50:

(28)

=  – 1.71 log10– p  +  3.0	(28)

Step 9:	Calculate the scintillation fade depth for the time percentage p by:

x( p)  =  a( p )  ×  s                dB	(29)

### 2.4.2	Signal variations at elevation angles lower than about 4°

Amplitude scintillation increases rapidly with decreasing elevation angle. At elevation angles below about 2° to 4°, large-scale fading due to the effect of tropospheric stratification becomes important.

### 2.4.3	Signal fading mixed with rain attenuation and scintillation

Although rain attenuation is the dominant effect at frequencies above 10 GHz, the effect of scintillation is by no means negligible, particularly at elevation angles lower than 10°. In this case, overall signal fading is estimated by:

dB	(30)

where:

A :		total attenuation excluding gaseous absorption (dB)

AR :		rain attenuation (dB)

AS :		signal fade due to scintillation (dB).

## 2.5	Attenuation by sand and dust storms

Very little is known about the effects of sand and dust storms on radio signals on slant-paths. Available data indicate that at frequencies below 30 GHz, high particle concentrations and/or high moisture contents are required to produce significant propagation effects.

# 3.	Noise temperature

As attenuation increases, so does emission noise. For earth stations with low-noise front-ends, this increase of noise temperature may have a greater impact on the resulting signal-to-noise ratio than the attenuation itself.

The atmospheric contribution to antenna noise in a ground station may be estimated with the equation:

(31)

where:

Ts :		sky-noise temperature (K) as seen by the antenna

A :		path attenuation (dB), and

Tm :		effective temperature (K) of the medium.

The effective temperature is dependent on the contribution of scattering to attenuation and on the physical extent of clouds and rain cells on the vertical variation of the physical temperature of the scatterers and, to a lesser extent, on the antenna beamwidth. By comparing radiometric observations and simultaneous beacon attenuation measurements, the effective temperature of the medium has been determined to lie in the range 260-280 K for rain and clouds along the path at frequencies between 10 and 30 GHz.

When the attenuation is known, the following effective temperatures of the mediums may be used to obtain an upper limit to sky-noise temperature at frequencies below 60 GHz:

Tm  =  280 K for clouds

Tm  =  260 K for rain

The noise environments of stations on the surface of the Earth and in space are treated in detail in Recommendation ITU-R PN.677.

For satellite telecommunication systems using the geostationary orbit, earth stations will find that the Sun and, to a lesser extent, the Moon, are significant noise sources at all frequencies and that the galactic background is a possibly significant consideration at frequencies below about 2 GHz (see Recommendation ITU-R PN.677). In addition, contributions to the sky background noise temperature may be given by Cygnus A and X, Cassiopeia A, Taurus and The Crab nebula.

To determine the system noise temperature of earth stations from the brightness temperatures discussed above, the equations of Recommendation ITU-R PN.677 may be used.

# 4.	Cross-polarization effects

Frequency re-use by means of orthogonal polarizations is often used to increase the capacity of space telecommunication systems. This technique is restricted, however, by depolarization on atmospheric propagation paths. Various depolarization mechanisms, especially hydrometeor effects, are important in the troposphere.

Faraday rotation of the plane of polarization by the ionosphere is discussed in Recommendation ITU-R PI.531. As much as 1° of rotation may be encountered at 10 GHz, and greater rotations at lower frequencies. As seen from the earth station, the planes of polarization rotate in the same direction on the up- and down-links. It is therefore not possible to compensate for Faraday rotation by rotating the feed system of the antenna, if the same antenna is used both for transmitting and receiving.

## 4.1	Calculation of long-term statistics of hydrometeor-induced cross-polarization

To calculate long-term statistics of depolarization from rain attenuation statistics the following parameters are needed:

Ap :	rain attenuation (dB) exceeded for the required percentage of time, p, for the path in question, commonly called co-polar attenuation, CPA

t :	tilt angle of the linearly polarized electric field vector with respect to the horizontal (for circular polarization use t = 45°)

¦ :	frequency (GHz)

q :	path elevation angle (degrees).

The method described below to calculate cross-polarization discrimination (XPD) statistics from rain attenuation statistics for the same path is valid for 8 GHz £ ¦ £ 35 GHz and q £ 60°. The procedure for scaling to frequencies down to 4 GHz is given in § 4.3 (and see Step 8 below).

Step 1:	Calculate the frequency-dependent term:

Cf  =  30 log f                for 8  £  f  £  35 GHz	(32)

Step 2:	Calculate the rain attenuation dependent term:

CA  =  V ( f ) log Ap	(33)

where:

V ( f )  =  12.8 f 0.19                for 28  £  f  £  20 GHz

V ( f )  =  22.6 f 0,19                for 20  <  f  £  35 GHz

Step 3:	Calculate the polarization improvement factor:

(34)

The improvement factor Ct = 0 for t = 45° and reaches a maximum value of 15 dB for t = 0° or 90°.

Step 4:	Calculate the elevation angle-dependent term:

Cq  =  – 40 log (cos q )                for q  £  60°	(35)

Step 5:	Calculate the canting angle dependent term:

Cs  =  0.0052 s2	(36)

s is the effective standard deviation of the raindrop canting angle distribution, expressed in degrees; s takes the value 0°, 5°, 10° and 15° for 1%, 0.1%, 0.01% and 0.001% of the time, respectively.

Step 6:	Calculate rain XPD not exceeded for p% of the time:

XPDrain  =  Cf  –  CA  +  Ct  +  Cq  +  Cs                dB	(37)

Step 7:	Calculate the ice crystal dependent term:

Cice  =  XPDrain   ´  (0.3  +  0.1 log p) / 2                dB	(38)

Step 8:	Calculate the XPD not exceeded for p% of the time, including the effects of ice:

XPDp  =  XPDrain  –  Cice	                dB	(39)

In this prediction method in the frequency band 4 to 6 GHz where path attenuation is low, Ap statistics are not very useful for predicting XPD statistics. For frequencies below 8 GHz, the frequency-scaling formula of § 4.3 can be used to scale cross-polarization statistics calculated for 8 GHz down to the 6 to 4 GHz band.

## 4.2	Joint statistics of XPD and attenuation

The conditional probability distribution of XPD for a given value of attenuation, Ap, can be modelled by assuming that the cross-polar to co-polar voltage ratio, r = 10–XPD/20, is normally distributed. Parameters of the distribution are the mean value, rm, which is very close to 10–XPDrain/20, with XPDrain given by equation (37), and the standard deviation, sr, which assumes the almost-constant value of 0.038 for 3 dB £ Ap £ 8 dB.

## 4.3	Long-term frequency and polarization scaling of statistics of hydrometeor-induced cross-polarization

Long-term XPD statistics obtained at one frequency and polarization tilt angle can be scaled to another frequency and polarization tilt angle using the semi-empirical formula:

for  4  £  f1, f2  £  30 GHz	(40)

where XPD1 and XPD2 are the XPD values not exceeded for the same percentage of time at frequencies ¦1 and ¦2 and polarization tilt angles, t1 and t2, respectively.

Equation (40) is based on the same theoretical formulation as the prediction method of § 4.1, and can be used to scale XPD data that include the effects of both rain and ice depolarization, since it has been observed that both phenomena have approximately the same frequency dependence at frequencies less than about 30 GHz.

## 4.4	Data relevant to cross-polarization cancellation

Experiments have shown that a strong correlation exists between rain depolarization at 6 and 4 GHz on Earth-space paths, both on the long term and on an event basis, and up-link depolarization compensation utilizing concurrent down-link depolarization measurements appears feasible. Only differential phase effects were apparent, even for severe rain events, and single-parameter compensation (i.e. for differential phase) appears sufficient at 6 and 4 GHz.

Measurements at 6 and 4 GHz have also shown that 99% of the XPD variations are slower than ±4 dB/s, or equivalently, less than ±1.5 degrees/s in the mean path differential phase shift. Therefore, the time constant of a depolarization compensation system at these frequencies need only be about 1 s.

# 5.	Propagation delays

Radiometeorologically-based methods for estimating the average propagation delay or range error, and the corresponding variations, for Earth-space paths through the troposphere are available in Recommendation ITU-R PN.834. The delay variance is required for satellite ranging and synchronization in digital satellite communication systems. At frequencies above 10 GHz, the ionospheric time delay (see Recommendation ITU-R PI.531) is generally smaller than that for the troposphere, but may have to be considered in special cases.

Range determination to centimetre accuracy requires careful consideration of the various contributions to excess range delay. The water vapour component amounts to 10 cm for a zenith path and a reference atmosphere with surface water vapour concentration of 7.5 g/m3 and 2 km scale height (see Recommendation ITU-R PN.676). This contribution is the largest source of uncertainty, even though the dry atmosphere adds 2.3 m of the zenithal excess range delay.

For current satellite telecommunication applications, additional propagation delays contributed by precipitation are sufficiently small to be ignored.

# 6.	Bandwidth limitations

In the vicinity of the absorption lines of atmospheric gases, anomalous dispersion produces small changes in the refractive index. However, these refractive index changes are small in the bands allocated to Earth-space communications, and will not restrict the bandwidth of systems.

Multiple scattering in rain can limit the bandwidth of incoherent transmission systems due to variation in time delays of the multiple-scattered signals; however, the attenuation itself under such circumstances will present a far more serious problem. A study of the problem of bandwidth limitations imposed by the frequency dependence of attenuation and phase shift due to rain on coherent transmission systems showed that such bandwidth limitations are in excess of 3.5 GHz for all situations likely to be encountered. These are greater than any bandwidth allocated for Earth-space communications below 40 GHz, and the rain attenuation will therefore be far more important than its frequency dependence.

# 7.	Angle of arrival

Elevation-angle errors due to refraction are discussed in Recommendation ITU-R PN.834. The total angular refraction (the increase in apparent elevation) is about 0.65°, 0.35° and 0.25°, for elevation angles of 1°, 3° and 5°, respectively, for a tropical maritime atmosphere. For a polar continental climate the corresponding values are 0.44°, 0.25° and 0.17°. Other climates will have values between these two extremes. The day-to-day variation in apparent elevation is of the order of 0.1° (r.m.s.) at 1° elevation, but the variation decreases rapidly with increasing elevation angle.

Short-term angle-of-arrival fluctuations are discussed in Recommendation ITU-R PN.834. Short-term variations, due to changes in the refractivity-height variation, may be of the order of 0.02° (r.m.s.) at 1° elevation, again decreasing rapidly with increasing elevation angle. In practice, it is difficult to distinguish between the effect of the short-term changes in the refractivity-height distribution and the effect of random irregularities superimposed on that distribution. A statistical analysis of the short-term angle-of-arrival fluctuation at 19.5 GHz and at an elevation angle of 48° suggests that both in elevation and azimuth directions, standard deviations of angle-of-arrival fluctuations are about 0.002° at the cumulative time percentage of 1%. The seasonal variation of angle-of-arrival fluctuations suggests that the fluctuations increase in summer and decrease in winter. The diurnal variation suggests that they increase in the daytime and decrease both in the early morning and the evening.

_________________