RECOMMENDATION  ITU-R  P.676-5

Attenuation by atmospheric gases

(Question ITU-R 201/3)

(1990-1992-1995-1997-1999-2001)

The ITU Radiocommunication Assembly,

considering

a)	the necessity of estimating the attenuation by atmospheric gases on terrestrial and slant paths,

recommends

1	that, for general application, the procedures in Annex 1 be used to calculate gaseous attenuation at frequencies up to 1 000 GHz. (Software code in MATLAB is available from the Radiocommunication Bureau);

2	that, for approximate estimates of gaseous attenuation in the frequency range 1 to 350 GHz, the simpler procedure given in Annex 2 be used.

ANNEX  1

Line-by-line calculation of gaseous attenuation

# 1	Specific attenuation

The specific attenuation at frequencies up to 1 000 GHz due to dry air and water vapour, can be evaluated most accurately at any value of pressure, temperature and humidity by means of a summation of the individual resonance lines from oxygen and water vapour, together with small additional factors for the non-resonant Debye spectrum of oxygen below 10 GHz, pressure-induced nitrogen attenuation above 100 GHz and a wet continuum to account for the excess water vapour-absorption found experimentally. Figure 1 shows the specific attenuation using the model, calculated from 0 to 1 000 GHz at 1 GHz intervals, for a pressure of 1 013 hPa, temperature of 15 C for the cases of a water-vapour density of 7.5 g/m3 (Curve A) and a dry atmosphere (Curve B).

Near 60 GHz, many oxygen absorption lines merge together, at sea-level pressures, to form a single, broad absorption band, which is shown in more detail in Fig. 2. This Figure also shows the oxygen attenuation at higher altitudes, with the individual lines becoming resolved at lower pressures.

For quick and approximate estimates of specific attenuation at frequencies up to 350 GHz, in cases where high accuracy is not required, simplified algorithms are given in Annex 2 for restricted ranges of meteorological conditions.

The specific gaseous attenuation is given by:

γ=γo+γw=0.1820f N \( f \) dBkm} {	(1)

where o and w are the specific attenuations (dB/km) due to dry air and water vapour, respectively, and where f is the frequency (GHz) and N ( f  is the imaginary part of the frequency-dependent complex refractivity:

N \( f \) = Sum cSub { size 8{i} }  {`S rSub { size 8{i} } `F rSub { size 8{i} } +N rSub { size 8{D} }  rSup { size 8{(f)+NW} }  \( f \) } } {	(2)

Si is the strength of the i-th line, Fi is the line shape factor and the sum extends over all the lines; ND} }  \( f \) } { and NW} }  \( f \) } { are dry and wet continuum spectra.

The line strength is given by:

Si a1 × 10–7 p θ3 exp  a2 (1 – θ)for oxygen= b1 × 10–1 e θ3.5 exp  b2 (1 – θ)for water vapour	(3)

where:

p :	dry air pressure (hPa)

e :	water vapour partial pressure in hPa (total barometric pressure P  p  e)

  	300/T

T :	temperature (K).

Local values of p, e and T measured profiles (e.g. using radiosondes) should be used; however, in the absence of local information, the reference standard atmospheres described in Recommendation ITU-R P.835 should be used.

The water-vapour partial pressure, e, may be obtained from the water-vapour density  using the expression:

e=ρ T216.7	(4)

The coefficients a1, a2 are given in Table 1 for oxygen, those for water vapour, b1 and b2, are given in Table 2.

The line-shape factor is given by:

Fi=ffi Δf–δ fi–ffi–f2+Δf2 + Δf–δ fi+ffi+f2+Δf2	(5)

where fi is the line frequency and f is the width of the line:

Δf a3 × 10–4 (p θ(0.8 – a4) + 1.1 e θ)for oxygen= b3 × 10–4 (p θb4 + b5 e θb6)for water vapour	(6)

and  is a correction factor which arises due to interference effects in oxygen lines:

δ (a5 + a6 θ) × 10–4 p θ0.8for oxygen= 0for water vapour	(7)

The spectroscopic coefficients are given in Tables 1 and 2.

TABLE  1

Spectroscopic data for oxygen attenuation

| f0 | a1 | a2 | a3 | a4 | a5 | a6 |
| --- | --- | --- | --- | --- | --- | --- |
| 50.474238 | 0.94 | 9.694 | 8.60 | 0 | 1.600 | 5.520 |
| 50.987749 | 2.46 | 8.694 | 8.70 | 0 | 1.400 | 5.520 |
| 51.503350 | 6.08 | 7.744 | 8.90 | 0 | 1.165 | 5.520 |
| 52.021410 | 14.14 | 6.844 | 9.20 | 0 | 0.883 | 5.520 |
| 52.542394 | 31.02 | 6.004 | 9.40 | 0 | 0.579 | 5.520 |
| 53.066907 | 64.10 | 5.224 | 9.70 | 0 | 0.252 | 5.520 |
| 53.595749 | 124.70 | 4.484 | 10.00 | 0 | – 0.066 | 5.520 |
| 54.130000 | 228.00 | 3.814 | 10.20 | 0 | – 0.314 | 5.520 |
| 54.671159 | 391.80 | 3.194 | 10.50 | 0 | – 0.706 | 5.520 |
| 55.221367 | 631.60 | 2.624 | 10.79 | 0 | –1.151 | 5.514 |
| 55.783802 | 953.50 | 2.119 | 11.10 | 0 | – 0.920 | 5.025 |
| 56.264775 | 548.90 | 0.015 | 16.46 | 0 | 2.881 | – 0.069 |
| 56.363389 | 1 344.00 | 1.660 | 11.44 | 0 | – 0.596 | 4.750 |
| 56.968206 | 1 763.00 | 1.260 | 11.81 | 0 | – 0.556 | 4.104 |
| 57.612484 | 2 141.00 | 0.915 | 12.21 | 0 | –2.414 | 3.536 |
| 58.323877 | 2 386.00 | 0.626 | 12.66 | 0 | –2.635 | 2.686 |
| 58.446590 | 1 457.00 | 0.084 | 14.49 | 0 | 6.848 | – 0.647 |
| 59.164207 | 2 404.00 | 0.391 | 13.19 | 0 | –6.032 | 1.858 |
| 59.590983 | 2 112.00 | 0.212 | 13.60 | 0 | 8.266 | –1.413 |
| 60.306061 | 2 124.00 | 0.212 | 13.82 | 0 | –7.170 | 0.916 |
| 60.434776 | 2 461.00 | 0.391 | 12.97 | 0 | 5.664 | –2.323 |
| 61.150560 | 2 504.00 | 0.626 | 12.48 | 0 | 1.731 | –3.039 |
| 61.800154 | 2 298.00 | 0.915 | 12.07 | 0 | 1.738 | –3.797 |
| 62.411215 | 1 933.00 | 1.260 | 11.71 | 0 | – 0.048 | – 4.277 |
| 62.486260 | 1 517.00 | 0.083 | 14.68 | 0 | – 4.290 | 0.238 |
| 62.997977 | 1 503.00 | 1.665 | 11.39 | 0 | 0.134 | – 4.860 |
| 63.568518 | 1 087.00 | 2.115 | 11.08 | 0 | 0.541 | –5.079 |
| 64.127767 | 733.50 | 2.620 | 10.78 | 0 | 0.814 | –5.525 |
| 64.678903 | 463.50 | 3.195 | 10.50 | 0 | 0.415 | –5.520 |
| 65.224071 | 274.80 | 3.815 | 10.20 | 0 | 0.069 | –5.520 |
| 65.764772 | 153.00 | 4.485 | 10.00 | 0 | – 0.143 | –5.520 |
| 66.302091 | 80.09 | 5.225 | 9.70 | 0 | – 0.428 | –5.520 |
| 66.836830 | 39.46 | 6.005 | 9.40 | 0 | – 0.726 | –5.520 |
| 67.369598 | 18.32 | 6.845 | 9.20 | 0 | –1.002 | –5.520 |
| 67.900867 | 8.01 | 7.745 | 8.90 | 0 | –1.255 | –5.520 |
| 68.431005 | 3.30 | 8.695 | 8.70 | 0 | –1.500 | –5.520 |
| 68.960311 | 1.28 | 9.695 | 8.60 | 0 | –1.700 | –5.520 |
| 118.750343 | 945.00 | 0.009 | 16.30 | 0 | – 0.247 | 0.003 |
| 368.498350 | 67.90 | 0.049 | 19.20 | 0.6 | 0 | 0 |
| 424.763124 | 638.00 | 0.044 | 19.16 | 0.6 | 0 | 0 |
| 487.249370 | 235.00 | 0.049 | 19.20 | 0.6 | 0 | 0 |
| 715.393150 | 99.60 | 0.145 | 18.10 | 0.6 | 0 | 0 |
| 773.839675 | 671.00 | 0.130 | 18.10 | 0.6 | 0 | 0 |
| 834.145330 | 180.00 | 0.147 | 18.10 | 0.6 | 0 | 0 |

TABLE  2

Spectroscopic data for water-vapour attenuation

| f0 | b1 | b2 | b3 | b4 | b5 | b6 |
| --- | --- | --- | --- | --- | --- | --- |
| 22.235080 | 0.1090 | 2.143 | 28.11 | 0.69 | 4.80 | 1.00 |
| 67.813960 | 0.0011 | 8.735 | 28.58 | 0.69 | 4.93 | 0.82 |
| 119.995941 | 0.0007 | 8.356 | 29.48 | 0.70 | 4.78 | 0.79 |
| 183.310074 | 2.3000 | 0.668 | 28.13 | 0.64 | 5.30 | 0.85 |
| 321.225644 | 0.0464 | 6.181 | 23.03 | 0.67 | 4.69 | 0.54 |
| 325.152919 | 1.5400 | 1.540 | 27.83 | 0.68 | 4.85 | 0.74 |
| 336.187000 | 0.0010 | 9.829 | 26.93 | 0.69 | 4.74 | 0.61 |
| 380.197372 | 11.9000 | 1.048 | 28.73 | 0.69 | 5.38 | 0.84 |
| 390.134508 | 0.0044 | 7.350 | 21.52 | 0.63 | 4.81 | 0.55 |
| 437.346667 | 0.0637 | 5.050 | 18.45 | 0.60 | 4.23 | 0.48 |
| 439.150812 | 0.9210 | 3.596 | 21.00 | 0.63 | 4.29 | 0.52 |
| 443.018295 | 0.1940 | 5.050 | 18.60 | 0.60 | 4.23 | 0.50 |
| 448.001075 | 10.6000 | 1.405 | 26.32 | 0.66 | 4.84 | 0.67 |
| 470.888947 | 0.3300 | 3.599 | 21.52 | 0.66 | 4.57 | 0.65 |
| 474.689127 | 1.2800 | 2.381 | 23.55 | 0.65 | 4.65 | 0.64 |
| 488.491133 | 0.2530 | 2.853 | 26.02 | 0.69 | 5.04 | 0.72 |
| 503.568532 | 0.0374 | 6.733 | 16.12 | 0.61 | 3.98 | 0.43 |
| 504.482692 | 0.0125 | 6.733 | 16.12 | 0.61 | 4.01 | 0.45 |
| 556.936002 | 510.0000 | 0.159 | 32.10 | 0.69 | 4.11 | 1.00 |
| 620.700807 | 5.0900 | 2.200 | 24.38 | 0.71 | 4.68 | 0.68 |
| 658.006500 | 0.2740 | 7.820 | 32.10 | 0.69 | 4.14 | 1.00 |
| 752.033227 | 250.0000 | 0.396 | 30.60 | 0.68 | 4.09 | 0.84 |
| 841.073593 | 0.0130 | 8.180 | 15.90 | 0.33 | 5.76 | 0.45 |
| 859.865000 | 0.1330 | 7.989 | 30.60 | 0.68 | 4.09 | 0.84 |
| 899.407000 | 0.0550 | 7.917 | 29.85 | 0.68 | 4.53 | 0.90 |
| 902.555000 | 0.0380 | 8.432 | 28.65 | 0.70 | 5.10 | 0.95 |
| 906.205524 | 0.1830 | 5.111 | 24.08 | 0.70 | 4.70 | 0.53 |
| 916.171582 | 8.5600 | 1.442 | 26.70 | 0.70 | 4.78 | 0.78 |
| 970.315022 | 9.1600 | 1.920 | 25.50 | 0.64 | 4.94 | 0.67 |
| 987.926764 | 138.0000 | 0.258 | 29.85 | 0.68 | 4.55 | 0.90 |

The dry air continuum arises from the non-resonant Debye spectrum of oxygen below 10 GHz and a pressure-induced nitrogen attenuation above 100 GHz.

ND} }  \( f \) ``=``f`p`θ rSup { size 8{2} } ` left [ {  {6 . 14`` times ``10 rSup { size 8{–5} } }  over  {d left [1``+`` left ( {  {f}  over  {d} }  right ) rSup { size 8{2} }  right ]} } ``+``1 . 4`` times ``10 rSup { size 8{–12} } ` \( 1`` - ``1 . 2`` times ``10 rSup { size 8{–5} } `f rSup { size 8{1 . 5} }  \) `p`θ rSup { size 8{1 . 5} }  right ]} {	(8)

where d is the width parameter for the Debye spectrum:

d=5.6 × 10–4(p+1.1 e) θ	(9)

The wet continuum, NW} }  \( f \) } { is included to account for the fact that measurements of water-vapour attenuation are generally in excess of those predicted using the theory described by equations (2) to (7), plus a term to include the effects of higher-frequency water-vapour lines not included in the reduced line base:

NW} }  \( f \) =f \( 3 . 57`θ rSup { size 8{7 . 5} } `e`+`0 . 113`p \) `10 rSup { size 8{–7} } `e`θ rSup { size 8{3} } } {	(10)

# 2	Path attenuation

## 2.1	Terrestrial paths

For a terrestrial path, or for slightly inclined paths close to the ground, the path attenuation, A, may be written as:

A=γ r0=γo+γw r0dB	(11)

where r0 is path length (km).

## 2.2	Slant paths

This section gives a method to integrate the specific attenuation calculated using the line-by-line model given above, at different pressures, temperatures and humidities through the atmosphere. By this means, the path attenuation for communications systems with any geometrical configuration within and external to the Earth's atmosphere may be accurately determined simply by dividing the atmosphere into horizontal layers, specifying the profile of the meteorological parameters pressure, temperature and humidity along the path. In the absence of local profiles, from radiosonde data, for example, the reference standard atmospheres in Recommendation ITU-R P.835 may be used, either for global application or for low (annual), mid (summer and winter) and high latitude (summer and winter) sites.

Figure 3 shows the zenith attenuation calculated at 1 GHz intervals with this model for the global reference standard atmosphere in Recommendation ITU-R P.835, with horizontal layers 1 km thick and summing the attenuations for each layer, for the cases of a moist atmosphere (Curve A) and a dry atmosphere (Curve B).

The total slant path attenuation, A(h, ), from a station with altitude, h, and elevation angle, , can be calculated as follows when   0:

Ah, ϕ =  γ Hsin Φ dH 	(12)

where the value of  can be determined as follows based on Snell's law in polar coordinates:

Φ=arccos c(r+H) × n(H)	(13)

where:

c=(r+h) × n(h) × cos ϕ	(14)

where n(h) is the atmospheric radio refractive index, calculated from pressure, temperature and water-vapour pressure along the path (see Recommendation ITU-R P.835) using Recommendation ITU-R P.453.

On the other hand, when   0, there is a minimum height, hmin, at which the radio beam becomes parallel with the Earth's surface. The value of hmin can be determined by solving the following transcendental equation:

r+hmin × nhmin=c	(15)

This can be easily solved by repeating the following calculation, using hmin  h as an initial value:

h'min = cnhmin – r	(16)

Therefore, A(h, ) can be calculated as follows:

Ah, ϕ=γ Hsin Φ dH+ γ Hsin Φ dH	(17)

In carrying out the integration of equations (12) and (17), care should be exercised in that the integrand becomes infinite at   0. However, this singularity can be eliminated by an appropriate variable conversion, for example, by using u4  H – h in equation (12) and u4  H – hmin in equation (17).

A numerical solution for the attenuation due to atmospheric gases can be implemented with the following algorithm.

To calculate the total attenuation for a satellite link, it is necessary to know not only the specific attenuation at each point of the link but also the length of path that has that specific attenuation. To derive the path length it is also necessary to consider the ray bending that occurs in a spherical Earth.

Using Fig. 4 as a reference, an is the path length through layer n with thickness n that has refractive index nn. n and n are the entry and exiting incidence angles. rn are the radii from the centre of the Earth to the beginning of layer n. an can then be expressed as:

an=− rn cos βn+12 4 rn2 cos2  βn+8 rn  δn+4 δn2	(18)

The angle n can be calculated from:

αn=π−arc cos −an2−2 rn δn−δn22 an rn+2 an δn	(19)

1 is the incidence angle at the ground station (the complement of the elevation angle ). n  1 can be calculated from n using Snell's law that in this case becomes:

βn+1=arcsin nnnn+1 sin αn	(20)

where nn and nn  1 are the refractive indexes of layers n and n  1.

The remaining frequency dependent (dispersive) term has a marginal influence on the result (around 1%) but can be calculated from the method shown in the ITU-R Handbook on Radiometeorology.

The total attenuation can be derived using:

Agas=n = 1kan γn dB	(21)

where n is the specific attenuation derived from equation (1).

To ensure an accurate estimate of the path attenuation, the thickness of the layers should increase exponentially, from 10 cm at the lowest layer (ground level) to 1 km at an altitude of 100 km, according to the following equation:

δi = 0.0001 exp i − 1100 km	(22)

from i  1 to 922, noting that 922  1.0 km and  δi≃100 km.

For Earth-to-space applications, the integration should be performed at least up to 30 km, and up to 100 km at the oxygen line-centre frequencies.

# 3	Dispersive effects

The effects of dispersion are discussed in the ITU-R Handbook on Radiometeorology, which contains a model for calculating dispersion based on the line-by-line calculation. For practical purposes, dispersive effects should not impose serious limitations on millimetric terrestrial communication systems operating with bandwidths of up to a few hundred MHz over short ranges (for example, less than about 20 km), especially in the window regions of the spectrum, at frequencies removed from the centres of major absorption lines. For satellite communication systems, the longer path lengths through the atmosphere will constrain operating frequencies further to the window regions, where both atmospheric attenuation and the corresponding dispersion are low.

ANNEX  2

Approximate estimation of gaseous attenuation
in the frequency range 1-350 GHz

This Annex contains simplified algorithms for quick, approximate estimation of gaseous attenuation for a limited range of meteorological conditions and a limited variety of geometrical configurations.

# 1	Specific attenuation

The specific attenuation due to dry air and water vapour, from sea level to an altitude of 5 km, can be estimated using the following simplified algorithms, which are based on curve-fitting to the line-by-line calculation, and agree with the more accurate calculations to within an average of about 15% at frequencies removed from the centres of major absorption lines. The absolute difference between the results from these algorithms and the line-by-line calculation is generally less than 0.1 dB/km and reaches a maximum of 0.7 dB/km near 60 GHz. For altitudes higher than 5 km, and in cases where higher accuracy is required, the line-by-line calculation should be used.

For dry air, the attenuation o (dB/km) is given by:

γo= 7.34 rp2 rt3f2+ 0.36 rp2 rt2 + 0.3429 b γo'(54)(54–f)a+bf2 × 10–3	(22a)

for  f    54 GHz

γo=exp [54–N ln(γo(54)) (f – 57) (f – 60) (f – 63) (f – 66)1944 –57–N ln(γo(57)) (f – 54) (f – 60) (f – 63) (f – 66)486 +60–N ln(γo(60)) (f – 54) (f – 57) (f – 63) (f – 66)324 –63–N ln(γo(63)) (f – 54) (f – 57) (f – 60) (f – 66)486 +66–N ln(γo(66)) (f – 54) (f – 57) (f – 60) (f – 63)1944]fN	(22b)

for  54 GHz    f    66 GHz

γo=0.2296 d γo' (66)(f–66)c+d + 0.286 rp2 rt3.8(f–118.75)2+2.97 rp2 rt1.6f2 × 10–3	(22c)

for  66 GHz    f    120 GHz

γo=3.02 × 10–4 rp2 rt3.5 + 1.5827 rp2 rt3(f–66)2 + 0.286 rp2 rt3.8(f–118.75)2+2.97 rp2 rt1.6f2 × 10–3	(22d)

for  120 GHz    f    350 GHz

with:

γo' (54)=2.128 rp1.4954 rt–1.6032 exp –2.5280 (1–rt)	(22e)

γo (54)=2.136 rp1.4975 rt–1.5852 exp –2.5196 (1–rt)	(22f)

γo (57)=9.984 rp0.9313 rt2.6732 exp 0.8563 (1–rt)	(22g)

γo (60)=15.42 rp0.8595 rt3.6178 exp 1.1521 (1–rt)	(22h)

γo (63)=10.63 rp0.9298 rt2.3284 exp 0.6287 (1–rt)	(22i)

γo (66)=1.944 rp1.6673 rt–3.3583 exp –4.1612 (1–rt)	(22j)

γo' (66)=1.935 rp1.6657 rt–3.3714 exp –4.1643 (1–rt)	(22k)

a=ln(η2η1)/ln 3.5	(22l)

b=4aη1	(22m)

η1=6.7665 rp–0.5050 rt0.5106 exp 1.5663 (1–rt)–1	(22n)

η2=27.8843 rp–0.4908 rt0.8491 exp 0.5496 (1–rt)–1	(22o)

c=ln(ξ2ξ1)/ln 3.5	(22p)

d=4cξ1	(22q)

ξ1=6.9575 rp–0.3461 rt0.2535 exp 1.3766 (1–rt)–1	(22r)

ξ2=42.1309 rp–0.3068 rt1.2023 exp 2.5147 (1–rt)–1	(22s)

N=0 for f≤60 GHz and N=–15 for f>60 GHz

where:

f :	frequency (GHz)

rp  	p / 1013

rt  	288/(273  t)

p :	pressure (hPa)

t :	temperature (C), where mean temperature values can be obtained from maps given in Recommendation ITU-R P.1510, when no adequate temperature data are available.

For water vapour, the attenuation w (dB/km) is given by:

γw=3.13 × 10–2 rp rt2+1.76 × 10–3 ρ rt8.5+rt2.5[3.84 ξw1 g22 exp (2.23 (1–rt))(f–22.235)2+9.42 ξw12 +10.48 ξw2 exp (0.7 (1–rt))(f–183.31)2+9.48 ξw22 + 0.078 ξw3 exp (6.4385 (1–rt))(f–321.226)2+6.29 ξw32 +3.76 ξw4 exp (1.6 (1–rt))(f–325.153)2+9.22 ξw42 + 26.36 ξw5 exp (1.09 (1–rt))(f–380)2 +17.87 ξw5 exp (1.46 (1–rt))(f–448)2 + 883.7 ξw5 g557 exp (0.17 (1–rt))(f–557)2 +302.6 ξw5 g752 exp (0.41 (1–rt))(f–752)2]f2 ρ × 10–4	(23a)

for  f    350 GHz

with:

ξw1=0.9544 rp rt0.69+0.0061 ρ	(23b)

ξw2=0.95 rp rt0.64+0.0067 ρ	(23c)

ξw3=0.9561 rp rt0.67+0.0059 ρ	(23d)

ξw4=0.9543 rp rt0.68+0.0061 ρ	(23e)

ξw5=0.955 rp rt0.68+0.006 ρ	(23f)

g22=1+(f–22.235)2 (f+22.235)2	(23g)

g557=1+(f–557)2 (f+557)2	(23h)

g752=1+(f–752)2 (f+752)2	(23i)

where  is the water-vapour density (g/m3).

Figure 5 shows the specific attenuation from 1 to 350 GHz at sea-level for dry air and water vapour with a density of 7.5 g/m3.

# 2	Path attenuation

## 2.1	Terrestrial paths

For a horizontal path, or for slightly inclined paths close to the ground, the path attenuation, A, may be written as:

A=γ r0=(γo+γw) r0dB	(24)

where r0 is the path length (km).

## 2.2	Slant paths

This section contains simple algorithms for estimating the gaseous attenuation along slant paths through the Earth's atmosphere, by defining an equivalent height by which the specific attenuation calculated in § 1 may be multiplied to obtain the zenith attenuation. The resulting zenith attenuations are accurate to within ±10% from sea level up to altitudes of about 2 km, using the pressure, temperature and water-vapour density appropriate to the altitude of interest. For altitudes higher than 2 km, and for frequencies within 0.5 GHz of the centres of resonance lines at any altitude, the procedure in Annex 1 should be used. The path attenuation at elevation angles other than the zenith may then be determined using the procedures described later in this section. For dry air, the equivalent height is given by:

ho=5.386–3.32734 × 10–2 f+1.87185 × 10–3 f2–3.52087 × 10–5 f3+83.26(f–60)2+1.2 km	(25a)

for  1 GHz    f    56.7 GHz

ho = 10 km	for  56.7 GHz    f    63.3 GHz	(25b)

ho=f0.039581–1.19751 × 10–3 f+9.14810 × 10–6 f21–0.028687 f+2.07858 × 10–4 f2 + 90.6(f–60)2 km	(25c)

for  63.3 GHz    f    98.5 GHz

ho=5.542–1.76414 × 10–3 f+3.05354 × 10–6 f2 + 6.815(f–118.75)2+0.321 km	(25d)

for  98.5 GHz    f    350 GHz

and for water-vapour, the equivalent height is:

hw=1.651 + 1.61(f–22.23)2+2.91 + 3.33(f–183.3)2+4.58 + 1.90(f–325.1)2+3.34 km	(26)

for  f    350 GHz

The zenith attenuation between 50 to 70 GHz is a complicated function of frequency, as shown in Fig. 7, and the above algorithms for equivalent height can provide only an approximate estimate, in general, of the minimum levels of attenuation likely to be encountered in this frequency range. For greater accuracy, the procedure in Annex 1 should be used.

The concept of equivalent height is based on the assumption of an exponential atmosphere specified by a scale height to describe the decay in density with altitude. Note that scale heights for both dry air and water vapour may vary with latitude, season and/or climate, and that water vapour distributions in the real atmosphere may deviate considerably from the exponential, with corresponding changes in equivalent heights. The values given above are applicable up to an altitude of 2 km.

The total zenith attenuation is then:

A=γo ho+γw hwdB	(27)

Figure 6 shows the total zenith attenuation at sea level, as well as the attenuation due to dry air and water vapour, using the mean annual global reference atmosphere given in Recommendation

ITU-R P.835. Between 50 and 70 GHz greater accuracy can be obtained from the 0 km curve in Fig. 7 which was derived using the line-by-line calculation as described in Annex 1.

### 2.2.1	Elevation angles between 5 and 90

#### 2.2.1.1	Earth-space paths

For an elevation angle, , between 5 and 90°, the path attenuation is obtained using the cosecant law, as follows:

For path attenuation based on surface meteorological data:

A=A+Awsinϕ dB	(28)

whereAo=ho γo and Aw=hw γw

and for path attenuation based on integrated water vapour content:

A(p)=Ao+Aw(p)sin ϕ dB	(29)

where  Aw(p) is given in § 2.3.

#### 2.2.1.2	Inclined paths

To determine the attenuation values on an inclined path between a station situated at altitude h1 and another at a higher altitude h2, where both altitudes are less than 2 000 m above mean sea level, the values ho and hw in equation (28) must be replaced by the following ho' and hw' values:

ho'=ho e–h1ho–e–h2ho km	(30)

hw'=hw e–h1hw–e–h2hw km	(31)

it being understood that the value  of the water vapour density used in equation (23) is the hypothetical value at sea level calculated as follows:

ρ=ρ1 × exp h12	(32)

where 1 is the value corresponding to altitude h1 of the station in question, and the equivalent height of water vapour density is assumed as 2 km (see Recommendation ITU-R P.835).

Equations (30), (31) and (32) use different normalizations for the dry air and water vapour equivalent heights. While the mean air pressure referred to sea level can be considered constant around the world (equal to 1 013 hPa), the water vapour density not only has a wide range of climatic variability but is measured at the surface (i.e. at the height of the ground station). For values of surface water vapour density, see Recommendation ITU-R P.836.

### 2.2.2	Elevation angles between 0º and 5º

#### 2.2.2.1	Earth-space paths

In this case, Annex 1 of this Recommendation should be used. The same Annex should also be used for elevations less than zero.

#### 2.2.2.2	Inclined paths

The attenuation on an inclined path between a station situated at altitude h1 and a higher altitude h2 (where both altitudes are less than 2 000 m above mean sea level), can be determined from the following:

A=γo ho Re+h1⋅F(x1) e–h1hocos ϕ – Re+h2F(x2) e–h2hocos ϕ +γw hw Re+h1⋅F(x1') e–h1hwcos ϕ – Re+h2F(x2') e–h2hwcos ϕ dB	(33)

where:

Re :	effective Earth radius including refraction, given in Recommendation ITU-R P.834, expressed in km (a value of 8 500 km is generally acceptable for the immediate vicinity of the Earth's surface)

1 :	elevation angle at altitude h1

F :	function defined by:

F(x) = 10.661 x + 0.339 x2 + 5.51	(34)

ϕ2=arccos Re+h1Re+h2 cos ϕ1	(35a)

xi=tan ϕi Re+hiho for i=1, 2	(35b)

xi'=tan ϕi Re+hihw for i=1, 2	(35c)

it being understood that the value  of the water vapour density used in equation (23) is the hypothetical value at sea level calculated as follows:

ρ=ρ1 ⋅ exp h12	(36)

where 1 is the value corresponding to altitude h1 of the station in question, and the equivalent height of water vapour density is assumed as 2 km (see Recommendation ITU-R P.835).

Values for 1 at the surface can be found in Recommendation ITU-R P.836. The different formulation for dry air and water vapour is explained at the end of § 2.2.

## 2.3	Slant path water-vapour attenuation

The above method for calculating slant path attenuation by water vapour relies on the knowledge of the profile of water-vapour pressure (or density) along the path. In cases where the integrated water vapour content along the path, Vt, is known, an alternative method may be used. The total water-vapour attenuation in the zenith direction can be expressed as:

Aw(p)=Vt(p) ⋅ γw(ρ)ρ dB	(37)

where Vt ( p) (kg/m2 or mm),  (g/m3) and w (dB/km) are respectively the integrated water vapour content at the required percentage of time p, the mean annual surface water-vapour density and the specific attenuation.

The specific attenuation w can be calculated using equation (23a).

Values for the integrated water vapour content Vt ( p) can be obtained either from radiosonde profiles or radiometric measurements. Statistics of Vt ( p) and  are given in Recommendation ITU-R P.836. For elevation angles other than the zenith, the attenuation must be divided by sin , where  is the elevation angle, assuming a uniform horizontally-stratified atmosphere, down to elevation angles of about 5°.