RECOMMENDATION  ITU-R  PN.681-1*

PROPAGATION  DATA  REQUIRED  FOR  THE  DESIGN  OF  EARTH-SPACE
LAND  MOBILE  TELECOMMUNICATION  SYSTEMS

(Question ITU-R 207/3)

(1990-1994)

Rec. ITU-R PN.681-1

The ITU Radiocommunication Assembly,

considering

a)	that for the proper planning of Earth-space land mobile systems it is necessary to have appropriate propagation data and prediction methods;

b)	that the methods of Recommendation ITU-R PN.618 are recommended for the planning of Earth-space telecommunication systems;

c)	that further development of prediction methods for specific application to land mobile-satellite systems is required to give adequate accuracy in all regions of the world and for all operational conditions;

d)	that, however, methods are available which yield sufficient accuracy for many applications,

recommends

1.	that the methods contained in Annex 1 be adopted for use in the planning of Earth-space land mobile telecommunications systems, in addition to the methods recommended in Recommendation ITU-R PN.618.

ANNEX  1

# 1.	Introduction

Propagation effects in the land mobile-satellite service (LMSS) differ from those of the fixed-satellite service (FSS) primarily because of the greater importance of terrain effects. In the FSS it is generally possible to discriminate against multipath, shadowing and blockage through the use of highly directive antennas placed at unobstructed sites. Therefore, in general, the LMSS offers smaller link availability percentages than the FSS. The prime availability range of interest to system designers is usually from 80% to 99%.

This Annex deals with data and models specifically needed for predicting propagation impairments in LMSS links, which include tropospheric effects, ionospheric effects, multipath, blockage and shadowing. It is based on measurements at 1.5 GHz (L-band) and 870 MHz in the UHF band.

# 2.	Tropospheric effects

## 2.1	Attenuation

Signal losses in the troposphere are caused by atmospheric gases, rain, fog and clouds. Except at low elevation angles, tropospheric attenuation is negligible at frequencies below about 1 GHz, and is generally small at frequencies up to about 10 GHz. Above 10 GHz, the attenuation can be large for significant percentages of the time on many paths. Prediction methods are available for estimating gaseous absorption (Recommendation ITU-R PN.676) and rain attenuation (Recommendation ITU-R PN.618). Fog and cloud attenuation is usually negligible for frequencies up to 10 GHz.

## 2.2	Scintillation

Irregular variations in received signal level and in angle of arrival are caused by both tropospheric turbulence and atmospheric multipath. The magnitudes of these effects increase with increasing frequency and decreasing path elevation angle, except that angle-of-arrival fluctuations caused by turbulence are independent of frequency. Antenna beamwidth also affects the magnitude of these scintillations. These effects are observed to be at a maximum in the summer season. A prediction method is given in Recommendation ITU-R PN.618.

# 3.	Ionospheric effects

Ionospheric effects on Earth-to-space paths are addressed in Recommendation ITU-R PI.531. Values of ionospheric effects for frequencies in the range of 0.1 to 10 GHz are given in Tables 1 and 2 of Recommendation ITU-R PN.680.

# 4.	Shadowing

## 4.1	Empirical roadside shadowing model

Cumulative 1.5 GHz fade distribution measurements have given rise to the empirical roadside shadowing model. The extent of trees along the roadside is represented by the percentage of optical shadowing caused by roadside trees at a path elevation angle of 45° in the direction of the signal source. The model is valid when this percentage is in the range 55-75%.

A (q, p)  =  a ( p)  +  b ( p) q  +  g ( p) q2	(1)

for

20°  £  q  £  60°

1%  £  p  £  20%

where:

A:	fade exceeded in dB with respect to free space propagation

p:	percentage of the distance travelled over which the fade is exceeded

q:	path elevation angle to the satellite.

The parameters, a( p), b( p), and g ( p) are tabulated in Table 1.

TABLE  1

Values a(p), b(p), and g (p) of the empirical roadside model

| Percentage (p) | a(p) | b(p) | g (p) |
| --- | --- | --- | --- |

| 20 | 24.45 | –0.7351 | 5.991 × 10-3 |
| --- | --- | --- | --- |

| 10 | 26.84 | –0.6775 | 4.605 × 10-3 |
| --- | --- | --- | --- |

| 05 | 29.22 | –0.6000 | 3.219 × 10-3 |
| --- | --- | --- | --- |

| 02 | 32.38 | –0.5106 | 1.386 × 10-3 |
| --- | --- | --- | --- |

| 01 | 34.76 | –0.4430 | 0.0 |
| --- | --- | --- | --- |

The empirical roadside model corresponds to an average propagation condition with the vehicle driving on lanes both sides of the highway (lanes close to and far from roadside trees are included). The model applies to highways and rural roads where the overall aspect of the propagation path is, for the most part, orthogonal to the lines of roadside trees and utility poles and it is assumed that the dominant cause of LMSS signal attenuation is canopy shadowing (see Recommendation ITU-R PN.833).

Figure 1 shows plots of fade exceeded versus the path elevation angle for several constant percentages, p.

FIGURE 1...[D01] = 3 CM

Since the measurements that resulted in the development of the model did not discriminate against multipath, it implicitly includes the effect of multipath by roadside trees. The receiving antenna employed in the measurements has an azimuthally omnidirectional radiation pattern with a peak gain of 4 dB.

Fade measurements in the United Kingdom for a mixture of trees and buildings have shown that the model is also valid for mixed environments and that the model may be extended to elevation angles up to 80°.

## 4.2	Attenuation frequency scaling model

Mobile fade measurements of roadside tree shadowing have shown that the ratio of fades at equal probability values is approximately consistent with the ratio of the square root of frequencies, f1 and f2:

(2)

for

0.8 GHz  £  f1 and f2  £  2.7 GHz

1%  £  p  £  30%

30°  £  q  £  60°

where:

A:	fade exceeded with respect to free space propagation (dB)

q:	elevation angle.

## 4.3	Fade duration distribution model

Optimal design of land mobile satellite receivers depends on knowledge of the statistics associated with fade durations, which can be represented in units of travelled distance, m, or in seconds. Fade duration measurements have given rise to the following empirical model which is valid for distance fade duration dd ³ 0.02 m.

P (FD  >  dd  |  A  >  Aq)  =  	(3)

where P(FD > dd ½ A > Aq) represents the probability that the distance fade duration, FD, exceeds the distance, dd m, under the condition that the attenuation, A, exceeds Aq. The designation “erf” represents the error function, s is the standard deviation of ln(dd), and ln(a) is the mean value of ln(dd). The left-hand side of equation (3) was estimated by computing the percentage number of “duration events” that exceed dd relative to the total number of events for which A > Aq in data obtained from measurements in the United States of America and Australia. The best fit regression values obtained from these measurements are a = 0.22 and s = 1.215.

Figure 2 contains a plot of p, expressed as a percentage, versus dd for a 5 dB threshold.

# 4.4	Non-fade duration distribution model

A “non-fade duration” event of distance duration, dd, is defined as the distance over which the fade levels are smaller than a specified fade threshold. The non-fade duration model is given by:

P ( NFD  >  dd  |  A  <  Aq )  =  b ( dd ) – g	(4)

where P(NFD > dd ½ A < Aq) is the percentage probability that a continuous non-fade distance, NFD, exceeds the distance, dd, given that the fade is smaller than the threshold, Aq. Table 2 contains the values of b and g for roads that exhibit “moderate and extreme” shadowing as defined in § 4.1. A 5 dB fade threshold is used.

FIGURE 2...[D02] = 3 CM

TABLE  2

Non-fade duration regression values for a 5 dB fade
threshold at a path elevation angle of 51°

| Shadowing level | b | g |
| --- | --- | --- |

| Moderate | 20.54 | 0.5800 |
| --- | --- | --- |

| Extreme | 11.71 | 0.8371 |
| --- | --- | --- |

# 5.	Multipath models for clear line-of-sight conditions

In many cases the mobile terminal has a clear line-of-sight (negligible shadowing) to the mobile satellite. Degradation to the signal can still occur under these circumstances, due to terrain-induced multipath. The mobile terminal receives a phasor summation of the direct line-of-sight signal and several multipath signals. These multipath signals may add constructively or destructively to result in signal enhancement or fade. The multipath signal characteristics depend on the scattering cross-sections of the multipath reflectors, their number, the distances to the receiving antenna, the field polarizations, and receiving antenna gain pattern.

The multipath degradation models introduced in the following sections are based on measurements made using an antenna with the following characteristics:

–	omnidirectional in azimuth;

–	gain variation between 15° and 75° elevation less than 3 dB;

–	below the horizon (negative elevation angles) the antenna gain was reduced by at least 10 dB.

## 5.1	Multipath in a mountain environment

The distribution of fade depths due to multipath in mountainous terrain is modelled by:

p  =  a A – b	(5)

for:

1%  <  p  <  10%

where:

p:	percentage of distance over which the fade is exceeded, and

A:	fade exceeded (dB).

The curve fit parameters, a and b, are shown in Table 3 for 1.5 GHz and 870 MHz. Note that the above model is valid when the effect of shadowing is negligible.

TABLE  3

Parameters for best fit cumulative fade distribution for
multipath in mountainous terrain

| Frequency | Elevation = 30° |  | Elevation = 45° |  |
| --- | --- | --- | --- | --- |
| (GHz) | a | b | a | b |
| 0.870 | 34.52 | 1.855 | 31.64 | 2.464 |
| 1.500 | 33.19 | 1.710 | 39.95 | 2.321 |

Figure 3 contains curves of the cumulative fade distributions for path elevation angles of 30° and 45° at 1.5 GHz and 870 MHz.

FIGURE 3...[D03] = 3 CM

## 5.2	Multipath in a roadside tree environment

Experiments conducted along tree-lined roads in the United States of America have shown that multipath fading is relatively insensitive to path elevation over the range of 30°-60°. The measured data have given rise to the following model:

p  =  u exp (– v A)	(6)

for

1%  <  p  <  50%

where:

p:	percentage of distance over which the fade is exceeded and

A:	fade exceeded (dB).

Note that the above model assumes negligible shadowing. The curve fit parameters, u and v, are shown in Table 4.

TABLE  4

Parameters for best exponential fit cumulative fade
distributions for multipath for tree-lined roads

| Frequency (GHz) | u | v | Fade range (dB) |
| --- | --- | --- | --- |

| 0.870 | 125.6 | 1.1160 | 1-4.5 |
| --- | --- | --- | --- |

| 1.500 | 127.7 | 0.8573 | 1-6.0 |
| --- | --- | --- | --- |

Figure 4 contains curves of the cumulative fade distributions for 1.5 GHz and 870 MHz. Enhanced fading due to multipath can occur at lower elevation angles (5° to 30°) where forward scattering from relatively smooth rolling terrain can be received from larger distances.

FIGURE 4...[D04] = 3 CM

_________________

*	This Recommendation should be brought to the attention of Radiocommunication Study Group 8.