Readme File for the Coefficients of the Excess Path Length Parameters along an Earth-space link

These coefficients are also applicable to Rec. ITU-R P.834-5 to -9.

These data files are to be used in equations 22 to 25 of Annex 1.

Notation of names of the data files

<met_param>_gd_<mod_par>.dat .

Where:

<met_param> indicates one of the following physical parameters:

pres = air total pressure at the Earth's surface, ps,

vapr = water vapour partial pressure at the Earth's surface, es,

tmpm = mean temperature of the water vapour column above the surface, Tms

lamd = vapour pressure decrease factor. ,

alfm = lapse rate of the mean temperature of water vapour from Earth surface, m

<mod_par> indicates one of the following model parameters

a1 = average value of the parameter

a2 = seasonal fluctuation of the parameter

a3 = day (of the year) of the minimum value of the parameter

In addition,

hreflev.dat = average height of reference level with respect to the mean sea level (m), href

Each data file and the hreflev.dat file contains 121 rows corresponding to latitudes from 90 to -90 degrees, and 241 columns corresponding to longitudes from 0 to 360 degrees in 1.5 degree increments.

Additional files are:

- lat.dat = reference latitude values from 90 deg to -90 deg in 1.5 deg increments

- long.dat = reference longitude values from 0 deg to 360 deg in 1.5 deg increments