RECOMMENDATION  ITU-R  P.838-1

SPECIFIC  ATTENUATION  MODEL  FOR  RAIN  FOR  USE  IN  PREDICTION  METHODS

(Question ITU-R 201/3)

(1992-1999)

Rec. ITU-R P.838-1

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to calculate the attenuation due to rain from a knowledge of rain rates,

recommends

1	that the following procedure be used.

Specific attenuation R (dB/km) is obtained from the rain rate R (mm/h) using the power-law relationship:

(1)

The frequency-dependent coefficients k and  are given in Table 1 for linear polarizations (horizontal: H, vertical: V) and horizontal paths. Values of k and  at frequencies other than those in Table 1 can be obtained by interpolation using a logarithmic scale for frequency, a logarithmic scale for k and a linear scale for .

The values in Table 1 have been tested and found to be sufficiently accurate for attenuation prediction up to frequencies of 55 GHz.

For linear and circular polarization, and for all path geometries, the coefficients in equation (1) can be calculated from the values in Table 1 using the following equations:

(2)

(3)

where  is the path elevation angle and  is the polarization tilt angle relative to the horizontal (  45 for circular polarization).

TABLE  1

Regression coefficients for estimating specific attenuation in equation (1)

| Frequency (GHz) | kH | kV | H | V |
| --- | --- | --- | --- | --- |
| 1 | 0.0000387 | 0.0000352 | 0.912 | 0.880 |
| 2 | 0.000154 | 0.000138 | 0.963 | 0.923 |
| 4 | 0.000650 | 0.000591 | 1.121 | 1.075 |
| 6 | 0.00175 | 0.00155 | 1.308 | 1.265 |
| 7 | 0.00301 | 0.00265 | 1.332 | 1.312 |
| 8 | 0.00454 | 0.00395 | 1.327 | 1.310 |
| 10 | 0.0101 | 0.00887 | 1.276 | 1.264 |
| 12 | 0.0188 | 0.0168 | 1.217 | 1.200 |
| 15 | 0.0367 | 0.0335 | 1.154 | 1.128 |
| 20 | 0.0751 | 0.0691 | 1.099 | 1.065 |
| 25 | 0.124 | 0.113 | 1.061 | 1.030 |
| 30 | 0.187 | 0.167 | 1.021 | 1.000 |
| 35 | 0.263 | 0.233 | 0.979 | 0.963 |
| 40 | 0.350 | 0.310 | 0.939 | 0.929 |
| 45 | 0.442 | 0.393 | 0.903 | 0.897 |
| 50 | 0.536 | 0.479 | 0.873 | 0.868 |
| 60 | 0.707 | 0.642 | 0.826 | 0.824 |
| 70 | 0.851 | 0.784 | 0.793 | 0.793 |
| 80 | 0.975 | 0.906 | 0.769 | 0.769 |
| 90 | 1.06 | 0.999 | 0.753 | 0.754 |
| 100 | 1.12 | 1.06 | 0.743 | 0.744 |
| 120 | 1.18 | 1.13 | 0.731 | 0.732 |
| 150 | 1.31 | 1.27 | 0.710 | 0.711 |
| 200 | 1.45 | 1.42 | 0.689 | 0.690 |
| 300 | 1.36 | 1.35 | 0.688 | 0.689 |
| 400 | 1.32 | 1.31 | 0.683 | 0.684 |