RECOMMENDATION  ITU-R  P.838-2

Specific attenuation model for rain for use in prediction methods

(Question ITU-R 201/3)

(1992-1999-2003)

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to calculate the attenuation due to rain from a knowledge of rain rates,

recommends

1	that the following procedure be used.

Specific attenuation R (dB/km) is obtained from the rain rate R (mm/h) using the power-law relationship:

γR=kRα	(1)

The frequency-dependent coefficients k and  are given in Table 1 for linear polarizations (horizontal: H, vertical: V) and horizontal paths.

The values in Table 1 have been tested and found to be sufficiently accurate for attenuation prediction up to frequencies of 55 GHz.

The coefficients k and  may alternatively be determined, as a function of frequency, from the following equations, which have been derived from curve-fitting to power-law coefficients derived from scattering calculations:

logk=j=13ajexp−logf−bjcj2+mklogf+ck	(2)

α=i=14aiexp−logf−bici2+mαlogf+cα	(3)

where:

f :	frequency (GHz)

k :	either kH or kV

 :	either H or V.

TABLE  1

Frequency dependent coefficients for estimating specific
attenuation using equations (4), (5) and (1)

| Frequency (GHz) | kH | kV | H | V |
| --- | --- | --- | --- | --- |
| 1 | 0.0000387 | 0.0000352 | 0.9122 | 0.8801 |
| 1.5 | 0.0000868 | 0.0000784 | 0.9341 | 0.8905 |
| 2 | 0.0001543 | 0.0001388 | 0.9629 | 0.9230 |
| 2.5 | 0.0002416 | 0.0002169 | 0.9873 | 0.9594 |
| 3 | 0.0003504 | 0.0003145 | 1.0185 | 0.9927 |
| 4 | 0.0006479 | 0.0005807 | 1.1212 | 1.0749 |
| 5 | 0.001103 | 0.0009829 | 1.2338 | 1.1805 |
| 6 | 0.001813 | 0.001603 | 1.3068 | 1.2662 |
| 7 | 0.002915 | 0.002560 | 1.3334 | 1.3086 |
| 8 | 0.004567 | 0.003996 | 1.3275 | 1.3129 |
| 9 | 0.006916 | 0.006056 | 1.3044 | 1.2937 |
| 10 | 0.01006 | 0.008853 | 1.2747 | 1.2636 |
| 12 | 0.01882 | 0.01680 | 1.2168 | 1.1994 |
| 15 | 0.03689 | 0.03362 | 1.1549 | 1.1275 |
| 20 | 0.07504 | 0.06898 | 1.0995 | 1.0663 |
| 25 | 0.1237 | 0.1125 | 1.0604 | 1.0308 |
| 30 | 0.1864 | 0.1673 | 1.0202 | 0.9974 |
| 35 | 0.2632 | 0.2341 | 0.9789 | 0.9630 |
| 40 | 0.3504 | 0.3104 | 0.9394 | 0.9293 |
| 45 | 0.4426 | 0.3922 | 0.9040 | 0.8981 |
| 50 | 0.5346 | 0.4755 | 0.8735 | 0.8705 |
| 60 | 0.7039 | 0.6347 | 0.8266 | 0.8263 |
| 70 | 0.8440 | 0.7735 | 0.7943 | 0.7948 |
| 80 | 0.9552 | 0.8888 | 0.7719 | 0.7723 |
| 90 | 1.0432 | 0.9832 | 0.7557 | 0.7558 |
| 100 | 1.1142 | 1.0603 | 0.7434 | 0.7434 |
| 120 | 1.2218 | 1.1766 | 0.7255 | 0.7257 |
| 150 | 1.3293 | 1.2886 | 0.7080 | 0.7091 |
| 200 | 1.4126 | 1.3764 | 0.6930 | 0.6948 |
| 300 | 1.3737 | 1.3665 | 0.6862 | 0.6869 |
| 400 | 1.3163 | 1.3059 | 0.6840 | 0.6849 |

The remaining coefficients are given in Tables 2 and 3.

TABLE  2

Coefficients in equations (2) and (3) for horizontal polarization

|  | a | b | c | mk | ck | m | c |
| --- | --- | --- | --- | --- | --- | --- | --- |
| j  1 | 0.3364 | 1.1274 | 0.2916 | 1.9925 | –4.4123 | – | – |
| 2 | 0.7520 | 1.6644 | 0.5175 |  |  |  |  |
| 3 | –0.9466 | 2.8496 | 0.4315 |  |  |  |  |
| i  1 | 0.5564 | 0.7741 | 0.4011 | – | – | –0.08016 | 0.8993 |
| 2 | 0.2237 | 1.4023 | 0.3475 |  |  |  |  |
| 3 | –0.1961 | 0.5769 | 0.2372 |  |  |  |  |
| 4 | –0.02219 | 2.2959 | 0.2801 |  |  |  |  |

TABLE  3

Coefficients in equations (2) and (3) for vertical polarization

|  | a | b | c | mk | ck | m | c |
| --- | --- | --- | --- | --- | --- | --- | --- |
| j  1 | 0.3023 | 1.1402 | 0.2826 | 1.9710 | –4.4535 | – | – |
| 2 | 0.7790 | 1.6723 | 0.5694 |  |  |  |  |
| 3 | –1.0022 | 2.9400 | 0.4823 |  |  |  |  |
| i  1 | 0.5463 | 0.8017 | 0.3657 | – | – | –0.07059 | 0.8756 |
| 2 | 0.2158 | 1.4080 | 0.3636 |  |  |  |  |
| 3 | –0.1693 | 0.6353 | 0.2155 |  |  |  |  |
| 4 | –0.01895 | 2.3105 | 0.2938 |  |  |  |  |

For linear and circular polarization, and for all path geometries, the coefficients in equation (1) can be calculated from the values in Table 1 using the following equations:

k = [kH+kV+(kH−kV)cos2 θ cos 2 τ]2	(4)

a =[kHaH + kVaV + kHaH – kVaV cos2 θ cos 2 τ] 2k	(5)

where  is the path elevation angle and  is the polarization tilt angle relative to the horizontal (  45 for circular polarization).

For convenience, a quick estimate of values of k and  at frequencies other than those in Table 1, can be obtained from Figs. 1 to 4.