RECOMMENDATION  ITU-R  P.839-1

RAIN  HEIGHT  MODEL  FOR  PREDICTION  METHODS

(Question ITU-R 201/3)

(1992-1997)

Rec. ITU-R P.839-1

The ITU Radiocommunication Assembly,

considering

a)	that knowledge of the height of the 0 C isotherm during rainy conditions is required in several prediction models used in the design of space telecommunication systems,

recommends

1	that the following model be used for the global estimation of h0 (mean 0 C isotherm height):

| h0 = | 5−0.075j−23555+0.1j+21 | for   –23 Northern Hemisphere for —0    –23 Northern Hemisphere for —0    –21 Southern Hemisphere for –71    –21 Southern Hemisphere for   –71 Southern Hemisphere |
| --- | --- | --- |

where h0 is in km above mean sea level and  is the latitude in degrees;

2	that for North America and Europe west of 60 E longitude the following model be used for the estimation of hFR (mean 0 C isotherm height during rainy conditions):

hFR    3.2  –  0.075 (  –  35)               for       35        70

where hFR is in km above ground;

3	that for other areas in the world, where no further information is available, hFR be equated to h0.

NOTE 1 – hFR, in general, is different from h0. The sign and magnitude of the difference is dependent on climatic conditions.