Recommendation ITU-R P.840-5 (02/2012)

Attenuation due to clouds and fog

P Series Radiowave propagation

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2012

ITU 2012

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  P.840-5

Attenuation due to clouds and fog

(Question ITU-R 201/3)

(1992-1994-1997-1999-2009-2012)

Scope

This Recommendation provides methods to predict the attenuation due to clouds and fog on Earth-space paths.

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to give guidance to engineers in the design of Earth-space telecommunication systems for frequencies higher than 10 GHz;

b)	that attenuation due to clouds may be a factor of importance especially for microwave systems well above 10 GHz or low-availability systems;

c)	that for the calculation of the time series of total attenuation and space-time prediction methods, an analytical expression for the statistics of the total columnar content of cloud liquid water is needed,

recommends

1	that the curves, models and maps given in Annex 1 should be used for the calculation of attenuation due to clouds and fog;

2	that the information in Annex 1 should be used for global calculations of propagation effects, required by, inter alia, space-time channel models, that require an analytic expression for the statistics of the total columnar content of cloud liquid water.

Annex 1

# 1	Introduction

For clouds or fog consisting entirely of small droplets, generally less than 0.01 cm, the Rayleigh approximation is valid for frequencies below 200 GHz and it is possible to express the attenuation in terms of the total water content per unit volume. Thus the specific attenuation within a cloud or fog can be written as:

dB/km	(1)

where:

c :	specific attenuation (dB/km) within the cloud;

Kl :	specific attenuation coefficient ((dB/km)/(g/m3));

M :	liquid water density in the cloud or fog (g/m3).

At frequencies of the order of 100 GHz and above, attenuation due to fog may be significant. The liquid water density in fog is typically about 0.05 g/m3 for medium fog (visibility of the order of 300 m) and 0.5 g/m3 for thick fog (visibility of the order of 50 m).

# 2	Specific attenuation coefficient

A mathematical model based on Rayleigh scattering, which uses a double-Debye model for the dielectric permittivity  ( f ) of water, can be used to calculate the value of Kl for frequencies up to 1 000 GHz:

(dB/km)/(g/m3)	(2)

where f is the frequency (GHz), and:

(3)

The complex dielectric permittivity of water is given by:

(4)

(5)

where:

0   77.6    103.3 (  –  1)	(6)

1   5.48	(7)

2   3.51	(8)

  300 / T	(9)

with T the temperature (K).

The principal and secondary relaxation frequencies are:

fp    20.09  –  142  (  –  1)    294 (  –  1)2	GHz	(10)

fs    590  –  1 500 (  –  1)	GHz	(11)

Figure 1 shows the values of Kl at frequencies from 5 to 200 GHz and temperatures between –8°C and 20°C. For cloud attenuations, the curve corresponding to 0°C should be used.

figure 1

Specific attenuation by water droplets at various
temperatures as function of frequency

# 3	Cloud attenuation

To obtain the attenuation due to clouds for a given probability, the statistics of the total columnar content of liquid water L (kg/m2) or, equivalently, mm of precipitable water for a given site must be known yielding:

dB     for   90    5	(12)

where  is the elevation angle and Kl is read from Fig. 1.

Statistics of the total columnar content of liquid water may be obtained from radiometric measurements or from radiosonde launches.

# 4	Total columnar content of cloud liquid water

The total columnar content of cloud liquid water, expressed in kg/m2 or, equivalently, in mm of liquid water, can be obtained from radiosonde soundings and radiometric measurements. Radiosonde data is widely available; however, it has limited time resolution and is only applicable to zenith paths. The total columnar content of cloud liquid water can be retrieved from radiometric measurements at appropriate frequencies along the desired path.

The annual values of total columnar content of cloud liquid water, L (kg/m2), exceeded for 0.1, 0.2, 0.3, 0.5, 1, 2, 3, 5, 10, 20, 30, 50, 60, 70, 80, 90, 95 and 99% of an average year are available in the form of digital maps from the Radiocommunication Study Group 3 website, in the data files ESAWRED_xx_v4.TXT, where xx  01, 02, 03, 05, 1, 2, 3, 5, 10, 20, 30, 50, 60, 70, 80, 90, 95 and 99. The data is from 0 to 360 in longitude and from 90 to –90 in latitude, with a resolution of 1.125º in both latitude and longitude. This data is to be used in conjunction with the companion data files ESALAT_1dot125.TXT and ESALON_1dot125.TXT containing the latitudes and longitudes of the corresponding entries (grid points) in data files ESAWRED_xx_v4.TXT. The total columnar content of cloud liquid water at any desired location on the surface of the Earth can be derived by the following interpolation method:

a)	determine the two probabilities, pabove and pbelow, above and below the desired probability, p, from the set: 0.1, 0.2, 0.3, 0.5, 1, 2, 3, 5, 10, 20, 30, 50, 60, 70, 80, 90, 95 and 99%;

b)	for the two probabilities, pabove and pbelow, determine the total columnar content of cloud liquid water, L1, L2, L3, and L4 at the four closest grid points;

c)	determine the total columnar content of cloud liquid water, Labove and Lbelow, at the probabilities, pabove and pbelow, by performing a bi-linear interpolation of the four values of total columnar content of cloud liquid water, L1, L2, L3, and L4 at the four grid points, as described in Recommendation ITU-R P.1144;

d)	determine the total columnar content of cloud liquid water, L, at the desired probability, p, by interpolating Labove and Lbelow vs. pabove and pbelow to p on a linear L vs. log p scale.

Note that the digital maps of total columnar content of cloud liquid water contain the symbol NaN (Not-a-Number) when there is no value of the total content of cloud liquid water corresponding to a given annual probability of exceedance.

Example contours of total columnar content of cloud liquid water are provided in Figures 2, 3, 4, 5, 6, and 7 for exceedance probabilities of 0.1, 0.5, 1, 5, 10 and 20%.

FIGURE 2

FIGURE 3

FIGURE 4

FIGURE 5

FIGURE 6

FIGURE 7

# 5	Statistical distribution of total columnar content of cloud liquid water

The statistics of the total columnar content of cloud liquid water can be approximated by a log-normal distribution. The mean, m, standard deviation, , and probability of liquid water, Pclw, parameters of the log-normal distribution are available in the form of digital maps from the Radiocommunication Study Group 3 website in the data files WRED_LOGNORMAL_MEAN_v4.TXT, WRED_LOGNORMAL_STDEV_v4.TXT, and WRED_LOGNORMAL_PCLW_v4.TXT. The data is from 0 to 360 in longitude and from 90 to –90 in latitude, with a resolution of 1.125º in both latitude and longitude. This data is to be used in conjunction with the companion data files ESALAT_1dot125.TXT and ESALON_1dot125.TXT containing the latitudes and longitudes of the corresponding entries (grid points) in data files WRED_LOGNORMAL_MEAN_v4.TXT, WRED_LOGNORMAL_STDEV_v4.TXT, and WRED_LOGNORMAL_PCLW_v4.TXT. The total columnar content of cloud liquid water at any desired location on the surface of the Earth can be derived by the following interpolation method:

a)	determine the parameters, m1, m2, m3, m4, 1, 2,3, 4, PCLW1, PCLW2, PCLW3 and PCLW4  at the four closest grid points;

b)	determine the total columnar content of cloud liquid water L1, L2, L3, and L4 for the desired probability, p, at the four closest grid points from the parameters m1, m2, m3, m4, 1, 2, 3, 4, PCLW1, PCLW2, PCLW3 and PCLW4 as follows:

for i = 1, 2, 3, 4	(13)

where:

(14)

c)	determine the total columnar content of cloud liquid water at the desired location by performing a bi-linear interpolation of the four values of total columnar content of cloud liquid water, L1, L2, L3, and L4 at the four grid points as described in Recommendation ITU-R P.1144.

Note that the digital maps of log-normal parameters contain the symbol NaN (Not-a-Number) when there is no value of total content of cloud liquid water corresponding to a given annual probability of exceedance in the digital maps of total columnar content of cloud liquid water.