RECOMMENDATION  ITU-R  PI.1059

METHOD  FOR  PREDICTING  SKY-WAVE  FIELD  STRENGTH
IN  THE  FREQUENCY  RANGE  1 605  TO  1 705  kHz

(Question ITU-R 215/3)

(1994)

Rec. ITU-R PI.1059

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to predict sky-wave field strengths in the broadcasting service at frequencies between 1 605 and 1 705 kHz;

b)	that portions of this band are being shared by broadcasting and other services in different Regions, and that an accurate method for predicting interference levels is needed to maintain efficient and orderly utilization of those portions of the spectrum;

c)	that the method given below is an extension of that developed by the ex-CCIR in its preparatory work for the Regional Administrative Radio Conference to Establish a Plan for the Broadcasting Service in the Band 1 605-1 705 kHz in Region 2 (Rio de Janeiro, 1988);

d)	that results of the most recent studies utilizing data from nearly 400 paths distributed throughout the world strongly indicate the method given below offers good results particularly when applied to long paths where the prediction heretofore has been very difficult,

recommends

that the following method be adopted for the prediction of sky-wave field strengths between 1 605 and 1 705 kHz.

# 1.	Introduction

This method predicts values of the night-time sky-wave field strength for a given power radiated from one or more vertical antennas, when measured by a loop antenna at ground level aligned in a vertical plane along the great-circle path to the transmitter. It applies for paths of length 50 to 12 000 km in the LF and MF bands.

# 2.	Annual median night-time field strength

The predicted sky-wave field strength is given by:

(1)

where:

E:	annual median of half-hourly median field strengths (dB(mV/m)) for a given transmitter cymomotive force, V, and at a given time, t, relative to sunset or sunrise as appropriate

E0:	annual median of half-hourly median field strengths (dB(mV/m)) for a transmitter cymomotive force of 300 V at the reference time defined in § 2.1

V:	transmitter cymomotive force (dB) above a reference cymomotive force of 300 V (see § 2.2)

GS:	sea-gain correction (dB) (see § 2.3)

Lp:	excess polarization-coupling loss (dB) (see § 2.4)

p:	slant-propagation distance (km) (see § 2.5)

k:	loss factor incorporating effects of ionospheric absorption, focusing and terminal losses, and losses between hops on multi-hop paths (see § 2.6)

Lt:	hourly loss factor (dB) (see § 2.7)

Lr:	loss factor incorporating effect of solar activity (see § 2.8).

## 2.1	Reference time

The reference time is taken as six hours after the time at which the Sun sets at a point S on the surface of the Earth. For paths shorter than 2 000 km, S is the mid-point of the path. On longer paths, S is 750 km from the terminal where the sun sets last, measured along the great-circle path.

## 2.2	Cymomotive force

The transmitter cymomotive force V (dB(300 V)) is given as:

(2)

where:

P:	radiated power (dB(1 kW))

G:	transmitting antenna gain factor (dB)

GV:	transmitting antenna gain factor (dB) due to vertical directivity, given in Fig. 1

GH:	transmitting antenna gain factor (dB) due to horizontal directivity. For directional antennas, GH is a function of azimuth. For omnidirectional antennas, GH = 0.

## 2.3	Sea gain

The sea gain GS is the additional signal gain when one or both terminals are situated near the sea, but it does not apply to propagation over fresh water. GS for a single terminal is given by:

(3)

(4)

where:

G0:	sea gain when the terminal is on the coast and the sea is unobstructed by further land (dB)

c1:	correction to take account of the distance between the terminal and the sea

c2:	correction to take account of the width of one or more sea channels, or the presence of islands.

If both terminals are near the sea, GS is the sum of the values for the individual terminals.

G0 is given in Fig. 2 as a function of d. G0 = 10 dB when d > 6 500 km, where d (km) is the ground distance between transmitter and receiver.

The correction c1 is given by:

(5)

where:

s1:	distance of terminal from sea, measured along great-circle path (km)

r1 = 103             km

¦:	frequency (kHz)

The correction c2 is given by:

(6)

(7)

where:

s2:	distance of terminal from next section of land, measured along great-circle path (km)

r2 = 103             km

a:		proportion of land in the section of path between r2 and s2 (0 < a £ 1).

If a computer is used but a terrain data bank is not available to calculate a, then a should be made equal to 0.5, which implies that land and sea are present in equal proportions in the section of path between r2 and s2.

## 2.4	Polarization coupling loss

Lp is the excess polarization coupling loss (dB). Lp for a single terminal is given by one of the following two formulae:

where I is the magnetic dip, N or S, in degrees at the terminal and q is the path azimuth measured in degrees from the magnetic E-W direction, such that | q | £ 90°. Lp should be evaluated separately for the two terminals, because of the different q and I that may apply, and the two Lp values added. The most accurate available values of magnetic dip and declination (e.g. Figs. 5 and 6) should be used in determining q and I.

Figure 7 shows values of Lp calculated from equation (8).

## 2.5	Slant propagation distance, p

For paths longer than 1 000 km, p (km) is approximately equal to the ground distance, d (km) between transmitter and receiver. For shorter paths:

(9)

Equation (9) may be used for paths of any length with negligible error. It should be used in all cases where the distances considered are both above and below 1 000 km, to avoid discontinuities in field strength as a function of distance.

## 2.6	Loss factor

The loss factor k is given by:

(10)

For paths up to 3 000 km in length, FM is the geomagnetic latitude of the mid-point of the path under study. Northern latitudes are considered positive, southern latitudes negative. Figure 4 may be used to obtain the geomagnetic latitude from known geographic coordinates of the mid-point. If the absolute value of FM is greater than 60°, equation (10) is evaluated for FM = 60°. Paths longer than 3 000 km are divided into two equal sections. k for each section is determined according to equation (10). The algebraic average of the two k’s (keff) is then used in calculating field strength.

## 2.7	Hourly loss factor

The hourly loss factor, Lt (dB) is given in Fig. 3. The time t is the time in hours relative to the sunrise or sunset time as appropriate. These are taken at the ground at the mid-path position for d < 2 000 km and at 750 km from the terminal where the Sun sets or rises for longer paths. The large values of hourly loss factor near midday are not defined (Fig. 3). For times in this period use a limit value of 30 dB. The hourly loss factor should not be calculated for high latitude paths and seasons when sunrise and sunset do not occur.

Equations generally equivalent to these curves to within 0.5 dB, are given in § 1 of Appendix 1. Figure 3 represents the average annual diurnal variation.

Figure 8 shows sunset and sunrise times for a range of geographic latitudes and months.

Equations equivalent to the hourly loss factor and the sunset and sunrise curves are given in § 1 and 2 of Appendix 1.

## 2.8	Loss factor incorporating effect of solar activity

Lr is the loss factor incorporating effect of solar activity and is given by the following formulae:

where:

(12)

Paths longer than 3 000 km are divided into two equal sections as described in § 2.6. The value of Lr for each section is derived and added together.

# 3.	Day-to-day and short-period variations of night-time field strengths

The difference between field strength exceeded for 10% of the time and the annual median value, D, is latitude dependent and is given by:

(13)

FIGURE 1/PI.1059...[D01] = 19 CM

FIGURE 2/PI.1059...[D02] = 12 CM

FIGURE 3/PI.1059...[D03] = 8 CM

FIGURE 4/PI.1059...à l'italienne

Sur demande du BR

MONTAGE:      Des anciens originaux (Corrections si nécessaire)

FIGURE 5/PI.1059...à l'italienne

Sur demande du BR

MONTAGE:      Des anciens originaux (Corrections si nécessaire)

FIGURE 6/PI.1059...à l'italienne

Sur demande du BR

MONTAGE:      Des anciens originaux (Corrections si nécessaire)

FIGURE 7/PI.1059...[D07] = à l'italienne

FIGURE 8/PI.1059...[D08] = 14 CM

APPENDIX  1

This Appendix contains equations that may be used in lieu of Figs. 3 and 8 for hourly loss factor, and sunset and sunrise times respectively. For the purpose of this Appendix, the following additional symbols are used.

# List of symbols

a:	geographic latitude of a point on the path (degrees)

b:	geographic longitude of a point on the path (degrees)

S:	local mean time of sunset or sunrise at a point (h).

North and East coordinates are considered positive, and South and West coordinates negative.

# 1.	Hourly loss factor: Lt

These equations may be used instead of the curves in Fig. 3, within the stated limits of t. For hours between these times (i.e. near midnight) set Lt = 0.

where t is the time in hours relative to sunset or sunrise at the path mid-point.

# 2.	Sunset and sunrise times

For non-polar locations, i.e. such that | a | < 65°, the times of sunset and sunrise can be calculated as follows, to an accuracy of ±2 min.

N:	day of the year, in days: e.g. 1 January = 1

S¢:	approximate local time of event, e.g. sunset = 1800 h, sunrise = 0600 h

Z:	Sun’s zenith distance (degrees) = 90.8333°(90°50') for sunset or sunrise.

Step 1:	calculate observer’s longitude, B:

B  =  b / 15               h

Step 2:	calculate the time of event, Y:

Y  =  N  +  (S¢  –  B) / 24               days

Step 3:	calculate Sun’s mean anomaly, M:

M  =  0.985600 Y  –  3.289               degrees

Step 4:	calculate Sun’s longitude, L:

L  =  M  +  1.916 sin M  +  0.020 sin 2 M  +  282.634               degrees

Note in which quadrant L occurs.

Step 5:	calculate Sun’s right ascension, RA:

tan RA  =  0.91746 tan L

Note that RA must be in the same quadrant as L.

Step 6:	calculate Sun’s declination s:

sin s  =  0.39782 sin L

from which:

Note that sin s may be positive or negative but cos s must always be positive.

Step 7:	calculate Sun’s local hour angle H:

cos  H  =  x  =  (cos Z  –  sin s  ·  sin a) / (cos s  ·  cos a)

Note that if | x | > 1, there is no sunset or sunrise.

From cos H, obtain H in degrees:	for sunrise 180 < H < 360;
for sunset 0 < H < 180.

Step 8:	calculate local mean time of event, S:

S  =  H / 15  +  RA / 15  –  0.065710  Y  –  6.622

Note that S is expressed in hours and that multiples of 24 should be added or subtracted until 0 < S < 24.

Note that S is the local time at the point concerned. The corresponding standard time is S - B + bm/15 h where bm is the longitude of the standard meridian for the desired time zone (degrees) so that, for example, universal time = S - B.

_________________