RECOMMENDATION  ITU-R  RA.1031*

PROTECTION  OF  THE  RADIOASTRONOMY  SERVICE  IN
FREQUENCY  BANDS  SHARED  WITH  OTHER  SERVICES

(Question ITU-R 145/7)

(1994)

Rec. ITU-R RA.1031

The ITU Radiocommunication Assembly,

considering

a)	that the radioastronomy service is based on the reception of natural emissions at much lower power levels than are generally used in other radio services, and that the radioastronomy service is therefore particularly vulnerable to harmful interference from transmitters in shared frequency bands;

b)	that many frequency bands allocated to the radioastronomy service are also allocated to other services which transmit in those bands;

c)	that protection from harmful interference is essential to the advancement of the radioastronomy service, and that the preferred frequency bands for radioastronomy are given in Recommendation ITU-R RA.314;

d)	that protection criteria used for radioastronomical measurements are given in Recommendation ITU-R RA.769;

e)	that the detailed characteristics of the interference and the particular type of radioastronomical measurement may need to be taken into account when developing sharing criteria;

f)	that radioastronomy sites are carefully chosen, and that site characteristics may need to be considered in sharing calculations;

g)	that frequency sharing is generally impossible for transmitters within direct line-of-sight of a radioastronomy observatory,

recommends

1.	that, in making assignments to services which share frequency bands with the radioastronomy service, administrations should take all practicable steps to avoid harmful interference to the radioastronomy service;

2.	that consideration be given to protecting radioastronomy sites from harmful interference in shared bands through the establishment of coordination zones for terrestrial services, as defined in Annex 1;

3.	that the size of a coordination zone be calculated, taking account of:

–	the protection criteria contained in Recommendation ITU-R RA.769;

–	an accurate interference model for the service sharing the band;

–	the latest available propagation models such as those contained in Recommendations ITU-R PN.452, ITU-R PN.526 and ITU-R PN.617; and,

–	the requirement that interference variations which may result from variable propagation should be based upon the probability that the interference should not exceed harmful thresholds for more than 10% of the time;

4.	that terrestrial transmissions within a coordination zone be inhibited unless technical means can be found to avoid harmful interference to the radioastronomy service.

ANNEX  1

Coordination considerations

# 1.	General

Radioastronomy sites are chosen specifically to minimize interference from other terrestrial services. The sites are usually at a considerable distance from the major fixed sources of terrestrial interference and may be screened by nearby high ground.

For the assessment of interference to radioastronomy from transmitters on the Earth’s surface, a value of 0 dBi is adopted for the gain of the radioastronomy antenna in the direction of the horizon. On this basis, potentially interfering signals at the threshold levels defined in Recommendation ITU-R RA.769 will not cause harmful interference to observations made at elevation angles greater than 19°.

Many radioastronomy measurements can tolerate levels of interference from a shared service which exceed the harmful thresholds for 10% of the time. However, certain types of measurements such as those involving transient phenomena and those depending upon simultaneous observations at many sites on Earth, may be severely damaged by interference at inopportune times.

# 2.	Separation distances required for sharing

If geographical sharing is to be successful the interfering transmitter and the interfered-with receiver must be separated by a distance at which the interference is not considered harmful. The attenuation over this distance must be sufficient to reduce the received signal below the appropriate level of Table 1, 2 or 3 of Recommendation ITU-R RA.769, for all but 10% of the time. Appendix 28 to the Radio Regulations defines a basic transmission loss Lb( p) as:

Lb( p)  =  Pt  +  Gt  +  Gr  –  Pr ( p)	(1)

where:

Lb( p) :	minimum permissible basic transmission loss (dB) for p% of the time; this value must be exceeded by the actual transmission loss for all but p% of the time

Pt :		transmitting power level (dBW) in the reference bandwidth at the input to the antenna

Gt :		gain (dBi) of the transmitting antenna

Gr :		gain (dBi) of the receiving antenna in the direction of the transmitter

Pr ( p) :	maximum permissible interference power (dBW) in the reference bandwidth to be exceeded for no more than p% of time at the receiver input.

With Gr = 0 dBi, equation (1) assumes the form:

Lb( p)  =  Pt  +  Gt  –  Pr ( p)	(2)

where Pr is to be taken from column 7 of Table 1 or Table 2 of Recommendation ITU-R RA.769.

Lb( p) should be calculated using appropriate models, such as those contained in Recommendations ITU-R PN.452, ITU-R PN.526 and ITU-R PN.617, using p = 10% in the case of time-variable propagation loss.

It should be noted that the free-space propagation loss is not variable and the percentage of time criterion is not pertinent in this case. For line-of-sight transmission, Lb has a simple analytical form and equation (2) may be written as:

20 log (4p d)  –  20 log l  =  Pt  +  Gt  –  Pr	(3)

where:

d :	distance between transmitter and receiver (m)

l :	wavelength (m).

In the above analysis Pt is the power transmitted by the active service within the bandwidth Br of the radioastronomy receiver. If the transmitter power PT  is distributed over a bandwidth Bt > Br then:

Pt (dBW)  =  PT (dBW)  –  10 log (Bt /Br)       for Bt  >  Br	(4)

assuming that the transmitter power has a uniform spectral density.

# 3.	Sharing within the line-of-sight

It is rarely possible for radioastronomy to share successfully with any other service whose transmitters are within line-of-sight of the observatory. Figure 1 illustrates this fact. The maximum e.i.r.p. which would not result in harmful interference to the radioastronomy service has been calculated using equations (3) and (4) for two distances. One distance is representative of a terrestrial transmitter at a large distance within line-of-sight, namely an airborne transmitter on the horizon at a height of 20 km. The other distance is that of the geostationary orbit, and is thus representative of the maximum distance of most spaceborne transmitters not on deep-space missions. The harmful interference levels of Recommendation ITU-R RA.769, Table 1, have been used in the case of the terrestrial transmitter. An additional protection of 15 dB is required for the case of the transmitter in the geostationary orbit, to allow observations within 5° of satellites in the orbit. The curves are applicable to a clear dry atmosphere.

FIGURE 1...[D01] = 3 CM

It is clear from Fig. 1 that sharing with a terrestrial transmitter within line-of-sight is unlikely to be possible at frequencies below 10 GHz because of the severe restriction sharing would impose on the transmitter e.i.r.p. Even for frequencies up to 40 GHz either the transmitter power must be of the order of a few milliwatts or the transmitting antenna must provide high discrimination towards the direction of the observatory, for sharing to be possible.

# 4.	Sharing beyond the line-of-sight using coordination zones

The establishment of coordination zones around radioastronomy sites provides a method of avoiding interference from other terrestrial services sharing the band beyond the line-of-sight.

A coordination zone associated with a radioastronomy station is defined as the area for which the sum total of emissions from users outside its boundary does not exceed the threshold levels of harmful interference measured at the radioastronomy antenna.

The size of the coordination zone depends upon a number of factors. The types of measurements being made at the radioastronomy site determine the corresponding harmful interference thresholds given in Recommendation ITU-R RA.769. The number and distribution of the users outside the zone, the e.i.r.p. of the user transmissions in the direction of the radioastronomy site, the fraction of the time they are active, and the propagation characteristics determine the interfering power flux-density at the radioastronomy site. The propagation characteristics depend upon factors such as the profile of the terrain, presence of trees, and the atmospheric conditions. Appropriate propagation models are suggested in § 2 above.

Because of the number of factors involved, the boundaries of the coordination zones should be established individually for each radioastronomy site at which such a zone is required. It should be realized that the coordination distance is likely to be 100 km or more. For many small countries the coordination zone required may extend beyond the national boundaries into countries where the frequency allocations may be different. Thus special conditions may need to be applied when determining coordination zones to protect radioastronomy in small countries.

The coordination zone defines a region around a radio observatory outside of which the users of the active service can transmit freely without causing harmful interference to radioastronomy observations. For users within the coordination zone technical means must be found to avoid harmful interference to the radioastronomy service.

In principle, coordination zones can also be set up to protect radioastronomy sites from mobile transmitters. In this case the mobile user must have some means of determining when the coordination zone is entered, and some means of reducing the power received at the radioastronomy site to a level below the harmful threshold for interference.

For the case of mobile transmitters on aircraft the sizes of the zones would be much larger than for ground-based transmitters since line-of-sight propagation conditions extend over much greater distances, and increase with aircraft height.

_________________

*	This Recommendation should be brought to the attention of Radiocommunication Study Groups 2 (ex SG 12), 3 (ex SG 5), 4, 8, 9, 10 and 11.