RECOMMENDATION  ITU-R  RA.314-9

Preferred frequency bands for radio astronomical measurements

(Question ITU-R 145/7)

(1953-1956-1959-1966-1970-1974-1978-1982-1986-1990-1992-2002)

The ITU Radiocommunication Assembly,

considering

a)	that the development of radio astronomy has led to major technological advances, particularly in receiving techniques and to improved knowledge of fundamental radio-noise limitations of great importance to radiocommunication, and promises further important results;

b)	that the advancement of radio astronomy requires the protection of certain frequency bands from interference;

c)	that the International Astronomical Union (IAU) is maintaining and updating the list of spectral lines of the greatest importance to radio astronomy;

d)	that radio astronomers study spectral lines both in bands allocated to the radio astronomy service and, as far as spectrum usage by other services allows, outside the allocated bands, and that this has resulted in the detection of more than 3 000 spectral lines as illustrated in Fig. 1 of Annex 1;

e)	that account should be taken of the Doppler shifts of the lines, due to the relative motion of source and observer;

f)	that certain frequency bands have been allocated for continuum observations, and that the exact positions of these bands in the spectrum are not of critical importance, but that their centre frequencies should be in the ratio not more than two to one, taking the width of relevant atmospheric windows into account;

g)	that radio astronomers have made useful astronomical observations from the Earth’s surface in all available atmospheric windows ranging from 2 MHz to 1 000 GHz and above;

h)	that the technique of space radio astronomy, which involves the use of radio telescopes on space platforms, provides access to the entire radio spectrum above about 10 kHz, including parts of the spectrum not accessible from the Earth due to absorption in the atmosphere;

j)	that some types of high-resolution interferometric observations require simultaneous reception, at the same radio frequency, by receiving systems located in different countries, on different continents, or on space platforms;

k)	that world administrative radio conferences and world radiocommunication conferences have made improved allocations for radio astronomy, but that protection in many bands, particularly those shared with other services, may still need careful planning,

recommends

1	that administrations should afford all practicable protection to the frequencies used by radio astronomers in their own and neighbouring countries;

2	that particular attention should be given to securing or maintaining adequate protection for the frequency bands listed in Tables 1 and 2, which contain rest frequencies and Doppler-shifted frequencies of the astrophysically most important spectral lines identified by the General Assembly of the IAU, and in Table 3, which contains the frequency bands allocated to the radio astronomy service that are preferred for continuum observations;

3	that administrations be asked to provide assistance in the coordination of observations of spectral lines in bands not allocated to radio astronomy.

TABLE  1

Radio-frequency lines of the greatest importance to radio astronomy
at frequencies below 275 GHz

| Substance | Rest frequency | Suggested minimum band | Notes(1) |
| --- | --- | --- | --- |
| Deuterium (DI) | 327.384 MHz | 327.0-327.7 MHz |  |
| Hydrogen (HI) | 1 420.406 MHz | 1 370.0-1 427.0 MHz | (2), (3) |
| Hydroxyl radical (OH) | 1 612.231 MHz | 1 606.8-1 613.8 MHz | (4) |
| Hydroxyl radical (OH) | 1 665.402 MHz | 1 659.8-1 667.1 MHz | (4) |
| Hydroxyl radical (OH) | 1 667.359 MHz | 1 661.8-1 669.0 MHz | (4) |
| Hydroxyl radical (OH) | 1 720.530 MHz | 1 714.8-1 722.2 MHz | (3), (4) |
| Methyladyne (CH) | 3 263.794 MHz | 3 252.9-3 267.1 MHz | (3), (4) |
| Methyladyne (CH) | 3 335.481 MHz | 3 324.4-3 338.8 MHz | (3), (4) |
| Methyladyne (CH) | 3 349.193 MHz | 3 338.0-3 352.5 MHz | (3), (4) |
| Formaldehyde (H2CO) | 4 829.660 MHz | 4 813.6-4 834.5 MHz | (3), (4) |
| Methanol (CH3OH) | 6 668.518 MHz | 6 661.8-6 675.2 MHz | (3) |
| Helium (3He+) | 8 665.650 MHz | 8 657.0-8 674.3 MHz | (3), (6) |
| Methanol (CH3OH) | 12.178 GHz | 12.17-12.19 GHz | (3), (6) |
| Formaldehyde (H2CO) | 14.488 GHz | 14.44-14.50 GHz | (3), (4) |
| Cyclopropenylidene (C3H2) | 18.343 GHz | 18.28-18.36 GHz | (3), (4), (6) |
| Water vapour (H2O) | 22.235 GHz | 22.16-22.26 GHz | (3), (4) |
| Ammonia (NH3) | 23.694 GHz | 23.61-23.71 GHz | (4) |
| Ammonia (NH3) | 23.723 GHz | 23.64-23.74 GHz | (4) |
| Ammonia (NH3) | 23.870 GHz | 23.79-23.89 GHz | (4) |
| Sulphur monoxide (SO) | 30.002 GHz | 29.97-30.03 GHz | (6) |
| Methanol (CH3OH) | 36.169 GHz | 36.13-36.21 GHz | (6) |
| Silicon monoxyde (SiO) | 42.821 GHz | 42.77-42.86 GHz |  |
| Silicon monoxyde (SiO) | 43.122 GHz | 43.07-43.17 GHz |  |
| Dicarbon monosulphide (CCS) | 45.379 GHz | 45.33-45.44 GHz | (6) |
| Carbon monosulphide (CS) | 48.991 GHz | 48.94-49.04 GHz |  |
| Oxygen (O2) | 61.1 GHz | 56.31-63.06 GHz | (5), (6), (7) |
| Deuterated water (HDO) | 80.578 GHz | 80.50-80.66 GHz |  |
| Cyclopropenylidene (C3H2) | 85.339 GHz | 85.05-85.42 GHz |  |
| Silicon monoxide (SiO) | 86.243 GHz | 86.16-86.33 GHz |  |
| Formylium (H13CO+) | 86.754 GHz | 86.66-86.84 GHz |  |
| Silicon monoxide (SiO) | 86.847 GHz | 86.76-86.93 GHz |  |
| Ethynyl radical (C2H) | 87.3 GHz | 87.21-87.39 GHz | (5) |
| Hydrogen cyanide (HCN) | 88.632 GHz | 88.34-88.72 GHz | (4) |
| Formylium (HCO+) | 89.189 GHz | 88.89-89.28 GHz | (4) |
| Hydrogen isocyanide (HNC) | 90.664 GHz | 90.57-90.76 GHz |  |

TABLE  1 (end)

| Substance | Rest frequency | Suggested minimum band | Notes(1) |
| --- | --- | --- | --- |
| Diazenylium (N2H+) | 93.174 GHz | 93.07-93.27 GHz |  |
| Carbon monosulphide (CS) | 97.981 GHz | 97.65-98.08 GHz | (4) |
| Sulphur monoxide (SO) | 99.300 GHz | 99.98-100.18 GHz |  |
| Methyl acetylene (CH3C2H) | 102.5 GHz | 102.39-102.60 GHz | (5) |
| Methanol (CH3OH) | 107.014 GHz | 106.91-107.12 GHz |  |
| Carbon monoxide (C18O) | 109.782 GHz | 109.67-109.89 GHz |  |
| Carbon monoxide (13CO) | 110.201 GHz | 109.83-110.31 GHz | (4) |
| Carbon monoxide (C17O) | 112.359 GHz | 112.25-112.47 GHz | (6) |
| Cyano radical (CN) | 113.5 GHz | 113.39-113.61 GHz | (5) |
| Carbon monoxyde (CO) | 115.271 GHz | 114.88-115.39 GHz | (4) |
| Oxygen (O2) | 118.750 GHz | 118.63-118.87 GHz | (7) |
| Formaldehyde (H213CO) | 137.450 GHz | 137.31-137.59 GHz | (6) |
| Formaldehyde (H2CO) | 140.840 GHz | 140.69-140.98 GHz |  |
| Carbon monosulphide (CS) | 146.969 GHz | 146.82-147.12 GHz |  |
| Nitric oxide (NO) | 150.4 GHz | 149.95-150.85 GHz | (5) |
| Methanol (CH3OH) | 156.602 GHz | 156.45-156.76 GHz |  |
| Water vapour (H2O) | 183.310 GHz | 183.12-183.50 GHz |  |
| Carbon monoxide (C18O) | 219.560 GHz | 219.34-219.78 GHz |  |
| Carbon monoxide (13CO) | 220.399 GHz | 219.67-220.62 GHz | (4) |
| Cyano radical (CN) | 226.6 GHz | 226.37-226.83 GHz | (5) |
| Cyano radical (CN) | 226.8 GHz | 226.57-227.03 GHz | (5) |
| Carbon monoxide (CO) | 230.538 GHz | 229.77-230.77 GHz | (4) |
| Carbon monosulphide (CS) | 244.953 GHz | 244.72-245.20 GHz | (6) |
| Nitric oxide (NO) | 250.6 GHz | 250.35-250.85 GHz | (5) |
| Ethynyl radical (C2H) | 262.0 GHz | 261.74-262.26 GHz | (5) |
| Hydrogen cyanide (HCN) | 265.886 GHz | 265.62-266.15 GHz |  |
| Formylium (HCO+) | 267.557 GHz | 267.29-267.83 GHz |  |
| Hydrogen isocyanide (HNC) | 271.981 GHz | 271.71-272.25 GHz |  |
| (1) If Notes (2) or (4) are not listed, the band limits are the Doppler-shifted frequencies corresponding to radial velocities of  300 km/s (consistent with line radiation occurring in our galaxy). (2) An extension to lower frequency of the allocation of 1 400-1 427 MHz is required to allow for the higher Doppler shifts for HI observed in distant galaxies. (3) The current international allocation is not primary and/or does not meet bandwidth requirements. See the Radio Regulations (RR) for more detailed information. (4) Because these line frequencies are also being used for observing other galaxies, the listed bandwidths include Doppler shifts corresponding to radial velocities of up to 1 000 km/s. It should be noted that HI has been observed at frequencies redshifted to 500 MHz, while some lines of the most abundant molecules have been detected in galaxies with velocities up to 50 000 km/s, corresponding to a frequency reduction of up to 17%. (5) There are several closely spaced lines associated with these molecules. The listed bands are wide enough to permit observations of all lines. (6) This line frequency is not mentioned in Article 5 of the RR. (7) These lines are observable only outside the atmosphere. |  |  |  |

TABLE  2

Radio-frequency lines of the greatest importance to radio astronomy at frequencies
 between 275 and 1 000 GHz (not allocated in the RR)

| Substance | Rest frequency (GHz) | Suggested minimum band (GHz) | Notes(1) |
| --- | --- | --- | --- |
| Diazenylium (N2H+) | 279.511 | 279.23-279.79 |  |
| Carbon monosulphide (CS) | 293.912 | 292.93-294.21 |  |
| Hydronium (H3O+) | 307.192 | 306.88-307.50 |  |
| Deuterated water (HDO) | 313.750 | 313.44-314.06 |  |
| Carbon monoxide (C18O) | 329.330 | 329.00-329.66 |  |
| Carbon monoxide (13CO) | 330.587 | 330.25-330.92 |  |
| Carbon monosulphide (CS) | 342.883 | 342.54-343.23 |  |
| Carbon monoxide (CO) | 345.796 | 345.45-346.14 |  |
| Hydrogen cyanide (HCN) | 354.484 | 354.13-354.84 |  |
| Formylium (HCO+) | 356.734 | 356.37-357.09 |  |
| Oxygen (O2) | 368.498 | 368.13-368.87 |  |
| Diazenylium (N2H+) | 372.672 | 372.30-373.05 | (2) |
| Water vapour (H2O) | 380.197 | 379.81-380.58 | (2) |
| Hydronium (H3O+) | 388.459 | 388.07-388.85 |  |
| Carbon monosulphide (CS) | 391.847 | 390.54-392.24 |  |
| Oxygen (O2) | 424.763 | 424.34-425.19 |  |
| Carbon monoxide (C18O) | 439.088 | 438.64-439.53 |  |
| Carbon monoxide (13CO) | 440.765 | 440.32-441.21 |  |
| Carbon monoxide (CO) | 461.041 | 460.57-461.51 |  |
| Deuterated water (HDO) | 464.925 | 464.46-465.39 |  |
| Carbon (CI) | 492.162 | 491.66-492.66 |  |
| Deuterated water (HDO) | 509.292 | 508.78-509.80 |  |
| Hydrogen cyanide (HCN) | 531.716 | 529.94-532.25 | (2) |
| Carbon monosulphide (CS) | 538.689 | 536.89-539.23 | (2) |
| Water vapour (H218O) | 547.676 | 547.13-548.22 | (2) |
| Carbon monoxide (13CO) | 550.926 | 549.09-551.48 | (2) |
| Water vapour (H2O) | 556.936 | 556.37-557.50 | (2) |
| Ammonia (15NH3) | 572.113 | 571.54-572.69 | (2) |
| Ammonia (NH3) | 572.498 | 571.92-573.07 | (2) |
| Carbon monoxide (CO) | 576.268 | 574.35-576.84 | (2) |
| Carbon monosulphide (CS) | 587.616 | 587.03-588.20 | (2) |
| Deuterated water (HDO) | 599.927 | 599.33-600.53 | (2) |
| Water vapour (H2O) | 620.700 | 620.08-621.32 | (2) |
| Hydrogen chloride (HCI) | 625.040 | 624.27-625.67 |  |
| Hydrogen chloride (HCI) | 625.980 | 625.35-626.61 |  |
| Carbon monosulphide (CS) | 636.532 | 634.41-637.17 |  |
| Carbon monoxide (13CO) | 661.067 | 658.86-661.73 |  |
| Carbon monoxide (CO) | 691.473 | 690.78-692.17 |  |
| Oxygen (O2) | 715.393 | 714.68-716.11 | (2) |
| Carbon monosulphide (CS) | 734.324 | 733.59-735.06 | (2) |
| Water vapour (H2O) | 752.033 | 751.28-752.79 | (2) |

TABLE  2 (end)

| Substance | Rest frequency (GHz) | Suggested minimum band (GHz) | Notes(1) |
| --- | --- | --- | --- |
| Oxygen (O2) | 773.840 | 773.07-784.61 | (2) |
| Hydrogen cyanide (HCN) | 797.433 | 796.64-798.23 |  |
| Formylium (HCO+) | 802.653 | 801.85-803.85 |  |
| Carbon monoxide (CO) | 806.652 | 805.85-807.46 |  |
| Carbon (CI) | 809.350 | 808.54-810.16 |  |
| Carbon monosulphide (CS) | 832.057 | 829.28-832.89 |  |
| Oxygen (O2) | 834.146 | 833.31-834.98 |  |
| Carbon monosulphide (CS) | 880.899 | 877.96-881.78 |  |
| Water vapour (H2O) | 916.172 | 915.26-917.09 | (2) |
| Carbon monoxide (CO) | 921.800 | 918.72-922.72 | (2) |
| Carbon monosulphide (CS) | 929.723 | 926.62-930.65 |  |
| Water vapour (H2O) | 970.315 | 969.34-971.29 | (2) |
| Carbon monosulphide (CS) | 978.529 | 977.55-979.51 | (2) |
| Water vapour (H2O) | 987.927 | 986.94-988.92 | (2) |
| (1) The band limits are the Doppler-shifted frequencies corresponding to radial velocities of  300 km/s (consistent with line radiation occurring in our galaxy). (2) These lines are observable only outside the atmosphere. |  |  |  |

TABLE  3

Frequency bands allocated to the radio-astronomy service
that are preferred for continuum observations

| Frequency band (MHz) | Frequency band (GHz) |
| --- | --- |
| 13.360-13.410 | 10.6-10.7 |
| 25.550-25.670 | 15.35-15.4 |
| 37.5-38.25(1) | 22.21-22.50 |
| 73-74.6(2) | 23.6-24.0 |
| 150.05-153(3) | 31.3-31.8 |
| 322-328.6 | 42.5-43.5 |
| 406.1-410 | 76-116(1) |
| 608-614(4) |  |
| 1 400-1 427 | 123-158.5(1) |
| 1 660-1 670 | 164-167 |
| 2 655-2 700(1) | 200-231.5 |
| 4 800-5 000(1) | 241-275(1) |
| (1) These bands include secondary allocations. (2) Allocation (primary) in Region 2, protection recommended in Regions 1 and 3. (3) Allocation (primary) in Region 1, Australia and India. (4) Allocation (primary) in Region 2, the African Broadcasting Area (606-614 MHz), China (606-614 MHz) and India. In Region 1 (except the African Broadcasting Area) and in Region 3 this band is allocated on a secondary basis. |  |

ANNEX  1