RECOMMENDATION  769

PROTECTION  CRITERIA  USED  FOR  RADIOASTRONOMICAL  MEASUREMENTS

(Question 145/7)

(1992)

Rec. 769

The CCIR,

considering

a)	that the development of radioastronomy has led to major technological advances, particularly in receiving techniques, and to improved knowledge of fundamental radio-noise limitations of great importance to radiocommunication, and promises further important results;

b)	that radioastronomers have made useful astronomical observations from the Earth's surface at frequencies as low as 2 MHz, as high as 800 GHz, and from space platforms at frequencies which extend down to lower than 10 kHz;

c)	that protection from interference is essential to the advancement of radioastronomy and the associated measurements;

d)	that the sensitivity of radioastronomical receiving equipment, which is still steadily improving, greatly exceeds the sensitivity of communications and radar equipment;

e)	that propagation conditions at frequencies below about 40 MHz are such that a transmitter operating anywhere on the Earth might cause harmful interference to radioastronomy;

f)	that some transmissions from spacecraft introduce problems of interference to radioastronomy and that these cannot be avoided by choice of site for an observatory or by local protection;

g)	that certain types of radioastronomical observation require long periods of uninterrupted recording, sometimes up to several days;

h)	that harmful interference to radioastronomy can be caused by terrestrial transmissions reflected by the Moon, by aircraft, and possibly by artificial satellites;

j)	that some types of high-resolution interferometric observations require simultaneous reception, at the same radio frequency, by receiving systems located in different countries or on different continents;

k)	that some degree of protection can be achieved by appropriate frequency assignments on a national rather than an international basis;

l)	that the Radio Conferences have made improved allocations for radioastronomy, but that protection in many bands, particularly those shared with other radio services, will need careful planning;

m)	that technical criteria concerning harmful interference in respect of the radioastronomy service have been developed, which are those set out in Tables 1, 2 and 3,

recommends

1.	that radioastronomers should be encouraged to choose sites as free as possible from interference;

2.	that administrations should afford all practicable protection to the frequencies used by radioastronomers in their own and neighbouring countries, taking due account of the levels of harmful interference given in Annex 1;

3.	that administrations, in seeking to afford protection to particular radioastronomical observations, should take all practical steps to reduce to the absolute minimum amplitude, harmonic radiations and other spurious emissions falling within the band of the frequencies to be protected for radioastronomy, particularly those emissions from aircraft, spacecraft and balloons;

4.	that when proposing frequency allocations, administrations take into account that it is very difficult for the radioastronomy service to share frequencies with any other service in which direct line-of-sight paths from the transmitters to the observatories are involved. Above about 40 MHz sharing may be practicable with services in which the transmitters are not in direct line-of-sight from the observatories, but coordination may be necessary, particularly if the transmitters are of high power.

ANNEX 1

Sensitivity of radioastronomy systems

# 1.	General considerations

The simplest way to define the sensitivity of an observation in radioastronomy is to state the smallest power level change DP at the radiometer input which can, with high certainty, be detected and measured by the radiometer. The sensitivity equation is:

(1)

where P and DP refer to noise powers, Df0 stands for the bandwidth and t is the integration time. Equation (1) also holds if P and DP are power spectral densities. Thus DP, the noise fluctuation in power spectral density in the sensitivity equation (1), is related to the total system sensitivity (noise fluctuations) expressed in temperature units through the Boltzmann constant, k, as shown in equation (2):

(2)

and we may express the sensitivity equation as:

(3)

where:

and represents the sum of TA, (the antenna noise temperature contribution from the cosmic background, the Earth's atmosphere and radiation from the Earth), and TR, the receiver noise temperature. Equations (1) or (3) can be used to estimate the sensitivities and harmful interference levels for radioastronomical observations. The results are listed in Tables 1 and 2: an observing (or integration) time t of 2 000 s is assumed. In Table 1 (continuum observations), Df is assumed to be the bandwidth of the allocated radioastronomy bands. In Table 2 (spectral line observations) Df is the channel bandwidth (corresponding to a velocity of 3 km/s) typical of a spectral line system.

The harmful interference levels given in Tables 1 and 2 are expressed as the interference level which introduces an error of 10% in the measurement of DP (or DT), i.e.:

(4)

In summary, the appropriate columns in Tables 1 and 2 may be calculated using the following methods:

–	DT, using equation (3),

–	DP, using equation (2),

–	DPH, using equation (4).

Harmful interference can also be expressed in terms of the power flux-density incident at the antenna, either in the total bandwidth or as a spectral power flux-density SH per 1 Hz of bandwidth. For convenience, the values are given for an antenna having a gain, in the direction of arrival of the interference, equal to that of an isotropic antenna (which has an effective area of c2/4p f 2, where c is the speed of the light and ¦ the frequency).

Values of SH Df, in dB(W/m2), are derived from DPH by adding:

mmmmmdB	(5)

where f  is in MHz. SH is then derived by subtracting 10 log Df  to allow for the bandwidth.

The calculated sensitivities and harmful interference levels presented in Tables 1 and 2 are based on assumed integration times of 2000 s. Integration times actually used in astronomical observations cover a wide range of values. Continuum observations made with telescopes operating singly (rather than in interferometric arrays) are reasonably well represented by the integration time of 2000 s. It is representative of good quality observations. There are many occasions when this time is exceeded by an order of magnitude. There are also certain types of observations, such as observations of solar bursts, for which the greatest attainable sensitivity may not be required. On the other hand 2000 s is less representative of spectral line observations. Improvements in receiver stability and the increased use of correlation spectrometers have resulted in the more frequent use of longer integration times. Spectral line observations lasting several hours are not quite common. A more representative value would be 10 h with a consequent improvement in sensitivity of 6 dB over that now shown in Table 2.

Changes in receiving systems can be expected to give improved performance in the future. At the high frequency end of the spectrum now being used by radioastronomers, improvements in receiver technology are likely to have their largest effect. If receiver temperatures of 10 K can be achieved at frequencies in excess of 30 GHz then improvements in sensitivity of approximately 6 dB will result.

The levels given in Tables 1 and 2 are applicable to terrestrial sources of interfering signals, and are valid for intentional as well as unwanted emissions. The harmful power flux-density and spectral power flux-density shown in Tables 1 and 2 are based on the 0 dBi side-lobe case and should be regarded as the general interference criteria for high sensitivity radioastronomy observations, when the interference does not enter the near side lobes.

A model of the typical side-lobe levels for large paraboloid antennas in the frequency range 2 to 10 GHz is given in Recommendation 509. In this model, the side-lobe level decreases with angular distance (degrees) from the main beam axis, and is equal to 32 – 25 log j (dBi) for 1° < j < 48°. A level of 0 dBi occurs at 19° from the main beam axis. A source of interference of power flux-density equal to the threshold values given in Table 1 would be harmful if such an antenna was pointed within 19° of it. Thus, in some situations, interference at the harmful thresholds in Table 1 can be a problem to radioastronomers.

# 2.	Special cases

## 2.1	Interference from geostationary satellites

Interference from geostationary satellites is a case of particular importance. Because the power levels in Tables 1 and 2 were calculated assuming 0 dBi antenna gain, harmful interference will be encountered when a reference antenna, such as described in Recommendation 509, is pointed within 19° of a satellite radiating at levels in accordance with those listed in the tables. A series of similar transmitters located at intervals of 30° around the geostationary-satellite orbit would preclude radioastronomy observations with high sensitivity from a band of sky 38° wide and centred on the orbit. The loss of such a large area of sky would impose severe restrictions on radioastronomy observations.

In general, it would not be practical to suppress the unwanted emissions from satellites to below the harmful level when the main beam of a radio telescope is pointed directly towards the satellite. A workable solution is suggested by observing the projection of the geostationary-satellite orbit in celestial coordinates as viewed from the latitudes of a number of major radioastronomy observations (see Recommendation 517). If it were possible to point a radio telescope to within 5° of the orbit without encountering harmful interference, then for that telescope a band of sky 10° wide would be unavailable for high-sensitivity observations. For a given observatory this would be a serious loss. However, for a combination of existing radio telescopes located at northern and southern latitudes, operating at the same frequencies, the entire sky would be accessible. A value of 5° should therefore be regarded as the requirement for minimum angular spacing between the main beam of a radioastronomy antenna and the geostationary-satellite orbit.

TABLE  1

Sensitivities and harmful interference levels for radioastronomy continuum observations with 2 000 s integration time

|  |  | Minimum |  | System sensitivity (noise fluctuations) |  | Harmful interference levels (2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Centre frequency (1) | Assumed bandwidth | antenna noise temperature | Receiver noise temperature | Temperature | Power spectral density | Input power | Power flux-density | Spectral power flux-density |
| ¦c (MHz) | D ¦A (MHz) | TA (K) | TR (K) | D T (mK) | D P (dB(W/Hz)) | D PH (dBW) | SH D ¦A (dB(W/m2)) | SH (dB(W/(m2 × Hz))) |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) |
| 0000 13.385 | 000 0 0.05 | 60 000 | 100 | 4 250.000 | – 222 | – 185 | – 201 | – 248 |
| 0000 25.610 | 000 0 0.120 | 20 000 | 100 | 917.000 | – 229 | – 188 | – 199 | – 249 |
| 0000 73.8 | 000 0 1.6 | 1 000 | 100 | 14.000 | – 247 | – 195 | – 196 | – 258 |
| 000 151.525 | 000 0 2.95 | 200 | 100 | 2.760 | – 254 | – 199 | – 194 | – 259 |
| 000 325.3 | 000 0 6.6 | 40 | 100 | 0.860 | – 259 | – 201 | – 189 | – 258 |
| 000 408.05 | 000 0 3.9 | 25 | 100 | 1.000 | – 259 | – 203 | – 189 | – 255 |
| 000 611 | 000 0 6.0 | 15 | 100 | 0.740 | – 260 | – 202 | – 185 | – 253 |
| 001 413.5 | 000 27 | 10 | 20 | 0.091 | – 269 | – 205 | – 180 | – 255 |
| 001 665 | 000 10 | 10 | 20 | 0.150 | – 267 | – 207 | – 181 | – 251 |
| 002 695 | 000 10 | 10 | 20 | 0.150 | – 267 | – 207 | – 177 | – 247 |
| 004 995 | 000 10 | 10 | 20 | 0.150 | – 267 | – 207 | – 171 | – 241 |
| 010 650 | 00 100 | 12 | 20 | 0.050 | – 272 | – 202 | – 160 | – 240 |
| 015 375 | 00 050 | 15 | 30 | 0.100 | – 269 | – 202 | – 156 | – 233 |
| 023 800 | 00 400 | 15 | 50 | 0.051 | – 271 | – 195 | – 147 | – 233 |
| 031 550 | 00 500 | 18 | 100 | 0.083 | – 269 | – 192 | – 141 | – 228 |
| 043 000 | 01 000 | 25 | 100 | 0.063 | – 271 | – 191 | – 137 | – 227 |
| 089 000 | 06 000 | 30 | 150 | 0.037 | – 273 | – 185 | – 125 | – 222 |
| 110 500 | 11 000 | 40 | 150 | 0.029 | – 274 | – 184 | – 121 | – 222 |
| 166 000 | 04 000 | 40 | 150 | 0.048 | – 272 | – 186 | – 120 | – 216 |
| 224 000 | 14 000 | 40 | 200 | 0.032 | – 274 | – 182 | – 114 | – 215 |
| 270 000 | 10 000 | 40 | 200 | 0.038 | – 273 | – 183 | – 113 | – 213 |

(1)	Calculation of harmful interference levels is based on the centre frequency shown in this column although not all regions have the same allocations.

(2)	The harmful interference levels given are those which apply for measurements of the total power received by a single antenna. Less stringent levels may be appropriate for other types of measurements, as discussed in § 2.2. For transmitters in the geostationary orbit, the levels should be 15 dB lower, as explained in § 2.1.

Note 1 – If an integration time of 15 min, 1 h, 2 h, 5 h or 10 h is used, the relevant values in the Table should be varied by + 1.7, – 1.3, – 2.8, – 4.8 or – 6.3 dB respectively.

TABLE  2

Sensitivities and harmful interference levels for radioastronomy spectral line observations* with 2 000 s integration

|  | Assumed | Minimum |  | System sensitivity (noise fluctuations) |  | Harmful interference levels (1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | spectral line channel bandwidth | antenna noise temperature | Receiver noise temperature | Temperature | Power spectral density | Input power | Power flux-density | Spectral power flux-density |
| ¦ (MHz) | D ¦c (kHz) | TA (K) | TR (K) | D T (mK) | D P (dB(W/Hz)) | D PH (dBW) | SH D ¦c (dB(W/m2)) | SH (dB(W/(m2 × Hz))) |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) |
| 1 327 | 1 010 | 40 | 100 | 22.100 | – 245 | – 215 | – 204 | – 244 |
| 1 420 | 1 020 | 10 | 20 | 3.350 | – 253 | – 220 | – 196 | – 239 |
| 1 665 | 1 020 | 10 | 20 | 3.350 | – 253 | – 220 | – 194 | – 237 |
| 4 830 | 1 050 | 10 | 20 | 2.120 | – 255 | – 218 | – 183 | – 230 |
| 14 500 | 1 150 | 15 | 30 | 1.840 | – 256 | – 214 | – 169 | – 221 |
| 22 200 | 1 250 | 40 | 50 | 2.850 | – 254 | – 210 | – 162 | – 216 |
| 23 700 | 1 250 | 40 | 50 | 2.850 | – 254 | – 210 | – 161 | – 215 |
| 43 000 | 1 500 | 25 | 100 | 2.800 | – 254 | – 207 | – 153 | – 210 |
| 48 000 | 1 500 | 30 | 100 | 2.910 | – 254 | – 207 | – 152 | – 209 |
| 88 600 | 1 000 | 30 | 150 | 2.850 | – 254 | – 204 | – 144 | – 204 |
| 98 000 | 1 000 | 40 | 150 | 3.000 | – 254 | – 204 | – 143 | – 203 |
| 115 000 | 1 000 | 50 | 150 | 3.160 | – 254 | – 204 | – 141 | – 201 |
| 140 000 | 1 500 | 40 | 150 | 2.450 | – 255 | – 203 | – 139 | – 200 |
| 178 000 | 1 500 | 40 | 150 | 2.450 | – 255 | – 203 | – 136 | – 198 |
| 220 000 | 2 500 | 40 | 200 | 2.400 | – 255 | – 201 | – 133 | – 197 |
| 265 000 | 2 500 | 40 | 200 | 2.400 | – 255 | – 201 | – 131 | – 195 |

*	This Table is not intended to give a complete list of spectral-line bands, but only representative examples throughout the spectrum.

(1)	The harmful interference levels given are those which apply for measurements of the total power received by a single antenna. Less stringent levels may be appropriate for other types of measurements, as discussed in § 2.2. For transmitters in the geostationary orbit, the levels should be 15 dB lower, as explained in § 2.1.

Note 1 – If an integration time of  15 min, 1 h, 2 h, 5 h or 10 h is used, the relevant values in the Table should be changed by + 1.7, – 1.3, – 2.8, – 4.8 or – 6.3 dB respectively.

COLUMN  DESCRIPTIONS  FOR  TABLES  1  AND  2

# Column

(1)	Centre frequency of the allocated radioastronomy band (Table 1) or nominal spectral line frequency (Table 2).

(2)	Assumed or allocated bandwidth (Table 1) or assumed typical channel widths used for spectral line observations (Table 2).

(3)	Minimum antenna noise temperature includes contributions from the ionosphere, the Earth’s atmosphere and radiation from the Earth.

(4)	Receiver noise temperature representative of a good radiometer system intended for use in high sensitivity radioastronomy observations.

(5)	Total system sensitivity in millikelvins as calculated from equation (1) using the combined antenna and receiver noise temperatures, the listed bandwidth and an integration time of 2 000 s.

(6)	Same as (5) above, but expressed in noise power spectral density using the equation D P  =  k D T, where k  =  1.38  ´  10–23 (J/K) (Boltzmann’s constant). The actual numbers in the table are the logarithmic expression of D P.

(7)	Power level at the input of the receiver considered harmful to high sensitivity observations (D PH). This is expressed as the interference level which introduces an error of not more than 10% in the measurement of D P; D PH  =  0.1 D P D f. The numbers in the table are the logarithmic expression of D PH.

(8)	Power flux-density in a spectral line channel needed to produce a power level of D PH in the receiving system with an isotropic receiving antenna. The numbers in the table are the logarithmic expression of SH D f.

(9)	Spectral power flux-density in a spectral line channel needed to produce a power level D PH in the receiving system with an isotropic receiving antenna. The numbers in the table are the logarithmic expression of SH.

In the model antenna response of Recommendation 509, the side-lobe level at an angle of 5° from the main beam is 15 dBi. Thus, to avoid harmful interference to a radio telescope pointed within 5° from the transmitter, the satellite emissions must be reduced 15 dB below the power flux-densities given in Tables 1 and 2. When satellites are spaced at intervals of only a few degrees along the geostationary-satellite orbit, the emission levels associated with the individual transmitters must be even lower to meet the requirement that the sum of the powers of all the interfering signals received should be 15 dB below DPH in Tables 1 and 2.

It is recognized that the emission limitations discussed above cannot, in practice, be achieved so as to enable sharing of the same frequency band between radioastronomy and down-link transmissions from satellites to take place. The limitations are, however, applicable to unwanted emission from the satellite transmitters which fall within the radioastronomy bands listed in Tables 1 and 2. These emission limitations have implications for the space services responsible for the interference, which require careful evaluation. Furthermore, the design of new radioastronomy antennas should strive to minimize the level of side-lobe gain near the main beam as an important means of reducing interference from transmitters in the geostationary-satellite orbit.

## 2.2	The response of interferometers and arrays to radio interference

Two effects reduce the response to interference. These are related to the frequency of the fringe oscillations that are observed when the outputs of two antennas are combined, and to the fact that the components of the interfering signal received by different and widely-spaced antennas will suffer different relative time delays before they are recombined. The treatment of these effects is more complicated than that for single antennas in § 1. Broadly speaking the major effect is that the effective integration time over which interference affects the measurement is reduced from

the total time of observation to the mean time of one natural fringe oscillation. This typically ranges from some seconds for a compact array with the longest projected spacing L¢ ~ 103 l , where l is the wavelength, to less than 1 ms for intercontinental arrays with L¢ ~ 107 l. Thus, compared to a single radio telescope, the interferometer has a degree of immunity to interference which, under reasonable assumptions increases with the array size expressed in wavelengths.

The greatest immunity from interference occurs for interferometers and arrays in which the separation of the antennas is sufficiently great that the chance of occurrence of correlated interference is very small (e.g. for very long baseline interferometry (VLBI)). In this case, the above considerations do not apply. The tolerable interference level is determined by the requirement that the power level of the interfering signal should be no more than 1% of the receiver noise power to prevent serious errors in the measurement of the amplitude of the cosmic signals. Harmful interference levels for typical VLBI observations are given in Table 3.

It must be emphasized that the use of large interferometers and arrays is generally confined to studies of discrete high brightness sources, with angular dimensions no more than a few tenths of a second of arc for VLBI. For more general studies of radio sources, the results in Tables 1 and 2 apply and are thus appropriate for the general protection of radioastronomy.

TABLE  3

Harmful interference levels for VLBI observations

| Centre frequency fc (1) (MHz) | Harmful interference level SH (dB(W/(m2 × Hz))) |
| --- | --- |
| 325.3 | – 215 |
| 611.3 | – 211 |
| 1 413.5 | – 209 |
| 2 695.3 | – 204 |
| 4 995.3 | – 198 |
| 10 650.3 | – 192 |
| 15 375.3 | – 187 |
| 23 800.3 | – 182 |
| 43 000.3 | – 173 |
| 86 000.3 | – 166 |

      - (1)	Harmful interference levels at other frequencies used for VLBI may be obtained by interpolation.

_________________