Recommendation ITU-R RS.1861 (01/2010)

Typical technical and operational characteristics of Earth exploration-satellite service (passive) systems using allocations between 1.4 and 275 GHz

RS Series Remote sensing systems

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2010

ITU 2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  RS.1861*

Typical technical and operational characteristics of Earth exploration-satellite service (passive) systems using allocations between 1.4 and 275 GHz

(Question ITU-R 243/7)

(2010)

Scope

This Recommendation provides typical technical and operational characteristics of Earth exploration-satellite service (passive) systems using allocations between 1.4 and 275 GHz for utilization in sharing studies.

The ITU Radiocommunication Assembly,

considering

a)	that Earth exploration-satellite service (EESS) (passive) observations may receive emissions from active services;

b)	that there are exclusive EESS (passive) allocations in which all emissions are prohibited by RR No. 5.340;

c)	that EESS (passive) is allocated on a co-primary basis with active services in certain bands;

d)	that studies considering protection for EESS (passive) systems are taking place within ITU-R;

e)	that in order to perform compatibility and sharing studies with EESS (passive) systems, the technical and operational characteristics of those systems must be known,

recommends

1	that the technical and operational parameters presented in Annex 1 of this Recommendation should be taken into account in studies considering EESS (passive) systems using allocations between 1.4 and 275 GHz.

Annex 1

# 1	Introduction

Passive sensors are used in the remote sensing of the Earth and its atmosphere by Earth exploration and meteorological satellites in certain frequency bands allocated to the Earth exploration-satellite service (EESS) (passive). The products of these passive sensor operations are used extensively in meteorology, climatology, and other disciplines for operational and scientific purposes. However, these sensors are sensitive to any emissions within their allocated band. Therefore, any RF emissions above a certain level may constitute interference to the passive sensors using those bands. This is mainly due to the fact that passive sensors may not be able to differentiate the wanted signal from the interference and that interference may not be identifiable in the passive sensor products.

# 2	Current missions and predicted deployments

Several administrations and at least one recognized international organization operated more than 24 satellites in the EESS (passive) at the end of the year 2007. An additional two to three are anticipated to be deployed per year for the foreseeable future. Individual satellites typically carry one to three passive sensing payloads operating below 275 GHz. Each payload may conduct measurements simultaneously at 3 to 15 frequencies as well as on two polarizations at a single frequency.

# 3	Typical orbits

EESS (passive) systems operate in non-geostationary satellite orbit (non-GSO). Orbits are typically circular with an altitude between 350 and 1 400 km. Many EESS (passive) systems operate in a sun-synchronous orbit. Some sensors make measurements at the same place on the Earth every day, while others will repeat observations only after a longer (often more than two weeks) repeat period.

In certain circumstances, multiple satellites operate in formation. Formation flying EESS satellites allow the capability to measure a portion of the atmosphere or surface of the Earth using both multiple instruments and multiple orientations. Measurements from multiple spacecraft will be separated within an amount of time shorter than the time constant of the phenomena being measured. Nominally this separation is on the order of 5 to 15 min, but can be as little as 15 s.

Two formations are used between multiple systems operating in non-GSO. In one formation, two or more satellites directly follow each other performing measurements of the same parcel of atmosphere or the Earth’s surface as demonstrated by satellites A and B in Fig. 1. In the other formation, a nadir pointing passive sensor conducts a measurement while another spacecraft conducts a near-simultaneous measurement at the Earth’s limb as demonstrated by satellites A and C in Fig. 1.

# 4	Types of measurements

All EESS passive sensing systems perform a form of radiometry. Radiometry senses how much energy a body radiates given its temperature. The amount of energy radiated from a perfect “blackbody” varies with frequency and is given by Planck’s equation. However, no substance is truly a perfect blackbody radiator. Frequencies of particular interest for EESS (passive) applications are provided in Recommendation ITU-R RS.515.

The amount of energy radiated is also dependent on the radiating substance. Within a passive sensor’s field of view, there may be multiple radiators in inter alia atmosphere, water vapour, suspended ice particles, and cloud liquid water, emitting in the sensor’s bandwidth. Measurements not conducted on the Earth’s limb will also receive background emissions from water, soil, surface ice, or some combination of all three.

A single passive sensor cannot by itself identify how much energy is radiated by each substance in its field of view. For this reason, data products of most value are derived by comparing measurements from multiple sensors operating at multiple frequencies. By performing radiometric measurements at multiple frequencies, the types of each natural emitter (e.g. water vapour, suspended ice, O3, etc.) and their concentrations may be derived. As the data from any one sensor may be compared with that of multiple other sensors, any interference received by one sensor may corrupt multiple other measurements.

FIGURE 1

Formation flying orientations

## 4.1	Fixed-pointing, multiple frequency and polarization radiometric sensing

Sensing concurrently at multiple frequencies and polarizations offers the possibilities of identifying the presence of multiple natural emitters present in the field of view of the sensor as well as to create profiles of their concentrations. Profiling (a.k.a. sounding) sensors may be nadir-pointing or pointed at the limb of the Earth. Applications of profiling sensors includes the determination of atmospheric chemistry profiles of H2O, O3, ClO, BrO, HCl, OH, HO2, HNO3, HCN, and N2O through limb measurements.

Fixed pointing radiometers are also used to determine path delay of the radar signals used for altimeters caused by atmospheric water vapour.

Radiometers designed for the whole Earth viewing perform continuous, hemispheric microwave soundings of temperature and humidity profiles as well as rain mapping.

## 4.2	Conical scanning radiometers

Many passive microwave sensors designed for imaging the Earth’s surface features use a conical scan configuration turning around the nadir direction because it is important, for the interpretation of surface measurements, to maintain a constant ground incidence angle along the entire scan-lines since the footprints will remain constant in size, and also because the polarization characteristics of the signal have an angular dependence. Conical scanning antennas gather information over wide areas as shown in Fig. 2. Scans are typically performed by rotating the antenna at an offset angle from the nadir direction. Conical scanning radiometers are used to monitor various water processes including precipitation, oceanic water vapour, cloud water, near-surface wind speed, sea surface temperature, soil moisture, snow cover, and sea ice parameters. They can also be used to provide information on the integrated column precipitation content, its area distribution, and its intensity.

FIGURE 2

Geometry of conical scan passive microwave radiometers

## 4.3	Cross-track scanning radiometers

Scanning radiometric measurements gather information over wide areas creating virtual maps of the parameter being measured. This data product determines the horizontal spatial variability of a parameter rather than measuring the parameters at specific points. Scanning measurements are also typically performed at multiple frequencies and polarizations.

Typical applications of cross-track scanning radiometers include the measurement of temperature profiles in the upper atmosphere (especially the stratosphere) and to provide a cloud-filtering capability for tropospheric temperature observations. They also are used to provide daily global observations of temperature and moisture profiles at high temporal resolution, and to measure cloud liquid water content and provide qualitative estimates of precipitation rate.

Scans are typically performed in a cross-track pattern across the surface of the Earth as shown in Fig. 3. Cross-track scanning is performed by physically rotating a reflector 360°. As the reflector is directed away from the surface of the Earth, sensor channels are still used as calibrations are performed by measuring the cosmic background (i.e. cold sky) in addition to a known “warm” source on the spacecraft, as shown in Fig. 4.

FIGURE 3

Typical cross-track Earth scanning pattern

FIGURE 4

Typical sensing scanning pattern over 360°

## 4.4	Push-broom radiometers

A “push-broom” (along track) sensor consists of a line of sensors arranged perpendicular to the flight direction of the spacecraft, as illustrated in Fig. 5. Different areas of the surface are detected as the spacecraft flies forward. The push-broom is a purely static instrument with no moving parts. The major feature of the push-broom is that all resolution elements in a scan line are acquired simultaneously, and not sequentially as with mechanically scanned sensors, enabling this type of sensor to significantly increase the achievable radiometric resolution. Push-broom sensors can be used for a variety of applications, including temperature profiles measurements of the atmosphere, and soil moisture and ocean salinity measurements.

FIGURE 5

Typical push-broom radiometer configuration

# 5	Definition of parameters

TABLE 1

List of technical and operational EESS parameters for passive sensors

Sensor type

Orbit parameters

Altitude

Inclination

Eccentricity

Repeat period

Sensor antenna parameters

Number of beams

Reflector diameter

Maximum antenna gain

Polarization

–3 dB beamwidth

Instantaneous field of view

Off-nadir pointing angle

Incidence angle at Earth

TABLE 1 (end)

–3 dB beam dimensions

Swath width

Main beam efficiency

Beam dynamics

Sensor antenna pattern

Cold calibration antenna gain

Cold calibration horizontal angle (degrees relative to satellite track)

Cold calibration vertical angle (degrees relative to nadir direction)

Sensor receiver parameters

Sensor integration time

Channel bandwidth

Horizontal resolution

Vertical resolution

TABLE 2

Definitions of parameters

| Parameter | Definition |
| --- | --- |
| Sensor type | Various types of radiometers are possible depending on the technology of the radiometer: interferometric radiometer, conical scan, nadir, push-broom, limb radiometer |
| Orbit parameters |  |
| Altitude | The height above the mean sea level |
| Inclination | Angle between the equator and the plane of the orbit |
| Eccentricity | The ratio of the distance between the foci of the (elliptical) orbit to the length of the major axis |
| Repeat period | The time for the footprint of the antenna beam to return to (approximately) the same geographic location |
| Sensor antenna parameters Antenna characteristics vary among sensors. Measured antenna patterns are provided in § 6, where available. A reference radiation pattern is currently being developed for use in other cases |  |
| Number of beams | The number of beams is the number of locations on Earth from which data are acquired at one time |
| Reflector diameter | Diameter of the antenna reflector |
| Maximum antenna gain | The maximum antenna gain can be the real one, or, if it is not known, it can be computed using the antenna efficiency and D diameter of the reflector (when applicable), with the formula: |
| Polarization | Specification of linear or circular polarization |
| −3 dB beamwidth | The −3 dB beamwidth, θ3dB, is defined as the angle between the two directions in which the radiation intensity is one-half the maximum value |

TABLE 2 (end)

| Parameter | Definition |
| --- | --- |
| Instantaneous field of view | The instantaneous field of view (IFOV) is the area over which the detector is sensitive to radiation. By knowing the altitude of the satellite, the dimension of the IFOV can be calculated on the Earth’s surface at the nadir point: the IFOV is generally expressed in km × km. The IFOV is a measure of the size of the resolution element. |
|  | In a scanning system the IFOV refers to the solid angle subtended by the detector when the scanning motion is stopped. For conical scan radiometers, two values are usually computed: – along-track: in the direction of the platform motion (along the in-track direction); – cross-track: in the direction orthogonal to the motion of the sensor platform. For nadir scan radiometers, such as that shown in Fig. 3, the nadir IFOV = Hθ3dB, where H is the height of the satellite and 3dB is the half-power beamwidth. See also Fig. 6 |
| Off-nadir pointing angle | The angle between the nadir and the pointing direction. It is the angle α in Fig. 6 |
| Incidence angle at Earth | The angle between the pointing direction and the normal to the Earth’s surface. It is the angle i as in Fig. 6 |
| –3 dB beam dimensions | The linear dimensions of the beam on the Earth (at the –3 dB level) |
| Swath width | The swath width is defined as the linear ground distance covered in the cross-track direction. For a scanning radiometer, it depends on the angular field of view (AFOV) or scanning angle. For a nadir radiometer, it depends on the off nadir angle. The field of view (FOV) is the total range of viewing of a sensor into the direction of the target. The cross-track component of the FOV is equivalent to the swath width |
| Main beam efficiency | The main beam area is defined as the angular size of a cone with an opening angle equal to 2.5 times the measured –3 dB beamwidth. The main beam efficiency is defined as the ratio of the energy received in the main beam to the energy received in the complete antenna pattern |
| Beam dynamics | The beam dynamics is defined as follows: – For conical scans, it is the rotating speed of the beam; – For mechanical nadir scans, it is the number of scans per second |
| Sensor antenna pattern | Antenna gain as a function of off-axis angle |
| Cold calibration antenna gain | Antenna gain in the direction of (cold) space. This could be the maximum gain of the primary antenna or the secondary antenna |
| Cold calibration horizontal angle | Horizontal angle (degrees relative to satellite track) of the cold calibration measurement. This angle is measured in the tangent plane relative to the along-track direction |
| Cold calibration vertical angle | Vertical angle (degrees relative to nadir direction) of the cold calibration measurement. This angle is measured out from the tangent plane |
| Sensor receiver parameters |  |
| Sensor integration time | The sensor integration time corresponds to the short period of time allocated for the radiative measurement of the instantaneous area of observation by the detector of a sensor |
| Channel bandwidth | The channel bandwidth is the range of frequencies around a centre frequency used by the passive sensor |
| Measurement spatial resolution |  |
| Horizontal resolution | The spatial resolution is often defined as the ability to distinguish between two closely spaced objects on an image. It is generally expressed in both horizontal (usually cross-track IFOV size) and vertical (along-track) resolutions. (Note that “vertical”, in this sense, does not refer to altitude.) |
| Vertical resolution |  |

FIGURE 6

Scanning configuration

Note that the field of view’s projection on the Earth’s surface becomes elliptical due to the increased incidence angle from nadir to the edge of the swath width (half swath).

# 6	Parameters of typical systems

This section provides typical parameters of passive sensors for EESS (passive) bands between 1 GHz and 275 GHz. Table 3 lists the EESS (passive) bands and the section in this text that contains the passive sensor parameters for each band. A consistent set of parameters is used for each band to support worst-case static analyses and dynamic analyses to determine interference levels into passive sensors.

TABLE 3

List of EESS (passive) bands

| EESS (passive) band | Section (§) containing passive sensor parameters |
| --- | --- |
| 1 400-1 427 MHz | 6.1 |
| 6 425-7 250 MHz | 6.2 |
| 10.6-10.7 GHz | 6.3 |
| 18.6-18.8 GHz | 6.4 |
| 21.2-21.4 GHz | 6.5 |
| 23.6-24 GHz | 6.6 |
| 31.3-31.8 GHz | 6.7 |
| 36-37 GHz | 6.8 |
| 50.2-50.4 GHz | 6.9 |
| 52.6-54.25 GHz | 6.10 |
| 54.25-59.3 GHz | 6.11 |
| 86-92 GHz | 6.12 |
| 114.25-122.25 GHz | 6.13 |
| 148.5-151.5 GHz | 6.14 |
| 155.5-158.5 GHz | 6.15 |
| 164-167 GHz | 6.16 |
| 174.8-191.8 GHz | 6.17 |

## 6.1	Typical parameters of passive sensors operating in the 1 400-1 427 MHz band

Frequencies near 1 400 MHz are ideal for measuring soil moisture, and also for measuring sea surface salinity and vegetation biomass. Soil moisture is a key variable in the hydrologic cycle with significant influence on evaporation, infiltration and runoff. In the vadose zone, soil moisture governs the rate of water uptake by vegetation. Sea surface salinity has an influence on deep thermohaline circulation and the meridional heat transport. Variations in salinity influence the near surface dynamics of tropical oceans. To date, there is no capability to measure soil moisture and sea surface salinity directly on a global basis, so the protection of this passive band is essential.

Some remote sensing missions will collect soil moisture data in the entire passive microwave band under consideration from 1 400 to 1 427 MHz. Others will use the same band to collect measurements of ocean salinity with the goal of observing and modelling the processes that relate sea surface salinity variations to climatic changes in the hydrologic cycle, and to understand how these variations influence the general ocean circulation. Still other missions will use a different technological approach and will measure both soil moisture and ocean salinity.

Table 4 provides the characteristics and parameters of sensors on these missions.

TABLE 4

EESS (passive) sensor characteristics in the 1 400-1 427 MHz band

|  | Sensor A1 | Sensor A2 | Sensor A3 |
| --- | --- | --- | --- |
| Sensor type | Interferometric radiometer | Conical scan | Push broom |
| Orbit parameters |  |  |  |
| Altitude | 757 km | 670 km | 657 km |
| Inclination | 98° |  |  |
| Eccentricity | 0 |  |  |
| Repeat period | 3 days | 3 days | 7 days |
| Sensor antenna parameters |  |  |  |
| Number of beams | 1 | 1 | 3 |
| Reflector diameter | N/A | 6.2 m | 2.5 m |
| Maximum beam gain | 9 dBi | 37 dBi | 29.1, 28.8, 28.5 dBi |
| Polarization | V, H |  |  |
| –3 dB beamwidth | 71.6° | 2.6° | 6.1, 6.3, 6.6° |
| Off-nadir pointing angle | 25° | 35.5° | 25.8°, 33.8°, 40.3° |
| Beam dynamics | Fixed | 14.6 rpm | Fixed |
| Incidence angle at Earth | 2°/48° | 39.9° | 28.7°, 37.8°, 45.6° |
| –3 dB beam dimensions | 50 km (35 km centre of FOV) | 50.1 × 38.5 km | 94 × 76 km, 120 × 84 km 156 × 97 km |
| Instantaneous field of view | 756 km | Same as –3 dB dimensions, above |  |
| Main beam efficiency | N/A | 91% | 94%, 92.4%, 90.4% |
| Swath width | 1 000 km | 1 000 km | 407 km |
| Sensor antenna pattern | Fig. 7a | Fig. 7b | Fig. 7c |
| Cold calibration ant. gain | N/A |  |  |
| Cold calibration angle (degrees re. satellite track) | N/A |  |  |
| Cold calibration angle (degrees re. nadir direction) | N/A |  |  |
| Sensor receiver parameters |  |  |  |
| Sensor integration time | 1.2 s | 84 ms | 6 s |
| Channel bandwidth | 27 MHz |  | 26 MHz |
| Measurement spatial resolution |  |  |  |
| Horizontal resolution | 40 km | 39 km | 64, 75, 90 km |
| Vertical resolution | N/A |  |  |

FIGURE 7a

Sensor A1 antenna pattern for the 1 400-1 427 MHz band

FIGURE 7b

Sensor A2 antenna patterns for the 1 400-1 427 MHz band

FIGURE 7c

Sensor A3 antenna patterns for the 1 400-1 427 MHz band

## 6.2	Typical parameters of passive sensors operating in the 6.425-7.25 GHz band

The 6-7 GHz band channel is essential for observing global soil moisture, global sea surface temperature, temperature of sea ice and sea surface wind through cloud, in combination with other channels.

In measurement of soil moisture, measurement in higher frequencies is strongly influenced by vegetation and the atmosphere, and the 6-7 GHz band is the most suitable for relatively higher spatial resolution measurements. In the case of measurement of sea surface temperature, measurement in higher frequencies is strongly influenced by the atmosphere and lower temperature is more difficult to measure in higher frequencies, making the 6-7 GHz band the most suitable.

Table 5 summarizes the parameters of passive sensors that are or will be operating in the 6.425-7.25 GHz band.

## 6.3	Typical parameters of passive sensors operating in the 10.6-10.7 GHz band

The band 10.6-10.7 GHz is of primary interest to measure rain, snow, sea state, and ocean wind. Table 6 summarizes the parameters of passive sensors that are or will be operating in the 10.6-10.68 GHz band.

TABLE 5

EESS (passive) sensor characteristics in the 6.425-7.25 GHz band

|  | Sensor B1 | Sensor B2 | Sensor B3 | Sensor B4 |
| --- | --- | --- | --- | --- |
| Sensor type | Conical scan |  |  |  |
| Orbit parameters |  |  |  |  |
| Altitude | 705 km | 828 km | 835 km | 699.6 km |
| Inclination | 98.2° | 98.7° | 98.85° | 98.186° |
| Eccentricity | 0.0015 | 0 | 0 | 0.002 |
| Repeat period | 16 days | 17 days | N/A | 16 days |
| Sensor antenna parameters |  |  |  |  |
| Number of beams | 1 |  |  |  |
| Reflector diameter | 1.6 m | 2.2 m | 0.6 m | 2.0 m |
| Maximum beam gain | 38.8 dBi |  |  | 40.6 dBi |
| Polarization | V, H |  |  |  |
| –3 dB beamwidth | 2.2° | 1.65° |  | 1.8° |
| Off-nadir pointing angle | 47.5° | 46.8 | 55.4° | 47.5° |
| Beam dynamics | 40 rpm | 31.6 rpm | 2.88 s scan period | 40 rpm |
| Incidence angle at Earth | 55° | 55.7° | 65° | 55° |
| –3 dB beam dimensions | 40 km (cross-track) | 24 km |  | 35 km (cross-track) |
| Instantaneous field of view | 43 km × 75 km | 68 km × 40 km | 112 km × 260 km | 35 km × 61 km |
| Main beam efficiency | 95.1% | 95% |  | 92% |
| Swath width | 1 450 km | 1 700 km | 2 000 km | 1 450 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 |  |  |  |
| Cold calibration ant. gain | 25.1 dBi | N/A |  | 25.6 dBi |
| Cold calibration angle (degrees re. satellite track) | 115.5º | N/A |  | 115.5º |
| Cold calibration angle (degrees re. nadir direction) | 97.0º | N/A |  | 97.0º |
| Sensor receiver parameters |  |  |  |  |
| Sensor integration time | 2.5 ms | 5 ms | N/A | 2.5 ms |
| Channel bandwidth | 350 MHz centred at 6.925 GHz | 350 MHz centred at 6.625 GHz | 350 MHz centred at 6.9 GHz | 350 MHz centred at 6.925 GHz and at 7.3 GHz |
| Measurement spatial resolution |  |  |  |  |
| Horizontal resolution | 43 km | 15-50 km | 38 km | 35 km |
| Vertical resolution | 74 km | 24 km | 38 km | 61 km |

TABLE 6

EESS (passive) sensor characteristics in the 10.6-10.7 GHz band

|  | Sensor C1 | Sensor C2 | Sensor C3 | Sensor C4 | Sensor C5 |
| --- | --- | --- | --- | --- | --- |
| Sensor type | Conical scan |  |  |  |  |
| Orbit parameters |  |  |  |  |  |
| Altitude | 817 km | 705 km | 833 km | 835 km | 699.6 km |
| Inclination | 98° | 98.2° | 98.7° | 98.85° | 98.186° |
| Eccentricity | 0 | 0.0015 | 0 | 0 | 0.002 |
| Repeat period | N/A | 16 days | 17 days | N/A | 16 days |
| Sensor antenna parameters |  |  |  |  |  |
| Number of beams | 1 |  | 2 | 1 |  |
| Reflector diameter | 0.9 m | 1.6 m | 2.2 m | 0.6 m | 2.0 m |
| Maximum beam gain | 36 dBi | 42.3 dBi | 45 dBi | 36 dBi | 44.1 dBi |
| Polarization | H, V |  | H, V, R, L | H, V |  |
| –3 dB beamwidth | 2.66° | 1.4° | 1.02° | 3.28 | 1.2° |
| Instantaneous field of view | 56 km × 30 km | 51 km × 29 km | 48 km × 28 km | 76 km × 177 km | 41 km × 21 km |
| Main beam efficiency |  | 94.8% | 95% |  | 93% |
| Off-nadir pointing angle | 44.3° | 47.5° | 47° | 55.4° | 47.5° |
| Beam dynamics | 20 rpm | 40 rpm | 31.6 rpm | 2.88 s scan period | 40 rpm |
| Incidence angle at Earth | 52° | 55° | 58.16° | 65° | 55° |
| –3 dB beam dimensions | 56.7 km (cross-track) | 27.5 km (cross-track) | 42.9 km (cross-track) | N/A | 23 km (cross-track) |
| Swath width | 1 594 km | 1 450 km | 1 600 km | 2 000 km | 1 450 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 | Fig. 8a | Fig. 8b | See Rec. ITU-R RS.1813 |  |
| Cold calibration ant. gain | N/A | 29.1 dBi | N/A |  | 29.6 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | 115.5º | N/A |  | 115.5º |
| Cold calibration angle (degrees re. nadir direction) | N/A | 97.0º | N/A |  | 97.0º |
| Sensor receiver parameters |  |  |  |  |  |
| Sensor integration time | 1 ms | 2.5 ms | 2.47 ms | N/A | 2.5 ms |
| Channel bandwidth | 100 MHz | 100 MHz centred at 10.65 GHz |  |  |  |
| Measurement spatial resolution |  |  |  |  |  |
| Horizontal resolution | 38 km | 27 km | 15 km | 38 km | 23 km |
| Vertical resolution | 38 km | 47 km | 15 km | 38 km | 41 km |

FIGURE 8a

Sensor C1 antenna pattern envelope for the 10.6-10.7 GHz band

FIGURE 8b

Sensor C2 antenna pattern envelope for the 10.6-10.7 GHz band

## 6.4	Typical parameters of passive sensors operating in the 18.6-18.8 GHz band

The 18.6-18.8 GHz band is essential for observing global rain rates, sea state, sea ice, water vapour, ocean wind speed, soil emissivity, and humidity. Table 7 summarizes the parameters of passive sensors that are or will be operating in the 18.6-18.8 GHz band.

TABLE 7

EESS (passive) sensor characteristics in the 18.6-18.8 GHz band

|  | Sensor D1 | Sensor D2 | Sensor D3 | Sensor D4 | Sensor D5 |
| --- | --- | --- | --- | --- | --- |
| Sensor type | Conical scan |  |  |  |  |
| Orbit parameters |  |  |  |  |  |
| Altitude | 828 km | 705 km | 865.6 km | 835 km | 699.6 km |
| Inclination | 98.7° | 98.2 | 20 | 98.85° | 98.186° |
| Eccentricity | 0 | 0.0015 | 0 | 0 | 0.002 |
| Repeat period | 17 days | 16 days | 7 days |  | 16 days |
| Sensor antenna parameters |  |  |  |  |  |
| Number of beams | 3 | 1 |  | 1 |  |
| Reflector diameter | 2.2 m | 1.6 m | 0.65 m | 0.6 m | 2.0 m |
| Maximum beam gain |  | 47.6 dBi |  |  | 49.4 dBi |
| Polarization | V, H, LHC, RHC, +45°, −45° | V, H |  |  |  |
| –3 dB beamwidth | 0.64° | 0.8 | 0.67 | 1.9 | 0.65 |
| Instantaneous field of view | 24 km × 15.5 km | 27 km × 16 km | 10 km | 45 km × 104 km | 22 km × 13 km |
| Main beam efficiency | 95% | 95.8% | 96% |  | 94% |
| Off-nadir pointing angle | 46.6° | 47.5 | 44.5 | 55.4 | 47.5 |
| Beam dynamics | 31.6 rpm | 40 rpm | 20 rpm | 2.88 s scan period | 40 rpm |
| Incidence angle at Earth | 53.6° | 55.0° | 52.3 | 65° | 55.0° |
| –3 dB beam dimensions | 9 km | 13 km (cross-track) | 10 km | 28 km | 16 km (cross-track) |
| Swath width | 1 700 km | 1 450 km |  | 2 000 km | 1 450 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 |  |  |  |  |
| Cold calibration ant. gain | N/A | 32.8 dBi | N/A |  | 33.9 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | 115.5º | N/A |  | 115.5º |
| Cold calibration angle (degrees re. nadir direction) | N/A | 97.0º | N/A |  | 97.0º |
| Sensor receiver parameters |  |  |  |  |  |
| Sensor integration time | 1.2 ms | 2.5 ms | N/A |  | 2.5 ms |
| Channel bandwidth | 200 MHz centred at 18.7 GHz |  | N/A | 200 MHz centred at 18.7 GHz |  |
| Measurement spatial resolution |  |  |  |  |  |
| Horizontal resolution | 9 km | 16 km | 40 km | 38 km | 13 km |
| Vertical resolution | 9 km | 27 km | 40 km | 38 km | 22 km |

## 6.5	Typical parameters of passive sensors operating in the 21.2-21.4 GHz band

The 21.2-21.4 GHz band in addition to the 23.6-24 GHz band are used for measurements of water vapour and liquid water both on the Earth’s surface and in the atmosphere. They are on either side of the 22.235 GHz water-vapour spectral line. Atmospheric measurements are used with oxygen, O2, temperature measurements to remove the effect of water vapour on temperature profiles. Table 8 summarizes the parameters of passive sensors that are or will be operating in the 
21.2-21.4 GHz band.

TABLE 8

EESS (passive) sensor characterstics in the 21.2-21.4 GHz band

|  | Sensor E1 | Sensor E2 |
| --- | --- | --- |
| Sensor type | Mechanical nadir scan | Push-broom(1) |
| Orbit parameters |  |  |
| Altitude | 833 km | 850 km |
| Inclination | 98.6° | 98 |
| Eccentricity | 0 |  |
| Repeat period | 9 days |  |
| Sensor antenna parameters |  |  |
| Number of beams | 1 beam; 30 earth fields per 8 s scan period | 90 |
| Maximum beam gain | 34.4 dBi | 45 dBi |
| Reflector diameter | 0.3 m | 0.9 m |
| Polarization | V | H, V |
| –3 dB beamwidth | 3.3° | 1.1° |
| Instantaneous field of view | Nadir FOV: 48.5 km Outer FOV: 149.1 × 79.4 km | 16 km × 2 282 km |
| Main beam efficiency | 95% |  |
| Off-nadir pointing angle | ±48.33° cross-track |  |
| Beam dynamics | 8 s scan period | N/A (beams are unchanging) |
| Incidence angle at Earth |  |  |
| –3 dB beam dimensions | 45 km | 16 km |
| Total FOV cross/along-track | Outer FOV: 149.1 × 79.4 km Nadir FOV: 48.5 km | 100/1.1° |
| Swath width | 2 343 km | 2 282 km |
| Sensor antenna pattern | −10 dBi back lobe gain | −12 dBi back lobe gain |
| (1) Push-broom is a concept that has not yet been implemented at this frequency. |  |  |

TABLE 8 (end)

|  | Sensor E1 | Sensor E2 |
| --- | --- | --- |
| Sensor antenna parameters (cont.) |  |  |
| Cold calibration ant. gain | 34.4 dBi | 35 dBi |
| Cold calibration angle (degrees re. satellite track) | 90° |  |
| Cold calibration angle (degrees re. nadir direction) | 83° |  |
| Sensor receiver parameters |  |  |
| Sensor integration time | 158 m | N/A |
| Channel bandwidth | 270 MHz centred at 23.8 GHz | N/A |
| Measurement spatial resolution |  |  |
| Horizontal resolution | 45 km | 16 km |
| Vertical resolution | N/A | 16 km |

## 6.6	Typical parameters of passive sensors operating in the 23.6-24 GHz band

In case of a sounder, passive measurements around frequencies 23.8 GHz (total water vapour content), 31.5 GHz (window channel) and 90 GHz (liquid water) provide auxiliary data which play a predominant role in the retrieval process of temperature measurements performed in the O2 absorption spectrum. These auxiliary measurements must have radiometric and geometric performances and availability criteria consistent with those of the temperature measurements. In case of a conical scanning radiometer, it is possible to measure horizontal water vapour distribution with other channels. The main characteristics of the sensors are given in Table 9.

TABLE 9

EESS (passive) sensor characteristics in the 23.6-24 GHz band

|  | Sensor F1 |  | Sensor F2 | Sensor F3 | Sensor F4 | Sensor F5 | Sensor F6 | Sensor F7 | Sensor F8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor type | Conical scan |  |  |  | Mechanical nadir scan |  | Conical scan | Push-broom | Conical scan |
| Orbit parameters |  |  |  |  |  |  |  |  |  |
| Altitude | 817 km | 705 km |  | 828 km | 833 km 822 km* | 824 km | 835 km | 850 km | 699.6 km |
| Inclination | 20° | 98.2° |  | 98.7° | 98.6° 98.7°* | 98.7° | 98.85° | 98 | 98.186° |
| Eccentricity | 0 | 0.0015 |  | 0 | 0 0.001 | 0 |  |  | 0.002 |
| Repeat period | 7 days | 16 days |  | 17 days | 9 days 29 days* | 9 days |  |  | 16 days |
| Sensor antenna parameters |  |  |  |  |  |  |  |  |  |
| Number of beams | 1 |  |  |  | 30 earth fields per 8 s scan period | 2 | 1 | 90 | 1 |
| Reflector diameter | 0.6 m | 1.6 m |  | 2.2 m | 0.3 m 0.274 m* | 0.203 m | 0.6 m | 0.9 m | 48.5 dBi |
| Maximum beam gain | 40 dBi | 46.7 dBi |  | 52 dBi | 34.4 dBi | 30.4 dBi | 43 dBi | 45 dBi | 2.0 m |
| Polarization | H, V |  |  |  | V QV* | QV | H, V |  | H, V |
| −3 dB beamwidth | 1. 81° | 0.9° |  | 0.64° | 3.3° | 5.2° | 1.5° | 1.1° | 0.75 |
| Instantaneous field of view | 63 km × 38 km | 32 km × 18 km |  | 18 km × 12 km | Nadir FOV: 48.5 km Outer FOV: 149.1 × 79.4 km 147 × 79 km* | Nadir FOV: 74.8 km Outer FOV: 323.1 × 141.8 km | 36 km × 86 km | 16 km × 2 282 km | 26 km × 15 km |
| Main beam efficiency | 96% | 94.8% |  | 95% |  |  |  |  | 94% |
| Off-nadir pointing angle | 44.5 | 47.5 |  | 46.6° | ±48.33° cross-track | ±52.725° cross-track | 55.4 |  | 47.5 |

TABLE 9 (end)

|  | Sensor F1 | Sensor F2 | Sensor F3 | Sensor F4 | Sensor F5 | Sensor F6 | Sensor F7 | Sensor F8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor antenna parameters (cont.) |  |  |  |  |  |  |  |  |
| Beam dynamics | 31.9 rpm | 40 rpm | 31.6 rpm | 8 s scan period | 8/3 s scan period cross-track; 96 earth fields per scan period | 2.88 s scan period | 90 resolution elements/ line | 40 rpm |
| Incidence angle at Earth | 52.3° | 55° | 53.63° | 0 (nadir) 57.5°* |  | 65° |  | 55° |
| –3 dB beam dimensions | 38.7 km (cross-track) | 18 km (cross-track) | 14.1 km (cross-track) | 45 km 48 km* | 76 km | 22 km | 16 km | 15 km (cross-track) |
| Swath width | 1 607 km | 1 450 km | 1 688 km | 2 343 km 2 186 km* | 2 503 km | 2 000 km | 2 282 km | 1 450 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 | Fig. 9b | See Rec. ITU-R RS.1813 | Fig. 9c | See Rec. ITU-R RS.1813 |  | 12 dBi back lobe gain | See Rec. ITU-R RS.1813 |
| Cold calibration ant. gain | N/A | 32.1 dBi | N/A | 34.4 dBi | 30.4 dBi | N/A | 35 dBi | 32.4 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | 115.5º | N/A | 90° −90° ± 3.9°* | 0 | N/A | 90° | 115.5º |
| Cold calibration angle (degrees re. nadir direction) | N/A | 97.0º | N/A | 83° | 82.175 | N/A | 83° | N/A |
| Sensor receiver parameters |  |  |  |  |  |  |  |  |
| Sensor integration time | 1 ms | 2.5 ms | 1.2 ms | 158 ms | 18 ms | N/A |  | 2.5 ms |
| Channel bandwidth | 400 MHz | 400 MHz centred at 23.8 GHz |  | 270 MHz centred at 23.8 GHz |  | 400 MHz centred at 23.8 GHz | N/A | 400 MHz centred at 23.8 GHz |
| Measurement spatial resolution |  |  |  |  |  |  |  |  |
| Horizontal resolution | 40 km | 18 km | 17.6 km | 45 km 48 km* | 75 km | 38 km | 16 km | 15 km |
| Vertical resolution | N/A | 30 km | N/A | 45 km 48 km* | 75 km | 38 km | 16 km | 25 km |
| NOTE 1 – * Indicates that a particular sensor is flown on different missions, with different orbit and sensor parameters. |  |  |  |  |  |  |  |  |

FIGURE 9a

Sensor F1 antenna pattern envelope for the 23.6-24 GHz band

FIGURE 9b

Sensor F2 antenna pattern envelope for the 23.6-24 GHz band

FIGURE 9c

Sensor F4 antenna pattern (23.8 GHz)

## 6.7	Typical parameters of passive sensors operating in the 31.3-31.8 GHz band

Passive measurements around frequencies 23.8 GHz (total water vapour content), 31.5 GHz (window channel) and 90 GHz (liquid water) provide auxiliary data which play a predominant role in the retrieval process of temperature measurements performed in the O2 absorption spectrum. These auxiliary measurements must have radiometric and geometric performances and availability criteria consistent with those of the temperature measurements.

This band is one of the bands used for close-to-nadir atmospheric sounding in conjunction with the bands such as 23.8 GHz and 50.3 GHz for the characterization each layer of the Earth’s atmosphere. The 31.3-31.5 GHz band will also be used in conjunction with the band 31.5-31.8 GHz as a “split window”. This will allow a comparison of the measurements conducted in the two sub-bands to check the quality of the data. This will then allow using the full band when the quality is expected good to increase the sensitivity of the sensor.

Table 10 summarizes the parameters of passive sensors that are or will be operating in the 31.3-31.8 GHz band.

TABLE 10

EESS (passive) sensor characteristics in the 31.3-31.8 GHz band

|  | Sensor G1 | Sensor G2 | Sensor G3 |
| --- | --- | --- | --- |
| Sensor type | Nadir scan |  | Conical scan |
| Orbit parameters |  |  |  |
| Altitude | 833 km 822 km* | 824 km | 835 km |
| Inclination | 98.6° | 98.7° | 98.85° |
| Eccentricity | 0.001 | 0 | 0 |
| Repeat period | 9 days 29 days* | 9 days |  |

TABLE 10 (end)

|  | Sensor G1 | Sensor G2 | Sensor G3 |
| --- | --- | --- | --- |
| Sensor antenna parameters |  |  |  |
| Number of beams | 30 earth fields per 8 s scan period | 2 | 1 |
| Maximum beam gain | 34.4 dBi | 30.4 dBi | 45 dBi |
| Reflector diameter | 0.30 m 0.274 m* | 0.203 m | 0.6 m |
| Polarization | V QV* | QV | H, V |
| –3 dB beamwidth | 3.3° | 5.2° | 1.1° |
| Off-nadir pointing angle | ±48.33° cross-track | ±52.725° cross-track | 55.4 |
| Beam dynamics | 8 s scan period | 8/3 s scan period cross-track; 96 earth fields per scan period | 2.88 s scan period |
| Incidence angle at Earth | 0 57.5°* | 0 | 65° |
| –3 dB beam dimensions | 49.1 km | 75 km | 16 km |
| Instantaneous field of view | Nadir FOV: 48.5 km Outer FOV: 149.1 × 79.4 km 147 × 79 km* | Nadir FOV: 74.8 km Outer FOV: 323.1.1 × 141.8 km | 30 km × 69 km |
| Main beam efficiency | 95% |  |  |
| Swath width | 2 343 km 2 186 km* | 2 500 km | 2 000 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 |  |  |
| Cold calibration ant. gain | 34.4 dBi | 30.4 dBi | N/A |
| Cold calibration angle (degrees re. satellite track) | 90 −90° ± 3.9°* | 0 | N/A |
| Cold calibration angle (degrees re. nadir direction) | 83.33 | 82.175 | N/A |
| Sensor receiver parameters |  |  |  |
| Sensor integration time | 158 ms | 18 ms | N/A |
| Channel bandwidth | 180 MHz centred at 31.4 GHz |  | 0.5 GHz |
| Measurement spatial resolution |  |  |  |
| Horizontal resolution | 44 km 48 km* | 75 km | 38 km |
| Vertical resolution | 44 km 48 km* | 75 km | 38 km |
| NOTE 1 – * Indicates that a particular sensor is flown on different missions, with different orbit and sensor parameters. |  |  |  |

## 6.8	Typical parameters of passive sensors operating in the 36-37 GHz band

The band 36-37 GHz is vital for the study of global water circulation, rain rates, snow, sea ice, and clouds. Table 11 summarizes the parameters of passive sensors that are or will be operating in the 36-37 GHz band.

TABLE 11

EESS (passive) sensor characteristics in the 36-37 GHz band

|  | Sensor H1 | Sensor H2 | Sensor H3 | Sensor H4 | Sensor H5 |
| --- | --- | --- | --- | --- | --- |
| Sensor type | Conical scan |  |  |  |  |
| Orbit parameters |  |  |  |  |  |
| Altitude | 865.6 km | 705 km | 828 km | 835 km | 699.6 km |
| Inclination | 20 | 98.2° | 98.7° | 98.85° | 98.186° |
| Eccentricity | 0 | 0.0015 | 0 | 0 | 0.002 |
| Repeat period | 7 days | 16 days | 17 days |  | 16 days |
| Sensor antenna parameters |  |  |  |  |  |
| Number of beams |  |  | 2 | 1 |  |
| Reflector diameter | 0.65 m | 1.6 m | 2.2 m | 0.6 m | 2.0 m |
| Maximum beam gain | 45 dBi | 53.1 dBi | 55 dBi | 46 dBi | 54.8 dBi |
| Polarization | H | H, V |  |  |  |
| –3 dB beamwidth | 1.8° | 0.42° | 0.44° | 1° | 0.35 |
| Instantaneous field of view | 62 km × 38 km | 14 km × 8 km | 16 km × 12 km | 26 km × 60 km | 12 km × 7 km |
| Main beam efficiency | 96% | 93.9% | 95% |  | 93% |
| Off-nadir pointing angle | 44.5° | 47.5° | 46.8° | 55.4 | 47.5° |
| Beam dynamics | 31.9 rpm | 40 rpm | 31.6 rpm | 2.88 s scan period | 40 rpm |
| Incidence angle at Earth | 52.3° | 55° | 55.7° | 65° | 55° |
| –3 dB beam dimensions | 38 km (cross-track) | 8.2 km (cross-track) | 12 km (cross-track) | 15 km | 6.8 km (cross-track) |
| Swath width | 1 607 km | 1 450 km | 1 700 km | 2 000 km | 40 rpm |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 | See Fig. 10 | See Rec. ITU-R RS.1813 |  |  |
| Cold calibration ant. gain | N/A | 36.5 dBi | N/A |  | 39.3 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | 115.5º | N/A |  | 115.5º |
| Cold calibration angle (degrees re. nadir direction) | N/A | 97.0 | N/A |  | 97.0 |

TABLE 11 (end)

|  | Sensor H1 | Sensor H2 | Sensor H3 | Sensor H4 | Sensor H5 |
| --- | --- | --- | --- | --- | --- |
| Sensor receiver parameters |  |  |  |  |  |
| Sensor integration time | 1 ms | 2.5 ms | 1.2 ms | N/A | 2.5 ms |
| Channel bandwidth | 1 GHz | 1 GHz centred at 36.5 GHz |  |  |  |
| Measurement spatial resolution |  |  |  |  |  |
| Horizontal resolution | 40 km | 8.2 km | 12 km | 38 km | 6.8 km |
| Vertical resolution | N/A | 14 km | 6 km | 38 km | 12 km |

FIGURE 10

Sensor H2 antenna pattern envelope for the 36-37 GHz band

## 6.9	Typical parameters of passive sensors operating in the 50.2-50.4 GHz band

This frequency band is one of several bands between 50 GHz and 60 GHz that are used collectively to provide three-dimensional temperature profiles of the atmosphere. Table 12 summarizes the parameters of passive sensors that are or will be operating in the 50.2-50.4 GHz band.

TABLE 12

EESS (passive) sensor characteristics in the 50.2-50.4 GHz band

|  | Sensor I1 | Sensor I2 | Sensor I3 | Sensor I4 |
| --- | --- | --- | --- | --- |
| Sensor type | Conical scan | Mechanical nadir scan | Push-broom | Mechanical nadir scan |
| Orbit parameters |  |  |  |  |
| Altitude | 828 km | 833 km 822 km* | 850 km | 824 km |
| Inclination | 98.7° | 98.6 98.7* | 98 | 98.7° |
| Eccentricity | 0 | 0 0.001* | 0 | 0 |
| Repeat period | 17 days | 9 days 29 days* |  | 9 days |
| Sensor antenna parameters |  |  |  |  |
| Number of beams | 1 | 30 earth fields per 8 s scan period | 90 | 2 |
| Reflector diameter | 2.2 m | 0.15 m | 0.5 m | 0.203 m |
| Maximum beam gain |  | 34.4 dBi | 45 dBi | 37.9 dBi |
| Polarization | V | V QV* | H, V | QH |
| −3 dB beamwidth | 0.39° | 3.3° | 1.1° | 2.2° |
| Instantaneous field of view | 16 km × 12 km | Nadir FOV: 48.5 km Outer FOV: 149.1 × 79.4 km 147 × 79 km* | 16 km × 2 282 km | Nadir FOV: 31.6 km Outer FOV: 136.7 × 60 km |
| Main beam efficiency | 95% |  |  | 95% |
| Off-nadir pointing angle | 46.8° | ±48.33° cross-track |  | ±52.725° cross-track |
| Beam dynamics | 31.6 rpm | 8 s scan period | 90 resolution elements per swath | 8/3 s scan period cross-track; 96 earth fields per scan period |
| Incidence angle at Earth | 55.7° | 57.5° |  |  |
| –3 dB beam dimensions | 6 km | 48 km (at nadir) | 16 km (at nadir) | 2.232 km |
| Swath width | 1 700 km | 2 343 km 2 186 km | 2 282 km | 2 500 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 |  |  |  |
| Cold calibration ant. gain | N/A | 34.4 dBi | 35 dBi | 37.9 dBi |

TABLE 12 (end)

|  | Sensor I1 | Sensor I2 | Sensor I3 | Sensor I4 |
| --- | --- | --- | --- | --- |
| Sensor antenna parameters (cont.) |  |  |  |  |
| Cold calibration angle (degrees re. satellite track) | N/A | 90 −90° ± 3.9°* | 90 | 0 |
| Cold calibration angle (degrees re. nadir direction) | N/A | 83.33 | 83 | 82.175 |
| Sensor receiver parameters |  |  |  |  |
| Sensor integration time | 1.2 ms | 165 ms | N/A | 18 ms |
| Channel bandwidth | 134 MHz centred at 50.3 GHz | 180 MHz centred at 50.3 GHz | N/A | 180 MHz centred at 50.3 GHz |
| Measurement spatial resolution |  |  |  |  |
| Horizontal resolution | 6 km | 48 km | 16 km | 32 km |
| Vertical resolution | 6 km | 48 km | 16 km | 32 km |
| NOTE 1 – * Indicates that a particular sensor is flown on different missions, with different orbit and sensor parameters. |  |  |  |  |

## 6.10	Typical parameters of passive sensors operating in the 52.6-54.25 GHz band

This band is one of the bands used for close-to-nadir atmospheric sounding in conjunction with the bands at 23.8 GHz, 31.5 GHz and 50.3 GHz to characterize each layer of the atmosphere.

Table 13 summarizes the parameters of passive sensors that are or will be operating in the 52.6-54.25 GHz band.

TABLE 13

EESS (passive) sensor characteristics in the 52.6-54.25 GHz band

|  | Sensor J1 | Sensor J2 | Sensor J3 | Sensor J4 |
| --- | --- | --- | --- | --- |
| Sensor type | Mechanical nadir scan | Conical scan | Mechanical nadir scan | Conical scan |
| Orbit parameters |  |  |  |  |
| Altitude | 833 km 822 km* | 828 km | 824 km | 835 km |
| Inclination | 98.6 98.7°* | 98.7° |  | 98.85° |
| Eccentricity | 0 0.001* | 0 |  |  |
| Repeat period | 9 days 29 days* | 17 days | 9 days | N/A |
| Sensor antenna parameters |  |  |  |  |
| Number of beams | 30 earth fields per 8 s scan period | 1 | 2 | 1 |
| Reflector diameter | 0.15 m | 2.2 m | 0.203 m | 0.6 m |

TABLE 13 (end)

|  | Sensor J1 | Sensor J2 | Sensor J3 | Sensor J4 |
| --- | --- | --- | --- | --- |
| Sensor antenna parameters (cont.) |  |  |  |  |
| Maximum beam gain | 34.4 dBi | 54 dBi | 37.9 dBi | 39 dBi |
| Polarization | V, H QV, QH* | V | QH | V |
| –3 dB beamwidth | 3.3° | 0.39° | 2.2° | 2.2 |
| Instantaneous field of view | Nadir FOV: 48.5 km Outer FOV: 149.1 × 79.4 km 147 × 79 km* | 16 km × 12 km | Nadir FOV: 31.6 km Outer FOV: 136.7 × 60 km | Outer FOV 18 × 43 km |
| Main beam efficiency | 95% | 95% | 95% |  |
| Off-nadir pointing angle | ±48.33° cross-track | 46.8° | ±52.725° cross-track | 55.4 |
| Beam dynamics | 8 s scan period | 31.6 rpm | 8/3 s scan period cross-track; 96 earth fields per scan period | 2.88 s scan period |
| Incidence angle at Earth | 0 57.5°* | 55.7° |  | 65 |
| –3 dB beam dimensions | 48 km | 6 km | 32 km | 32 km |
| Swath width | 2 343 km 2 186 km* | 1 700 km | 2 500 km | 2 000 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 |  |  |  |
| Cold calibration ant. gain | 34.4 dBi | N/A | 37.9 dBi | N/A |
| Cold calibration angle (degrees re. satellite track) | 90 −90° ± 3.9°* | N/A | 0 | N/A |
| Cold calibration angle (degrees re. nadir direction) | 83.33 | N/A | 82.175 | N/A |
| Sensor receiver parameters |  |  |  |  |
| Sensor integration time | 165 ms | 1.2 ms | 18 ms | N/A |
| Channel bandwidth | 400 MHz centred at 52.8 GHz 170 MHz centred at 53.596 GHz | 960 MHz centred at 53.57 GHz | 400 MHz centred at 52.8 GHz 170 MHz centred at 53.596 GHz | 400 MHz centred at 52.8 GHz, 53.3 GHz, 53.8 GHz |
| Measurement spatial resolution |  |  |  |  |
| Horizontal resolution | 47 km 48 km* | 6 km | 32 km | 32 km |
| Vertical resolution | 47 km 48 km* | 6 km | 32 km | 32 km |
| NOTE 1 – * Indicates that a particular sensor is flown on different missions, with different orbit and sensor parameters. |  |  |  |  |

## 6.11	Typical parameters of passive sensors operating in the bands between 54.25 and 59.3 GHz

The band 54.25-59.3 GHz is of primary interest for atmospheric temperature profiling (O2 absorption lines). Table 14 summarizes the parameters of passive sensors that are or will be operating between 54.25 and 59.3 GHz. The frequency range from 54.25 to 60.3 GHz is covered by many smaller bands with varying bandwidths and polarizations (see Tables 15 and 16).

TABLE 14

EESS (passive) sensor characteristics operating between 54.25 and 59.3 GHz

|  | Sensor K1 | Sensor K2 | Sensor K3 | Sensor K4 |
| --- | --- | --- | --- | --- |
| Sensor type | Conical scan | Mechanical nadir scan | Mechanical nadir scan | Conical scan |
| Orbit parameters |  |  |  |  |
| Altitude | 828 km | 824 km | 833 km 822 km* | 835 km |
| Inclination | 98.7° |  | 98.6 98.7°* | 98.85° |
| Eccentricity | 0 |  | 0 0.001* | 0 |
| Repeat period | 17 days | 9 days | 9 days 29 days* |  |
| Sensor antenna parameters |  |  |  |  |
| Number of beams | 2 |  | 30 earth fields per 8 s scan period | 1 |
| Reflector diameter | 2.2 m | 0.203 m | 0.15 m | 0.6 m |
| Maximum beam gain | 60 dBi | 37.9 dBi | 34.4 dBi | 51 dBi |
| Polarization | See Table 15 | See Table 16 | See Table 17 | See Table 18 |
| –3 dB beamwidth | 0.39° | 2.2° | 3.3 | 0.6° |
| Instantaneous field of view | 16 km × 12 km | Nadir FOV: 31.6 km Outer FOV: 136.7 × 60 km | Nadir FOV: 48.5 km (3.3) Outer FOV: 149.1 × 79.4 km 147 × 79 km* | Outer FOV 18 × 43 km |
| Main beam efficiency | 95% |  |  |  |
| Off-nadir pointing angle | 46.8° | ±52.73° cross-track | ±48.33° cross-track | 55.4 |
| Beam dynamics | 31.6 rpm | 8/3 s scan period cross-track; 96 earth fields per scan period | 8 s scan period | 2.88 s scan period |
| Incidence angle at Earth | 55.7 |  | 0 57.5* | 65 |

TABLE 14 (end)

|  | Sensor K1 | Sensor K2 | Sensor K3 | Sensor K4 |
| --- | --- | --- | --- | --- |
| Sensor antenna parameters (cont.) |  |  |  |  |
| −3 dB beam dimensions | 3 km | 31.6 km | 48.5 km 48 km* | 18 km × 43 km |
| Swath width | 1 700 km | 2 500 km | 2 343 km | 2 000 km |
| Sensor antenna pattern | See Rec. ITU-R RS.1813 |  |  |  |
| Cold calibration ant. gain | N/A | 37.9 dBi | 34.4 dBi | N/A |
| Cold calibration angle (degrees re. satellite track) | N/A | 0 | 90 −90° ± 3.9°* | N/A |
| Cold calibration angle (degrees re. nadir direction) | N/A | 82.175 | 83.33 | N/A |
| Sensor receiver parameters |  |  |  |  |
| Sensor integration time | 1.2 ms | 18 ms | 165 ms | N/A |
| Channel bandwidth | See Table 15 | See Table 16 | See Table 17 | See Table 18 |
| Measurement spatial resolution |  |  |  |  |
| Horizontal resolution | 3 km | 32 km | 48 km | 18 km |
| Vertical resolution | 3 km | 32 km | 48 km | 18 km |
| NOTE 1 – * Indicates that a particular sensor is flown on different missions, with different orbit and sensor parameters. |  |  |  |  |

TABLE 15

Sensor K1 passive sensor characteristics for channels 
between 54.25 and 60.5 GHz

| Centre frequency (GHz) | Channel bandwidth (MHz) | Polarization |
| --- | --- | --- |
| 54.380 | 440 | V |
| 54.905 | 350 | V |
| 55.490 | 340 | V |
| 56.660 | 300 | V |
| 59.380 | 280 | V |
| 59.940 | 440 | V |
| 60.3712 | 57.6 | L |
| 60.4080 | 16 | L |
| 60.4202 | 8.4 | L |
| 60.5088 | 44.8 | L |
| 60.434776 | 25 | L |

TABLE 16

Sensor K2 passive sensor characteristics for channels 
between 54.25 and 59.3 GHz

| Centre frequency (GHz) | Channel bandwidth (MHz) | Polarization |
| --- | --- | --- |
| 54.4 | 400 | QH |
| 54.94 | 400 | QH |
| 55.5 | 330 | QH |
| 57.290344 | 330 | QH |
| 57.073344, 57.507344 | 78 | QH |
| 57.660544, 57.564544, 57.016144, 56.920144 | 36 | QH |
| 57.634544, 57.590544, 56.990144, 56.946144 | 16 | QH |
| 57.622544, 57.602544, 56.978144, 56.958144 | 8 | QH |
| 57.617044, 57.608044, 56.972644, 56.963644 | 3 | QH |

TABLE 17

Sensor K3 passive sensor characteristics for channels
 between 54.25 and 59.3 GHz

| Centre frequency (GHz) | Channel bandwidth (MHz) | Polarization |
| --- | --- | --- |
| 54.4 | 400 | H, QH* |
| 54.94 | 400 | V, QV* |
| 55.5 | 330 | H, QH* |
| 57.290344 | 330 | H, QH* |
| 57.073344, 57.507344 | 78 | H, QH* |
| 57.660544, 57.564544, 57.016144, 56.920144 | 36 | H, QH* |
| 57.634544, 57.590544, 56.990144, 56.946144 | 16 | H, QH* |
| 57.622544, 57.602544, 56.978144, 56.958144 | 8 | H, QH* |
| 57.617044, 57.608044, 56.972644, 56.963644 | 3 | H, QH* |
| NOTE 1 – * Indicates that a particular sensor is flown on different missions, with different parameters. |  |  |

TABLE 18

Sensor K4 passive sensor characteristics for channels 
between 54.25 and 60.5 GHz

| Centre frequency (GHz) | Channel bandwidth (MHz) | Polarization | Altitude of peak sensitivity (km) |
| --- | --- | --- | --- |
| 54.64 | 400 MHz | V | 10 |
| 55.63 | 400 MHz | V | 14 |
| 57.290344 ± 0.322 ± 0.1 | 50 MHz | V | 20 |
| 57.290344 ± 0.322 ± 0.05 | 20 MHz | V | 25 |
| 57.290344 ± 0.322 ± 0.025 | 10 MHz | V | 29 |
| 57.290344 ± 0.322 ± 0.001 | 5 MHz | V | 35 |
| 57.290344 ± 0.322 ± 0.005 | 3 MHz | V | 42 |

## 6.12	Typical parameters of passive sensors operating in the bands between 86 and 92 GHz

The 86-92 GHz passive sensor band is essential for the measurement of clouds, oil spills, ice, snow, and rain. It is also used as a reference window for temperature soundings near 118 GHz.  Table 19 summarizes the parameters of passive sensors that are or will be operating between 86 and 92 GHz.

TABLE 19

EESS (passive) sensor characteristics operating between 86 and 92 GHz

|  | Sensor L1 | Sensor L2 | Sensor L3 | Sensor L4 | Sensor L5 | Sensor L6 | Sensor L7 | Sensor L8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor type | Conical scan |  |  | Mechanical nadir scan |  |  | Conical scan |  |
| Orbit parameters |  |  |  |  |  |  |  |  |
| Altitude | 867 km | 705 km | 833 km | 833 km 822 km* |  | 824 km | 835 km | 700 km |
| Inclination | 20 | 98.2° | 98.7° | 98.6° 98.7°* |  | 98.7° | 98.85° | 98.2° |
| Eccentricity | 0 | 0.0015 | 0 | 0 0.001* |  | 0 |  | 0.002 |
| Repeat period | 7 days | 16 days | 17 days | 9 days 29 days* |  | 9 days | N/A | 16 days |
| Sensor antenna parameters |  |  |  |  |  |  |  |  |
| Number of beams | 1 | 2 | 1 | 30 earth fields per 8 s scan period | 30 earth fields per 8 s scan period 1 beam (steerable in 90 earth fields per scan period)* | 2 |  |  |
| Reflector diameter | 0.65 m | 1.6 m | 2.2 m | 0.15 m | 0.3 m 0.22 m* | 0.203 m | 0.6 m | 2 m |
| Maximum beam gain | 50 dBi | 60.5 dBi | 56 dBi | 34.4 dBi | 47 dBi 44.8 dBi* | 37.9 dBi | 54 dBi | 62.4 dBi |
| Polarization | H, V |  |  | H QV* |  | QV | H, V |  |
| –3 dB beamwidth | 0.43° | 0.18° | 0.39° | 3.3° | 1.1° | 2.2° | 0.4° | 0.15° |
| Instantaneous field of view | 10 km × 17 km | A: 6.2 km × 3.6 km B: 5.9 km × 3.5 km | 16 km × 12 km | Nadir FOV: 48.5 km Outer FOV: 149.1 × 79.4 km 147 × 79 km* | Nadir FOV: 16 km (1.1) Outer FOV: 53 × 27 km* | Nadir FOV: 31.6 km × 31.6 km Outer FOV: 136.7 × 60 km | 12 km × 28 km | A: 5.1 km × 2.9 km B: 5.0 km × 2.9 km |
| Main beam efficiency | 96.2% | 96% | 95% |  |  |  | N/A | 91% |

TABLE 19 (end)

|  | Sensor L1 | Sensor L2 | Sensor L3 | Sensor L4 | Sensor L5 | Sensor L6 | Sensor L7 | Sensor L8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor antenna parameters (cont.) |  |  |  |  |  |  |  |  |
| Off-nadir pointing angle | 44.5 | 47.5° | 46.98° | ±48.33 cross-track | ±48.95 49.4°* | ±52.725 cross-track | N/A | 47.5° |
| Beam dynamics | 20 rpm | 40 rpm | 31.6 rpm | 8 s scan period | 8/3 s scan period | 8/3 s scan period cross-track; 96 earth fields per scan period | 2.88 s scan period | 40 rpm |
| Incidence angle at Earth | 53.5 | A :55.0° B :54.5° | 55.77° | 30 positions 57.5°* | Various angles from 0 59°* |  | 35° | 55° |
| Swath width | 1 700 km | 1 450 km | 1 700 km | 2 343 km 2 186 km* | 2 343 km 2 193 km* | 2 500 km | 2 000 km | 1 450 km |
| Cold calibration ant. Gain | N/A | 40.4 dBi | N/A | 34.4 dBi | 34.4 dBi 44.8 dBi* | 37.9 dBi | N/A | 43.4 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | 115.5º | N/A | 90 −90° ± 3.9°* | End of scan (at 48.95) −90° ± 3.9°* | 0 | N/A | 115.5º |
| Cold calibration angle (degrees re. nadir direction) | N/A | 97.0º | N/A | 83.33 | 83.33 73.6 (66° to 81°)* | 82.175 | N/A | 97.0º |
| Sensor receiver parameters |  |  |  |  |  |  |  |  |
| Sensor integration time | 2 ms | 1.2 ms |  | 180 ms 165 ms* | 185 ms 18 ms* | 18 ms | N/A | 1.2 ms |
| Channel bandwidth | 2 700 MHz centred at 89 GHz | 3 000 MHz centred at 89 GHz | 6 000 MHz centred at 89 GHz |  | Centred at 89 GHz 500 MHz, each with a bandwidth of 1 000 MHz 2 800 MHz centred at 89 GHz* | 2 000 MHz centred at 87-91.9 GHz | 2 GHz | 3 000 MHz centred at 89 GHz |
| Measurement spatial resolution |  |  |  |  |  |  |  |  |
| Horizontal resolution | 10 km | 3.5 km | 6 km | 40.5 km 48 km* | 40.5 km 16 km* | 32 km | 19 km | 2.9 km |
| Vertical resolution | N/A | 6.1 km | 6 km | 48 km | 16 km | 32 km | 6 km | 5.1 km |
| NOTE 1 – * Indicates that a particular sensor is flown on different missions, with different orbit and sensor parameters. |  |  |  |  |  |  |  |  |

## 6.13	Typical parameters of passive sensors operating in the bands between 114.25 and 122.25 GHz

The band 114.25-122.25 GHz is of primary interest for atmospheric temperature profiling (O2 absorption lines). Table 20 summarizes the parameters of passive sensors that are or will be operating between 114.25 and 122.25 GHz.

TABLE 20

EESS (passive) sensor characteristics operating 
between 114.25 and 122.25 GHz

|  | Sensor M1 |
| --- | --- |
| Sensor type | Limb sounder |
| Orbit parameters |  |
| Altitude | 705 km |
| Inclination | 98.2° |
| Eccentricity | 0.0015 |
| Repeat period | 16 days |
| Sensor antenna parameters |  |
| Number of beams | 2 |
| Reflector diameter | 1.6 m × 0.8 m |
| Maximum beam gain | 60 dBi |
| Polarization | 2 orthogonal |
| –3 dB beamwidth | 0.19° × 0.245 |
| Instantaneous field of view | 6.5 km × 13 km |
| Main beam efficiency | N/A |
| Off-nadir pointing angle | Limb |
| Beam dynamics | N/A |
| Incidence angle at Earth | N/A |
| –3 dB beam dimensions | 3 km |
| Swath width | N/A |
| Sensor antenna pattern | N/A |
| Cold calibration ant. gain | N/A |
| Cold calibration angle (degrees re. satellite track) | N/A |
| Cold calibration angle (degrees re. nadir direction) | N/A |
| Sensor receiver parameters |  |
| Sensor integration time | 0.166 s |
| Channel bandwidth | N/A |
| Measurement spatial resolution |  |
| Horizontal resolution | 13 km |
| Vertical resolution | 6.5 km |

## 6.14	Typical parameters of passive sensors operating in the bands between 148.5 and 151.5 GHz

The 148.5-151.5 GHz passive sensor band is essential for the measurement of N2O, Earth’s surface temperature, and cloud parameters. It is also used as a reference window for temperature soundings. Table 21 summarizes the parameters of passive sensors that are or will be operating between 148.5 and 151.5 GHz.

TABLE 21

EESS (passive) sensor characteristics operating 
between 148.5 and 151.5 GHz

|  | Sensor N1 |
| --- | --- |
| Sensor type | Cross-track nadir scan |
| Orbit parameters |  |
| Altitude | 705 km |
| Inclination | 98.2° |
| Eccentricity | 0.0015 |
| Repeat period | 16 days |
| Sensor antenna parameters |  |
| Number of beams | 1 |
| Reflector diameter | 0.219 m |
| Maximum beam gain | 45 dB |
| Polarization | Linear |
| –3 dB beamwidth | 1.1° |
| Main beam efficiency | > 95% |
| Off-nadir pointing angle | ±48.95° |
| Beam dynamics | Scan period of 8/3 s |
| Incidence angle at Earth | 56.9 |
| –3 dB beam dimensions | 13.5 km |
| Swath width | 1 650 km |
| Sensor antenna pattern | See Fig. 11 |
| Cold calibration ant. gain | 45 dB |
| Cold calibration angle (degrees re. satellite track) | 90° |
| Cold calibration angle (degrees re. nadir direction) | 65-81 |
| Sensor receiver parameters |  |
| Sensor integration time | 18 ms |
| Channel bandwidth | 4 000 MHz @ 150 GHz |
| Measurement spatial resolution |  |
| Horizontal resolution | 13.5 km |
| Vertical resolution | 13.5 km |

FIGURE 11

Sensor N1 antenna pattern for the 148.5 and 151.5 GHz band

## 6.15	Typical parameters of passive sensors operating in the bands between 155.5-158.5 GHz

The band 155.5-158.5 GHz is of primary interest to measure Earth and cloud parameters. Table 22 summarizes the parameters of passive sensors that are or will be operating between 155.5-158.5 GHz.

TABLE 22

EESS (passive) sensor characteristics operating between 155.5-158.5 GHz

|  | Sensor O1 | Sensor O2 |
| --- | --- | --- |
| Sensor type | Conical scan | Cross-track nadir scan |
| Orbit parameters |  |  |
| Altitude | 865 km | 822 km |
| Inclination | 20 | 98.7° |
| Eccentricity | 0 | 0.001 |
| Repeat period | 7 days | 29 days |
| Sensor antenna parameters |  |  |
| Number of beams |  | 1 |
| Reflector diameter | 0.65 m | 0.22 m |
| Maximum beam gain | 60 dBi | 44.8 dBi |
| Polarization | H, V | QV |
| –3 dB beamwidth |  | 1.1° |
| Instantaneous field of view |  | Nadir FOV: 16 km Outer FOV: 53 × 27 km |
| Main beam efficiency | 96% | 95% |
| Off-nadir pointing angle | 44.5° | 49.45° |
| Beam dynamics | 20 rpm | Scan period of 8/3s |
| Incidence angle at Earth | 52.3° | 59° |
| –3 dB beam dimensions | 3 km | 16 km |
| Swath width |  | 2 193 km |
| Sensor antenna pattern |  |  |
| Cold calibration ant. gain | N/A | 44.8 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | −90° ± 3.9° |
| Cold calibration angle (degrees re. nadir direction) | N/A | 73.6 (66° to 81°) |
| Sensor receiver parameters |  |  |
| Sensor integration time | N/A | 18 ms |
| Channel bandwidth | 2 GHz | < 2.8 GHz |
| Measurement spatial resolution |  |  |
| Horizontal resolution | 6 km | 16 km |
| Vertical resolution | 6 km | 16 km |

## 6.16	Typical parameters of passive sensors operating in the bands between 164 and 167 GHz

The band 164-167 GHz is of primary interest to measure N2O, cloud water and ice, rain, CO, and ClO. Table 23 summarizes the parameters of passive sensors that are or will be operating between 164 and 167 GHz.

TABLE 23

EESS (passive) sensor characteristics operating between 164 and 167 GHz

|  | Sensor P1 | Sensor P2 |
| --- | --- | --- |
| Sensor type | Conical scan | Mechanical nadir scan |
| Orbit parameters |  |  |
| Altitude | 828 km | 824 km |
| Inclination | 98.7° |  |
| Eccentricity | 0 |  |
| Repeat period | 17 days | 9 days |
| Sensor antenna parameters |  |  |
| Number of beams | 2 |  |
| Reflector diameter | 0.48 × 0.71 m | 0.127 m |
| Maximum beam gain | 54 dBi | 43.9 dBi |
| Polarization | V | QH |
| –3 dB beamwidth | 0.39° | 1.1° |
| Instantaneous field of view | 16 km × 12 km | Nadir FOV: 15.8 km Outer FOV: 68.4 × 30 km |
| Main beam efficiency | 95% |  |
| Off-nadir pointing angle | 46.8° | ±52.725° cross-track |
| Beam dynamics | 31.6 rpm | 8/3 s scan period cross-track; 96 earth fields per scan period |
| Incidence angle at Earth | 55.5° | 0 |
| –3 dB beam dimensions | 6 km | 1.116 km |
| Swath width | 1 700 km | 2 500 km |
| Sensor antenna pattern |  |  |
| Cold calibration ant. gain | N/A | 43.9 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | 0 |
| Cold calibration angle (degrees re. nadir direction) | N/A | 82.175 |
| Sensor receiver parameters |  |  |
| Sensor integration time | 1.2 ms | 18 ms |
| Channel bandwidth | 1 425 MHz centred at 166 ± 0.7875 GHz | 3 000 MHz centred at 164-167 GHz |
| Measurement spatial resolution |  |  |
| Horizontal resolution | 12 km | 32 km |
| Vertical resolution | 12 km | 32 km |

## 6.17	Typical parameters of passive sensors operating in the bands between 174.8 and 191.8 GHz

The 174.8-191.8 GHz passive sensor band is essential for N2O and O3 measurements, in addition to water vapour profiling. Table 24 summarizes the parameters of passive sensors that are or will be operating between 174.8 and 191.8 GHz.

TABLE 24

EESS (passive) sensor characteristics operating between 174.8 and 191.8 GHz

|  | Sensor Q1 | Sensor Q2 | Sensor Q3 | Sensor Q4 | Sensor Q5 | Sensor Q6 | Sensor Q7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor type | Conical scan | Cross-track scan | Limb sounder | Mechanical nadir scan | Conical scan | Nadir scan |  |
| Orbit parameters |  |  |  |  |  |  |  |
| Altitude | 828 km | 705 km |  | 824 km | 835 km | 867 km | 822 km |
| Inclination | 98.7° | 98.2° |  | 98.7° | 98.85° | 20 | 98.7° |
| Eccentricity | 0 |  |  |  |  |  | 0.001 |
| Repeat period | 17 days | 16 days |  | 9 days | N/A | 7 days | 29 days |
| Sensor antenna parameters |  |  |  |  |  |  |  |
| Number of beams | 2 | 1 | 2 | 96 earth fields per scan period | 6 |  | 1 (steerable in 90 earth fields per scan period) |
| Reflector diameter | 0.48 × 0.71 m | 0.219 m | 1.6 × 0.8 m | 0.127 m | 0.6 m | 0.2 m | 0.22 m |
| Maximum beam gain | 54 dBi | 45 dBi | 60 dBi | 43.9 dBi | 60 dBi | 49 dBi | 44.8 dBi |
| Polarization | V | Linear | V | QH | V | H | QV |
| −3 dB beamwidth | 0.39° | 1.1° | 0.19° × 0.245 | 1.1° | 0.2° | 0.66° | 1.1° |
| Instantaneous field of view | 16 km × 12 km | 14 km | 4.5 km × 9 km | Nadir FOV: 15.8 km Outer FOV: 68.4 × 30 km | Outer FOV: 8 × 19 km | At nadir 10 km × 10 km At swath limit 14 km × 22 km | Nadir FOV: 16 km Outer FOV: 53 × 27 km |
| Main beam efficiency | 95% |  | N/A | 95% | N/A | 97% | 95% |
| Off-nadir pointing angle | 46.8° | ±48.95° | N/A | ±52.725° cross-track | 55.4° | 42 | 49.4 |

TABLE 24 (continued)

|  | Sensor Q1 | Sensor Q2 | Sensor Q3 | Sensor Q4 | Sensor Q5 | Sensor Q6 | Sensor Q7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor antenna parameters (cont.) |  |  |  |  |  |  |  |
| Beam dynamics | 31.6 rpm | 8/3 s scan period | Scans continuously in tangent height from the surface to ~92 km in 24.7 s 240 scans/ orbit | 8/3 s scan period cross-track | 2.88 sscan period | 1 revolution per 1.639 s | 8/3 s scan period cross-track |
| Incidence angle at Earth | 55.5 | 56.9° | N/A |  | 65° | 55 | 59 |
| –3 dB beam dimensions | 3 km | 13.5 km | 3 km | 16 km | 8 km × 19 km | 10 km × 10 km | 16 km |
| Swath width | 1 700 km | 1 650 km | N/A | 2 500 km | 2 000 km | 1 700 km | 2 193 km |
| Sensor antenna pattern |  | See Fig. 12 |  |  |  |  |  |
| Cold calibration ant. gain | N/A | 45 dB | N/A | 43.9 dBi | N/A |  | 44.8 dBi |
| Cold calibration angle (degrees re. satellite track) | N/A | 90° | N/A | 0 | N/A |  | −90° ± 3.9° |
| Cold calibration angle (degrees re. nadir direction) | N/A | 65° to 81° | N/A | 82.175 | N/A |  | 73.6 (66° to 81°) |

TABLE 24 (end)

|  | Sensor Q1 | Sensor Q2 | Sensor Q3 | Sensor Q4 | Sensor Q5 | Sensor Q6 | Sensor Q7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor receiver parameters |  |  |  |  |  |  |  |
| Sensor integration time | 1.2 ms | 18 ms | 0.166 s | 18 ms | N/A | 7.34 ms | 18 ms |
| Channel bandwidth | 1 275 MHz centred at 183.31 ± 0.7875 GHz, 3 500 MHz centred at 183.31 ± 3.1 GHz, 4 500 MHz centred at 183.31 ± 7.7 GHz | 1 000 MHz centred at 183.31 ± 1.00 GHz, 2 000 MHz centred at 183.31 ± 3.00 GHz, 4 000 MHz centred at 183.31 ± 7.00 GHz | N/A | See Table 25 | 1.5 GHz centred at 183.31 ± 7 GHz, 1.0 GHz centred at 183.31 ± 3 GHz, 0.5 GHz centred at 183.31 ± 1 GHz | 6 channels from 200 MHz to 2 GHz centred at 183.31 GHz | 0.5 GHz centred at 183.311 ± 1 GHz, 1.0 GHz centred at 183.311 ± 3 GHz, 1.1 GHz centred at 190.311 ± 1 GHz |
| Measurement spatial resolution |  |  |  |  |  |  |  |
| Horizontal resolution | 6 km | 13.5 km | 9 km | 16 km | 8 km | 10 km cross-track | 16 km |
| Vertical resolution | 6 km | 13.5 km | 4.5 km | 16 km | 8 km | 10 km | 16 km |

FIGURE 12

Sensor Q2 antenna pattern for the 174.8 and 191.8 GHz band

TABLE 25

Sensor Q4 passive sensor characteristics for channels 
between 174.8 and 191.8 GHz

| Centre frequency (GHz) | Channel bandwidth (MHz) | Polarization |
| --- | --- | --- |
| 183.31 ± 4.5 | 2 000 | QH |
| 183.31 ± 1.8 | 1 000 | QH |
| 190.31 | < 2 200 | V |

*	This Recommendation should be brought to the attention of Radiocommunication Study Group 1.

The “vadose zone” is the portion of Earth between the land surface and the zone of saturation which extends from the top of the ground surface to the water table.