RECOMMENDATION  ITU-R  S.1328-1

SATELLITE  SYSTEM  CHARACTERISTICS  TO  BE  CONSIDERED  IN  FREQUENCY
SHARING  ANALYSES  BETWEEN  GEOSTATIONARY-SATELLITE  ORBIT  (GSO)
AND  NON-GSO  SATELLITE  SYSTEMS  IN  THE
FIXED-SATELLITE  SERVICE  (FSS)  INCLUDING  FEEDER  LINKS
FOR  THE  MOBILE-SATELLITE  SERVICE  (MSS)*

(Questions ITU-R 205/4, ITU-R 206/4 and ITU-R 231/4)

(1997-1999)

Rec. ITU-R S.1328-1

The ITU Radiocommunication Assembly,

considering

a)	that the World Radiocommunication Conference (Geneva, 1995) (WRC-95), in Resolutions 116 (WRC-95) and 117 (WRC-95), allocated frequencies to the FSS for use by feeder links of non-GSO MSS systems;

b)	that WRC-95, in Resolution 118 (WRC-95), provided for parts of the 30/20 GHz bands in the FSS to be used by the non-GSO FSS without the restrictions of Radio Regulations (RR) No. S22.2;

c)	that WRC-95, in Resolution 120 (WRC-95), provided for parts of the 30/20 GHz band in the FSS to be shared with feeder links of the non-GSO MSS;

d)	that WRC-95, in Resolution 121 (WRC-95), advocated the development of interference criteria and methodology for sharing between feeder links of the non-GSO MSS and networks of the GSO FSS;

e)	that the World Radiocommunication Conference (Geneva, 1997) in Resolution 130 (WRC-97), advocated the development of interference criteria and methodology for sharing between non-GSO FSS and networks of the GSO FSS,

recommends

1	that, in the planning and development of new FSS networks, both GSO and non-GSO, and feeder links for MSS systems affecting the FSS allocations, the technical characteristics of existing and planned satellite systems in Annexes 1 to 11 be taken into consideration;

2	that, in studies pertaining to the development of sharing criteria between satellite systems, the technical characteristics of existing and planned systems in Annexes 1 to 11 be used in interference analyses;

3	that administrations planning modifications to these systems or proposing future satellite system networks in FSS bands are urged to submit their technical characteristics to the ITU-R to update this data source.

ANNEX  1

TABLE  1

Technical characteristics of several LEO and GSO satellite networks

| Parameters | Non-GSO MSS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LEO A | LEO B | LEO C | LEO D | LEO E |  | LEO F | LEO G |
| 1 Orbital parameters |  |  |  |  |  |  |  |  |
| Shape of orbit | Circular | Circular | Circular | Circular | Elliptical | Circular | Circular | Circular |
| Height (km) | 780 | 10 355 | 2 000 | 1 414 | 7 846  520 | 80-90 | 10 355 | 1 500 |
| Inclination angle (degrees) | 86 | 50 | 55 | 52 | 116.6 | 0 | 45 | 74 |
| Coherence (track repeat (h)) | – | – | – | 47.5 | 3 | 4.8 | – | – |
| Number of satellites per plane | 11 | 4 | 5 | 6 | 5 | 7 | 5 | 12 |
| Number of orbital planes | 6 | 3 | 8 | 8 | 2 | 1 | 2 | 4 |
| Satellite separation within plane (degrees) | 32.7 | 90 | 45 | 60 | 72 | 51 | – | 30 |
| Satellite phasing between planes (degrees) | 31.6 (22) | 30 | – | 7.5 | 36 | – | 0 | 90 |
| 2 Targeted frequency range and polarization |  |  |  |  |  |  |  |  |
| Uplink frequency (GHz) | 29.1-29.3 | 29.1-29.5 | 5.091-5.250 | 5.091-5.250 | 15.45-15.65 |  | 5.100-5.250 | 14 |
| Uplink polarization | RHCP | LHCP | RHCP/LHCP | LHCP/RHCP | – |  | – | LHCP |
| Downlink frequency (GHz) | 19.4-19.6 | 19.3-19.7 | 6.875-7.075 | 6.875-7.055 | 6.875-7.075 |  | 6.925-7.075 | 11 |
| Downlink polarization | LHCP | RHCP | RHCP/LHCP | LHCP/RHCP | – |  | – | RHCP |
| 3 Spectrum required in each direction (MHz) | 200 | 400 | 200 | 159/180 | 200 |  | 100 | 50 |
| 4 Carrier transmission parameters |  |  |  |  |  |  |  |  |
| Modulation type | FDMA/QPSK Rate ½, coded Rate 6.25 Mbit/s | CDMA | CDMA/FDMA | CDMA/FDMA | QPSK | QPSK | TDMA/QPSK | CDMA/FDMA |
| Number of service link beams |  | – | 32 | 16 | 61 | 37 | 16,3 | – |
| Number of feeder-link segments/polarization |  | – | 1 | 8 | 31 | 31 | – | – |
| Segment bandwidth (MHz) |  | – | 12 | 16.5 | 12 | 12 | – | – |
| Receiver bandwidth (kHz) | 3 000 | 2 500 | 200 | 1 230 | 3 000/7 000 | 3 000/7 000 | 25 | 5 800 |
| Transmission bandwidth (kHz) | 4 370 | 2 500 | 2 500 | 1 230 | 3 000/7 000 | 3 000/7 000 | 25 | 5 800 |
| Overall (C/N0) per user (dB/Hz) or (C/N) (dB) | – | – | 44.7-46.6 | 44 | – | – | 48 | 42 (Eb/N0) |

TABLE  1 (continued)

| Parameters | Non-GSO MSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LEO A |  | LEO B |  | LEO C |  | LEO D |  | LEO E |  |  | LEO F |  | LEO G |
| 4 Carrier transmission parameters (continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink e.i.r.p./carrier (dBW) | ARC 34.0 to 43.5 |  | 54.25 |  | 40.2 |  | 54 |  | 50 | 50 |  | 47.5 |  | 49 |
| Downlink e.i.r.p./carrier (dBW) | ARC 4.5 to 15.0 |  | 5.31 |  | –8.5 |  | –5 |  | – | – |  | 0 |  | –6 (nadir) |
| Type of satellite transponder | Regenerative |  | Transparent |  | Transparent |  | Transparent |  | Transparent | Transparent |  | – |  | – |
| 5 Satellite antenna parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tx maximum gain (dBi) | 26.9, 2 dB axial ratio |  | 35.7 |  | 3 |  | 2 |  | 11 | 15 |  | 13 |  | 5 (nadir) |
| Rx maximum gain (dBi) | 30.1, 2 dB axial ratio |  | 38.5 |  | 3 |  | 2 |  | 11 | 15 |  | 10 |  | 5 (nadir) |
| Main lobes | RR Appendix S8 |  | – |  | –3 dB |  | ISO flux  2 dB |  | – | – |  | RR Appendix S30B |  | – |
| Side lobes | RR Appendix S8 |  | – |  | –3 dB |  | –14 dB |  | –16 dB | –16 dB |  | – |  | – |
| Back lobes | – |  | – |  | –10 dB |  | –35 dB |  | –38 dB | –38 dB |  | – |  | – |
| Steerable antenna or not | (Programmed pointing) Steerable (4/SAT) |  | Steerable |  | No |  | No |  | No | No |  | No |  | No |
| 6 Earth station antenna parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Peak Tx gain (dBi) | 56.3 (beamwidth 0.24) | 64.8 |  | 47.7 |  | 47.5 |  | 55.3 |  |  | 47.8 |  | 41 |  |
| Peak Rx gain (dBi) | 53.2 (beamwidth 0.36) | 60.8 |  | 50.2 |  | 50.0 |  | 48.2 |  |  | 50.7 |  | 42 |  |
| Radiation pattern | RR Appendix S8 | Rec. ITU-R S.465 |  | Rec. ITU-R S.580 |  | Rec. ITU-R S.465 |  | – |  |  | Rec. ITU-R S.465 |  | 32.3 – 25 log  |  |
| Minimum operating elevation angle (degrees) | 5Acquisition 8.3 Service (auto tracking) | 10 |  | 10 |  | 10 |  | 5 |  |  | 5 |  | 10 |  |
| 7 Number of earth stations and distribution | 25 ww 7 USA | 8-12 |  | 50-100 |  | 100-200 |  | 20-40 |  |  | 6-30 |  | 3 or more |  |
| 8 Earth station switching strategy | Nom. 10 minimum, make before break |  minimum elevation angle |  | Highest elevation angle |  | Select highest elevation angle |  | Highest and 2nd highest elevation angle |  |  |  minimum elevation angle |  |  |  |

TABLE  1 (continued)

| Parameters | Non-GSO FSS |  |  |  | LEO N user and feeder-links |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LEO N |  | MEO V |  | User links | Feeder-links |
| 1 Orbital parameters |  |  |  |  |  |  |
| Shape of orbit | Circular |  | Circular |  | Circular |  |
| Height (km) | 700 |  | 10 360 |  | 700 |  |
| Inclination angle (degrees) | 82 |  | 82.5 |  | 82 |  |
| Coherence (track repeat (h)) | 46 |  | 24 |  | 46 |  |
| Number of satellites per plane | 13 |  | 6 |  | 13 |  |
| Number of orbital planes | 7 |  | 4 |  | 7 |  |
| Satellite separation within plane (degrees) | 27.7 |  | 60 |  | 27.7 |  |
| Satellite phasing between planes (degrees) | 25.7 |  | 45 |  | 25.7 |  |
| 2 Targeted frequency range and polarization |  |  |  |  |  |  |
| Uplink frequency (GHz) | 12.75-13.25 |  | 29.5-29.9 |  | 1.98-2.01 2.675-2.69 | 19.3-19.6 |
| Uplink polarization | Circular |  | Circular |  | Circular | Circular |
| Downlink frequency (GHz) | 10.7-10.95 |  | 19.8-20.2 |  | 2.17-2.20 | 15.43-15.63 |
| Downlink polarization | Circular |  | Circular |  | Circular | Circular |
| 3 Spectrum required in each direction (MHz) | 200 |  | 400 |  | 45(1)/30(2) | 300(1)/200(2) |
| 4 Carrier transmission parameters |  |  |  |  |  |  |
| Modulation type | FDM/TDMA/QPSK |  | FDM/CDMA/TDMA; FDM/TDMA/QPSK |  | CDMA/QPSK | TDMA/QPSK |
| Number of service link beams | 37 |  | 625 |  | 37 | – |
| Number of feeder-link segments/polarization | – |  | – |  | – | 1 |
| Segment bandwidth (MHz) | – |  | – |  | – | – |
| Receiver bandwidth (kHz) | Up 50 000(3); 4 160(4) | Down 50 000(3); 4 920(4) | Up 40 000(3); 50 000(4) | Down 50 000(3); 40 000(4) | 1 250(1), 15 000(2) | 20 000(1); 20 000(2) |
| Transmission bandwidth (kHz) | 50 000(3); 4 160(4) | 50 000(3); 4 920(4) | 40 000(3); 50 000(4) | 50 000(3); 40 000(4) | 1 250(1), 15 000(2) | 20 000(1); 20 000(2) |
| Overall (C/N0) per user (dB/Hz) or (C/N) (dB) | 6.5 dB (Eb/N0) |  | 6 dB (Eb/N0) |  | 6.5 dB (Eb/N0) | 6.5 dB (Eb/N0) |
| Uplink e.i.r.p./carrier (dBW) | 53.7(3); 40.7(4) |  | 57-67 (ARC)(3); 45.8-55.8 (ARC)(4) |  | 1.5 | 67.2 |
| Downlink e.i.r.p./carrier (dBW) | 90º:10.5; 65º:12.3; 45º:16.2; 20º:20(3) 90º:11.9; 65º:13.7; 45º:17.6; 20º:21.4(4) |  | 43.6(3), 50.4(4) |  | 31.7 | 15.8 |
| Type of satellite transponder | On-board processing |  | Transparent |  | Processing | Processing |

TABLE  1 (continued)

| Parameters | Non-GSO FSS |  | LEO N user and feeder-links |  |
| --- | --- | --- | --- | --- |
|  | LEO N | MEO V | User links | Feeder-links |
| 5 Satellite antenna parameters |  |  |  |  |
| Tx maximum gain (dBi) | 16.4 | 43.9 | 16.4 | 5.2 |
| Rx maximum gain (dBi) | 16.4 | 43.9 | 16.4 | 5.2 |
| Main lobes | – | – | – | – |
| Side lobes | – | – | – | – |
| Back lobes | – | – | – | – |
| Steerable antenna or not | Yes | Yes | No | No |
| 6 Earth station antenna parameters |  |  |  |  |
| Peak Tx gain (dBi) | 45(3); 32(4) | 55.6(3); 43.2(4) | 3 | 48.4 |
| Peak Rx gain (dBi) | 43.5(3); 32(4) | 52.1(3); 39.7(4) | 3 | 48.4 |
| Radiation pattern | Rec. ITU-R S.580(3) Rec. ITU-R S.465(4) | Rec. ITU-R S.580(3) Rec. ITU-R S.580(4) | Not yet defined | Rec. ITU-R S.465 |
| Minimum operating elevation angle (degrees) | 10 | 40 | 10 | 10 |
| 7 Number of earth stations and distribution | – | – | Up to 4 million | Up to several dozens |
| 8 Earth station switching strategy | By programme | By programme |  minimum operating elevation angle | Maximum duration of communication session |

TABLE  1 (continued)

| Parameters | Non-GSO/MSS | GSO FSS |  |  |  |  | GSO MSS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LEO | GSO 1 | GSO 2 | GSO 3 | GSO 13 | GSO 20 |  |
| 1 Orbital parameters |  |  |  |  |  |  |  |
| Shape of orbit | Circular |  |  |  | Circular | Circular | Circular |
| Height (km) | 1 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 |
| Inclination angle (degrees) | 83 | – | – | – | 0 | – | – |
| Coherence (track repeat (h)) | – | – | – | – | – | – | – |
| Number of satellites per plane | 1 | – | – | – | 17 | – | – |
| Number of orbital planes | 7 | – | – | – | 1 (GSO) | – | – |
| Satellite separation within plane (degrees) | – | – | – | – | At least 2 for co-channel sharing | – | – |
| Satellite phasing between planes (degrees) | 51.4 | – | – | – | Not applicable | – | – |
| 2 Targeted frequency range and polarization |  |  |  |  |  |  |  |
| Uplink frequency (GHz) | 7 | 6 | 6 | 14 | 30 | 30 | 30 |
| Uplink polarization | RHCP | Circular | Circular | V and H | Circular with re-use | LHCP/RHCP | LHCP/RHCP |
| Downlink frequency (GHz) | 5 | 4 | 4 | 12 | 20 | 20 | 20 |
| Downlink polarization | LHCP | Circular | Circular | V and H | Circular with re-use | LHCP/RHCP | RHCP |
| 3 Spectrum required in each direction (MHz) | 50 up 66 down | 500 | 500 | 500 | 1 000-2 500 | 1 000 | 500 |
| 4 Carrier transmission parameters |  |  |  |  |  |  |  |
| Modulation type | CDMA/FDMA | FM/TV | 64 kbit/s | FM/TV | FDMA up/FDMA down | FDMA/QPSK | FDMA/QPSK |
| Number of service link beams | – | – | – | – | 48 | 8 | 4 |
| Number of feeder-link segments/polarization | – | – | – | – | Not applicable | – | – |
| Segment bandwidth (MHz) | – | – | – | – | Not applicable | – | – |

TABLE  1 (continued)

| Parameters | Non-GSO/MSS | GSO FSS |  |  |  |  | GSO MSS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LEO | GSO 1 | GSO 2 | GSO 3 | GSO 13 | GSO 20 |  |
| 4 Carrier transmission parameters (continued) |  |  |  |  |  |  |  |
| Receiver bandwidth (kHz) | 2 050 | 30 000 | 51.2 | 27 000 | 500 uplink/ 120 000 downlink | 241 up | 1 800 |
| Transmission bandwidth (kHz) | 2 050 | 30 000 | 51.2 | 27 000 | 340 uplink/ 81 000 downlink | 1 800 down | 1 800 |
| 5 Satellite antenna parameters |  |  |  |  |  |  |  |
| Tx maximum gain (dBi) | 3 |  |  |  | 46.5/35.0 | 48.9 | 40.9 |
| Rx maximum gain (dBi) | 3 |  |  |  | 46.5/35.0 | 48.9 | 40.9 |
| Main lobes | – |  |  |  | ex-CCIR Report 558 | RR Appendix S30B | RR Appendix S30B |
| Side lobes | – |  |  |  | ex-CCIR Report 558 | – | – |
| Back lobes | – |  |  |  | ex-CCIR Report 558 | – | – |
| Steerable antenna or not | No |  |  |  | No, fixed pointed array | Yes | Yes |
| 6 Earth station antenna parameters |  |  |  |  |  |  |  |
| Peak Tx gain (dBi) | 43.1 | 57.8 | 51.6 | 62.3 | 44.5/53.3 | 34.0-43.5 | 37.5-43.5 |
| Peak Rx gain (dBi) | 48.5 | 54.0 | 47.7 | 60.2 | 44.5/53.3 | 30.5-40.0 | 34.0-40.0 |
| Radiation pattern | 29 – 25 log  | – | – | – | 29 – 25 log  | 32 – 25 log  | 32 – 25 log  |
| Minimum operating elevation angle (degrees) | 10 | – | – | – | 5 | 10 | 10 |
| 7 Number of earth stations and distribution | 4 or more |  |  |  | Unlimited, through urban and suburban areas and certain rural areas |  |  |
| 8 Earth station switching strategy |  |  |  |  | Not needed/used | Not applicable | Not applicable |

TABLE  1 (continued)

| Parameters | Non-GSO FSS |  |  | FSS |  |
| --- | --- | --- | --- | --- | --- |
|  | MEO J | MEO K | LEO SAT-1 | GSO 30 | QUASI-GSO 31 |
| 1 Orbital parameters |  |  |  |  |  |
| Shape of orbit | Circular | Circular | Circular | Circular | Elliptical |
| Height (km) | 13 900 | 13 900 | 700 | 36 000 | 1 000-43 000 |
| Inclination angle (degrees) | 75 | 75 | 98.2 | – | 63 |
| Coherence (track repeat (h)) | 8 | 8 | Not applicable | – | 12 |
| Number of satellites per plane | 1 | 1 | 40 | 1 | 1 |
| Number of orbital planes | 9 | 9 | 21 | 12 | 8 |
| Satellite separation within plane (degrees) | Not applicable | Not applicable | 9 | – | – |
| Satellite phasing between planes (degrees) | 73.5 | 73.5 | Random | – | Varies |
| 2 Targeted frequency range and polarization |  |  |  |  |  |
| Uplink frequency (GHz) | 29.5-30 | 27.6-28.6 | 28.6-29.1 | 30 | 30 |
| Uplink polarization | LHCP/RHCP | LHCP/RHCP | LHCP/RHCP | RHCP/LHCP | RHCP/LHCP |
| Downlink frequency (GHz) | 19.7-20.2 | 17.8-18.8 | 18.8-19.3 | 19 | 19 |
| Downlink polarization | LHCP/RHCP | LHCP/RHCP | LHCP/RHCP | RHCP/LHCP | RHCP/LHCP |
| 3 Spectrum required in each direction (MHz) | 500 | 1 000 | 500 | 1 000-3 200 | 1 000-3 200 |
| 4 Carrier transmission parameters |  |  |  |  |  |
| Modulation type | QPSK FDMA/TDMA up TDM down | TCM TDM | Shaped QPSK FDMA up/ TDMA down | Phase | Phase |
| Number of service link beams | 256 | 256 | < 49 per satellite | – | – |
| Number of feeder-link segments/polarization | – | – | – | – | – |
| Segment bandwidth (MHz) | – | – | – | – | – |

TABLE  1 (end)

| Parameters | Non-GSO FSS |  |  | FSS |  |
| --- | --- | --- | --- | --- | --- |
|  | MEO J | MEO K | LEO SAT-1 | GSO 30 | QUASI-GSO 31 |
| 4 Carrier transmission parameters (continued) |  |  |  |  |  |
| Receiver bandwidth (kHz) | 250 000 | 125 000 | 0.275-35.2 (MHz) | 3 200 000 | 3 200 000 |
| Transmission bandwidth (kHz) | 125 000 | 125 000 | 500 (MHz) | 3 200 000 | 3 200 000 |
| 5 Satellite antenna parameters |  |  |  |  |  |
| Tx maximum gain (dBi) | – | – | 29.8 (90), 30.9, 32.0 (40) | 55 | 55 |
| Rx maximum gain (dBi) | – | – | 29.8 (90), 30.9, 32.0 (40) | 55 | 55 |
| Main lobes | – | – | – | ex-CCIR Report 558 | ex-CCIR Report 558 |
| Side lobes | – | – | – | ex-CCIR Report 558 | ex-CCIR Report 558 |
| Back lobes | – | – | – | ex-CCIR Report 558 | ex-CCIR Report 558 |
| Steerable antenna or not | No | No | 64 steerable beams | Yes | Yes |
| 6 Earth station antenna parameters |  |  |  |  |  |
| Peak Tx gain (dBi) | 45 | 62.1 | 36 | 70 | 70 |
| Peak Rx gain (dBi) | 41.5 | 58.6 | 33 | 70 | 70 |
| Radiation pattern | 29 – 25 log  | 29 – 25 log  | RR Appendix S8, Annex III | RR Appendix S8 | RR Appendix S8 |
| Minimum operating elevation angle (degrees) | – | 10 | 40 | 5 | 5 |
| 7 Number of earth stations and distribution |  90° of coverage latitude bounds Unlimited within coverage |  90° of coverage latitude bounds | Up to 20 million, worldwide | Global | Global |
| 8 Earth station switching strategy | Best elevation | – | Track nearest satellite |  |  |
| ARC: Automatic Range Compensation LHCP: left-hand circular polarization RHCP: right-hand circular polarization (1) Uplink. (2) Downlink. (3) Feeder link/gateway. (4) Service link/user station. |  |  |  |  |  |

ANNEX  2

TABLE  2

Technical characteristics of feeder links of GSO MSS satellite networks

| Parameters | GSO-C | GSO-D | GSO-E | GSO-F | GSO-G | GSO-H |
| --- | --- | --- | --- | --- | --- | --- |
| 2 Frequency range (GHz) |  |  |  |  |  |  |
| Uplink | 12.75-13.25 | 12.75-13.25 | 6 | 28.75-28.6/29.5-30.0 | 27.5-29.5 | 27.5-29.5 |
| Downlink | 10.7-10.95 | 11.2-11.45 | 4 | 18.55-18.80/19.7-20.2 | 18.4-19.7 | 18.4-19.7 |
| 3 Spectrum requirements in each direction (MHz) | 200 | 150 | 50 | 250/500 up 250/500 down | 1 300 | 1 300 |
| 4 Carrier transmission parameters |  |  |  |  |  |  |
| Carrier type | 600HG1ECF, 2K40G1EDF, 5K25G1EDF, 12K0G1EDF, 56K0G1EDF | 2K40G1EDF, 4K80G1EDF, 32K0M7EDT, 144KG1EDF, 1M25G1EDC | QPSK/TDMA | FDM/TDMA/QPSK up TDM/QPSK down | QPSK | QPSK |
| Receiver bandwidth (kHz) | 0.75-70 | 3-90 | 30 | 125 000 | 76.8 | 76.8 |
| Allocated bandwidth (kHz) | 5-100 | 5-1 250 | 33 | 125 000 | 84.5 | 84.5 |
| Overall (C/N0) (dB(Hz)) | 32-58 | 41-57 | 48 | 19.4 up/14.1 down per user | 66.6 | 66.6 |
| Uplink e.i.r.p./carrier (dBW) | 34-47 | 36-52.0 | 53.2 | 53.2 | 67.5 | 61.5 |
| Downlink e.i.r.p./carrier (dBW) | –8 to 5 | –2 to 14 | 3.8 | 61.8 | 40.6 | 34.6 |
| 5 Satellite antenna parameters |  |  |  |  |  |  |
| Peak Tx gain (dBi) | 30 at all frequencies | 30 at all frequencies | 20 | 49.0 | – | – |
| 3 dB bandwidth (degrees) | – | – | 17.8 | – | – | – |
| Peak Rx gain (dBi) | – | – | 20 | 49.0 | – | – |
| 3 dB bandwidth (degrees) | – | – | 17.8 | – | – | – |
| Sidelobe gain or pattern |  |  | RR Appendix S30B standard | RR Appendix S30B | Rec. ITU-R S.672 | Rec. ITU-R S.672 |
| Roll-off pattern |  |  | RR Appendix S30B standard | – | Square law | Square law |
| Steerable antenna or not | None | None | No | Fixed points |  |  |
| 6 Earth station antenna parameters |  |  |  |  |  |  |
| Maximum Tx gain (dBi) | 61.3 at 13 GHz | 61.3 at 13 GHz | 50.2 | 57.2 | 49.2 | 55.2 |
| Maximum Rx gain (dBi) | 60.0 at 11 GHz | 60.0 at 11 GHz | 45.5 | 53.5 | 45.7 | 51.7 |
| Radiation pattern | Rec. ITU-R S.580 | Rec. ITU-R S.580 | Rec. ITU-R S.580 | Rec. ITU-R S.465 | Rec. ITU-R S.580 | Rec. ITU-R S.580 |
| Minimum elevation angle (degrees) | 25 | 25 | 5 | 10 | 10 | 10 |
| 7 Number of earth stations and distribution | Diversity pair | Diversity pair | Up to 30 | Unlimited | – | – |

TABLE  3

Technical characteristics of GSO satellite networks using FSS frequency bands

| 1a Frequency band (GHz) | 6 /4 (Conv.) |  |  |  | 6/4 (Allot.) | 14/12 (Conv.) |  | 14/12 (Allot.) | 30/20 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1b System | GSO 1 | GSO 2 | GSO 5 | GSO 6 | GSO 9 | GSO 3 | GSO 7 | GSO 10 | GSO 11 | GSO 12 | GSO 13 |
| Number of co-located satellites | 1 |  |  |  | 1 | 1 |  | 1 | 1 | 1 | 2 |
| Number of beams/satellite and polarization | 2 and RHCP + 2 and LHCP |  |  |  |  | V and H |  | V and H | 7V and 7H | 1 circular polarization | 24 and RHCP  24 and LHCP |
| 2 Spectrum required in each direction (MHz) | 500 |  |  |  | 300 | 500 |  | 500 | 800 | 1 000 | 1 000-2 500 |
| 3a Uplink carrier | FM/TV |  | 64 kbit/s IDR |  |  | FM/TV | 64 kbit/s |  | 147 Mbit/s | 65 Mbit/s | 384 kbit/s |
| Beam identification | Global | Hemi | Global | Hemi |  | Spot | Spot |  | Spot | Spot | Spot (global) |
| Occupied bandwidth | 30 MHz | 30 MHz | 51.2 kHz | 51.2 kHz |  | 27 MHz | 51.2 kHz |  | 110 MHz | 110 MHz | 500 kHz |
| Minimum required C/N or Eb/N0 (dB) |  |  |  |  |  |  |  |  |  |  | 8 |
| 3b Downlink carrier | Same as uplink |  |  |  | Same as uplink | Same as uplink |  | Same as uplink | Same as uplink |  | Data musk |
| Occupied bandwidth (MHz) | Same as uplink |  |  |  | Same as uplink | Same as uplink |  | Same as uplink | Same as uplink |  | 120 |
| Minimum required C/N or Eb/N0(1) (dB) | 17.7 |  | 9.7 |  |  | 17.7 | 9.7 |  |  |  | 5 |
| 4 Uplink parameters |  |  |  |  |  |  |  |  |  |  |  |
| Transmitter power to antenna (dBW) |  |  |  |  |  |  |  |  | 20 |  | –11.5 to –3.5 (power control) |
| Transmitting antenna size (m) |  |  |  |  |  |  |  |  | 5 | 11.5 | 0.66 |
| Transmitting antenna gain (dBi) | 57.8 | 57.8 | 51.6 | 51.6 | 52.5 | 62.3 | 55.5 | 50.7 | 61.9 | 69.0 | 44.3 |
| e.i.r.p. (dBW) | 85.4 | 87.8 | 48.3 | 46.1 | 6.6(2) | 86.3 | 40.9 | 14.6(2) | 81.9 | 91.0 | 32.8-40.8 (power control) |
| Peak system G/T (dB(K–1)) |  |  |  |  |  |  |  |  | 21.8 |  | 18.9 |
| Receiving antenna beamwidth (degrees) |  |  |  |  |  |  |  |  |  |  | 0.9 |
| Receiving antenna side-lobe pattern |  |  |  |  |  |  |  |  |  |  |  |
| Steerable or not |  |  |  |  |  |  |  |  |  |  | No |

TABLE  3 (end)

| 5 Downlink parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter power to antenna (dBW) |  |  |  |  |  |  |  |  | 6 |  | 12.5 |
| Peak transmitting antenna gain (dBi) |  |  |  |  |  |  |  |  | 49.5 | 33.0 | 46.5 |
| Peak e.i.r.p. (dBW) | 30.5 | 35.0 | 0.5 | 0.9 | –35.6(2) | 50.0 | 7.7 | –21(2) | 55.5 | 39.0 | 59 |
| Transmitting antenna beamwidth (degrees) |  |  |  |  |  |  |  |  | 0.44 |  | 1.4 |
| Receiving antenna size (m) |  |  |  |  |  |  |  |  | 5 |  | 0.66 |
| Peak receiving antenna gain (dBi) | 54.0 | 54.0 | 47.7 | 47.7 | 49.1 | 60.2 | 53.5 | 49.4 | 58.4 | 33.0 | 41.0 |
| 6 Earth segment |  |  |  |  |  |  |  |  |  |  |  |
| Number of earth stations |  |  |  |  |  |  |  |  |  |  | 600 000 |
| Distribution of earth stations |  |  |  |  |  |  |  |  |  |  | Home and business location |
| IDR: intermediate data rate. (1) For GSO 1-GSO 12, this is the C/N for the overall link (uplink and downlink). (2) The e.i.r.p. is given in dB(W/Hz). The total e.i.r.p. is given in Annex 1 of Appendix S30B as the indicated e.i.r.p. density averaged over the necessary bandwidth. |  |  |  |  |  |  |  |  |  |  |  |

ANNEX  3

TABLE  4

Technical characteristics of planned 30/20 GHz GSO FSS networks

| Satellite system | A | A' | B | J | K | L | M | N | S | T | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General parameters |  |  |  |  |  |  |  |  |  |  |  |
| Nominal frequency (GHz) | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 |
| Number of transponders per satellite | 64 | 64 | 48 | 48 | 40 |  |  |  |  |  | 48 |
| Transponder bandwidth (MHz) | 120 | 250 | 120 | 120 | 120 | 120 | 120 | 24 | 120 | 54 | 36 |
| Transponder high power amplifier (W) | 30 | 30 | 20 | 30 | 60 |  |  | 95 |  | 40-60 | 60-90 |
| Satellite e.i.r.p. (dBW)(1) | 61 | 61 | 59 | 61 | 61 | 60 | 60.2 | 54 | 58 | 61 | 51-55 |
| Polarization(2) | Circular | Circular | Circular | Circular | Circular | Circular | Circular | Circular | Circular | Linear | Linear |
| Downlink parameters |  |  |  |  |  |  |  |  |  |  |  |
| Modulation/access | TDM | CDMA | TDM | TDM | TDM | TDM | TDM | TDM | TDM | PSK/ MCPC | PSK/ MCPC |
| Central frequency (GHz) | 19.5 | 19.5 | 19.5 | 19.5 | 19.7 | 19.5 | 19.95 | 19.5 | 20.0 | 19.92 | 18.95 |
| Data rate (Mbit/s) | 92 | 95.04 | 92 | 155 | 92 | 155 | 130 | 40 | 90 | 51.84 | 38.88 |
| Digital service(3) | Narrow-band data | Narrow-band data | Narrow-band data | Narrow-band data | Narrow-band data | Narrow-band data | Narrow-band data | Narrow-band data | Narrow-band data | SC/TV | SC/TV |
| e.i.r.p. spectral density (dB(W/Hz)) | –23.6 | –26.6 | –21.3 | –25.9 | –20.8 | –20.8 | –20.9 | –22.0 | –22.6 | –16.3 | –18.0 |
| Uplink parameters |  |  |  |  |  |  |  |  |  |  |  |
| Central frequency (GHz) | 29.5 | 29.5 | 29.5 | 29.5 | 29.5 | 29.5 | 29.75 | 29.5 | 30.0 | 29.72 | 28.75 |
| Modulation/access | CDMA | CDMA | FDM | FDM | FDM | FDM | FDM | FDM | FDM | TV/MCPC | TV/MCPC |
| Nominal rate (kbit/s) | 384 | 384 | 384 | 384 | 384 | 384 | 128 | 384 | 384 | 51 840 | 38 880 |
| Minimum earth station antenna diameter (m) | 0.66 | 0.66 | 0.66 | 0.66 | 0.70 | 0.70 | 0.65 | 0.75 | 0.70 | 4.6/0.6 | 4.6/0.6 |
| Up/down earth station antenna discrimination at 2° (dB) | 22.5/19 | 22.5/19 | 22.5/19 | 22.5/19 | 23/19.5 | 23/19.5 | 22.5/19 | 23.5/20 | 23/19.5 | 40/18 | 40/18 |
| Maximum power spectral density (dB(W/Hz)) | –61.7 | –70 | –60.5 | –58.9 | –57.9 | –57.9 | –65.9 | –63.7 | –58.9 | –61.1 | –61.9 |

TABLE  4 (end)

| Satellite system | V | W | X | Y | Z |
| --- | --- | --- | --- | --- | --- |
| General parameters |  |  |  |  |  |
| Nominal frequency (GHz) | 20/30 | 20/30 | 20/30 | 20/30 | 20/30 |
| Number of transponders per satellite |  |  |  |  | 360(4) |
| Transponder bandwidth (MHz) | 125 |  |  |  | 25-120 |
| Transponder high power amplifier (W) | 40 |  |  |  |  |
| Satellite e.i.r.p. (dBW)(1) | 60-62 | 22.6 | 62.8 |  | 70(5) |
| Polarization(2) | Circular |  |  |  | Linear |
| Downlink parameters |  |  |  |  |  |
| Modulation/access | TDM | TDM | FDM |  | TDM/FDM/CDMA |
| Central frequency (GHz) | 20 | 20 | 20.2 | 17.7-20.2 | (6) |
| Data rate (Mbit/s) | 92.16 | 0.064 | 1.544/0.384 |  | To be determined |
| Digital service(3) | Narrow-band data | Narrow-band data | Narrow-band data/video |  |  |
| e.i.r.p. spectral density (dB(W/Hz)) | –19.1 | –25.5 | –23.1/–16.0 | –63/–38 | –4.1 |
| Uplink parameters |  |  |  |  |  |
| Central frequency (GHz) | 30 | 30 | 30 | 27.5-30.0 | (6) |
| Modulation/access | FDM | FDM | FDM |  | TDM/FDM/CDMA |
| Nominal rate (kbit/s) | 384 | 64 | 1 544/384 |  |  |
| Minimum earth station antenna diameter (m) | 0.70 | 0.60 | 1.0/0.3 |  | 0.3-12.0 |
| Up/down earth station antenna discrimination at 2°(dB) | 23/19.5 | 21.5/18 | 26/12 |  | Rec. ITU-R S.580 |
| Maximum power spectral density (dB(W/Hz)) | –58.8 | –45.1 | –65.9/–56.8 | –40 | –44.0 |
| MCPC: multi-channel per carrier SC: single carrier (1) Estimated peak e.i.r.p. values of spot beams, with edge of coverage of spot beams from 3 to 5 dB below peak. (2) Left- and right-hand circular polarization used for up and downlinks. (3) Narrow-band digital data rates may include mixtures of 64, 128, 384 and 1 544 kbit/s. (4) Co-location of six or more satellites providing a total of 360 transponders with 25 MHz bandwidth. Where larger bandwidths are used the number of transponders will vary. (5) In compliance with No. 2578 of the Radio Regulations (edition of 1994). (6) Uplink frequency (GHz): 18.1-18.2/27.5-28.0/28.0-31.0. Downlink frequency (GHz): 21.9-22.0(7), 21.4-21.9(7)/18.2-21.2. (7) Downlink operation in accordance with RR Resolution 525 (WARC-92). |  |  |  |  |  |

TABLE  5

Technical characteristics of planned 30/20 GHz GSO FSS networks
Satellite system P

| Type of satellite/service | GSO FSS |  |  |  |
| --- | --- | --- | --- | --- |
| Type of transponder | Transparent |  | Regenerative |  |
| Type of carrier | Ka-1 | Ka-2 | Ka-3 | Ka-4 |
| Up and/or downlink | Up and down | Up and down | Up | Down |
| Frequencies (GHz) Earth-to-space/space-to-Earth | 30/20 | 30/20 | 30/20 | 30/20 |
| Uplink e.i.r.p. (dBW) Minimum Maximum | 73.0 77.0 | 66.1 70.1 | 37.7 41.7 | – – |
| Earth station peak Tx gain (dBi) | 63.7 | 63.7 | 45.1 | – |
| Uplink free-space path length (dB) Minimum Maximum | 213.1 214.2 | 213.1 214.2 | 213.1 214.2 | 213.1 214.2 |
| Downlink e.i.r.p. (dBW) Maximum Minimum | 62.0 58.0 | 57.0 53.0 | – – | 62.5 58.5 |
| Satellite peak Tx/Rx gain (dBi) | 53.2 | 53.2 | 47.7 | 46.2 |
| Earth station peak Rx gain (dBi) | 60.1 | 60.1 | – | 41.6 |
| Earth station Tx/Rx diameter (m) | 6.0/6.0 | 6.0/6.0 | 0.66/– | –/0.66 |
| Satellite Tx/Rx 3 dB beamwidth (degrees) | 0.3/0.3 | 0.3/0.3 | –/0.6 | 0.6/– |
| Occupied bandwidth (MHz) | 186.6 | 186.6 | 0.333 | 115 |
| Downlink free-space path length (dB) Minimum Maximum | 209.5 210.6 | 209.5 210.6 | 209.5 210.6 | 209.5 210.6 |
| Earth station elevation angle to satellite (degrees) Minimum Maximum | 10 90 | 10 90 | 10 90 | 10 90 |
| Intersatellite links (Yes or No) | Yes | Yes | Yes | Yes |
| On-board processing (Yes or No) | No | No | Yes | Yes |
| Message type (voice, data, video, paging, messaging, etc.) | All | All | All | All |
| Reference information data rate (Mbit/s)(1) | 155.52 | 155.52 | 0.384 | 120 |
| Modulation | QPSK | QPSK | QPSK | QPSK |
| FEC (type) | 1/2 convolutional | 1/2 convolutional | 3/4 convolutional and Reed-Solomon | 2/3 convolutional and Reed-Solomon |
| Spectral shaping factor | 1.2 | 1.2 | 1.2 | 1.2 |
| Network performance (design values): BER and performance mask 4% of year 1% of year 0.6% of year 0.5% of year 0.04% of year | 1  10–9 Not applicable 1  10–8 Not applicable 1  10–6 | 1  10–9 Not applicable 1  10–8 Not applicable 1  10–6 | Not applicable 1  10–10 Not applicable 1  10–8 Not applicable | Not applicable 1  10–10 Not applicable 1  10–8 Not applicable |
| BER threshold Required C/N (dB) Required margin (dB) | 1  10–6 6.9 18 | 1  10–6 6.9 12 | 1  10–8 5.8 2.5 | 1  10–8 4.8 2.5 |
| (1) The above information data rates may vary according to traffic/user requirements, and should be taken as reference for general analysis. |  |  |  |  |

TABLE  6

Technical characteristics of a planned 30/20 GHz GSO/FSS networks using on-board processing
Satellite system Q

|  | Residential terminals | Business VSAT | Gateway |
| --- | --- | --- | --- |
| 1 Frequency range and polarization |  |  |  |
| Uplink frequency (GHz) | 29.5-30.0 | 28.35-30.0 | 28.35-29.5 |
| Uplink polarization | LHCP/RHCP | LHCP/RHCP | LHCP/RHCP |
| Downlink frequency (GHz) | 19.7-20.2 | 18.55-20.2 | 18.55-19.7 |
| Downlink polarization | LHCP/RHCP | LHCP/RHCP | LHCP/RHCP |
| Spectrum in each direction (MHz) | Up to 500 | Up to 1 500 | Up to 1 000 |
| Transponder bandwidth (MHz) | 110-120 | 110-120 | 110-120 |
| 2 Uplink transmission parameters |  |  |  |
| Access technique | MF-TDMA | MF-TDMA | TDMA |
| Modulation type | O-QPSK/PFMO | O-QPSK/PFMO | O-QPSK/PFMO |
| User bit rate (Mbit/s) | 0.512-2.048 | 2.048 | 65.536 |
| Nominal carrier bandwidth (MHz) | 0.8-3.0 | 3.0 | 110.0-115.0 |
| Transmit earth station size (m) | 0.45-0.75 | 1.2-1.8 | 3.0 |
| Transmit earth station gain (dBi) | 41.0-45.5 | 49.5-53.0 | 57.5 |
| Uplink e.i.r.p. per carrier (dBW) | 44.0-48.5 | 49.5-53.0 | 72.5-79.5 |
| Typical clear sky C/N0 (dB/Hz) | 70.5-74.5 | 82.0 | 103.0 |
| 3 Satellite antenna parameters |  |  |  |
| Beamwidth | 0.55 diameter | 0.55 diameter | 0.55 diameter |
| Beam shape | Circular | Circular | Circular |
| Maximum Rx/Tx gain (dBi) | 49.0 | 49.0 | 49.0 |
| Radiation pattern | Rec. ITU-R S.672 | Rec. ITU-R S.672 | Rec. ITU-R S.672 |
| Peak satellite G/T (dB(K–1)) | 20.0 | 20.0 | 20.0 |
| Peak satellite e.i.r.p. (dBW) | 65.0 | 65.0 | 65.0 |
| Type of transponder | On-board processing | On-board processing | On-board processing |
| 4 Downlink transmission parameters |  |  |  |
| Modulation type | TDM/QPSK | TDM/QPSK | TDM/QPSK |
| User bit rate (Mbit/s) | 65.536 | 65.536 | 65.536 |
| Nominal carrier bandwidth (MHz) | 109.5 | 109.5 | 109.5 |
| Receive earth station size (m) | 0.45-0.75 | 1.2-1.8 | 3.0 |
| Receive earth station gain (dBi) | 37.5-42.0 | 46.0-49.5 | 54.0 |
| Receive earth station G/T (dB(K–1)) | 14.5-19.0 | 23.0-26.5 | 31.5 |
| Typical clear sky C/N0 (dB/Hz) | 95.0 | 99.0 | 102.0 |
| 5 Earth station parameters |  |  |  |
| Expected number of earth stations | Millions | Thousands | Less than 50 |
| Minimum elevation angle (degrees) | 10 | 10 | 20 |
| Antenna pattern | 32 – 25 log  | 32 – 25 log  | 29 – 25 log  |
| 6 Fade compensation strategy | UPC, adaptive FEC, rate reduction | UPC, adaptive FEC, rate reduction | UPC, adaptive FEC, site diversity |
| PFMO: pulse frequency modulation (offset) UPC: upward power control |  |  |  |

APPENDIX  1

TO  ANNEX  3

TABLE  7

Typical one-way link availability objectives (BER of 1  10–9)

| Connection between | One-way availability (%) |
| --- | --- |
| Residential terminal and residential terminal | 99.5 |
| Residential terminal and business VSAT | 99.6 |
| Residential terminal and gateway | 99.7 |
| Business terminal and business terminal | 99.7 |
| Business terminal and gateway | 99.8 |
| Gateway and gateway | 99.8-99.9 |

ANNEX  4

TABLE  8

Technical characteristics of some current 30/20 GHz GSO FSS networks
Satellite system R

| Satellite system | R-1 | R-2 | R-3 | R-4 | R-5 | R-6 |
| --- | --- | --- | --- | --- | --- | --- |
| General parameters |  |  |  |  |  |  |
| Nominal frequency (GHz) | 20/30 |  |  |  |  |  |
| Number of transponders per satellite(1) | 10 |  |  |  |  |  |
| Transponder bandwidth (MHz) | 100 |  |  |  |  |  |
| Transponder high power amplifier (W) | 29 |  |  |  |  |  |
| Satellite e.i.r.p. (dBW)(2) | 55 |  |  |  |  |  |
| Polarization | Circular |  |  |  |  |  |
| Downlink parameters |  |  |  |  |  |  |
| Modulation/access | TDM | FDM | FDM | FDM | FDM | FM |
| Central frequency (GHz)(3) |  |  |  |  |  |  |
| Data rate (Mbit/s) | 20.4 | 0.032 | 0.032 | 1.544 | 7.2 | Not applicable |
| Digital service | Voice, data and fax | Voice, data and fax | Voice, data and fax | Voice, data and fax | Digital video | TV-FM |
| e.i.r.p. spectral density (dB(W/Hz)) | –9.1 | –3.1 | –5.3 | –14.6 | –17.0 | –3.9(4) |
| Uplink parameters |  |  |  |  |  |  |
| Central frequency (GHz)(3) |  |  |  |  |  |  |
| Modulation/access | TDMA | FDMA | FDMA | FDMA | FDMA | FM |
| Nominal rate (Mbit/s) | 20.4 | 0.032 | 0.032 | 1.544 | 7.2 | Not applicable |
| Minimum earth station antenna diameter (m) | 7.6 | 1.4 | 3.6 | 3.6 | 1.4 | 3.0 |
| Up/down earth station antenna discrimination at 2° (dB) | Rec. ITU-R S.465 |  |  |  |  |  |
| Maximum power spectral density (dB(W/Hz)) | –31.1 | –23.3 | –25.3 | –27.6 | –28.2 | –14.0(4) |
| (1) Include 3 transponders connected to spot beams. (2) Peak e.i.r.p. of national beam. (3) Not specified. (4) Modulated by 1 MHz peak-to-peak energy dispersal signal. |  |  |  |  |  |  |

ANNEX  5

TABLE  9

Technical characteristics of a planned 10.95-11.2, 11.45-11.7/13.75-14.5 GHz non-GSO FSS network: FSAT-MULTI 1-B

| 1 Orbital parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Shape of orbit | Circular |  |  |  |
| Height (km) | 1 457 |  |  |  |
| Inclination angle (degrees) | 55 |  |  |  |
| Coherence (track repeat (h)) | 665 |  |  |  |
| Number of satellites per plane | 4 |  |  |  |
| Number of orbital planes | 16 |  |  |  |
| Satellite separation (degrees) within plane | 90 |  |  |  |
| Satellite phasing between planes | Not applicable |  |  |  |
| 2 Targeted frequency range and polarization |  |  |  |  |
| Uplink frequency (GHz) | 13-14 |  |  |  |
| Uplink polarization | Circular |  |  |  |
| Downlink frequency (GHz) | 11-12 |  |  |  |
| Downlink polarization | Circular |  |  |  |
| 3 Spectrum required in each direction (MHz) | 1 000 within the above frequency range |  |  |  |
| 4 Carrier transmission parameters |  |  |  |  |
| Modulation type | SSMA |  |  |  |
| Number of service link beams | < 45 |  |  |  |
| Number of feeder-link segments/polarization | – |  |  |  |
| Segment bandwidth (MHz) | – |  |  |  |
| Receiver bandwidth (kHz) | Forward: 41 000; return: 5 200 |  |  |  |
| Transmission bandwidth (kHz) | Forward: 41 000; return: 5 200 |  |  |  |
| Overall (C/N0) per user (dB/Hz) or (C/N) (dB) | 4 dB (Eb/N0) |  |  |  |
| Uplink e.i.r.p./carrier (dBW) | 63.8(1) | 68(1) (13.75-14 GHz) |  | 35.5(2) |
| Downlink e.i.r.p./carrier (dBW) | 90: 17.5(2) 4.8(1) | 75: 19.7(2) 7.1(1) | 50: 21.2(2) 8.6(1) | 32: 23.1(2) 10.2(1) |
| Type of satellite transponder | Transparent |  |  |  |
| 5 Satellite antenna parameters |  |  |  |  |
| Tx maximum gain (dBi) | 90: 17.2 | 75: 19.8 | 50: 21.7(2) | 32: 23 |
| Rx maximum gain (dBi) | 90: 16.9 | 75: 19.5 | 50: 21.4 | 32: 22.7 |
| Main lobes | – |  |  |  |
| Side lobes | – |  |  |  |
| Back lobes | – |  |  |  |
| Steerable antenna or not | Yes |  |  |  |
| 6 Earth station antenna parameters |  |  |  |  |
| Peak Tx gain (dBi) | 51.6(1) | 54.1(1) (13.75-14 GHz) |  | 33(2) |
| Peak Rx gain (dBi) | 50.5(1) | 53(1) (13.75-14 GHz) |  | 31.8(2) |
| Radiation pattern | Rec. ITU-R S.580 |  |  | – |
| Minimum operating elevation angle (degrees) | 5 |  |  | 10 |
| 7 Number of earth stations and distribution | Up to 20 millions |  |  |  |
| 8 Earth station switching strategy | Track best elevation satellite within the operating range |  |  |  |
| (1) Feeder link/gateway. (2) Service link/user station. |  |  |  |  |

TABLE  10

Technical characteristics of a planned 30/20 GHz non-GSO satellite network: FSAT-MULTI 1-A

a)	Orbital parameters

The satellites are distributed between two types of planes, as shown in the following Table:

| Number of planes | 161 |
| --- | --- |
| Number of satellites per plane | 1 |
| Right ascension node (degrees) | 0  1 315 i for i  0-160 |
| Altitude (km) | 1 675 |
| Eccentricity | 0 |
| Inclination (degrees) | 87 1133 |

b)	Communication parameters

| 2 Frequency range (GHz) |  |  |  |
| --- | --- | --- | --- |
| Uplink frequency | 27.5-30.0 |  |  |
| Downlink frequency | 17.3-20.2 |  |  |
| 4 Carrier transmission parameters |  |  |  |
| Modulation type | QPSK  Viterbi and Reed-Solomon coding |  |  |
| Information rate (Mbit/s) | 0.384 | 2.048 | 33.0 |
| Transmission bandwidth (MHz) | 0.500 | 2.66 | 42.9 |
| Required Eb/N0 (dB) | 6.0 | 6.0 | 6.0 |
| Uplink e.i.r.p./carrier (dBW) Minimum Maximum (depends on the antenna diameter and on the elevation angle (rain fades compensation)) | 32.4 | 29.5 53.4 |  |
| Downlink e.i.r.p./carrier (dBW) depends on the elevation angle (3 dB edge of the beam value) | – | – | 41 at 20, 36 at 30, 31.2 at 50, 29.2 at 70, 28.9 at 90 |
| Type of satellite transponder | On-board processing |  |  |
| 5 Satellite antenna parameters |  |  |  |
| Tx maximum gain (dBi) (beam edge) | 31.7 at 20, 28.6 at 30, 24.5 at 50, 22.7 at 70, 22.4 at 90 |  |  |
| Rx maximum gain (dBi) (beam edge) | 31.7 at 20, 28.6 at 30, 24.5 at 50, 22.7 at 70, 22.4 at 90 |  |  |
| Main/side/back lobes |  |  |  |
| 6 Earth station antenna parameters |  |  |  |
| Radiation pattern | Rec. ITU-R S.580 (side lobes), RR Appendix S30B (main beam) |  |  |
| Minimum operating elevation angle (degrees) (depends on the latitude) | 20 at the Equator |  |  |

ANNEX  6

TABLE  11

Additional parameters for the feeder links of the non-GSO MSS and FSS systems of LEO-G

|  | FL MSS | FL FSS | FSS |
| --- | --- | --- | --- |
| 1 Orbital parameters |  |  |  |
| Shape of orbit |  | Circular |  |
| Height (km) |  | 1 500 |  |
| Inclination angle (degrees) |  | 74 |  |
| Satellites per plane |  | 12 |  |
| Number of orbital planes |  | 4 |  |
| Satellite separation within plane (degrees) |  | 30 |  |
| Satellite phasing between planes (degrees) |  | 90 |  |
| 2 Targeted frequency range and polarization |  |  |  |
| Uplink frequency (GHz) | 19.3-19.6 | 28.6-29.1 | 28.6-29.1 |
| Uplink polarization | LHCP | LHCP | LHCP |
| Downlink frequency (GHz) | 15.45-15.65 | 18.8-19.3 | 18.8-19.3 |
| Downlink polarization | RHCP | RHCP | RHCP |
| 3 Spectrum required in each direction (MHz) | 200 | 300 | 200 |
| 4 Carrier transmission parameters |  |  |  |
| Receiver bandwidth (kHz) | 48 000 | 64 000 | 32 000 |
| Transmission bandwidth (kHz) | 48 000 | 64 000 | 32 000 |
| Overall C/N0 per user (dB/Hz) | 46 | 46 | 46 |
| Uplink e.i.r.p./carrier (dBW) Maximum Minimum | 67 29.6 | 63.9 28.2 | 60.9 28.2 |
| Downlink e.i.r.p./carrier (dBW) Maximum Minimum | 24.9 –3.8 | 29.1 2.0 | 29.7 5.6 |
| 5 Satellite antenna parameters |  |  |  |
| Tx maximum gain (dBi) | 22 | 30 | 30 |
| Rx maximum gain (dBi) | 22 | 30 | 30 |
| Main, side and back lobes |  |  |  |
| Steerable antenna or not | Yes | Yes | Yes |
| 6 Earth station antenna parameters |  |  |  |
| Peak Tx gain (dBi) | 49 | 49 | 45 |
| Peak Rx gain (dBi) | 49 | 49 | 45 |
| Radiation pattern |  |  |  |
| Minimum operating elevation angle (degrees) | 10 | 10 | 10 |
| 7 Number of earth stations and distribution | 6 or more | 6 or more | Multiple |
| 8 Earth station switching strategy |  minimum elevation angle |  |  |

ANNEX  7

TABLE  12

Technical characteristics of two current 30/20 GHz GSO FSS networks: Ka-J1 and Ka-J2

| Satellite system | Ka-J1 | Ka-J2 |
| --- | --- | --- |
| General parameters |  |  |
| Nominal frequency (GHz) | 20/30 | 20/30 |
| Number of transponders per satellite | 5 | 6 |
| Transponder bandwidth (MHz) | 100 | 200 |
| Input power of satellite transmit antenna (dBW) | 11 | 11 |
| Satellite e.i.r.p. (dBW) | 52 | 58 |
| Polarization | Circular | Circular |
| Digital service (voice, data, video, paging, etc.) | All | All |
| Downlink parameters |  |  |
| Modulation/access | QPSK/TDMA | QPSK/TDM |
| Central frequency (GHz)(1) |  |  |
| Occupied bandwidth (MHz) | 35 | 140 |
| Earth station antenna diameter (m): Minimum Maximum | 3.0 11.5 | 1.8 4.2 |
| e.i.r.p. spectral density (dB(W/Hz)) | –23.4 | –23.5 |
| Uplink parameters |  |  |
| Modulation/access | QPSK/TDMA | QPSK/TDM |
| Central frequency (GHz) |  |  |
| Occupied bandwidth (MHz) | 35 | 140 |
| Earth station antenna diameter (m): Minimum Maximum | 3.0 11.5 | 1.2 4.2 |
| Maximum power spectral density (dB(W/Hz)) | 4.6 | –1.9 |
| (1) Not specified. |  |  |

TABLE  13

Technical characteristics of a planned 30/20 GHz GSO FSS network: GSO-EKX

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Circular |
| Height (km) | 35 786 |
| Inclination angle (degrees) | 0 inclination to equatorial plane |
| Number of satellites per plane | 8-2 each in 4 orbit slots |
| Requested orbital locations | 99° E, 117° W, 69° W, 26.2° W |
| Satellite separation (degrees) within plane | > 2 |
| 2 Frequency range and polarization |  |
| 3 Spectrum required (GHz) | 1.0 for United States of America domestic network 1.5 for international network |
| 4 Carrier transmission parameters |  |
| Uplink Earth-to-space centre frequencies (United States of America) (GHz) | 28.475, 29.375, 29.625 and 29.875 |
| Downlink space-to-Earth centre frequencies (United States of America) (GHz) | 17.925, 18.175, 18.425, 18.675, 19.825 and 20.075 |
| Uplink Earth-to-space centre frequencies (Europe and S-E Asia) (GHz) | 27.975, 28.225, 28.475, 29.375, 29.625 and 29.875 |
| Downlink space-to-Earth centre frequencies (Europe and S-E Asia) (GHz) | 17.925, 18.175, 18.425, 18.675, 19.825 and 20.075 |
| Number of service link beams | 32 dual polarization, receive and transmit |
| Modulation type | QPSK |
| User access | FDMA/TDMA |
| Receiver bandwidth (MHz) | 250 |
| Number of FDMA carriers per beam | 8, 4  2 polarizations |
| Number of TDMA users | Up to 100 T1 users |
| Overall C/N0 per user (dB/Hz) | 89.5 |
| Uplink e.i.r.p./carrier in clear (dBW) | 62.2 |
| Uplink e.i.r.p./carrier in rain (dBW) | 67.2 |
| Downlink e.i.r.p./carrier (dBW) | 56.4 |
| 5 Satellite antenna parameters |  |
| Tx maximum gain (dBi) | 48.4 |
| Rx maximum gain (dBi) | 48.4 |
| Tx side lobes (dB) | –15 relative to peak |
| Rx side lobes (dB) | –20 relative to peak |
| 6 Earth station antenna parameters |  |
| Tx maximum gain (dBi) | 58.2 (typical value) |
| Rx maximum gain (dBi) | 54.8 (typical value) |
| Tx side lobes (dB) | –15 relative to peak |
| Rx side lobes (dB) | –20 relative to peak |
| 7 Number of earth stations and distribution | Number of sites is unlimited. Distribution of earth stations shall be in North and South America, Europe, Africa, South-East Asia and Australia, plus sites for transpacific and transatlantic |
| 8 Earth station switching strategy | Interference avoidance while operating at the highest possible elevation angle |

TABLE  14

Technical characteristics of a planned 30/20 GHz non-GSO FSS network: NGSO-KX

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Circular |
| Height (km) | 10 352 |
| Inclination angle (degrees) | 55 |
| Repeatable track | Approximately every 6 h |
| Number of satellites per plane | 5 |
| Number of orbital planes | 4 |
| Satellite separation (degrees) within plane | 72 |
| 2 Frequency range and polarization |  |
| Uplink frequency (GHz) | 28.6-29.1 |
| Uplink polarization | LHCP/RHCP |
| Downlink frequency (GHz) | 18.8-19.3 |
| Downlink polarization | LHCP/RHCP |
| 3 Spectrum required in each direction (MHz) | 500 |
| 4 Carrier transmission parameters |  |
| Modulation type | O-QPSK |
| Number of service link beams | 20 |
| Receiver bandwidth (MHz) | 3-200 |
| Transmission bandwidth (kHz) | 3-200 |
| Overall C/N | Depends on antenna size: uplink at 30° elevation: over 6 dB for users downlink at 30° elevation: over 6 dB |
| Uplink e.i.r.p./carrier (dBW) | 37-45 |
| Downlink e.i.r.p./carrier (dBW) | 55-60 |
| Type of satellite transponder | On-board processing |
| 5 Satellite antenna parameters |  |
| Minimum antenna gain (dBi) | 39 |
| Beamwidth | 1° at the 3 dB point |
| Steerable antenna or not | Yes |
| 6 Earth station antenna parameters |  |
| Peak Tx gain (dBi) | 39.7-53.8 (for 36 cm to 2 m) |
| Peak Rx gain (dBi) | 35.9-50.1 (for 36 cm to 2 m) |
| Radiation pattern | Rec. ITU-R S.465 |
| Minimum operating elevation angle (degrees) | 30 |
| 7 Number of earth stations and distribution | Global/unlimited |
| 8 Earth station switching strategy | Interference avoidance/highest elevation angle |

TABLE  15

Technical characteristics of a planned 30/20 GHz system: LEOSAT-2

| 1 Orbital parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Satellites |  |  | 63 |  |  |  |
| Shape of orbit |  |  | Circular |  |  |  |
| Planes |  |  | 7 |  |  |  |
| Altitude |  |  | 1 400 km (870 miles) |  |  |  |
| Inclination (degrees) |  |  | 48 |  |  |  |
| Plane phasing (degrees) |  |  | +28.57 |  |  |  |
| Orbit period |  |  | 6 825 s (1.9 h) |  |  |  |
| Usable capacity (Gbit/s) |  |  | 80 |  |  |  |
| Service region at 16° elevation angle |  |  | 60° N and S latitude extends to 70° by mitigating affect of elevation angle (view or antenna) |  |  |  |
| Peak d.c. power (kW) |  |  | 13.6 |  |  |  |
| Average d.c. power (kW) |  |  | 4.6 |  |  |  |
| Mission life (years) |  |  | 8 (10 expendable) |  |  |  |
| Stabilization and position sensing |  |  | 3-axis stabilized; GPS |  |  |  |
| Dimensions (length width height, stored) |  |  | 3  2  5 m (120  80  200 in) |  |  |  |
| Wet mass |  |  | 3 100 kg (6 834 lb) |  |  |  |
| Dry mass |  |  | 2 500 kg (5 512 lb) |  |  |  |
| Propellant |  |  | 600 kg (1 323 lb) |  |  |  |
| 2 Frequency range and polarization |  |  |  |  |  |  |
| Number of beams per satellite - uplink |  |  | 432 |  |  |  |
| Frequency – uplink (GHz) |  |  | 28.6-29.1 and 29.5-30.0 |  |  |  |
| Number of beams per satellite – downlink |  |  | 260 |  |  |  |
| Frequency – downlink (GHz) |  |  | 18.8-19.3 and 19.7-20.2 |  |  |  |
| Number of optical inter-satellite links per satellite |  |  | 6 |  |  |  |
| Optical intersatellite link data rate (Gbit/s) |  |  | 4.5 |  |  |  |
| 3 Spectrum required in each direction (MHz) |  |  | 1 000 |  |  |  |
| 4 Carrier transmission parameters |  |  |  |  |  |  |
| Modulation type | QPSK, 8-PSK |  |  |  |  |  |
| Information rate (Mbit/s) | 2.048 | 10.0 |  | 16.384 | 51.84 | 155.52 |
| Transmission bandwidth(MHz) | 4.10 | 20.0 |  | 32.8 | 104 | 311 |
| Required Eb/N0 (dB) | 6.2 | 6.2 |  | 8.9 | 8.0 | 14.3 |
| Uplink e.i.r.p./carrier (dBW): |  |  |  |  |  |  |
| Nominal | 35.3 | 39.8 |  | – | 48.5 | 50.8 |
| Maximum (rain faded) | 39.0 | 52.0 |  | – | 60.7 | 60.4 |
| Downlink e.i.r.p./carrier (dBW): |  |  |  |  |  |  |
| Nominal | – | – |  | 41.3 | 32.6 | 35.9 |
| Maximum (rain faded) | – | – |  | 44.8 | 40.6 | 42.9 |
| Type of satellite transponder |  |  | On-board processing |  |  |  |
| Satellite switch rate (Gbit/s) |  |  | 17.5 |  |  |  |
| Aggregate data rate (Gbit/s) |  |  | 8.9 |  |  |  |
| 5 Satellite antenna parameters |  |  |  |  |  |  |
| Tx maximum gain (dBi) |  |  | 32.8(1), 37.4(2) |  |  |  |
| Rx maximum gain (dBi) |  |  | 35.3(1), 40.9(2) |  |  |  |
| 6 Earth station antenna parameters |  |  |  |  |  |  |
| Radiation pattern |  |  |  |  |  |  |
| Minimum operating elevation angle (degrees) |  |  | 16 |  |  |  |
| (1) For data rates of 2.048, 10.0, 16.384 and 51.84 Mbit/s. (2) For data rate of 155.52 Mbit/s. |  |  |  |  |  |  |

TABLE  16

Technical characteristics of the MEOSAT-X system in the 20/30 GHz bands

| Band | 20/30 GHz | 40/50 GHz |
| --- | --- | --- |
| 1 Orbital parameters |  |  |
| Shape of orbit | Circular |  |
| Height (km) | 10 352 |  |
| Inclination angle (degrees) | 50 |  |
| Coherence (track repeat (h)) | 6 |  |
| Number of satellites per plane | 8 |  |
| Number of orbital planes | 4 |  |
| Satellite separation within plane (degrees) | 30 and 60 (within each plane satellites will fly in pairs with 30° between members of a pair and 60° between pairs) |  |
| Satellite phasing between planes (degrees) | 67.5 |  |
| 2 Frequency range and polarization |  |  |
| Uplink frequency (GHz) | 28.35-29.1 and 29.5-30.0 | 47.2-50.2 |
| Uplink polarization | LHCP/RHCP |  |
| Downlink frequency (GHz) | 18.05-18.3, 18.8-19.3 and 19.7-20.2 | 37.5-42.5 (3 GHz of spectrum in this band) |
| Downlink polarization | LHCP/RHCP |  |
| 3 Spectrum required in each direction (MHz) | 1 250 | 3 000 |
| 4 Carrier transmission parameters |  |  |
| Modulation type | O-QPSK |  |
| Number of service link beams | 360 (MBA multi-beam antenna) beams for the downlink and 360 (MBA multi-beam antenna) beams for the uplink, 2 separate links for gateway applications | 4 beams supporting 16 channels |
| Number of feeder-link segments/polarization |  | Not available |
| Receiver bandwidth (MHz) | Uplink: 1.5, 3,5, 17, 120 (depends on data rate); downlink: 120 | 500 |
| Transmission bandwidth | Uplink: 100 kHz-125 MHz downlink: 125 MHz | Uplink: 500 MHz; downlink: 100 kHz, 500 MHz |
| Overall C/N0 per user (dB/Hz) or C/N (dB) | Uplink at 20° elevation: 20 dB for users 34 dB for gateways; downlink at 20° elevation: 10-19 dB (depends on antenna size); gateway uplink: 34 dB gateway downlink: 26.9 dB | Uplink at 20° elevation: 32.7 dB Downlink at 20° elevation: 27.3 dB |

TABLE  16 (end)

| Band | 20/30 GHz | 40/50 GHz |
| --- | --- | --- |
| 4 Carrier transmission parameters (continued) |  |  |
| Uplink e.i.r.p./carrier (dBW) | 51-66 (for 65 cm to 180 cm user antennas); 81.23 (for gateways) | 88 (4.8 m antenna) |
| Downlink e.i.r.p./carrier (dBW) | 53 (users); 54 (gateways) | 60 (4.8 m antenna) |
| Type of satellite transponder | On-board processing |  |
| 5 Satellite antenna parameters |  |  |
| Tx maximum gain (dBi) | 37.5 (MBA), 38.32 (steerable beam) | 45.19 (steerable beam) |
| Rx maximum gain (dBi) | 37.48 (MBA), 42.21 (steerable beam) | 47.86 (steerable beam) |
| Main lobes | 2° beamwidth at the 3 dB point for the MBA |  |
| Side lobes (dB) | 14-17 first side lobe for the MBA; 20 for the steerable beam | 20, first side lobe |
| Back lobes (dB) | 25 for the MBA and the steerable beam | 25 |
| Steerable antenna or not | No for MBA; yes for steerable beam | Yes |
| 6 Earth station antenna parameters |  |  |
| Peak Tx gain (dBi) | 44-53 (for 65 cm to 180 cm) | 66.08 (for 4.8 m) |
| Peak Rx gain (dBi) | 40-50 (for 65 cm to 180 cm) | 45.19 (for 4.8 m) |
| Radiation pattern | Rec. ITU-R S.580 (29 – 25 log (angle)) |  |
| Minimum operating elevation angle (degrees) | 20 |  |
| 7 Number of earth stations and distribution | Unlimited/global |  |
| 8 Earth station switching strategy | Interference avoidance/highest elevation angle |  |

ANNEX  8

TABLE  17

Technical characteristics of a planned GSO FSS network above 30 GHz: GSO-VX

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Circular |
| Height (km) | 35 786 (geosynchronous) |
| Inclination angle (degrees) | 0 (referenced to Equator) |
| Coherence (track repeat (h)) | Not applicable |
| Number of satellites per plane | 14 |
| Number of orbital planes | 1 (GSO) |
| Satellite separation (degrees) within plane | At least 2 |
| Satellite phasing between planes (degrees) | Not applicable |
| Satellite locations (number per location) | 99° W(2), 101° W(2), 103° W(2) 63° W(1), 53° W(1), 8.5° E(2), 48° E(1) |
| 2 Frequency range and polarization |  |
| Uplink frequency (GHz) V-band Ku-band | 47.2-50.2 12.75-13.25 |
| Uplink polarization V-band Ku-band | LHCP/RHCP LHCP/RHCP |
| Downlink frequency (GHz) V-band Ku-band | 39.5-42.5 10.70-10.95, 11.2-11.45 |
| Downlink polarization V-band Ku-band | LHCP/RHCP LHCP/RHCP |
| 3 Spectrum required in each direction (MHz) |  |
| V-band Ku-band | 3 000 500 |
| 4 Carrier transmission parameters |  |
| Modulation type V-band (uplink and downlink) Ku-band (uplink and downlink) | Differential QPSK, TDMA/FDMA Differential QPSK, TDMA/FDMA |
| Number of service link beams V-band Ku-band | 40 16 |
| FDM channels per beam V-band Ku-band | 10 1 |
| Bandwidth per FDM channel (MHz) V-band Ku-band | 300 250 |
| TDM channels per FDM channel V-band Ku-band | 100 100 |
| TDMA burst rate (Mbit/s) V-band Ku-band | 155.52 to 200 155.52 to 200 |

TABLE  17 (end)

| 4 Carrier transmission parameters (continued) |  |
| --- | --- |
| Receiver bandwidth (MHz) V-band Ku-band | 300 250 |
| Transmitter bandwidth (MHz) V-band Ku-band | 300 250 |
| Overall C/N0 per user (dB/Hz) V-band Ku-band | Uplink (clear air): 96.8 Uplink (rain): 93.9 Downlink (clear air): 95.4 Downlink (rain): 91.7 Uplink (clear air): 96.3 Uplink (rain): 93.7 Downlink (clear air): 97.2 Downlink (rain): 94.2 |
| Uplink e.i.r.p./carrier (dBW) V-band Ku-band | 73.8 68.5 |
| Downlink e.i.r.p./carrier (dBW) V-band Ku-band | 56.0 47.0 |
| Type of satellite transponder | Processor controlled SS-TDMA switch |
| 5 Satellite antenna parameters |  |
| Transmit antenna maximum gain (dBi) V-band Ku-band | 49.0 33.5 |
| Receive antenna maximum gain (dBi) V-band Ku-band | 49.0 48.8 |
| Transmit side-lobe level (dB below peak) V-band Ku-band | 18 18 |
| Back lobes (dB below peak) | 25 |
| Steerable antenna or not | No, V-band and Ku-band antennas are satellite body mounted |
| 6 Earth station antenna parameters |  |
| Peak transmit gain (dBi) V-band Ku-band | 59.5 for 2.5 m 48.8 for 2.5 m |
| Peak receive gain (dBi) V-band Ku-band | 58.0 for 2.5 m 47.3 for 2.5 m |
| Radiation pattern | Rec. ITU-R S.580 (29 – 25 log (angle)) |
| Minimum operating elevation angle (degrees) | 20 |
| 7 Number of earth stations and distribution | Global/unlimited |
| 8 Earth station switching strategy | Interference avoidance/highest elevation angle |
| SS: satellite switched. |  |

TABLE  18

Technical characteristics of a planned GSO FSS network above 30 GHz: GEO-SV(1)

| System | GEO-SV |
| --- | --- |
| 1 Orbital parameters |  |
| Shape of orbit | Circular |
| Height (km) | 35 786 |
| Inclination angle (degrees) | 0 |
| Coherence (track repeat (h)) | – |
| Number of satellites per plane | 6 |
| Number of orbital planes | 1 (GSO) |
| Satellite separation (degrees) within plane | At least 2 |
| Satellite phasing between planes (degrees) | – |
| 2 Frequency range and polarization |  |
| Uplink frequency (GHz) | 47.2-50.2 |
| Uplink polarization | LHCP/RHCP |
| Downlink frequency (GHz) | 39.5-42.5 |
| Downlink polarization | LHCP/RHCP |
| 3 Spectrum required in each direction (MHz) | 3 000 |
| 4 Carrier transmission parameters |  |
| Modulation type | D-QPSK |
| Number of service link beams | 40, 0.15° uplink and downlink beams |
| User data rate per TDMA carrier (Mbit/s) | 155 (1 to 2.5 m ground terminals) 26.4 (45 cm ground terminals) |
| Coding rate | 0.609 |
| Effective overall rate (coding and data overhead) | 0.517 |
| Carrier occupied bandwidth (MHz) | 199.85 (1 to 2.5 m terminals) 34.04 (45 cm terminals) |
| Required Eb/(N0 + I0) without loss (dB) | 5.0 |
| Modem implementation loss (dB) | 1.5 |
| Receiver/transmitter bandwidth (MHz) | 257 (1 to 2.5 m ground terminals) 43.7 (45 cm terminals) |
| Uplink e.i.r.p. per carrier (dBW) | 75.5(2) |
| Downlink e.i.r.p. per carrier (dBW) | 62.0 |
| Type of satellite transponder | SS-TDMA |
| 5 Satellite antenna parameters |  |
| Receiver sensitivity, G/T (dB(K–1)) | 26.4 |
| Tx maximum gain (dBi) | 58.0 |
| Rx maximum gain (dBi) | 58.0 |
| Antenna downlink beamwidth | 0.15° at the 3 dB point |
| Steerable antenna or not | Not |
| Radiation pattern |  |
| 6 Earth station antenna parameters |  |
| Apertures | Tx: 2.5 m only Rx: 2.5 m; 1 m; 45 cm |
| Peak Tx gain (dBi) | 59.5 |
| Peak Rx gain (dBi) | 50.8 (1 m) 43.8 (45 cm) |
| Receiver sensitivity G/T (dB(K–1)) | 23.8 (1 m) 16.8 (45 cm) |
| Radiation pattern | Rec. ITU-R S.580 (29 – 25 log (angle)) |
| Minimum operating elevation angle (degrees) | 15 |
| 7 Number of earth stations and distribution | Global/unlimited |
| 8 Earth station switching strategy |  |
| (1) The GEO-SV system is a hybrid system that also includes a Ku-band payload. (2) Uplink power control. |  |

TABLE  19

Technical characteristics of a planned GSO FSS network above 30 GHz: GEO-LV

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Circular |
| Height (km) | 35 786 |
| Inclination angle (degrees) | 0 |
| Number of satellites per plane |  |
| Number of orbital planes | 1 (GSO) |
| Satellite separation within plane (degrees) | At least 2 |
| 2 Frequency range and polarization |  |
| Uplink frequency (GHz) | 45.5-46.7 |
| Uplink polarization | LHCP/RHCP |
| Downlink frequency (GHz) | 37.5-38.6 |
| Downlink polarization | LHCP/RHCP |
| 3 Spectrum required in each direction (GHz) | 1.1 |
| 4 Carrier transmission parameters |  |
| Modulation type | QPSK |
| Number of service link beams |  |
| Receiver bandwidth (MHz) | Depends on data rate: Uplink: minimum of 3 Downlink: minimum of 3 |
| Transmission bandwidth (MHz) | Depends on data rate: Uplink: minimum of 3 Downlink: minimum of 3 |
| Overall C/N0 per user (dB/Hz) | Minimum of 59 |
| Uplink e.i.r.p./carrier (dBW) | Depends on antenna size and data rate. Over 41 |
| Downlink e.i.r.p./carrier (dBW) | Depends on antenna size and data rate. Over 55 |
| Type of satellite transponder | On-board processing |
| 5 Satellite antenna parameters |  |
| Satellite minimum antenna gain (dBi) | 52 |
| Main lobes | 0.15° beamwidth at the 3 dB point |
| Steerable antenna or not | Yes |
| 6 Earth station antenna parameters |  |
| Minimum Tx gain (dBW) | 33 |
| Minimum Rx gain (dBW) | 31 |
| Radiation pattern | RR Appendix S8, Annex III |
| Minimum operating elevation angle (degrees) | 30 |
| 7 Number of earth stations and distribution | Global/unlimited |

TABLE  20

Technical characteristics of a planned GSO FSS network above 30 GHz: MEO-LV

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Circular |
| Height (km) | 10 352 |
| Inclination angle (degrees) | 55 |
| Coherence (track repeat (h)) | 6 |
| Number of satellites per plane | 5 |
| Number of orbital planes | 4 |
| Satellite separation (degrees) within plane | 72 |
| 2 Frequency range and polarization |  |
| Uplink frequency (GHz) | 45.5-46.7 |
| Uplink polarization | LHCP/RHCP |
| Downlink frequency (GHz) | 37.5-38.6 |
| Downlink polarization | LHCP/RHCP |
| 3 Spectrum required in each direction (GHz) | 1.1 |
| 4 Carrier transmission parameters |  |
| Modulation type | QPSK |
| Receiver bandwidth (MHz) | Depends on data rate: Uplink: minimum of 3 Downlink: minimum of 3 |
| Transmission bandwidth (MHz) | Depends on data rate: Uplink: minimum of 3 Downlink: minimum of 3 |
| Overall C/N0 per user (dB/Hz) | Minimum of 57 |
| Uplink e.i.r.p./carrier (dBW) | Depends on antenna size and data rate. Over 41 |
| Downlink e.i.r.p./carrier (dBW) | Depends on antenna size and data rate. Over 55 |
| Type of satellite transponder | On-board processing |
| 5 Satellite antenna parameters |  |
| Satellite minimum antenna gain (dBi) | 41 |
| Main lobes | 6° beamwidth at the 3 dB point |
| Steerable antenna or not | Yes |
| 6 Earth station antenna parameters |  |
| Minimum Tx gain (dBW) | 33 |
| Minimum Rx gain (dBW) | 31 |
| Radiation pattern | RR Appendix S8, Annex III |
| Minimum operating elevation angle (degrees) | 30 |
| 7 Number of earth stations and distribution | Global/unlimited |
| 8 Earth station switching strategy | Interference avoidance/highest elevation angle |

TABLE  21

Technical characteristics of several LEO and GSO satellite networks above 38 GHz

| Parameters | Non-GSO above 38 GHz FSS |  |  | GSO above 38 GHz FSS |  |
| --- | --- | --- | --- | --- | --- |
|  | LEO V1 |  | LEO V2 | OSG V1 |  |
| 1 Orbital parameters |  |  |  |  |  |
| Shape of orbit | Circular |  | Circular | Circular |  |
| Apogee/perigee (km) | 1 350/1 350 |  | 10 355/10 355 | 35 878 |  |
| Inclination angle (degrees) | 47 |  | 50 |  |  |
| Number of satellites per plane | 6 |  | 5 |  |  |
| Number of orbital planes | 12 |  | 3 |  |  |
| Satellite separation within plane | 60 + 25 plane inclination |  | 72 |  |  |
| Satellite phasing per plane, RAAN (degrees) |  + 30 |  | 120 |  |  |
| 2 Targeted frequency range and polarization |  |  |  |  |  |
| Uplink frequency (GHz) | 47.2-50.2 |  | 47.2-50.2 | 47.2-50.2 |  |
| Uplink polarization/axial ratio (dB) | RHCP / __ dB |  | RHCP/LHCP | RHCP/LHCP |  |
| Downlink frequency (GHz) | 37.5-40,5 |  | 37.5-42.5 | 37.5-42.5 |  |
| Downlink polarization/axial ratio (dB) | LHCP / __ dB |  | RHCP/LHCP | RHCP/LHCP |  |
| 3 Carrier transmission parameters |  |  |  |  |  |
| Modulation type | QPSK |  | O-QPSK | O-QPSK |  |
| Coding rate | 0.449 |  |  0.5 | 0.5 |  |
| Number of service link beams | 16 |  | 48 | 24 |  |
| Target BER | 1  10–6 | 1  10–9 | 1  10–7 to 1  10–10 | 1  10–7 to 1  10–10 |  |
| Burst information data rate (Mbit/s) | 10.24 | 51.84 | OC3 to 10  OC3 | OC3 to 10  OC3 |  |
| Bandwidth per channel (MHz) | 18 | 90 | 300 to 3 000 | 300 to 3 000 |  |
| Required Eb/(N0 + I0) without loss (dB) | 2.2 | 2.7 | 6.0 | 6.0 |  |
| Modem implementation loss (dB) | 1.5 | 1.5 (+0.55/hop) | 2.0 | 2.0 |  |
| Receiver bandwidth (MHz) |  |  |  |  |  |
| Transmission bandwidth (MHz) |  |  |  |  |  |
| Uplink e.i.r.p./carrier (dBW) | 44.9 to 50.5 ARC | 60.2 to 65.8 ARC | 69.7 to 78.9 | 73.7 to 80.0 |  |
| Downlink e.i.r.p./carrier (dBW) | 28.3 to 33.9 ARC | 46.9 to 52.5 ARC | 49.5 to 58.8 | 57.5 to 66.4 |  |
| Type of satellite transponder | Transparent |  | Processed payload | Regenerative |  |
| 4 Satellite antenna parameters |  |  |  |  |  |
| Effective aperture (m) | 0.45 Tx/0.36 Rx |  | 0.65 Tx/0.52 Rx | 1.36 Tx/1.1 Rx |  |
| Tx maximum gain (dBi) | 40.6 |  | 45.5 | 53 |  |
| Rx maximum gain (dBi) | 40.6 |  | 45.2 | 53 |  |
| Receiver sensitivity G/T (dB(K–1)) | 10.5 |  | 16.9 | 24.4 |  |
| Radiation pattern | RR Appendix S8 |  | Ex-CCIR Report 558 | Ex-CCIR Report 558 |  |
| Steerable antenna or not | Yes |  | Yes | Yes |  |
| 5 Earth station antenna parameters |  |  |  |  |  |
| Effective aperture (m) | 0.66 | 1.5 | 2.2 | 2.2 |  |
| Peak Tx gain (dBi) | 49.3 | 56.4 | 59.7 | 59.7 |  |
| Peak Rx gain (dBi) | 47.3 | 54.4 | 57.8 | 57.8 |  |
| Receiver sensitivity, G/T (dB(K–1)) | 19.3 | 26.4 | 32.4 | 32.4 |  |
| Radiation pattern | Rec. ITU-R S.465(1) | Rec. ITU-R S.465 | Rec. ITU-R S.465 | Rec. ITU-R S.465 |  |
| Minimum operating elevation angle (degrees) | 22 |  | 30 | 30 |  |
| 6 Number of earth stations and distribution | 43 000 active worldwide | 1 500 active worldwide | Unknown (< 50 000) | Unknown (< 30 000) |  |
| 7 Earth station switching strategy | Make before break |  | Make before break | Make before break |  |
| OC3: optical carrier system operating at the 3rd hierarchical level. RAAN: right ascension of the ascending node. (1) Recommendation ITU-R S.465 radiation pattern is consistent with the proposed frequency of operation when extrapolated to the proposed frequency band. |  |  |  |  |  |

TABLE  22

Technical characteristics of the GEOSAT-X system in the 40/50 GHz bands

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Circular |
| Height (km) | 35 786 |
| Inclination angle (degrees) | – |
| Coherence (track repeat (h)) | – |
| Number of satellites per plane | 9 |
| Number of orbital planes | 1 (GSO) |
| Satellite separation within plane (degrees) | At least 2 |
| Satellite phasing between planes (degrees) | – |
| 2 Frequency range and polarization |  |
| Uplink frequency (GHz) | 47.2-50.2 |
| Uplink polarization | LHCP/RHCP |
| Downlink frequency (GHz) | 39.5-42.5 (3 GHz of spectrum in this band) |
| Downlink polarization | LHCP/RHCP |
| 3 Spectrum required in each direction (MHz) | 3 000 |
| 4 Carrier transmission parameters |  |
| Modulation type | QPSK |
| Number of service link beams | 48 |
| Number of feeder-link segments/polarization | 8 beams for gateway communication 2 downlink beams for HCDR service |
| Receiver bandwidth | User service beams: Uplink: 500 kHz, 3 MHz, 11 MHz, depending on data rate Downlink: 125 MHz Gateway beams: Uplink and downlink: 125 MHz HCDR beams: Downlink: 1 GHz |
| Transmission bandwidth | User service beams: Uplink: 500 kHz, 3 MHz, 11 MHz, depending on data rate Downlink: 125 MHz Gateway beams: Uplink and downlink: 125 MHz HCDR beams: Downlink: 1 GHz |
| Overall C/N0 per user(dB/Hz) or C/N (dB) | User service beams C/N0: Uplink (clear air): 75.6, 76.9, 84.3, 93.2; (rain): 67.8, 67.8, 75.5, 81.6, depending on data rate Downlink (clear air): 89.8, 92.1, 94.8, 103.5; (rain): 82.8, 82.8, 86.2, 90.3, depending on terminal size Gateway beams C/N0: Uplink (clear air): 111.8; (rain): 92.3 Downlink (clear air): 110.5; (rain): 93.0 HCDR beams C/N0: Downlink (clear air): 115.8; (rain): 101.6 |
| Uplink e.i.r.p./carrier (dBW) | User service beams (edge of coverage): 49.9, 53.0, 60.4, 69.2, depending on terminal size Gateway beams: 83.9 |

TABLE  22 (end)

| 4 Carrier transmission parameters (continued) |  |
| --- | --- |
| Downlink e.i.r.p./carrier (dBW) | User service beams (edge of coverage): 61.2 Gateway beams: 62.7 HCDR beams: 62.2 |
| Type of satellite transponder | On-board processing |
| 5 Satellite antenna parameters |  |
| Tx maximum gain (dBi) | 56.5 for user and gateway links 53.7 for high data rate |
| Rx maximum gain (dBi) | 53.8 |
| Main lobes |  |
| Side lobes (dB) | Tx: 14 below peak Rx: 25 below peak |
| Back lobes (dB) | 25 below peak |
| Steerable antenna or not | Yes, phase array antenna for user and gateway beams |
| 6 Earth station antenna parameters |  |
| Peak Tx gain (dBi) | 44.8-59.3 for user earth stations from 45 cm to 240 cm 64.8 for gateway earth stations |
| Peak Rx gain (dBi) | 43.3-57.8 for user earth stations from 45 cm to 240 cm 63.3 for gateway earth stations 69.3 for HCDR earth stations |
| Radiation pattern | Rec. ITU-R S.580 (29 – 25 log (angle)) |
| Minimum operating elevation angle (degrees) | 20 |
| 7 Number of earth stations and distribution | Global/unlimited |
| 8 Earth station switching strategy | Interference avoidance/highest elevation angle |
| HCDR: High capacity data rate. |  |

ANNEX  9

TABLE  23

Technical characteristics of non-GSO FSS systems: System USAKU-L1

| 1 Orbital parameters |  |  |  |
| --- | --- | --- | --- |
| Shape of orbit | Circular |  |  |
| Height (km) | 1 457 |  |  |
| Inclination angle (degrees) | 55 |  |  |
| Coherence (track repeat (h)) | 665 |  |  |
| Number of satellites per plane | 4 |  |  |
| Orbital planes | 16 |  |  |
| Satellite separation (degrees) within plane | 90 |  |  |
| 2 Targeted frequency range and polarization |  |  |  |
| Uplink frequency (GHz) | 12/17 |  |  |
| Uplink polarization | Circular |  |  |
| Downlink frequency (GHz) | 10/12 |  |  |
| Downlink polarization | Circular |  |  |
| 3 Spectrum required in each direction (MHz) | 1 050 within the above frequency range |  |  |
| 4 Carrier transmission parameters |  |  |  |
| Modulation type | QPSK/BPSK |  |  |
| Number of service link beams | < 45 |  |  |
| Number of uplink transponders/polarization | 4 |  |  |
| Transponder bandwidth (MHz) | 250 or 300 |  |  |
| Earth station noise bandwidth (MHz) | Forward: 22.6, return: 2.93 |  |  |
| Overall Eb/N0 (dB) | 3.5 |  |  |
| Earth station useful e.i.r.p. (dBW) | 64.6(1) to 68(2) 30.3 to 43.4(3) |  |  |
| Satellite useful e.i.r.p./carrier (dBW) Forward Return | 16.4 to 18.5 (residential), 11 to 17.6 (provincial, international) –3.5 to –1.7 (residential), –7.2 to –2.4 (provincial, international) |  |  |
| Type of satellite transponder | Transparent bent pipe |  |  |
| 5 Satellite antenna parameters |  |  |  |
| Tx maximum gain (dBi) | 15 at Nadir –22.8 at EOC |  |  |
| Rx maximum gain (dBi) | 18.2 at Nadir –25.7 at EOC |  |  |
| Steerable antenna or not | Yes |  |  |
| 6 Earth station antenna parameters | Provincial | International(3) | Residential(3) |
| Peak Tx gain (dBi) | 49.4(1)/53.8(2) | 37 | 29.5-32.2 |
| Peak Rx gain (dBi) | 48.1(1)/52.5(2) | 35.7 | 28.1-30.8 |
| Radiation pattern | Rec. ITU-R S.580 | RR Appendix S8 | RR Appendix S8 |
| Minimum specified elevation angle (degrees) | 5 | 5 | 10 |
| 7 Number of earth stations and distribution | Up to 20 million worldwide |  |  |
| 8 Earth station switching strategy | Track best elevation satellite within the operating range outside the non-operating zone |  |  |
| EOC: edge of coverage (1) Gateway. (2) Gateway in 13.75-14 GHz. (3) User terminal. |  |  |  |

TABLE  24

Technical characteristics of a quasi-GSO FSS network: USAKU-H1

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Elliptical |
| Apogee/perigee (km) | 41 449/4 100 |
| Inclination (degrees) | 63.4 |
| Coherence (track repeat (h)) | 14 (approximately) |
| Number of satellites per plane | 4 |
| Number of orbital planes | 3 |
| Satellite phasing between planes (degrees) | 120 |
| Argument of perigee (degrees) | 270 |
| Minimum satellite separation from geosynchronous plane (degrees) | 40 |
| Maximum latitude/longitude operating range (degrees) | 19/3 |
| Nominal operational longitude | 100° W/30° W/120° E/170° E |
| 2 Frequency range |  |
| Uplink frequency (GHz) | 17.3-17.8 |
| Uplink polarization | LHCP/RHCP |
| Downlink frequency (GHz) | 10.7 to 12.7 |
| Downlink polarization | RHCP/LHCP |
| 3 Spectrum required in each direction (MHz) | 500 |
| 4 Carrier transmission parameters |  |
| Modulation | QPSK |
| Access | FDMA/TDM |
| Number of service link beams | 10 |
| Segment (transponder) bandwidth (MHz) | 24 |
| Receive bandwidth (MHz) | 24 |
| Transmission bandwidth (MHz) | 24 |
| Overall C/N0 (dB/Hz) | 86 |
| Uplink e.i.r.p./carrier (dBW) | 77 |
| Downlink e.i.r.p./carrier (dBW) | 58 |
| 5 Satellite communication parameters |  |
| Type of satellite transponder | Transparent |
| Transmit maximum gain (dBi) | 38 |
| Receive maximum gain (dBi) | 24 |
| Steerable antenna or not | Yes (phased array) |
| 6 Earth station characteristics |  |
| Feeder link transmit gain (dBi) | 55 |
| Feeder link e.i.r.p. per 24 MHz (dBW) | 77 |
| Receive antenna gain (minimum) (dBi) | 30 |
| Antenna pattern | 29 – 25 log (θ) |

TABLE  25

Technical characteristics of a quasi-GSO FSS network: Tanya

| 1 Orbital parameters |  |
| --- | --- |
| Shape of orbit | Elliptical |
| Apogee/perigee (km) | 41 449/4 100 |
| Argument of perigee (degrees) | 270 |
| Inclination (degrees) | 63.4 |
| Coherence (track repeat (h)) | 12 (approximately) |
| Number of satellites per plane | 1 |
| Number of orbital planes | 4 |
| Satellite phasing between planes (degrees) | 90 |
| Minimum satellite separation from geostationary orbit plane (degrees) | 53.3 |
| Operational stationary latitude/longitude locations | 58.3 N/90 E 58.3 N/90 W |
| Maximum latitude/longitude operating range (degrees) | 10/2 |
| Minimum elevation angle from earth stations (degrees) | 25 |
| Short-term/long-term pfd at Earth's surface (dB(W/(m2  MHz))) | Identical |
| 2 Frequency range |  |
| Uplink frequency (GHz) | 13.75-14.5/17.3-17.8 |
| Uplink polarization | LHCP/RHCP |
| Downlink frequency (GHz) | 10.7-12.7 |
| Downlink polarization | RHCP/LHCP |
| 3 Spectrum required in each direction (MHz) | 1 250 |
| 4 Carrier transmission parameters |  |
| Modulation | QPSK |
| Access | FDMA/TDM |
| Number of service link beams | 10 |
| Segment (transponder) bandwidth (MHz) | 24 |
| Receive bandwidth (MHz) | 24 |
| Transmission bandwidth (MHz) | 24 |
| Overall C/N0 (dB/Hz) | 86 |
| Uplink e.i.r.p./carrier (dBW) | 77 |
| Downlink e.i.r.p./carrier (dBW) | 58 |
| 5 Satellite communication parameters |  |
| Type of satellite transponder | Transparent |
| Transmit maximum gain (dBi) | 38 |
| Receive maximum gain (dBi) | 24 |
| Steerable antenna or not | Yes (phased array) |
| 6 Earth station characteristics |  |
| Feeder link transmit gain (dBi) | 55 |
| Feeder link e.i.r.p. per 24 MHz (dBW) | 77 |
| Receive antenna gain (minimum) (dBi) | 30 |
| Antenna pattern | 29 – 25 log (θ) |

ANNEX  10

TABLE  26

Technical characteristics of two GSO satellite telemetry and command sub-systems

|  | Emergency |  | On-station |  |
| --- | --- | --- | --- | --- |
|  | GSO-A | GSO-B | GSO-A | GSO-B |
| Telemetry |  |  |  |  |
| Frequency (GHz) | 11.7 |  | 11.7 |  |
| Modulation | PM |  | PM |  |
| Satellite e.i.r.p. (dBW) | 7.2 | 10.1 | 12.4 | 16 |
| Earth station G/T (dB(K–1)) | 37 | 35 | 37 | 35 |
| C/N0 (dB/Hz) | 66.5 | 67.4 | 71.7 | 73.5 |
| Losses (dB) | 7 | 5 | 7 | 5 |
| Telemetry bit rate (dB/Hz) | 27.1 | 33.1 | 27.1 | 33.1 |
| Eb/N0 (dB) | 32.4 | 29.3 | 37.6 | 35.2 |
| Required Eb/N0 (dB/Hz) | 9.6 | 12 | 9.6 | 12 |
| Command |  |  |  |  |
| Frequency (GHz) | 14.5 | 17.3 | 14.5 | 17.3 |
| Modulation | FM |  | FM |  |
| Earth station e.i.r.p. (dBW) | 88 | 89 | 59 | 65 |
| Satellite antenna gain (dBi) | 0 | 0 | 43 | 29 |
| Receiver input power (dBW) | –125.9 | –126.3 | –122.9 | –125 |
| Receiver threshold (dBW) | –141.5 | –143 | –141.5 | –143 |
| System noise temperature (dB(K–1)) | 28 | 27.4 | 29.6 | 28.9 |
| C/N0 (dB/Hz) | 74.7 | 74.9 | 76.1 | 74.7 |
| Required C/N0 (dB/Hz) | 58.5 | 57 | 58.5 | 57 |
| PM: phase modulation. |  |  |  |  |

ANNEX  11

TABLE  27

Technical characteristics of a planned V-band system: GSOV-B1

| 2 Frequency range and polarization |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink frequency (GHz) |  |  | 47.2-50.2 |  |  |  |
| Uplink polarization |  |  | Linear |  |  |  |
| Downlink frequency (GHz) |  |  | 37.5-40.5 |  |  |  |
| Downlink polarization |  |  | Linear |  |  |  |
| 3 Spectrum required in each direction (MHz) |  |  | 3 000 |  |  |  |
| 4 Carrier transmission parameters |  |  |  |  |  |  |
| Modulation type |  |  | QPSK/SS-TDMA |  |  |  |
| Number of service link beams |  |  | 24  0.3° beams |  |  |  |
| User data rate per TDMA carrier |  |  | 88 Mbit/s 128 kbit/s |  |  |  |
| Occupied bandwidth |  |  | 105.6 MHz 167 kHz |  |  |  |
| Coding |  |  | R 1/2 |  |  |  |
| 5 Satellite antenna parameters |  |  |  |  |  |  |
| Antenna beam gain (dBi) |  |  | 51.5 |  |  |  |
| Satellite G/T (dB(K–1)) |  |  | 23.4 |  |  |  |
| Satellite noise temperature (K) |  |  | 650 |  |  |  |
| Beam size (degrees) |  |  | 0.3 |  |  |  |
| Number of beams |  |  | 24 |  |  |  |
| 6 Earth station antenna parameters |  |  |  |  |  |  |
| Antenna gain |  |  | See item 7 below |  |  |  |
| Minimum elevation angle (degrees) |  |  | 20 |  |  |  |
| 7 Power characteristics |  |  |  |  |  |  |
|  |  |  | 194KG7W |  | 123MG7W |  |
| Uplink/downlink | Earth station diameter (m) | Earth station gain (dBi) | Maximum power density (dB(W/Hz)) | Minimum power density (dB(W/Hz)) | Maximum power density (dB(W/Hz)) | Minimum power density (dB(W/Hz)) |
| Uplink clear sky | 0.45 0.9 1.2 6 | 45.2 51.2 53.7 67.7 | –47.3 –53.3 –55.8 –69.8 | –65.8 –71.8 –74.3 –88.3 | –48.7 –54.7 –57.2 –71.2 | –51.7 –57.7 –60.2 –74.2 |
| Uplink degraded | 0.45 0.9 1.2 6 | 45.2 51.2 53.7 67.7 | –37.3 –43.3 –55.8 –59.8 | –65.8 –71.8 –74.3 –88.3 | –38.7 –44.7 –47.2 –61.2 | –51.7 –57.7 –60.2 –74.2 |
| Downlink | 0.45 0.9 1.2 6 | 43.7 49.7 52.2 66.2 | –60.3 (2) 67.3 –75.8 | –69.3 (2) –75.5 –78.8 | (1) –66.2 –66.2 –66.2 | (1) –73.3 –74.9 –77.5 |
| (1) Not provided. (2) Out of range. |  |  |  |  |  |  |

TABLE  28

Technical characteristics of a planned V-band system: GSO V-B2

Technical characteristics of CANSAT GSO FSS satellite networks in the 50/40 GHz bands

| GSO FSS network name – “CANSAT–_ _” | 2A | 2B | 3A | 3B | 3C | 3D | 3E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Orbital position (° W) | 70.5 | 82.0 | 91.0 | 111.1 | 114.9 | 118.7 | 107.3 |
| 1 Orbital parameters |  |  |  |  |  |  |  |
| Shape of orbit | Circular |  |  |  |  |  |  |
| Height (km) | 35 786 (geosynchronous) |  |  |  |  |  |  |
| Inclination angle (degrees) |  0.1 |  |  |  |  |  |  |
| Number of orbital planes | 1 (GSO) |  |  |  |  |  |  |
| Separation within plane | – |  |  |  |  |  |  |
| 2 Frequency range and polarization |  |  |  |  |  |  |  |
| Uplink frequency (GHz) | 47.2-50.2 |  |  |  |  |  |  |
| Uplink polarization | – |  |  |  |  |  |  |
| Downlink frequency (GHz) | 37.5-40.5 |  |  |  |  |  |  |
| Downlink polarization | – |  |  |  |  |  |  |
| 3 Spectrum required in each direction (MHz) | 3 000 |  |  |  |  |  |  |
| 4 Carrier transmission parameters |  |  |  |  |  |  |  |
| Modulation type | QPSK or 8-PSK |  |  |  |  |  |  |
| Number of service link beams | Up to 80 |  |  |  |  |  |  |
| FDM channels/beam | Up to 5 |  |  |  |  |  |  |
| Number of bandwidth/FDM channel (MHz) | 150 |  |  |  |  |  |  |
| Number of user rate/TDM channel (Mbit/s) | 0.144 to 155.52 |  | 1.544 to 155.52 |  |  |  |  |
| Receiver bandwidth (MHz) | 0.150 to 150 (depending upon data rate) |  | 1.50 to 150 (depending upon data rate) |  |  |  |  |
| Transmission bandwidth (MHz) | 0.150 to 150 (depending upon data rate) |  | 1.50 to 150 (depending upon data rate) |  |  |  |  |
| Overall C/N per user (dB) | Minimum of 5.0 (depends on coding) |  |  |  |  |  |  |
| Uplink e.i.r.p./carrier (dBW) | 49.6 (144 kbit/s) 60.0 (1.544 Mbit/s) 76.2 (44.736 Mbit/s) 84.7 (155.52 Mbit/s) |  | 60.0 (1.544 Mbit/s) 76.2 (44.736 Mbit/s) 84.7 (155.52 Mbit/s) |  |  |  |  |
| Downlink e.i.r.p./carrier (dBW) | 70.1 (144 kbit/s) 63.8 (1.544 Mbit/s) 47.1 (44.736 Mbit/s) 37.1 (155.52 Mbit/s) |  | 63.8 (1.544 Mbit/s) 47.1 (44.736 Mbit/s) 37.1 (155.52 Mbit/s) |  |  |  |  |
| Type of transponder | – |  |  |  |  |  |  |
| 5 Satellite antenna parameters |  |  |  |  |  |  |  |
| Transmit gain (maximum) (dBi) | 53.1 |  |  |  |  |  |  |
| Receive gain (maximum) (dBi) | 53.1 |  |  |  |  |  |  |
| Main lobes | – |  |  |  |  |  |  |
| Side lobes | – |  |  |  |  |  |  |
| Steerable antenna or not | Yes |  |  |  |  |  |  |
| 6 Earth station antenna parameters |  |  |  |  |  |  |  |
| Transmit gain (maximum) (dBi) | 45.5 (46 cm) to 65.4 (4.5 m) |  | 52.3 (1.0 m) to 65.4 (4.5 m) |  |  |  |  |
| Receive gain (maximum) (dBi) | 43.8 (46 cm) to 63.6 (4.5 m) |  | 50.5 (1.0 m) to 63.6 (4.5 m) |  |  |  |  |
| Radiation pattern | 29 – 25 log () |  |  |  |  |  |  |
| Minimum operating elevation angle (degrees) | Generally ³ 20 |  | Minimum elevation determined by climatic zone, and desired availability |  |  |  |  |
| 7 Number of earth stations and distribution | Unlimited/visible Earth |  | Unlimited/anywhere on visible Earth |  |  |  |  |
| 8 Earth station switching strategy | – |  |  |  |  |  |  |

* NOTE 1 –	References to Ku bands refer to the 10-15 GHz bands.

References to Ka bands refer to the 17-30 GHz bands.

References to V bands refer to the 40-50 GHz bands.