RECOMMENDATION  ITU-R  S.1418

METHOD  FOR  CALCULATING  SINGLE  ENTRY  CARRIER-TO-INTERFERENCE
RATIOS  FOR  LINKS  IN  INTER-SATELLITE  SERVICE
USING  GEOSTATIONARY  ORBIT

(Question ITU-R 239/4)

(1999)

Rec. ITU-R S.1418

The ITU Radiocommunication Assembly,

considering

a)	that Question ITU-R 239/4 for the World Radiocommunication Conference (Geneva, 1997) (WRC-97) calls for “very urgent and priority studies” to determine sharing criteria and coordination guidelines for systems utilizing the inter-satellite service (ISS);

b)	that there is extensive planned use of the ISS between satellites in the geostationary orbit (GSO);

c)	that GSO ISS networks typically require wide bandwidths, and typically form high data rate trunk links for the space segments of networks;

d)	that high data rate trunk lines must have high availability;

e)	that a method is required to evaluate the level of interference between such systems,

recommends

1	that the method described in Annex 1 be used to calculate C/I ratios for inter-satellite links in the GSO.

ANNEX  1

Method for calculating single entry C/I for links
of ISS using GSO

# 1	Inputs

The required inputs for the calculation of single entry C/I are shown in Table 1.

TABLE  1

Inputs for methodology for calculation
of single entry C/I

| Item | Symbol |
| --- | --- |
| Centre frequency (MHz) | fc |
| Victim system |  |
| Longitude of receiver (degrees) | Vrx |
| Longitude of transmitter (degrees) | Vtx |
| Transmit power (dBW) | Pv |
| Transmit gain (dBi) | Gv,tx |
| Transmit bandwidth (MHz) | Bv |
| Receive gain (dBi) | Gv,rx |
| Receive antenna diameter (m)(1) | dv,rx |
| Interfering system |  |
| Longitude of receiver (degrees) | Irx |
| Longitude of transmitter (degrees) | Itx |
| Transmit power (dBW) | Pi |
| Transmit gain (dBi) | Gi,tx |
| Transmit bandwidth (MHz) | Bi |
| Receive gain (dBi) | Gi,rx |
| Transmit antenna diameter (m)(1) | di,tx |
| (1) The antenna diameters are sometimes required for calculation of antenna discrimination. |  |

# 2	Method

## 2.1	Carrier power

To calculate carrier power, it is necessary to calculate the range loss between the satellites. The range between victim satellites is given by:

R = 2rGSO sin  Vtx – Vrx|2km

where:

Vtx :	longitude of the victim transmitter

Vrx :	longitude of the victim receiver

rGSO :	radius of the GSO (42 164 km)

R :		range between satellites (km).

The free space loss is given by:

Lfs = 20 log f + 20 log R + 32.45               dB

where f is the carrier frequency (MHz).

The carrier power is then computed:

C = Pv + Gv,tx – Lfs + Gv,rx               dB

## 2.2	Interference power

To calculate interference power, it is necessary to calculate the range loss between the satellites. The range from the interfering transmitter to the victim receiver is given by:

R = 2rGSO sin  Itx – Vrx 2km

where:

Itx :	longitude of the interfering transmitter

Vrx :	longitude of the victim receiver

rGSO :	radius of the GSO (42 164 km)

R :		range between satellites (km).

The free space loss is given by:

Lfs = 20 log f + 20 log R + 32.45               dB

where f is the carrier frequency (MHz).

To facilitate calculation of off-boresight angles, the coordinates of all satellites are transformed from polar coordinates to the rectangular coordinate system within the equatorial plane, with the origin at geocentre.

The x axis component of each satellite is given by:

Sx = rGSO cos(longitude)

and the y axis component is given by:

Sy = rGSO sin(longitude)

where S is the position of the satellite in question, and rGSO is the GSO radius (42 164 km).

If A is the vector from satellite 1 to satellite 2, and B is the vector from satellite 1 to satellite 3, the angle () between A and B (measured at satellite 1) is given by:

θ = cos–1A ⋅ BA B

as illustrated in Fig. 1. | A | and | B | are the magnitudes of A and B, and A  B is the inner product of the two vectors. For example, for A and B expressed in rectangular coordinates then:

A ⋅ B = AxBx + AyBy + AzBz

For example, if the antenna discrimination at the victim receiver is required, let A represent the vector from the victim receiver to the victim transmitter, and let B represent the vector from the victim receiver to the interfering transmitter. The resulting angle  is input to an appropriate antenna pattern model to compute the antenna discrimination.

The bandwidth reduction factor is given by:

LBW = 10 log BiBvif BiBv > 10otherwise

The interference power is then computed:

I = Pi + Gi,txθtx – Lfs + Gv,rxθrx – LBW dB

where tx and rx are the off-boresight angles at the interfering transmitting and victim receiving ends, respectively, and Gi,tx(tx) and Gv,rx(rx) are the off-boresight gains of the interfering transmitting and victim receiving antennas.

# 3	Sample computation of C/I for the GSO ISS

The antenna pattern model used for this example is Recommendation ITU-R S.672 (Annex 1), with first side lobe of 20 dB below maximum gain.

The inputs for this example are shown in Table 2. It is assumed that Bi/Bv = 1.

TABLE  2

Inputs for example

| Item |  | Symbol |
| --- | --- | --- |
| Centre frequency (Hz) | 60  109 | fc |
| Victim system |  |  |
| Longitude of receiver (degrees) | 12 | Vrx |
| Longitude of transmitter (degrees) | 0 | Vtx |
| Transmit power (dBW) | 13 | Pv |
| Transmit gain (dBi) | 49 | Gv,tx |
| Receive gain (dBi) | 49 | Gv,rx |
| Receive antenna diameter (m) | 0.75 | dv,rx |
| Interfering system |  |  |
| Longitude of receiver (degrees) | 10 | Irx |
| Longitude of transmitter (degrees) | 2 | Itx |
| Transmit power (dBW) | 13 | Pi |
| Transmit gain (dBi) | 55.7 | Gi,tx |
| Receive gain (dBi) | 55.7 | Gi,rx |
| Transmit antenna diameter (m) | 1 | di,tx |

Table 3 shows the results of the C/I calculation.

TABLE  3

C/I calculation

| Item |  | Symbol |
| --- | --- | --- |
| Carrier power |  |  |
| Transmit power (dBW) | 13.0 | Pv |
| Transmit antenna gain (dBi) | 49.1 | Gv,tx |
| Free space loss (dB) | –206.9 | L |
| Receive antenna gain (dBi) | 49.1 | Gv,rx |
| Carrier power (dBW) | –95.7 | C |
| Interference power |  |  |
| Transmit power (dBW) | 13.0 | Pi |
| Transmit off-angle (degrees) | 1.0 | θtx |
| Transmit gain (dB) | 35.7 | Gi,tx (θtx) |
| Free space loss (dB) | –205.3 | L |
| Receive off-angle (degrees) | 1.0 | θrx |
| Receive gain (dB) | 29.1 | Gv,rx (θrx) |
| Interference power (dBW) | –127.5 | I |
| C/I (dB) | 31.8 |  |