Recommendation ITU-R S.1503-4

(09/2023)

S Series: Fixed-satellite service

Functional description to be used in developing software tools for determining conformity of non-geostationary-satellite orbit fixed-satellite service systems or networks with limits contained in Article 22 of the Radio Regulations

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at https://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2023

ITU 2023

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  S.1503-4

Functional description to be used in developing software tools for determining conformity of non-geostationary-satellite orbit fixed-satellite service systems or networks with limits contained in Article 22 of the Radio Regulations

(2000-2005-2013-2018-2023)

Scope

This Recommendation provides a functional description of the software for use by the Radiocommunication Bureau of ITU to conduct examination of non-geostationary-satellite orbit (non-GSO) fixed-satellite service (FSS) system notifications for their compliance with the validation limits specified in the Radio Regulations.

Keywords

epfd, non-GSO, methodology

Abbreviations/Glossary

Alpha angle (): 	The minimum angle between the line to the non-GSO satellite and the lines to the GSO arc as seen at the earth station communicating with a geostationary-satellite orbit (GSO).

e.i.r.p. mask: 		Equivalent isotropic radiated power mask used to define the emissions of the non-GSO earth station in the epfd(up) calculation or the emissions of the non-GSO satellite for the epfd(IS) calculation.

epfd: 		Equivalent power flux-density, as defined in Radio Regulations (RR) No. 22.5C.1, of which there are three cases to consider:

epfd(down): emissions from the non-GSO satellite system into a GSO satellite earth station;

epfd(up): emissions from the non-GSO earth station into a GSO satellite;

epfd(IS): inter-satellite emissions from the non-GSO satellite system into a GSO satellite system.

pfd mask: 		power flux-density mask used to define the emissions of the non-GSO satellite in the epfd(down) calculation.

WCG: 		Worst Case Geometry, the location of the GSO earth station and GSO satellite that analysis suggests would cause the highest single entry epfd values for given inputs.

Related ITU Recommendations, Reports

Recommendation ITU-R BO.1443 – Reference BSS earth station antenna patterns for use in interference assessment involving non-GSO satellites in frequency bands covered by RR Appendix 30

Recommendation ITU-R S.672 – Satellite antenna radiation pattern for use as a design objective in the fixed-satellite service employing geostationary satellites

Recommendation ITU-R S.1428 – Reference FSS earth-station radiation patterns for use in interference assessment involving non-GSO satellites in frequency bands between 10.7 GHz and 30 GHz

The ITU Radiocommunication Assembly,

considering

a)	that WRC-2000 adopted, in Article 22 of the Radio Regulations (RR), single-entry limits applicable to non-geostationary satellite orbit (non-GSO) fixed-satellite service (FSS) systems in certain parts of the frequency range 10.7-30 GHz to protect geostationary satellite orbit (GSO) networks operating in the same frequency bands from unacceptable interference;

b)	that these frequency bands are currently used or planned to be used extensively by geostationary-satellite orbit systems (GSO systems);

c)	that during the examination under Nos. 9.35 and 11.31, the Bureau examines non-GSO FSS systems to ensure their compliance with the single-entry epfd limits given in Tables 22-1A, 22-1B, 22-1C, 22-1D, 22-1E, 22-2 and 22-3 of Article 22 of the RR;

d)	that to perform the regulatory examination referred to in considering c), the Radiocommunication Bureau (BR) requires a software tool that permits the calculation of the power levels produced by such systems, on the basis of the specific characteristics of each non-GSO FSS system submitted to the Bureau for coordination or notification, as appropriate;

e)	that GSO FSS and GSO broadcasting-satellite service (BSS) systems have individual characteristics and that interference assessments will be required for multiple combinations of antenna characteristics, interference levels and probabilities;

f)	that designers of satellite networks (non-GSO FSS, GSO FSS and GSO BSS) require knowledge of the basis on which the BR will make such checks;

g)	that WRC-19 adapted a methodology which assesses the impact of interference from non-GSO systems into GSO networks in the frequency range 37.5-51.4 GHz that utilises epfd statistics as an input;

h)	that such tools may be already developed or under development and may be offered to the BR,

recommends

that the functional description specified in Annex 1 should be used to develop software tools calculating the power levels produced by non-GSO FSS systems and the compliance of these levels with the limits contained in Tables 22-1A, 22-1B, 22-1C, 22-1D, 22-1E, 22-2 and 22-3 of Article 22 of the RR.

NOTE – The ITU-R agreed to continue working on a further revision of this Recommendation and has prepared a draft Workplan (in relation with this Recommendation) and a Working Document.

Annex 1

Functional description of software for use by the BR in checking 
compliance of non-GSO FSS systems with epfd limits

TABLE OF CONTENTS

Page

Annex 1 – Functional description of software for use by the BR in checking  compliance of non-GSO FSS systems with epfd limits		3

PART A – Fundamental constraints and basic assumptions		5

A1	General		5

A2	Fundamental assumptions		7

A3	Modelling methodology		9

PART B – Input parameters		10

B1	Introduction		10

B2	BR supplied parameters to software		10

B3	Non-GSO system inputs to software		11

B4	pfd/e.i.r.p. masks		15

B5	Validation of input parameters		16

Attachment to Part B		18

PART C – Generation of pfd/e.i.r.p. masks		25

C1	Definition		25

C2	Generation of satellite pfd masks		25

C3	Generation of e.i.r.p. masks		34

C4	pfd and e.i.r.p. mask format		36

PART D – Software for the examination of non-GSO filings		46

D1	Introduction		46

D2	Determination of runs to execute		47

D3	Worst-case geometry		50

D4	Calculation of time step size and number of time steps		81

D5	epfd calculation description		94

D6	Geometry and algorithms		112

D7	Structure and format of results		135

PART E – Testing of the reliability of the software outputs		137

E1	Evaluation of the computation accuracy of the candidate software		137

E2	Evaluation of the epfd(↓/↑) statistics obtained by the BR		138

E3	Verification of the pfd masks		138

E4	Re-testing of the BR software after any modifications or upgrades		138

PART F – Software implementing this Recommendation		138

F1	Operating system		138

F2	Interfaces to existing software and databases		138

F3	User manual		139

PART A

Fundamental constraints and basic assumptions

# A1	General

## A1.1	Purpose

The software algorithm described in this Annex is designed for its application by the BR to conduct examination of the non-GSO FSS system notifications for their compliance with the limits contained in Tables 22-1A, 22-1B, 22-1C, 22-1D, 22-1E, 22-2 and 22-3 of Article 22 of the RR.

The algorithm could also under certain conditions permit examination of whether coordination is required between non-GSO FSS systems and large earth stations under Articles 9.7A and 9.7B using the criteria in RR Appendix 5.

The algorithm uses the parameters of the non-GSO system to derive statistics of epfd. These epfd statistics could be used as the input to other methodologies that assess interference from non-GSO systems into GSO network(s) in bands where there are no epfd limits in Article 22 of the RR.

The algorithm in this Recommendation was developed based upon a reference GSO satellite in equatorial orbit with zero inclination angle. The analysis to determine whether a non-GSO satellite system meets the epfd limits in Article 22 of the RR is made by calculating the epfd levels at the reference GSO satellite or at an earth station pointing towards it. A GSO satellite network operating with non-zero inclination angles could receive higher epfd levels than those predicted by this algorithm, noting that the analysis under RR Nos 22.5C, 22.5D, 22.5F are conducted assuming zero inclination of the GSO satellite. Analysis under RR Nos. 9.7A and 9.7B, however, is to determine if coordination is required by comparing against the trigger level in Appendix 5 of the RR, and therefore in this case other methodologies, including those that assume non-zero GSO satellite inclination, could be acceptable alternatives.

## A1.2	Software block-diagram

The block-diagram of the software algorithm described in this Annex is shown in Fig. 1. It comprises initial data and calculation for the notifying administration and the BR. The data section contains the whole set of parameters relevant to the notified non-GSO system, a set of reference GSO system parameters as well as epfd limits provided by the BR.

The calculation section is designed for estimations required to examine notified non-GSO systems compliance with the epfd limits. The calculation section is based on a concept of a downlink power flux-density (pfd) mask (see Note 1), an uplink effective isotropic radiated power (e.i.r.p.) mask (see Note 2) and inter-satellite e.i.r.p. mask (see Note 3).

NOTE 1 – A pfd mask is a maximum pfd produced by a non-GSO space station and is defined in Part C.

NOTE 2 – An e.i.r.p. mask is a maximum e.i.r.p. radiated by a non-GSO earth station and is a function of latitude and the angle from the boresight of the transmitting antenna main beam to a point on the GSO arc.

NOTE 3 – An inter-satellite e.i.r.p. mask is a maximum e.i.r.p. radiated by a non-GSO space station and is a function of latitude and the angle between the line to the sub-satellite point and a point on the GSO arc.

The pfd/e.i.r.p. masks are calculated by the filing administration as identified in Block 1 and then supplied with the other non-GSO system parameters in Blocks a and b. The BR supplies additional parameters, in particular the epfd limits in Block c.

FIGURE 1

Stages in epfd verification – key logic blocks

## A1.3	Allocation of responsibilities between Administrations and the BR for software employment

Taking into account significant complexity regarding specific features of different non-GSO system configurations in the software it would seem appropriate to impose some burden of responsibility relevant to testing for epfd limits on administrations notifying appropriate non-GSO systems. Therefore the examination procedure for meeting epfd limits would consist of two stages. The first stage would include derivation of a mask for pfd/e.i.r.p. produced by interfering non-GSO network stations. The mask would account for all the features of specific non-GSO systems arrangements (such as possible beam pointing and transmit powers). The first stage would be finalized by the delivery of the pfd/e.i.r.p. mask to the BR.

The second stage calculations would be effected at the BR. The second stage would feature the following operations:

–	Identification of the runs required for a non-GSO network taking into account the frequencies for which it has filed and the frequency ranges for which there are epfd limits in RR Article 22 (Block 2).

–	Definition of the maximum epfd geometry of a GSO space station and an earth station of that network (Block 3). It would ensure verification of sharing feasibility for a notified non-GSO network with any GSO network in the FSS and BSS.

–	epfd statistics estimation (Block 4).

–	Making a decision on interference compliance with appropriate epfd limits.

The estimations are based on the non-GSO system parameters (Blocks a and b) delivered by a notifying administration and the initial data (Block c) available at the BR.

Any administration may use software that uses the algorithms defined in this Annex together with data on the non-GSO networks to estimate statistics for interference into its own GSO networks and check for compliance with epfd limits. It would assist in solving probable disputes between the BR and administrations concerned.

The elements of the software block-diagram discussed are presented hereinafter in detail. The Parts are as follows:

Part A	–		Basic limitations and main system requirements for the software as a whole are presented.

Part B	–		Non-GSO networks parameters and initial data for Blocks a and b are discussed.

Part C	–		Definitions and estimation algorithms for pfd/e.i.r.p. masks relative to non-GSO network earth and space stations are presented. Specifics of those masks applying in simulation is also discussed (Block 1).

Part D	–		This Part deals with general requirements on the software related to examination of non-GSO networks notifications, algorithms to estimate epfd statistics, and the format for output data presentation. Part D covers issues of Blocks 2, 3 and 4.

Parts E, F	–	These Parts define requirements on the software related to valuation of delivered software and to verification of the software output on validity.

# A2	Fundamental assumptions

## A2.1	Units of measurement

To provide for adequate simulation results and to avoid errors a common measurement units system is used in Table 1 for the software description. The list of measurement units for the basic physical parameters is shown in Table 1.

TABLE 1

The system of measurement units for basic physical parameters used
for the software performance description

| Parameter | Units |
| --- | --- |
| Distance | km |
| Angle | degree |
| Time | s |
| Linear rotation velocity | km/s |
| Angular rotation velocity | degree/s |
| Frequency | MHz |
| Frequency bandwidth | kHz |
| Power | dBW |
| Power spectral density | dB(W/Hz) |
| pfd | dB(W/(m2 · BWref)) |
| Average number of co-frequency non-GSO earth stations per unit area | 1/km2 |
| epfd, epfd or epfdis | dB(W/BWref) |
| Antenna gain | dBi |
| Geographical position on the Earth’s surface | degrees |

## A2.2	Constants

The functional description of the software for examination of non-GSO networks notification at the BR uses the constants shown in Table 2.

TABLE 2

Constants to be used by software

| Parameter | Notation | Numerical value | Units |
| --- | --- | --- | --- |
| Radius of the Earth | Re | 6 378.145 | km |
| Radius of geostationary orbit | Rgeo | 42 164.2 | km |
| Gravitational constant |  | 3.986 012  105 | km3/s2 |
| Speed of light | c | 2.997 924 58  105 | km/s |
| Angular rate of rotation of the Earth | e | 4.178 074 582 3  10–3 | degree/s |
| Earth rotation period | Te | 86 164.090 54 | s |
| Factor of the Earth non-sphericity | J2 | 0.001 082 636 | – |

## A2.3	The Earth model

The force of the Earth attraction is the main factor to define a satellite orbital motion. Additional factors include:

–	orbit variations due to the Earth’s oblateness and its mass distribution irregularities;

–	solar and lunar attraction;

–	medium drag for satellite;

–	solar radiation pressure, etc.

The function description of software in this Annex accounts orbit perturbations only due to the Earth oblateness. It is motivated by the fact that effect of other perturbing factors is significantly less. The Earth’s oblateness causes secular and periodic perturbations of ascending node longitude and orbit perigee argument. Section D6.3 describes expressions to account the Earth’s oblateness effect.

The orbits of some repeating ground tracks can be very sensitive to the exact orbit model used. Administrations could also provide the BR with their own independently determined average precession rates that could be used by the software instead of the values calculated using the equation in § D6.3.

## A2.4	Constellation types

The algorithm in this Recommendation has been developed to be applicable to at least the non-GSO satellite systems shown in Table 3.

Constellations can contain sub-constellations with different orbit parameters and shape, but all sub-constellation within a constellation have to be either repeating or non-repeating. If the constellation is repeating then the repeat period specified must be appropriate for all non-GSO satellites, including all sub-constellations.

TABLE 3

Orbit type classification

| Type | Orbit shape | Equatorial ? | Repeating ? |
| --- | --- | --- | --- |
| A | Circular | No | Yes |
| B | Circular | No | No |
| C | Circular | Yes | n/a |
| D | Elliptical(1) | No | Yes |
| E | Elliptical(1) | No | No |
| (1) Assuming that the elliptical system has perigee and apogee at the extrema in latitude, i.e. with active arc at highest or lowest latitude. |  |  |  |

# A3	Modelling methodology

The approach described in this Annex involves time simulation in which interference levels are evaluated on a time step by time step basis. Section D4 defines the method to calculate the size of time steps and the total number of time steps to be used. That section also identifies an optional dual time step approach to reduce run times without altering the resulting decision.

PART B

Input parameters

# B1	Introduction

## B1.1	Background

Certain parameters for a non-GSO network and other data must be specified in order to accomplish the requisite software functions:

–	Function 1: Provide the pfd masks for the non-GSO satellites (downlink) and the e.i.r.p. mask for the earth stations transmitting to those satellites (uplink) or non-GSO satellite (intersatellite).

–	Function 2: Apply the pfd/e.i.r.p. mask in the calculation of downlink epfd↓, uplink epfd↑ and/or intersatellite epfd levels (cumulative time distributions of epfd).

–	Function 3: Determine whether the pfd/e.i.r.p. mask levels are consistent with basic transmission parameters of the non-GSO network, only in the case of dispute.

The roles of the administration of the non-GSO network and the BR are discussed in § A1.3.

Detailed parameters are needed by the BR in support of Function 2 and so this section focuses on the parameters needed to fulfil that requirement.

The parameters provided should be consistent, so that if the administrations modify its network (e.g. if there are changes to the constellation) so that the pfd/e.i.r.p. changes then a new mask would need to be supplied to the BR.

## B1.2	Scope and overview

This section identifies inputs to the software in four main sections:

–	Section B2 defines the inputs from the BR;

–	Section B3 defines the inputs from the non-GSO operator excluding the pfd/e.i.r.p. masks;

–	Section B4 defines the pfd/e.i.r.p. masks.

–	An Attachment to Part B then maps the parameters to the SRS database tables.

Note that in the following tables, square brackets in variable names indicate an index for that variable and not tentative text.

# B2	BR supplied parameters to software

The BR supplies two types of data, firstly the type of run to execute:

| RunType | One of {Article 22, 9.7A, 9.7B} |
| --- | --- |
| SystemID | ID of system to examine (either non-GSO or large earth station) |

The second data is to provide the threshold epfd levels to use as the pass / fail criteria. These are accessed by the software when generating the runs and consists of a series of records as follows:

| epfddirection | One of {Down, Up, IS} |
| --- | --- |
| VictimService | One of {FSS, BSS} |
| StartFrequencyMHz | Start of frequency range for which epfd threshold applies |
| EndFrequencyMHz | End of frequency range for which epfd threshold applies |
| VictimAntennaType | Reference code for antenna pattern to be used in calls to ITU supplied antenna gain pattern DLL |
| VictimAntennaDishSize | Dish size of victim antenna pattern to be used in calls to ITU supplied antenna gain pattern DLL |
| VictimAntennaBeamwidth | Beamwidth of victim antenna pattern to be used in calls to ITU supplied antenna gain pattern DLL |
| RefBandwidthHz | Reference bandwidth in Hz of epfd level |
| NumPoints | Number of points in epfd threshold mask |
| epfdthreshold[N] | epfd level in dBW/m2/Reference bandwidth |
| epfdpercent[N] | Percentage of time associated with epfd threshold |

# B3	Non-GSO system inputs to software

These are split into constellation parameters, orbit parameters for each space station and one or more sets of system operating parameters.

## B3.1	Non-GSO constellation parameters

| Nsat | Number of non-GSO satellites |
| --- | --- |
| H_MIN | Minimum operating height (km) |
| DoesRepeat | Flag to identify that constellation repeats using station keeping to maintain track |
| AdminSuppliedPrecession | Flag to identify that the constellation orbit model precession field is supplied by the admin |
| Wdelta | Station keeping range (degrees) |
| ORBIT_PRECESS | Administration supplied precession rate (degrees/second) |

## B3.2	Non-GSO space station parameters

For each of the non-GSO satellites, the following parameters to define the location of the constellation at the start of the simulation:

| A[N] | Semi-major axis of orbit (km) |
| --- | --- |
| E[N] | Eccentricity of orbit |
| I[N] | Inclination of orbit (degrees) |
| O[N] | Longitude of ascending node of orbit (degrees) |
| W[N] | Argument of perigee (degrees) |
| V[N] | True anomaly (degrees) |

## B3.3	Non-GSO system operating parameters

These represent the set of parameters required to define the non-GSO system’s operations. There could be different sets of parameters at different frequency bands, but only one set of operating parameters for any frequency band used by the non-GSO system.

| Freq_Min | Minimum frequency for which this set of parameters applies |
| --- | --- |
| Freq_Max | Maximum frequency for which this set of parameters applies |
| MIN_EXCLUDE[Latitude] | Exclusion zone angle (degrees), the minimum angle to the GSO arc at the non-GSO ES at which the non-GSO system can provide a service to that location, defined at the ES ( angle) by latitude. The MIN_EXCLUDE at a specific latitude will be derived using linear interpolation between data points. This field could vary between non-GSO system orbit planes via the orb_id field. If the orb_id field equals 0 then the data exclusion zone data applies to all orbit planes |
| MIN_ELEV[Latitude][Azimuth] | Minimum elevation angle of the non-GSO earth station when it is receiving or transmitting (degrees) by latitude and azimuth. The nearest latitude to that in the table will be used and then linear interpolation in azimuth |
| MIN_DURATION[Latitude] | Minimum satellite tracking duration at latitude (seconds): the nearest latitude to that in the table will be used |
| MAX_CO_FREQ[Latitude] | Maximum number of co-frequency tracked non-GSO satellites by latitude: the nearest latitude to that in the table will be used |
| ES_DENSITY | Average number of non-GSO earth stations active at the same time (/km2) |
| ES_DISTANCE | Average distance between cell or beam footprint centre (km) |
| ES_LAT_MIN | Minimum limit of the latitude range of non-GSO ES (degrees) |
| ES_LAT_MAX | Maximum limit of the latitude range of non-GSO ES (degrees) |
| MIN_ANGLE_AT_ES | Minimum angle in degrees at the surface of the Earth between the lines to any two active non-GSO satellites. Assumed to be zero if not provided. Not applicable if the MIN_DURATION[Latitude] is non-zero |
| MIN_ANGLE_AT_SAT | The minimum angle in degrees at the non-GSO satellite between the lines to any two active non-GSO earth stations. Assumed to be zero if not provided |
| MAX_CO_FREQ_SAT | The maximum number of non-geostationary earth stations tracked co-frequency by a non-geostationary satellite within the EPFD(up) calculation. If a value is not provided, it is assumed that the maximum number of earth stations tracked co-frequency by a non-geostationary satellite is equal to the number of earth stations created for the epfd↑ run |

These parameters would be supplied in XML format with header as follows:

<non_gso_operating_parameters es_lat_max="+90" es_lat_min="-90" es_distance="200" es_density="0.00001" c_name="orb_id" b_name="azimuth" a_name="latitude" high_freq_mhz="F2" low_freq_mhz="F1" param_id="1" angle_at_es = "5" min_angle_at_sat = 5>

where:

| Field | Type or range | Units |  | Example |
| --- | --- | --- | --- | --- |
| ntc_id | Integer | – |  | 12345678 |
| sat_name | String | – |  | My satellite network |
| param_id | Integer | – |  | 1 |
| low_freq_mhz | Double precision | MHz |  | 10 000 |
| high_freq_mhz | Double precision | MHz |  | 12 000 |
| a_name | {latitude} range −90 to +90 degrees | – |  | latitude |
| b_name | {azimuth} range 000 to 360 degrees | – |  | azimuth |
| c_name | {orb_id} range 01 to 9999 0 should be used to indicate that parameters will be applicable to all orbits | – |  | orb_id |
| es_density | Double precision | km2 |  | 0.000 1 |
| es_distance | Double precision | km |  | 200 |
| es_lat_min | Double precision | degree |  | −90 |
| es_lat_max | Double precision | degree |  | +90 |
| min_angle_at_sat | Double precision range 0 to +90 degrees | degree | 15 |  |
| min_angle_at_es | Double precision range 0 to +90 degrees | degree | 5 |  |
| max_co_freq | Integer range 0 to 9999 | - | 2 |  |
| max_co_freq_sat | Integer, range 0 to 9999 | - | 2 |  |
| min_duration | Integer, range 0 to 99999 | second | 400 |  |
| elev_angle | Double precision, range 0 to +90 degrees | degree | 5 |  |

After the header the XML contains arrays of MIN_EXCLUDE which can vary by orb_id and latitude while the MIN_DURATION, MAX_CO_FREQ values vary only by latitude. The MIN_ELEV array can vary by both latitude and azimuth.

Note that if the non-GSO ES type defined in the e.i.r.p. mask is specific rather than typical, then the fields es_density and es_distance are not used.

The non-GSO operating parameters would be stored in the same database as the pfd and e.i.r.p. masks.

An example non-GSO operating parameters XML file would therefore be:

<?xml version="1.0"?>

<satellite_system sat_name="MySatName" ntc_id="12345678">

<non_gso_operating_parameters es_lat_max="+90" es_lat_min="-90" es_distance="200" es_density="0.00001" c_name="orb_id" b_name="azimuth" a_name="latitude" high_freq_mhz="F2" low_freq_mhz="F1" param_id="1" min_angle_at_sat = "0" min_angle_at es =0>

<min_exclude c="1">

<exclusion_zone_angle a="-75">0</exclusion_zone_angle>

<exclusion_zone_angle a="-45">3</exclusion_zone_angle>

<exclusion_zone_angle a="-15">5</exclusion_zone_angle>

<exclusion_zone_angle a="15">5</exclusion_zone_angle>

<exclusion_zone_angle a="45">3</exclusion_zone_angle>

<exclusion_zone_angle a="75">0</exclusion_zone_angle>

</min_exclude>

<min_exclude oc="2">

<exclusion_zone_angle a="-75">0</exclusion_zone_angle>

<exclusion_zone_angle a="-45">4</exclusion_zone_angle>

<exclusion_zone_angle a="-15">6</exclusion_zone_angle>

<exclusion_zone_angle a="15">6</exclusion_zone_angle>

<exclusion_zone_angle a="45">6</exclusion_zone_angle>

<exclusion_zone_angle a="75">0</exclusion_zone_angle>

</min_exclude>

<max_co_freq a="0">2</max_co_freq>

<min_duration a="-50">400</min_duration>

<min_duration a="0">1000</min_duration>

<min_duration a="50">400</min_duration>

<min_elev a="-30">

<elev_angle b="0">30</elev_angle>

<elev_angle b="90">40</elev_angle>

<elev_angle b="280">30</elev_angle>

<elev_angle b="370">40</elev_angle>

</min_elev>

<min_elev a="0">

<elev_angle b="0">20</elev_angle>

<elev_angle b="90">30</elev_angle>

<elev_angle b="280">20</elev_angle>

<elev_angle b="370">30</elev_angle>

</min_elev>

<min_elev a="30">

<elev_angle b="0">30</elev_angle>

<elev_angle b="90">40</elev_angle>

<elev_angle b="280">30</elev_angle>

<elev_angle b="370">40</elev_angle>

</min_elev>

</non_gso_operating_parameters>

</satellite_system>

# B4	pfd/e.i.r.p. masks

## B4.1	Non-GSO downlink pfd mask

| FreqMin | Minimum of frequency range in MHz for this pfd mask |
| --- | --- |
| FreqMax | Maximum of frequency range in MHz for this pfd mask |
| RefBW | The power level in the pfd mask should be given in kHz with respect to the same reference bandwidth as the epfd thresholds in the tables in Article 22 relevant for the frequency ranges covered. If the tables in Article 22 give two reference bandwidths (e.g. 40 kHz and 1 MHz) then the smaller bandwidth should be used. |
| MaskType | One of {, or (az,el)} |
| Option 1 pfd_mask (satellite, latitude, , L) | The pfd mask is defined by: – the non-GSO satellite – the latitude of the non-GSO sub-satellite point – the separation angle between this non-GSO space station and the GSO arc, as defined in § D6.4.4 – the difference L in longitude between the non-GSO sub-satellite point and the point on the GSO arc where the  angle is minimised as defined in § D6.4.4 |
| Option 2 pfd_mask (satellite, latitude, Az, El) | The pfd mask is defined by: – the non-GSO satellite – the latitude of the non-GSO sub-satellite point – the azimuth angle, defined in § D6.4.5 – the elevation angle, defined in § D6.4.5 |

## B4.2	Non-GSO uplink e.i.r.p. mask

| FreqMin | Minimum of frequency range in MHz for this pfd mask |
| --- | --- |
| FreqMax | Maximum of frequency range in MHz for this pfd mask |
| RefBW | The power level in the e.i.r.p. mask should be given in kHz with respect to the same reference bandwidth as the epfd thresholds in the tables in Article 22 relevant for the frequency ranges covered. If the tables in Article 22 give two reference bandwidths (e.g. 40 kHz and 1 MHz) then the smaller bandwidth should be used. |
| ES_ID | Reference of non-GSO ES or –1 if using generic ES |
| MaskFormat | Either LatitudeAndAngleGSOArc or LatitudeAzimuthElevationDeltaLong |
| ES_e.i.r.p. [Lat] [] | Non-GSO earth station e.i.r.p. as a function of latitude and the angle between the line from the non-GSO ES boresight line and the line from the non-GSO ES to a point on the GSO arc. |
| ES_e.i.r.p.[Lat][Az][El][DeltaLongES] | Non-GSO ES e.i.r.p. as a function of non-GSO ES latitude, current pointing (azimuth, elevation) and difference in longitude between non-GSO ES and point on the GSO arc |

## B4.3	Non-GSO inter-satellite e.i.r.p. mask

| FreqMin | Minimum of frequency range in MHz for this e.i.r.p. mask |
| --- | --- |
| FreqMax | Maximum of frequency range in MHz for this e.i.r.p. mask |
| RefBW | The power level in the e.i.r.p. mask should be given in kHz with respect to the same reference bandwidth as the epfd thresholds in the Tables in Article 22 relevant for the frequency ranges covered. If the Tables in Article 22 give two reference bandwidths (e.g. 40 kHz and 1 MHz) then the smaller bandwidth should be used. |
| SAT_e.i.r.p.[Lat] [] | Non-GSO satellite e.i.r.p. as a function of latitude and the angle seen from the non-GSO satellite between the non-GSO sub-point and a point on the GSO arc. |

# B5	Validation of input parameters

This section describes the minimum met of validation of the input parameters: additional checks could also be undertaken.

## B5.1	Non-GSO space station parameters

This methodology is applicable for the types of non-GSO systems with orbital characteristics as defined in Table 3. To ensure consistency with this assumption, the following tests are required to be undertaken for each non-GSO satellite:

Test for circular or near-circular orbit:

If the e > 0 and e < MAX_CIRCULAR_E then

{

WarningMessage: setting orbit to be circular from eccentricity = e

Set e = 0 and continue

}

Test HEO system has  = ±/2:

If the eccentricity >= MAX_CIRCULAR_E then

{

Ensure w in range {−, +}

If (abs(/2 – abs(w))) > MAX_HELO_DELTAW

{

ErrorMessage: orbit apogree not at maximum latitude

Exit

}

It is assumed that:

MAX_CIRCULAR_E = 0.01

MAX_HELO_DELTAW = 1e-5 degrees

With systems that use multiple sub-constellations, it must be checked that all are repeating or all are non-repeating.

## B5.2	Non-GSO system operating parameters ranges

The following non-GSO system operating parameters are to be checked:

MIN_EXCLUDE[Latitude]  0

MIN_ELEV[Latitude, Azimuth]  0

MIN_DURATION[Latitude]  1 second

MAX_CO_FREQ[Latitude]  0

ES_DENSITY > 0

ES_DISTANCE  0 (ES_DISTANCE = 0 means that only one earth station can be distributed inside the victim GSO footprint)

+90º > ES_LAT_MIN  −90º

+90º  ES_LAT_MAX > −90º

ES_LAT_MAX > ES_LAT_MIN

MIN_ANGLE_AT_ES  0

MIN_ANGLE_AT_SAT  0

MAX_CO_FREQ_SAT  0

## B5.3	Masks and system operating parameters XML files

The following non-GSO system operating parameters are to be checked:

–	That there is only a single non-GSO system operating parameters set per frequency range.

–	That there is a non-GSO system operating parameters set for each frequency range that is to be examined.

–	That if the MIN_EXCLUDE varies by orbit plane, that a value is defined for each orbit plane.

It is also necessary to check that any ES e.i.r.p. masks using the LatitudeAndAngleGSOArc format are monotonically decreasing.

Attachment 
to Part B

This Attachment to Part B details the parameters that the epfd software uses from the SRS database. If the extended set of system operating parameters are provided, then values should be taken from that XML file rather than the relevant SRS table.

Table 4 lists the current RR Appendix 4 information for non-GSO satellite systems included in the BR space networks system (SNS) database. The relationship between the database tables is shown in Fig. 2. Mask information and link tables are not shown in Fig. 2 but are described in Table 4.

Format description

| Value | Description |
| --- | --- |
| X | Used to describe alphanumeric data. e.g. X(9) specifies a 9-character field containing alphanumeric data XXX is equivalent to X(3). |
| 9 | Used to describe digits |
| ‘.’ | Shows the position of a decimal point |
| S | Implies a sign (sign leading separate) e.g. S999.99 implies a numeric field with a range of values from –999.99 to +999.99 99 implies a numeric field with a range of values from 0 to 99 |

Figure 2

Extract from SRS entity relationship

TABLE 4

SRS data for epfd analysis

Notice

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Primary Key |
| ntc_type | Text | X | Code indicating if the notice is of a geostationary satellite [G], Non-geostationary satellite [N], specific earth station [S] or typical earth station [T] | value != Null |
| d_rcv | Date/Time | 9(8) | Date of receipt of the notice |  |
| ntf_rsn | Text | X | Code indicating that the notice has been submitted under RR1488 [N], RR1060 [C], RR1107 [D], 9.1 [A], 9.6 [C], 9.7A [D], 9.17 [D], 11.2 [N], 11.12 [N], AP30/30A-Articles 2A, 4 and 5 [B], AP30B-Articles 6 and 7 [P], AP30B-Article 8 [N] or Res49 [U] | The software looks a value that is ‘C’ or ‘N’ |
| st_cur | Text | XX | Current processing status of the notice | The software looks for a value that is ‘50’ in the Article 9.7A check |

Non-geo

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Primary Key |
| sat_name | Text | X(20) | Name of the satellite |  |
| nbr_sat_td | Number | 9 | Nco for the uplink case |  |
| density | Number | 9(9) | Average density of non-GSO ES for the uplink case |  |
| avg_dist | Number | 9(9) | Average distance between co-frequency non-GSO ES for the uplink case |  |
| f_x_zone | Text | X | Should be ‘Y’ for alpha based exclusion zone |  |
| x_zone | Number | 9(9) | Size of exclusion zone |  |

orbit

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Foreign Key |
| orb_id | Number | 99 | Sequence number of the orbital plane | Primary Key |
| nbr_sat_pl | Number | 99 | Number of satellites per non-geostationary orbital plane | value != Null && value > 0 |
| right_asc | Number | 999.99 | Angular separation in degrees between the ascending node and the vernal equinox | value != Null |
| inclin_ang | Number | 999.9 | Inclination angle of the satellite orbit with respect to the plane of the Equator | value != Null |

orbit (continued)

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| apog | Number | 9(5).99 | The farthest altitude of the non-geostationary satellite above the surface of the Earth or other reference body – expressed in kilometres Distances > 99 999 km are expressed as a product of the values of the fields “apog” and “apog_exp” (see below) e.g. 125 000 = 1.25 × 105 | value != Null && value > 0 |
| apog_exp | Number | 99 | Exponent part of the apogee expressed in power of 10 To indicate the exponent; give 0 for 100, 1 for 101, 2 for 102, etc. | value != Null && value >= 0 |
| perig | Number | 9(5).99 | The nearest altitude of the non-geostationary satellite above the surface of the Earth or other reference body – expressed in kilometres Distances > 99 999 km are expressed as a product of the values of the fields “perigee” and “perig_exp” (see below) e.g. 125 000 = 1.25 × 105 | value != Null && value > 0 |
| perig_exp | Number | 99 | Exponent part of the perigee expressed in power of 10 To indicate the exponent; give 0 for 100, 1 for 101, 2 for 102, etc. | value != Null && value >= 0 |
| perig_arg | Number | 999.9 | Angular separation (degrees) between the ascending node and the perigee of an elliptical orbit. If RR No. 9.11A applies |  |
| op_ht | Number | 99.99 | Minimum operating height of the non-geostationary satellite above the surface of the Earth or other reference body – expressed in kilometres Distances > 99 km are expressed as a product of the values of the fields “op_ht” and “op_ht_exp” (see below) e.g. 250 = 2.5 × 102 | value != Null && value > 0 |
| op_ht_exp | Number | 99 | Exponent part of the operating height expressed in power of 10 To indicate the exponent; give 0 for 100, 1 for 101, 2 for 102, etc. | value != Null && value >= 0 |
| f_stn_keep | Text | X | Flag indicating if the space station uses [Y] or does not use [N] station-keeping to maintain a repeating ground track | value != Null && (value == ‘Y’ ¦¦ ‘N’) |
| rpt_prd_dd | Number | 999 | Day part of constellation repeat period (s) |  |
| rpt_prd_hh | Number | 99 | Hour part of constellation repeat period (s) |  |
| rpt_prd_mm | Number | 99 | Minute part of constellation repeat period (s) |  |
| rpt_prd_ss | Number | 99 | Second part of constellation repeat period (s) |  |
| f_precess | Text | X | Flag indicating if the space station should [Y] or should not [N] be modelled with specific precession rate of the ascending node of the orbit instead of the J2 term | value != Null && (value == ‘Y’ ¦¦ ‘N’) |

orbit (end)

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| precession | Number | 999.99 | For a space station that is to be modelled with specific precession rate of the ascending node of the orbit instead of the J2 term, the precession rate in degrees/day measured counter-clockwise in the equatorial plane | If f_precess == ‘Y’ then value != Null && value > = 0 |
| long_asc | Number | 999.99 | Longitude of the ascending node for the jth orbital plane measured counter-clockwise in the equatorial plane from the Greenwich meridian to the point where the satellite orbit makes its south-north crossing of the equatorial plane (0° = j < 360°) | value != Null && value > = 0 |
| keep_rnge | Number | 99.9 | Longitudinal tolerance of the longitude of the ascending node | If f_stn_keep == ‘Y’ then value != Null && value > = 0 |

Phase

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Foreign Key |
| orb_id | Number | 99 | Sequence number of the orbital plane | Foreign Key |
| orb_sat_id | Number | 99 | Satellite sequence number in the orbital plane | value != Null && value > = 0 |
| phase_ang | Number | 999.9 | Initial phase angle of the satellite in the orbital plane If RR No. 9.11A applies | value != Null && value > = 0 |

Grp

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Foreign Key |
| grp_id | Number | 9(9) | Unique identifier of the group | Primary Key |
| emi_rcp | Text | X | Code identifying a beam as either transmitting [E] or receiving [R] | value != Null && (value == ‘E’ ¦¦ ‘R’) |
| beam_name | Text | X(8) | Designation of the satellite antenna beam |  |
| elev_min | Number | S9(3).99 | Minimum elevation angle at which any associated earth station can transmit to a non-geostationary satellite or minimum elevation angle at which the radio astronomy station conducts single-dish or VLBI observations | value != Null && value > = 0 |
| freq_min | Number | 9(6).9(6) | Minimum frequency in MHz (assigned frequency – half bandwidth) (of all frequencies for this group) | value != Null && value > 0 |
| freq_max | Number | 9(6).9(6) | Maximum frequency in MHz (assigned frequency + half bandwidth) (of all frequencies for this group) | value != Null && value > 0 |
| d_rcv | Date/Time | 9(8) | Date of receipt of the list of frequency assignments pertaining to the group |  |
| noise_t | Number | 9(6) | Receiving system noise temperature | Only validated for 9.7A/B checks |

srv_cls

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| grp_id | Number | 9(9) | Unique identifier of the group | Foreign Key |
| seq_no | Number | 9(4) | Sequence number | value != Null && value > = 0 |
| stn_cls | Text | XX | Class of station |  |

sat_oper

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the group | Foreign Key |
| lat_fr | Number | 9(9) | Start of latitude range | −90 <= lat_fr < lat_to |
| lat_to | Number | 9(9) | End of latitude range | lat_fr < lat_to <= 90 |
| nbr_op_sat | Number | 9 | Nco for the down direction to use for latitudes in the range lat_fr to lat_to | > 0 |

mask_info

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the group | Foreign Key |
| mask_id | Number | 9(4) | Sequence number | Foreign Key |
| f_mask | Text | X | Code identifying a mask as either earth station e.i.r.p. ‘E’ for space station e.i.r.p. or ‘S’ for space station or ‘P” for pfd | value != Null && (value == ‘E’ ¦¦ ‘S’¦¦ ‘P’) |
| f_mask_type | Text | X | Code describing format of mask, where ‘A’ = Alpha, ‘Z’ = Azimuth/elevation , ‘O’ = off-axis and ‘D’ = azimuth, elevation and deltaLong | value != Null && (value == ‘A’ ¦¦ ‘Z’¦¦ ‘O’¦¦’D’) |

e_as_stn

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| grp_id | Number | 9(9) | Unique identifier of the group | Foreign Key |
| seq_no | Number | 9(4) | Sequence number | value != Null && value >= 0 |
| stn_name | Text | X(20) | Name of the transmitting or receiving station |  |
| stn_type | Text | X | Code indicating if the earth station is specific ‘S’ or typical ‘T’ | value != Null && (value == ‘S’ ¦¦ ‘T’) |
| bmwdth | Number | 999.99 | Angular width of radiation main lobe expressed in degrees with two decimal positions | value != Null && value > 0 |

mask_lnk1

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| grp_id | Number | 9(9) | Unique identifier of the group | Foreign Key |
| mask_id | Number | 9(4) | Unique identifier of the mask | Foreign Key |
| orb_id | Number | 99 | Sequence number of the orbital plane | Foreign Key |
| sat_orb_id | Number | 99 | Satellite sequence number in the orbital plane | value != Null && value > = 0 |

mask_lnk2

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| grp_id | Number | 9(9) | Unique identifier of the group | Foreign Key |
| seq_e_as | Number | 9(4) | Sequence number of the associated earth station | Foreign Key |
| mask_id | Number | 9(4) | Unique identifier of the mask | Foreign Key |

mask_lnk3

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Foreign Key |
| param_id | Number | 9(4) | Unique identifier of the system operating parameters | Foreign Key |

Tables used in the Article 9.7A/9.7B calculations

e_stn

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Foreign Key |
| stn_name | Text | X(20) | Name of the earth station | value != Null |
| sat_name | Text | X(20) | Name of the associated space station | value != Null |
| lat_dec | Number | S9(2).9(4) | Latitude in degrees with four decimals | value != Null |
| long_dec | Number | S9(2).9(4) | Longitude in degrees with four decimals | value != Null |
| long_nom | Number | S999.99 | Nominal longitude of the associated space station, give '–' for West, '+' for East | value != Null |

e_ant

| Data Item | Data Type | Format | Description | Validation |
| --- | --- | --- | --- | --- |
| ntc_id | Number | 9(9) | Unique identifier of the notice | Foreign key |
| emi_rcp | Text | X | Code identifying a beam as either transmitting [E] or receiving [R] | value != Null |
| bmwdth | Number | 999.99 | Beamwidth of the earth station antenna |  |
| gain | Number | S99.9 | Maximum isotropic gain of the earth station antenna |  |

PART C

Generation of pfd/e.i.r.p. masks

# C1	Definition

The purpose of generating pfd/e.i.r.p. masks is to define an envelope of the power radiated by the non-GSO space stations and the non-GSO earth stations so that the results of calculations encompass what would be radiated regardless of what resource allocation and switching strategy are used at different periods of a non-GSO system life.

These masks are regulatory constraints in that the non-GSO FSS system should not exceed those values at any time and could be derived using the methodology below. They represent an envelope of the power produced by a system which will constrain how rapidly the pfd or e.i.r.p. can change between data points and how low values for the extreme points in arrays can get. Note that it is feasible for a system not to transmit at certain latitudes: in this case a null value of −1000 dBW should be used.

# C2	Generation of satellite pfd masks

## C2.1	General presentation

The satellite pfd mask is defined by the maximum pfd generated by any space station in the interfering non-GSO system as seen from any point at the surface of the Earth. A four dimensional pfd mask is recommended for use by the BR verification software and is defined following one of the two options:

Option 1: As a function of:

–	the non-GSO satellite;

–	the latitude of the non-GSO sub-satellite point;

–	the separation angle  between this non-GSO space station and the GSO arc, as seen from any point on the surface of the Earth, as defined in § D6.4.4;

–	the difference L in longitude between the non-GSO sub-satellite point and the point on the GSO arc where the  angle is minimised, as defined in § D6.4.4.

Option 2: As a function of:

–	the non-GSO satellite;

–	the latitude of the non-GSO sub-satellite point;

–	the non-GSO satellite azimuth angle, defined in § D6.4.5;

–	the non-GSO satellite elevation angle, defined in § D6.4.5.

Whatever parameters are used to generate the pfd mask, the resulting pfd mask should be converted to one of the format options above.

Because the non-GSO space station can generate simultaneously a given maximum number of beams, it should be taken into account in order to better fit the system design and not be too constraining for non-GSO systems.

The mitigation techniques used by the non-GSO system, such as the GSO arc avoidance, are implemented in the calculation of the pfd mask. The GSO arc avoidance defines a non-operating zone on the ground in the field of view of a non-GSO space station. The location of this non-operating zone on the ground will move as a function of the latitude of the non-GSO sub-satellite point. To get a more accurate model of a non-GSO system, the latitude of the non-GSO sub-satellite point is taken as a parameter to the pfd mask calculation.

## C2.2	Mitigation techniques description

The mitigation technique implemented within the non-GSO system should be accurately explained in this section in order to be fully modelled in the calculation of the epfd↓.

With regard to the use of a non-operating zone around the GSO arc, there are at least two different ways of modelling a non-GSO system based on a cell architecture:

–	Cell-wide observance of a non-operating zone: a beam of a non-GSO space station is switched off if the separation angle between this non-GSO space station and the GSO arc, at any point of the non-GSO cell, is less than 0 (GSO arc avoidance angle).

–	Cell-centre observance of a non-operating zone: a beam of a non-GSO space station is switched off when the centre of the cell sees this non-GSO space station at less than 0 from the GSO arc.

Other mitigation techniques may be used by a non-GSO system which are not listed here. Information on these techniques will be provided by the non-GSO administration for the description and verification of the pfd mask.

## C2.3	pfd calculation

### C2.3.1	pfd calculation

The pfd radiated by a non-GSO space station at any point on the Earth’s surface is the sum of the pfd produced by all illuminating beams in the co-frequency band.

Some non-GSO systems have tracking antennas which point to cells fixed on the Earth’s surface and do not move with the spacecraft. However, since the pfd mask is generated with respect to the non-GSO location, assumptions must be made in the development of the pfd mask. Making the simplifying assumption that the cells move with the spacecraft can lead to inaccurate geographic distributions of epfd levels.

As non-GSO systems use mitigation techniques, there will be no main beam-to-main beam alignment. Therefore, de-polarization effects mean that both co-polarization and cross-polarization contributions must be included as sources of interference.

The implementation of the pfd mask explicitly accounts for both co-polarization and cross-polarization from non-GSO satellites into GSO earth stations for like types of polarization (circular-to-circular or linear-to-linear). Isolation between systems of different types of polarization (circular-to-linear) is not directly covered. A study has shown that the average total interference power over all axial ratios and polarization ellipse orientations is a very small net increase in the received interference power in the BSS antenna of 0.048 dB. The bounds of any cross-polarization contributions, that are very unlikely to be reached, are from –30 dB to 3 dB.

Then:

pfd=10 logiNco10pfd_coi/10 +jNcross10pfd_crossj/10

where:

pfd :	pfd radiated by a non-GSO space station (dB(W/m2)) in the reference bandwidth

i :	index of the beams illuminated in the polarization considered

Nco :	maximum number of beams which can be illuminated simultaneously in the polarization considered

pfd_coi :	pfd produced at the point considered at the Earth’s surface by one beam in the polarization considered (dB(W/m2)) in the reference bandwidth

j :	index of the beams illuminated in the opposite polarization to the polarization considered

Ncross :	maximum number of beams which can be illuminated simultaneously in the opposite polarization to the polarization considered

pfd_crossj :	pfd produced at the point considered at the Earth’s surface by one beam in the opposite polarization to the polarization considered (dB(W/m2)) in the reference bandwidth

and

pfd_coi= Pi+Gi-10log10(4 π d2)

where:

Pi :	maximum power emitted by the beam i in the reference bandwidth (dB(W/BWref))

BWref :	reference bandwidth (kHz)

Gi :	gain generated by the beam i in the polarization considered at the point considered at the Earth’s surface (dBi)

d : 	distance between the non-GSO space station and the point considered at the Earth’s surface (if the non-GSO satellite antenna gain is in isoflux, d is the altitude of the non-GSO space station) (m)

and

pfd_crossj= Pj+G_crossj-10log10(4 π d2)

where:

G_crossj :	cross-polarization gain generated by the beam j illuminated in the opposite polarization to the polarization considered, at the point considered at the Earth’s surface (dBi).

It is expected that the parameters used to generate the pfd/e.i.r.p. mask correspond to the performance of the non-GSO satellite over its anticipated lifetime. The pfd levels should be an envelope over all possible traffic and beam combinations and represent the peak pfd in the given direction feasible over the lifetime of the system. Non-GSO satellite systems that employ adaptive antennas in which the beam size and sidelobe can be adjusted should take the combination that would result in the highest pfd expected during the anticipated lifetime in each direction when generating the pfd mask. The pfd for all angles for which the satellite operates will therefore be the highest pfd that would be generated, for example, when there is a traffic hot spot in that direction. This approach is consistent with the algorithm in Part D to calculate the epfd which takes account of frequency re-use constraints within the non-GSO system.

### C2.3.2	Satellite antenna gain at the point considered at the Earth’s surface

The objective of this section is to determine the gain in the direction of a point M at the Earth’s surface when the satellite antenna points towards a cell i. The antenna coordinate can be defined by four ways of the coordinate system:

 :	spherical coordinate

v :	u  sin  cos , v  sin  sin 

B :	A   cos , B   sin 

(Az, El) :	sin (El)  sin  sin , tan (Az)  tan  cos 

The antenna gain data can be provided in a number of formats and coordinate systems. As the key input into the epfd calculation is the pfd mask, not the antenna gain pattern used to derive the pfd mask, the format and coordinate system of this antenna gain pattern could be different from those used elsewhere in this document. In particular, it is important to note that the above equations related to the conversion between (θ, ) and (Az, El) are different from those included in § D3.1.3.1 and which are used in the Worst-Case Geometry Algorithm.

As an example, the following calculations are performed in the antenna reference (A, B).

The sampling of the non-GSO antenna pattern should be adapted so that interpolation does not lead to gain level significantly different from real values.

Figure 3 presents the geometry in the antenna plane (A, B), while Fig. 4 presents the geometry between the (θ, ) and (Az, El) angles using the definitions above.

Figure 3

Antenna plane (A, B)

Figure 4

(θ, φ) and (Az, El) geometry, illustrated using a reference frame centred at the satellite

The coordinates of the point M at the Earth’s surface are (a, b) in the antenna plane (A, B), corresponding to (M, M) in the polar reference.

The coordinates of the point C centre of the cell i, are (Ac, Bc) in the antenna plane (A, B), and (c, c) in the spherical reference.

For satellite antenna gain patterns with functional descriptions (i.e. equations), the gain into the point M may be computed directly from the coordinates C(Ac, Bc) and M(a, b). For other patterns, the satellite antenna gains are provided in a grid of (A, B) points, and the point M(a, b) may be located between four points of the grid (A, B).

In general, it is therefore necessary to undertake interpolation between data points. Consider a grid of values P for a range of x values = {x1, x2, …} and y values = {y1, y2,…} as in Fig. 5.

figure 5

Interpolation between data points

The value of parameter P at point (x, y) can be derived by identifying the bounding values and hence:

λx= x - x1x2 - x1

λy= y - y1 y2 - y1

Then P can be interpolated using:

P=1-λx1-λyP11+λx1-λyP21+1-λxλyP12+λxλyP22

The sampling of the non-GSO satellite antenna pattern should be adapted so that interpolation does not lead to significant approximation.

The same criteria should be used when sampling the pfd mask.

## C2.4	Methodology

The pfd mask is defined by the maximum pfd generated by any space station in the interfering non-GSO system and as a function of the parameters defined either in option 1 or option 2. For the generation of the pfd mask, the cells in the non-GSO satellite footprint are located according to the beam pointing utilized by the non-GSO system. For satellites with steerable antennas, the satellite can point to the same area of the Earth throughout its track through the sky.

These cells are fixed relative to the Earth’s surface. For satellites that have antenna-pointing angles fixed relative to the satellite, the cell pattern is the same relative to the satellite but is moving relative to the Earth.

### C2.4.1	Option 1

Option 1 has been described for a pfd mask defined as a function of the separation angle , as an example.

The pfd mask is defined as a function of the separation angle  between this non-GSO space station and the GSO arc, as seen from any point on the surface of the Earth, and the difference L in longitude between the non-GSO sub-satellite point and the GSO satellite.

The angle  is therefore the minimum topocentric angle measured from this particular earth station between the interfering non-GSO space station and any point in the visible GSO arc.

The objective of the mask is to define the maximum possible level of the pfd radiated by the non-GSO space station as a function of the separation angle between the non-GSO space station and the GSO arc at any point on the ground, per interval of L.

At each point of the non-GSO satellite footprint, the pfd value depends on:

–	the configuration of the spot beams which are illuminated by the satellite;

–	the maximum number of co-frequency beams which can be illuminated simultaneously;

–	the maximum number of co-frequency, co-polarization beams which can be illuminated simultaneously;

–	the maximum power available at the satellite repeater.

The proposed methodology for the generation of the pfd mask is explained in the following steps:

Step 1:  At any given time, in the field of view of a non-GSO space station, Ntotal is the maximum number of cells that can be seen with the minimum service elevation angle.

Step 2:  In the field of view of the non-GSO space station, it is possible to draw iso- lines, i.e. the points on the surface of the Earth which share the same value of  (see Figs 6 and 7).

Figure 6

Field of view of a non-GSO space station (Option 1)

Step 3:  Along the iso- line, define intervals of L: difference in longitude between the non-GSO sub-satellite point and the point on the GSO arc where the  angle is minimised.

Step 4:  Per each interval of L, the iso- line can be defined by a set of n points M,k for k  1, 2,...n. To determine the maximum pfd corresponding to a given value of ,it is necessary to calculate the maximum pfd at each of the points M,k for k  1, 2,...n. The maximum pfd at a given M,k is determined by first finding the pfd contributed by each celli toward M,k taking into account the dependency of the sidelobe patterns on the beam tilt angle. The maximum pfd contributions toward M,k are then summed, with the number of contributions constrained by the physical limitations of the space station:

–	Out of the Ntotal cells that can be seen within the coverage area of the space station under a minimum elevation angle for communication, only Nco can be illuminated at the same frequency bandwidth, in one sense of polarization, and Ncross in the other sense of polarization. This characterizes the limitation of the antenna system on the non-GSO space station. To calculate the mask in one polarization, the cells which can be illuminated in the polarization concerned are identified, and the cross-polarization level is considered for other cells.

–	Out of these Nco and Ncross cells, only a given number can be powered simultaneously. This characterizes the limitation of the repeater system of the non-GSO space station.

–	If applicable, the limitations in terms of frequency reuse pattern and polarization reuse pattern also need to be clarified.

–	If applicable, the power allocated to one cell may vary taking into account the elevation angle relative to this cell, for example:

Figure 7

View in 3D of the iso- line

Step 5:  The generation of the pfd mask also needs to take into account accurately the mitigation technique implemented within the non-GSO system.

With regard to the use of a non-operating zone around the GSO arc, there are three different ways of modelling a non-GSO system based on a cell architecture:

–	cell-wide observance of a non-operating zone: a beam is switched off when one point on the Earth sees a non-GSO satellite within 0 of the GSO arc. In this particular case, any beam illuminating a cell which is crossed by an iso- line corresponding to a value   0 is switched off;

–	cell-centre observance of a non-operating zone: a beam is switched off when the centre of the cell sees a non-GSO satellite within 0 of the GSO arc. In this case, any beam illuminating a cell with its centre inside the non-operating zone bounded by the two iso-0 lines is switched off;

Step 6:  The maximum pfd value corresponding to a given value  within an interval of L is:

pfdα, ∆L=max1,2,…npfdMα,k

Step 7:  The location of an iso- line, hence the value of the maximum pfd along this line depends on the latitude of the non-GSO sub-satellite point. Therefore, a set of pfd masks will need to be provided, each corresponding to a given latitude of the sub-satellite point.

Step 8:  A set of pfd masks may be needed (one per non-GSO satellite).

### C2.4.2	Option 2

The pfd mask is defined in a grid in azimuth and elevation, per latitude of the non-GSO sub-satellite as shown in Fig. 8.

The objective of the mask is to define the maximum possible level of the pfd radiated by the non-GSO space station in this azimuth elevation grid.

At each point of the non-GSO satellite footprint, the pfd value depends on:

–	the configuration of the spot beams which are illuminated by the satellite;

–	the maximum number of co-frequency beams which can be illuminated simultaneously;

–	the maximum number of co-frequency, co-polarization beams which can be illuminated simultaneously;

–	the maximum power available at the satellite repeater.

Figure 8

Field of view of a non-GSO space station (Option 2)

The proposed methodology for the generation of the pfd mask is explained in the following steps:

Step 1:  At any given time, in the field of view of a non-GSO space station, Ntotal is the maximum number of cells that can be seen with the minimum service elevation angle.

Step 2:  For each point M(Az, El), determine the maximum pfd. The maximum pfd at a given M,k is determined by first finding the pfd contributed by each celli toward MAz, El) taking into account the dependency of the sidelobe patterns on the beam tilt angle. The maximum pfd contributions toward M,k are then summed, with the number of contributions constrained by the physical limitations of the space station:

–	Out of the Ntotal cells that can be seen within the coverage area of the space station under a minimum elevation angle for communication, only Nco cells can be illuminated at the same frequency bandwidth, in one sense of polarization, and Ncross cells in the other sense of polarization. This characterizes the limitation of the antenna system on the non-GSO space station. To calculate the mask in one polarization, the cells which can be illuminated in the polarization concerned are identified, and the cross-polarization level is considered for other cells.

–	Out of these Nco and Ncross cells, only a given number can be powered simultaneously. This characterizes the limitation of the repeater system of the non-GSO space station.

–	If applicable, the limitations in terms of frequency reuse pattern and polarization reuse pattern also need to be clarified.

–	If applicable the power allocated to one cell may vary taking into account the elevation angle relative to this cell, for example.

Step 3:  The generation of the pfd mask also needs to take into account accurately the mitigation technique implemented within the non-GSO system.

With regard to the use of a non-operating zone around the GSO arc, there are three different ways of modelling a non-GSO system based on a cell architecture:

–	cell-wide observance of a non-operating zone: a beam is switched off when one point on the Earth sees a non-GSO satellite within 0 of the GSO arc. In this particular case, any beam illuminating a cell which is crossed by an iso- line corresponding to a value   0 is switched off;

–	cell-centre observance of a non-operating zone: a beam is switched off when the centre of the cell sees a non-GSO satellite within 0 of the GSO arc. In this case, any beam illuminating a cell with its centre inside the non-operating zone bounded by the two iso-0 lines is switched off.

Step 4:  A set of pfd masks may need to be provided as a function of the latitude of the sub-satellite point.

Step 5:  A set of pfd masks may be needed (one per non-GSO satellite).

# C3	Generation of e.i.r.p. masks

## C3.1	Generation of earth station e.i.r.p. masks

### C3.1.1	General presentation

The earth station e.i.r.p. mask is defined by a set of tables of the maximum e.i.r.p. as a function of latitude and one of:

1)	The off-axis angle in the direction of the GSO arc as generated by an earth station. There can be different such e.i.r.p. tables applicable at different latitudes.

2)	The azimuth and elevation of the non-GSO satellite that the earth station is currently pointing at and difference in longitude between the earth station and reference point on the GSO arc. There can be different such e.i.r.p. tables applicable at different latitudes.

The non-GSO earth station is located in a non-GSO cell which is served by a maximum number of non-GSO space stations. The density of non-GSO earth stations which can operate co-frequency simultaneously is also used as an input to the calculation.

### C3.1.2	Mitigation techniques description

The mitigation technique implemented within the non-GSO system should be accurately explained in this section in order to be fully modelled in the calculation of the epfd↑ (see § C2.2).

### C3.1.3	Earth station antenna pattern

The earth station antenna pattern used needs to be identified to calculate the earth station e.i.r.p. mask.

### C3.1.4	Methodology

Step 1:

The earth station gain pattern could be defined as a function of the off-axis angle at the non-GSO earth station between the boresight direction towards the non-GSO space station and a point on the GSO arc, and its e.i.r.p mask could be calculated as follows:

The earth station e.i.r.p. mask at a given latitude is defined by the maximum e.i.r.p. radiated in the reference bandwidth by the earth station as a function of the off-axis angle, and is given by:

ES_ e.i.r.p.Lat, θ=GLat, θ+PLat

where:

ES_e.i.r.p.(Lat, ):	equivalent isotropic radiated power in the reference bandwidth (dB(W/BWref))

Lat:	latitude of ES for that e.i.r.p. mask

:	off-axis angle at the non-GSO earth station between the boresight line towards non-GSO space station and a point on the GSO (e.g. the GSO space station) (degrees)

G(Lat, ):	earth station directional antenna gain (dBi) at given latitude

P(Lat):	maximum power delivered to the antenna, in the reference bandwidth (dB(W/BWraf)) at given latitude

BWraf:	reference bandwidth (kHz).

Alternatively, the gain pattern could be provided in the format ES_e.i.r.p.(Lat, az, el, DeltaLongES) and the corresponding e.i.r.p mask could be calculated as follows:

The earth station e.i.r.p. mask at a given latitude is defined by the maximum e.i.r.p. radiated in the reference bandwidth by the earth station as a function of the azimuth, elevation and delta longitude angles, and is given by:

ES_ e.i.r.p.Lat, az, el, ∆LongES=GLat, az, el, ∆Long+PLat

where:

ES_e.i.r.p.(Lat, az, el, longES) :	equivalent isotropic radiated power in the reference bandwidth (dB(W/BWref))

Lat :	latitude of ES for that e.i.r.p. mask

az :	azimuth component of the antenna pointing direction (degree)

el :	elevation component of the antenna pointing direction (degree)

LongES :	difference in longitude between non-GSO earth station and point on the GSO arc (degree)

G(Lat, az, el, Long) :	earth station directional antenna gain (dBi) at given latitude

P(Lat) :	maximum power delivered to the antenna, in the reference bandwidth (dB(W/BWraf)) at given latitude

BWraf :	reference bandwidth (kHz).

Step 2:  Assuming that the non-GSO cells are uniformly distributed on the Earth’s surface, the simultaneous co-frequency transmit non-GSO earth stations are evenly distributed over the cell. Therefore the interferer can be located at the centre of the cell to perform the simulation.

This exercise would be repeated for all latitudes for which the ES_e.i.r.p. could be different.

## C3.2	Generation of space station e.i.r.p. masks

The space station e.i.r.p. mask is defined by the maximum e.i.r.p. generated by a non-GSO space station as a function of the angle seen from the non-GSO space station between the line to the sub-point of the non-GSO space station considered and a point on the GSO arc.

The space station e.i.r.p. mask is defined by the maximum e.i.r.p. radiated in the reference bandwidth by the space station as a function of this angle, and is given by:

NON-GSO_SS_e.i.r.p.θ=Gθ+P

where:

NON-GSO_SS_e.i.r.p. :	equivalent isotropic radiated power in the reference bandwidth (dB(W/BWref))

	angle seen from the non-GSO space station between the line to the sub-point of the non-GSO space station considered and a point on the GSO arc (degrees)

G():	space station antenna gain pattern (dBi) corresponding to the aggregation of all beams

P:	maximum power, in the reference bandwidth (dB(W/BWrif))

BWrif:	reference bandwidth (kHz).

# C4	pfd and e.i.r.p. mask format

## C4.1	General structure of masks

The pfd and e.i.r.p. masks are key inputs to Recommendation ITU-R S.1503 with format:

–	For epfd(down) runs, the pfd mask(s), containing tables of pfd(, long) or of pfd(azimuth, elevation) together with the latitude for which each table is valid.

–	For epfd(up) runs, the non-GSO earth station e.i.r.p. mask(s), each one contains tables of e.i.r.p. () or e.i.r.p. (.az, el, long) together with the latitude for which each table is valid.

–	For epfd(IS) runs, the non-GSO satellite e.i.r.p. masks, each one contains tables of e.i.r.p. () together with the latitude for which each table is valid.

During the simulation, the software will calculate the relevant parameters, such as latitude and off-axis angle or  angle, and then use the mask to calculate a pfd or e.i.r.p. using the following approach:

1)	The array of {Latitude, Table} is searched and the table which has the nearest latitude to the value calculated in the simulation is selected.

2)	Using the selected table, the pfd or e.i.r.p. is then calculated by interpolation using:

a)	pfd: calculated using bi-linear interpolation in either pfd(, long) or pfd(azimuth, elevation);

b)	e.i.r.p.(): calculated using linear interpolation in e.i.r.p. ();

c)	e.i.r.p.(az, el, long): using the methodology in § D5.2.7.

Each table is independent i.e. at different latitudes it can use a different grid resolution and range. The mask does not need to cover the whole range: outside the supplied values the last valid value is assumed to be used.

However, it should be noted that for latitude and {azimuth, elevation, , long} regions where no actual pfd is produced, in order to avoid using nearest latitude table containing operational pfd values it is advisable to provide extremely low pfd values for these ranges to simulate no transmission scenario.

The pfd mask table is not assumed to be symmetric in {azimuth, elevation, , long} and should be given for the full range from positive to negative extremes. In the case that the {azimuth, elevation, , long, off-axis angle} calculated in the simulation is outside the ranges given in the pfd or e.i.r.p. masks, then the last valid value should be used.

For the ES e.i.r.p. masks, there is the option to specify the position by (latitude, longitude) rather than density via a reference to a specific ES in the SRS. Note that it is not permitted to mix types: either all non-GSO ES are to be defined via specific ES or all via the density field.

Each mask has header information giving:

–	Notice ID

–	Satellite name

–	Mask ID

–	Lowest frequency mask is valid in MHz

–	Highest frequency mask is valid in MHz

–	Reference bandwidth of mask in kHz

–	Mask type

–	Parameters of mask.

The reference bandwidth of the mask is required as there are more than one bandwidth in the epfd thresholds in Article 22 of the Radio Regulations. The mask is scaled assuming the pfd or e.i.r.p. has a constant spectral power density, so that the pfd or e.i.r.p. to use in the calculations is:

pfd_Calc = pfd_Mask + 10log10[ Threshold_Bandwidth / refbw_khz]

If no refbw_khz is present in the mask than ref_bw_khz=40 is assumed.

e.i.r.p._Calc = e.i.r.p._Mask + 10log10[ Threshold_Bandwidth / refbw_khz]

The pfd and e.i.r.p. levels can vary by latitude: at least one latitude value (by default, 0 degree) should be given.

Each XML file should contain one mask (pfd or e.i.r.p.).

The masks relationships are shown in Figs 9 to 12.

Figure 9

Structure of pfd mask data for epfd(down)

Figure 10

Structure of off-axis based e.i.r.p. mask data for epfd(up)

Figure 11

Structure of azimuth elevation based e.i.r.p. mask data for epfd(up)

Figure 12

Structure of e.i.r.p. mask data for epfd(IS)

The pfd masks are to be provided to the ITU BR in XML format as:

–	It is both machine readable and human readable

–	Allows both format and type checking

–	Is an international standard for exchange of data.

The XML format is plain text with opening and closing blocks, as in:

<satellite_system>

</satellite_system>

Within each section there are then fields relevant to that block. All angles should be given in degrees.

At the top level the satellite system is identified via its notice ID and name using:

<satellite_system ntc_id="NNNNNNN" sat_name="NAME">

[Header]

[Tables]

</satellite_system>

Within this structure there is the header followed by each of the tables.

The format for each mask is described in more detail in the sections below.

## C4.2	pfd mask for epfd(down)

The header format of the pfd mask is as follows:

<pfd_mask mask_id="N" low_freq_mhz="F1" high_freq_mhz="F2" refbw_khz = “BW” type="Type" a_name="latitude" b_name="B" c_name="C">

where (see Table 5):

TABLE 5

pfd mask header format

| Field | Type or range | Units | Example |
| --- | --- | --- | --- |
| mask_id | Integer | – | 3 |
| low_freq_mhz | Double precision | MHz | 10 000 |
| high_freq_mhz | Double precision | MHz | 12 000 |
| refbw_khz | Double precision | kHz | 40 |
| type | {alpha_deltaLongitude, azimuth_elevation} | – | alpha_deltaLongitude |
| a_name | {latitude} | – | latitude |
| b_name | {alpha, azimuth} | – | alpha |
| c_name | {deltaLongitude, elevation} | – | deltaLongitude |

For each of a, b, c there are then arrays of values, such as:

<by_a a="N">

</by_a>

The values within that open/close structure then all relate to a = N: a similar structure is used for b values.

The innermost group gives the actual pfd value, such as:

<pfd c="0">–140</pfd>

An example pfd mask would therefore be:

<satellite_system ntc_id="12345678" sat_name="MySatName">

<pfd_mask mask_id="3" low_freq_mhz="10000" high_freq_mhz="40000" refbw_khz = “40” type="alpha_deltaLongitude" a_name="latitude" b_name="alpha" c_name="deltaLongitude">

<by_a a="0">

<by_b b="–180">

<pfd c="–20">–150</pfd>

<pfd c="0">–140</pfd>

<pfd c="20">–150</pfd>

</by_b>

<by_b b="–8">

<pfd c="–20">–165</pfd>

<pfd c="0">–155</pfd>

<pfd c="20">–165</pfd>

</by_b>

<by_b b="–4">

<pfd c="–20">–170</pfd>

<pfd c="0">–160</pfd>

<pfd c="20">–170</pfd>

</by_b>

<by_b b="0">

<pfd c="–20">–180</pfd>

<pfd c="0">–170</pfd>

<pfd c="20">–180</pfd>

</by_b>

<by_b b="4">

<pfd c="–20">–170</pfd>

<pfd c="0">–160</pfd>

<pfd c="20">–170</pfd>

</by_b>

<by_b b="8">

<pfd c="–20">–165</pfd>

<pfd c="0">–155</pfd>

<pfd c="20">–165</pfd>

</by_b>

<by_b b="180">

<pfd c="–20">–150</pfd>

<pfd c="0">–140</pfd>

<pfd c="20">–150</pfd>

</by_b>

</by_a>

</pfd_mask>

</satellite_system>

The XML format can be used to submit a shorthand form to avoid duplicated data but the table will be completed by interpolation or repeated use of the previous value in an array. The form of the pfd mask used by the epfd calculation algorithm in Part D is a grid at each latitude of pfd(x, y) where (x, y) are the two variables of the mask.

At each latitude there will be an array of x and y values:

xi = {x1, x2, .. xn}

yi = {y1, y2, .. yn}

These arrays can vary between latitudes.

Then for each (xi, yi) there is an associated pfd value and hence there are n × m pfd values in the table, as in the simplified 4 × 4 table below:

| (x, y) array | x1 | x2 | x3 | x4 |
| --- | --- | --- | --- | --- |
| y1 | pfd(1,1) | pfd(2,1) | pfd(3,1) | pfd(4,1) |
| y2 | pfd(1,2) | pfd(2,2) | pfd(3,2) | pfd(4,2) |
| y3 | pfd(1,3) | pfd(2,3) | pfd(3,3) | pfd(4,3) |
| y4 | pfd(1,4) | pfd(2,4) | pfd(3,4) | pfd(4,4) |

The data submitted can be a subset of this, for example:

| (x, y) array | x1 | x2 | x3 | x4 |
| --- | --- | --- | --- | --- |
| y1 | – | pfd(2,1) | pfd(3,1) | – |
| y2 | pfd(1,2) | – | – | pfd(4,2) |
| y3 | pfd(1,3) | – | – | pfd(4,3) |
| y4 | – | pfd(2,4) | pfd(3,4) | – |

In this case, the pfd table would be completed by a combination of assumptions. Firstly, if there are edge of table values not specified, to extend the mask by using the nearest defined value and assuming it continues:

pfd(1,1) = pfd(2,1)

pfd(4,1) = pfd(3,1)

pfd(1,4) = pfd(2,4)

pfd(4,4) = pfd(3,4)

Then if there are central table values not specified, to complete using linear interpolation from the surrounding values as follows:

pfd(2,2) = Interpolate{x2, pfd(1,2), pfd(4,2), x1, x4}

pfd(2,3) = Interpolate{x3, pfd(1,2), pfd(4,2), x1, x4}

pfd(3,2) = Interpolate{x2, pfd(1,3), pfd(4,3), x1, x4}

pfd(3,3) = Interpolate{x3, pfd(1,3), pfd(4,3), x1, x4}

This completion of the pfd mask could be undertaken when it is read in or on-the-fly during the calculation of the pfd for a specific geometry.

## C4.3	e.i.r.p. mask for epfd(up)

The header format of the e.i.r.p.(up) mask is as follows:

<eirp_mask_es mask_id=“N” low_freq_mhz=“F1” high_freq_mhz=“F2” refbw_khz = “BW” a_name=“latitude” b_name = “offaxis angle” ES_ID = “–1”>

where (see Table 6):

TABLE 6

Non-GSO ES e.i.r.p. mask header format

| Field | Type or range | Units | Example |
| --- | --- | --- | --- |
| mask_id | Integer | – | 1 |
| low_freq_mhz | Double precision | MHz | 10 000 |
| high_freq_mhz | Double precision | MHz | 12 000 |
| refbw_khz | Double precision | kHz | 40 |
| format | Character | – | “T” or “A” |
| a_name | {latitude} | degrees | latitude |
| b_name | {offaxis angle} | degrees | angle |
| ES_ID | Integer | – | 12345678 –1 if not specific ES |
| c_name | {azimuth angle} | degrees | 45 |
| d_name | {elevation angle} | degrees | 15 |
| e_name | {deltalongES angle} | degrees | 5 |

If the format =“T” then there are then for each relevant latitude arrays of e.i.r.p. values for given off-axis angles, such as:

<eirp b="0">30.0206</eirp>

The e.i.r.p. mask should be monotonically decreasing.

An example e.i.r.p.(up) mask would therefore be:

<satellite_system ntc_id="12345678" sat_name="MySatName">

<eirp_mask_es mask_id="1" low_freq_mhz="10000" high_freq_mhz="40000" refbw_khz = “40” format = "T" a_name = "latitude" b_name="offaxis angle", ES_ID=–1>

<by_a a="0">

<eirp b="0">30.0206</eirp>

<eirp b="1">20.0206</eirp>

<eirp b="2">12.49485</eirp>

<eirp b="3">8.092568</eirp>

<eirp b="4">4.9691</eirp>

<eirp b="5">2.54634976</eirp>

<eirp b="10">–4.9794</eirp>

<eirp b="15">–9.381681</eirp>

<eirp b="20">–12.50515</eirp>

<eirp b="30">–16.90743</eirp>

<eirp b="50">–18.9471149</eirp>

<eirp b="180">–18.9471149</eirp>

</by_a>

</eirp_mask_es>

</satellite_system>

If the format = “A” then at each latitude there would be a set of tables for every (azimuth, elevation) pointing angle covered, with each table of e.i.r.p. against the difference in longitude between the ES and the point on the GSO arc.

An example e.i.r.p.(up) mask would therefore be:

<satellite_system ntc_id="12345678" sat_name="MySatName">

<eirp_mask_es mask_id="1" low_freq_mhz="10000" high_freq_mhz="40000" refbw_khz = "40" format = "A" a_name = "latitude" c_name="azimuth angle" d_name = "elevation angle" e_name = "DeltaLongES", ES_ID=–1>

<by_a a="0">

<by_c c = “0”>

<by_d d= “0”>

<eirp e="0">30.0206</eirp>

<eirp e="1">20.0206</eirp>

<eirp e="2">12.49485</eirp>

<eirp e="3">8.092568</eirp>

<eirp e="4">4.9691</eirp>

<eirp e="5">2.54634976</eirp>

<eirp e="10">–4.9794</eirp>

<eirp e="15">–9.381681</eirp>

<eirp e="20">–12.50515</eirp>

<eirp e="30">–16.90743</eirp>

<eirp e="50">–18.9471149</eirp>

<eirp e="180">–18.9471149</eirp>

</by_d>

## C4.4	e.i.r.p. mask for epfd(IS)

The header format of the e.i.r.p.(IS) mask is as follows:

<eirp_mask_ss mask_id="N" low_freq_mhz="F1" high_freq_mhz="F2" refbw_khz = “BW” a_name= “latitude” b_name="offaxis angle">

where (see Table 7):

TABLE 7

Non-GSO satellite e.i.r.p. mask header format

| Field | Type or range |  | Units | Example |
| --- | --- | --- | --- | --- |
| mask_id | Integer |  | – | 1 |
| low_freq_mhz | Double precision |  | MHz | 10 000 |
| high_freq_mhz | Double precision |  | MHz | 12 000 |
| refbw_khz |  | Double precision | kHz | 40 |
| a_name | {latitude} |  | – | latitude |
| b_name | {angle} |  | – | angle |

There are then for each relevant latitude arrays of e.i.r.p. values for given off-axis angles, such as:

<eirp b="0">30.0206</eirp>

The e.i.r.p. mask should be monotonically decreasing.

An example e.i.r.p.(IS) mask would therefore be:

<satellite_system ntc_id="12345678" sat_name="MySatName">

<eirp_mask_ss mask_id="2" low_freq_mhz="10000" high_freq_mhz="40000" refbw_khz = “40” a_name = “latitude” b_name="offaxis angle">

<by_a a="0">

<eirp b="0">30.0206</eirp>

<eirp b="1">20.0206</eirp>

<eirp b="2">12.49485</eirp>

<eirp b="3">8.092568</eirp>

<eirp b="4">4.9691</eirp>

<eirp b="5">2.54634976</eirp>

<eirp b="10">–4.9794</eirp>

<eirp b="15">–9.381681</eirp>

<eirp b="20">–12.50515</eirp>

<eirp b="30">–16.90743</eirp>

<eirp b="50">–18.9471149</eirp>

<eirp b="180">–18.9471149</eirp>

</by_a>

</eirp_mask_ss>

</satellite_system>

PART D

Software for the examination of non-GSO filings

# D1	Introduction

## D1.1	Scope

The scope of this section is to specify part of a software requirements document (SRD) for a computer program that can be used by the BR to calculate whether a specific non-GSO system proposed by an administration meets epfd limits.

There are three key tasks that the software must complete as identified in Fig. 1:

1)	determination of the runs to execute;

2)	for each run, determination of worst case geometry;

3)	for each run, calculation of epfd statistics and checking compliance with the limits.

## D1.2	Background

This section assumes that the following approaches are used:

epfd calculation: Each non-GSO satellite has a pfd mask and the pfd for each satellite is used to calculate the aggregate epfd↓ at an earth station of a GSO system. This is repeated for a series of time steps until a distribution of epfd↓ is produced. This distribution can then be compared with the limits to give a go/no go decision.

epfd calculation: The Earth is populated with a distribution of non-GSO earth stations. Each earth station points towards a non-GSO satellite using pointing rules for that constellation, and transmits with a defined e.i.r.p.. From the e.i.r.p. mask for each earth station, the epfd↑ at the GSO can be calculated. This is repeated for a series of time steps until a distribution of epfd↑ is produced. This distribution can then be compared with the limits to give a go/no go decision.

epfdis calculation: From the e.i.r.p. mask for each space station, the epfdis at the GSO space station can be calculated. This is repeated for a series of time steps a distribution of epfdis is produced. This distribution can then be compared with the limits to give a go/no go decision.

The SRD provides detailed algorithms that would allow it to be implemented in software by any interested parties without reference to any specific development methodology.

## D1.3	Overview

This Part D is structured into the following sections:

Section D2:	Determination of runs to execute

Section D3:	Determination of worst case geometry for each run

Section D4:	Calculation of time step size and number of time steps

Section D5:	Calculation of epfd statistics and limit compliance checking

Section D5.1:	Defines the epfd↓ algorithm

Section D5.2:	Defines the epfd↑ algorithm

Section D5.3:	Defines the epfdis algorithm

Section D6:	Defines the core geometry and algorithms used by both epfd calculations including gain patterns

Section D7: 	Specifies the output formats and process to obtain a go/no-go decision.

Note that where square brackets are included as part of a parameter name, this indicates an index into an array, not tentative text.

## D1.4	General assumptions and limitations

A general limitation on the generation of epfd statistics is:

Bin size:	SB = 0.1 dB

To be consistent with the evaluation algorithm in § D7.1.3 epfd values calculation for each time step should be rounded to the lower values with a maximum precision of 0.1 dB.

The calculation of angle to GSO arc, , as described in § D6.4.4 should, where feasible, be calculated using the analytic method. If this method is unable to provide a solution, then the iterative approach may be used based upon a number of test points, with specified separation between them:

Separation between GSO test points: 1e-6 radians.

The test points should be a located at integer multiples of 1e-6 radians.

## D1.5	Database and interface

Automatic verification analysis should take input data from the SRS or other databases, combined with BR resources such as DLLs to define epfd limits and calculate antenna gains. Custom analysis can request some parameters, such as GSO satellite and ES locations, from the user.

# D2	Determination of runs to execute

## D2.1	Article 22 runs

For an Article 22 run a key task is to determine which runs to execute given a non-GSO filing and the epfd limits specified in the RR.

In any direction, if there are no masks, whether pfd or e.i.r.p., then it is not necessary to undertake any runs.

It is necessary to look in the SRS grp, freq tables to:

–	Identify the date involved

–	Identify the list of frequencies.

It is also necessary to check the frequencies of the system operating table: if there are different sets of parameters at different frequencies then a run will have to be executed for each unique set of pfd/e.i.r.p. mask, orbit elements and system operating characteristics.

For each {freq_min, freq_max, date} combination is used to call the LimitsAPI. If there are duplicate limits returned, then only the minimum frequency case need be run.

For all unique Satellite {freq_min, freq_max, date} in non-GSO notice

{

From LimitsAPI request all FSS epfd(down) limits for {freq_min, freq_max, date}

For all unique epfd(down) limits returned

{

Set FrequencyRun = max(fmin(mask), fmin(limits)) + RefBW/2

CreateRun:

Direction = Down

Service = FSS

Frequency = FrequencyRun

ES_DishSize = From Limits API

ES_GainPattern = From Limits API

epfd_Threshold = From Limits API

Ref_BW = From Limits API

}

From LimitsAPI request all BSS epfd(down) limits for {freq_min, freq_max, date}

For all unique epfd(down) limits returned

{

Set FrequencyRun = max(fmin(mask), fmin(limits)) + RefBW/2

CreateRun:

Direction = Down

Service = BSS

Frequency = FrequencyRun

ES_DishSize = From Limits API

ES_GainPattern = From Limits API

epfd_Threshold = From Limits API

Ref_BW = From Limits API

}

}

For all unique ES {freq_min, freq_max} in non-GSO notice

{

From LimitsAPI request all epfd(up) limits for {freq_min, freq_max, date}

For all unique epfd(up) limits returned

{

Set FrequencyRun = max(fmin(mask), fmin(limits)) + RefBW/2

CreateRun:

Direction = Up

Frequency = FrequencyRun

Sat_Beamwidth = From Limits API

Sat_GainPattern = From Limits API

epfd_Threshold = From Limits API

Ref_BW = From Limits API

}

}

For all unique Satellite {freq_min, freq_max, date} in non-GSO notice

{

From LimitsAPI request all epfd(is) limits for {freq_min, freq_max, date}

For all unique epfd(is) limits returned

{

Set FrequencyRun = max(fmin(mask), fmin(limits)) + RefBW/2

CreateRun:

Direction = Intersatellite

Frequency = FrequencyRun

Sat_Beamwidth = From Limits API

Sat_GainPattern = From Limits API

epfd_Threshold = From Limits API

Ref_BW = From Limits API

}

}

## D2.2	Number 9.7A of the Radio Regulations

For RR No. 9.7A runs the criteria and threshold is defined in RR Appendix 5 and runs are generated as follows:

If the selected earth station meets the criteria in Appendix 5

{

Get the frequency range of the selected ES(fmin, fmax)

Get all non-GSO networks in the SRS that overlap that frequency range

For each non-GSO network returned

{

Query Limits API with the selected ES(fmin, fmax)

{

Get RefBW from Appendix 5 Data

Set FrequencyRun = max(ES_fmin, Mask_fmin) + RefBW/2

CreateRun:

Direction = Down

Frequency = FrequencyRun

ES_DishSize = From ES filing

ES_GainPattern = From ES filing

epfd_Threshold = From Appendix 5

Ref_BW = From Appendix 5

}

}

}

## D2.3	Number 9.7B of the Radio Regulations

For RR No. 9.7B runs the criteria and threshold is defined in RR Appendix 5 and runs are generated as follows:

Get (fmin, fmax) from non-GSO notice

{

Get all ES in the SRS that overlap that frequency range

For each ES returned

{

If the earth station meets the criteria in Appendix 5

{

Query Limits API with ES(fmin, fmax)

Get RefBW from Appendix 5 Data

Set FrequencyRun = max(ES_fmin, Mask_fmin) + RefBW/2

CreateRun:

Direction = Down

Frequency = FrequencyRun

ES_DishSize = From ES filing

ES_GainPattern = From ES filing

epfd_Threshold = From Appendix 5

Ref_BW = From Appendix 5

}

}

}

}

# D3	Worst-case geometry

The epfd limits in RR Article 22 are applicable for all GSO ESs and all pointing angles towards that part of the GSO arc visible from that ES. It is, however, not feasible to model all such geometries within the verification software. The worst-case geometry (WCG) is a reference GSO satellite location and either an ES or boresight of the GSO satellite’s beam which is used when examining a non-GSO system for compliance with the epfd limits in RR Article 22. It remains necessary for the non-GSO operator to meet the epfd limits in RR Article 22 for all other geometries including the testing of specific GSO networks as noted in § A1.3.

The WCG is selected by an algorithm, the worst-case geometry algorithm (WCGA), which undertakes an examination of the pfd/e.i.r.p. masks together with the non-GSO satellite orbital parameters to identify the highest single entry epfd value. Where there are multiple geometries with the same highest single entry epfd value, then the geometry is selected that should have this highest single entry epfd for the greatest percentage of time, identified by considering the angular velocity or elevation angle. These assumptions are based on the critical epfd levels being the highest ones which are those that are most readily measurable.

The WCGA is based upon iterating over a set of positions, typically of the non-GSO satellite. The geometry is assumed to be symmetric in longitude and the satellite can be set at the required latitude exactly using a simple point-mass model. However the epfd calculation engine described in § D5 can use a range of orbit models with a specific time step and therefore the longitude at which a satellite reaches a specified latitude will be different from that in the WCGA. Hence an additional stage is required which for the non-GSO satellite that results in the highest single entry epfd the difference in longitude is calculated between:

–	The longitude of the non-GSO satellite when it reaches the specified latitude using a point mass model in the WCGA, converting to (latitude, longitude) using a static time t = simulation start time (e.g. relative time = 0).

–	The longitude at which the non-GSO satellite is closest to the specified latitude using the full orbit model and the fine time step calculated for the given run, converting to (latitude, longitude) using the relevant simulation time.

This difference in longitude is shown in Fig. 13.

Figure 13

Adjusting longitude of WCG to take account of orbit model and simulation time

This difference in longitude is then used to adjust the position of the GSO satellite and ES calculated in the WCGA so that the non-GSO satellite with the pfd mask that causes the highest epfd goes through the geometry that causes this epfd value during its first orbit. When iterating for latitude using the time step, the time step that gives the nearest latitude is used in the longitude calculation.

Note that the algorithm in this Recommendation is not designed to take account of either ITU-R Regions or specific longitudes as the epfd limits in RR Article 22 are meant to be applicable for all GSO ES locations and visible parts of the GSO arc.

Care is required when comparing floating point numbers to check for rounding errors. In the WCGA it is acceptable to round to the nearest 0.1 dB rather than round up. The search grid is in steps of 0.1° and binary search routines terminate when the difference in angles is less than 1e-5 radians.

## D3.1	WCG epfd↓

### D3.1.1	Inputs

The inputs to the algorithm include:

pfd_mask:	the pfd mask to check

0[lat] :	the GSO arc avoidance angle of the non-GSO satellite by latitude = MIN_EXCLUDE[Latitude]

h :	the minimum operating height of the non-GSO satellite

0[lat, az] :	the minimum elevation angle of the non-GSO satellite by latitude and azimuth

{a, e, i}:	the orbit parameters of the non-GSO system

ES:	the parameters of the ES including gain pattern.

### D3.1.2	Algorithm

This section describes the algorithm to determine the WCG for the epfd(down) direction.

Note that there could be different ranges of frequency in the pfd masks: this process is assumed to be repeated for each valid frequency range. For each valid frequency ranges there could be different pfd masks, multiple sets of {a, e, i} or system operating parameters (e.g. GSO avoidance angles that vary by non-GSO satellite): the process should be repeated over each such set.

The WCG is based upon search in (, φ) as seen by the non-GSO satellite, with particular care taken for the region (–0, +0) including  = 0. This search is repeated at a number of test non-GSO satellite latitudes. In addition, specific checks are made for the highest latitudes for which  = {−0, 0, +0} to ensure compatibility with the methodology in Recommendation ITU-R S.1714.

For each test point considered, the algorithm calculates the epfd using the pfd mask and receive antenna gain, and compares that to the threshold for the relevant latitude. The gain is calculated using the  angle for the off-axis angle : for the BSS ES antenna gain pattern, which might not be symmetric around the boresight, the  should be the value calculated assuming the ES points at the location at the point correspond to the  angle. Note that the algorithm can be implemented in a way that calculates the WCG for multiple dish sizes using vectorisation.

It is likely that multiple test points will result in the same difference between epfd level and threshold. To assess which should be used as the WCG, the angular velocity of the non-GSO satellite as seen by the ES is calculated, and the geometry selected is that:

1.	Gives the highest difference between epfd level and threshold to the resolution of the resulting statistics (0.1 dB).

2.	If multiple geometries meet point 1, select the one that would result in geometry that gives the lowest angular velocity of the satellite as seen by the ES.

The search algorithm is shown in Figs 14 and 15, where:

Δα=α-α0lat

Δε=ε-ε0lat, az

Figure 14

Search (, φ) grid for WCGD_CalcAtLat

Figure 15

Geometry for WCGD_CheckExtremeCase

The algorithm is described in the following pseudo code for specified GSO ES type:

WCGA_Down:

Set WorstEPFDBin = –9999

Set WorstAngularVelocity = +9999

Identify 0,min = minimum value over all values in the 0[lat, az] table

For all satellites in the order listed in ITU DB

{

Determine PFD mask to use for this satellite

If this PFD mask and satellite orbit (a, e, i) combination has not been checked so far then or this satellite uses a different 0[lat] then

Call GetWCGA_Down(SystemParams)

End if

}

Next satellite

Note that SystemParams = (PFD_Mask, 0[lat], 0[lat, az], ES, OrbitParams, 0,min) as required

GetWCGA_Down (SystemParams):

StepSize = 0.1°

If (i = 0)

{

WCGD_CalcAtLat(SystemParams, latitude = 0)

}

Else

{

LatNumSteps = RoundUp(i / StepSize)

For n = 0 to LatNumSteps inclusive

{

latitude = i * n / LatNumSteps

WCGD_CalcAtLat(SystemParams, latitude)

If (n > 0)

{

WCGD_CalcAtLat(SystemParams, -latitude)

}

}

WCGD_CheckExtremeCase(SystemParams, 0,  = +/2}

WCGD_CheckExtremeCase(SystemParams, 0,  = -/2}

WCGD_CheckExtremeCase(SystemParams, +1,  = +/2}

WCGD_CheckExtremeCase(SystemParams, -1,  = –/2}

WCGD_CheckExtremeCase(SystemParams, +1,  = +/2}

WCGD_CheckExtremeCase(SystemParams, -1,  = -/2}

}

}

WCGD_CalcAtLat(SystemParams, latitude):

Locate non-GSO satellite at latitude

Calculate height of non-GSO satellite from its radius, r

If height of non-GSO satellite < minimum operating height then return	Calculate φ0 for elevation angle 0, min and radius r

WCGD_CheckCase(SystemParams, latitude,  = 0, φ = 0)

NumPhiSteps = RoundUp(φ0 / StepSize)

PhiStepSize = φ0 / NumPhiSteps

For φ = PhiStepSize to φ0 inclusive in NumPhiSteps steps

{

ThetaMin = /2

ThetaMax = +3/2

If the PFD mask is symmetric in DeltaLong or Azimuth and elevation table is also symmetric between east and west

{

ThetaMax = /2

}

NumThetaSteps = max(16, RoundUp(2φ/PhiStepSize))

ThetaStepSize = (ThetaMax-ThetaMin)/NumThetaSteps

For ThetaStep = 0 to NumThetaSteps inclusive

{

= ThetaMin + ThetaStep*ThetaStepSize

WCGD_CheckCase(SystemParams, latitude, , φ)

}

WCGD_CheckAlphaPhiCase(SystemParams, latitude, φ, 0, RHS)

WCGD_CheckAlphaPhiCase(SystemParams, latitude, φ, +1, RHS)

WCGD_CheckAlphaPhiCase(SystemParams, latitude, φ, -1, RHS)

If the PFD masks is not symmetric then

{

WCGD_CheckAlphaPhiCase(SystemParams, latitude, φ, 0, LHS)

WCGD_CheckAlphaPhiCase(SystemParams, latitude, φ, +1, LHS)

WCGD_CheckAlphaPhiCase(SystemParams, latitude, φ, -1, LHS)

}

}

WCGD_CheckAlphaElevCase(SystemParams, latitude, 0, RHS)

WCGD_CheckAlphaElevCase(SystemParams, latitude, +1, RHS)

WCGD_CheckAlphaElevCase(SystemParams, latitude, -1, RHS)

If the PFD mask is not symmetric then

{

WCGD_CheckAlphaElevCase(SystemParams, latitude, 0, LHS)

WCGD_CheckAlphaElevCase(SystemParams, latitude, +1, LHS)

WCGD_CheckAlphaElevCase(SystemParams, latitude, -1, LHS)

}

WCGD_CheckCase(SystemParams, latitude, , φ):

Convert (,φ) to (az, el) in the satellite reference frame

Create line from non-GSO satellite N in direction (az, el)

Identify point P in which line intersects Earth

Calculate the latitude of P, latP

If Absolute(latP) > 81.2 degrees then exit this function

If latP < ES_LAT_MIN then exit this function

If latP > ES_LAT_MAX then exit this function

If number of non-GSO satellites that operate at this latitude is zero then exit this function

Calculate the (aznGSO, elnGSO) of the non-GSO satellite as seen by the ES

Find the nearest latitude to latP in the 0[latP, AznGSO] table

At point P calculate (, long) angles wrt point N

At point P calculate AngularVelocity using methodology below

Calculate PFD from mask, latitude & (az, el, , long)

Calculate G() and G(0[latP])

Calculate the elGSO of the point on the GSO arc associated with the calculated

GSO = the appropriate minimum elevation angle in Table 8

If ((  0[latP] and elnGSO  0[latP, AznGSO] and elGSO >= εGSO)

or G() > min(Gmax -30 dB, G(0[latP])) ) then

{

Calculate EPFDThreshold from latitude of point P on the Earth’s surface

Calculate EPFDMargin = PFD + Grel() - EPFDThreshold

Calculate EPFDbin = EPFDMargin/BinSize

If WorstEPFDBin < EPFDBin

{

WorstEPFDBin = EPFDBin

Worst AngularVelocity = AngularVelocity

Store this (N, P)

}

Else if (WorstEPFDBin = EPFDBin &&

WorstAngularVelocity > AngularVelocity)

{

WorstAngularVelocity = AngularVelocity

Store this (N, P)

}

}

WCGD_CheckAlphaPhiCase(SystemParams, Latitude, φ, Sign, Side):

Set  range according to side to check (left or right)

If bracket  = 0 then

{

Use binary search to iterate on  until WCGD_GetDeltaAlpha(SystemParams, Sign, , φ) = 0

WCGD_CheckCase(SystemParams, latitude, , φ)

}

WCGD_CheckAlphaElevCase(SystemParams, Latitude, Sign, Side):

Set  range according to side to check (left or right)

If bracket  = 0 then

{

Use binary search to iterate on  until within 1e-5 radians

{

For each i

{

Call WCGD_CalcPhiFromThetaElev(SystemParams, i) to determine φi					Call WCGD_GetDeltaAlpha(SystemParams, Sign, i φi) to determine i

}

} selecting  that brackets  = 0

WCGD_CheckCase(SystemParams, latitude, , φ)

}

WCGD_CalcPhiFromThetaElev(SystemParams, ThetaTest, PhiMax):

φ0 = 0

φ1 = PhiMax

Use binary search to iterate on φ until within 1e-5 radians

{

For each φi

{

Call WCGD_CalcDeltaElev(SystemParams, ThetaTest, φi)

}

} selecting φ that brackets  = 0

Return φ

WCGD_GetDeltaAlpha(SystemParams, Sign, , φ):

Convert (,φ) to (az, el)

Create line in direction (az, el) from non-GSO satellite

Identify point P where line intersects Earth

Calculate latitude of P, latP

Determine exclusion zone size at this point, 0[latP]

At point P calculate

deltaA = Sign*0[latP]

Return  - deltaA

WCGD_GetDeltaElev(SystemParams, ,φ):

Convert (,φ) to (az, el)

Create line in direction (az, el) from non-GSO satellite

Identify point P where line intersects Earth

Calculate latitude of P, latP

Calculate (aznGSO, elnGSO) of non-GSO satellite as seen by point P

At point P calculate 0[lat, az]

Return elnGSO - 0[lat, az]

WCGD_CheckExtremeCase(SystemParams, Sign, ):

Set latitude range according to sign (north or south hemisphere)

If bracket  = 0 then

{

Use binary search to iterate until latitude range less than 1e-5 radians

{

For each test latitude, Lat

{

Call WCGD_CalcDeltaAlphaFromLatElev(SystemParams, Lat, Sign, i) to determine i and corresponding φi

}

} Selecting latitudes that bracket  = 0

WCGD_CheckCase(SystemParams, latitude, , φ)

}

WCGD_CalcDeltaAlphaFromLatElev(SystemParams, Latitude, Sign, ):

Set satellite at Latitude

Get φ using WCGD_CalcPhiFromThetaElev SystemParams, , PhiMax)

Calculate  from WCGD_CalcDeltaAlpha(SystemParams, Sign, , φ)

Return , φ

The algorithm uses the geometry given in the sections below.

### D3.1.3	Geometry

#### D3.1.3.1	Conversion between (az, el) and (, φ)

The following equations can be used:

cosφ=cosazcosel

sinel=sinθsinφ

Note that it is necessary to check the sign of the az or φ to ensure that the arccos / arcsin is calculated correctly.

It should also be noted that the equations above should be used in the application of the worst-case geometry algorithm and that they are different from the equations converting (θ, ) into (Az, El) included in § C2.3.2.

The (, φ) coordinate system and its relationship to the (az, el) coordinate system is shown in Figs 16 and 17 below.

Figure 16

Conversion between (Azimuth, Elevation) and (, φ) from satellite viewpoint

Figure 17

Conversion between (Azimuth, Elevation) and (, φ) in 3D

The φ angle is between the two lines:

–	from the satellite to the sub-satellite point

–	from the satellite to the test direction.

The  angle is around the boresight in an anti-clockwise direction from the elevation = 0 plane.

#### D3.1.3.2	Setting satellite at latitude

Key steps in this algorithm are the calculation of the position and velocity vectors of the non-GSO satellite and ES. For circular orbit systems the latitude can be used to derive the true anomaly, , using:

sinω+ = sinlatsini

For elliptical systems, it is necessary to use binary search. Assuming the system is designed to have with argument of perigee = ±/2, then the satellite will go from minimum to maximum latitude when the mean anomaly varies from [0, ]. Therefore, the binary search can start with M = (0, ) and iterate from there.

To derive the position and velocity vectors, the following equations could be used:

In the satellite plane:

rsat= rsat(cosP+sin Q)

sat = μp(–sin P+(e+cos)Q)

where:

P, Q:	unit vectors in the orbit plane with origin the centre of the Earth and P aligned with the orbit’s major axis as described in § D6.3.3

a, e, : 	orbital elements.

Also:

rsat= p1+e cos

p=a1-e2

The non-GSO’s position and velocity vectors can then be converted from the PQW orbit plane based frame to Earth centred vectors using the standard rotation matrix via the (, , i) orbital elements. For the WCG calculation it can be assumed that second order effects including the J2 factor need not be considered.

The position vector equation can also be used to calculate a latitude from a true anomaly, , and hence by iteration locate the satellite at the required latitude.

#### D3.1.3.3	Calculation of maximum φ in satellite viewpoint

For a given latitude and hence satellite radius, the maximum angle at the satellite from sub-satellite point φ0 can be derived from the elevation angle  using:

sinφ0= Rersatsinπ2+ε

#### D3.1.3.4	Calculation of angular velocity

The inputs are as follows:

ES position vector:	res

ES velocity vector:	es

Non-GSO satellite position vector:	rsat

Non-GSO satellite velocity vector:	sat

From these, it is possible to calculate the apparent velocity and vector from the ES to the satellite:

r = rsat – res

= sat – es

The angle between these two vectors can then be calculated:

cos ψ= r∙ r

The instantaneous angular velocity is then:

θ= rsin ψ

The various terms are shown in Fig. 18.

Figure 18

Vectors to calculate non-GSO satellite apparent angular velocity

Note that a low angular velocity will result in higher likelihoods of interference and hence for a given epfd value the WCG that gives the least apparent angular velocity would be the one to use.

The ES’s velocity vector can be derived from its position vector (x, y, z) as follows:

es =we –yx0

where we is the Earth’s angular velocity in radians per second.

## D3.2	WCG epfd↑

Note that as the epfd limits in Article 22 are for 100% of the time there is no need to consider the likelihood of particular geometries, only the maximum epfd(up) value.

If there are multiple sub-constellations with alternative orbital elements or some satellites use different exclusion zone angles, then the process should be repeated for each unique set.

### D3.2.1	Inputs

The inputs to the algorithm are as follows:

ES_e.i.r.p.:	the non-GSO ES e.i.r.p. mask to check. This could vary by latitude, with each latitude range having an e.i.r.p. pattern, defined as a table of e.i.r.p. vs off-axis angle towards the GSO arc or by latitude, azimuth and elevation, with each latitude/azimuth/elevation having an e.i.r.p pattern defined as a table of e.i.r.p. against difference in longitude between the ES and GSO satellite

θadB :	the GSO satellite’s beam half power beamwidth

0,GSO :	the minimum elevation angle of the GSO system

0[lat, az] :	the minimum elevation angle of the non-GSO system, which could vary by latitude and azimuth

0[lat] :	the minimum exclusion angle of the non-GSO system, which could vary by latitude and non-GSO satellite

Nco[lat] :	the number of non-GSO satellites that can provide co-frequency service at a specific location on the Earth’s surface, which could vary by latitude

ES_DISTANCE, ES_DENSITY :	the average distance on the Earth’s surface between co-frequency beams from the non-GSO system (km) and density of co-frequency non-GSO ES or:

ES(lat, long) :	the latitude and longitude of the specific ES of the non-GSO system

{a,i,e, ,,} :	the orbit parameters of the non-GSO system including whether the system’s ground track repeats and if so the repeat time.

### D3.2.2	GSO system parameters

It is assumed that the GSO system’s minimum operating elevation angle and beamwidth are as in Table 8.

TABLE 8

WCG(up) GSO system parameters

| Frequency band | f < 10 GHz | 10 GHz ≤ f < 17 GHz | f 17 GHz |
| --- | --- | --- | --- |
| Beamwidth (degrees) | 1.5 | 4 | 1.55 |
| Minimum elevation angle (degrees) EOC | 10 | 10 | 20 |

Table 9 shows the geocentric angle that corresponds to the minimum elevation angle and hence the maximum angle at the GSO satellite towards the beam boresight φBS:

TABLE 9

WCG(up) derived minimum and maximum values

| Frequency band | f < 10 GHz | 10 GHz ≤ f < 17 GHz | f 17 GHz |
| --- | --- | --- | --- |
| φEOC at GSO satellite (degrees) | 8.567 | 8.567 | 8.172 |
| φBS at GSO satellite (degrees) | 7.817 | 6.567 | 7.397 |
| Geocentric angle for φBS (degrees) | 56.230 | 42.552 | 50.934 |

These were generated using the geometry in Fig. 19 and the following equations.

FIGURE 19

GSO pointing angles for WCGA(UP) geometry

Here the φBS can be derived as follows:

sinφEOC= ReRgeosinπ2+ ε

φBS= φEOC – θ3dB2

Then, noting that as  > /2:

sin(π – ψ)= RgeoRe sin⁡(φBS)

Hence:

χBS=π-φBS-ψ

### D3.2.3	Algorithm

The WCGA for the epfd(up) case is as follows:

WCGA_UP:

Calculate φBS from EOC

From φBS calculate BS

If ES from density

Call WCGA_UP_General

Else

If non-GSO satellite repeats

Call WCGA_UP_SpecifcES_Repeating

Else

Call WCGA_UP_SpecifcES_NonRepeating

Endif

Endif

The various cases, their functions and geometries are described in the following sections.

#### D3.2.3.1	Aggregate epfd calculation

In the general case the aggregate epfd(up) can be calculated using:

epfdup=i=1i=NESe.i.r.p.lat-LS+Grel,rx+10log10NUMES+10log10Nco,ESlat

Note the summation is in absolute though the equation is given in dB terms with addition and where:

e.i.r.p. (lat) :	e.i.r.p. towards the GSO for the given non-GSO ES latitude either as a function of latitude and off-axis angle towards the GSO arc or function of latitude, boresight (azimuth, elevation) and difference in longitude between the non-GSO ES and GSO satellite

LS :	spreading factor

Grel,rx :	relative gain at the GSO satellite using the Rec. ITU-R S.672 gain pattern

NUM_ES :	is a factor for systems using density rather than specific ES (and which typically relates to the access method) derived from the non-GSO system’s density and distance fields as given in § D5.2.5

NUM_ES = ES_DISTANCE * ES_DISTANCE * ES_DENSITY

Nco,ES(lat) :	maximum number of co-frequency non-GSO ES that can transmit at a given location.

The summation is over the NES non-GSO deployed within the GSO satellites 15 dB beamwidth footprint using the algorithm in § D5.2.5. This depends upon the separation distance between co-frequency beams which is a parameter provided by the non-GSO system.

For the WCG(up) calculation, it is assumed that the aggregate epfd is dominated by the single entry epfd at the boresight plus an aggregation factor and so this can be estimated by extracting the non-GSO parameters from the summation as in:

epfdup≅e.i.r.p.lat+10log10NUMES++10log10Nco,ESlat+i=1i=NESGrel,rx-LS

This last term is only dependent upon the geometry (in particular the geocentric angle, ) and the gain pattern in Recommendation ITU-R S.672 but not upon any of the non-GSO parameters and hence can be pre-computed.

DES1002F672x=i=1i=NESGrel,rx-LS

A method to calculate based upon an assumed non-GSO ES separation distance ES_DISTANCE = DES = 100 km is via the following Pade approximation:

y=a+bx+cx2+dx3+ex4+fx51+gx+hx2

where:

TABLE 10

F672 Pade approximation parameters

| Parameter | Beamwidth = 4 | Beamwidth = 1.55 x < 35° | Beamwidth = 1.55 x > 35° | Beamwidth = 1.5 |
| --- | --- | --- | --- | --- |
| a | −133.536851 | −133.323814 | −133.323814 | −142.1952459 |
| b | 0.001384021 | 0.017909858 | 0.02314611 | −0.001235207 |
| c | 0.000637798 | −0.011981864 | −0.001336397 | 0.00121213 |
| d | 6.9531E-07 | 0.002350044 | 2.26511E-05 | −4.77102E-05 |
| e | −1.94494E-07 | −4.61428E-05 | −6.95017E-08 | 6.5926E-07 |
| f | 1.41944E-09 | −3.30E-07 | −7.75011E-10 | −2.83069E-09 |
| g | −0.033027982 | −0.408584467 | −0.036720978 | −0.033787173 |
| h | 0.000434998 | 0.054553642 | 0.000370144 | 0.000306156 |

For the specific ES case the DES is not defined and so a value of 100 should be used while NUM_ES set to 1 so this term does not contribute.

Given the F672 factor, the epfd(up) for a given geometry can be estimated using:

epdfup≅e.i.r.p.lat+10log10NUMES++10log10Nco,ESlat+DES1002F672(χ)

#### D3.2.3.2	Worst pointing

The epfd calculation above relies on the ability to calculate the maximum e.i.r.p. towards the GSO. As well as the e.i.r.p. mask, this will depend upon a number of additional factors including:

–	Latitude of the non-GSO ES

–	Minimum elevation angle(s) at that latitude 0[az, lat]

–	Exclusion zone size 0[lat]

–	Constellation (or sub-constellation) orbit parameters.

The influence of some of the factors can be seen in the examples below assuming the non-GSO ES is in the northern hemisphere so that the centre of the Figure is pointing south.

Equatorial orbit, low latitude

Figure 20

Equatorial orbit low latitude

Here the non-GSO satellite never intersects the GSO orbit or the exclusion zone and so the min is the angle at the non-GSO ES between the equatorial orbit and GSO arc in the azimuth of consideration.

Note that a purely equatorial orbit would be repeating so would be handled by a separate case within the WCG. However it is included to show the edge case and gain an understanding of the geometry involved.

Equatorial orbit, high latitude

Figure 21

Equatorial orbit high latitude

Here again the non-GSO satellite orbit never intersects the GSO orbit or exclusion zone but for the azimuth of the GSO satellite the non-GSO satellite would not be active as it is below the minimum elevation angle. Hence the min is between the nearest azimuth for which the non-GSO satellite would be above the horizon. However this point would be considered as the applicable point for another position on the GSO satellite where the minimum off-axis angle would be smaller and hence e.i.r.p. larger. It is therefore suggested that if the non-GSO satellite is below the minimum elevation angle for that azimuth it need not be considered.

Polar orbit, low latitude

Figure 22

Polar orbit low latitude

In this case the polar orbiting satellite could be located at any pointing angle (az, el) as seen by the non-GSO ES. The limiting case is then the edge of the exclusion zone, so that min = 0[lat].

Polar orbit, high latitude

Figure 23

Polar orbit high latitude

In this case the GSO arc is always below the minimum elevation angle and hence the minimum off-axis angle min is the difference between the minimum elevation angle and elevation angle of the GSO arc in the azimuth / latitude of interest.

Note this also the case for the previous scenario (polar orbit, low latitude) for the extreme edge case where the GSO arc has a low elevation angle as seen by the non-GSO ES.

Low inclination, no in-line geometry

Figure 24

Low inclination, no in-line geometry

This example is similar to the equatorial case in that there is no in-line geometry, and hence the min angle to use is the angular separation between the edge of the visible zone and GSO arc or the exclusion zone angle 0[lat], whichever is larger.

Low inclination with in-line geometry

Figure 25

Low inclination with in-line geometry

This is an extension of the previous case with the inclination increased until the visible zone spreads either side of the GSO arc. The minimum off-axis angle is then the exclusion zone size so that min = 0[lat].

General case

The geometry of the general case is shown in Fig. 26.

Figure 26

General case with orbit shell

The general method iterates over the field of view of the GSO satellite and determines the associated ES location.

For non-GSO systems with non-repeating orbits, the result will be the same for all longitudes of the GSO system, and hence a GSO longitude = 0° can be used.

For non-GSO systems with repeating orbit, an initial run should be undertaken and the average longitude of each of the non-GSO satellites calculated over the repeat period together with the likelihood of the non-GSO satellite being within 1 degree of that average longitude. The GSO satellite should be located at the average longitude of the non-GSO satellite with the highest likelihood of being within 1 degree of that satellite’s average longitude.

Then given:

–	GSO satellite longitude

–	Non-GSO ES φ (latitude, longitude)

–	Maximum or minimum latitude of the non-GSO satellite

–	Radius of the non-GSO satellite when at the maximum/minimum latitude.

It is then possible to iterate on the longitude of the non-GSO satellite which minimises the angle to the GSO satellite as seen at the ES. This is the converse of the  angle and hence is called the  angle. This can be derived using iteration or the analytic method as described in § D6.4.4.4. There will be two positions, the + associated with the maximum latitude and the - associated with the minimum latitude of the non-GSO system.

It is also possible to determine points that are in the direction of the + and - points with angle 0 from the GSO satellite as in Figs 27, 28 and 29. This results in points {a, b, c, d} that can then be checked to see if they are valid, in particular that:

–	They are at least 0 away from the GSO satellite (arc)

–	They are within the [+, -] range of the non-GSO satellite as seen by the ES

–	They are above the minimum non-GSO elevation angle for the non-GSO ES latitude and azimuth of the test point.

If they are valid then they can be considered as possible options for the minimum off-axis angle towards the GSO satellite at the non-GSO ES when pointing at a non-GSO satellite.

Figure 27

WCG(up) general case when [+, -] bracket the GSO arc

Figure 28

WCG(up) general case when [+, -] do not bracket the GSO arc or intersect minimum

Figure 29

WCG(up) general case when [+, -] do not bracket the GSO arc but does intersect minimum

There is also a fifth test point {e} to handle the case when the GSO arc is below the minimum elevation angle as shown in Fig. 30.

Figure 30

WCG(up) general case when [+, -] bracket the GSO arc but it is below the minimum elevation angle

#### D3.2.3.3 	WCG_Up_General

The general case iterates over the field of view of the victim GSO satellite as shown in Fig. 31.

Figure 31

Search of GSO satellite field of view

The search is undertaken in (, φ) as described below:

WCGA_UP_General:

CheckCaseUpGeneral(0, 0)

NumberOfPhiSteps = Integer(Degrees(φ0)/ 0.1)

For PhiStep = 1 to NumberOfPhiSteps inclusive

φ = φ0 * PhiStep / NumberOfPhiSteps

ThetaStepSizeDegrees = 0.1 * φ0 / φ

NumberOfThetaSteps = max(16, Integer(360 /  ThetaStepSizeDegrees))

For ThetaStep = 0 to NumberOfThetaSteps-1 inclusive

= radians(ThetaStepSizeDegrees * ThetaStep)

CheckCaseUpGeneral(, φ)

Next ThetaStep

Next PhiStep

CheckCaseUpGeneral(, φ):

Convert (, φ) to (az, el) at a GSO satellite set at longitude = 0

Use (az, el) to create line from the GSO satellite

Calculate the intersection point of that line and the spherical Earth

Calculate the (lat, long) of the non-GSO ES at the intersection point

Check that the latitude is in the range of the non-GSO system i.e between ES_LAT_MIN and ES_LAT_MAX

Check that the Nco(lat) > 0

If the latitude is ok then

If the non-GSO system uses a repeating orbit then

{

Call WCGA_UP_SpecifcES_Repeating

Return

}

For this (lat, long) calculate the geocentric angle

For the GSO system’s beamwidth, calculate the F672()

If the e.i.r.p. mask is defined by offaxis angle then

{

Call CalcMinOffaxisAngle to calculate  for this location

If find a minimum offaxis angle then calculate the epfd(up) using:

epfdup=e.i.r.p.lat,+10log10NUM_ES+10log10Ncolat+DES1002F672(χ)

If this is the highest epfd(up) then store this value and (, φ)

}

Else the ES e.i.r.p. mask is defined by (az, el, DeltaLongES) then

{

Call CalcES_e.i.r.p.(Non-GSO ES lat, long) to calculate e.i.r.p.(lat) for this location

Calculate the epfd(up) using:

epfdup=e.i.r.p.lat+10log10NUM_ES+10log10Ncolat+DES1002F672(χ)

If this is the highest epfd(up) then store this value and (, φ)

Endif

Endif

Note that the method to calculate the maximum latitude of the non-GSO system is given in § D3.2.3.6.

CalcES_e.i.r.p.(non-GSO ES lat, long)

Calculate the DeltaLongES

Create a vector rES of the non-GSO ES

Set Maxe.i.r.p. = -9999

Identify the ES_e.i.r.p. table for this non-GSO latitude

Set StepSizeDeg = 0.5

Set NumAz = 1 + integer(360 degrees / StepSizeDeg)

Set NumEl = 1 + integer(90 degrees / StepSizeDeg)

For all az_i = 0 to NumAz not inclusive

Az = -180 + az_i * 360 / NumAz

For all el_i = 0 to NumEl inclusive

El = 0 + el_i * 90 / NumEl

Create a vector razel in direction (az, el)

Select the appropriate es e.i.r.p. (az, el) array

If the (az, el) direction is outside the exclusion zone and above the minimum elevation for this azimuth and there could be a satellite in this direction then

{

Calculate the ES_e.i.r.p.az,el towards the GSO for this (az, el, DeltaLong)

If ES_e.i.r.p.az,el is greater than ES_e.i.r.p. then use this value instead

}

Next el_i

Next az_i

To determine if there could be a non-GSO satellite in a given (az, el) direction from a non-GSO ES at position rES, the following algorithm can be used, assuming that the vectors rES and razel are in Earth centred coordinates (x, y, z):

Inputs:

Non-GSO ES position vector = rES

Vector in direction (az, el) = razel

non-GSO satellite orbit inclination angle = i

non-GSO satellite semi-major axis = a

non-GSO satellite eccentricity = e

If the non-GSO satellite eccentricity = 0 (i.e. circular orbit):

There could be a satellite in direction (az, el) if

−i  Latitude(a)  +i

Otherwise, if the non-GSO satellite eccentricity > 0 (i.e. elliptical orbit):

There could be a satellite in direction (az, el) if the following are both true:

−i  Latitude(a(1+e))  +i

−i  Latitude(a(1-e))  +i

Where the function CalcLatitude(d) is:

Create a line from the non-GSO ES in direction (az, el) using parameter :

r=rES+λrazel

The value of  for which the line is at distance d from the centre of the Earth can be calculated by solving the quadratic equation:

razel2λ2+2rES.razelλ+rES2-d2=0

The root for which  > 0 should be used to calculate the position vector r.

From this the latitude can be calculated using:

Latitude(d) = asin(r.z / r)

CalcMinOffaxisAngle(non-GSO ES lat, long)

Calculate the (azGSO, GSO) of the GSO satellite as seen by the non-GSO ES

Determine the radius Rn,+ of the non-GSO satellite when its lat = +i

Determine the radius Rn,- of the non-GSO satellite when its lat = -i

If Rn,+ or Rn,- are below the minimum operating height then determine the latitude of the non-GSO satellite when at this height and use this instead

For each of {Rn,+, Rn,+} and {lat+, lat-}, calculated the {+, -} angles and associated non-GSO satellite positions identified as (a, b) in Figs. 27-29

For each of the {+, -} positions, use spherical geometry to derive points (c, d) which have angle at the ES of 0 along the lines to point (a, b) respectively

For each of (a, b, c, d) points, calculate the (azimuth, elevation) as seen by the ES

Set the MinimumAngle to be +9999

For each of points (a, b, c, d):

If point is within {+, -} range and at least 0 away from GSO satellite and above the 0(lat, azimuth) then

{

MinimumAngle = min(MinimumAngle, angle for this point)

}

Next point

If {+, -} bracket the GSO arc and the elevation of the GSO satellite is less than the minimum elevation angle for the GSO satellite azimuth then

{

MinimumAngle = min(MinimumAngle, max(0, 0[Lat, AzGSO] - GSO))

}

Return MinimumAngle or if not found, an error code

#### D3.2.3.4	WCGA_UP_SpecifcES_Repeating

If there are specific ES locations and the non-GSO satellite network using a repeating track orbit then there will be a highly limited number of geometries that are feasible. It is therefore possible to fly the satellite for the repeat period and then, for each ES, for each non-GSO satellite, calculate the . If  ≤ 0, or the elevation angle is below the minimum, then the ES would not transmit, otherwise the epfd towards that location can be derived.

Not all geometries will be feasible. For example:

–	Non-GSO ES above 81.29° N or S will not see the GSO arc and so can be excluded.

–	There will be a maximum difference in longitude between that of the non-GSO ES determined by the edge of coverage (EOC) elevation angle of the GSO system.

–	The non-GSO ES will not transmit to the non-GSO satellite when it is within the exclusion zone defined by  < 0.

–	The non-GSO ES will not transmit to the non-GSO satellite when its elevation angle is below the minimum  < 0[lat, az].

–	The non-GSO satellite is below the minimum operating height hmin.

–	The number of non-GSO satellites that can ES at this latitude is zero.

–	These are therefore checked in the following algorithm:

WCGA_UP_SpecifcES_Repeating

Calculate time step according to algorithm in § D4.3

For t = 0 to repeat time of constellation

Update positions of non-GSO satellites for this time step

For each non-GSO ES

If the non-GSO ES lat is <+81.29° && >-81.29° && Nco[Lat] > 0 then

For each non-GSO satellite

If satellite is visible and above minimum operating height

Calculate the elevation angle  and azimuth

Calculate the exclusion zone angle

If (  0[lat, az] and   0[lat) then

EPFDup=EIRP,lat+10log10Ncolat

If this is the worst epfd so far then store this geometry

Endif

Endif

Next satellite

Endif

Next ES

Next time step

#### D3.2.3.5	WCGA_UP_SpecifcES_NonRepeating

For the non-repeating case the non-GSO satellites will populate an orbit shell but only some of the geometries considered in the general case will be feasible as the non-GSO ES will only be at specific latitudes. Therefore this approach searches across the GSO arc as seen by each of the non-GSO ES then takes a similar approach to the general case.

WCGA_UP_SpecifcES_NonRepeating:

For each non-GSO ES

If the non-GSO ES latitude is <+81.29° or >-81.29° then

Calculate MaxDeltaLong = acos((Re/Rgeo)/cos(ES_lat))

NumLongSteps = (integer)(degrees(MaxDeltaLong)/0.1°)

For DeltaLongStep = -NumLongSteps to +NumLongSteps

DeltaLong = MaxDeltaLong * DeltaLongStep / NumLongSteps

GSO_Long = ES_Long + DeltaLong

Calculate (az, el) of ES as seen by GSO satellite

Convert (az, el) to (, φ)

Call CheckCaseUpGeneral(, φ)

Next DeltaLongStep

Endif

Next ES

#### D3.2.3.6	Range in latitude

When calculating the WCG(up) and also the epfd(up), it is necessary to identify where the ES could be located. While most systems in types A and B have global coverage, non-GSO networks of type C will be limited in the latitude range. For systems with multiple sub-constellations, this range could vary between sub-constellations.

The latitude range can be derived from the satellite’s height, its inclination angle and the minimum operating elevation angle for the ES, as in Fig. 32 below.

Figure 32

Calculation of maximum latitude for ESs

For elliptical systems, there will be two values, one for the apogee and another for the perigee, and so the inputs would be:

Semi-major axis of orbit (km):	a

Eccentricity of orbit:	e

Minimum elevation angle (radians):

Inclination angle (radians):	i

From these parameters, the following calculations can be undertaken:

ra=a1+e

ψ= π2+ ε

φa=sin–1 Rerasin ψ

θa=π-ψ+φa

Then:

Latmax=i+θa

Similarly, using:

rp=a1-e

Using the same equations but replacing suffix (a) with (p), the following can be derived:

Latmin=-i-θp

This assumes that for elliptical systems, the apogee is in the northern hemisphere, i.e. that one of the following is true:

e = 0

= 270°

where:

:	argument of perigee.

In the case that:

e > 0

= 90°

Then the following adjustments should be made:

Latmax’ = –Latmin

Latmin’ = –Latmax

In the case that the orbit inclination is zero and eccentricity zero (i.e. for an equatorial circular orbit) then these equations reduce to:

Latmax=θ

Latmin=-θ

## D3.3	WCG epfdIS

### D3.3.1	Inputs

The inputs to the algorithm are as follows:

SS_e.i.r.p.:	the satellite e.i.r.p. mask to check

θadB :	the GSO satellite’s beam half power beamwidth

:	the minimum elevation angle of the GSO system

a,i,e, ,, :	the orbit parameters of the non-GSO system.

The GSO satellite’s beam half power beamwidth and minimum elevation angle can be selected using the same method as in § D3.2.2 for the WCG epfd(up).

If there are multiple sub-constellations with alternative orbital elements then the process should be repeated for each unique set of {a, e, i}.

### D3.3.2	Algorithm

WCGA_IS:

From the epfd limits get the gain pattern to use

From the epfd limits get the GSO beamwidth θadB

From  θadB calculate φ1, φ2

Using the gain pattern calculate Grel(φi) for i = 1,2

From φ1 calculate LatBS

If for all satellites i = 0 then

{

Worst Case Geometry:

BS.Latitude = 0

BS.Longitude = LatBS

GSO.Longitude = 0

}

Else

{

Set WorstEPFDBin = -9999

Set WorstAngularVelocity = +9999

For all satellites in the order listed in ITU DB

{

Determine e.i.r.p. mask to use for this satellite

If this e.i.r.p. mask has not been checked so far then

Call GetWCGA_IS(e.i.r.p._mask, i)

End if

Next satellite

}

If no solution has been found then

{

Convert (=i, φ=φ1) to (az, el)

Using (az, el), create line from the GSO satellite

Put the ES at the first intersection point with the Earth

}

Rotate GSO, BS in longitude to ensure inline event

GetWCGA_IS(e.i.r.p._Mask, i):

LatStep = i / RoundUp(i) in degrees

For lat=i to +i in LatStep steps

{

Set satellite at latitude to calculate r, v

If satellite is above minimum operating height

{

From r, φi calculate ψi

From φi, ψi calculate Di, θi

Try to calculate ∆longi

In the cases that the geometries are feasible

{

From the GSO gain pattern calculate Grel(φi)

From the e.i.r.p. mask calculate e.i.r.p.(ψi)

Calculate epfdi

Calculate rgso, gso

Calculate θ of non-GSO satellite as seen by GSO

If epfdi is higher than Worstepfd

{

Store this geometry

WorstAngularVelocity = θ

Worstepfd = epfdi

}

Else if epfdi is the same bin as Worstepfd

{

If θ is lower than WorstAngularVelocity

{

Store this geometry

WorstAngularVelocity = θ

}

}

}

}

}

### D3.3.3	Geometry

There are two potentially significant geometries, which is when the non-GSO satellite just becomes visible as seen by the GSO satellite and when it traverses the GSO satellite beam, as in Fig. 33.

Figure 33

Two WCG(IS) non-GSO satellite locations

In some cases, the same WCG location will handle both geometries – e.g. for an equatorial satellite system a beam at the extreme in azimuth will be aligned for both geometries.

From the radius of the non-GSO satellite at each of the positions, it is possible to calculate the off-axis angle at the satellite and hence e.i.r.p.() together with the distance as in Fig. 34.

Figure 34

Two WCG(IS) calculation of satellite off-axis angle

where:

φ1=φBS (from above)

sinφ2= ReRgso

Hence:

sin ψi= RgeoRngso,isinφi

where i = {1, 2} noting that ψ1 > π2 and that ψ2 < π2 so that:

ψ1=π-sin-1 RgeoRngso,1sinφ1

ψ2=sin-1 RgeoRngso,2sinφ2

Then:

θi=π-φi- ψi

Di= Rngso,isin θisinφi

Hence given a non-GSO satellite with radius distance Rngso,i at the two specified geometries, the two single entry epfd levels can be calculated as:

epfdi=e.i.r.p.ψi+ Grel,i –10log10(4πDi2)

Note that Grel,1=0 and Grel,2=Grelφ2-φ1 and D is in metres.

By setting the non-GSO satellite at a specified latitude, lat, (where it has longitude long), it is therefore possible to derive the single entry epfd from the radius vector and the two geometries identified above.

In some cases, there will be no in-line geometry – for example for elliptical systems at apogee the line from the non-GSO satellite to the GSO arc will at no point intersect the Earth. This can be checked by calculating the difference in longitude between the non-GSO satellite and the point on the GSO arc from the geometry above and the non-GSO satellite’s latitude using:

cos ∆longi= cos θicos lati

If there is no solution to this equation, then there is no position that meets the required geometry. Other positions could be excluded if the non-GSO satellite was below the minimum operating height.

If there is a solution, then the position of the non-GSO and GSO satellite can be calculated from:

rngso= Rngsocoslatcoslongcoslatsinlongsin(lat)

rgso= Rgsocoslong-longsinlong-long0

For the i = 1 case the boresight is where the line L from the GSO to non-GSO satellite intersects the Earth’s surface.

L1λ= rgso+λrngso-rgso

For the i=2 case the boresight is on a line created using an adjusted non-GSO satellite position, calculated using:

R'ngso,2=Rgeo sinφsin ψ'2

where:

ψ'2=π-φ1-θ2

Given multiple locations with the same single entry epfd, then the one to use would be the one with the lowest angular velocity, using the same method as for WCGA(down), noting that the velocity vector of the GSO satellite can be derived in the same way as for earth stations, i.e.:

gso= we–yx0

The position of the GSO satellite would be chosen such that one of the non-GSO satellites with the e.i.r.p. mask identified traverses the critical geometry during its first orbit, using the same methodology as for the WCGA(up).

Additional geometry for the WCG for epfd(IS) is described in §§ D3.1.3 and D3.2.3 above.

# D4	Calculation of time step size and number of time steps

## D4.1	Simulation time increment and accuracy

The simulation time increment is one of the most important parameters for the determination of a distribution function of interference from non-GSO networks on the basis of the simulation model. Its specified value should guarantee absence of cases when high-level short-term interference exceeding an acceptable level is missed and is not considered. Otherwise results of simulation analysis will be inaccurate and sometimes erroneous. The decrease of a simulation time increment allows to increase accuracy of obtained results but at the same time results in increase of total number of simulation time increments and amount of required calculations.

The description of calculation algorithms for simulation time increment in uplink, downlink and intersatellite cases are shown below.

The algorithms are based upon a set of orbital characteristics, such as the inclination angle. If there are multiple sets, e.g. for multiple sub-constellations, then the longest run time and smallest time step over all sub-constellations should be used.

When an artificial precession is used, the same value should be used for all satellites of the non-GSO system. The value used should be the one calculated for the satellite used to derive the time step size and run duration.

In order to reduce run times, the following procedure should be used to adjust the time step size for non-repeating orbits cases where the number of time steps exceeds 1e8:

Calculate time step and run time using Nhit = 16

If orbit is non-repeating

If number of time steps exceeds 1e8 then:

Nhit'=NhitminNcourse,Nsatellites

Re-calculate time step and run time

Ncourse'=floorNhit'NhitNcourse

TScourse'=TS'.Ncourse'

Endif

Endif

## D4.2	Description of the procedure for the determination of minimum downlink simulation time increment

The value of simulation time increment should guarantee acquisition and description of the most short-term interference scenarios with the required accuracy. The high-level short-term interference is caused by emission of a non-GSO space station which is in-line situation (a non-GSO satellite passes through the main beam of a GSO earth station antenna). Therefore, the method for determining a simulation time increment tref  is based on ensuring the required number Nhit of epfd↓ estimations during the time interval t when a non-GSO satellite passes through the main beam of a GSO earth station antenna:

Δtref=ΔtNhit	(1)

tref  should be rounded to the nearest non-zero millisecond.

The time required for a non-GSO satellite passing through the main beam of a GSO earth station antenna depends on mutual location of the earth and space stations of the GSO network as well as on orbital parameters of the non-GSO network. The value of t should be computed at the location where the time for a non-GSO satellite to pass through the GSO main beam is smallest. Since this occurs when a GSO earth station is located directly under a GSO space station, pass time t is determined by equations (2) and (3) (see Fig. 35).

∆t= 2φω	(2)

where:

φ= 12θ3dB –arcsinReRe+h sin12θ3dB

(3)

s:	non-GSO satellite angular velocity of rotation around the Earth at the minimum operational altitude (degrees/s); for multiple orbits, the greatest such s should be selected

e:	Earth rotation angular velocity at the equator (degrees/s)

i:	orbit inclination (degrees)

3dB:	GSO earth station antenna 3 dB beamwidth (degrees)

Re:	Earth radius (km)

h:	orbit altitude (km) (see Note 1).

NOTE 1 – In the case that the constellation has multiple values of h for different sub-constellations or planes, the lowest value should be used. In the case of elliptical orbits the minimum operating height should be used.

Figure 35

Calculation of epfd(down) time step size

The Nhit value defines simulation accuracy. The higher the Nhit value the higher the accuracy of the final results.

Nhit should be set to 16 as derived in § D4.5. In a case when a non-GSO network satellite constellation consists of satellites with different orbital parameters it is necessary to determine a simulation time increment for each type of the orbits concerned and to define a minimum one.

TABLE 11

Input data

| Parameter | Designation | Units |
| --- | --- | --- |
| Orbit inclination | i | degree |
| Orbit altitude or for elliptical orbits the minimum operating height | h | km |
| GSO earth station antenna 3 dB beamwidth | 3dB | degree |
| Number of epfd↓ calculations required during the time a non-GSO satellite is passing through the main beam of a GSO earth station antenna | Nhit | – |

## D4.3	Description of the procedure for the determination of minimum uplink simulation time increment

High-level short-term uplink interference would be caused by emissions from a non-GSO earth station during an in-line event (when a GSO SS is in the main beam of a non-GSO earth station antenna). The required number Nhit of epfd↑ measurements should be effected within the period of the GSO satellite staying in the main beam of the non-GSO earth station antenna to ensure acquisition and definition of the in-line event. If the non-GSO earth station were directly below the GSO satellite (see Fig. 36) then the value of the minimum simulation time increment could be calculated using equations (1) and (2). In that case a width of the non-GSO earth station antenna main beam should be taken instead of that of the GSO earth station antenna main beam.

Figure 36

Calculation of epfd(up) time step size

TABLE 12

Input data

| Parameter | Designation | Units |
| --- | --- | --- |
| Orbit inclination | i | degree |
| Orbit altitude | h | km |
| Non-GSO earth station antenna 3 dB beamwidth | 3dB | degree |
| Number of epfd calculations required during the time a GSO satellite is passing through the main beam of a non-GSO earth station antenna | Nhit | – |

## D4.4	Description of the procedure for the determination of minimum inter-satellite simulation time increment

The time step size for epfdis calculations is derived by considering that there should be at least Nhit time steps during which the non-GSO satellite is within the main beam of the GSO satellite. Given that the shortest time step is when the beam at the GSO is pointed as far as possible away from the sub-satellite point, and given the following:

Re:	radius of the Earth

h:	height of non-GSO orbit

Rgeo:	radius of geostationary orbit

3dB:	half power beamwidth of GSO beam.

For elliptical orbit systems, calculate the height when the non-GSO satellite crosses the equator, i.e. when  = −  or + such that (+) = 0 or integer multiples of . In addition, it is necessary to consider the minimum operating height, so that the height to use is the maximum of {minimum operating height, height when crossing the equator}.

Then the time step can be calculated using the following algorithm (see Fig. 37).

Figure 37

Geometric parameters involved in the equations

Calculate:

D2=2D1sinθ3dB2

D3=D2cos180-θ2

Then calculate the value:

φ= 2arctanD3/2Re+h –D2/2sin(180 – θ2) 	(4)

which can be used in equation (2) to calculate the step size to use.

## D4.5	Derivation of Nhit

The time step size is selected to ensure there is sufficient resolution of the epfd within the victim’s main beam. The necessary resolution is driven by the bin size of 0.1 dB and from this the number of steps within the main beam can be derived.

Nhit should be selected so that the highest epfd will be detected in the simulation and identified to be within the right bin. This implies a resolution in the calculations of (0.1 dB)/2 = 0.05 dB.

The greatest error would occur when two-time steps are located equally spaced on either side of the main beam, as in Fig. 38.

Figure 38

Calculation of Nhits

The spacing between samples will be:

∆θ= θ3dBNhit

The gain pattern within the main beam can be assumed to be parabolic and hence following:

Grel=12θθ3dB2

The slope on this curve is:

ΔGrelΔθ=24θ3dB2θ

Hence, Nhit required for a difference in gain of 0.05 dB can be derived as follows:

∆Grel=0.05=24∙ θθ3dB∙ ∆θθ3dB=24∙θθ3dB∙ 1Nhit

So:

Nhit=480∙ θθ3dB

Therefore for the time step nearest the main beam when:

θ=12θ3dBNhit

Then:

Nhit=RoundUp240 =16

This approach can also be used for cross-track sampling, and hence Ntrack = Nhit = 16.

## D4.6	Total simulation run time

This section describes the calculation of number of time steps for the epfd↓ and epfd↑ algorithms specified in § D5. The basic approach first considers constellations with repeating and non-repeating ground track separately, where systems that use repeating ground tracks use station keeping to ensure that the satellites follow a single Earth trace. For example there will be minor launch errors and perturbations that would make the orbit drift unless station keeping was used operationally to ensure the track repeats. Administrations therefore must indicate to the BR whether station keeping is used to maintain a single track.

Some constellations have different values for inclination, height, or eccentricity between the planes. In this case, it is assumed that, to maintain coverage, the constellation would be designed so that the separation between planes does not change significantly. For the case of repeating ground tracks, this means that there will be a single repeat period for the constellation. This is the time for all satellites within the constellation to return to the same position relative to the Earth and each, within the constraints of station keeping. For the case of non-repeating ground tracks, there will be a single period for all the orbit planes to process around the equator.

This approach is to be used for constellations with both circular and elliptical orbits with orbit inclination non-zero. For equatorial orbit constellations in which all satellites have the same altitude, then it is sufficient to run for a single repeat period. This can be calculated as follows:

Trun= 2πws – we

where ws and we are the rotation angular velocities of the satellite and Earth as given in § D2.

Table 13 shows the input parameters used in all other constellation types.

TABLE 13

Input data

| Parameter | Designation | Units |
| --- | --- | --- |
| Constellation repeats yes/no | Type | – |
| Minimum samples taken to get statistical significance | NS  10 | – |

In both cases the time step can be calculated using the method described above. The number of time steps should be at least:

Nmin  	NS  /(100 – (maximum % in the Tables of Article 22 of the RR less than 100%))

So, for example, for the 99.999% case the number of steps would be:

Nmin=1 000 000

### D4.6.1	Repeating orbits

For those orbits specified as being repeating, the orbit predictor has to be accurate to ensure repeatability. Thus, there is an option for administrations to specify the exact longitude precession rate with respect to a point mass orbit predictor that ensures the orbit would repeat. The definition and use of this parameter is described in § D6.3.

With this parameter, a simulated orbit would be repeated but in reality there would be a slight drift due to longitudinal station keeping errors. It is expected that station keeping changes within the orbit plane would make no difference and so are not included.

The result should be as shown in Fig. 39.

Figure 39

Track of repeating orbit non-GSO satellite through GSO ES beam

In Fig. 39, it can be seen that the result would be a series of samples within the main beam of the GSO earth station that is sufficiently fine to resolve the main beam, includes station keeping drift and produces sufficient samples to generate the required statistics.

TABLE 14

Input data

| Parameter | Designation | Units |
| --- | --- | --- |
| Constellation repeat period | Prepeat | s |

Given the following parameters:

Nmin:	minimum number of time steps required for statistical significance

Prepeat:	time period that the constellation repeats (s)

Tstep:	time step (s)

Ntracks:	number of tracks through the main beam, = 16, as specified in § D4.5.

For this case the time step should not exactly divide the constellation repeat period. If:

Nrepsteps=PrepeatTstep

is an integer, then calculate a revised time step (used in the following equations in place of Tstep) equal to:

Tstep'= Tstep1+ Nrepsteps/ Nrepsteps

Calculate the time period required to get the minimum number of time steps for statistical significance:

Tsig=Nmin∙Tstep

This corresponds to the following number of constellation repeats:

Nrep  round (Tsig/Prepeat) to nearest integer above.

The number of repeats of the constellation is the largest of Nrep or Ntracks i.e.:

Nrun    max (Nrep, Ntracks)

Then the total run time is:

Trun  =  Nrun · Prepeat

So the number of time steps is then:

Nsteps  round (Trun/Tstep) to nearest integer below

Trun  =  Nsteps * Tstep

### D4.6.2	Non-repeating orbits

In this case the longitudinal spacing between successive ascending node passes must be examined so that there are sufficient tracks within the main beam. The time step size and number of time steps can be used to determine how far a particular orbit will have processed within the run. The same numbers can be used to determine how many time steps are required for the orbit to drift around the equator. The orbit period can then be used to work out the difference between tracks.

The constant that specifies the required number of points within the main beam can be used to specify the number of tracks though the main beam required (i.e. Ntrack  Nhits). If the gap between tracks is too large or too fine (resulting in either insufficient samples or excessive run times respectively) then artificial precession can be used.

It is expected that station keeping drift should cancel out in the long term and so would not be required for these calculations.

The result should be as shown in Fig. 40.

Figure 40

Track of non-repeating orbit non-GSO satellite through GSO ES beam

In Fig. 40, it can be seen that the result would be a series of tracks within the main beam of the GSO earth station that is sufficiently fine to resolve the main beam and produces sufficient samples to generate the required statistics.

TABLE 15

Input data

| Parameter | Designation | Units |
| --- | --- | --- |
| Orbit inclination | i | degree |
| Orbit semi-major axis | a | km |
| GSO earth station antenna 3 dB beamwidth(1) | 3dB | degree |
| Required number of tracks of a non-GSO satellite passing through the main beam of a GSO earth station antenna | Ntracks | – |
| (1) In the case of calculating run length for epfd↓. In the case of epfdis and epfd↑: epfd↑: calculate  using the beamwidth of the non-GSO earth station as specified in its e.i.r.p. mask using the calculation in equation (3) epfdis: calculate  using the beamwidth of the GSO satellite in the calculation in equation (4). |  |  |

Two parameters are required:

Spass	longitudinal spacing between successive ascending passes through the equatorial plane

Sreq	required resolution of passes through the equatorial plane based upon GSO earth station beam size.

These are calculated using the following steps:

Step 1:	Using the equations in § D6.3.2, calculate the n,  Ωr, ωr in radians / second

Step 2:	Convert the n,  Ωr, ωr  into degrees per minute

Step 3:	Calculate the nodal period of the orbit in minutes using:

Pn= 360wr+ n

Step 4:	Calculate the longitudinal spacing between successive ascending passes through the equatorial plane, S, given the Earth’s rotation rate (e  0.250684 degrees/min):

Spass=Ωe-Ωr∙Pn          degrees

The equations above apply to circular orbits. For elliptical orbit systems where the calculations above would be significantly different, the value of Spass should be supplied by the administration.

Step 5:	From the GSO earth station beamwidth and height, Sreq can be calculated using equation (3):

Sreq= 2φNtracks

Step 6:	Calculate the number of orbits to fully populate around the equator, taking into account that each plane has ascending and descending nodes:

Norbits= 180Sreq

Step 7:	Round Norbits to next highest integer.

Step 8:	Calculate total angle orbit has rotated round during this time:

Stotal=Norbits∙Spass

Step 9:	Calculate the number of multiples of 360 that this corresponds to, rounding to nearest integer below:

N360=intStotal360

Step 10:	Calculate the separation between planes that this corresponds to:

Sactual= 360N360Norbits

Step 11:		To ensure that the orbit drifts with the required precession rate, the following additional artificial precession should be included:

Sartificial=Sactual-Spass           degrees/orbit

or:

Dartificial= SartificialTperiod               degrees/s

Step 12:		Part D gives more information on how this parameter is used. The total run time is then the time to process around the equator using the orbit period from either §§ D6.3.1 or D6.3.2 depending upon or bit model, namely:

T’run    Tperiod · Norbits

Step 13:		The total number of time steps is then:

Nsteps    Round (T’run / Tstep) to nearest integer below

Trun    Nsteps *  Tstep.

## D4.7	Dual time step option

### D4.7.1	Dual time step option epfd(down)

In order to improve simulation performance an option to the algorithm is to implement two time steps. A coarse time step would be used except when any non-GSO satellite is near the main beam of the GSO ES or edge of exclusion zone, defined as:

GRX() > min[Gmax − 30 dB, GRX(0[Latitude]])

Figure 41 shows where to use the finer time step:

Figure 41

Use of fine time step when at with GSO ES main beam

A coarse step size is used for non-critical regions far from the GSO earth station main beam. This step size is defined as a topocentric angle:

φcoarse=1.5°

This coarse step size is used for all antenna beamwidths and all non-GSO systems.

The size of the coarse step needs to be an integer multiple of fine steps for statistical purposes. Since the coarse step size is constant, the ratio of coarse steps to fine steps is dependent only upon the beamwidth of the GSO earth station (3dB). This ratio is defined as:

Ncoarse=FloorNhits*φcoarseφ3dB

where floor is a function that truncates the decimal part of the ratio and outputs the integer part of the ratio. This produces a conservative ratio of fine steps to coarse steps to ensure that a coarse step is never larger than the target topocentric size of 1.5°.

### D4.7.2	Dual time step option epfd(up)

In order to improve simulation performance an option to the algorithm is to implement two-time steps. A coarse time step would be used except when the gain from any non-GSO ES towards the GSO satellite is greater than −30 dB.

A coarse step size is used for non-critical regions when the non-GSO earth station main beam is pointing away from the GSO satellite. This step size is defined as a topocentric angle:

φcoarse=1.5°

This coarse step size is used for all antenna beamwidths and non-GSO systems.

The size of the coarse step needs to be an integer multiple of fine steps for statistical purposes. Since the coarse step size is constant, the ratio of coarse steps to fine steps is dependent only upon the beamwidth of the non-GSO earth station (3dB). This ratio is defined as:

Ncoarse=FloorNhits*φcoarseφ3dB

where floor is a function that truncates the decimal part of the ratio and outputs the integer part of the ratio. This produces a conservative ratio of fine steps to coarse steps to ensure that a coarse step is never larger than the target topocentric size of 1.5 degrees.

# D5	epfd calculation description

## D5.1	epfd↓ software description

This section describes the algorithm to calculate the epfd↓ from a non-GSO constellation into a GSO downlink. It is assumed that each non-GSO satellite has a pfd mask. From the pfd for each satellite the aggregate epfd↓ at an earth station of a GSO system is calculated. This is repeated for a series of time steps until a distribution of epfd↓ is produced. This distribution can then be compared with the limits to give a go/no go decision.

Figure 42 shows the geometry with constellation of non-GSO satellites and test GSO satellite transmitting to a GSO earth station.

Figure 42

Example of epfd(down) scenario

### D5.1.1	Configuration parameters

These run definition parameters are:

| Parameter name | Parameter value | Parameter units and ranges |
| --- | --- | --- |
| Frequency | F_DOWN | MHz |
| GSO satellite longitude | GSO_LONG | degree |
| GSO earth station latitude | GSO_ES_LAT | degree |
| GSO earth station longitude | GSO_ES_LONG | degree |
| GSO earth station dish size | GSO_ES_D_ANT | m |
| GSO earth station gain pattern | GSO_ES_PATTERN | One of those in § D6.5 |
| Reference bandwidth | REFBW | kHz |
| Number of epfd↓ points | Nepfd_DOWN | – |
| Array of Nepfd_DOWN epfd↓ values | epfd_DOWN[I] | dB(W/(m2 · BWref)) |
| Array of Nepfd_DOWN percentages | PC[I] | % |

### D5.1.2	Non-GSO system parameters

The following parameters, as specified in § B3.1 would be used.

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Satellite pfd mask | See Part C for definition and format |  |
| Number of non-GSO satellites | Nsat | – |
| Transmit centre frequency | F_DOWNsat | MHz |
| Exclusion zone angle by latitude, potentially varying by satellite | MIN_EXCLUDE[Latitude] | degree |
| Minimum satellite tracking duration by latitude | MIN_DURATION [Latitude] | second |
| Maximum number of satellites operating co-frequency at Earth location, potentially varying by latitude | MAX_CO_FREQ [Latitude] | – |
| Orbit has repeating ground track maintained by station keeping | Yes or No | – |
| Administration is supplying specific node precession rate | Yes or No | – |
| Station keeping range for ascending node as half total range | Wdelta | degree |
| Minimum operating height | H_MIN | km |
| Minimum operating elevation angle by latitude and azimuth | ES_MINELEV[Latitude] [Azimuth] | degree |
| Minimum angle between active non-GSO satellites at ES | MIN_ANGLE_AT_ES | degree |

For each satellite the following parameters specified in § B3.2 would be used, where the definitions of the parameters are specified in § D6.3.1 at the time of the start of the simulation.

Note that in the Table below, the indices [N] are present to indicate that there would be a different value for each satellite, and the N-th value corresponds to the N-th satellite. For the pfd mask it indicates that the pfd data is structured in such a way that the pfd[N] entry is a reference that points to a particular sub-set. For example each satellite in the constellation could reference the same pfd(lat, az, el), pfd(lat, X, long), or pfd(lat, , long) table.

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| pfd mask to use | pfd[N] | – |
| Semi-major axis | A[N] | km |
| Eccentricity | E[N] | – |
| Inclination | I[N] | degree |
| Longitude of ascending node | O[N] | degree |
| Argument of perigee | W[N] | degree |
| True anomaly | V[N] | degree |

### D5.1.3	Run time step parameters

The following run parameters should be calculated using the algorithm in § D4:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Time step | TSTEP | s |
| Number of time steps | NSTEPS | – |

The time step and statistics take account of the time windows as shown in Fig. 43.

Figure 43

Run time, fine and course time steps and time windows

The following are calculated in § D4:

–	Run duration

–	Fine time step

–	Course time step.

In the case that the non-GSO satellite selection method is defined by track duration (e.g. for the MIN_DURATION [Latitude] to be non-zero for at least one latitude), then it is also necessary to consider a set of time windows to ensure all combinations of satellite selection are analysed.

The MIN_DURATION is provided for the constellation (possibly dependent upon latitude, which in this case of the ES). The length of the time window in fine time steps is calculated as the integer ratio of this track duration divided by fine time step size. The MIN_SLIDING_TIME is also calculated as an integer number of fine time steps.

The minimum sliding time is:

MIN_SLIDING_TIME=max1 second,MinimumOrbitalPeriod100.Nsatellites

Where MinimumOrbitalPeriod is the smallest orbit period over all sub-constellations.

The run duration is derived in § D4 so that the constellation has returned to the initial conditions and hence statistics are complete. This could require that the last time window is only partially included: i.e. only those time steps within the run duration would be included in the epfd statistics. However the simulation would continue until the time window is complete to determine the satellites to track. In addition, as each window starting time is offset it is necessary to run for other windows additional steps beyond that used by the first-time window.

The length of the sliding window in number of fine time steps is:

NSW = RoundDown(MIN_DURATION / Tfine)

The number of fine time steps for the MIN_SLIDING_TIME can be calculated as:

NMSL = RoundUp(MIN_SLIDING_TIME / Tfine)

The number of time windows to use in the calculations is then:

NTW = RoundUp(Nsw / NMSL)

The number of repeats of the time window for the run duration is then:

NRepeat = RoundUp(Nstep / NSW)

The total number of fine time steps required to simulate is the run duration plus the time required to complete all the time windows, namely:

NTotalSteps = Nrepeat * NSW + (NTW  – 1) * NMSL

The total run duration required to get all the sliding windows to complete is then:

TTotalDuration = NTotalSteps * Tfine

### D5.1.4	Algorithms and calculation procedures

In the case that the non-GSO satellite selection method is defined by track duration (e.g. for the MIN_DURATION [Latitude] to be non-zero for at least one latitude) then the algorithm and calculation procedures are as in § D5.1.4.2, otherwise they are as in § D5.1.4.1.

The operating non-GSO satellites are those outside the exclusion zone, above their minimum operating elevation angle and transmitting towards (i.e. height above or equal to MIN_OPERATING_HEIGHT) the GSO earth station. The maximum number of operating non-GSO satellites is the maximum number of non-GSO satellites allowed to transmit co-frequency towards the same area on the ground.

#### D5.1.4.1	Algorithms and calculation procedures when not using track duration

To calculate epfd↓ values from a non-GSO system that is not using the track duration into one GSO system earth station the following algorithm should be used. The algorithm can be used on multiple GSO systems in parallel if required.

Step 1:	Read in parameters for non-GSO system as specified in § D5.1.2.

Step 2:	Read in GSO parameters as specified in § D5.1.1.

Step 3:	If required, calculate the maximum epfd GSO location using the algorithm in § D3.1 otherwise use the GSO satellite and ES location requested.

Step 4:	Calculate the number of time steps and time step size using algorithm in § D4 and hence calculate end time.

Step 4bis:	Initialize statistics by zeroing all bins of epfd↓ values.

Step 5:	If a dual time step algorithm is included then use Sub-step 5.1, otherwise Ncoarse = 1 all the time.

Sub-step 5.1:	Calculate coarse step size Tcoarse = Tfine * Ncoarse.

Step 6:	If a dual time step algorithm is included then repeat Sub-step 6.1 to Step 22 until end time is reached, otherwise repeat Steps 7 to 22 until end time is reached.

Sub-step 6.1:	If it is the first time step then set Tstep = Tfine.

Sub-step 6.2:	Otherwise if there are less than Ncoarse steps remaining then set Tstep = Tfine.

Sub-step 6.3:	Otherwise, if any of the GRX() for the last time step were within 30 dB of peak then set Tstep = Tfine; otherwise, set Tstep = Tcoarse.

Step 7:	Update position vectors of all earth stations based on coordinate system in § D6.1.

Step 8:	Update position vectors of all GSO satellites based on coordinate system in § D6.2.

Step 9:		Update position and velocity vectors of all non-GSO satellites based on coordinate system, orbit prediction model and station keeping algorithm in § D6.3.

Step 10:		Set epfd↓ = 0.

Step 11:		Select all non-GSO satellites visible to the GSO earth station using the algorithm in § D6.4.1.

Step 12:	 	Repeat Steps 13 to 18 for each visible non-GSO satellite.

Step 13:		Calculate the parameters required by the pfd mask, either (lat, , long) or (lat, azimuth, elevation) as required, using the definition of angles in § D6.4.4 or § D6.4.5.

Step 13bis:	Calculate the (AzimuthNGSO, NGSO) of the non-GSO satellite as seen at the GSO ES location using the definition of angles in § D6.4.4.

Step 14:		Using the pfd mask for the selected non-GSO satellite, calculate pfd(lat, , long) or pfd(lat, azimuth, elevation) at the GSO earth station using the non-GSO satellite pfd mask as specified in § D5.1.5.

Step 15:		Calculate the off-axis angle  at GSO earth station between line to the GSO satellite and the non-GSO satellite.

Step 16:		Calculate GRX() = Receive gain (dB) at GSO earth station using relevant gain pattern specified in algorithms in § D6.5.

Step 17:		Calculate epfd↓i for this non-GSO satellite using:

epfd↓i = pfd + GRX() – Gmax

where Gmax is the peak gain of the GSO earth station antenna.

Step 18:		Store the epfd↓i entries for each satellite that:

•	either has an  ≥ 0[latitude] for that satellite and has an NGSO ≥ 0[latitude][AzimuthNGSO]

•	or for which GRX() > min[Gmax − 30 dB, GRX(o[Latitude]])

Step 19:		Repeat Steps 20 and 21 for each time step as long as no more than MAX_CO_FREQ[lat] satellites are included in Step 23.

MAX_CO_FREQ[lat] is the maximum number of operating non-GSO satellites at the latitude of GSO_ES considered corresponding to the maximum number of satellites allowed to transmit at the same frequency towards the same area on the ground, fulfilling the GSO exclusion zone, the minimum elevation angle and the MIN_ANGLE_AT_ES requirements as defined for the non-GSO system.

Step 20:	 	Apply Step 23 for the satellite which has an  ≥ 0[latitude] and an NGSO ≥ 0[latitude][AzimuthNGSO] and with the highest single epfd↓i.

Step 21: 	If the MIN_ANGLE_AT_ES between non-GSO satellites has been specified, then remove the non-GSO satellites from those in Step 18 which have an  ≥ 0[latitude] and an NGSO ≥ 0[latitude][AzimuthNGSO] that do not meet this criterion with respect to the satellite used in Step 20.

Step 22: 	Repeat Step 23 for those satellites for which GRX() > min[Gmax − 30 dB, GRX(o[Latitude]]).

Note:		There should be no double-counting of satellites in Step 23. In particular, if a satellite is on the list of the highest MAX_CO_FREQ[lat] satellites and also for which GRX() > min[Gmax − 30 dB, GRX(o[Latitude]]) it should be included only once as part of the MAX_CO_FREQ[lat] satellites in Step 20.

Step 23:		Increment epfd↓ by the epfd↓i value in linear.

Step 24:		Increment epfd↓ statistics by epfd↓ for this time step by (Tstep/Tfine) entries.

Step 25:		Generate the epfd↓ CDF from the epfd↓ PDF using the algorithm in § D7.1.2.

Step 26:		Compare epfd↓ statistics with limits using the algorithm in § D7.1.

Step 27:		Output results in format specified in § D7.3.

#### D5.1.4.2	Algorithms and calculation procedures when using track duration

To calculate epfd↓ values from a non-GSO system that is using the track duration into one GSO system earth station the following algorithm should be used. The algorithm can be used on multiple GSO systems in parallel if required.

Step 1:		Read in parameters for non-GSO system as specified in § D5.1.2.

Step 2:		Read in GSO parameters as specified in § D5.1.1.

Step 3:		If required, calculate the maximum epfd GSO location using the algorithm in § D3.1 otherwise use the GSO satellite and ES location requested.

Step 4:		Calculate the number of time steps and time step size using algorithm in § D4 and hence calculate end time. As described in § D5.1.3, adjust the MIN_SLIDING_TIME and MIN_DURATION to be an integer number of fine time step and hence calculate the NUM_SLIDE_WINDOWS. Increment the run duration by the integer number of fine time steps so that all sliding windows will have complete statistics.

Step 4bis:	Initialize statistics by zeroing all bins of epfd↓ values for each of the NUM_SLIDE WINDOWS.

Step 5:		If a dual time step algorithm is included then use Sub-step 5.1, otherwise Ncoarse = 1 all the time.

Sub-step 5.1:	Calculate coarse step size Tcoarse = Tfine * Ncoarse.

Step 6:		If a dual time step algorithm is included then repeat Sub-step 6.1 to Step 22 until end time is reached, otherwise repeat Steps 7 to 22 until end time is reached.

Sub-step 6.1:	If it is the first time step then set Tstep = Tfine.

Sub-step 6.2:	Otherwise if there are less than Ncoarse steps remaining then set Tstep = Tfine.

Sub-step 6.3:	Otherwise, if any of the GRX() for the last time step were within 30 dB of peak then set Tstep = Tfine; otherwise, set Tstep = Tcoarse.

Step 7:		Update position vectors of all earth stations based on coordinate system in § D6.1.

Step 8:		Update position vectors of all GSO satellites based on coordinate system in § D6.2.

Step 9:		Update position and velocity vectors of all non-GSO satellites based on coordinate system, orbit prediction model and station keeping algorithm in § D6.3.

Step 10:		Set epfd↓ = 0.

Step 11:		Select all non-GSO satellites visible to the GSO earth station using the algorithm in § D6.4.1.

Step 12:		Repeat Steps 13 to 18 for each visible non-GSO satellite.

Step 13:		Calculate the parameters required by the pfd mask, either (lat, , long) or (lat, azimuth, elevation) as required, using the definition of angles in § D6.4.4 or § D6.4.5.

Step 13bis:	Calculate the (AzimuthNGSO, NGSO) of the non-GSO satellite as seen at the GSO ES location using the definition of angles in § D6.4.4.

Step 14:		Using the pfd mask for the selected non-GSO satellite, calculate pfd(lat, , long) or pfd(lat, azimuth, elevation) at the GSO earth station using the non-GSO satellite pfd mask as specified in § D5.1.5.

Step 15:		Calculate the off-axis angle  at GSO earth station between line to the GSO satellite and the non-GSO satellite.

Step 16:		Calculate GRX() = Receive gain (dB) at GSO earth station using relevant gain pattern specified in algorithms in § D6.5.

Step 17:		Calculate epfd↓i for this non-GSO satellite using:

epfd↓i = pfd + GRX() – Gmax

where Gmax is the peak gain of the GSO earth station antenna.

Step 18:		Store the epfd↓i entries for each satellite that:

•	either has an  ≥ 0[latitude] for that satellite and has an NGSO ≥ 0[latitude][AzimuthNGSO]

•	or for which GRX() > min[Gmax − 30 dB, GRX(0[Latitude]])

Step 19:		If a window is closing this time step then identify which non-GSO satellites met the 0, 0 constraints for the full duration of the window.

Step 19bis:	For each of those satellites that met the 0, 0 constraints for the full duration of the window, calculate the highest epfd↓[nSat] over the time window and sort this list of satellites by maximum epfd↓[nSat] per satellite.

Step 20:		Repeat Steps 21 and 22 for each time step in the window for the MAX_CO_FREQ[lat] satellite’s epfd↓[nSat] contributions on this list plus those satellites for which GRX() > min[Gmax − 30 dB, GRX(0[Latitude]]), where MAX_CO_FREQ[lat] is the maximum number of operating non-GSO satellites at the latitude of GSO_ES considered corresponding to the maximum number of satellites allowed to transmit at the same frequency towards the same area on the ground, fulfilling the GSO exclusion zone and minimum elevation angle requirements as defined by for the non-GSO system.

Note: There should be no double-counting of satellites in Step 21. In particular, if a satellite is on the list of the highest MAX_CO_FREQ[lat] satellites and also for which GRX() > min[Gmax − 30 dB, GRX(0[Latitude]]) it should be included only once as part of the MAX_CO_FREQ[lat] satellites in Step 21.

Step 21:		Increment epfd↓ by the epfd↓i value in linear.

Step 22:		Increment epfd↓ statistics for the relevant slide window by epfd↓ for this time step by (Tstep/Tfine) entries. If last time step was a course time step and the window closed during that time step, then the epfd↓ statistics should be updated by that part of Tstep that was in the window and the remaining part stored for the following window. If the run since start time for this slide window exceeds the run duration then only include that part of the time window that is within the run duration in the statistics.

Step 23:		Generate the epfd↓ CDF for all slide windows from the epfd↓ PDF using the algorithm in § D7.1.2.

Step 24:		Compare epfd↓ statistics for all slide windows with limits using the algorithm in § D7.1.

Step 25:		Output results in format specified in § D7.3.

### D5.1.5	pfd mask calculation

The pfd mask is defined as a table of pfd values for various angles and latitudes.

Note that the latitude range should be:

Minimum:	–i

Maximum:	+i

where i is the inclination of the non-GSO satellite’s orbit.

In general, the (azimuth, elevation) or (,long) angles calculated at each time step will be between two values in the arrays. In this case bilinear interpolation between pfd values should be used with equations:

pfd=1-λx1-λypfd11+λx1-λypfd21+1-λxλypfd12+λxλypfd22

where:

λx= x - x1x2 - x1

λy= y - y1 y2 - y1

And (x, y) are the two dimensions of the pfd mask.

If the angles are outside the pfd mask, the software calculates pfd from the highest angle in the mask (i.e. at the mask edge).

The mask that is closer in latitude to that of the reference satellite should be used. Part C gives more information about the format and sampling of the pfd mask.

### D5.1.6	Outputs

The result of the algorithm is two arrays in format:

| Array of epfd↓ values | epfd_DOWN_CALC[I] | dB(W/(m2 · BWref)) |
| --- | --- | --- |
| Array of percentages | PC_CALC[I] | % |

where PC_CALC[I] is the percentage of time epfd_DOWN_CALC[I] is exceeded.

## D5.2	epfd↑ software description

This section describes the algorithm to calculate epfd↑ from non-GSO earth stations into a GSO uplink. The locations of the ES can be defined in one of two ways:

1)	It is assumed that the Earth is populated with a uniform distribution of non-GSO earth stations. In this case the ES_ID in the e.i.r.p. mask should be set to −1.

2)	The locations of specific ES are used via a field in the ES e.i.r.p. mask. In this case the density field is not used.

Each earth station points towards a non-GSO satellite using pointing rules for that constellation, and transmits with a defined e.i.r.p. From the e.i.r.p. and off-axis gain pattern for each earth station, the epfd↑ at the GSO can be calculated. This is repeated for a series of time steps until a distribution of epfd↑ is produced. This distribution can then be compared with the limits to give a go/no go decision.

Figure 44 shows the geometry with population of non-GSO earth stations transmitting to a constellation of non-GSO satellites, together with a test GSO satellite receiving from a GSO earth station.

Figure 44

Example of epfd(up) scenario

### D5.2.1	Configuration parameters

This sub-section specifies the parameters required for all epfd↑ calculations defined in the RR. This would be a data-set of N sets of limits that can be shared between runs. The Table could be queried so that the required values can be used depending upon non-GSO system frequency.

For each set of limits the following would be defined as generated by in § D2.1.

| Parameter name | Parameter value | Parameter units and ranges |
| --- | --- | --- |
| Frequency | FREQ | MHz |
| GSO gain pattern | FEND_UP | One of those in § D6.5 |
| GSO peak gain | GSO_SAT_PEAKGAIN | dBi |
| GSO half power beamwidth | GSO_SAT_BEAMWIDTH | degree |
| Reference bandwidth | RAFBW | kHz |
| Number of epfd↑ points | Nepfd_UP | – |
| Array of Nepfd_UP epfd↑ values | epfd_UP[I] | dB(W/(m2 · BWref)) |
| Array of Nepfd_UP percentages | PC_UP[I] | % |

### D5.2.2	Determination of maximum epfd configuration

The maximum epfd location of the GSO satellite and beam centre is defined in § D3.2.

### D5.2.3	Calculation of run steps

The time step and number of time steps are calculated using the algorithm in § D4 which also describes the optional dual time step option.

### D5.2.4	Inputs

#### D5.2.4.1	Input parameters

This section defines the input parameters for a particular non-GSO system scenario. In this case, input is a generic term that could include files or user input. Information is required for:

–	non-GSO system;

–	GSO system;

–	run configuration.

#### D5.2.4.2	Non-GSO system parameters

The following parameters, as specified in § B3.1 would be used:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Number of non-GSO satellites | Nsat | – |
| Orbit has repeating ground track maintained by station keeping | Yes or No | – |
| Administration is supplying specific node precession rate | Yes or No | – |
| Station keeping range for ascending node as half total range | Wdelta | degrees |

For each satellite the following parameters specified in § B3.2, would be used, where the definitions of the parameters are specified in § D6.3.1 at the time of the start of the simulation.

Note that in the Table below, the indices [N] are present to indicate that there would be a different value for each satellite, and the N-th value corresponds to the N-th satellite.

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Semi-major axis | A[N] | km |
| Eccentricity | E[N] | – |
| Inclination | I[N] | degrees |
| Longitude of ascending node | O[N] | degrees |
| Argument of perigee | W[N] | degrees |
| True anomaly | V[N] | degrees |

Each satellite must have an independent set of six orbital parameters for orbit definition and subsequent propagation.

To define the characteristics of non-GSO earth stations, the following parameters, as specified in § B4.2 would be used:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Maximum number of co-frequency tracked non-GSO satellites | MAX_CO_FREQ [Latitude] | – |
| Earth station e.i.r.p. mask by latitude | ES_e.i.r.p.[lat] | dB(W/BWref) |
| Minimum elevation angle | ES_MINELEV[Latitude] [Azimuth] | degree |
| Exclusion zone angle by latitude, potentially varying by satellite | MIN_EXCLUDE [Latitude] | degree |
| Average number of non-GSO ES active at the same time per km2 | ES_DENSITY | km2 |
| Average distance between cell or beam foot-print centres | ES_DISTANCE | km |
| The maximum number of non-geostationary earth stations tracked co-frequency by a non-geostationary satellite within the EPFD(up) calculation. If a value is not provided, it is assumed that the maximum number of earth stations tracked co-frequency by a non-geostationary satellite is equal to the number of earth stations created for the epfd↑ run | MAX_CO_FREQ_SAT | – |
| The minimum angle in degrees at the non-GSO satellite between the lines to any two active non-GSO earth stations. Assumed to be zero if not provided | MIN_ANGLE_AT_SAT | degree |
| Minimum angle in degrees at the surface of the Earth between the lines to any two active non-GSO satellites. Assumed to be zero if not provided. Not applicable if the MIN_DURATION[Latitude] is non-zero | MIN_ANGLE_AT_ES | degree |

Note that the minimum track duration is not used for the epfd(up) case.

#### D5.2.4.3	GSO system parameters

The GSO system can be either calculated or use worst-case parameters using the algorithm in § D3.2 or entered values. The required parameters as specified are:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| GSO satellite longitude | GSO_SAT_LONG | degree |
| GSO boresight latitude | BS_LAT | degree |
| GSO boresight longitude | BS_LONG | degree |
| GSO reference gain pattern | GSO_SAT_PATTERN | One of those in § D6.5 |

These parameters are defined in §§ D6.1 and D6.2.

#### D5.2.4.4	Run parameters

The following run parameters should be calculated using the algorithm in § D4:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Time step | TSTEP | s |
| Number of time steps | NSTEPS | – |

### D5.2.5	Production of non-GSO earth station distribution

In the case that the non-GSO ES locations are defined by a distribution, the following method should be used:

Step 1:	Calculate the number of actual operating non-GSO earth stations that the representative earth station will represent using:

NUM_ES = ES_DISTANCE * ES_DISTANCE * ES_DENSITY

Step 2:	Calculate e.i.r.p. to use for each representative non-GSO earth station using:

REP_e.i.r.p. = ES_e.i.r.p. + 10log10(NUM_ES)

Step 3:	Define the GSO service area as the region enclosed by the contour representing a relative gain of 15 dB.

Step 4:	For every distance ES_DISTANCE in latitude and distance ES_DISTANCE in longitude within the service area defined in Step 3, locate a representative non-GSO earth station with radiating with REP_e.i.r.p.

If the supplied ES_DISTANCE is zero, then in Step 1 set NUM_ES = 1 and at Step 4 locate a single non_GSO ES at the boresight of the GSO satellite.

The NUM_ES is typically 1 for TDMA and FDMA systems and for CDMA systems equal to the number of co-frequency ES all operating on the same frequency at the same time and location. The ES_DISTANCE relates to the average distance between co-frequency beams.

The deployment method should be symmetric around the (latitude, longitude) of the GSO satellite’s boresight, as shown in Fig. 45.

Figure 45

Deployment method for non-GSO earth stations

No non-GSO ESs should be deployed below the minimum latitude or above the maximum latitude, where these two extreme values are derived using the methodology in § D3.2.3.

The separation in latitude in radians between non-GSO ES can be calculated from the distance using:

∆lat= dRe

The separation in longitude in radians between non-GSO ES can be calculated using:

∆long= dRecos lat

### D5.2.6	Algorithms and calculation procedures

To calculate epfd↑ values from one non-GSO system into one GSO system satellite the following algorithm should be used. The algorithm can be used on multiple GSO systems in parallel if required:

Step 1:	Read in parameters for non-GSO system as specified in § D5.2.4.2.

Step 2:	Read in GSO parameters as specified in § D5.2.4.3.

Step 3:	If required calculate maximum epfd GSO location using the algorithm in § D3.2 otherwise use the GSO satellite and ES location requested.

Step 4:	If required calculate locations of non-GSO earth stations using the algorithm in § D5.2.5.

Step 5:	Initialize statistics by zeroing all bins of epfd↑ values.

Step 6:	If required calculate number of time steps and time step size using the algorithm in § D4 and hence calculate end time.

If a dual time step algorithm is included then use Sub-step 6.1, otherwise Ncoarse  1 all the time.

Sub-step 6.1:	Calculate coarse step size Tcoarse = Tfine * Ncoarse.

Step 7:	Repeat Steps 8 to 24 for all time steps.

If a dual time step algorithm is included, then repeat Sub-step 7.1 to Step 22 until end time is reached.

Sub-step 7.1:	If it is the first-time step then set Tstep = Tfine.

Sub-step 7.2:	Otherwise if there are less than Ncoarse steps remaining then set Tstep = Tfine.

Sub-step 7.3:	Otherwise if any of the  angles for the last time step were within coarse of the exclusion zone angle then set the Tstep = Tfine; otherwise, use Tstep = Tcoarse.

Step 8:	Update position vectors of all earth stations using algorithm in § D6.1.

Step 9:	Update position and velocity vectors of all non-GSO satellites using algorithm in § D6.3.2.

Step 10:		Update position vectors of GSO satellite using algorithm in § D6.2.

Step 11:		Initialise a list of possible links comprising triples of {non-GSO satellite, non-GSO ES, epfd↑I } to be an empty list.

Step 12:		Set the counter of times each satellite is used to zero for all satellites.

Step 13:		Set the counter of times each non-GSO ES is used to zero for all non-GSO ES.

Step 14:		Repeat Steps 15 to 24 for all non-GSO earth stations.

Step 15:		Determine if this non-GSO earth station is visible to the GSO satellite using the algorithm in § D6.4.1.

Step 16:		If the non-GSO earth station is visible to the GSO satellite then do Steps 17 to 24.

Step 17:		For all non-GSO satellites repeat Steps 18 to 24.

Step 18:		If this non-GSO satellite is a) visible to the non-GSO ES and b) above the minimum elevation angle 0[latitude][AzimuthNGSO] at the non-GSO for this latitude and the non-GSO satellite’s azimuth and c) outside the exclusion zone for this latitude 0[latitude], undertake Steps 19 to 24.

Step 19:		Calculate ES_e.i.r.p.[lat] (dB(W/BWraf)) of non-GSO earth station at its given latitude in direction of GSO satellite using non-GSO earth station e.i.r.p. mask in § C3.

REP_e.i.r.p. = ES_e.i.r.p.[lat, off-axis angle] + 10log10 (NUM_ES)

Step 20:		Calculate GRX = receive relative gain (dB) at GSO satellite using relevant gain pattern specified in the algorithms in § D6.5.

Step 21:		Calculate D = distance (km) between the non-GSO earth station and the GSO satellite using the algorithm in § D6.4.1.

Step 22:		Calculate the spreading factor LFS = 10log(4D2) + 60.

Step 23:		Calculate epfd↑i for this non-GSO satellite:

epfd↑i = REP_e.i.r.p. – LFS + GRX – Gmax

Step 24:		Add to the list of possible links this {non-GSO Satellite, non-GSO ES, epfd↑i }.

Step 25:		Sort the list of possible links by epfd↑i with the highest epfd↑i at the top.

Step 26:		Set epfd↑ = 0.

Step 27:		If the list of possible links is empty jump to Step 35 otherwise repeat Steps 28 to 34 for the link {non-GSO satellite, non-GSO ES, epfd↑i } on the list of possible link with the highest epfd↑i

Step 28:		Increment the epfd↑ by this link’s epfd↑i

Step 29: 	Increment the counter of the number of times this link’s non-GSO satellite has been used.

Step 30:		If this non-GSO satellite counter is equal to MAX_CO_FREQ_SAT then remove all remaining links on the list of possible links that use this non-GSO satellite.

Step 31: 	Increment the counter of the number of times this link’s non-GSO ES has been used.

Step 32:		If this non-GSO ES counter is equal to MAX_CO_FREQ then remove all remaining links on the list of possible links that use this non-GSO ES.

Step 33:		If the MIN_ANGLE_AT_ES is non-zero, then remove all links from the list that have an angle at the ES to this link’s non-GSO satellite is less than MIN_ANGLE_AT_ES.

Step 34:		If the MIN_ANGLE_AT_SAT is non-zero, then remove all links from the list that have an angle at the satellite to this link’s non-GSO satellite is less than MIN_ANGLE_AT_SAT.

Step 35:		Increment epfd↑ statistics by this epfd↑.

If a dual time step algorithm is included, then the step below should be used:

Sub-step 35.1:	Increment epfd↑ statistics by the epfd↑ for this time step by Tstep/Tfine entries.

Step 36:		Generate the epfd↑ CDF from the epfd↑ pdf using the algorithm in § D7.1.2.

Step 37:		Compare epfd↑ statistics with limits using the algorithm in § D7.1.

Step 38:		Output results in format specified in § D7.2.

### D5.2.7	Calculation of e.i.r.p.

If the ES_e.i.r.p. mask is of the form ES_e.i.r.p.(lat)() then find the table with the nearest latitude to the non-GSO ES under consideration, and then use that to undertake linear interpolation in ES_e.i.r.p. against the off-axis angle at the non-GSO ES towards the GSO satellite.

If the ES_e.i.r.p. mask is of the form ES_e.i.r.p.(lat)(az)(el)(DeltaLongES) then find the table with the nearest latitude to the non-GSO ES under consideration. Within that table, find the (az)(el) array with the smallest angular offset to the (azimuth, elevation) of the non-GSO satellite as seen by the non-GSO ES. Use that array to undertake linear interpolation in ES_e.i.r.p. against the difference in longitude between the non-GSO ES and the GSO satellite.

### D5.2.8	Outputs

The result of the algorithm is two arrays of size Nepfd↑ in format:

| Array of Nepfd_UP epfd↑ values | epfd_UP_CALC[I] | dB(W/(m2 · BWref)) |
| --- | --- | --- |
| Array of Nepfd_UP percentages | PC_CALC[I] | % |

where PC_CALC[I] is the percentage of time epfd_UP_CALC[I] is exceeded.

## D5.3	epfdis software description

This section describes the algorithm to calculate epfdis from non-GSO space stations into a GSO uplink. From the e.i.r.p. and off-axis angle for each space station, the epfdis at the GSO space station can be calculated. This is repeated for a series of time steps until a distribution of epfdis is produced. This distribution can then be compared with the limits to give a go/no go decision.

### D5.3.1	Configuration parameters

This sub-section specifies the parameters required for all epfdis calculations. This would be a data-set of N sets of limits that can be shared between runs. The Table could be queried so that the required values can be used depending upon non-GSO system frequency.

For each set of limits the following would be defined as derived in § D2.1.

| Parameter name | Parameter value | Parameter units and ranges |
| --- | --- | --- |
| Frequency band start | FREQ | MHz |
| GSO gain pattern | FEND_IS | One of those in § D5.5 |
| GSO peak gain | GSO_SAT_PEAKGAIN | dBi |
| GSO half power beamwidth | GSO_SAT_BEAMWIDTH | degrees |
| Reference bandwidth | RIFBW | kHz |
| Number of epfdis points | Nepfd_IS | – |
| Array of Nepfd_IS epfdis values | epfd_IS[I] | dB(W/(m2 · BWrif)) |
| Array of Nepfd_IS percentages | PC_IS[I] | % |

### D5.3.2	Determination of maximum epfd configuration

The maximum epfd location of GSO satellite and beam centre is defined in § D3.3.

### D5.3.3	Calculation of run steps

A single time step and number of time steps are calculated using the algorithm in § D4.

### D5.3.4	Input parameters

This sub-section defines the input parameters for a particular non-GSO system scenario. In this case, input is a generic term that could include files or user input. Information is required for:

–	non-GSO system;

–	GSO system;

–	run configuration.

#### D5.3.4.1	Non-GSO system parameters

The following parameters, as specified in § B2.1 would be used:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Number of non-GSO satellites | Nsat | – |
| Orbit has repeating ground track maintained by station keeping | Yes or No | – |
| Administration is supplying specific node precession rate | Yes or No | – |
| Station keeping range for ascending node as half total range | Wdelta | degrees |

For each satellite the following parameters specified in § B2.1 would be used, where the definitions of the parameters are specified in § D6.3.1 at the time of the start of the simulation.

Note that in the Table below, the indices [N] are present to indicate that there would be a different value for each satellite, and the N-th value corresponds to the N-th satellite.

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Semi-major axis | A[N] | km |
| Eccentricity | E[N] | – |
| Inclination | I[N] | degrees |
| Longitude of ascending node | O[N] | degrees |
| Argument of perigee | W[N] | degrees |
| True anomaly | V[N] | degrees |

Each satellite must have an independent set of six orbital parameters for orbit definition and subsequent propagation.

To define the characteristics of non-GSO space stations, the following parameters, as specified in § B4.3 would be used:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| e.i.r.p. per space station by latitude | non-GSO_SS_e.i.r.p.[Lat][] | dB(W/BWrif) |
| Minimum transmit frequency(1) | IS_F | GHz |
| (1) The filing administration can supply a set of space station e.i.r.p. masks and associated frequency range for which the mask is valid. |  |  |

#### D5.3.4.2	GSO system parameters

The GSO system can either use worst-case parameters using the algorithm in § D5.2 or entered values. The required parameters are:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| GSO satellite longitude | GSO_SAT_LONG | degrees |
| GSO boresight latitude | BS_LAT | degrees |
| GSO boresight longitude | BS_LONG | degrees |
| GSO reference gain pattern | GSO_SAT_PATTERN | One of those in § D5.5 |

These parameters are defined in §§ D6.1 and D6.2.

#### D5.3.4.3	Run parameters

The following run parameters should be calculated using the algorithm in § D4:

| Parameter description | Parameter name | Parameter units |
| --- | --- | --- |
| Time step | TSTEP | s |
| Number of time steps | NSTEPS | – |

### D5.3.5	Algorithms and calculation procedures

In the calculation of the dual time step for the epfdis computation, Ncoarse = 1.

To calculate epfdis values from one non-GSO system into one GSO system satellite the following algorithm should be used. The algorithm can be used on multiple GSO systems in parallel if required:

Step 1:	Read in parameters for non-GSO system as specified in § D5.3.4.2.

Step 2:	Read in GSO parameters as specified in § D5.3.4.3.

Step 3:	If required calculate worst-case GSO location using algorithm in § D3.3.

Step 4:	Initialize statistics by zeroing all bins of epfdis values.

Step 5:	If required calculate number of time steps and time step size using algorithm in § D4 and hence calculate end time.

If a dual time step algorithm is included then use Sub-step 5.1, otherwise Ncoarse = 1 all the time.

Sub-step 5.1:	Calculate coarse step size Tcoarse = Tfine * Ncoarse.

Step 6:	Repeat Steps 7 to 19 for all time steps.

If a dual time step algorithm is included, then repeat Sub-step 6.1 to Step 17 until end time is reached.

Sub-step 6.1:	If it is the first-time step then set Tstep = Tfine.

Sub-step 6.2:	Otherwise if there are less than Ncoarse steps remaining then set Tstep = Tfine.

Sub-step 6.3:	Otherwise, if any of the α angles for the last time step were within coarse of the exclusion zone angle then set the Tstep = Tfine; otherwise, use Tstep = Tcoarse.

Step 7:	Update position and velocity vectors of all non-GSO satellites using algorithm in § D6.3.

Step 8:	Update position vectors of GSO satellite using algorithm in § D6.2.

Step 9:	Set epfdis = 0.

Step 10:	Repeat Steps 10 to 18 for all non-GSO space stations.

Step 11:	Determine if this non-GSO space station is visible to the GSO satellite using the algorithm in § D6.4.1.

Step 12:	If the non-GSO space station is visible to the GSO satellite then do Steps 13 to 18.

Step 13:	Calculate e.i.r.p. (dB(W/BWrif)) of non-GSO space station in direction of GSO satellite using the e.i.r.p. mask in § C3 for the non-GSO space station’s latitude.

Step 14:	Calculate GRX = receive relative gain (dB) at GSO satellite using relevant gain pattern specified in algorithms in § D6.5.

Step 15:	Calculate D = distance (km) between the non-GSO space station and the GSO satellite using the algorithm in § D6.4.1.

Step 16:	Calculate the spreading factor LFS = 10 log(4 D2) + 60.

Step 17:	Calculate epfdisi for this non-GSO satellite:

epfdisi = e.i.r.p. – LFS + GRX – Gmax

Step 18:	Increment epfdis by epfdisi.

Sub-step 19:	Increment epfdis statistics by this epfdis.

If a dual time step algorithm is included then the step below should be used:

Sub-step 19.1:	Increment epfdis statistics by the epfdis for this time step by Tstep/Tfine entries.

Step 20:	Generate the epfdis CDF from the epfdis pdf using the algorithm in § D7.1.2.

Step 21:	Compare epfdis statistics with limits using algorithm in § D7.1.

Step 22:	Output results in format specified in § D7.2.

### D5.3.6	Outputs

The result of the algorithm is two arrays in format:

| Array of Nepfd_IS epfdis values | epfd_IS_CALC[I] | dB(W/(m2 · BWrif)) |
| --- | --- | --- |
| Array of Nepfd_IS percentages | PC_CALC[I] | % |

where PC_CALC[I] is percentage of time epfd_IS_CALC[I] is exceeded.

# D6	Geometry and algorithms

This section describes the geometry that defines the core algorithms used in the software. One aspect is the conversion into a generic cartesian vector-based coordinate system. The precise orientation of the X vector is not specified in this Recommendation to allow alternative implementations by developers. The axis chosen should not impact the results as satellite and Earth coordinates are defined relative to the Earth.

To aid developers examples coordinate systems are used to show how to convert to and from generic vectors.

## D6.1	Earth coordinates system

Figure 46 shows the reference coordinate system for earth stations.

Figure 46

Definition of latitude

The Earth is defined as a sphere with radius as specified in § A2.2 = Re. The Earth rotates around an axis, the Z axis, at a rate defined in § A2.2 = e. Perpendicular to the Z axis, crossing the Earth at the Equator, is the XY plane.

Earth stations are located on this sphere based upon two angles:

Latitude: angle between line from centre of Earth to earth station and XY plane;

Longitude: angle as shown in Fig. 47.

Figure 47

Definition of longitude

Earth stations are assumed to have constant (latitude, longitude) positions in time.

The orientation within the XY plane of the X and Y axes is not specified in this Recommendation as all locations are referenced to the Earth rather than one particular inertial frame. This allows different implementations to use different reference points if required without impacting on the results.

One possible implementation is what is described as the geocentric inertial system. For this example case, conversion from geographic coordinates is achieved using:

Long=arccosxx2+y2        if y  0	(5)

Long=–arccosxx2+y2        if y  0	(6)

Lat=arctanzx2+y2	(7)

If this example coordinate system is used, then the conversion from geographic coordinates into geocentric inertial system coordinates is:

x = Re cos(lat) cos(long)	(8)

y = Re cos(lat) sin(long)	(9)

z = Re sin(lat)	(10)

where:

(x, y, z):	coordinates in the geocentric inertial system

long:	geographic longitude

lat: 	geographic latitude.

In this example geocentric inertial reference system, the equation for motion of a mass point on the Earth’s surface would be as:

xyz= Recoslat cos⁡(lon+ Ωet)Recoslat sin⁡(lon+ Ωet)Resinlat                              	(11)

where:

lat:	geographic latitude of the mass point on the Earth’s surface

lon:	geographic longitude of the mass point on the Earth’s surface

t:	time

e:	angular rate of rotation of the Earth.

## D6.2	GSO satellite coordinate system

The geostationary arc is a circle in the XY plane at a distance of Rgeo from the Earth’s centre where Rgeo is specified in § A2.2. Individual geostationary satellites have location on this circle defined by a longitude as shown in Fig. 48.

Figure 48

Definition of GSO satellite longitude

Geostationary satellites are assumed to have constant longitude in time. The conversion to and from vectors can use the same algorithms as in the section above by setting the latitude to zero.

## D6.3	Non-GSO satellite coordinate system

### D6.3.1	Non-GSO satellite orbit parameters

This section defines the parameters that specify an orbit for non-GSO satellites. Non-GSO satellites move in a plane as shown in Fig. 49.

Figure 49

Orbit plane angles

The plane of the orbit is referenced to the Earth by two angles:

:	longitude of ascending node: This defines where the ascending orbit plane intersects the equatorial plane. As the orbit is fixed in inertial space while the Earth rotates, a time reference for which this angle is valid must be given. In this case it is the start of the simulation

i:	inclination angle: This is defined as the angle between the plane of the orbit and the equatorial plane.

The orbit and position of the non-GSO satellite within the orbit is then defined by further parameters as shown in Fig. 50.

Figure 50

Definition of non-GSO satellite in-plane angles

The shape of the orbit is defined by:

a = (Ra + Rp)/2	(12)

e = (Ra – Rp) / (Ra + Rp)	(13)

where:

a:	semi-major axis

e:	eccentricity

Ra:	distance from the centre of the Earth to the satellite at apogee

Rp:	distance from the centre of the Earth to the satellite at perigee.

The position of the perigee within the orbit plane is defined by:

:	argument of perigee, angle between the line of the nodes and perigee.

The position of a non-GSO satellite within the plane at a particular time is defined by:

v0:	angle between perigee and specified point on orbit.

For circular orbits,  can be set to zero and v0 assumed to be the same as the argument of latitude defined as:

μ0=ω+υ0	(14)

Other useful terms are:

p=a1-e2	(15)

M=E-esinE	(16)

tan2= 1+e1 – etanE2	(17)

R= p1 +e cos() 	(18)

T=2πa3/μ	(19)

where:

p:	focal parameter

E:	eccentric anomaly

M:	mean anomaly

T:	period of orbit

R:	distance from centre of Earth to satellite when satellite is at position .

These can be used by the algorithm to predict the future position of the non-GSO satellite as described in § D5.

### D6.3.2	Non-GSO satellite orbit predictor

Given the orbital elements in the section above, standard orbit mechanics can be used to predict the position of the satellite at future times. In addition there are three additional precession factors for the ascending node and argument of perigee as described below.

Line of nodes

n= n01+ 32 J2Re2p21 – 32sin2i(1 – e2)1/2	(20)

where:

J2 = 	0.001082636

n0=μa3

Orbit precession in ascending node longitude

The rate of ascending node longitude secular drift is defined as:

Ωr= –32 J2Re2p2n cos⁡(i)	(21)

It follows from the above that polar orbits have zero precession rate and equatorial ones have a maximum precession rate. With direct satellite motion (i  90°) the ascending node shifts to the west (to  decreasing) and with reverse satellite motion (i  90°) it shifts to the east (to  increasing).

Perigee argument precession

Perigee argument secular shift rate is defined as:

ωr= 32 J2Re2p2n 2 – 52 sin2(i)	(22)

Perigee argument precession rate at i = 0 and i = 180 is maximum. For i1 = 63 26' 06'' or i2 = 116° 33' 54'' the precession rate is zero. If i  i1 or i  i2, then the perigee precession is along a satellite motion direction, and if i1  i  i2, then it is in the opposite direction.

Use of precession terms

Perigee argument is defined as:

ω = ω0 + ωrt	(23)

where:

0:	perigee argument at an initial moment

r:	perigee argument precession rate.

A current value of an ascending node longitude is defined as:

Ω = Ω0 + Ωrt	(24)

where:

0:	ascending node longitude at an initial moment

r:	ascending node longitude precession rate.

The revised orbit period is then:

TP=2πwr+n	(25)

The conversion to generic cartesian-based vector would depend upon the direction of the X vector. For an example coordinate system and for circular orbits, the satellite motion expression in the geocentric inertial reference system could be defined as:

xyz= R(cos+ωcosΩ–sin+ωsinΩcosi)R(cos+ωsinΩ+sin+ωcosΩcosi)Rsin+ωsini                                                           	(26)

And the velocity vector could be defined as:

xyz=xheRpsinν-hRcosΩsinω+ν+sinΩcosω+νcos⁡iyheRpsinν-hRsinΩsinω+ν-cosΩcosω+νcos⁡izheRpsinν+hRsinicosω+ν	(27)

h=μp

h: 	specific angular momentum

ECF (Earth Centered and Fixed) components of the position and velocity vectors could be defined as:

xyzECF=-xcosθ+ysinθxsinθ+ycosθz	(28)

xyzECF=xcosθ+ysinθ+ΩeyECF-xsinθ+ycosθ-ΩexECFz	(29)

where Ωe is a rotation rate of the earth 7,2921158553*10-5radsec and θ is the Greenwich hour angle of the Earth’s prime meridian, i.e. the angle between the inertial x axis and ECF x axis.

A satellite motion in an elliptical orbit is non-uniform; therefore, the Kepler expression and a concept of a mean anomaly would be used in the model to define the real anomaly as a function of time. Since an explicit dependence of the true anomaly on time is unavailable; the numerical methods of solving the below expressions were used for its definition. The expression is:

M= M0+ nt	(30)

Kepler’s equation M=E-esin(E) can be solved with Newton-Raphson method to obtain the true anomaly:

E0=M0= arccose+cos01+cos0	(31)

Ei+1=Ei-Ei-esin(Ei)-M1-ecosEi	(32)

### D6.3.3	Conversion to Generic Cartesian-based Vectors

The conversion to generic Cartesian-based vector would depend upon the direction of the X vector, but an approach based upon the X vector aligned with the direction in which the ascending node longitude is zero is as follows:

1.	For the relevant time t in seconds since simulation start, calculate the values of the processing terms (, , ) as required using the decision tree in § D6.3.5.

2.	From M calculate the eccentric anomaly E using equation (16) and iteration.

3.	From E calculate the true anomaly  using equation (17).

4.	Hence calculate the radius vector R using equation (18).

5.	Calculate the position of the satellite within the orbital plane in (P, Q) coordinates defined as in Fig. 51 below using:

pq0= RcosνRsinν0	(33)

6.	Create the rotation matrix from satellite orbit coordinates to inertial xyz coordinates using:

R= R11R12R13R21R22R23R31R32R33	(34)

where:

R11= cosΩcosω–sinΩsinωcosi	(35)

R12= -cosΩsinω–sinΩcosωcosi	(36)

R13= sinΩsini	(37)

R21= sinΩcosω+cosΩsinωcosi	(38)

R22= -sinΩsinω+cosΩcosωcosi	(39)

R23= -cosΩsini	(40)

R31= sinωsini	(41)

R32= cosωsini	(42)

R33= cosi	(43)

7.	Hence calculate the position of the satellite in xyz coordinates using:

xyz= Rpq0	(44)

Figure 51

Definition of satellite P, Q coordinates

### D6.3.4	Non-GSO satellite orbit station keeping

An important aspect to station keeping is to simulate multiple passes of the non-GSO satellite through an earth station’s main beam with slightly different crossing directions. As changing the position within the plane does not affect this, then the main parameter to vary is the longitude of the ascending node.

The approach proposed is to give range Wdelta in longitude of ascending node. At the start of the simulation all stations in the constellation have this parameter offset by Wdelta. During the simulation this field would increase to 0 (at the run’s mid-point) and then increase to Wdelta.

This is implemented by rotating the station’s position and velocity vectors around the Z axis by the required angle as specified in § D6.3.4.

### D6.3.5	Forced orbit precession

The standard orbit prediction algorithm is based upon point Earth mass plus correcting factors for J2 perturbations. There are two cases where this requires to be over-ridden:

a)	where administrations supply a detailed value of the orbit precession rate with respect to a point Earth mass to ensure a repeat ground track;

b)	for non-repeat orbits where an artificial precession rate is used to ensure the required spacing between equatorial passes.

### D6.3.6	Combining the orbit models

The various options for the orbit model can be combined in three ways as shown Fig. 52.

Figure 52

Flowchart of orbit model options

Note that the equatorial orbit i = 0 constellation is a special case in that there is no station keeping but the ground track of each satellite repeats just after one orbit. It should therefore be treated as Case (1) but with forced precession set to zero as described in § D4.

The three cases have their key orbit angles in radians updated as follows:

Case 1

ωt=w0+ωrt	(45)

Ωt=Ω0+Ωrt+π180Dartificialt	(46)

Mt=M0+nt	(47)

Case 2

ωt=w0+ωrt	(48)

Ωt=Ω0+Ωrt+π180.Wdelta2tTrun-1	(49)

Mt=M0+nt	(50)

Case 3

ωt=w0	(51)

Ωt=Ω0+π180Dadmint+π180.Wdelta2tTrun-1	(52)

Mt=M0+n0t	(53)

where:

r  = 	J2 precession of longitude of the ascending node in radians / second

r = 	J2 precession of the argument of perigee in radians / second

n = 	orbit motion including the J2 term in radians / second

n0 = 	orbit motion for point mass in radians / second

Dartificial = 	artificial precession in degrees / second

Dadmin = 	admin supplied precession in degrees / second

Wdelta = 	station keeping range in degrees

t = 	simulation time in seconds

Trun = 	total run time of the simulation in seconds.

### D6.3.7	Mapping orbit parameters from SRS data

The following orbit parameters are given in the SRS / IFIC database:

Table orbit:

–	Apogee height (km) = ha

–	Perigee height (km) = hp

–	Inclination angle (degrees) = INC

–	Right ascension (degrees) = RA

–	Longitude of ascending node (degrees) = LAN

–	Argument of perigee (degrees) = AP.

Table phase:

–	Phase angle (degrees) = PA.

For most of these fields it is possible to map nearly directly to the orbit parameters required, such as:

a= Re+ ha+ hp2

e= ha – hp2a

i = INC

= LAN

= AP

Note that this algorithm uses the ascending node definition based upon longitude rather than right ascension to ensure the orbit is correctly referenced to earth station longitude.

The last field to define is the true anomaly, , which can be derived from the phase angle. The phase angle is defined in RR Appendix 4 as:

| A.4.b.5.b: | the initial phase angle (i) of the i-th satellite in its orbital plane at reference time t = 0, measured from the point of the ascending node (0° ≤ i < 360°) |
| --- | --- |

The phase angle is shown in Fig. 53.

Figure 53

Definition of phase angle

The true anomaly can therefore be derived from the phase angle as follows:

0=PA – ω

or:

PA= ω+0=μ0

## D6.4	Geometry

### D6.4.1	Distance between two stations

Given two station’s position vectors in the form (x, y, z), the distance D between them can be calculated using:

D=x1-x22+y1-y22+z1-z22

### D6.4.2	Distance to Earth horizon

The distance to the horizon Dh for a station with Earth centred position vector of magnitude R can be calculated as follows:

Dh= R2 – Re2

### D6.4.3	Satellite visibility check

Two stations, whether earth stations or satellites, are visible if the direct distance between them is less than the sum of the distance to the horizon for each station, using the spherical Earth model described in § D6.1.

### D6.4.4	Angle to GSO arc and Longitude

#### D6.4.4.1	Definition

Figure 54 shows the definition of the  angle.

Figure 54

Definition of  angle

The Figure shows a test earth station and non-GSO satellite.

For each test point Pi on the GSO arc, there is a line from the earth station that intersects that point. There is then an angle i between that line and a line from the earth station to the non-GSO satellite.

The  angle is the minimum of all the test points for which the line does not intersect the Earth, i.e.

α=minα1

The sign of  is determined by whether the line from the earth station to the non-GSO satellite intersects the XY plane at a distance less than or greater than the GSO radius as follows:

Given:

Earth station position vector: 	RES

Non-GSO satellite position vector: 	RNS

Construct line:

R=RES+λREN

where:

This line crosses the XY plane when:

R(z) = 0

i.e. when

λz=0= –RES(z)REN(z)

Hence:

The sign of , X is then determined by:

If λz=0 < 0 then Rz=0 = Infinity

In the case when the ES is in the northern hemisphere:

If Rz=0 < Rgeo then  is positive

If Rz=0 = Rgeo then  is zero

If Rz=0 > Rgeo or if z=0 ≤ 0 then  is negative.

In the case when the ES is in the southern hemisphere:

If Rz=0 > Rgeo then  is positive

If Rz=0 = Rgeo then  is zero

If Rz=0 < Rgeo or if z=0  0 then  is negative.

In the case when the ES is at the equator:

is the negative of the sign of RENz

From the test point identified to give the  angle, the longitude can be calculated between the non-GSO sub-satellite point and the point on the GSO arc where the  angle is minimised, as shown in Fig. 55.

Figure 55

Definition of longitude

Hence:

Long = LongAlpha – LongNGSO

The  iteration should use test points that meet the requirements in § D1.4.

If there are two points on the GSO arc that give the same  (possibly the two edge of visibility points), then the one with the minimum absolute Long should be selected. If both have the same Long but with different signs, then the positive one should be used.

#### D6.4.4.2	Alpha search range

Note that the GSO arc will be visible to the non-GSO at a height of hnGSO and latitude latnGSO if the difference in longitude is less than:

cos∆long=cosxcoslatnGSO

where:

x=x1+x2

and:

cosx1=ReRgso

cosx2=ReRe+hnGSO

The range of the GSO arc that will be visible to an ES can be calculated in a similar manner by setting x2 = 0.

#### D6.4.4.3	Sign of Alpha

Figures 56 and 57 below are provided to give clarity as to the sign of  as seen from the perspective of a non-GSO satellite or non-GSO ES in the northern and southern hemispheres.

When the non-GSO satellite is in the northern hemisphere:

Figure 56

Sign of  as seen by non-GSO satellite in Northern Hemisphere

Figure 57

Sign of  as seen by non-GSO ES in Northern Hemisphere looking South

The geometry for the case when the non-GSO satellite is in the southern hemisphere is shown in Figs 58 and 59 below.

Figure 58

Sign of  as seen by non-GSO satellite in Southern Hemisphere

Figure 59

Sign of  as seen by non-GSO ES in Southern Hemisphere looking North

#### D6.4.4.4	Analytic method to calculate  and

This section describes an analytic method to calculate the alpha angle and also associated  angles.

Analytic calculation of

The analytic method to calculate  starts with the following two inputs:

P = Position vector of GSO ES, typically with magnitude = radius of the Earth = Re

N = Position vector of non-GSO satellite

These are defined as:

P=xpypzp

N=xnynzn

Then assume:

G = Position vector of a point on GSO arc specified by angle  and radius of geostationary orbit Rgeo:

G=RgeocosθRgeo sinθ0

Then  is the minimum over all  of the angle between the lines PN and PG where:

PN=xn-xpyn-ypzn-zp

PG=Rgeocosθ-xpRgeosinθ-yp-zp

This can be calculated using:

cosα=PN.PGPNPG

This will be minimised over  when:

ddθ=0

Which is also when:

ddθPN.PGPNPG=0

Writing this as:

ddθfg=0

Then f can be written as:

f=A+Bcosθ+Csinθ

where:

A=-xn-xpxp+yn-ypyp+zn-zpzp

B=xn-xpRgeo

C=yn-ypRgeo

And g can be written:

g=E+Fcosθ+Gsinθ

where:

E=Rgeo2+Re2

F=-2xpRgeo

G=-2ypRgeo

Note that D = magnitude of PN is not dependent upon  and so is a constant which need not be considered further.

using:

g=gr

and noting that:

ddθfg=0

when:

ddθfg2=0

i.e. when:

ddθf2gr=0

using f’ to represent f differentiated by , this can be expanded using standard methods as:

2ff'gr-gr'f2gr2=0

This can be simplified to:

2f'gr=gr'f

using:

x=sinθ

Note there is an alternative solution that uses cos  instead of sin .

Simplified using a new set of parameters {a, b, c, d, e} this is:

a+bx21-x2=c+dx+ex22

where:

a=AG-2CE

b=BF-CG

c=2CF-BG

d=AF-2BE

e=-BG-CF

This can be expanded and then re-arranged into:

a4x4+a3x3+a2x2+a1x+a0=0

where:

a4=e2+b2

a3=2de+2ab

a2=d2+2ce+a2-b2

a1=2cd-2ab

a0=c2-a2

This quartic equation can be solved using a number of techniques, including the use of the Newton-Raphson method. This approach aims to solve a function f of a variable x, using iteration, given an initial value and f’ = the derivative of f using:

xn+1=xn-fxnf'xn

where:

x=sinθ

fx= a4x4+a3x3+a2x2+a1x+a0

f'x= 4a4x3+3a3x2+2a2x+a1

The Newton-Raphson method is considered to have converged when the absolute difference between xn and xn+1 is less than 1e-6.

For the starting value, both x = +1 and x = −1 can be used and two solutions derived, matching the two cases of the line from the GSO ES position P to the non-GSO satellite N forward or backwards.

This analytic method does not take account of the range of visibility of the GSO arc as seen by the GSO ES. It is therefore necessary to identify the maximum range of the visible GSO arc, using:

cosθmax=ReRgeocosLatES

The points on the GSO arc that correspond to max can be considered additional possible solutions. The solution to use is one that is within the range −max to max that results in the smallest alpha angle.

The iterative method may be used as a fall-back should these methods fail to derive suitable solutions to the quartic equation but the preferred approach is analytic.

Analytic calculation of

The beta angle is defined in a similar way to that for  but is minimised over all possible positions of the non-GSO satellite with radius vector Rn and at latitude = lat but unknown longitude, for a given position of the GSO satellite.

The analytic method to calculate  starts with the following two inputs:

P = Position vector of GSO ES, typically with magnitude Rp = radius of the Earth = Re

G = Position vector of GSO satellite with magnitude Rg

These are defined as:

P=xpypzp

G=xgyg0

Then assume:

N = 	Position vector of non-GSO satellite at latitude = lat and angle  and radius of geostationary orbit Rn:

N=RncoslatcosθRn coslatsinθRnsinlat

Then  is the minimum over all  of the angle between the lines PN and PG where:

PN=Rncoslatcosθ-xpRn coslatsinθ-ypRnsinlat-zp

PG=xg-xpyg-yp-zp

This can be calculated using as similar approach above, with f and gr functions:

f=A+Bcosθ+Csinθ

gr=E+Fcosθ+Gsinθ

where:

A=RP2-xGxP+yGyP+zpRNsinlat

B=xG-xPRNcoslat

C=yG-yPRNcoslat

and:

E=RN2+RP2-2zPRNsinlat

F=-2xPRNcoslat

G=-2yPRNcoslat

Note that D = magnitude of PG is not dependent upon  and so is a constant which need not be considered further.

When the parameter set {A, B, C, E, F, G} has been defined then a similar methodology can be used to solve for  as for  above. There can be cases, such as its use in the worst case geometry algorithm, where it is not required to check for visibility.

#### D6.4.4.5	DeltaLongES

The difference in longitude for a non-GSO ES and a point on the GSO arc, as used in the definition of the ES_e.i.r.p. mask, is defined as follows:

DeltaLongES = Longitude(GSO point) – Longitude(Non-GSO ES)

### D6.4.5	Satellite and earth station azimuth and elevation

Figure 60 shows the definition of the azimuth and elevation angles used for the non-GSO satellite:

Figure 60

Definition of azimuth and elevation for non-GSO satellite

It should be noted that direction of the cartesian X, Y, Z vectors in this diagram are:

X:	+ve in the East direction from the non-GSO satellite

Y:	towards the centre of the Earth from the non-GSO satellite

Z:	+ve towards the North direction from the non-GSO satellite.

For the earth station, the definition of the azimuth and elevation angles are as in Fig. 61.

Figure 61

Definition of azimuth and elevation for earth station

It should be noted that direction of the cartesian X, Y, Z vectors in this diagram are:

X:	+ve in the East direction from the earth station in the horizontal plane

Y:	towards the North direction from the earth station in the horizontal plane

Z:	+ve towards the Zenith of the earth station perpendicular to the horizontal plane.

Furthermore, the normalized coordinates, u and v, can be computed from the azimuth and elevation angles as follows:

u=cosEl*cosAz

v=cosEl*sin(Az)

## D6.5	Gain patterns

This section defines the gain patterns used in the algorithms for earth stations and satellites. Note that all formula includes the peak gain, so where relative gain is required, the peak gain should be subtracted.

### D6.5.1	GSO earth station gain patterns

#### D6.5.1.1	FSS earth station gain pattern

The FSS earth station gain pattern to use is specified in Recommendation ITU-R S.1428.

#### D6.5.1.2	BSS earth station gain pattern

The BSS earth station gain pattern to use is specified in Recommendation ITU-R BO.1443.

### D6.5.2	GSO satellite gain pattern

The values of peak gain and half power beamwidth and the antenna reference radiation pattern to use are specified in RR Article 22 based upon Recommendation ITU-R S.672.

The peak gain to use in analysis should be selected using Table 16.

TABLE 16

Peak gain to use with Recommendation ITU-R S.672

| Half power beamwidth in Article 22 | Peak gain to use in analysis |
| --- | --- |
| 1.5 | 41.0 dBi |
| 1.55 | 40.7 dBi |
| 4 | 32.4 dBi |

# D7	Structure and format of results

## D7.1	Go/No-go decision

### D7.1.1	Overall description of the decision process

The simulation produces a probability distribution function (PDF) of the pfd. The PDF records, for each pfd level, the number of simulation time-steps at which that pfd level occurred divided by the sum of all bins. The PDF should be converted into a cumulative distribution function (CDF) which records for each pfd level the number of simulation time-steps at which that pfd level was exceeded normalized by the total number of simulation time steps.

Note that the term cumulative distribution function is meant to include the concept of the complementary cumulative distribution function depending upon context.

### D7.1.2	Production of the CDF

The process detailed in § D5 generated a PDF of the pfd values. This PDF should be converted into a CDF which records for each pfd level an estimate of the percentage of time during which that pfd level is exceeded.

For each pfd value, the CDF should be calculated by:

CDFi = 100 (1 – SUM (PDFmin: PDFi))

where:

PDFx:	PDF table entry for a pfd value of X dB, normalized so that the total sum for all PDFx is 1.

### D7.1.3	Comparison procedure

The next stage is the comparison of the pfd limit values in the RR with those in the probability table.

Step 1:	Perform Steps 2 through 4 for each specification limit i.

Step 2:	Read the pfd value/probability pair (Ji and Pi) from the database.

Step 3:	If the pfd value Ji has a higher precision than SB (currently 0.1 dB) round Ji to a lower value with a maximum precision of 0.1 dB.

Step 4:	From the CDF find Pt, the probability that pfd value Ji was exceeded as obtained by the software.

Step 5:	If Pi  Pt then record Pass: the CDF complies with this specification point. Else record Fail: the CDF does not comply with this specification point.

The final stage is the comparison of the maximum pfd value recorded during the software run with the limit specified for 100% time (if any).

From the CDF, find the maximum pfd value recorded during the software run, Jmax. Compare it with the pfd limit specified for 100% time, J100. If Jmax  J100 then record Pass: the CDF complies with this specification point. If Jmax  J100 then record Fail: the CDF does not comply with this specification point.

### D7.1.4	Decision process

If a Pass result was recorded for all of the specification limits, then the non-GSO network complies with the specification. If any Fail results were recorded, then the non-GSO network does not comply with the specification.

## D7.2	Background information to decision

The background information required is:

–	pfd data generated in the software run (including antenna diameter) and reference antenna pattern;

–	table of specification limits for various antenna diameters and reference antenna pattern.

## D7.3	Format for output

The output format should be:

–	statement of the result of the test;

–	summary table;

–	CDF table (for information only).

### D7.3.1	Statement of the result of the compliance test

The overall conclusion of the evaluation (Pass or Fail) as defined in § D7.1.4 should be output.

### D7.3.2	Summary table

The summary table should show the following data (see Table 17):

TABLE 17

Summary table

| Specification point |  | Result | Simulation point |
| --- | --- | --- | --- |
| pfd value | Probability |  | Probability |
| J1 dB(W/(m2 · BWref)) | P1 | Pass/fail | Py |
| : | : | : | : |
| Ji dB(W/(m2 · BWref)) | Pi | Pass/fail | Py |
| where: Ji and Pi: pfd value/probability specification values from the database Pass/fail: test result Py: probability value from the probability table. |  |  |  |

### D7.3.3	Probability table

The output should include for information the calculated CDF which was used in the decision-making process.

PART E

Testing of the reliability of the software outputs

# E1	Evaluation of the computation accuracy of the candidate software

These tests could be performed by the software developer, and the results supplied to the BR along with the candidate software.

Software functions to be evaluated:

Orbit projection – Using a set of simplified parameters which result in a defined repeat period, run the software for the required simulation interval and check the actual (satellite vectors) against the predicted values.

Offset angles – Using appropriate sets of earth station and satellite locations, check the actual beam offset angle values against the predicted values. The sets of test data should cover the most complex trigonometrical cases: for example sites around longitude zero and longitude 180°.

Time step and simulation duration – Using appropriate sets of non-GSO network parameters, check the time step and simulation duration values generated by the software against the predicted values.

CDF generation – Using sets of test input files with known CDF results, verify the CDF generation software.

Go/no-go decision process – Using sets of CDF test input files, verify the accuracy of the go/no-go decision process.

Should multiple implementations be available then sensitivity analysis could be used to evaluate them, and their output can be compared to ensure consistency.

# E2	Evaluation of the epfd(↓/↑) statistics obtained by the BR

These are tests which will be performed automatically by the software as part of each run, to confirm that the run did find the worst-case interference events.

epfd value for 100% time – the epfd↓ value for 100% time obtained during the run should be compared with a value calculated from analysis of the non-GSO constellation. The obtained value should be within ±0.X dB of the expected value.

# E3	Verification of the pfd masks

The pfd masks are inputs to the BR validation tool to be provided by the notifying administration to the BR together with the software used for its calculation, the complete software description and parameters. The information required to generate the pfd mask could be made available to interested administrations to be used in case of dispute.

# E4	Re-testing of the BR software after any modifications or upgrades

A set of tests should be defined for use on any occasions when the BR software or its operating environment has been modified or upgraded. Such tests could include:

a)	some or all of the tests defined in § E1 for the initial evaluation of the computational accuracy of the candidate software;

b)	repetition of a representative set of evaluations of actual non-GSO filings, and comparison of the results obtained by the original and modified software systems.

PART F

Software implementing this Recommendation

# F1	Operating system

The software should run on Microsoft platforms under the Windows 10 or higher operating system.

# F2	Interfaces to existing software and databases

The BR captures all incoming notices related to space services into one central database for alphanumeric data (SNS) and into another database for graphical data (GIMS) such as antenna diagrams and service areas. These databases are used for the publication of the data on DVD, in the Weekly Circular and its Special Sections. They are also used to provide input data to software packages performing RR Appendix 8 and pfd examinations. Graphical Interface for Batch Calculations (GIBC) is used to carry out examination using these different modules. This guarantees that the data published are also the data used in these examinations. The BR considers this important for both the notifying administration and for administrations the services of which may be affected by the new station. This is not only for the convenience of the BR, but to ensure consistency and transparency towards administrations.

# F3	User manual

The purpose of the user manual is to tell the user how to run different tests to obtain certain results. Given the complexity of these tests, they need to be given in detail.

https://en.wikipedia.org/wiki/Newton%27s_method