RECOMMENDATION  ITU-R  S.1715

Guidelines developed in response to the studies requested
in Resolution 140 (WRC-03)

(2005)

The ITU Radiocommunication Assembly,

considering

a)	that Article 22 of the Radio Regulations (RR) contains epfd limits on non-GSO FSS systems which, if met, protect GSO FSS networks from unacceptable interference;

b)	that in the development of RR Article 22 epfd limits for the 20 GHz FSS band, different considerations were made for the lower portion of the band (17.8-18.6 GHz) as compared to the upper portion (i.e. 19.7-20.2 GHz) of the band;

c)	that the temporal characteristics of the interference caused to GSO FSS networks by certain types of non-GSO FSS systems may be significantly different than those caused by other types of non-GSO FSS systems;

d)	that any non-GSO FSS use of the band 19.7-20.2 GHz is subject to RR No. 5.484A;

e)	that protection of GSO FSS networks in the band 19.7-20.2 GHz from unacceptable interference may be able to be provided by a certain type of non-GSO FSS1 system that does not meet the epfd limits in RR No. 22.5C (Table 22-1C) subject to the agreement of the affected administration(s) under RR No. 22.5CA;

f)	that Resolution 140 (WRC-03) invited ITU-R to develop “criteria which would protect GSO FSS networks in the band 19.7-20.2 GHz from unacceptable interference by HEO FSS systems taking account of the combined effect of interference into downlinks of the GSO FSS networks from HEO FSS systems and other non-GSO FSS systems”;

g)	that Resolution 140 (WRC-03) invited administrations “to consider using the relevant ITU-R Recommendations regarding the protection of GSO FSS satellite networks from interference by non-GSO FSS systems as a guideline for consultation between administrations, to fulfil their obligations under RR No. 22.2 in the band 19.7-20.2 GHz, and in the case where an administration responsible for a non-GSO FSS system requests the application of RR No. 22.5CA”,

noting

a)	that GSO FSS networks with specific receive earth stations having very large antennas and non-GSO FSS systems operating in the 19.7-20.2 GHz frequency band are subject to coordination under RR No. 9.7A and No. 9.7B, as specified in Table 5-1 of RR Appendix 5;

b)	that in order to provide the required protection to GSO FSS networks, the long-term portion of the epfd limits in RR Article 22 in the 19.7-20.2 GHz band is about 15 dB more stringent than the corresponding limit in the 17.8-18.6 GHz band;

c)	that the effect on co-frequency sharing between non-GSO FSS systems of having some non-GSO FSS systems meet the RR Article 22 epfd limits and having other systems meet the levels included in this Recommendation has not yet been evaluated,

recommends

1	that the following guidelines could be applied in the event that an administration planning a certain type of non-GSO FSS system for operation in the 19.7-20.2 GHz band, which system does not meet the epfd limits specified in RR No. 22.5C (Table 22-1C) for this band, requests the application of RR No. 22.5CA in connection with Resolution 140 (WRC-03) with respect to its system:

–	that any such system should provide protection to GSO FSS networks equivalent to that provided by the single entry epfd limits specified in Table 22-1C of RR No. 22.5C for this band;

–	that the set of epfd masks shown in Table 1, which has been derived for a certain type of non-GSO FSS system that does not otherwise meet the epfd limits in the 19.7-20.2 GHz band based on the objective of providing, to GSO FSS networks, protection equivalent to that provided by the epfd masks contained in RR No. 22.5C (Table 22-1C), may be taken into consideration by an administration receiving such a request for application of RR No. 22.5CA, subject to its agreement, in any discussions concerning the possible application of RR No. 22.5CA;

2	that administrations take note that, if they agree to permit a non-GSO FSS system to operate at the epfd levels of Table 1 on their territory, they are also agreeing (see Note 1), under resolves 2 of Resolution 76 (WRC-2000), to a level of aggregate epfd on their territory caused by all non-GSO systems (including those indicated in recommends 1) that is higher than the corresponding level in Table 1C of Resolution 76 (WRC-2000) by an amount which accommodates the higher epfd levels in Table 1 in addition to the epfd caused by systems operating in compliance with the single entry epfd limits in Table 22-1C of RR No. 22.5C;

3	that the following Notes be considered as part of this Recommendation.

NOTE 1 – The single entry epfd levels in Table 1 are higher than the aggregate long-term limits in Table 1C of Resolution 76 (WRC-03).

NOTE 2 – This alternative Table is not meant to be considered in conjunction with the epfd limits of RR Article 22, but instead is to be used on its own in situations where a non-GSO FSS system does not meet the epfd limits and application of RR No. 22.5CA is requested.

NOTE 3 – Administrations operating or planning to operate non-GSO FSS systems that would rely on RR No. 22.5CA and this Recommendation for use of the 19.7-20.2 GHz band should not request protection or accommodation in intersystem coordination with other co-frequency non-GSO FSS systems that do not rely on RR No. 22.5CA at levels beyond those which a system that meets the applicable limits could request.

TABLE 1

An illustrative example of the single-entry epfd radiated by a certain type 
of non-GSO satellite system in the FSS
in the frequency band 19.7-20.2 GHz

| Frequency band (GHz) | epfd (dB(W/m2)) | Percentage of time during which epfd may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter and reference radiation pattern (1) |
| --- | --- | --- | --- | --- |
| 19.7-20.2 | –172.4 | 100 | 40 | 70 cm Recommendation ITU-R S.1428 |
|  | –158.4 | 100 | 1 000 |  |
|  | –174.0 | 100 | 40 | 90 cm Recommendation ITU-R S.1428 |
|  | –160.0 | 100 | 1 000 |  |
|  | –182.8 | 100 | 40 | 2.5 m Recommendation ITU-R S.1428 |
|  | –168.8 | 100 | 1 000 |  |
|  | –187.6 | 100 | 40 | 5 m Recommendation ITU-R S.1428 |
|  | –173.6 | 100 | 1 000 |  |
| (1) For this Table, reference patterns of Recommendation ITU-R S.1428 should be used only for the calculation of interference from non-GSO satellite systems in the FSS into GSO satellite networks in the FSS. |  |  |  |  |

1	Non-GSO FSS systems with an inclination between 35° and 145°, an apogee greater than 18 000 km, operating in a limited active arc, and not meeting the epfd limits in RR No. 22.5C (Table 22-1C).