RECOMMENDATION  ITU-R  S.1844

Cross-polarization reference gain pattern for linearly polarized very small
aperture terminals (VSAT) for frequencies in the range 2 to 31 GHz

(2009)

Scope

This Recommendation provides a cross-polarization reference gain pattern that, in the absence of particular information concerning the cross-polarization radiation pattern of a linearly polarized very small aperture terminal (VSAT) earth station antenna, should be used as a reference for interference calculations involving VSAT earth stations in the fixed-satellite service and stations of other services sharing the same frequency band, as well as coordination studies and interference assessment between systems in the fixed-satellite service.

The ITU Radiocommunication Assembly,

considering

a)	that, for studies of mutual interference between radiocommunication-satellite systems and between earth stations of such systems and stations of other services sharing the same frequency band, it may be necessary to use a single cross-polarization radiation pattern for the very small aperture terminal (VSAT) earth station antenna;

b)	that, for coordination studies and for the assessment of interference between VSAT earth stations and space stations, a cross-polarization radiation pattern for the angular region near the main beam based on the envelope of the peak power of the side lobes in this region may be appropriate;

c)	that the use of antennas with the best achievable cross-polarization radiation patterns will lead to the most efficient use of the radio-frequency spectrum and the geostationary-satellite orbit,

noting

a)	that increased utilization of the orbital arc has resulted in the widespread use of very small aperture antennas,

recommends

1	that, in the absence of particular information concerning the radiation pattern of the linearly polarized very small aperture terminal antenna for the earth station involved, a single reference radiation pattern should be used for coordination and interference studies involving linearly polarized VSAT earth stations in the fixed-satellite service and stations of other services sharing the same frequency band;

2	that the following cross-polarization reference gain pattern should be used for linearly polarized VSAT antennas with a diameter-to-wavelength (D/λ) less than 100, for different angles between the directions considered and the axis of the main beam (boresight direction), for frequencies in the range 2 to 31 GHz:

Gx()  =  Gmax – 25	dBi	for		      0.3

Gx()  =  Gmax – 20	dBi	for	0.3	      20

Gx()  =  Gmax – 0.0025Dλϕ2	dBi	for	20	      SXP

Gx()  =  23 – 20 log 	dBi	for	SXP	      7°

Gx()  =  20.2 – 16.7 log 	dBi	for	7°	      26.3°

Gx()  =  32 – 25 log 	dBi	for	6	      48°

Gx()  =  –10	dBi	for		      180°

where:

Gmax: 	antenna maximum gain (in the boresight direction) = 10 logηπ2Dλ2

D:	antenna diameter (m)

λ:	wavelength (m)

η:	efficiency as a fraction

0.3:	angular limit of the –0.3 dB co-polar beamwidth = 10.95 /D

20:	angular limit of the –20 dB co-polar beamwidth = 89.44 /D

SXP:	angle of intersection between:

F1() = Gmax – 0.0025Dλϕ2and F2() = 23 – 20 log 

and can be approximated by the following expression:

ϕSXP=101Dλ−0.99