Recommendation ITU-R S.2029 (12/2012)

Statistical methodology to assess time-varying interference produced by a geostationary fixed-satellite service network of earth stations operating with MF-TDMA schemes to geostationary fixed-satellite service networks

S Series Fixed-satellite service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2012

ITU 2012

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  S.2029

Statistical methodology to assess time-varying interference produced by a geostationary fixed-satellite service network of earth stations
operating with MF-TDMA schemes to geostationary
fixed-satellite service networks

(Question ITU-R 208/4)

(2012)

Scope

This Recommendation provides a statistical methodology to assess time-varying interference resulting from a geostationary network of earth stations, operating with MF-TDMA schemes, over a geostationary-satellite orbit fixed-satellite service network. The methodology considers the potential interference to another GSO FSS network. Furthermore, the methodology can be used to adjust the power levels of the interfering terminals such that the performance objectives of the interfered-with satellite network are not impacted.

The ITU Radiocommunication Assembly,

considering

a)	that FSS GSO satellites are well suited to provide broadband communication applications including Internet and data services;

b)	that satellite networks use a variety of network topologies and multiple access schemes, including the multi-frequency time division multiple access (MF-TDMA) scheme;

c)	that through the use of efficient modulation and coding, higher satellite e.i.r.p. levels, and other techniques, some networks can support full mesh (point-to-point) connectivity with small aperture terminals;

d)	that it is necessary to protect networks of the FSS from any potential interference from these terminals;

e)	that it would be useful to have methodologies to assess the time-varying interference from a GSO FSS network to another GSO FSS network;

f)	that it would be useful to have methodologies for assessing interference levels to satellite networks resulting from earth stations operating with MF-TDMA schemes;

g)	that many of the technical characteristics of these networks which affect performance and orbit spectrum/utilization have time-varying characteristics which are best modelled by stochastic processes,

noting

a)	that maximum permissible levels of inter-network interference from GSO networks to GSO/FSS networks operating in the same frequency band are provided in Recommendation ITU-R S.1323;

b)	that maximum permissible levels of inter-network interference and the methodology for determining this interference, from non-GSO systems and to GSO/FSS networks operating in the same frequency band are provided in Recommendation ITU-R S.1323;

c)	that time invariant interference is typically estimated using the T /T method, described in Recommendation ITU-R S.738;

d)	that methodologies to estimate the off-axis e.i.r.p. density levels and to assess the time-varying interference towards adjacent satellites resulting from pointing errors of a vehicle mounted earth station are provided in Recommendation ITU-R S.1857,

recommends

1	that the methodology given in the Annex should be used to assess the time-varying interference due to multiple earth stations operating in a MF-TDMA scheme;

2	that the methodology provided should be used to determine the off-axis emission levels of the interfering earth stations such that these satisfy the performance objectives of the interfered-with satellite network;

3	that the methodology provided should be used so that the type of MF-TDMA networks described in this Recommendation would not create interference to other FSS networks operating in the same frequency bands beyond the level accepted by administrations;

4	that the following Notes should be regarded as part of this Recommendation.

NOTE 1 – The methodology given in the Annex provides a statistical approach to assess the potential interference impacts of an MF-TDMA network to a neighbouring co-frequency GSO FSS network.

NOTE 2 − The parameters and the examples provided in the Annex represent a hypothetical system that operates in the 20/30 GHz frequency band. However, the methodology may also be used for other frequency bands after appropriate modifications of some parameters.

NOTE 3 – The methodology of this Recommendation does not apply to networks operating with code division multiple access (CDMA) schemes.

NOTE 4 – To verify that the mathematical model described in the methodology truly represents the time-varying characteristics of an MF-TDMA network, it may be useful to obtain the statistical characteristics of operational networks.

NOTE 5 – The apportionment of short-term interference for the MF-TDMA GSO/FSS networks considered in this Recommendation may be mutually agreed through the process of coordination.

NOTE 6 – The time allowance and the short-term interference criteria for GSO/FSS networks may be a subject for further study.

Annex

Statistical methodology to assess time-varying interference produced by a geostationary fixed-satellite service network of earth stations
operating with MF-TDMA schemes to geostationary
fixed-satellite service networks

# 1	Introduction

In recent years, the demand for satellite-based two-way Internet services has increased significantly. These services, especially for residential and small business users, are provided using small aperture satellite terminals. Typically, a single satellite network may consist of a large number of small aperture terminals deployed over a wide geographical area. According to the location within the satellite footprint, the varying weather conditions, and the user’s data rates, these terminals may operate on a range of aperture sizes and may require different transmit power levels. To utilize network resources efficiently, these networks may employ time- and frequency-division multiple access methods. A particular characteristic of small aperture terminals is that they have large antenna beamwidths and hence may produce uplink interference towards adjacent satellites if the transmit power levels are not adjusted properly. Additionally, some small terminals mounted on air/sea vessels, trains, or ground vehicles as well as stationary terminals may produce antenna pointing errors which may result in potential interference that must be mitigated. These combined effects contribute to a time-varying interference pattern from the network of terminals to a victim receiver in another satellite network.

This Annex presents a statistical approach to determine the interference to a GSO network from another GSO network consisting of multiple terminals operating using a time-division multiple access scheme and antenna pointing errors. The Annex discusses a long-term interference criterion and criteria to satisfy the short-term performance objectives, it provides some examples to illustrate the impacts on the neighbouring satellite network, and it presents a step-by-step approach on how to compute the resulting interference. The methodology presented may be useful to determine the off-axis emission levels of the interfering terminals such that these satisfy the short-term and long-term performance objectives of the victim satellite system.

# 2	Long- and short-term components of interference

The interference signal at the victim receiver consists of signals that belong to a large number of transmit terminals from a single interfering network that operate using a time-division multiple access protocol. The terminals may employ antenna apertures of different sizes and may transmit at different power levels depending on their location within the footprint of the satellite beam. Additionally, these terminals may have small antenna pointing errors. Therefore, when the observation interval is sufficiently large to contain transmissions from several interfering terminals, the interference level at the victim receiver is time-varying.

In such cases, for illustrative purposes, the interference signal at the victim receiver, Itot, may be expressed as the sum of a long-term interference component, Ilong, and a short-term interference component, Ishort, so that Itot=Ilong+Ishort. The long-term interference component is constant over short time intervals but it may exhibit small variations when observed over long time intervals (of the order of several minutes). These variations are statistical in nature resulting from the slow changing characteristics of the transmit signals. On the other hand, the short-term interference component is due to the transmissions from different terminal types and may vary over very short time intervals, for example over fractions of a second. Note that this short-term and long-term interference components are used only for illustrative purposes; interference analysis is carried out for the total interference.

Figure 1 shows the interference levels at the victim receiver due to transmissions from Terminals T1, T2, T3, T4 and T5. In general, as shown in this figure, the interference levels and the transmission durations depend on the particular terminal. The long-term component shown here represents the average level of the interference and the short-term interference component is given by the difference between the total interference and this long-term interference component.

To quantify and limit the effects of interference, this Annex gives methods for assessing and limiting the long-term interference, short-term interference, and total interference. Specifically, long-term interference and the criteria to satisfy the short-term performance objectives are given to limit the effects of interference at the victim receiver.

Figure 1

Interference observed at the victim receiver due to time-division multiple access transmissions
from Terminals T1, T2, T3, T4 and T5

# 3	Long-term interference criterion

Assessment of time invariant interference is typically carried out using the ΔT/T method, for example as described in Recommendation ITU-R S.738. In order to make use of a similar approach consider the hypothetical situation when the interference level at the victim receiver is not time-varying: that is the e.i.r.p. density levels at the terminals are adjusted so that the interference level seen at the victim receiver is given by long-term interference level, Ilong. Also, in this case the terminals do not produce antenna pointing errors. The long-term interference criterion in this case is expressed in terms of the ΔT/T ratio as:

(1)

where Ilong is defined as the ensemble-averaged interference power spectral density computed in a bandwidth of Wlong, k is the Boltzmann constant, Θv is the victim receiver noise temperature referred to the output of its antenna, Θvs is the noise temperature at the receiver of the victim satellite referred to the output of its antenna, and γv is the transmission gain from the output of the victim satellite’s input antenna to the output of the victim receiver’s antenna.

Clearly, when the ensemble-averaged power of the interference is a constant, the interference power-to-noise ratio considered in this hypothetical case is time invariant.

However, in practice, rather than the ensemble-average value the time-averaged value of the long-term interference component is generally available. This time average value may exhibit small variations when computed over different time intervals. This time-averaged value, Ilong, when computed over the long-term time interval of duration Tlong may exhibit variations because the underlying interference is a statistical process. Additionally, statistical characteristics of the terminals may change during this interval resulting in small variations about this average value. These variations can be limited by imposing constraints on the cumulative distribution function (CDF) of the variable as follows:

(2)

where , and Tlong are system parameters.

For illustrative purposes, Recommendation ITU-R S.523-4 specifies an averaging interval of 10 min for computing the interference to PCM encoded Telephony systems. Moreover, Annex 1 of Recommendation ITU-R S.1432-1 specifies the maximum levels of interference-to-noise ratio (I / N) that may be exceeded in any month as: (I / N) > 0 dB for 0.005% of any month; (I / N) > −2.4 dB for 0.03% of any month; (I / N) > −10 dB for 20% of any month; and (I / N) > −12 dB for 100% of any month.

# 4	Criteria to satisfy short-term performance objectives

In the preceding section, limits were imposed on the long-term interference. In this section, a criterion is presented to limit the total interference so that it meets the short-term performance objectives of the victim receiver. The total interference may exhibit variations over a few milliseconds. According to Recommendation ITU-R S.1323-2, victim links should contain sufficient link margins to overcome degradations due to the combined effects of propagation and time-varying interference. Degradations due to propagation effects should not account for more than 90% of the time allocated in the short-term performance objectives. Additionally, this Recommendation states that time-varying interference “should be responsible for at most 10% of the time allowance for the BER (or C/N value) specified in the short-term performance objectives of the desired network and corresponding to the shortest percentage of time (lowest C/N value).” In this Annex, a criterion similar to the above is presented to derive an acceptable limit for the short-term interference.

The short-term performance objectives are usually stated in terms of the bit-error-rate (BER) levels or the carrier-to-noise (C/N) ratio levels that may not be degraded for a specified time period. For example, for a given C/N ratio and time percentage pairs, C/Nj,pj%, j=1,2,…, J, the C/N ratio may be less than C/Nj for only pj% of the time of any month. Similar to considering o) in Recommendation ITU-R S.1323-2, let us consider propagation effects to result in degradation of the link for at most 1-pshort/100×pj%, j=1,2,…,J, of the time of any month where pshort represents the fraction of the time allowance specified in the short-term performance objective allocated to short-term interference (for example, pshort=10). Noting that the long-term interference is limited as proposed in the previous section, the proposed criteria to limit the short-term interference and comply with the short-term performance objectives are expressed as:

a)	In the presence of propagation effects and the long-term interference, the C/N ratio should not be less than C/Nj for more than 1-pshort/100×pj%, j=1,2,…,J, (for example, pshort=10) of the time of any month.

b)	In the presence of short-term interference, the C/N ratio should not be less than C/Nj for more than pshort/100×pj% of the time of any month, where j corresponds to the smallest C/Nj value.

c)	In the presence of propagation effects and total interference, the C/N ratio should not be less than C/Nj for more than pj%, j=1,2,…,J, of the time of any month.

Note that in order to comply with the above conditions, the victim link should contain a sufficient link margin to satisfy condition a) and the e.i.r.p. density level of the interferer should be limited to satisfy conditions b) and c). Also note that according to condition c), the combined impact of propagation degradation and the total interference is such that C/N ratio still meets the short-term performance objective.

## 4.1	Expressing the C/N ratios

The C/N ratio under clear-sky conditions, considering the long-term interference component can be expressed as:

where Ccs is the carrier power under clear-sky conditions, Ncs noise power at the victim receiver under clear-sky conditions and Ilong is the long-term interference power component under clear-sky conditions.

Next, consider the C/N ratio under rain fading conditions. The rain attenuation factors in the uplink and downlink of the victim signal link are denoted by A↑ and A↓, respectively. The carrier power at the victim receiver under these conditions will be attenuated by the factor FA↑,A↓ and is denoted by CcsFA↑,A↓. The noise power at the receiver is denoted by the function NNcs,A↓. This function includes the sky noise due to rain and the noise components from the wanted and adjacent satellites. Note that the attenuation factors due to rain in the downlink from adjacent satellites may not be the same as A↓. In such cases this noise function should take into account these different rain attenuation factors. Under rain fading conditions the long-term interference component is denoted by IIlong,A↓,A↑,i where A↑,i is the uplink rain attenuation factor from the interfering terminal to the wanted satellite. Note that when the uplink rain attenuation factors are not the same for the different interfering terminals, these different rain attenuation factors should be considered in this expression. Moreover, when the downlink rain attenuation components from the adjacent and wanted satellites are not the same, the different downlink rain attenuation factors from the adjacent satellites should be taken into account. Combining these, the C/N ratio under rain fading conditions and the long-term interference component is expressed as:

(3)

Finally, consider the rain attenuation in the presence of the total interference, Itot. The C/N ratio can be expressed as:

(4)

where IItot,A↓,A↑,i is the total interference in the presence of rain.

## 4.2	Expressing the short-term performance objective criterion

In this subsection, the criteria to satisfy the short-term performance objectives stated in § 4 are expressed in the terms of the degradations of the C/N ratio. The Criterion a) above can be expressed as:

For analysis purposes, it is convenient to consider the degradation of the C/N ratio with respect to its clear-sky values. Denote the degradations in the presence of the long-term interference component and the total interference as Zs=C/NcsC/Ns and Zt=C/NcsC/Nt, respectively. Also, define the variables Zj=C/NcsC/Nj, j=1,2,…,J. Then the above can be expressed in the following equivalent form:

(5)

Similarly, the Criterion c) can be expressed as:

(6)

Finally, the Criterion b) can be satisfied when the victim link is designed such that the propagation effects make use of the maximum allocated time for the smallest specified C/N ratio level, which is denoted by C/Njm. Hence, Criterion b) can be expressed as:

(7)

# 5	List of parameters and notation

This section contains a list of parameters and the notation employed in this Annex.

λu, λd (m):	wavelengths in the uplink and downlink directions, respectively

ϕr (degrees):	antenna pointing error at Tr: angle between the actual and desired directions of antenna boresight.

ϕr,ε, ϕr,a (degrees):	antenna pointing error in the elevation and azimuth directions at Tr: difference between actual and desired values of the elevation and azimuth angles.

(degrees):	off-axis angle at Ti measured from its boresight direction.

ψr,x, x=i,v (degrees):	angle at Tr between its boresight direction and direction to Sx.

ψv,i (degrees):	angle at Rv between its boresight direction and direction to Si.

δv,x, x=i,v (degrees):	angle at receive antenna of Sv between its boresight direction and direction to Tx.

δi,r (degrees):	angle at receive antenna of Si between its boresight direction and direction to Tr.

ηx, x=i,v (degrees):	angle at transmit antenna of Sx between its boresight direction and direction to Rv.

γx, x=i,v:	transmission gain of satellite downlink measured from receive antenna output of Sx to receive antenna output of Rv.

θspace (degrees): 	orbital separation between Satellites Sv to Si.

Θr (K): 	sky noise temperature due to rain at Rv referred to the output of its receive antenna.

Θv (K): 	system noise temperature at Rv referred to the output of its receive antenna.

Θxs, x=i,v (K):	system noise temperature at Sx referred to the output of its receive antenna.

A↓: 	attenuation factor due to rain in the downlink from Sv to Rv.

A↑: 	attenuation factor due to rain in the uplink from Tv to Sv.

A↑,i: 	attenuation factor due to rain in the uplink from Tr to Sv.

Bx , x=r,v (W/Hz): 	boresight e.i.r.p. density at Tx.

Bxs , x=r,v (W/Hz): 	boresight e.i.r.p. density at Sx.

C (W/Hz): 	carrier power spectral density under clear-sky conditions at receive antenna output of Rv.

Ccs (W): 	carrier power under clear-sky conditions at receive antenna output of Rv.

Ci, Cv: 	receive beam centres of Si and Sv on the Earth surface.

C/Nj :	(C/N) ratio specified in the short-term objectives. The C/N ratio should not be less than pj% of time.

C/Ncs :	(C/N) ratio at victim receiver under clear-sky conditions and in the presence of the long-term interference.

C/Ns :	(C/N) ratio at victim receiver under rain-fading conditions and in the presence of the long-term interference.

C/Nt :	(C/N) ratio at victim receiver under rain-fading conditions and in the presence of total interference.

E (W/Hz): 	off-axis e.i.r.p. density pattern at Tr.

EIRP() (W/Hz): 	e.i.r.p. density in off-axis direction  .

Gt,r :	normalized transmit antenna gain at Tr. Gt,r0= 1 .

Gr,v : 	receive antenna gain at Rv.

Gr,is,Gr,vs:	receive antenna gain patterns at Si and Sv, respectively.

Gt,i,s, Gt,vs:	normalized transmit antenna gain at Si and Sv, respectively.

Gt,is0= Gt,vs0= 1 .

Iavg (W/Hz): 	ensemble-averaged value of interference power spectral density at Rv due to all terminals Tv.

Ilong (W): 	long-term interference power at Rv due to all terminals Tv.

Ilong (W/Hz): 	long-term interference power spectral density at Rv due to all terminals Tv.

Itot (W): 	total interference power at Rv due to all terminals Tv.

Itot (W/Hz): 	total interference power spectral density at Rv due to all terminals Tv.

Itot,0 (W/Hz): 	total interference power spectral density in the absence of antenna pointing errors at Rv due to all terminals Tv.

Ixr, x=i,v (W/Hz): 	interference power spectral density at Rv due to Tr and received via Sx.

k: 	Boltzmann constant. k = 1.38065  10–23 W/K/Hz.

Ld :	downlink path loss from Si or Sv to Rv. Ld=4πdd/λd2 + other losses, where dd is downlink range.

Lu,x, x=r,v : 	uplink path loss from Tx to Sv. Lu,x=4πdu,x/λu2 + other losses, where du,x is uplink range.

N↑ (W/Hz): 	noise power spectral density at Sv at the output of receive antenna of Rv.

N↓ (W/Hz): 	noise power spectral density at Rv referred to the output of its receive antenna.

N↑,i (W/Hz): 	noise power spectral density at Si at the output of receive antenna of Rv.

Ncs (W):	noise power under clear-sky conditions at receive antenna output of Rv.

Nr (W/Hz): 	sky noise power spectral density due to rain at the output of receive antenna of Rv.

pX: 	probability density function (PDF) of the variable X .

PX: 	cumulative distribution function (CDF) of the variable X.

qτt : 	rectangular pulse such that qτt=1 for t in 0,τ, and zero elsewhere.

R: 	region where interfering terminals are distributed.

r (m): 	location vector at Tr measured from origin, O.

Rv : 	victim receive terminal.

Si , Sv : 	satellites of the interfering network and the victim link, respectively.

Tlong (s):	averaging interval for long-term interference.

Tr , Tv: 	interfering terminal located r and the wanted transmit terminal.

Wlong (Hz):	bandwidth for determining the long-term interference power spectral density.

X (dB): 	value of parameter X in dB, 10 log10(X).

Zs :	Zs=C/NcsC/Ns , degradation of the C/N ratio due to rain fading in the presence of the long-term interference component.

Zt :	Zt=C/NcsC/Nt , degradation of the C/N ratio due to rain fading in the presence of the total interference.

# 6	Statistical model for the analysis of interference

The interfering and the victim satellite networks are shown in Fig. 2. The transmit terminals of the interfering network are denoted by T1,T2,…,Tr, as shown here. This figure shows the victim and interfering satellites, Sv and Si; wanted terminal, Tv ; and victim receiver, Rv . The analysis aims at quantifying the interference generated by the network of terminals T1,T2,…,Tr, to the victim satellite network. The interfering terminals are operating in a time-division multiple access manner with only a single terminal transmitting at a particular time instant in a narrow frequency band of interest. Note that the terminals may operate in a wide frequency band and in a frequency multiple access manner; the interference in this wide frequency band is computed by summing the interference in each narrow frequency band. In Fig. 2, it is assumed that satellites Si and Sv employ the same frequency translation from the uplink to the downlink.

Figure 2

Interference paths from Terminals T1, T2, … , Tr , to the victim receiver Rv, via Satellites Si and Sv. Ci and Cv denote 
the beam centres of Si and Sv on the ground; O is the origin from which the distances
to the terminal locations are measured

Assuming the interfering terminals are transmitting in a random manner, the time instant at which a particular interfering terminal is transmitting can be represented by a location dependent random variable. The probability density function (PDF) of this random variable is denoted by pr. When all the interfering terminals are in a Region R, it follows that Rprrdr=1.

The interfering terminals may consist of terminals with different antenna aperture sizes. The off-axis e.i.r.p. density pattern of a general terminal located at r is denoted by E. The PDF of the e.i.r.p. density patterns, when considered over all interfering terminals, is denoted by pE. Since this is a PDF, it follows that EpEEdE=1, where the integral is over all possible values of E.

The antenna pointing error at a terminal located r, which is the angle between the desired and actual directions of the antenna boresight, is denoted by ϕr. These antenna pointing errors may vary slowly and are statistically independent for different terminals. In this Annex it is assumed that elevation and azimuth components of this antenna pointing error, denoted by ϕr,ε and ϕr,a, are available. Also, the PDFs of these antenna pointing error components, pε and pa, are assumed to be known.

In the long-term interference criterion it is necessary to compute the time-averaged value of the interference. To facilitate this it is useful to represent the time-dependent transmission pattern of the interfering terminals as shown in Fig. 1. Suppose the Terminal Trn, located at rn, transmits in the time interval tn,tn+1. The transmission sequence of these terminals is Tr0, Tr1, Tr2,…., and the corresponding sequence of transmission intervals are: t0,t1, t1,t2, t2,t3, …. In order to represent this denote by qτ(t) a unit pulse of width τ so that qτt=1 in the interval 0,τ, and zero outside this interval. Then the time-dependent transmission pattern of the terminals can be expressed as nTrnqτn(t-tn) where τn=tn+1-tn.

# 7	Determining the long-term interference

The long-term interference is the time-averaged value, in a time interval of Tlong, of the interference in the absence of antenna pointing errors. The signal paths from the interfering terminals to the victim receiver via the interfering and victim satellites are shown in Fig. 2. As noted before, in this analysis translation between the uplink and the downlink frequencies is assumed to be the same at both satellites. The interference power spectral densities at the victim receiver, via Satellites Sv and Si, due to the interfering terminal Tr can be expressed in terms of the transmission gains from the satellites to the victim receiver, γv and γi, and the link parameters as follows:

(8)

where the satellite link transmission gains γv and γi are given as:

(9)

Note that the transmission gain γi does not depend on the location of the interfering terminal because it is the gain from the output of the receiving antenna at Satellite Si to the output of the receiving antenna at the victim terminal, Rv.

The interference terms Ivr and Iir depend on the specific location of the interfering terminal, Tr, and occur with a small probability prrdr. Since the interfering terminals may transmit at different off-axis e.i.r.p. density levels and they are located at different spatial locations, the composite interference signal due to all the terminals is time-varying. The ensemble-averaged value of the interference power spectral density when considered over all interfering terminals in the network in the desired Region R is expressed as:

(10)

Note that in this and subsequent sections, the interference power spectral density is computed; the corresponding interference power can be obtained by multiplying by the bandwidth of interest.

In this section, it is necessary to compute the time-averaged value of the interference. Therefore, it is necessary to express the interference signal as a function of time, in the absence of antenna pointing errors. Using the rectangular function qτt this can be expressed as:

where rn is the spatial location of the interfering terminal transmitting in the time interval tn,tn+1 and τn=tn+1-tn. The desired long-term interference component is the time-averaged value of the above and is expressed as:

(11)

As noted in § 3, because Itot,0 is a statistical process, the above value, Ilong, will exhibit small variations when computed over different time intervals. The CDF of the long-term interference-to-noise ratio presented in § 3 imposes limits on these variations.

# 8	Expressing the short-term performance objective criterion

The short-term performance objective criterion in terms of the C/N ratio degradation variables was expressed in § 4.2. In this section, expressions will be given to determine these C/N ratio degradations in terms of the link variables of the satellite network shown in Fig. 2.

## 8.1	Degradation of the C/N ratio due to rain fading in the presence of long-term interference

The degradation of the C/N ratio due to rain fading in the presence of the long-term interference component was expressed as Zs=C/NcsC/Ns in § 4.1. In this subsection, this degradation will be computed in terms of the specific link variables.

The C/N ratio under clear-sky conditions can be expressed as:

(12)

where the variables C, N↓, N↑ and N↑,i are given by:

Note that in equation (12), and later in § 8.2, the C/N ratio is expressed in terms of the carrier, noise and interference power spectral densities. The corresponding power can be obtained by multiplying these by the bandwidth of interest.

Next, consider the C/N ratio in the presence of rain fading

(13)

Here it is assumed that the orbital separation between the satellites Sv and Si is very small so that the downlink fade terms from these satellites are the same. Also, it is assumed that the uplink fade terms from the interfering terminals are approximately the same and given by A↑,i. This is reasonable for a coverage area of a few hundred kilometres. If this is not the case, the last term in the denominator should be appropriately modified to account for the location-dependent rain attenuation term, A↑,ir.

The degradation of the C/N ratio in the static case is defined by Zs=C/NcsC/Ns. Substituting the values for C/Ncs and C/Ns from equations (12) and (13) it can be shown that:

(14)

where the variables d1, d2 and d3 are defined as:

Note that d1+d2+d3=1. In order to express the parameters d1, d2 and d3 in terms of the satellite link variables, introduce the following variables:

Substituting these variables in d1, d2 and d3 above:

The variables c1, c2 and c3 can be expressed in terms of the satellite link parameters as:

Since the rain fades are typically available in dB units, the C/N ratio degradation variable Zs in equation (14) can be analysed conveniently when it is expressed in logarithmic units. Expressing Zs and the rain fades in dB units:

(15)

The CDF of , can be determined analytically when the PDFs of the rain attenuation factors, and are known. Alternatively, a Monte-Carlo simulation method may be used to estimate the CDF of Zs.

## 8.2	Degradation of the C/N ratio due to rain fading in the presence of total interference

In this section, the degradation of the C/N ratio due to rain fading in the presence of total interference, Zt=C/NcsC/Nt is determined in terms of the satellite link parameters.

The long-term interference component in § 7 was determined when the interfering terminals are transmitting without antenna pointing errors. Antenna pointing errors of the terminals are taken into consideration in this section. The antenna pointing error at Tr is denoted by ϕr. In the presence of antenna pointing errors the interference terms in equation (8) are expressed as:

(16)

where the dependence of the off-axis angles ψr,v and ψr,i on ϕr is shown explicitly. The total interference in the presence of antenna pointing errors is now Itot=Ivr+Irr. The antenna pointing errors are usually available in terms of their errors in the azimuth and elevation directions, ϕr,a and ϕr,ε. Annex 1 of Recommendation ITU-R S.1857 gives a method for determining the angles ψr,vϕr and ψr,iϕr using the available azimuth and elevation error angles.

The C/N ratio at the victim receiver due to rain fading in the presence of total interference is expressed as:

(17)

Similar to the derivation in the preceding section, the degradation in the C/N ratio in this case,, is expressed as:

(18)

where and the variables d1, d2 and d3 are as given in the preceding section. The CDF of Zt,  PZtz=PrZt≤z, can be determined analytically when the PDFs of the rain attenuation factors and the PDFs noted in § 6 are available. Alternatively, a Monte-Carlo simulation method may be used to estimate the CDF of Zt.

# 9	Increase in link degradation due to the short-term interference

The criterion for the short-term performance objectives given in § 4.2 is stated in terms of the complementary CDFs of the link C/N ratio degradation variables, and . Consider a C/N ratio degradation level of zj in rain fading and in the presence of the long-term interference. The link degradation, that is when Zs exceeds zj, in terms of a time percentage in this case is 1-PZszj×100%. Next, consider the total interference to this link.

The link degradation, for the same C/N ratio degradation level of zj is 1-PZtzj×100%. Therefore, the relative increase in the link degradation due to the presence of the short-term interference is:

(19)

For example, suppose a satellite link is designed to operate so that the C/N ratio for the link is less than C/Nj for only pj% of the time. According to § 4.2 a link margin should be incorporated so that under rain fading conditions and long-term interference the degradations are limited to at most pi%×1-pshort/100 of the time. The link margin necessary, zj, to satisfy this condition can be computed using the CDF of the degradation variable as 1-PZszj=pj×1-pshort/100. Then the short-term interference should be limited so that 1-PZtzj≤pj. It can be see from equation (19) that for these values RS%≤pshort%.

# 10	Increase in average interference due to antenna pointing errors

Notice that the long-term interference component, Ilong, is computed in the absence of antenna pointing errors and the total interference term, as computed in § 8.2, takes into account antenna pointing errors. The short-term variations of the interference are because of the antenna pointing errors and the time-division multiple access operation of the terminals. Variations due to the latter can be neglected if the average value of Ilong, denoted by Ilong and given in equation (10) as Iavg, is considered. This value can be realized when Tlong is very large with respect to the average transmission duration of each terminal. The following measure can be used to determine the effect of antenna pointing errors on the average interference:

(20)

where Itot is the average value of the total interference. Observe that, in the absence of antenna pointing errors Itot≈Ilong, so RL is negligible.

# 11	Simulation examples

This section provides illustrative computer simulation results obtained from the methodology presented in this Annex. Figure 3 shows the locations of the victim receiver and the interfering terminals with respect to the beam centres of the receive antennas of Si and Sv. As shown here, in this computer simulation the satellite beam centres are coincident and the victim receiver is positioned at this point. The Region R, where the interfering terminals are distributed, is obtained by distributing the transmit terminals uniformly in a circular area with centre Cv (or Ci) and a radius of 100 km. The aperture diameters of the interfering terminals are selected randomly from the set {0.2, 0.25, 0.3, 0.35, 0.4} m and their e.i.r.p. density pattern is limited by the following:

(21)

where  is the off-axis angle and  is a parameter that can be used to increase or decrease the off-axis emission levels from the terminals. Note that when E=0 the off-axis emission level corresponds to that specified in recommends 4 of Recommendation ITU-R S.524-9 for earth stations operating in GSO networks in the FSS transmitting in the 27.5-30 GHz frequency band. The following simulation results are given for (T/T)long, RS and RL as a function of . The satellite link parameters and the statistical parameters used in the computer simulations are given in Tables 1 and 2, respectively.

Figure 3

Footprints of the victim and interfering satellites and the distribution of the interfering terminals for this simulation. Here Cv and Ci are coinciding and Rv is also assumed to coincide with this point

TABLE 1

Satellite link parameters used in the computer simulation

| Uplink frequency Uplink loss Victim transmit terminal, Tv, (latitude, longitude) Site altitudes at Tv, Tr Rain rate and rain height at Tv, Tr | 28.75 GHz 213.09 dB (40 N, 102.8 E) 200 m above mean sea level Obtained using Recommendations ITU-R P.837 and ITU-R P.839 |
| --- | --- |
| Longitudes at Satellites Sv, Si Satellite beam centres, Cv and Ci, (latitude, longitude) Receive antenna gains at satellites, Noise temperatures at satellites, Receive antennas at satellites Satellite Sv e.i.r.p. density | 102.8 E and (102.8 + space) E (40 N, 102.8 E), (40N, 102.8 E) 51.83 dBi 1 000 K 1.75 m circular aperture with parabolic illumination 30 dBW/40 kHz |
| Downlink frequency Downlink loss Victim receiver antenna gain, Gr,v Site altitude at Rv Rain rate and rain height at Rv | 18.95 GHz 209.47 dB 50.96 dBi (2.4 m), 44.96 dBi (1.2 m) 200 m above mean sea level Obtained using Recommendations ITU-R P.837 and ITU-R P.839 |
| Satellite link transmission gains, (v, i) | (–7.25 dB, –76.62 dB) for Rv aperture diameter 2.4 m (–13.27 dB, –64.49 dB) for Rv aperture diameter 1.2 m |

TABLE 2

Statistical parameters used in the simulation

| Region R pr pE r, and r,a | Circular area with centre at Cv, Ci and radius 100 km Uniformly distributed in Region R Terminals with apertures of diameters {0.2, 0.25, 0.3, 0.35, 0.4} m equally distributed and e.i.r.p. density limited by equation (21) Gaussian random variables with mean zero and standard deviation |
| --- | --- |

Figures 4 and 5 show the values of (T/T)long and Rs given in equations (1) and (19), respectively. The value of Ilong in the numerator of (T/T)long is determined assuming a very large value of Tlong. Therefore, statistical variations of Ilong can be neglected. It can be seen that for the illustrative parameters given in Tables 1 and 2, and when the orbital separation of the satellite is 4, an e.i.r.p. density level corresponding to  gives rise to (T/T)long = 5%, for a victim receiver with aperture diameter 2.4 m. If the orbital separation were 3°, to maintain the same level of (T/T)long the value of  has to be decreased to –1.3 dB. These figures show that (T/T)long is less if the victim receiver aperture diameter is 1.2 m. Figures 4 and 5 also show the value of RS used in the short-term criterion. In this case pj% and pshort% were set to 2% and 10%, respectively. For the parameters considered in this illustrative example the value of RS is less than 4%. From equation (19), this corresponds to a link degradation level of less than 1.88%.

Figure 4

Variations of (T/T)long and Rs with in (21) for space = 4 and  = 0.5

Figure 5

Variations of (T/T)long and Rs with in (21) for space = 3 and  = 0.5

The value of RL used in equation (20) is shown in Fig. 6 for different values of . As noted before, for sufficiently large values of Tlong, the variations of RL are negligible in the absence of antenna pointing errors. This figure shows the gradual increase of RL with increasing values of .

Figure 6

Variation of RL with standard deviation of antenna pointing errors
for orbital separations of 3 and 4

# 12	Conclusions

A new statistical approach to assess the interference of a time variant system consisting of a network of earth stations operating on a time division multiple access scheme was presented in this Annex. The results served to illustrate the potential degradation on a victim satellite network and showed that the transmit levels of the interfering network terminals can be adjusted to meet the allowable interference and the performance objectives of the victim satellite system. The Appendix to this Annex provides an illustrative step-by-step process to estimate the CDF of ΔTTlong and RS.

Appendix

An illustrative step-by-step process to estimate the CDF of
ΔTTlong and RS

This Appendix provides an illustrative step-by-step process to estimate the CDF of ΔTTlong expressed in § 3 and the relative increase in degradation due to short-term interference, Rs, as expressed in equation (19) of the Annex. These values are estimated for a given off-axis e.i.r.p. density level. The approach presented here is based on Monte-Carlo simulations.

# 1	Input to the estimation process

Input 1. Satellite link parameters

Uplink and downlink wavelengths λu, λd; longitudes of Sv, Si; latitude and longitude pair at Ci, Cv; receive antenna gain patterns Gr,is,Gr,vs; noise temperatures Θis, Θvs; transmission gains γi, γv.

Noise temperature Θv.

Input 2. Interfering terminals

PDF of spatial distribution of terminals pr; PDF of e.i.r.p. density distribution pE. Note that the e.i.r.p. density depends on the aperture size of the terminal and the off-axis e.i.r.p. density limit considered.

Antenna pointing errors: PDFs of azimuth and elevation components of antenna pointing error, pϕa, pϕε. Alternatively, these components may be available as vectors of length Nmc (defined in Input 5), ϕr,a, ϕr,ϵ.

Transmission pattern of the terminals: PDF of the transmission duration of the terminals, pτ, where τ is the transmission duration of a terminal as discussed in § 6.

Input 3. Rain parameters

Rain rate, altitude above mean seal level and rain height for locations of Tv , Rv , and the representative center of Region R defined by pr. These parameters can be computed using Recommendations ITU-R P.837 and ITU-R P.839.

Sky noise temperature due to rain Θr.

Input 4. Parameters to compute long- and short-term interference levels

Observation interval for long-term interference, Tlong; time percentage for link degradations in short-term performance objectives, pi%; and maximum time percentage for short-term interference, pshort%.

Input 5. Monte-Carlo simulation parameter: sample size of the random vector Nmc

# 2	Estimation of CDF of ΔTTlong

Step 1. Generate the transmission times for the interfering terminals

Generate Nlong transmission times, τn, according to the PDF, pτ, such that the sum of all transmission times satisfies n=1Nlong-1τn<Tlong≤n=1Nlongτn.

Step 2. Generate the interfering transmit terminals

a)	Generate the Nlong-dimensional location vector r according to the PDF pr.

b)	Select the e.i.r.p. density of the terminal at each location point r according to the PDF pE.

Step 3. Compute the interference terms Iir and Ivr

a)	Angle ψr,v. This is computed using the latitudes and longitudes at r, Si and Sv.

b)	Angles δi,r and δv,r. These are computed using the latitudes and longitudes at r, Si, Sv, Ci and Cv.

c)	Compute the uplink path loss Lu,r.

d)	Interference signal Iir+Ivr using equation (8). Note that BrGt,rψr,v is the e.i.r.p. density in the direction of Sv and BrGt,r0 is the e.i.r.p. density in the direction of Si.

The Nlong-dimensional vector obtained by computing Iir+Ivr at all r location points gives the instantaneous values of the interference in the absence of antenna pointing errors.

Step 4. Compute the long-term interference Ilong

a)	Construct the interference signal, Itot,0t, as a function of time as described in § 7.

Itot,0t=n=1NlongIirn+Ivrnqτnt-tn, where rn is the nth component of r, t1=0 
and tn=i=1n-1τi.

b)	Compute Ilong=1Tlongt=0TlongItot,0t dt.

Step 5. Estimate CDF of ΔTTlong

a)	Construct a Nmc-dimensional vector Ilong by repeating, Nmc times, Steps 1 to 4 above.

b)	Construct the Nmc-dimensional vector ∆TTlong using (1) and the Ilong vector.

c)	Estimate the CDF of the vector ∆TTlong.

# 3	Estimation of Rs

Step 1. Generate the interfering transmit terminals

a)	Generate the Nmc-dimensional location vector r according to the PDF pr.

b)	Select the e.i.r.p. density of the terminal at each location point r according to the PDF pE.

Step 2. Compute the Nmc-dimensional interference vector Iir+Ivr

Follow Step 3 in § 2 above.

Step 3. CDF of

a)	Determine variables c1, c2 and c3 given in § 8.1 using the satellite link parameters and the Nmc-dimensional vector Ilong estimated in Step 5 of § 2 above.

b)	Determine the variables d1, d2 and d3 using c1, c2 and c3 as shown in § 8.1. Note that d1, d2 and d3 are Nmc-dimensional vectors.

c)	Generate the Nmc-dimensional rain attenuation vectors using Recommendation ITU-R P.618-8. Here  corresponds to a single representative location in the Region R as defined by the PDF pr.

d)	For each realization of  and d1, d2, d3, compute  expressed in equation (15). This gives a Nmc-dimensional vector for.

e)	Estimate the CDF of  using this vector.

Step 4. CDF of

a)	Generate the Nmc-dimensional antenna pointing error vector ϕr

Generate the Nmc-dimensional vectors ϕr,a and ϕr,ϵ using their respective PDFs, pϕa, pϕε. For each realization ϕr,a, ϕr,ϵ, compute ϕr using the procedure described in Annex 1 of Recommendation ITU-R S.1857.

b)	Generate the Nmc-dimensional interference vector Iir+Ivr given in equation (16)

For each realization r and ϕr compute Iir and Ivr as given in equation (16). Construct the Nmc-dimensional interference vector from this.

c)	Compute the Nmc-dimensional vector  using this as given in equation (18).

d)	Estimate the CDF of  using this vector.

Step 5. Estimate Rs.

a)	Compute the link margin necessary under rain fading conditions and long-term interference, zj, such that the maximum time allowed for degradations is
 pj%×1-pshort/100

b)	Compute the link degradation time with total interference for this link margin,

c)	Compute RS using equation (19).