RECOMMENDATION  ITU-R  S.736-1

ESTIMATION  OF  POLARIZATION  DISCRIMINATION  IN  THE  INTERFERENCE
CALCULATIONS  BETWEEN  GEOSTATIONARY-SATELLITE
NETWORKS  IN  THE FIXED-SATELLITE  SERVICE

(Question ITU-R 42/4)

(1992-1994)

Rec.  ITU-R S.736-1

The ITU Radiocommunication Assembly,

considering

a)	that geostationary-satellite networks in the fixed-satellite service (FSS) operate in the same frequency bands;

b)	that interference between networks in the FSS contributes to noise in the network;

c)	that it is necessary to protect a network in the FSS from interference by other such networks;

d)	that the detailed estimation of mutual interference between satellite networks, due to increased orbit occupancy requires more accurate values of polarization discrimination, resulting from the use of different or identical polarizations by wanted and interfering systems;

e)	that the use for actual coordination requirements of the values of polarization isolation factors of Appendix 29 to the Radio Regulations, would not provide a precise estimation of polarization discrimination in the calculation of actual interference margins,

recommends

1.	that, to estimate the polarization discrimination between two satellite networks considered, the method described in Annex 1 should be used.

ANNEX  1

Estimation of polarization discrimination

# 1.	Definition of the polarization of a wave

Polarization is defined as a vector of the electric field wave which is located in a plane orthogonal to the direction of the wave propagation.

Generally, this vector describes an ellipse. Two particular cases arise, firstly circular polarization where the two axes of the ellipse are equal, secondly linear polarization where one of the axes is zero.

If the radiated wave is linearly polarized, two orthogonal polarization planes exist, each polarization vector keeping a fixed direction. The polarization plane, for a linearly polarized wave, is the plane containing the direction of the wave propagation and the polarization vector.

If the radiated wave is circularly polarized, right- and left-hand rotations exist.

# 2.	Definition of polarization angle and of relative alignment angle

The polarization angle e is the angle between the vertical plane including the propagation direction (pointing of the earth station towards the satellite) and the polarization plane of the linearly polarized wave transmitted by the satellite or by the earth station pointed towards the satellite.

The relative alignment angle b is, in linear polarization, the angle between:

–	the planes of polarization of the wanted and interfering signals (e1 – e2) (see Appendix 2),

–	or the plane of polarization of the received signal and the plane of polarization of the receiving antenna (see Appendix 1).

In the co-polarized case, the angle b is given by:

b  =  ½e1  -  e2½  +  d

with:

d:	tolerances.

For calculation of the angles b and e, see Appendices 1 and 2.

# 3.	Definition of polarization decoupling ratio and polarization discrimination

The ratio of polarization decoupling Dp(j) of an earth station or a satellite antenna is the ratio of the field component in the wanted polarization to the field component in the orthogonal polarization. j is the angle between the directions of wanted and interfering signals.

The polarization discrimination Y (factor of polarization/isolation) of a receive antenna is the ratio of the received power of the two waves of different direction and polarization.

It should be noted that, if the interfering or the wanted network (or both of them) operates a set of transponders on one polarization, and a set of co-frequency transponders on the orthogonal polarization, then it is not valid to include the full polarization discrimination in calculations of interference between the two networks. The degree of such discrimination will depend on the extent of the overlap of the pass-bands of transponders in one network with those of the other network. In the worst case, i.e., when the transponders of the two networks are exactly aligned in frequency and bandwith, no inter-network polarization discrimination should be included in interference calculations between the two networks.

# 4.	Calculation of polarization discrimination Y in linear polarization

## 4.1	Calculation of polarization discrimination Yd in the down-link

The purpose of this calculation is to determine, in the case of a wanted receiving earth station, the discrimination with respect to an interfering wave. Earth-station radiation patterns have been established for co- and cross-polarization planes using experimental data.

The polarization angles are calculated for wanted and interfering signals using the coordinates of the two pointing directions of wanted and interfering satellite antennas and the coordinates of the reference earth station.

The derived value of discrimination Yd takes into account the co-polarized wave coming from the interfering satellite, received by the earth-station receiver (co-polarized A//(j) and cross-polarized A+(j) patterns).

The cross-polarized wave coming from the interfering satellite intercepted by the co-polarized pattern of the station is also taken into account. However, the additional isolation afforded by the ratio of crossed polarization transmit-to-crossed polarization receive may be neglected.

(1)

where:

jb :			topocentric separation between satellites

Dp(jb) :		polarization decoupling of the wanted earth station:

Dpsat :		polarization decoupling (dB) of the interfering satellite in the coverage area where the wanted station is located.

## 4.2	Calculation of the polarization discrimination Yu in the up-link

The purpose of this calculation is to determine in a similar way to the preceding section, for a receive antenna of the wanted satellite, the discrimination Yu with respect to an interfering wave.

(2)

where:

yb :				angle between the main radiation direction and the direction of interfering earth station

Dp(yb) :		polarization decoupling of the wanted space station:

S// and S+ :		co-polarized and cross-polarized patterns of the wanted satellite antenna

Dpst :			polarization decoupling (dB) of the interfering earth station.

# 5.	Calculation of polarization discrimination Y in the case of two polarizations: one circular, the other linear

In the case of an interfering wave in linear polarization (the linear polarization vector can be derived from  two circular polarization vectors, right- and left-hand rotation), the discrimination obtained at the receive wanted antenna operating in circular polarization can be expressed in a way similar to the above:

(3)

where:

Dp(j) :		polarization decoupling of the receive antenna (dB).

Similarly, in the case of an interfering wave in circular polarization (the circular polarization vector can be composed of two orthogonal linear polarization vectors), the discrimination obtained at the receive wanted antenna operating in linear polarization is described by the same formula.

# 6.	Application to actual cases

From the above relationships, the calculation of the polarization discrimination in various cases permits estimation of the improvement obtained in interference calculations. In particular the use of a polarization orthogonal to the one of the interference, may be considered. These relationships do not take into account the effects of propagation conditions on the signal polarization plane; these are included in Appendix 1.

# 7.	Orientation of polarization planes

It is possible to orient the polarization plane of a receive (linearly polarized) satellite or earth-station antenna for two different optimum conditions, one to minimize clear-sky interference from an orthogonally polarized signal and the other to minimize the effects of depolarization due to rain. The means of calculating the optimum alignments in the two cases are described in Appendix 1.

APPENDIX  1
TO  ANNEX  1

Optimization of polarization orientation

# 1.	Simple model of a satellite-to-Earth link

Depolarization effects on satellite-to-Earth (or Earth-to-satellite) links have been extensively analyzed and mathematical procedures which take into account all known contributions to depolarization have been formulated. These procedures are, however, extremely complex and cumbersome. A simplified model, suitable for use in a first-order analysis of interference has therefore been devised which uses the concept of equivalent gain of a partial link (i.e. either the Earth-to-space or the space-to-Earth connection).

The equivalent gain (as a power ratio) for one partial link can be represented by the following approximation:

G  =  G1  ·  cos2 b  +  G2  ·  sin2 b

G1  =  Gtp  ·  Grp  ·  A  +  Gtc  ·  Grc  ·  A  +  Gtp  ·  Grc  ·  A  ·  X  +  Gtc  ·  Grp  ·  A  ·  X

(4)

where:

b :		relative alignment angle, for linear polarization, between the received signal polarization plane and the plane of polarization of the receive antenna

Gtp :		co-polar gain characteristic of the transmit antenna expressed as a power ratio (Recommendations ITU-R S.465, ITU-R S.580 for earth stations and Recommendation ITU-R S.672 for satellites)

Gtc :		cross-polar gain characteristic of the transmit antenna expressed as a power ratio

Grp :		co-polar gain characteristic of the receive antenna expressed as a power ratio (Recommendations ITU-R S.465, ITU-R S.580 and ITU-R S.672)

Grc :		cross-polar gain characteristics of the receive antenna expressed as a power ratio

A :		rain fade as a power ratio £ 1

X :		rain depolarization as a power ratio << 1.

The following sections give derivations for the angle, b, for the two scenarios:

–	polarization aligned to minimize effects of rain fade, and

–	polarization aligned to minimize interference.

Using the equivalent gain concept, the wanted carrier power, C, or the single-entry interfering power, I, on each partial link is simply given by:

(5)

where:

PT :		wanted (interfering) transmitting antenna power (dBW)

LFS :		free-space loss on the wanted (interfering) link (dB)

LCA :	clear-air absorption on the wanted (interfering) link (dB)

G :		equivalent gain on the wanted (interfering) link (dBi).

# 2.	Polarization orientation to minimize interference due to rain depolarization

As illustrated in Fig. 1, the apparent polarization plane at the Earth’s surface is a function of the geographical coordinates of the boresite, the test point under consideration, and the satellite. The following formula for this polarization angle e can be used:

tan e  =  	(6)

where:

y :	latitude

l :	longitude

b :	boresight

p :	test point

s :	satellite.

In the above, it is assumed that the polarization plane is optimized for minimum rain fade at the boresight, i.e. aligning with either the local horizontal or the local vertical. In functional form, equation (6) may be rewritten:

e  =  f(yb, lb, yp, lp, ls)

To determine the interference component at test point, p, the difference in polarization angles is required.

Thus, if the wanted and interfering signals are “co-polar”, the angle, b, in equation (4) may be expressed as:

b  =  ½e1  -  e2½  +  d	(7)

where:

e1  =  f(yb1, lb1, yp, lp, ls1)

e2  =  f(yb2, lb2, yp, lp, ls2)

b1 :		boresight of wanted satellite s1

b2 :		boresight of interfering satellite s2

d :		allowance for mis-alignment of earth-station antenna and rotational tolerances of satellite beams.

If the wanted and interfering signals are “cross-polar” the angle bc in the worst case, is:

-  | e1  -  e2 |  -  d	(8)

Under some circumstances the distinction between co-polar and cross-polar may be academic. Thus for the purposes of this analysis the following definition will be assumed:

–	if both wanted signals are aligned to the respective local horizontals or both are aligned to the respective local verticals, they will be considered co-polar.

Figure [D02] = 13.5 cm

# 3.	Polarization orientation to minimize clear-sky interference

Minimum interference occurs when the polarization planes at the satellite orbit are orthogonal, i.e. when the polarization planes are either in the equatorial plane or in the plane of the Earth’s North-South axis. The following formula for the angle of polarization, e¢ , can be used when the polarization vector of the transmitted wave is parallel to the equatorial plane:

(9)

cos x  =  cos (lp  -  ls)  ·  cos yp

a :	radius of earth divided by radius of orbit ~ 0.151

Thus, in functional form:

e¢  =  g(yp, lp, ls)

When the wanted and interfering signals are “co-polar”, i.e. when both have polarization planes parallel to the equatorial plane or when both are perpendicular to the equatorial plane, the relative angle, b, between wanted and interfering polarization planes is given by:

(10)

where:

s1 :		wanted satellite

s2 :		interfering satellite.

Similarly when the two signals are cross-polar, the relative angle bc is given by:

(11)

The variation of e¢ is given in Fig. 1.

APPENDIX  2
TO  ANNEX  1

Calculation of polarization angle and relative alignment angle

# 1.	Calculation of polarization angle a of a wave in linear polarization

## 1.1	Definition of orthonormal coordinate systems (see Fig. 2)

The orthonormal coordinate systems used are the following:

–	the base Earth-centre coordinate system Rg: , Y, Z: Z is directed to the North and X in the equatorial plane is determined by the direction GS, G is the Earth centre and the origin of the system and S the satellite;

–	the coordinate system Rs connected with the satellite S: , Y,Z: Z  =   and Y is directed to the East in the equatorial plane;

–	the coordinate system Ra connected with the antenna A of satellite S: , Y, Z: Z is directed to boresight Pv and Y is directed to the East in the equatorial plane;

and the system Ra(g) linked with the polarization vector of the antenna A of satellite S: (g), Y(g), Z: the transformation from coordinate system Ra into coordinate system Ra(g) is made by a rotation (–g) around Z. That axis Y(g) is the axis of symmetry of the reflector of antenna A and defines the direction of the linear main polarization of antenna A for a propagation direction Z i.e. towards the boresight Pv of the antenna A of satellite S;

–	the coordinate system Rp connected with earth station P and satellite S on which is connected the earth station:  , Y, Z: Z is directed to satellite S and X is directed to the left of an observer who, placed at station P, faces at satellite S: direction X is V ^ Z;

V  =  GP gives the direction of the local vertical on the Earth at earth station P

V  =  [Rt cosyp cos (lp – ls); Rt cosyp sin (lp – ls); Rt sin yp] in system Rg

hence:

X  =   ^ Z / || V ^ Z ||  =   ^ PS / ||  =  GP ^ PS ||

Y  =  Z ^ X

Z  =  PS / || PS || avec PS  =  h X  –  GP

Paramers and notations are as follows::

index 1:	wanted satellite S1 and earth station P1

index 2:	interfering satellite S2 and earth station P2

index d:	down-link

index m:	up-link

Pv:		boresight of the antenna A of satellite S

P:		any point on the Earth

Rt:		radius of the Earth: 6 378 km

h:		radius of the geostationary-satellite orbit (GSO) : 42 164 km

k =		6.62 = h / Rt : GSO radius divided by radius of the Earth

yp:		latitude of earth station P

lp:		longitude of earth station P

ypv:		latitude of boresight Pv

lpv:		longitude of boresight Pv

ls:		longitude of satellite S

qp:		geocentric angle SGP: angle at the centre of the Earth between the sub-satellite point and earth station P

g :		inclination angle of the ellipse of the satellite antenna coverage pattern: angle between the equatorial plane and the polarization vector (or direction of the ellipse major axis), and is chosen to be positive for the trigonometric direction from the equator towards the ellipse major axis

d:		allowance for mis-alignment of earth station antenna and rotational tolerance of satellite beams

(y, q, (j – g)): Euler angles appearing in the transformation matrix M  from system Rs to system Ra(g)

^:		cross product

|| Z ||:	magnitude of vector Z.

## 1.2	Particular case: the earth station is at the boresight: P = Pv

#### 1.2.1	Definition of main polarization of a linearly polarized wave transmitted from a satellite antenna A

In this particular case, the planes of the antenna of earth station Pv , Y and of the satellite antenna A  are parallel, polarization vector V is parallel to the plane of GSO and is directed towards the East, i.e. normal toZ therefore:

Z

with:

V

in system Rg

with:

### 1.2.2	Approximate definition of polarization angle in the system Rpv of a receiving earth station at boresight Pv: determination of polarization angle of a linearly polarized wave transmitted from a satellite antenna A

The approximate polarization angle is defined in system Rpv by:

(12a)

Vectors V, X and Y must be expressed in the same system.

Taking into account the angle g characterizing the polarization of the transmitting satellite antenna, the value of the polarization angle may be expressed as:

e(g)  =  e  +  g	(12b)

## 1.3	General case: the earth station is distinct from the boresight: P  ¹  Pv

### 1.3.1	Definition of main polarization and cross-polarization of a lineraly polarized wave transmitted from a satellite antenna A

The electric field E(P) is defined in the antenna systema Ra of satellite S. The most used definition is the third definition of Arthur C. Ludwig for a wave transmitted by an antenna. The main polarization and cross-polarization components Ep(P) and Ec(P) at any point P are expressed as follows:

The expressions of vectors  are the following in the case of the antenna A of satellite S transmitting a linearly polarized field parallel to Y:

and

with eand e: usual unit vectors of spherical coordinates in system Ra:

e

and

Direction SP (satellite S – any earth station P) in system Ra is given by vector :

[qa; ja]  =  [arc cos Za; arc tan (Ya / Xa)]

Xa, Ya and Za are the components of vector  in antenna system Ra

Xg, Yg and Zg are the components of vector  in earth base system Rg

Xs  =  Zg, Ys  =  Yg and Zs  =  –Xg are the components of vector  in satellite system Rs.

Transformation matrix M from system Rs into system Ra(g) takes into account the Euler angles(y, q, (j – g)) defined as follows:

–	rotation y around Z:	transformation from system Rs into intermediate system R1:

=

–	rotation q around X:	transformation from system R1 into intermediate system R2:

=  Z

–	rotation (j – g) around Z:	transformation from system R2 into system Ra(g):

(g), Y (g), Z

with:

q  =  – arc cos (Zpvs / Zpva)

y  =  – sign (ypv)  ·  ½ arc cos [Ypvs / (sin q  ·  Zpva)] ½ and

tg j  =  – tg y cos q

Xpva, Ypva and Zpva  are the coordinates of boresight Pv in system Ra

Xpvs, Ypvs and Zpvs  are the coordinates of boresight Pv in system Rs

with:

Zpvs  =  h  –  Rt cos ypv cos (lpv  –  ls)

Zpva  =  [h2  +  Rt2  –  2h Rt cos ypv cos (lpv  –  ls)]½  and

Ypvs  =  Rt sin (lpv  –  ls) cos ypv

Therefore transformation (3 × 3) matrix M is defined as follows:

i.e. V  =  M V(g), with vector V expressed in satellite and antenna systems.

### 1.3.2	Precise definition of polarization angle in the system Rp of a receiving earth station at any point P: determination of the polarization angle of a linearly polarized wave transmitted from a satellite antenna A

Therefore in the case of linear polarization parallel to Y, the components of main polarization vector are expressed in system Ra by:

The polarization angle is defined in system Rp by:

(13a)

Vectors , X must be expressed in the same coordinate system.

Taking into account the angle, g, characterizing the polarization vector  of the transmitting satellite antenna, the value of the polarization angle may be expressed as:

(13b)

Vectors , X must be expressed in the same coordinate system.

# 2.	Calculation of relative alignment angle b for the linear polarization

## 2.1	Down-link case: reception by a wanted earth station P1 (see Fig. 3): 	Calculation, for an earth station P1, of the alignment angle between the signals coming from a wanted satellite S1 and an interfering satellite S2

Polarization angles ed1,1, and ed2,1 are defined in earth station coordinate system Rp1 connected with the wanted earth station considered P1, which is pointed at the wanted satellite S1.

The above calculations can be applied directly for polarization angle ed1,1 hence taking into account the polarization vector  of the wave transmitted from the antenna A1 of the wanted satellite S1 characterized by angle g1,:

(14)

Vectors , X are expressed in the same system Rg1.

Similarly, for calculation of polarization angle ed2,1 of the wave transmitted by the antenna A2 of the interfering satellite S2 with polarization vector  characterized by angle g2:

(15)

Vector  is initially calculated in system Rg2 then in system Rg1.

Polarization angles ed1,1(g1), and ed2,1(g2) being defined in system Rp1, the value of alignment angle bd is given in the co-polar case by the expression:

bd  =  ½ed1,1 (g1)  -  ed2,1  (g2)½  +  d	(16)

## 2.2	Up-link case: reception by a wanted satellite S1 (see Fig. 4): 	Calculation for a wanted satellite S1 of the alignment between the signals coming from a wanted earth station P1 and an interfering earth station P2

Polarization angles em1,1 and em2,1 are, in this case, defined in antenna coordinate system Ra1, connected with the antenna A1 of the wanted satellite S1 which is pointed towards its boresight Pv1.

Similar calculations to the above can also be applied for polarization angle em1,1 of the wave transmitted from the wanted earth station P1. Hence taking account the polarization vector  of the wave received by the antenna A1 of the wanted satellite S1 characterized by angle g1:

(17)

The vectors ,X are expressed in the same system Rg1.

Similarly, for calculation of the polarization angle em2,1 of the wave transmitted by an interfering earth station P2 with polarization vector  towards the receiving antenna A1 of the wanted satellite S1, (earth station P2 is pointed to the receiving antenna A2 of the interfering satellite S2 characterized by angle g2):

(18)

Vector  is also calculated in system Rg1.

Polarization angles em1,1(g1) and em2,1(g2) being defined in system Ra1, the value of alignment angle bm is given in the co-polar case by the expression:

bm  =  ½em1,1(g1)  -  em2,1(g2)½  +  d	(19)

FIGURE 2/S.736...[D02] = 12.5 CM

FIGURE 3/S.736...[D03] = PLEINE PAGE

FIGURE 4/S.736...[D04] = PLEINE PAGE

________________