RECOMMENDATION  ITU-R  SA.1024-1

NECESSARY  BANDWIDTHS  AND  PREFERRED  FREQUENCY  BANDS  FOR
DATA  TRANSMISSION  FROM  EARTH  EXPLORATION  SATELLITES
(NOT  INCLUDING  METEOROLOGICAL  SATELLITES)

(Question ITU-R 139/7)

Rec. ITU-R SA.1024-1

(1994-1997)

The ITU Radiocommunication Assembly,

considering

a)	that sensors on Earth exploration satellites, currently operating or under development, produce data rates ranging from 2 kbit/s up to 1 Gbit/s;

b)	that, although on-board data compression techniques could allow a reduction of symbol rates, it may be necessary to use error correcting coding techniques to achieve the performance objectives thus resulting in increased bandwidths;

c)	that several frequency bands are allocated to the Earth exploration-satellite (EES) service (space-to-Earth) and to the inter-satellite service for use by the EES service;

d)	that the band 2 200-2 290 MHz is extensively utilized by space and terrestrial services;

e)	that transmission through the Earth’s atmosphere at frequencies above around 10 GHz is affected by rain;

f)	that additional EES systems requiring space-to-Earth data links are planned, some of which will transmit higher resolution images, requiring high data rates resulting in bandwidths greater than can be accommodated in the 8 025-8 400 MHz band;

g)	that certain future EES systems will require both space-to-space links to data relay satellites and space-to-Earth data links near 26 GHz,

recommends

1	that Earth exploration-satellite space-to-Earth and space-to-space links requiring low data rates use assignments in the allocated bands below 2 300 MHz;

2	that Earth exploration-satellite space-to-Earth links requiring medium and high data rates use assignments in the 8 025-8 400 MHz band;

3	that Earth exploration-satellite space-to-space links requiring high data rates use assignments in the 25.25-27.5 GHz band;

4	that Earth exploration-satellite space-to-Earth links requiring high data rates but with a possible reduction of transmission performance in case of rain use assignments in frequency bands above 25 GHz that are allocated to the Earth exploration-satellite service;

5	that EES systems requiring wide bandwidth data relay satellite links as well as wide bandwidth direct-to-Earth links use assignments near 26 GHz.