RECOMMENDATION  ITU-R  SA.1028

PERFORMANCE  CRITERIA  FOR  SATELLITE  PASSIVE  REMOTE  SENSING

(Question ITU-R 140/7)

(1994)

Rec. ITU-R SA.1028

The ITU Radiocommunication Assembly,

considering

a)	that certain frequency bands have been allocated for spaceborne passive microwave remote sensing;

b)	that some of these bands are also allocated to other radio services;

c)	that performance criteria are a necessary prerequisite to the establishment of interference and sharing criteria;

d)	that surface brightness temperature, the atmospheric temperature at points along a path, and absorption coefficients can be determined from measurements of the sensor antenna temperature, TA;

e)	that the surface brightness temperature and the absorption coefficients, in turn, depend upon the physical properties of the surface or atmosphere that are to be sensed;

f)	that studies have established measurement sensitivity requirements;

g)	that performance requirements for passive sensors can be stated in terms of measurement sensitivity, DTe, and availability, measured at the satellite, assuming that degradation from other elements in the system will be small,

recommends

1.	that the measurement sensitivities suitable for passive sensing of the Earth’s land, oceans and atmosphere are as shown in Table 1;

TABLE  1

| Frequency (GHz) | Primary application | Required DTe (K) |
| --- | --- | --- |
| Near 1.4 | Soil moisture, salinity | 0.1 |
| Near 2.7 | Salinity, soil moisture | 0.1 |
| Near 5 | Estuary surface temperature | 0.3 |
| Near 6 | Sea surface temperature | 0.3 |
| Near 11 | Rain(1), snow, ice, wind | 1.0 |
| Near 15 | Water vapour, rain | 0.2 |
| Near 18 | Rain(1), snow, ice, wind, water vapour | 1.0 |
| Near 21 | Water vapour, liquid water | 0.2 |
| 22.235 | Water vapour, liquid water | 0.4 |
| Near 24 | Water vapour, liquid water | 0.2 |
| Near 30 | Ice, oil spills, clouds | 0.2 |
| Near 37 | Rain, snow, ice | 1.0 |
| 50 to 66 | Atmospheric temperature profiling | 0.3/0.1(2) |
| Near 90 | Clouds, oil spills, ice, snow | 1.0 |
| Above 100 | Nitrous oxide, O3, CO, H2O, ClO, temperature | 0.2 |

- (1) Parameters given for this application.

- (2) Second number for pushbroom sensors.

2.	that in shared frequency bands, availability of passive sensor data as enumerated above shall exceed 95% of all locations in the sensor service area in the case where the loss occurs randomly and shall exceed 99% of all locations in the case where the loss occurs systematically at the same locations.

_________________