RECOMMENDATION ITU-R  SA.1158*

SHARING  OF  THE  1 675-1 710 MHz  BAND  BETWEEN  THE  
METEOROLOGICAL-SATELLITE  SERVICE  (SPACE-TO-EARTH)
AND  THE  MOBILE-SATELLITE  SERVICE  (EARTH-TO-SPACE)

(Question ITU-R 204/7)

(1995)

Rec. ITU-R SA.1158

The ITU Radiocommunication Assembly,

considering

a)	that the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) has allocated the 1 675-1 710 MHz band on a primary basis in Region 2 to the mobile-satellite service (MSS) (Earth-to-space) and maintained the primary status of the meteorological-satellite service (METSAT) (space-to-Earth);

b)	that each of these two services may be provided by geostationary satellite systems and non-geostationary satellite systems;

c)	that for more than 20 years the international group of METSAT service operators (CGMS) have agreed to separate the band 1 675-1 710 MHz into three sub-bands which are being used and are expected to continue to be used as follows:

1 675-1 690 MHz:	main earth stations at fixed locations for reception of raw image data, data collection data and spacecraft telemetry from geostationary meteorological satellites;

1 690-1 698 MHz:	user stations for direct readout services from geostationary meteorological satellites. (Some METSAT service operators currently use frequencies below 1 690 MHz to provide direct readout services from geostationary meteorological satellites.);

1 698-1 710 MHz:	user stations for direct readout services and prerecorded image data at main earth stations from non-geostationary meteorological satellites;

d)	that there exist hundreds of METSAT earth stations that receive direct data readout and data dissemination transmissions in parts of the 1 690-1 710 MHz band using small antennas, and that, moreover, thousands of similar stations are foreseen for some functions delivered by the METSAT service;

e)	that for different functions provided by the METSAT service, METSAT earth stations in the 1 690-1 710 MHz band can be fixed, mobile, or transportable;

f)	that Recommendation ITU-R SA.1027 provides sharing criteria for current METSAT systems using satellites in low-Earth orbit;

g)	that Recommendation ITU-R SA.1161 provides sharing criteria for current METSAT systems using satellites in geostationary orbit;

h)	that a large number of MSS earth station transmitters are expected to be deployed near or within a METSAT service area;

j)	that some meteorological agencies plan to increase the channel bandwidths and revise the frequency assignment plans for new generations of METSAT satellites;

k)	that geostationary METSAT space stations, which initially serve a certain area, may be relocated from time to time in order to provide coverage of another area;

l)	that Annexes 1, 2 and 3 provide a view pertaining to the technical sharing aspects of the METSAT and MS services operating in the 1 675-1 710 MHz band;

m)	that mobile-satellite techniques are either available or may be able to be developed to automatically and dynamically avoid transmissions from earth stations in the vicinity of receiving METSAT earth stations and that such techniques are described in Annex 3;

n)	that MSS systems can be developed that avoid the use of frequencies employed by METSAT systems for data dissemination to hundreds of small earth stations where dynamic interference protection may not be feasible,

noting

1	that techniques are available to operators of METSAT systems and of MS systems to further enhance the capability to share this frequency band,

recognizing

1	that WARC-92 decided that, in the band 1 675-1 710 MHz, stations in the MSS shall not cause harmful interference to, nor constrain the development of the meteorological-satellite and meteorological aids services and the use of this band shall be subject to the provisions of Resolution No. 46 (WARC-92) (Footnote 735A (WARC-92) to the Radio Regulations (RR));

2	that studies have indicated that potential interference from co-frequency MSS earth stations would be acceptable with typical separation distances of around 40 km and that the required separation distance is considerably reduced when the MSS earth station transmits on adjacent channels to those received by the METSAT earth station,

recommends

1	that additional studies be carried out to further advance possible sharing arrangements between MSS and METSAT earth stations, taking into account § c);

2	that transmitting MSS earth stations provide sufficient separation (spatial, time or frequency) such that unacceptable interference does not occur to METSAT earth stations already operating or planned to be implemented;

3	that additional studies be carried out to further evaluate interference from METSATs to MSS satellites, taking into account Annex 2.

ANNEX  1

The feasibility of co-channel and adjacent channel sharing between 
transmitting earth stations in the mobile-satellite service and
receiving earth stations in the meteorological-satellite
service in the frequency band 1 675-1 710 MHz

Part A  –  Co-channel sharing analyses

# 1	Introduction

Part A of this Annex addresses an example of co-channel sharing between transmitting low-Earth orbit (LEO) earth stations in the MSS and LEO receiving stations in the METSAT service.

However, the results of this section apply generally to other combinations of MSS and METSAT stations, e.g.:

–	LEO earth stations in the MSS, GEO earth stations in the METSAT service;

–	GEO earth stations in the MSS, LEO earth stations in the METSAT service;

–	GEO earth stations in the MSS, GEO earth stations in the METSAT service.

This is due to the fact that the distance estimates are relatively independent of the specific characteristics of the systems involved, and because of the exponential decrease in interference signal strength with increasing distance in the diffraction-attenuation over-the-horizon zone.

# 2	Typical High Resolution Picture Transmission (HRPT) link budget calculation

TABLE  1

| Carrier frequency 1 700 MHz | 5° elevation |
| --- | --- |
| e.i.r.p. (TIROS N) (dBm) | 38.2 |
| Space losses (dB) | –166 |
| Ground losses (dB) | –3.7 |
| Receiver station gain (2.44 m) (dBi) | 29.8 |
| Received signal level (2 668 kHz) (dBm) | –102 |

# 3	Estimated noise level in an HRPT receiver from MSS earth station transmissions (typical case)

The MSS communications density hypothesis of 0.025 E/km2 has been taken for the following interference calculation. This density gives a 6.3 km interval grid (see Fig. 1) between two MSS ground-transmitters and this results in a maximum distance from a METSAT earth station of 4.5 km. Considering that the METSAT antenna is directed towards the zenith, the interference link budget is as shown in Table 2.

FIGURE 1...[D01] = 8 CM

Within a 4.5 km radius, there could be typically about 4 MSS transmitters. Similarly, within a 10 km radius, there could be about 8 MSS transmitters. Therefore the total interference level will be approximately –66 dBm, as shown in Table 2.

TABLE  2

| Carrier frequency 1 700 MHz | Spread spectrum MSS (5 MHz)(1) | Narrow-band MSS (25 kHz) |
| --- | --- | --- |
| MSS ground transmitter e.i.r.p. (in 2 668 MHz HRPT receiver band) (dBm) | 37 | 40 |
| Space losses (4.5 km) (dB) | –110 | –110 |
| Receiving antenna gain towards horizon (dBi) | 0 | 0 |
| Interference level due to 1 MSS station (at 4.5 km) (dBm) | –73 | –70 |
| Interference level due to 4 MSS stations (at 4.5 km) (dBm) | – 67 | – 64 |
| Interference level due to all MSS stations (dBm) | – 66 | – 63 |
| (1) There are other bandwidths being considered for low-Earth orbiting MSS systems. |  |  |

# 4	Protection area for a METSAT earth station (LEO) operating in a shared band with MSS transmitting earth stations

HRPT service is provided by LEO satellites, where the system performance requires the link to be established for elevation angles ³5°.

## 4.1	Calculation of the minimum radius of the protection area of a HRPT receiving station to ensure that MSS interference does not exceed 20% of the time

This calculation is carried out in order to obtain the minimum separation distance necessary to ensure that an HRPT receiving station does not suffer constant interference from an MSS ground transmitter (this calculation does not apply to airborne transmitters).

For this purpose it is assumed that:

–	the MSS ground transmitter is transmitting in the band used by the HRPT receiver;

–	there is only one such transmitter in the vicinity of the HRPT receiving station; and

–	the gain of the MSS earth station is isotropic in the upper hemisphere of the antenna.

The minimum path loss required to prevent interference to a METSAT HRPT station is shown in Table 3.

TABLE  3

| Carrier frequency 1 700 MHz | Spread spectrum MSS (5 MHz)(1) | Narrow-band MSS (25 kHz) |
| --- | --- | --- |
| HRPT required signal level for 5° elevation angle(2) (dBm) | –102 | –102 |
| Permissible interference level(3) (dBm) | –115 | –115 |
| METSAT antenna gain towards horizon (directed towards 5° elevation angle)(4) (dBi) | 0 | 0 |
| MSS ground transmitter e.i.r.p. (in 2 668 MHz HRPT receiver band) (dBm) | 37 | 40 |
| Minimum path loss required (dB) | –152 | –155 |
| (1) There are other bandwidths being considered for low-Earth orbiting MSS systems. (2) See Table 1. (3) Value from Table 1 of Recommendation ITU-R SA.1027 (Terrestrial). (4) 2.44 m antenna: Gmax = 30 dBi, D/l = 13.8. |  |  |

The gain indicated in Table 3 corresponds to the value in the direction of the interference, that is not exceeded for at least 20% of the time. It was calculated using antenna position statistics for an 800 km LEO METSAT spacecraft.

The following calculation of the separation distance d (km) between METSAT and MSS stations employs free-space losses (Le) and Earth-diffraction losses (Ls), following Recommendations ITU-R P.452 and ITU-R P.526.

Le  =  – 60  +  20 log (l  /  4p)  –  20 log d	(1)

where l is in metres and d in kilometres,

Ls  =  F(X )  +  G(Y1)  +  G(Y 2)	(2)

with:

X  =  2.2  b  f 1/3  ae–2/3  d	(3)

where:

b  =	1 because f > 20 MHz

f :	frequency (MHz)

ae :	equivalent terrestrial radius (km)(8 500 km)

d :	distance (km)

with:

F(X )  =  11  +  10 log  X  –  17.6 X               and X  =  0.063 d

F(X )  =  –1  +  10 log d  –  1.1 d               dB	(4)

with:

Yi  =  9.6  ´  10–3  ´  b  f 2/3 ae–1/3  Hi	(5)

where:

Hi :  station or mobile antenna height (m).

In this example Y1 (10 m) = 0.67 and Y2 (3 m) = 0.2

with:

G(Y)  =  20 log (Y  +  0.1 Y 3)               for 10 K (here K  <  0.02)  <  Y  <  2	(6)

that gives G1(Y1) = –3.1 dB and G2(Y2) = –14 dB.

The equation to be solved is:

(7)

### 4.1.1	One single MSS transmitter case

Considering only one MSS narrow-band transmitter located near the HRPT station, the result for L = –155 dB is d = 24 km.

### 4.1.2	Multiple MSS transmitters case

With several MSS transmitters, at a density of 0.025 E/km2, the interferers shall be considered circularly distributed around the HRPT receive antenna of 0 dBi gain. On this protection circle of radius d km, there will be a minimum of (0.025 ´ 2p d ´ 6.3) transmitters spaced at 6.3 km (see § 3). The influence of MSS transmitters located beyond the

protection circle is disregarded. Thus the minimum path loss of –155 dB (from Table 3) has to be shared by (0.025 ´ 2p d ´ 6.3) transmitters. Based on the above assumptions, the calculations result in equation (9), where d is the minimum protection distance (km).

L(d) is the elemental attenuation (for just one transmitter).

–155  =  10 log d  +  L(d )               L(d )  =  –115.1  –  10 log d  –  1.1 d	(8)

–155  =  –115.1  –  1.1 d	(9)

A distance of 36 km is found for a narrow-band MSS transmitter (and 34 km for a spread-spectrum MSS transmitter).

NOTE 1 – For a given mobile earth station e.i.r.p., spread-spectrum MSS has no significant advantages over narrow-band MSS with respect to co-channel sharing with meteorological receivers. This is because in all cases, except for WEFAX, the victim receivers have wide bandwidths. In the case of WEFAX, the interfering spread-spectrum e.i.r.p. in the receiver passband bandwidth is much lower than that of the narrow-band system, but the distance separations are not proportionally lower because Fresnel zone clearance increases on the interfering signal path with reduced separation distances such that the dB/km losses are reduced.

## 4.2	Calculation of the minimum radius of the protection area of a HRPT receiving station to ensure that MSS interference does not exceed 0.002% of the time

The assumptions made for calculation purposes are the same as in § 4.1.

The minimum path loss required to prevent interference to a METSAT earth station is shown in Table 4.

TABLE  4

|  | Spread-spectrum MSS (5 MHz)(1) | Narrow-band MSS (25 kHz) |
| --- | --- | --- |
| HRPT required signal level for 5° elevation angle(2) (dBm) | –102 | –102 |
| Permissible interference level for an S/I of 6 dB according to ITU-R Recommendation SA.1027(3) (dBm) | –108 for less than 0.002% of the time | –108 for less than 0.002% of the time |
| HRPT ground antenna gain towards horizon 0° (directed towards 5° elevation angle)(4) (dBi) | 23.5 | 23.5 |
| MSS ground transmitter e.i.r.p. (in 2 668 MHz receiver band) (dBm) | 37 | 40 |
| Minimum path loss required (dB) | –168.5 | –171.5 |
| (1) There are other bandwidths being considered for low-Earth orbiting MSS systems. (2) See Table 1. (3) The level is –108 dBm for 2 668 MHz of passband and a 30 dBi antenna. (4) 2.44 m antenna: Gmax = 30 dBi, D/l = 13.8. |  |  |

Considering only one MSS ground transmitter operating with a 25 kHz bandwidth, the required separation is 38 km.

The calculation for MSS ground transmitters with 0.025 E/km2 density does not lead to a significantly higher separation distance, because only one MSS transmitter will be located within the METSAT main lobe of the antenna for an elevation angle of 5° (see Fig. 2).

FIGURE 2...[D02] = 18 CM

# 5	Summary

5.1	From the analysis in § 4, typical separation distances to prevent interference are in the range of 40 km (see Fig. 3).

5.2	As indicated in Fig. 3, reductions in MSS earth stations e.i.r.p. toward the horizon do not significantly change the separation distances.

5.3	It should be noted that in portions of the 1 675-1 710 MHz band, that METSAT earth stations, such as the HRPT or WEFAX, may be found in large numbers in certain parts of the world. In such cases, coordination with MSS earth stations may be difficult.

5.4	Annex 3 provides additional methods for potential interference avoidance.

FIGURE 3...[D03] = 13 CM

Part B  –  Non-co-channel analysis

# 1	Introduction

This section summarizes the feasibility of non-co-channel sharing between transmitting mobile earth stations and receiving earth stations in the METSAT service. This sharing situation is encountered not only in sharing techniques based on MSS use of frequencies that are not used by the METSAT service, but also in all co-channel sharing situations (co-channel sharing with one set of meteorological earth stations generally is accompanied by adjacent channel sharing with other meteorological earth stations); thus, non-co-channel sharing must be deemed feasible in order for co-channel sharing to be considered feasible. Although the parameters assumed herein for mobile earth stations pertain to narrow-band transmissions to a geostationary satellite, the same principles apply to any other kinds of earth stations in the MSS.

The interference mechanisms considered in this section include mobile earth station out-of-band emissions falling within the receiver passband and meteorological receiver gain compression (desensitization), spurious responses, and intermodulation products. The probability of a mobile earth station transmitting while located in the area around a meteorological earth station where interference may be perceptible was considered for each interference interaction and compared with the applicable sharing criteria. It is concluded that sharing on a non-co-channel basis is feasible, provided that appropriate limits are placed on mobile earth station e.i.r.p. and unwanted emission levels.

# 2	Separation distances for the prevention of interference

## 2.1	Desensitization

The separation distances between a meteorological earth station and the MSS earth terminal that result in an MSS signal level equal to the receiver saturation level (1 dB compression level) with the receiver operating at maximum sensitivity (e.g., during signal acquisition) range from 2 m to 56 m. The smallest distances being associated with CDA stations and the largest distances being associated with the WEFAX stations. Perceptible interference could occur through desensitization with higher than the assumed MSS interfering signal levels (i.e. the levels yielding 1 dB of gain compression).

## 2.2	Receiver spurious responses

The distance from the meteorological receiver where the MSS signal level at the receiver IF is at the same level as the receiver sensitivity level was calculated for a variety of conditions. The desired received signal level was assumed to be the same as the receiver sensitivity (this is valid for meteorological receivers acquiring the desired signals at low elevation angles). The off-tuning is assumed to be sufficient to achieve the 60 dB spurious rejection of the receiver. Separation distances were calculated for three types of meteorological earth stations (CDA, HRPT, and WEFAX) to provide a range of distances that result from different configurations (reflecting the different earth station bandwidths and gains). The three different receiver antenna gains considered reflect the discrimination of the receiver antenna when the interferer is in the mainbeam azimuth (i.e. at off-axis angles of 5° and 10° due to meteorological receiver antenna elevation), and in the backlobe of the receiver antenna. The results of separation distance calculations for receiver generated spurious responses assuming a 60 dB level of receiver spurious rejection, which is typical of the maximum achievable IF filter attenuation in the subject band, varied considerably. The distances for the CDA station were least sensitive, ranging from 26 m to 505 m; those for an HRPT station, from 95 m to 1 157 m and those for a WEFAX station, from 511 m to more than 10 km.

## 2.3	Receiver response to MSS spurious emission

Separation distances were calculated for three types of meteorological earth stations, CDA, HRPT, and WEFAX, due to a MSS spurious emission level in the receiver passband of –63 dBc based on the spurious emission level. Three types of earth stations were considered in order to provide a range of distances that result from different permissible levels of interference and antenna diameters. The receiver antenna gain reflects the discrimination of the antenna when the interferer is in the mainbeam azimuth (at off-axis angles of 5° and 10° due to the elevation of the meteorological receiver antenna) and in the backlobe of the receiver antenna. The signal level at the receiver antenna was compared to the permissible level of interference of –92 dBm for a CDA station, –111 dBm for an HRPT station and –120 dBm for a WEFAX station. These levels were taken from the sharing and coordination criteria given in Recommendation ITU-R SA.1027 for the non-geostationary METSAT systems and the criteria for the WEFAX station (operating with a geostationary satellite) were calculated using the method of Recommendation ITU-R SA.1023. The range of separation distances calculated were, for a CDA station, 10 m to 160 m; for an HRPT station, 228 m to 2 418 m; and for a WEFAX station, 658 m to 6 974 m.

# 3	Resulting probabilities of interference

Table 5 lists the probabilities of a MSS earth terminal being within the areas of potentially perceptible interference, as calculated using the largest above distances. The largest separation distances are associated with receiver spurious responses and greatly exceed the distances associated with other interference interactions; thus, the results encompass potential additive effects of interfering signal power associated with different interference interactions. The areas associated with two concurrently interfering MSS earth stations would be about twice as large as the area for a single interferer, but the probabilities of having two interfering earth stations would be much less than one-half the indicated probabilities (i.e. because of the low transmission duty cycles for each interfering earth station). The results indicate a lower probability of interference than is generally considered to be permissible in the METSAT service (e.g., 0.003% of the time, as in Recommendation ITU-R SA.1027).

TABLE  5

Probability of potentially perceptible interference

| Earth station type | Interference area (Aac) (km2) | Probability of interference |
| --- | --- | --- |
| CDA | 0.0142 | 2 ´ 10–8 |
| HRPT | 0.5298 | 7.3 ´ 10–6 |
| WEFAX | 3.27616 | 4.5 ´ 10–5 |
| NOTE 1 – The following were used for the above calculations: – Number of MSS channels = 666.7 – MSS satellite beam coverage area = 1.7 ´ 107 km2 – Transmission duty factor = 0.35. |  |  |

ANNEX  2

The feasibility of co-channel sharing between transmitting space stations 
in the meteorological-satellite service and receiving space stations 
in the mobile-satellite service in the frequency band 1 675-1 710 MHz

# 1	Introduction

If the 1 675-1 710 MHz band is to be used by the METSAT service in the downlink and by the MSS in the uplink, two interference paths can occur. One is from the transmitting mobile-satellite (MSS) earth station into the receiving METSAT earth station. That interference path and how to contain its effects is discussed in Annex 1. The other interference path is from the transmitting METSAT space station to the receiving MSS space station. The magnitudes of the interference that might be expected through this interference path are discussed in this Annex.

There are two modes of space-station-to-space-station interference that may occur. The first is between a transmitting and a receiving satellite in close proximity, and the second is between two such satellites when they are in almost antipodal locations with respect to one another. For each of these two modes there is a further division in that both services may employ satellites in either geostationary or non-geostationary orbit.

# 2	Interference between relatively antipodal satellites

Consider a near-antipodal configuration where the ray joining the two satellites is tangential to the Earth, such that their respective antenna discriminations are a minimum. As the satellites are moved closer together, the satellite antenna off-axis angles increase resulting in higher combined antenna discrimination. This total discrimination increases faster than the reduction in path loss, until in the case of both satellites in GSO, the two satellites can operate only a few degrees apart. For example, the gain at 90° off-axis for a GSO METSAT may be about –5 dBi for a discrimination of 23 dB. The gain at 90° off-axis for a 1° spot beam GSO MSS may be about 0 dBi for a discrimination of 44 dB giving a net discrimination of about 67 dB. The differential free space path loss between antipodal and 1° orbit spacing is about 41 dB. Therefore the close proximity case will generally be better than the antipodal case, until the angular separation between the two satellites is quite small.

For an assessment of the interference potential in the antipodal case, consider the geometry of Fig. 4. The path length from the METSAT satellite, orbit radius of R1, to the Earth horizon is d1, B1 is the METSAT boresight and j1 is the off-axis angle to the horizon. Similarly, the MSS radius, path length, boresight and off-axis angle to the horizon are R2, d2,

B2 and j2, respectively. Thus the interference signal path length for the antipodal configuration is d1 + d2. The wanted signal path length is d where the MSS service area is assumed to extend to an elevation angle of e.

FIGURE 4...[D04] = 9 CM

Then, at the MSS satellite, the wanted signal level is:

C  =  e.i.r.p.MSS(MES)  –  FSL(d )  +  GMSS(eoc)	(10)

The interference signal level is:

I  =  e.i.r.p.MET  –  FSL(d1  +  d2 )  –  DMET (j1)  +  GMSS  –  DMSS (j2)  –  BW factor	(11)

Thus the carrier-to-interference ratio is:

(12)

The variable meanings and assumptions of their values are given below. Due to the large variety of different systems, either worst case or mean values are used in the calculations.

e.i.r.p.MSS(MES) :  MSS earth station e.i.r.p.

–	for GSO MSS, ranges from 6 dBW (ETS-V) to 36 dBW (Inmarsat service A);

–	for LEO MSS, ranges from 13 dBW to –6 dBW;

–	assumed to be + 10 dBW for MSS(GSO) (see MSAT) and –6 dBW for MSS(LEO) (based on published parameters for IRIDIUM);

e.i.r.p.MET :  METSAT satellite e.i.r.p.

10 log [(d1 + d2) / d]2 :  differential free space loss between the wanted and interfering paths

–	LEO satellites in the METSAT service have altitudes ranging from 815 km to 925 km, an average value of 870 km is assumed;

–	LEO MSS satellites may have altitudes as low as 520 km. However, a typical value of 760 km is assumed;

–	edge of coverage (eoc) arrival angle is assumed to be at least 25° for mobile satellite systems in all modes of operation;

GMSS – GMSS(eoc) :  MSS satellite discrimination at its own edge of coverage;

–	assumed to be 3 dB for both LEO and GSO;

DMET (j1) :  METSAT satellite antenna discrimination to Earth horizon;

–	assumed to be 3 dB for METSAT(GSO) (Earth coverage) and 0 dB for METSAT(LEO) (omni antenna on LEO offering service down to 5° arrival angle);

DMSS (j2) :  MSS satellite antenna discrimination to Earth horizon;

–	for GSO MSS, satellite peak antenna gain ranges from 18 dBic (Earth coverage beam) to 34 dBic (spot beam);

–	for LEO MSS, satellite peak antenna gain ranges from 18.5 dBi (Earth coverage beam) to 28.8 dBi (spot beam);

–	assumed to be 20 dB for MSS(GSO) (spot beam) and 6 dB for MSS(LEO);

BW factor :  bandwidth factor = 10 log (bwMETSAT / bwMSS);

–	for bwMETSAT > bwMSS this term represents the fraction of the interfering signal that falls in band to the wanted signal;

–	for bwMSS > bwMETSAT this term represents the number of interfering signals that fall in band to the wanted signal assuming contiguous interfering channels;

–	the bandwidth for the MSS varies between about 5 kHz for continuous transmission to about 32 kHz for TDMA burst mode transmission. An average bandwidth of 18 kHz will be assumed.

The resulting C/I calculations are summarized in Table 6.

Under the assumption that a required C/I of 20 dB is adequate, antipodal space-station-to-space-station sharing between the METSAT service downlink and the MSS uplink appears quite feasible for the two scenarios in which the METSAT is in a low-Earth orbit.

For the case of GEO METSATS and GEO MSS it appears that sharing is feasible for the vast majority of the band. About 0.2% of the band is not usable by GEO MSS satellites due to excessive long term interference levels. MSS channels affected may be permanently disabled without significantly reducing traffic capacity. The meteorological agencies’ plans to increase bandwidths will help alleviate the problem if the satellites’ e.i.r.p. is not raised above the present levels. However, the meteorological agencies’ desire to revise frequency assignments will require MSS satellites to be reprogrammable so that channels may be enabled or disabled as required.

For the case of GEO METSATS and LEO MSS it appears that sharing is feasible for the vast majority of the band. Less than 0.9% of the band is not usable by LEO MSS satellites due to excessive interference levels. The effect on the MSS satellites and requirements to facilitate sharing are similar to those discussed in the previous paragraph. However, if further study shows that the interference by the METSATS at a LEO MSS satellite is of a short-term nature, it is possible that no MSS channels are adversely affected by interference. Otherwise the interference would be “systematic”, in that it would occur whenever the satellites were in the antipodal parts of their orbits.

To facilitate sharing with MSS in general, the GEO METSATS may standardize the frequencies used on future METSAT satellites so that transmissions that would cause high interference at the MSS satellite are known. Then, MSS systems could permanently disable the affected MSS channels, removing the need for in-orbit changes to MSS channel plans.

TABLE  6

Carrier-to-interference ratio for METSAT functions

| METSATS system | Function | Frequency (MHz) | RF bandwidth (MHz) | e.i.r.p. (dBW) | LEO MSS C/I (dB) | GEO MSS C/I (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Sensor | 1681.600 | 20.000 | 27.0 | 33.0 | 40.1 |
|  | S-VISSR | 1687.100 | 6.000 | 25.0 | 29.7 | 36.8 |
|  | WEFAX 1 | 1691.000 | 0.260 | 17.0 | 24.1 | 31.2 |
| GMS | WEFAX 2 | 1691.000 | 0.032 | 7.0 | 25.0 | 32.1 |
| (GSO) | Ranging 1 | 1684.000 | 1.000 | 17.0 | 29.9 | 37.0 |
|  | Ranging 2 | 1688.200 | 1.000 | -4.5 | 51.4 | 58.5 |
|  | Ranging 3 | 1690.200 | 1.000 | -4.5 | 51.4 | 58.5 |
|  | DCP report | 1694.500 | 0.400 | 4 | 39.0 | 46.1 |
|  | Telemetry | 1694.000 | 0.400 | 10 | 33.0 | 40.1 |
|  | Sensor W/B | 1676.000 | 5.000 | 19.0 | 34.9 | 42.0 |
|  | Sensor Raw Image | 1681.600 | 25.000 | 27.9 | 33.0 | 40.1 |
|  | Sensor Multi | 1681.478 | 0.500 | 19.0 | 24.9 | 32.0 |
|  | Sensor Mode AAA | 1685.700 | 5.000 | 19.0 | 34.9 | 42.0 |
|  | Ranging 1 | 1684.000 | 1.000 | 27.9 | 19.0 | 26.1 |
|  | Ranging 2 | 1688.200 | 1.000 | 27.9 | 19.0 | 26.1 |
| GOES | Ranging 3 | 1690.200 | 1.000 | 27.9 | 19.0 | 26.1 |
| (GSO) | Direct readout | 1687.100 | 3.500 | 27.9 | 24.5 | 31.6 |
|  | WEFAX | 1691.000 | 0.026 | 27.9 | 3.2 | 10.3 |
|  | Telemetry | 1694.000 | 0.020 | 19.0 | 11.0 | 18.1 |
|  | DCP report 1 | 1694.450 | 0.400 | 19.0 | 24.0 | 31.1 |
|  | DCP report 2 | 1694.500 | 0.400 | 21.1 | 21.9 | 29.0 |
|  | DCP report 3 | 1694.800 | 0.400 | 19.0 | 24.0 | 31.1 |
|  | DCP reports | 1675.281 | 0.435 | 12.5 | 30.8 | 37.9 |
|  | Telemetry | 1675.929 | 0.030 | 5.0 | 26.7 | 33.8 |

TABLE  6 (continued)

| METSATS system |  | Function | Frequency (MHz) | RF bandwidth (MHz) | e.i.r.p. (dBW) | LEO MSS C/I (dB) | GEO MSS C/I (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sensor |  | 1686.833 | 5.300 | 10.7 | 43.5 | 50.6 |
|  | Ranging 1 |  | 1691.000 | 0.660 | 21.3 | 23.8 | 30.9 |
|  | Ranging 2 |  | 1694.500 | 0.660 | 21.3 | 23.8 | 30.9 |
|  | Fax high res 1 |  | 1691.000 | 0.660 | 21.3 | 23.8 | 30.9 |
| METEOSAT | Fax high res 2 |  | 1694.500 | 0.660 | 21.3 | 23.8 | 30.9 |
| (GSO) | WEFAX 1 |  | 1691.000 | 0.026 | 21.3 | 9.8 | 16.9 |
|  | WEFAX 2 |  | 1694.500 | 0.026 | 21.3 | 9.8 | 16.9 |
|  | MDD |  | 1695.770 | 0.720 | 9.0 | 36.5 | 43.6 |
|  | HRIT |  | 1695.150 | 1.960 | 18.4 | 31.5 | 38.6 |
|  | LRIT |  | 1691.000 | 0.660 | 16.6 | 28.5 | 35.6 |
|  | Sensor |  | 1685.000 | 5.000 | 23 | 31.0 | 38.1 |
|  | WEFAX 1 |  | 1671.48 1690.8 | 0.018 | 18.8 | 10.7 | 17.8 |
| GOMS | WEFAX 2 |  | 1674.48 1691.4 | 0.018 | 18.8 | 10.7 | 17.8 |
| (GSO) | Fax high 1 |  | 1672.48 1691.0 | 0.0024 | 12.3 | 18.5 | 25.6 |
|  | Fax high 2 |  | 1673.48 1691.2 | 0.0024 | 12.3 | 18.5 | 25.6 |
|  | DCP 1 |  | 1697.0 | 2.000 (300 ´ 3 kHz) | 9.7 | 40.3 | 47.4 |
|  | DCP 2 |  | 1688.5 | 1.000 (100 ´ 10 kHz) | 12 | 35.0 | 42.1 |
| Typical LEO METSAT | Worst case |  | __ | 3.000 | 9.0 | 23.2 | 41.4 |

# 3	Interference between satellites in similar orbital positions

The other set of relative positions of transmitting METSAT and receiving MSS in which there may be harmful interference into the MSS receiver is when the satellites are in close proximity. For the GSO/GSO scenario this means that the two satellites must be at least a certain minimum distance apart on the geostationary orbital arc. For the other three scenarios, when one or more of the satellites is in a LEO orbit, the worst relative position of the two satellites is the transient condition in which the separation between them is minimized, i.e. when the interfering meteorological satellite is directly above or below the mobile satellite.

## 3.1	Required separation between a METSAT and a MSS on the GSO

When both the meteorological satellite and the mobile satellite are both on the GSO in close proximity, the desired carrier strength C is again:

C  =  e.i.r.p.MSS(MES)  –  FSL(d )  +  GMSS(eoc)	(13)

the interference level I is given by the equation:

I  =  e.i.r.p.MET  –  FSL(d3)  –  DMET (j1)  +  GMSS (0)  –  DMSS (j2)  –  BW factor	(14)

where all terms of the equations are the same as defined above in § 2 with the addition of the terms:

d3 :	distance between the two satellites

j1 :	angle off boresight of the MSS as seen from the METSAT

j2 :	angle off boresight of the METSAT as seen from the MSS.

From equations (13) and (14) the C/I at the MSS receiver is:

(15)

For the angles j1 and j2 , assumed to be large enough that the path between satellites is in the far sidelobes of both antennas, DMET (j1) and DMSS (j2) are both assumed to be at least 25 dB. The free-space loss FSL (d) between the MSS earth station and its space station, is assumed to be 189 dB, the value for this frequency band at low elevation angles. The required single-entry C/I is assumed to be 20 dB. The free-space loss FSL (d3) for transmissions from the METSAT to the MSS on the GSO is given by the expression:

FSL(d3)  =  92.5  +  20 log f  +  20 log d3	(16)

where f is the carrier frequency, assumed to be 1 685 GHz. Other parameters are assumed to have the same values as those discussed in § 2 above. Substituting (16) and values for all parameters except the unknown d3, equation (15) becomes:

20 log d3  =  75               or d3  =  5 623 km	(17)

The necessary angular spacing between the satellites to achieve this distance is 7.54°.

This required separation is not large in terms of the expected number of geostationary meteorological satellites and the necessary spacing between geostationary mobile satellites using the same spectrum in the 1 675-1 710 MHz band, and can easily be coordinated between operators of geostationary meteorological and mobile satellites by reasonable selection of their orbital positions. A second avenue open for coordination between them is carrier selection, such that the parameter BW factor is better than 0 dB. Also, it is noted that even with maximum squint between the satellites at 7.5° separation on the GSO, the off-axis angles j1 and j2 of the satellite antennas is at least 77°, consistent with the assumption that DMET(j1) and DMSS(j2) are at least 25 dB.

Thus it is concluded that satellite-to-satellite interference between geostationary METSAT and MSS satellites may require some coordination, but is not a significant factor in sharing of the band between the two services.

## 3.2	Transient interference into the mobile satellite when either satellite, or both, are in a LEO orbit

In any one of the GSO/LEO, LEO/LEO, or LEO/GSO scenarios the worst “nearby” alignment of satellites is when the lower satellite is in the main beam of the higher satellite. In each of these scenarios the C/I at the mobile-satellite receiver is given by the equation:

(18)

where:

FSLC :	free-space loss of the carrier from the mobile-satellite earth station to the mobile satellite

FSLI :	free-space loss of the interfering signal from the METSAT to the mobile satellite

DMET (j3) :	antenna discrimination of the METSAT antenna j3 from boresight in the direction of the mobile satellite

DMSS (j4) :	discrimination of the mobile satellite antenna j4 from boresight in the direction of the METSAT.

It is assumed that the backlobe discrimination of either satellite 180° off boresight is at least 20 dB. Thus the term (DMET(j3) + DMSS(j4)) is 20 dB for each of the three scenarios, independent of which satellite is the higher. Assuming all other terms to be the same as those indicated in § 2 above, the worst-case C/I for each scenario is given in Table 7.

TABLE  7

Worst-case C/I for “in-line” relative or bit positions

| Interference mode METSAT ® MSS | MSS MES e.i.r.p. (dBW) | METSAT e.i.r.p. (dBW) | D LFSL (dB) | MSS EOC discrimination (dBW) | METSAT antenna discrimination (dBW) | MSS antenna discrimination (dBW) | BW factor (dB) | C/I (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSO/LEO | –6.0 | 20.0 | +27.6 | 3.0 | 0.0 | 20.0 | 0.0 | 18.6 |
| LEO/LEO | –6.0 | 9.0 | –22.6 | 3.0 | 0.0 | 20.0 | 22.2 | 1.4(1) |
| LEO/GSO | 10.0 | 9.0 | –0.2 | 3.0 | 20.0 | 0.0 | 22.2 | 40.0 |
| (1) This low C/I occurs in rare transient events lasting of the order of 2 min, as a result of the two LEO satellites being temporarily in similar locations. 1.4 dB is the minimum C/I that would be experienced in any such event, if and when the two satellites were directly above the same position on the Earth. Most transient events would experience a minimum C/I greater than this value because the orbits of the two satellites would be such that the minimum separation between them during the event would be greater. Over the long term, the single-entry C/I due to interference from one meteorological satellite in a LEO would be less than 3 dB for less than 0.001% of the time, and would be less than 15 dB for less than 0.1% of the time. |  |  |  |  |  |  |  |  |

In the two scenarios in which either the METSAT or the MSS is in the GSO the worst transient interference is either above the required 20 dB or close enough to that requirement so that minor corrections to system parameters in the coordination between them could result in an acceptable interference environment for the mobile-satellite network.

In the scenario in which both satellite networks are in a low-Earth orbit, there will be relatively rare short-duration events in which the MSS would experience C/Is as low as 2 dB. However, there are techniques available to the mobile-satellite operator to avoid these predictable transient events, and to constrain their seriousness in the design of the system. Avoidance techniques include dynamic mobile channel selection, and temporary use of another mobile satellite. Design options available to the mobile-satellite operator include improved backlobe characteristics of the spacecraft antenna, or choice of a LEO orbit at significantly different altitudes than that used by METSATs.

# 4	Summary

The interference from transmitting METSAT space stations to receiving mobile-satellite space stations is examined for typical satellite system characteristics in all combinations of GSO and LEO orbits of each type of satellite. The results of this examination are summarized in Table 8.

TABLE  8

Summary of interference from METSAT to MSS space stations

| METSAT orbit | MSS orbit | From antipodal position | From nearby position |
| --- | --- | --- | --- |
| GSO | GSO | C/I = 10.3 dB | Spacing of 7.5° |
| GSO | LEO | C/I = 3.2 dB | C/I = 18.6 dB |
| LEO | LEO | C/I = 23.2 dB | C/I =1.4 dB minimum(1) |
| LEO | GSO | C/I = 41.4 dB | C/I = 40.0 dB |
| (1) Single-entry C/I levels may be as low as 1.4 dB in relatively rare transient events lasting of the order of 2 min. In the long term the C/I will be less than 3 dB for less than 0.001% of the time and less than 15 dB for less than 0.1% of the time. |  |  |  |

When the mobile-satellite is in a GSO orbit single-entry interference from a METSAT is expected to be greater than 20 dB or close enough to that requirement that coordination between the networks could achieve that objective. When the mobile-satellite is in a LEO orbit there may be significant transient interference from a GSO METSAT in a relatively antipodal location, or from a LEO METSAT in close proximity. There are design and operational measures that a LEO MSS operator can take to avoid these low C/I predictable transient events.

ANNEX  3

Sharing techniques for mobile-satellite earth station and meteorological-satellite
earth stations in the 1 675-1 710 frequency band

A number of techniques have been studied by the ITU-R to enhance the capability to share the radio spectrum between mobile or mobile-satellite systems and systems of other services. The basic problem addressed in these studies is that when the mobile service or SMS shares a frequency band with another service, the mobile station or the mobile-satellite earth station has been assumed to be operating anywhere in the service area of the victim system, whilst transmitting at the same frequency as the victim unit receives. Thus, these studies found that within the service area, the mobile or mobile-satellite earth station could cause harmful interference to stations of the other service.

These mobile or mobile-satellite earth stations must be assumed to be used by persons not accustomed to taking measures to avoid harmful radio interference between stations. For that reason the techniques implemented to control the magnitude of the interference within agreed-to limits must function without action being required by the user of the mobile or mobile-satellite earth station. Several such techniques that could be applied to limit the interference from a transmitting mobile-satellite earth station into a receiving meteorological-satellite earth station are described briefly here. The techniques which can be employed individually or jointly are:

1.	Frequency assignment by location.

2.	Beacon-actuated protection zones.

3.	Interference avoidance by frequency selection.

4.	Using frequencies in an MSS beam coverage area only when the METSAT earth stations are not using them (i.e., time sharing).

# 1	Frequency assignment by location

## 1.1	Method of assuring adequate frequency-distance separation (for the fixed exclusion zone case)

Using an interference-free signalling channel, the mobile earth station reports its location to the network operations centre (this capability is inherent in some planned non-GEO MSS systems). Interference-free working channels are then assigned, based on a computer “look-up” table indicating the frequencies whose use will not cause interference in the reported location and a list of frequencies not already assigned in the beam coverage area. The “look-up” table is based on known location and frequency assignments for the meteorological-satellite earth stations.

## 1.2	Comments

–	MSS signalling channels that will not cause harmful interference must be available for use throughout each MSS satellite coverage area.

–	MSS earth stations must inherently have, or be equipped with, position determination capabilities.

–	MSS earth station location must be known by the network control centre prior to being assigned a service channel.

–	Software and a database for assignment based on MSS earth station location must be integrated with the provisions for other channel assignment algorithms.

–	The network control computer system should be able to maintain acceptable network access delay.

# 2	Beacon-actuated protection zones

## 2.1	A flexible method of assuring adequate frequency-distance separation

A beacon transmitter is co-located with each METSAT receiving earth station to be protected with minimum acceptable frequency offsets between the beacon and the METSAT earth station receiver. The mobile-satellite earth station uses the beacon signal to determine whether it is in a restricted-frequency zone. This information is conveyed to the network operation centre, which assigns a channel that will not cause interference for use in the restricted-frequency zone when necessary.

## 2.2	Comments

–	MSS signalling channels that will not cause harmful interference must be available for use throughout each MSS satellite coverage area.

–	Beacons must be installed (practical only if there are a small number of receivers to be protected) at each METSAT earth station to be protected.

–	MSS earth stations must be equipped with beacon-signal processing capabilities.

–	MSS earth stations location (or the specific beacon zone the MSS earth station is within) must be known by the network operation centre prior to channel assignment.

–	Software and a database for assignment based on MSS earth station location in relation to specific beacons must be integrated with the provisions for other channel assignment algorithms.

–	The network control computer system should be able to maintain acceptable network access delay.

–	The technique also may facilitate time sharing.

# 3	Interference avoidance by frequency selection

## 3.1	Method to avoid interference to METSAT earth station types with many installations

The above interference avoidance techniques are appropriate for the case where only a few METSAT earth stations are used to receive signals from a METSAT (e.g., raw image data). However, these techniques are not suitable for the case where there are hundreds or thousands of small earth stations used in meteorological data distribution, e.g., for WEFAX, HRPT etc. These frequencies may be different for different METSAT systems and moreover, there may be some METSAT data distribution services that may not become ubiquitous.

These data distribution channels are generally quite narrow. Interference to these ubiquitous meteorological earth stations is avoided by having the MSS system not use the frequencies employed by the METSAT data distribution channels and a suitable guardband around them.

## 3.2	Comments

–	MSS signalling channels that will not cause harmful interference must be available.

–	Because the data distribution channels have a narrow bandwidth, the diminution of frequencies and capacity to an MSS system will probably be acceptable.

–	For non-GEO MSS systems, their network control centres must have the capability to recognize and adopt flexible frequency assignment protocols because different METSAT systems with different coverage areas may employ different frequencies and bandwidths for their data distribution channels.

–	Some parts of the world may not ubiquitously install small meteorological data distribution earth stations. MSS earth stations may be useful in such areas.

# 4	Using frequencies in an MSS beam coverage area only when the METSAT earth stations are not using them

## 4.1	Time sharing of frequencies

This is an old idea that has been in use in the METSAT field by non-GEO space stations for some time. That is, a non-GEO space station only serves a small part of the Earth’s surface at any instant of time. Thus, the same frequencies employed by the space station at that time can be employed on the rest of the world’s surface at that time. In other words, time share the use of the frequencies at all locations on the surface of the Earth between non-GEO METSATs and MSS systems.

## 4.2	Comments

–	MSS signalling channels that will not cause harmful interference must be available.

–	In the case at hand, there is a potential for interference from the METSAT space stations into the receivers of the MSS space stations. That concern is discussed in Annex 2.

–	The MSS network control centre must keep track of orbital locations and coverage of its own as well as the non-GEO METSAT space stations.

–	This technique may be used in conjunction with the beacon and fixed exclusion zone methods described above.

–	Good liaison channels must be established between MSS and METSAT system operators.

–	For multibeam MSS systems, this method may be used on a beam-by-beam basis.

_________________

* 	This Recommendation should be brought to the attention of Radiocommunication Study Group 8.