RECOMMENDATION  ITU-R  SA.1160-2

INTERFERENCE  CRITERIA  FOR  DATA  DISSEMINATION  AND  DIRECT
DATA  READOUT  SYSTEMS  IN  THE  EARTH  EXPLORATION-SATELLITE  AND  
METEOROLOGICAL-SATELLITE  SERVICES  USING  SATELLITES
IN  THE  GEOSTATIONARY  ORBIT

(Question ITU-R 141/7)

(1995-1997-1999)

Rec. ITU-R SA.1160-2

The ITU Radiocommunication Assembly,

considering

a)	that interference criteria are needed to ensure that systems can be designed to achieve adequate performance in the presence of interference;

b)	that interference criteria may be determined using the methodology described in Recommendation ITU-R SA.1022 and the performance objectives listed in Recommendation ITU-R SA.1159;

c)	that interference criteria assist in the development of criteria for sharing bands among systems, including those operating in other services;

d)	that systems in the Earth exploration-satellite service (EESS) and meteorological-satellite (MetSat) service must specify interference thresholds at levels greater than or equal to the permissible levels;

e)	that Annex 1 presents the parameters of representative systems that provide the basis for permissible levels of interference for pertinent transmissions in the EESS and MetSat service,

recommends

1	that the interference levels specified in Table 1 be used as the permissible total levels of interfering signal power at the antenna output of stations operating in the EESS and MetSat service.

TABLE  1

Interference criteria for stations in the EESS and MetSat service
using spacecraft in the geostationary orbit

| Frequency band (MHz) | Function and type of earth station | Interfering signal power (dBW) in the reference bandwidth to be exceeded for no more than 20% of the time | Interfering signal power (dBW) in the reference bandwidth to be exceeded for no more than p% of the time |
| --- | --- | --- | --- |
|  | Direct data readout High gain antenna | –153.8 dBW per 2.6 MHz | –148.6 dBW per 2.6 MHz p  0.025 |
| 1 670-1 710 space-to-Earth | Data dissemination Low gain antenna | –167.5 dBW per 4 kHz | –160.4 dBW per 4 kHz p  0.025 |
|  | Data dissemination High gain antenna | –153.4 dBW per 2.11 MHz | –148.1 dBW per 2.11 MHz p  0.025 |
| 2 025-2 110 Earth-to-space | Command data acquisition (CDA) station High resolution data | –136.7 dBW per 2.11 MHz | –133.4 dBW per 2.11 MHz p  0.025 |
|  | CDA station WEFAX data | –147.8 dBW per 4 kHz | –140.7 dBW per 4 kHz p  0.025 |
| 25 500-27 000 space-to-Earth | Direct data readout | –131.7 dBW per 10 MHz | –116.3 dBW per 10 MHz p  0.25 |

NOTE 1 – The interfering signal powers (dBW) in the reference bandwidths are specified for reception at elevation angles  3. NOTE 2 – The total interfering signal power level that may be exceeded for no more than x% of the time, where x is less than 20% but greater than the specified short-term time percentage (p% of the time), may be determined by interpolation between the specified values using a logarithmic scale (base 10) for percentage of time and a linear scale for interfering signal power density (dB). NOTE 3 – The sharing criteria can be expressed as permissible power flux-densities into the main beam of the receive antenna by subtracting 10 log(G 2/4) from the value given in Table 1, where G is the receive antenna gain and  is the wavelength. NOTE 4 – Although the interference criteria are based on the systems described in Annex 1, the interference criteria apply to all systems that operate in the subject frequency bands and which provide the specified service functions.

ANNEX  1

Basis for interference criteria

This Annex presents the parameters used as inputs to the methodology of Recommendation ITU-R SA.1022 to determine the interference criteria for data dissemination and direct readout systems.

# 1	Direct readout systems

Table 2 develops these criteria for direct readout systems, in which all of the interference enters the receiving station directly, and none is received at the station via the satellite that originates the data.

The interference criteria can be expressed as permissible power flux-densities into the main beam of the receive antenna by subtracting 10 log(G 2/4) from the values given in Table 2, where G is the receive antenna gain and  is the wavelength.

TABLE  2

Direct data readout system performance used as a basis for interference criteria
of stations operating with satellites in geostationary orbit

a) Frequency band 1 670-1 710 MHz

| Link parameter |  | Value | Notes |
| --- | --- | --- | --- |
| Down-link e.i.r.p. |  | 16.1 dBW |  |
| Down-link loss |  | 190.1 dB | Free-space, polarization, and antenna pointing |
| Down-lnk G/T |  | 24.4 dB(K–1) |  |
| Down-link C/N0 |  | 79.0 dB/Hz |  |
| Data rate |  | 2.6 Mbit/s |  |
| Required C/N0 |  | 78.1 dB/Hz | BER  1  10–6 2.2 dB implementation loss 1 dB modulation loss |
| Margin |  | 0.9 dB | Long-term and short-term |
| Receive antenna gain |  | 45.1 dBi |  |
| Receiver noise density |  | –207.9 dB(W/Hz) |  |
| Interference criteria | Long-term | –153.8 dB(W/2.6 MHz) | q  1/3 and Mmin  1.2 dB |
|  | Short-term | –148.6 dB(W/2.6 MHz) | q  1 and Mmin  1.2 dB |

TABLE  2 (continued)

b) Frequency band 25.5-27.0 GHz

| Link parameter |  | Value | Notes |
| --- | --- | --- | --- |
| Down-link e.i.r.p. |  | 50.8 dBW |  |
| Down-link loss | Long-term | 213.2 dB | Free-space, polarization, and antenna pointing |
|  | Short-term | 220.3 dB | 7.1 dB excess path loss |
| Down-link G/T |  | 31.6 dB(K–1) |  |
| Down-link C/N0 | Long-term | 97.8 dB/Hz |  |
|  | Short-term | 90.7 dB/Hz |  |
| Data rate |  | 15 Mbit/s |  |
| Required C/N0 |  | 83.8 dB/Hz | BER  1  10–7 0.5 dB implementation loss 1 dB modulation loss |
| Margin | Long-term | 14.0 dB |  |
|  | Short-term | 6.9 dB |  |
| Receive antenna gain |  | 60.1 dBi |  |
| Receiver noise density |  | –200.1 dB(W/Hz) |  |
| Interference criteria | Long-term | –131.7 dB(W/10 MHz) | q  1/3 and Mmin  1.2 dB |
|  | Short-term | –116.3 dB(W/10 MHz) | q  1 and Mmin  1.2 dB |

# 2	Data dissemination systems

Dissemination of high-resolution processed data and lower-resolution WEFAX data is affected by interference received at the station via the satellite as well as by interference transmitted directly into the station in the 1 670-1 710 MHz band. The high-resolution processed data and the WEFAX data are up-linked to the satellite in the 2 025-2 110 MHz band, and relayed, along with interfering signals entering the satellite in the same band, to the Earth station receivers via fixed-gain satellite transponders.

The up-link and down-link carrier-to-noise plus interference density ratios are respectively:

CN + I0 up = CN0up1 + I01k T1

and

CN0 + I0 down = CN0down1 + I02k T2

where:

I01 and I02 :	interference densities transmitted into the satellite and station receivers

T1 and T2 :	system noise temperatures of the satellite and station receivers

k :		Boltzmann’s constant.

The composite carrier-to-noise plus interference density ratio is:

CN0 + I0 = CN0 + I0 up–1 + CN0 + I0 down–1–1

From Recommendation ITU-R SA.1022 this can also be written:

CN0 + I0 = M–q CN0

where:

M :	interference-free margin

q :	action of the interference-free margin that the interference is allowed to consume

C/N0 :	composite carrier-to-noise density ratio given by:

CN0 = CN0up–1 + CN0down–1–1

From the foregoing equations:

Mq = 1 + I01k T1 CN0 up + I02k T2 CN0 downCN0 up + CN0 down

Assume that the up-link and down-link interference are allocated so that a fraction p of the interference received at the earth station is received via the satellite, and that a fraction 1 – p is transmitted directly into the station. It is desirable for p to be near 1/2 in order to provide a reasonable balance in the interference allocated to the up-link and to the down-link. For a fixed-gain transponder it can be shown that:

I02k T2 = 1 – pp I01k T1 CN0 downCN0 up

so that:

Mq = 1 + 1p I01k T1 1 + CN0 upCN0 down–1

Accordingly, the permissible up-link interference density  becomes:

I01 = 1 + p k T1 1 + CN0 upCN0 down Mq – 1	for M  Mmín

where, according to Recommendation ITU-R SA.1022, Mmin is the smallest interference-free margin for which only a fraction q of the margin is consumed by the interference. Correspondingly, the permissible down-link interference density is:

I02 = 1 – p k T2 1 + CN0 downCN0 up Mq – 1	for M  Mmin

Tables 3 and 4 summarize the calculation of I01 and I02 for high-resolution and WEFAX data respectively, assuming that p  1/2, q  1/3, and Mmin  1.2 dB for long-term interference, and that p  1/2, q  1, and Mmin  1.2 dB for Short-term interference.

The interference criteria can be expressed as permissible power flux-densities into the main beam of the receive antenna by subtracting 10 log(G 2/4) from the values given in Tables 3 and 4, where G is the receive antenna gain and  is the wavelength.

TABLE  3

Performance analysis used as a basis for interference criteria
of high-resolution data dissemination systems using geostationary satellites

| Link parameter |  | Value | Notes |
| --- | --- | --- | --- |
| Up-link e.i.r.p. |  | 72.1 dBW |  |
| Up-link loss |  | 191.7 dB | Free-space, polarization, and antenna pointing |
| Up-link G/T |  | –17.5 dB(K–1) | Post-launch measurement |
| Up-link C/N0 |  | 91.5 dB/Hz |  |
| Down-link e.i.r.p. |  | 23.8 dBW |  |
| Down-link loss |  | 190.1 dB | Free-space, polarization, and antenna pointing |
| Down-link G/T |  | 15.2 dB(K–1) |  |
| Down-link C/N0 |  | 77.5 dB/Hz |  |
| Composite C/N0 |  | 77.3 dB/Hz |  |
| Data rate |  | 2.11 Mbit/s |  |
| Required C/N0 |  | 75.9 dB/Hz | BER  1  10–6 1.9 dB implementation loss |
| Margin |  | 1.4 dB |  |
| Up-link receive antenna gain |  | 9.5 dBi |  |
| Up-link noise density |  | –201.6 dB(W/Hz) | T  500 K |
| Up-link interference criterion | Long-term | –136.7 dB(W/2.11 MHz) | q  1/3 |
|  | Short-term | –133.4 dB(W/2.11 MHz) | q  1 |
| Down-link receive antenna gain |  | 39.5 dBi |  |
| Down-link noise density |  | –204.3 dB(W/Hz) | T  269 K |
| Down-link interference criterion | Long-term | –153.4 dB(W/2.11 MHz) | q  1/3 |
|  | Short-term | –148.1 dB(W/2.11 MHz) | q  1 |

TABLE  4

Performance analysis used as a basis for interference criteria
of WEFAX data dissemination systems using geostationary satellites

| Link parameter | Value | Notes |
| --- | --- | --- |
| Up-link e.i.r.p. | 66.7 dBW |  |
| Up-link loss | 191.7 dB | Free-space, polarization, and antenna pointing |
| Up-link G/T | –17.5 dB(K–1) | Post-launch measurement |
| Up-link C/N0 | 86.1 dB/Hz |  |
| Down-link e.i.r.p. | 24.3 dBW |  |
| Down-link loss | 189.6 dB | Free-space and polarization |
| Down-link G/T | –0.3 dB(K–1) |  |
| Down-link C/N0 | 63.0 dB/Hz |  |

TABLE  4 (continued)

Performance analysis used as a basis for interference criteria
of WEFAX data dissemination systems using geostationary satellites

| Link parameter |  | Value | Notes |
| --- | --- | --- | --- |
| Composite C/N0 |  | 63.0 dB/Hz |  |
| IF bandwidth |  | 50 kHz |  |
| Required C/N0 |  | 57.0 dB/Hz | 10 dB FM threshold |
| Margin |  | 6.0 dB |  |
| Up-link receive antenna gain |  | 9.5 dBi |  |
| Up-link noise density |  | –201.6 dB(W/Hz) | T  500 K |
| Up-link interference criterion | Long-term | –147.8 dB(W/4 kHz) | q  1/3 |
|  | Short-term | –140.7 dB(W/4 kHz) | q  1 |
| Down-link receive antenna gain |  | 30.1 dBi |  |
| Down-link noise density |  | –198.2 dB(W/Hz) | T  1 100 K |
| Down-link interference criterion | Long-term | –167.5 dB(W/4 kHz) | q  1/3 |
|  | Short-term | –160.4 dB(W/4 kHz) | q  1 |