Recommendation ITU-R SA.509-3 (12/2013)

Space research earth station and radio astronomy reference antenna radiation pattern for use in interference calculations, including coordination procedures, for frequencies less than 30 GHz

SA Series Space applications and meteorology

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2014

ITU 2014

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  SA.509-3

Space research earth station and radio astronomy reference antenna radiation pattern for use in interference calculations, including coordination
procedures, for frequencies less than 30 GHz

(1978-1990-1998-2013)

Scope

This Recommendation gives reference antenna radiation patterns for space research and radio astronomy services for calculating interference from a single source or from multiple sources for frequencies less than 30 GHz.

The ITU Radiocommunication Assembly,

considering

a)	that the application of coordination procedures between space research earth stations or radio astronomy observatories and stations of other services is dependent upon specific antenna radiation patterns;

b)	that where this information does not exist, it may be desirable to use a reference antenna radiation pattern which represents the side-lobe gain levels that are not expected to be exceeded at most off-axis angles in the majority of antennas used in the service;

c)	that measured data from some large (D/  100) parabolic Cassegrain antennas used in the Space Research Service indicate an off-axis discrimination that is as good as, or better than, that of the reference antenna radiation pattern;

d)	that, in the case of aggregate interference from multiple interferers, using the peak envelope radiation pattern could overestimate the interference,

recommends

1	that in the absence of measured data on the levels of main-lobe or side-lobe response of a space research earth station or radio astronomy antenna which is subject to interference analyses or coordination procedures, the following reference antenna radiation patterns be used only for large parabolic antennas with D/  100 and for frequencies between about 1 and 30 GHz:

1.1	in cases of a single entry interference, to predict the worst-case interference from this source, the following reference antenna radiation pattern (see Fig. 1) be used:

Gφ=G0-3(φφ0)2:0≤φ<φ1G0-17 :φ1≤φ<φ232-25logφ :φ2≤φ<48-10 :48≤φ<80-5 :80≤φ<120-10 :120≤φ≤180

where:

G():	gain (dBi) relative to an isotropic antenna

:	off-axis angle (degrees)

G0:	maximum boresight gain of the antenna (dBi)

0:	½ of the 3-dB beamwidth of the antenna (degrees)

φ1=	φ0173 (degrees)

φ2=	10(49-G0)/25 (degrees);

1.2	in cases of multiple entry interference, to predict the aggregate interference from these multiple sources, the following reference antenna radiation pattern (see Fig. 2) be used:

Gφ= G0-3(φφ0)2:0≤φ<φ1G0-20 :φ1≤φ<φ229-25logφ :φ2≤φ<48-13 :48≤φ<80-8 :80≤φ<120-13 :120≤φ≤180

where:

G():	gain (dBi) relative to an isotropic antenna

:	off-axis angle (degrees)

G0:	maximum boresight gain of the antenna (dBi)

0:	½ of the 3-dB beamwidth of the antenna (degrees)

φ1=	φ0203 (degrees)

φ2=	10(49-G0)/25 (degrees);

1.3	if the actual G0 and φ0 parameters are not available, then the following equations be used to estimate them:

G0=10logηπDλ2 (dBi) φ0=20√3D/λ (degrees)

where:

η:	aperture efficiency of the antenna

D:	antenna diameter (m)

λ:	wavelength (m);

2	that administrations be invited to submit measured antenna radiation patterns (see Annex) which may be used, if necessary, to revise the reference antenna radiation patterns of recommends 1.

FIGURE 1

Reference antenna radiation pattern to be used in the absence of
measured data (single entry interference)

FIGURE 2

Reference antenna radiation pattern to be used in the absence
of measured data (multiple entry interference)

Annex 

Measured radiation patterns of space research
earth station and radio astronomy antennas

# 1	Lovell Mk1A radio astronomy antenna

Figure A.1 below shows the measured gain of the Lovell Mk1A radio astronomy antenna at 1 420 MHz. This antenna has a single reflector of circular aperture and a diameter of 76.2 m. The peak in the measured response at around 95° is due to spillover.

FIGURE A.1

Measured side-lobe pattern at 1 420 MHz