RECOMMENDATION  ITU-R  SA.514-2

INTERFERENCE  CRITERIA  FOR  COMMAND  AND  DATA  TRANSMISSION  SYSTEMS  
OPERATING  IN  THE  EARTH  EXPLORATION-SATELLITE  AND
METEOROLOGICAL-SATELLITE  SERVICES

(Questions ITU-R 139/7 and ITU-R 141/7)

(1978-1990-1994)

Rec. ITU-R SA.514-2

The ITU Radiocommunication Assembly,

considering

a)	that Recommendation ITU-R SA.1026 contains interference criteria for space-to-Earth links in some frequency bands;

b)	that studies are needed to establish interference criteria for other bands and for Earth-to-space links, according to the methodology of Recommendation ITU-R SA.1022;

c)	that in the meantime, it is necessary to have interference criteria for all command and data transmission systems operating in the Earth exploration-satellite and meteorological-satellite services,

recommends

1.	that, for space-to-Earth data transmission links in bands other than those treated in Recommendation ITU-R SA.1026, the interference criteria for Earth receiving sites be established as follows: for frequencies between 1 and 10 GHz, the power spectral density of noise-like interference or the total power of CW-type interference in any single band or in all sets of bands shall not exceed –154 dB(W/MHz) at the receiver input for more than 1% of the time; for frequencies less than 1 GHz, the permissible interference may increase at the rate of 20 dB per decreasing frequency decade;

2.	that, for Earth-to-space command and data transmission links, the interference criteria for near-Earth spacecraft receivers be established as follows: for frequencies between 300 MHz and 10 GHz, the power spectral density of noise-like interference or the total power of CW-type interference in any single band or in all sets of bands 1 kHz wide shall not exceed –161 dB(W/kHz) at the receiver input for more than 0.1% of the time; for frequencies less than 300 MHz, the permissible interference may increase at the rate of 20 dB per decreasing frequency decade.

_________________