RECOMMENDATION  ITU-R  SA.514-3

INTERFERENCE  CRITERIA  FOR  COMMAND  AND  DATA  TRANSMISSION  
SYSTEMS  OPERATING  IN  THE  EARTH  EXPLORATION-SATELLITE  
AND  METEOROLOGICAL-SATELLITE  SERVICES

(Questions ITU-R 139/7 and ITU-R 141/7)

(1978-1990-1994-1997)

Rec. ITU-R SA.514-3

The ITU Radiocommunication Assembly,

considering

a)	that Recommendations ITU-R SA.1026, ITU-R SA.1160 and ITU-R SA.1163 contain interference criteria for space-to-Earth links in some frequency bands;

b)	that Recommendations ITU-R SA.1160 and ITU-R SA.1163 contain interference criteria for Earth-to-space links in some frequency bands;

c)	that it is necessary to have interference criteria for all command and data transmission systems operating in the Earth exploration-satellite and meteorological-satellite services,

recommends

1	that, for space-to-Earth data transmission links in bands other than those treated in Recommendations ITU-R SA.1026, ITU-R SA.1160 and ITU-R SA.1163, the interference criteria for Earth receiving sites be established as follows:

–	for frequencies between 1 and 10 GHz, the power spectral density of noise-like interference or the total power of CW-type interference in any single band or in all sets of bands shall not exceed – 154 dB(W/MHz) at the receiver input for more than 1% of the time;

–	for frequencies less than 1 GHz, the permissible interference may increase at the rate of 20 dB per decreasing frequency decade;

2	that, for Earth-to-space command and data transmission links in bands other than those treated in Recommendations ITU-R SA.1160 and ITU-R SA.1163, the interference criteria for near-Earth spacecraft receivers be established as follows:

–	for frequencies between 300 MHz and 10 GHz, the power spectral density of noise-like interference or the total power of CW-type interference in any single band or in all sets of bands 1 kHz wide shall not exceed _ 161 dB(W/kHz) at the receiver input for more than 0.1% of the time;

–	for frequencies less than 300 MHz, the permissible interference may increase at the rate of 20 dB per decreasing frequency decade.