2
Rec. ITU-R SF.356-4


Rec. ITU-R SF.356-4
1

RECOMMENDATION  ITU-R  SF.356-4*
MAXIMUM  ALLOWABLE  VALUES  OF  INTERFERENCE  FROM
LINE-OF-SIGHT  RADIO-RELAY SYSTEMS  IN  A  TELEPHONE  CHANNEL  OF  A  SYSTEM
IN  THE  FIXED-SATELLITE  SERVICE  EMPLOYING  FREQUENCY  MODULATION,
WHEN  THE  SAME  FREQUENCY  BANDS  ARE  SHARED  BY  BOTH  SYSTEMS

(1963-1966-1970-1974-1978)

Rec. 356-4

The ITU Radiocommunication Assembly,


considering

a)
that systems in the fixed-satellite service and line-of-sight radio-relay systems share frequency bands in the range above 1 GHz;

b)
that mutual interference would increase the noise in both types of system beyond that which would exist in the absence of frequency sharing;

c)
that it is desirable that the noise due to interference in the telephone channels of systems in the fixed-satellite service because of the transmitters of radio-relay systems should, during most of the time, be a small fraction of the total noise in those systems, as set out in Recommendation ITU-R S.353;

d)
that it is necessary to specify the maximum allowable interference power in a telephone channel, to determine the maximum transmitter power and equivalent isotropically radiated power of line-of-sight radio-relay stations, and to determine whether specific locations for satellite-earth stations and terrestrial radio-relay stations would be satisfactory;

e)
that a distribution of one-minute mean power, as exemplified in Fig. 1 would allot to interference an appropriate fraction of the total noise power permitted in the hypothetical reference circuit;

f)
that systems in the fixed-satellite service may receive interference both through the satellite receiver and through the earth-station receiver, but will receive the higher levels of interference associated with small percentages of time primarily through the earth-station receivers,


recommends

1.
that systems in the fixed-satellite service and radio-relay systems sharing the same frequency bands, be designed in such a manner that the interference noise power, at a point of zero relative level in any telephone channel of a hypothetical reference circuit of a system in the fixed-satellite service, caused by the aggregate of the transmitters of radio-relay stations, conforming to Recommendation ITU-R SF.406, should not exceed:

1.1
1000 pW0p psophometrically-weighted one-minute mean power for more than 20% of any month;

1.2
50 000 pW0p psophometrically-weighted one-minute mean power for more than 0.03% of any month.

2.
that the following Note should be regarded as part of the Recommendation.

Note – The way in which the above values are to be taken into account in the general noise objective for systems in the fixed-satellite service is defined in Note 6 of Recommendation ITU-R S.353.

[image: image1.wmf]D

0

1

–

 

6

0

–

 

5

5

–

 

5

0

–

 

4

5

–

 

4

0

1

2

5

1

0

2

0

3

0

4

0

5

0

6

0

7

0

8

0

9

0

9

5

9

8

9

9

1

0

5

1

0

3

1

0

4

9

9

.

5

0

.

0

1

0

.

1

0

.

5

9

9

.

9

9

9

.

9

9

P

e

r

c

e

n

t

a

g

e

 

o

f

 

a

n

y

 

m

o

n

t

h

F

I

G

U

R

E

 

1

 

–

 

E

x

a

m

p

l

e

 

o

f

 

p

o

s

s

i

b

l

e

 

i

n

t

e

r

p

o

l

a

t

i

o

n

N

o

i

s

e

 

p

o

w

e

r

 

(

d

B

m

0

p

)

p

W

0

p


*	Radiocommunication Study Groups 4 and 9 made editorial amendments to this Recommendation in 2000 in accordance with Resolution ITU-R 44.





