RECOMMENDATION  357-3

MAXIMUM  ALLOWABLE  VALUES  OF  INTERFERENCE  IN  A  TELEPHONE  CHANNEL  OF
AN  ANALOGUE  ANGLE-MODULATED RADIO-RELAY  SYSTEM  SHARING  THE  SAME
FREQUENCY  BANDS  AS  SYSTEMS  IN  THE  FIXED-SATELLITE  SERVICE

(Questions 32/4 and 17/9)

(1963-1966-1974-1978)

Rec. 357-3

The CCIR,

CONSIDERING

(a)	that systems in the fixed-satellite service and line-of-sight radio-relay systems share certain frequency bands above 1 GHz;

(b)	that mutual interference would increase the noise in both types of system beyond that which would exist in the absence of frequency sharing;

(c)	that it is desirable that the noise, due to interference in the telephone channels of existing radio-relay systems, emanating from transmitters of satellites and earth stations, should be a fraction of the total noise in those systems, such that it would not be necessary to change the design objectives for radio-relay systems, as set out in Recom-mendation 393;

(d)	that it is necessary to specify the maximum allowable interference power in a telephone channel, to determine the maximum power flux from communication satellites which can be allowed at the surface of the Earth and to determine whether specific locations for satellite-earth stations and terrestrial radio-relay stations would be satisfactory;

(e)	that a distribution of one-minute mean power, as exemplified in Fig. 1, would allot to interference a reasonable fraction of the total noise power permitted in the hypothetical reference circuit,

UNANIMOUSLY  RECOMMENDS

1.	that systems in the fixed-satellite service and line-of-sight analogue angle-modulated radio-relay systems which share the same frequency bands, should be designed in such a manner, that in any telephone channel of a 2500 km channel hypothetical reference circuit for frequency-division multiplex, analogue angle-modulated radio-relay systems, the interference noise power at a point of zero relative level, caused by the aggregate of the emission of earth stations and space stations of the systems in the fixed-satellite service, including associated telemetering, telecommand and tracking transmitters, should not exceed:

1.1	1000 pW0p psophometrically-weighted one-minute mean power for more than 20% of any month;

1.2	50 000 pW0p psophometrically-weighted one-minute mean power for more than 0.01% of any month.

2.	that the following Note should be regarded as part of the Recommendation.

Note – The way in which the above values are to be taken into account in the general noise objective for radio-relay systems is defined in Recommendation 393.

Figre 1, p.13

–––––––––––