RECOMMENDATION  ITU-R SM.1269

CLASSIFICATION  OF  DIRECTION  FINDING  BEARINGS

(Question ITU-R 28/1)

(1997)

Rec. ITU-R SM.1269

The ITU Radiocommunication Assembly,

considering

a)	that direction finding measurements have great significance in the investigation of harmful interference and efficient use of the radio spectrum;

b)	that knowledge of the accuracy of a bearing is important in determining the most likely position of an emission;

c)	that knowledge of the accuracy of a bearing will also be useful to the international monitoring system;

d)	that many modern automatic direction finders rely on statistical averaging to classify bearings for fix purposes,

recommends

1	that Tables 1 and 2 be used when deciding and classifying the accuracy that should be ascribed to the measurement of a bearing;

2	that the accuracy of the bearing is indicated by appending the appropriate letter from the tables to the numerical value of the bearing;

3	that administrations provide statistical data to support assigning numerical averaging values to the observational characteristics, e.g. standard deviation, number of samples, actual error, mean average of the sample.

TABLE  1

Classifications of bearings of frequencies below 30 MHz

|  |  | Observational characteristics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Bearing error (degrees) | Signal strength | Bearing indication | Fading | Interference | Bearing swing (degrees) | Duration of observation |
| A | ± 2 | Very good or good | Definite | Negligible | Negligible |  3 | Adequate |
| B | ± 5 | Fairly good | Bearing fluctuation | Slight | Slight |  3  5 | Short |
| C | ± 10 | Weak | Severely fluctuating bearing | Strong | Strong |  5  10 | Very short |
| D |  ± 10 | Scarcely perceptible | Ill-defined | Very strong | Very strong |  10 | Inadequate |
| Class A: probability of less than 5% that error exceeds 2° Class B: probability of less than 5% that error exceeds 5° Class C: probability of less than 5% that error exceeds 10° Class D: bearing with an error greater than those in Class C. NOTE 1 – For more explanation of the Classification of Bearing see Chapter 3.6 of the Spectrum Monitoring Handbook (1995). |  |  |  |  |  |  |  |

TABLE  2

Classifications of bearings of frequencies above 30 MHz

|  |  | Observational characteristics |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Bearing error (degrees) | Signal strength | Bearing indication | Interference | Bearing swing (degrees) | Duration of observation |
| A | ± 1 | Very good or good | Definite | Negligible |  1 | Adequate |
| B | ± 2 | Fairly good | Bearing fluctuation | Slight |  1  3 | Short |
| C | ± 5 | Weak | Severely fluctuating bearing | Strong |  3  5 | Very short |
| D |   5 | Scarcely perceptible | Ill-defined | Very strong |  5 | Inadequate |
| Class A: probability of less than 5% that error exceeds 1. Class B: probability of less than 5% that error exceeds 2. Class C: probability of less than 5% that error exceeds 5. Class D: bearing with an error greater than those in Class C. NOTE 1 – For more explanation of the Classification of Bearing see Chapter 3.6 of the Spectrum Monitoring Handbook (1995). |  |  |  |  |  |  |