Recommendation ITU-R SM.2110-2

(09/2025)

SM Series: Spectrum management

Guidance on frequency ranges for operation of non-beam wireless power transmission for electric vehicles

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from https://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at https://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2025

ITU 2025

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  SM.2110-2

Guidance on frequency ranges for operation of non-beam 
wireless power transmission for electric vehicles

(Question ITU-R 210-4/1)

(2017-2019-2025)

Scope

This Recommendation provides guidance on frequency ranges for the operation of non-beam wireless power transmission (non-beam WPT) for electric vehicles (EV) and the protection of radiocommunication services.

Keywords

Wireless power transmission, short-range devices, ISM, non-beam WPT

Abbreviations/Glossary

CISPR	In French “Comité International Spécial des Perturbations Radioélectriques”, International Special Committee on Radio Interference

ICES	International Committee on Electromagnetic Safety

ICNIRP	International Commission on Non-ionizing Radiation Protection

IEC	International Electrotechnical Commission

IEEE	Institute of Electrical and Electronics Engineers

ISO	International Organization for Standardization

ISM	Industrial Scientific Medical

RR	Radio Regulations

SAE	Society of Automotive Engineers

SFTS	Standard Frequency and Time Signal service (RR No. 1.53)

WHO	World Health Organization

WPT	Wireless power transmission

WPT-EV	Wireless power transmission for electric vehicles

Related ITU Recommendations, Reports

Recommendation ITU-R SM.1056 – Limitation of radiation from industrial, scientific and medical (ISM) equipment

Recommendation ITU-R SM.1896 – Frequency ranges for global or regional harmonization of short-range devices

Recommendation ITU-R SM.2129 – Guidance on frequency ranges for the operation of non-beam wireless power transmission for mobile and portable devices

Report ITU-R SM.2153 – Technical and operating parameters and spectrum use for short-range radiocommunication devices

Report ITU-R SM.2303 – Wireless power transmission using technologies other than radio frequency beam

Report ITU-R SM.2451 – Assessment of impact on radiocommunication services from wireless power transmission for electric vehicle operating below 30 MHz

Report ITU-R TF.2487 – Protection criteria for systems in the standard frequency and time signal services

The ITU Radiocommunication Assembly,

considering

a)	that wireless power transmission (WPT) is defined as the transmission of power from a power source to an electrical load wirelessly using the electromagnetic field;

b)	that WPT technologies utilize various mechanisms, such as transmission via radio frequency radiated transmissions in the far-field (WPT beam) and near-field inductive, resonant and capacitive coupling (non-beam WPT);

c)	that WPT technologies are being considered for applications such as charging of electric vehicles;

d)	that WPT regulations and standards are currently being developed at national, regional, and international levels;

e)	that industrial alliances, consortia, and academia have investigated several fundamental frequency bands for WPT technologies, including; 19-21 kHz and 79-90 kHz for electric vehicles;

f)	that studies have been conducted on the impact of non-beam WPT to radiocommunication services and are contained in Report ITU-R SM.2451;

g)	that for the purpose of WPT studies the radio astronomy service is to be treated as a radiocommunication service;

h)	that as more WPT devices proliferate globally, the use of WPT technologies may have an impact on radiocommunication services including the standard frequency and time signal service and the radio astronomy service, WPT must not cause harmful interference to radiocommunication services;

i)	that to mitigate the impact of WPT devices on the operation of radiocommunication services some solutions utilize frequency bands already designated for industrial, scientific and medical (ISM) applications,

recognizing

a)	that WPT is not a radiocommunication service and has no status in the Radio Regulations (RR), but may be regarded as subject to RR Nos. 15.12 or 15.13;

b)	that the criteria to protect various radiocommunication services from harmful interference are specified in existing ITU-R Recommendations;

c)	that both consumers and manufacturers may benefit from harmonized frequency ranges and technical conditions for WPT technologies;

d)	that some Administrations classify the non-beam WPT energy transfer as an ISM application, even for operation outside bands designated for ISM use;

e)	that some Administrations classify non-beam WPT systems as radio application such as a short-range radio device;

f)	that some non-ISM bands are taken into consideration for the global or regional harmonized use of specific WPT applications;

g)	that the energy transfer of non-beam WPT systems can be treated separately from data communications, especially when the receiving device receives data communications at a frequency different from that for the energy transfer;

h)	that in the absence of a load, the WPT-EV shuts off and does not radiate;

i)	that for non-beam WPT-EV, the radiated RF power external to the WPT-EV system is much lower than RF power transferred to the vehicle. Most power is transferred to the receiver through mechanisms such as capacitive, resonant and inductive coupling;

j)	that Recommendation ITU-R SM.1056 on the limitation of radiation from ISM equipment recommends that administrations consider the use of the latest edition of CISPR 11. These limits do not necessarily protect radiocommunication services,

noting

a)	that the IEC TC 69 published IEC International Standard (IS) 61980-1 on “general requirement of electric vehicle wireless power transfer systems”, IEC IS 61980-2 on “specific requirements for communication between electric road vehicle (EV) and infrastructure”, and IEC IS 61980-3 on “specific requirements for the magnetic field power transfer systems of WPT-EV”;

b)	that the International Organization for Standardization (ISO/TC22/SC37) published ISO International Standard (IS) 5474-4 on “electrically propelled road vehicles – functional and safety requirements for power transfer between vehicle and external electric circuit part 4: magnetic field wireless power transfer”;

c)	that SAE International published the Standard J2954 on “wireless power transfer for light-duty plug-in/electric vehicles and alignment methodology”;

d)	that issues of non-ionizing radiation exposure are dealt with by international organizations such as the WHO, ICNIRP and IEEE;

e)	and that “ICNIRP Guidelines” (2010) addresses limiting exposure to time-varying electric and magnetic fields from 1 Hz to 100 kHz and IEEE C95.1 provides a standard for limiting exposure in the frequency range 0 Hz to 300 GHz,

recommends

that the frequency ranges (or portions thereof) listed in Table 1 below, as well as Note of Table 1 should be considered as guidance for the operation of non-beam WPT-EV systems and the protection of radiocommunication services taking into account considering f) and h).

TABLE 1

Frequency ranges for operation of non-beam WPT systems for electric vehicles

| Frequency range(1) | Suitable non-beam WPT-EV technologies |
| --- | --- |
| 19-21 kHz | Magnetic induction or Magnetic resonance |
| 79-90 kHz | Magnetic resonance |
| (1) For protection of SFTS operation at 60 kHz all forms of energy emissions from WPT-EV in the 60 kHz ± 4 kHz range (56-64 kHz) should be avoided. In cases where any form of WPT-EV energy emission occurs in the proximity of 60 kHz SFTS receivers, and assuming a minimum separation distance of 50 m, any form of WPT-EV energy emission falling within the 64-65 kHz and 55-56 kHz frequency ranges should not exceed a total of 35 dBµA/m at 10 m. Where the separation distance is guaranteed to be greater than 100 m between WPT-EV and SFTS receivers, any form of WPT-EV energy emission falling within the 63-65 kHz and 55-57 kHz ranges, should not exceed a total of 44 dBµA/m at 10 m. |  |