RECOMMENDATION  329-6

SPURIOUS  EMISSIONS*

(Question 55/1)

(1951-1953-1956-1959-1963-1966-1970-1978-1982-1986-1990)

Rec. 329-6

The CCIR,

CONSIDERING

(a)	that Recommendation 328 distinguishes between out-of-band emissions and spurious emissions, and specifies limits for out-of-band emissions;

(b)	that Appendix 8 to the Radio Regulations, specifies the maximum permitted level of spurious emissions in terms of the mean power supplied by a transmitter to the antenna transmission line at the frequency, or frequencies, of each spurious emission;

(c)	that Article 5, (Nos. 304 to 306), of the Radio Regulations stipulates that stations must conform to the tolerances specified in Appendix 8 for spurious emissions; that, moreover, every effort should be made to keep spurious emissions and out-of-band emissions at the lowest values which the state of the technique and the nature of the service permit;

(d)	that measurements of power, at frequencies other than the fundamental frequencies supplied to a transmitting antenna or to a test load, are useful in the analysis of transmitter performance as regards purity of emissions under specific conditions, and that such measurements will encourage the use of certain means of reducing spurious emissions;

(e)	that the relation between the power of the spurious emission supplied to a transmitting antenna and the field strength of the corresponding signals, at locations away from the transmitter, may differ greatly, due to such factors as the horizontal and vertical antenna directivity at the frequencies of the spurious emissions, propagation over various paths and radiation from parts of the transmitting apparatus other than the antenna itself;

(f)	that field-strength measurements of spurious emissions, at locations distant from the transmitter, are recognized as the direct means of expressing the intensities of interfering signals due to such emissions;

(g)	that, in dealing with emissions on the fundamental frequencies, administrations customarily establish the power supplied to the antenna transmission line, and measure the field strength at a distance, to aid in determining when an emission is causing interference with another authorized emission; that a similar procedure would be helpful in dealing with spurious emissions (see Article 18, No. 1813, of the Radio Regulations);

(h)	that for the most economical use of the frequency spectrum, it is necessary to lay down general maximum limits of spurious emissions, while recognizing that specific services may need lower limits for technical and operational reasons,

UNANIMOUSLY  RECOMMENDS

1.	Terminology and definitions

that the following terms and definitions should be used to designate emissions that are regarded as spurious:

1.1	Spurious emission (Article 1, No. 139 of the Radio Regulations)

emission on a frequency, or frequencies, which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information. Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products but exclude out-of-band emissions;

1.2	Harmonic emissions

spurious emissions at frequencies which are whole multiples of those contained in the frequency band occupied by an emission;

1.3	Spurious intermodulation products

spurious intermodulation products are intermodulation products at frequencies resulting from:

1.3.1	intermodulation between:

–	the oscillations at the carrier, characteristic, or harmonic frequencies of an emission, or the oscillations resulting from the generation of the carrier or characteristic frequency, and

–	oscillations of the same nature, of one or several other emissions, originating from the same transmitting system or from other transmitters or transmitting systems, and

1.3.2	intermodulation between any oscillation generated to produce the carrier or characteristic frequency of an emission;

1.4	Spurious frequency conversion products

spurious emissions, not including harmonic emissions, at the frequencies, or whole multiples thereof, of any oscillations generated to produce the carrier or characteristic frequency of an emission;

1.5	Parasitic emissions

spurious emissions, accidentally generated at frequencies which are independent both of the carrier or characteristic frequency of an emission and of frequencies of oscillations resulting from the generation of the carrier or characteristic frequency.

Note – Non-linearity in amplitude modulated transmitters (including single sideband transmitters) may result in out-of-band emissions which are immediately adjacent to the necessary bandwidth, due to odd order intermodulation products.

Recommendation 328 distinguishes between out-of-band emissions and spurious emissions. Whereas the limits for spurious emissions are specified below, the limits applicable to out-of-band emissions due to intermodulation are specified in Recommendation 326.

2.	Application of limits

2.1	that, as the limits for out-of-band emissions are covered by Recommendation 328, the limits given for spurious emissions given below should apply only for spurious emissions in accordance with the definition;

2.2	that for the time being, the maximum permitted levels of spurious emissions continue to be expressed in terms of the mean power supplied by the transmitter to the antenna feeder at the frequencies of the spurious emission concerned;

2.3	that spurious emission from any part of the installation, other than the antenna system, i.e. the antenna and its feeder, shall not have an effect greater than would occur if this antenna system were supplied with the maximum permissible power at that spurious emission frequency;

2.4	that, in the event that the standards of performance in § 3 below are adopted by an Administrative Radio Conference as revised limits for Appendix 8 to the Radio Regulations, a period of at least 3 years from the coming into force of the revised Regulations might be necessary, to enable all administrations to attain these limits for new transmitters;

2.5	that, where a transmitting system comprises more than one transmitter, the application of the limits specified in § 3 should apply with all transmitters operating normally in accordance with Appendix 8 to the Radio Regulations.

3.	Limits for the power of spurious emissions (see Notes 1 to 15)

3.1	that the following limits are applicable to transmitters with fundamental frequencies between 9 kHz and 30 000 kHz (from Radio Regulations Appendix 8, Table, Columns A and/or B):

For any spurious emission, the mean power supplied to the antenna transmission line should be at least 40 dB below the power of the fundamental emission, without exceeding the value of 50 mW (for exceptions see Notes 2, 3, 4, 7 and 8);

3.2	that the following limits are applicable to transmitters having fundamental frequencies between 30 MHz and 235 MHz (see Radio Regulations, Appendix 8, Table, Columns A and/or B):

3.2.1	Transmitters with output power greater than 25 W at the fundamental frequencies

For any spurious emission, the mean power supplied to the antenna transmission line should be at least 60 dB below the power of the fundamental emission, without exceeding the value of 1 mW (for exceptions, see Notes 5 and 9);

3.2.2	Transmitters with output power 25 W or less at the fundamental frequencies

For any spurious emission, the mean power supplied to the antenna transmission line should be at least 40 dB below the power of the fundamental emission, without exceeding the value of 25 mW (for exceptions, see Notes 5 and 6);

3.3	that the following limits are realizable for new transmitters with fundamental frequencies between 235 MHz and 960 MHz (see Radio Regulations, Appendix 8, Table, Column B):

3.3.1	Transmitters with output power greater than 25 W at the fundamental frequencies

For any spurious emission, the mean power supplied to the antenna transmission line should be at least 60 dB below the power of the fundamental emission, without exceeding the value of 20 mW (for exceptions, see Notes 10 and 11);

3.3.2	Transmitters with output power 25 W or less at the fundamental frequencies

For any spurious emission the mean power supplied to the antenna transmission line should be at least 40 dB below the power of the fundamental emission, without exceeding the value of 25 mW (for exceptions, see Notes 10 and 11);

3.4	that the following limits are realizable for new transmitters with fundamental frequencies between 960 MHz and 17.3 GHz:

3.4.1	Transmitters with output power greater than 10 W at the fundamental frequencies

For any spurious emission, the mean power supplied to the antenna transmission line should be at least 50 dB below the power of the fundamental emission, without exceeding the value of 100 mW (for exceptions, see Notes 10, 11, 12 and 13);

3.4.2	Transmitters with output power 10 W or less at the fundamental frequencies

For any spurious emission, the mean power supplied to the antenna transmission line should not exceed the value of 100 mW (for exceptions, see Notes 10, 11, 12 and 13).

3.5	that the limits adopted by the World Administrative Radio Conference, Geneva, 1979, should also be shown in the Radio Regulations, in the form of a graph as indicated in Fig. 1.

Figure 1, p.

Note 1 – When checking compliance with the provisions of the Table, it shall be verified that the bandwidth of the measuring equipment is sufficiently wide to accept all significant components of the spurious emission concerned.

Note 2 – For transmitters of mean power exceeding 50 kW and which operate below 30 MHz over a frequency range approaching an octave or more, a reduction below 50 mW is not mandatory, but a minimum attenuation of 60 dB shall be provided and every effort should be made to comply with the level of 50 mW.

Note 3 – For hand-portable equipment of mean power less than 5 W which operates below 30 MHz, the attenuation shall be at least 30 dB but every effort should be made to attain 40 dB attenuation.

Note 4 – For mobile transmitters which operate below 30 MHz any spurious component shall have an attenuation of at least 40 dB without exceeding the value of 200 mW but every effort should be made to comply with the level of 50 mW wherever practicable.

Note 5.1 – For frequency modulated maritime mobile radiotelephone equipment which operates above 30 MHz, the mean power of any spurious emission falling in any other international maritime mobile channel, due to products of modulation, shall not exceed a level of 10 mW. In addition, where, exceptionally, transmitters of mean power above 20 W are employed, these levels may be increased in proportion to the mean power of the transmitter.

Note 5.2 – The mean power of any other spurious emission on any discrete frequency within the international maritime mobile band shall not exceed a level of 2.5 mW. Where, exceptionally, transmitters of mean power above 20 W are employed, these levels may be increased in proportion to the mean power of the transmitter.

Note 6 – For transmitters having a mean power of less than 100 mW it is not mandatory to comply with an attenuation of 40 dB provided that the mean power level does not exceed 10 mW.

Note 7 – For transmitters of a mean power exceeding 50 kW which can operate on two or more frequencies covering a frequency range approaching an octave or more, whilst a reduction below 50 mW is not mandatory, a minimum attenuation of 60 dB shall be provided.

Note 8 – For hand-portable equipment of mean power less than 5 W the attenuation shall be 30 dB but every practicable effort should be made to attain 40 dB attenuation.

Note 9 – Administrations may adopt a level of 10 mW provided that harmful interference is not caused.

Note 10 – Where several transmitters feed a common antenna or closely spaced antennas on neighbouring frequencies, every practicable effort should be made to comply with the levels specified.

Note 11 – Since these levels may not provide adequate protection for receiving stations in the radioastronomy and space services, more stringent levels might be considered in each individual case in the light of the geographical position of the stations concerned.

Note 12 – These levels are not applicable to systems using digital modulation techniques, but may be used as a guide. Values for these systems may be provided by the relevant CCIR Recommendations, when available (see Recommendation No. 66 of the WARC-79 and Appendix 8 to the Radio Regulations).

Note 13 – Intermodulation emissions and thermal noise from earth and space stations in the space services occurring outside the frequency band allocated by the Radio Regulations and referenced to a 4 kHz bandwidth or a 1 MHz bandwidth, as appropriate and consistent, shall be at least 30 dB below the saturated transmitter output power (or, in the case of a solid-state amplifier, the rated output power) as produced by a single unmodulated carrier. All other spurious emissions, excluding thermal noise, from stations in the space services outside the assigned frequency band and referenced to a 4 kHz bandwidth or a 1 MHz bandwidth, as appropriate and consistent, shall be at least 50 dB below the power of the fundamental. The 100 mW and 100 mW limits in § 3.4 shall not apply, as these require further study by the CCIR.

Note 14 – It is recognized that specific services may need lower limits for technical and operational reasons.

Note 15 – For radiodetermination stations, until acceptable methods of measurement exist, the lowest practicable power of spurious emission should be achieved. Single-sideband maritime mobile limits are listed in Appendix 17, and pertinent aeronautical mobile service information is found in Appendix 27 and Appendix 27 Aer2-1 to the Radio Regulations.

4.	Methods of measurement of spurious emission by measurement of power supplied to the antenna*

that, together with other known methods of measuring the power of spurious emission, either the substitution method or a direct power measuring method should be used, when the transmitter is operated under normal conditions and when connected to its normal antenna or to a test load. When the measurements are performed with the transmitter connected to a test load, the power of the spurious oscillations supplied to the test load might differ considerably from the spurious emission supplied to the antenna used for actual transmission.

4.1	Substitution method

In the substitution method, an auxiliary generator, the output power of which can be varied, is employed and its frequency is adjusted to be equal to the mean frequency of the spurious emission in question. This auxiliary generator is used as follows:

The generator is substituted for the radio transmitter and is adjusted until it produces the same field at the mean frequency of the spurious emission as was produced by the radio transmitter (in intensity and polarization). This field is measured by means of a radio receiver tuned to the spurious emission and located at a distance of several wavelengths from the transmitting antenna. The power supplied by the generator is then equal to the power originally supplied by the transmitter itself, on condition that non-linearity of the radiating system does not introduce harmonic emission. To obtain the same conditions with the generator, account must be taken of any stray coupling from the original transmitter to the radiating system and of any direct radiation from the transmitter and from feeder lines or other apparatus that may become excited by direct coupling. It is also necessary to take into account the possibility of the power of a spurious emission being supplied in a push-pull or push-push mode or combination of both. More than one generator may be necessary when the method of excitation is complex. It is also necessary to determine the impedance of the feeder input circuit at the spurious frequencies, in order that the power supplied to the antenna may be measured accurately. It is necessary that several sets of measurements be made using different receiver locations.

When a test load is used, an indicator coupled to the load is required.

4.2	Direct methods

That the following three direct methods of measurement can be used:

4.2.1	First method. (See [CCIR, 1953e].) The voltage, current and power factor are determined at one point on the feeder using a selective radio receiver tuned to the mean frequency of the spurious emission concerned, and coupled to the desired point of the feeder.

4.2.2	Second method. (See [CCIR, 1962a].) The forward and reflected powers are determined by using a pair of inverse directional couplers, inserted directly in the feeder line or the test load; a selective power-measuring device is switched alternatively to the couplers and tuned to the mean frequency of the spurious emission concerned. The difference between these two measured powers gives the power supplied to the antenna at the frequencies of the spurious emission.

The voltages or power from the directional couplers can be measured using a comparison-method with the aid of a receiver and a signal generator. The principle of this method and a formula for determining the possible error of the measuring result are contained in [CCIR, 1966-69].

For coaxial lines a directional coupler may consist of a conductor (linear antenna), arranged within the feeder, parallel to its axis, and provided at one of its ends with a reflectionless termination relative to the external conductor. A voltage appears at the open end that is due entirely to the voltage wave in the feeder, which extends from the open end of the linear antenna to the closed end. The dimensions and spacing between the conductors of the coupler and the external wall depend on the maximum permissible input level and on the input impedance of the measuring set to be connected.

The method enables the power of spurious emission transferred from a transmitter to the antenna to be measured, regardless of whether it is generated directly in the transmitter concerned or in a secondary manner, e.g. by interaction with other transmitters.

For balanced feeder lines (see [CCIR, 1963-66b]), a directional coupler may consist of a pair of parallel conductors (symmetrical linear antenna) arranged symmetrically near the feeder in a plane parallel to the plane containing the feeder. The directional coupler is provided with a reflectionless termination at one end.

A voltage, balanced relative to ground, appears at the open end due entirely to the push-pull mode of the wave on the feeder. For selective measurement of the voltage it is preferable to transform it from balance to unbalance by a transformer.

If the couplers are arranged as mentioned above, the push-push mode of the wave on the feeder has a negligible influence on the measurement. The extent of this influence depends on the balance of the transformer used.

The spacing between the coupler and the feeder and the distance between the two conductors of the coupler depend on the maximum permissible input level of the selective level meter to be connected and the transformation ratio of the transformer used.

When power components of substantial magnitude transferred in a push-push mode can be expected, these components should be measured separately by using another appropriate method.

Another measuring equipment suitable for balanced feeder lines (see [CCIR, 1963-66c]) uses two coaxial feeder sections, each of which is fitted with two directional couplers. Thus the forward and reflected powers can be determined separately for each of the two conductors.

The sum of the forward powers is equal to the total power transferred to the transmission line. The method does not, however, distinguish between the power in the push-pull and push-push modes.

A special type of directional coupler can be used for measuring the power of spurious emissions over a wide range of frequencies.

4.2.3	Third method. (See [CCIR, 1962c].) Measurement is made of electromotive force values at the points of a node and anti-node on a symmetrical open-wire feeder and these values are converted into power values of the spurious emission at the frequency to be measured. The electromotive force values are measured by means of a coupling element and a selective radio receiver tuned to the mean frequency of the spurious emission concerned. The coupling element is a screened loop placed symmetrically between the feeder conductors and moved at will along the feeder to locate the nodes and anti-nodes. By changing the position of the plane of the loop in relation to the plane of the feeder conductors, it is possible to measure the power of the push-pull and push-push modes of the spurious emission.

The coefficient used for conversion of the measured values into power values is found from a graph plotted previously, at the time of calibration of the device.

4.3	Measurements of spurious emission at frequencies close to the fundamental frequencies [CCIR, 1962b]

4.3.1	In view of the difficulty of measuring spurious emissions which are relatively close to the necessary band, it may not be possible to ensure that the limitations set forth in § 3 are met in such cases (see Question 55/1).

4.3.2	In many cases, adequate suppression of oscillations which unduly disturb the measurement of spurious emissions on nearby frequencies can be effected by the insertion of suitable band-pass filters. Additional selective suppression of the carrier oscillation, for instance when measuring spurious emission on nearby frequencies, can be realized by cancelling the carrier in the monitoring receiver with an unmodulated carrier in antiphase, obtained from a low power stage (see [CCIR, 1963-66d]).

4.3.3	When several transmitters work on nearby frequencies in the same station, and may even be feeding the same antenna, as may happen, for instance, in sound broadcasting stations using frequency modulation at the frequencies in band 8, intermodulation products may be found with a frequency separation less than 1 MHz from the carrier frequencies in use.

4.3.3.1	In such cases measurements can be made using a directional coupler and a tuned filter. The principle of the measuring equipment is shown in [CCIR, 1966-69]. With this method a filter is used which is tuned to attenuate the more distant of the fundamental signals contributing to the generation of the spurious component to be measured. This prevents the generation of additional spurious components in the receiver.

The errors from overloading the receiver by the remaining fundamental signals can be overcome by measuring the level of the spurious component in the presence of the fundamental signals using the comparison method. The signal generator will be adjusted to increase the power output of the receiver by a factor of two. In this case the power of the spurious component and of the comparison signal are equal.

4.3.3.2	In some cases, the methods of measurement described above may be somewhat difficult to apply. It may then be preferable to measure the field strength on the spurious frequency and on a nearby carrier frequency at a convenient distance (a few, or a few tens of kilometres), with a sufficiently selective measuring instrument. If all the signals are radiated by the same antenna and share similar radiation patterns then the power of the spurious components can be determined by measurement of the field strengths. If the powers of the fundamental signals cannot be measured separately the sum of these powers can be used to calculate the power of the unknown spurious component by using the ratio of the squared value of field strength of the spurious component to the sum of the squared values of field strength for the fundamental signals. This ratio will be equal to the ratio of the power of the spurious component to the sum of the powers of the fundamental signals.

5.	Further improvements

that administrations and private operating agencies should continue to improve the degree of suppression of spurious emission, where this is economically possible, to reduce interference to other services to a greater extent than that given in § 3. Guidance on the means of reducing the level of spurious emission from transmitters is given in Report 838.

6.	Radioastronomy

that radioastronomy, because of its unique, passive, and sensitive nature, needs special consideration in so far as spurious emissions are concerned; radioastronomers frequently encounter signal-to-noise ratios of –30 dB and in extreme cases as low as –60 dB using long integration intervals. Administrations are urged, as far as practicable, to take into consideration the need to avoid spurious emissions which could cause harmful interference to radioastronomy operating in accordance with Article 36 of the Radio Regulations. For the purpose of resolving interference from spurious emissions, radioastronomy should be treated as a radiocommunication service and given protection to the extent radiocommunications services afford each other protection.

7.	Space service

that information regarding spurious emissions from space services transmitters, located both at earth and space stations, should contain intermodulation and general spurious emission limits. These spurious emission limits should apply up to a frequency of 17.3 GHz, excluding the 100 mW and 100 mW limits specified in § 3.4. Administrations should also endeavour to reduce spurious emission levels from such transmitters above 17.3 GHz to the lowest possible values, compatible with technical and economic constraints of the space transmitters used.

8.	Amplitude modulated sound broadcast transmitters which employ pulse width modulation techniques

that particular attention should be paid to spurious emissions which may be produced at multiples of the switching frequency on either side of the carrier as the result of the pulse width modulation technique;

that the switching frequency should be a multiple of the channel spacing in order to reduce interference within the broadcasting bands.

REFERENCES

CCIR Documents

[1953]: London, a. 65; b. 80; c. 101; d. 124; e. 130; f. 340.

[1956]: Warsaw, 313.

[1958]: Geneva, a. I/22; b. I/28; c. I/34.

[1962]: Geneva, a. I/1; b. I/17, c. I/23.

[1963-66]: a. I/54 (OIRT); b. I/1 (Federal Republic of Germany); c. I/40 (USSR); d. I/55 (OIRT).

[1966-69]: I/67 (USSR).

_________________

*	Note by the Editorial Committee. – The terminology used in Recommendation 329 is in conformity, in the three working languages, with that of Article 1 of the Radio Regulations (No. 139), namely:

– French: Rayonnement non essentiel
	– English: Spurious emission
	– Spanish: Emisión no esencial.

*	Relevant documents are: [CCIR, 1953a, b, c, d, e and f; 1956; 1958a, b and c; 1962a, b and c; 1963-66a].