RECOMMENDATION  ITU-R  TF.1153

THE OPERATIONAL USE OF TWO-WAY SATELLITE TIME AND
FREQUENCY TRANSFER EMPLOYING PN CODES

(Question ITU-R 201/7)

(1995)

Rec. ITU-R TF.1153

The ITU Radiocommunication Assembly,

considering

a)	the potential for high-accuracy time and frequency transfer of the two-way satellite method (TWSTFT) as expressed in Question ITU-R 201/7;

b)	the demonstrated high performance of TWSTFT systems using telecommunication satellites in the 14/11 and 14/12 GHz bands;

c)	that other frequency bands are becoming important;

d)	the demonstrated time calibration results of TWSTFT systems;

e)	that theoretical background is available to calculate the corrections for the effect of the propagation delay through the troposphere and the ionosphere, the correction for the Sagnac-effect, and other reciprocity factors;

f)	that the number of participants is increasing;

g)	the need for standardizing:

–	measuring procedures;

–	data processing;

–	format for the exchange of data and relevant information between pairs of participants,

recommends

1	that the measuring and data processing procedures for accurate time and frequency transfer be followed as outlined in Annex 1;

2	that the data format for the exchange of the relevant data between pairs of participants should include the information as outlined in Annex 2.

ANNEX  1

Procedures for TWSTFT

# 1	Introduction

Radio links can be used to transfer time from one clock to another. In radio links however, the signal delays are changing with distance, ionosphere, troposphere, temperature, earth conductivity and so on. To cancel these influences to first order the two-way scheme has been introduced: at both clock sites the time signals are transmitted at the same instant and on both sides the signal from the other clock is received and measured. After the exchange of the measured data, the difference of the two clocks is calculated. The delays cancel due to the complete reciprocity, to first order, of the signal paths. The accuracy of the result then depends on the residual effects due to the incomplete reciprocity. Some of these effects are known and others are still unknown. In some cases corrections for these effects can be used to improve accuracy.

FIGURE 1...[D01] = 3 CM

From Fig. 1, it can be seen how the difference of the clocks at stations 1 and 2 can be determined.

TA(k):	time-scale of the 1PPSTX reference point at station k, k being 1 for station 1 and 2 for station 2

TI(k):	time interval reading

TT(k):	transmitter delay, including the modem delay

TR(k):	receiver delay, including the modem delay

TU(k):	uplink propagation delay

TD(k):	downlink propagation delay

TS(k):	satellite delay

TCU(k):	Sagnac correction in the uplink

TCD(k):	Sagnac correction in the downlink.

The difference of the time-scale (1PPSTX) at station 2 from the time-scale (1PPSTX) at station 1 expressed by TA(1) – TA(2) is determined as follows.

The time interval counter (TIC) reading at station 1 is:

TI(1) = TA(1) – TA(2) + TT(2) + TU(2) + TCU(2) + TS(2) + TD(1) + TCD(1) + TR(1)

The TIC reading at station 2 is:

TI(2) = TA(2) – TA(1) + TT(1) + TU(1) + TCU(1) + TS(1) + TD(2) + TCD(2) + TR(2)

Subtracting the expression of station 2 from that of station 1, gives:

TI(1) – TI(2) = 2×TA(1) – 2×TA(2) + TT(2) – TT(1) + TU(2) – TU(1) + TS(2) – TS(1) 
+ TD(1) – TD(2) + TR(1) – TR(2) + TCD(1) – TCU(1) – TCD(2) + TCU(2)

The time-scale difference is given by:

TA(1) – TA(2) =	1/2[TI(1)]	(=	TIC reading at station 1)
-1/2[TI(2)]	(=	TIC reading at station 2)
+1/2[TS(1) – TS(2)]	(=	Satellite delay difference)
+1/2[TU(1) - TD(1)]	(=	Up/down difference at station 1)
–1/2[TU(2) - TD(2)]	(=	Up/down difference at station 2)
+1/2[TT(1) - TR(1)]	(=	Transmit/receive difference at station 1)
–1/2[TT(2) - TR(2)]	(=	Transmit/receive difference at station 2)
-1/2[TCD(1) - TCU(1)]	(=	Sagnac correction for station 1 including the correction 
		for satellite movement)
+1/2[TCD(2) - TCU(2)]	(=	Sagnac correction for station 2 including the correction 
		for satellite movement)

The last seven terms are the corrections for non-reciprocity. The corrections can be determined and grouped in corrections per station. Each station exchanges the TI(k) data together with the assessment of its own corrections to the other station.

The non-reciprocity factors are further addressed in the following sections.

# 2	Non-reciprocity due to satellite equipment delays

When the satellite receive antenna, transponder channel and transmit antenna are common to both signal paths, the satellite signal delays are equal, i.e. TS(1) = TS(2).

This is not the case when different frequencies, transponders or different spot beams are used for the reception and/or transmissions from each station, e.g. the transatlantic Intelsat satellites. In this case TS(1) and TS(2) or at least the difference TS(1) - TS(2) should be measured before the launch of the satellite or using another accurate time transfer method.

# 3	Sagnac-effect correction

Due to the movement around the rotation axis of the Earth, both of the earth stations and of the satellite during the propagation of a time signal to and from the satellite, a correction has to be applied to the propagation time of the signal. The Sagnac correction for the one-way path from satellite s to ground k is given by:

TCD(k)  =  (W / c2)  ´  [Y(k)  ´  X(s)  –  X(k)  ´  Y(s)]

where:

W :	Earth rotation rate = 7.2921 ´ 10-5 rad/s

c :	speed of light = 299 792 458 m/s

X(k) :	geocentric x-coordinate of station (m)

=	r cos[LA(k)] ´ cos[LO(k)]

X(s) :	geocentric x-coordinate of satellite (m)

=	R cos[LA(s)] ´ cos[LO(s)]

Y(k) :	geocentric y-coordinate of station (m)

=	r cos[LA(k)] ´ sin[LO(k)]

Y(s) :	geocentric y-coordinate of satellite (m)

=	R cos[LA(s)] ´ sin[LO(s)]

r :	Earth radius = 6 378 140 m

R :	satellite orbit radius = 42 164 000 m

LA :	latitude

LO :	longitude.

For geostationary satellites LA(s) = 0, so TCD(k) = (W/c2) ´ R ´ r ´ cos[LA(k)] ´ sin[LO(k)] – LO(s)).

Total Sagnac correction TC(12) for clock at station 1 as reference to measure clock at station 2 is:

TC(12) = 1/2(TCU(1) + TCD(2) – [TCU(2) + TCD(1)])

Further, the sign of the Sagnac correction for the downlink is opposite to the sign of the Sagnac correction for the uplink due to the opposite directions of the signals: TCU(k) = –TCD(k), so TC(12) = –TCD(1) + TCD(2).

Example for a satellite at 307°E:

LA(VSL) = 52 °N, LO(VSL) = 4 °E, LO(sat) = 307 °E, difference in LO = 57 °, TCD(VSL) = +112.42 ns

LA(USNO) = 39 °N, LO(USNO) = 283 °E, LO(sat) = 307 °E, difference in LO = –24°, TCD(USNO) = – 68.83 ns

TC(VSL®USNO): –TCD(VSL) + TCD(USNO) = –181.25 ns

TC(USNO®VSL): –TCD(USNO) + TCD(VSL) = +181.25 ns

VSL:	NMi Van Swinden Laboratory, Delft, the Netherlands

USNO:	US Naval Observatory, Washington DC, United States of America.

# 4	Non-reciprocity effects due to satellite movement in an earth fixed frame

Two-way paths between earth stations via the satellite are not reciprocal if the satellite is in motion relative to the Earth's surface and if the two arriving signals do not pass through the satellite at the same instant. This effect may be compensated for by a slight adjustment (<17 ms) of emission times. If synchronization errors are not compensated for in this way, the error itself, which may be of the order of 1 ns, requires correction.

# 5	Ionospheric correction

The up and downlink signals at each station differ in carrier frequency and they experience a different ionospheric delay equal to:

where:

TEC :	Total Electron Content along the signal path

c :	speed of light

fd and fu :	down and uplink frequencies.

Example: For a high TEC of 1 ´ 1018 electrons/m2 and for fu = 14.5 and fd = 12.5 GHz this ionospheric delay = 0.859 ns – 0.639 ns = 0.220 ns. So the correction for ½[TU(k) – TD(k)] is typically smaller than –0.11 ns.

# 6	Tropospheric correction

The troposphere gives a delay depending on the elevation angle, the water content of the air, air density and temperature, but up to 20 GHz this delay is not frequency dependent and so its influence on the up and down propagation delays is equal and no correction is needed.

# 7	Earth station delay measurement

The difference of the transmit and receive section [TT(k) – TR(k)] including the up and down converters, modulator and demodulator (modem), feeds, wiring, etc., has to be determined at each station. Methods to obtain this are:

–	co-location of both stations; or

–	subsequent co-location of a third (transportable) earth station at both stations;

–	the use of a satellite-simulator and calibrated cable.

The last method is the least expensive and can be used frequently. This method consists of the calibration of an auxiliary cable, measurement of the sum of the transmit and receive delay, measurement of the sum of the auxiliary cable delay and the receive delay and calculation of the receive and transmit delay from the measurements.

The internal transmit and receive delay difference of the modem have to be determined as well. This can be done by:

–	co-locating the modems and measuring the sum of the transmit delay of one modem and the receive delay of the other;

–	by measuring the sum of transmit and receive delay by connecting the IF output signal to the IF input of each of the modems. The transmit delay between the 1 pps transmitted and the appropriate phase reversal in the IF phase modulated output signal of the modem is measured, e.g. by an oscilloscope. The receive delay is found by subtracting the transmit delay from the measured sum of the delays.

# 8	Data processing

In order to determine the difference between the UTC(k) of the two participating laboratories, one must carefully measure and document the delays which occur in the timing chain of each laboratory. In some laboratories, UTC(k) is a mathematical time-scale. All laboratories have a master clock (clock(k)) which is the physical representation of this time-scale. Each laboratory computes a table of values with the relationship of its master clock to its time-scale [UTC(k) – clock(k)]. Clock(k) can be used as input ref(k) to the modem which generates the PN code which is transmitted. There is a delay [clock(k) – ref(k)] caused by the cabling associated with connecting clock(k) to the modem. The modem generates a 1PPS TX which is related to the transmitted signal which allows the difference [ref(k) – 1PPS TX(k)] with respect to the ref(k) to be measured.

To calculate the difference between the UTC time scales of the two laboratories from [TA(1) – TA (2)] (see § 1), the following applies:

UTC(1) – UTC(2)	=	TA(1) – TA(2)

–	[UTC(1) – clock(1)]

–	[clock(1) – ref(1)]

–	[ref(1) – 1PPS TX(1)]

+	[UTC(2) – clock(2)]

+	[clock(2) – ref(2)]

+	[ref(2) – 1PPS TX(2)]

# 9	Determination of accuracy and stability of the result

The general guidelines of the ISO/1993 “Guide to the expression of uncertainty in measurement” should be followed along with the characterization of performance procedures outlined in Recommendation ITU-R TF.538.

# 10	Performance characteristics of the earth station

There are two levels of earth station performance and operation that must be considered for two-way time transfers. The first level concerns meeting the basic requirements of the modem to produce the desired output phase jitter. The second level concerns the performance characteristics of the earth station as required by the regulatory organizations.

Meeting regulatory requirements do not by themselves guarantee satisfactory two-way operation. Only when the required C/N0 and carrier level C is delivered to the modem will one get satisfactory results. Operating parameters are determined from a link budget. From the link budget, one can determine the size of antenna required, the transmit power required and the required noise temperature of the receive system. The link budget may be calculated following the direction and examples given in § 2.2 and Annex II of the CCIR Handbook on Satellite Communications (Fixed-Satellite Service), Geneva, 1988.

Regulatory requirements usually originate with the administrative bodies responsible for the general management of the electromagnetic spectrum at national and international level and with the satellite operator. The rules and regulations often dictate when and where an earth station may be erected, the quality of that earth station, and its ownership and operation. These rules and regulations usually are intended to guarantee that the earth station will not generate interference to other earth stations and to other adjacent satellites. Issues involved usually include antenna patterns and antenna gain, G/T, e.i.r.p., polarization discrimination criteria, and the training and licensing of operating staff. Much of the technical matter concerned with the earth station can be guaranteed by the procurement of systems from manufacturers who have already qualified the equipment for operation with specific satellite systems and standards.

Electrical performance standards are usually defined by one or more of the following: Recommendations ITU-R S.580 and ITU-R S.465; US FCC regulation 25-209 from the Code of Federal Regulations, Title 47, Parts 20-39; and INTELSAT Earth Station Standard documents for standard C (IESS-203), standard E (IESS-205), standard G (IESS-601), and standard Z (IESS-602) earth stations. Some regions of the world may have other requirements relating to the satellite system, location, classification of user, and other criteria.

ANNEX  2

Information on the data format for data exchange

# 1	Introduction

The data file begins with a header which should contain all data that are not considered to change during the measurement session. The subsequent data lines contain all data that are expected to change during the session. Some data lines contain information for experimental purposes (e.g. the once-per-second readings) whereas other data lines contain information directly for operational use (e.g. the quadratic "best fit" for a session).

# 2	Required information

### 2.1	Station identification and assigned characters, codes and offset-frequencies

The identification of the participating stations by an ASCII character should be done, and PN codes and a clean carrier offset frequency should be assigned.

As an example, see the Table 1.

TABLE  1

| Laboratory designation (ASCII char.) | TX code (MITREX) | Clean carrier offset (kHz from centre frequency) |
| --- | --- | --- |
| A | 0 | - – 20 |
| - B | 1 | - 0 |
| - C | 2 | - + 20 |
| - D | 3 | - – 40 |
| - E | 4 | - + 40 |
| - F | 5 | - + 60 |
| - G | 6 | - + 80 |
| - H | 7 | - – 80 |

## 2.2	Session parameters

A plan for a session should contain the following information:

–	satellite identification: name, longitude, orbital parameters, transponder channel, satellite equipment non-reciprocity;

–	station designations;

–	TX-codes and RX-codes;

–	link budget;

–	earth station TX and RX frequencies;

–	operator's name;

–	start date and time;

–	preparation time for each session (equal to the pause between successive sessions);

–	duration of each session;

–	the way to exchange the generated data files.

## 2.3	Local earth station(k) parameters

Each station should keep on file the following information:

–	antenna coordinates (x,y,z in the IERS terrestrial reference frame);

–	earth station designator;

–	transmit power e.i.r.p. (W);

–	G/T of the receiver;

–	modem: manufacturer, model, type, serial number;

–	time interval counter: manufacturer, type, serial number;

–	antenna: manufacturer, type, diameter, gain;

–	delay calibration: date, method, results;

–	optional: record during the sessions the measured TX-RX delay, the C/N0, the environmental parameters at the antenna (temperature, humidity, barometric pressure, weather conditions).

Any change in station parameters and equipment should be reported.

# 3	Data format

## 3.1	General

For both kinds of data files (see § 3.2 and 3.3), the following is adopted.

The following legend explains the symbols used to characterize the various parameters:

LAB:	laboratory identification (e.g., according to BIPM convention)

jjjjj:	modified Julian date

hh:	UTC hour

mm:	UTC minute

ss:	UTC second

L:	designation of Local laboratory by an ASCII character

R:	designation of Remote laboratory by an ASCII character

*:	indication of the start of a line of text

S:	indication of the sign of the subsequent value

[ ]:	designation of an option

|:	designation of a choice

0.nnnnnnnnnnnn:	value of a time interval in s (12 decimals = 1 ps resolution).

DATA FILE NAME:

Ljjjjjhh.mmR

where jjjjj, hh, mm give the NOMINAL UTC start date of the TWSTFT session.

HEADER:

*	Ljjjjjhh.mmR

*	UTC (LAB) – CLOCK	=	S0.nnnnnnnnnnnn [jjjjj hhmmss]

*	CLOCK – 1PPSREF	=	S0.nnnnnnnnnnnn [jjjjj hhmmss]

*	1PPSREF – 1PPSTX	=	S0.nnnnnnnnnnnn [jjjjj hhmmss]

*	Any other parameter of the form: PARAMETER = Value [units] [jjjjj hhmmss]

*	DATA = [1PPSTX – 1PPSRX] | [1PPSREF – 1PPSRX] | [..]

where jjjjj, hhmmss optionally give the UTC data at which the indicated value is taken.

## 3.2	Report of the individual 1 s measurements

DATA FILE FORMAT:

the HEADER immediately followed by the DATA.

DATA:

| jjjjj | hhmmss | 0.nnnnnnnnnnnn |
| --- | --- | --- |
| jjjjj | hhmmss | 0.nnnnnnnnnnnn |
| µ § | µ § | µ § |
| jjjjj | hhmmss | 0.nnnnnnnnnnnn |

where jjjjj, hhmmss give the UTC date at which the data is taken.

EXAMPLE:

File A4926610.56B (data measured at station A from a TWSTFT session with station B on MJD 49266, scheduled at 1056 min UTC):

*	A4926610.56B

*	UTC (LAB A) – CLOCK	=	–	0.000000123456 49266 101000

*	CLOCK – 1PPSREF	=		0.000000012345

*	1PPSREF – 1PPSTX	=	+	0.000000001234 49266 102059

*	DATA – 1PPSTX – 1PPSRX

49266	105616 0.270924666406

49266	105617 0.270924663805

49266	105618 0.270924660170

49266	105619 0.270924657628

49266	105620 0.270924654270

49266	105621 0.270924651106

## 3.3	Report of averaged results after a quadratic fit

From the data taken the average time interval (i.e. 1PPSTX – 1PPSRX) at mid-point UTC date using a quadratic fit is determined, as well as its signed linear coefficient (s/s) and its signed quadratic coefficient (s/s2).

DATA FILE FORMAT:

the HEADER immediately followed by the average data, the linear coefficient and the quadratic coefficient.

DATA:

jjjjj hhmmss 0.nnnnnnnnnnnn S0.nnnnnnnnnnnn S0.nnnnnnnnnnnn

where jjjjj, hhmmss give the mid-point UTC date of the data.

EXAMPLE:

File A4926610.56B (data measured at station A from a TWSTFT session with station B on MJD 49266, scheduled at 1056 h UTC):

*	A4926610.56B

*	UTC (LAB A) – CLOCK	=	–	0.000000123456 49266 101000

*	CLOCK – 1PPSREF	=		0.000000012345

*	1PPSREF – 1PPSTX	=	+	0.000000001234 49266 102059

*	DATA = 1PPSTX – 1PPSRX

49266	105619 0.270924657628 + 0.000000056789 – 0.000000000321

__________________