RECOMMENDATION  ITU-R  TF.374-4

STANDARD-FREQUENCY  AND  TIME-SIGNAL  EMISSIONS

(Question ITU-R 102-1/7)

(1951-1953-1956-1959-1963-1966-1970-1974-1998)

Rec. ITU-R TF.374-4

The ITU Radiocommunication Assembly,

considering

a)	that the World Administrative Radio Conference (WARC) (Geneva, 1979), allocated the frequencies 20 kHz  0.05 kHz, 2.5 MHz  5 kHz (2.5 MHz  2 kHz in Region 1), 5 MHz  5 kHz, 10 MHz  5 kHz, 15 MHz  10 kHz, 20 MHz  10 kHz and 25 MHz  10 kHz, to the standard-frequency and time-signal service;

b)	that the same Conference allocated the following frequencies for use by the standard-frequency and time-signal satellite service:

400.1	MHz  25 kHz,
4 202	MHz  2 MHz (space-to-Earth),
6 427	MHz  2 MHz (Earth-to-space),
13.4	to 14.0 GHz (Earth-to-space),
20.2	to 21.2 GHz (space-to-Earth),
25.25	to 27.0 GHz (Earth-to-space),
30.0	to 31.3 GHz (space-to-Earth);

c)	that additional standard frequencies and time signals are emitted in other frequency bands, e.g. at frequencies 14-19.95 kHz and 20.05-70 kHz and in Region 1 also in the bands 72-84 kHz and 86-90 kHz, which have been designated by other Conferences (see RR S5.56);

d)	the provisions of Article S.26 [33] of the Radio Regulations;

e)	that transmissions in the bands mentioned in considering a) and predominantly those in considering c) provide widely accepted means of distributing time signals and standard frequencies;

f)	that for many purposes worldwide time synchronization with an uncertainty of less than 1 ms is required, which in an ideal case should be based on simple and inexpensive equipment;

g)	that interference may reduce the usefulness of standard-frequency and time-signal services to a serious degree,

recommends

1	that ITU-R Study Group 7 continue its study of worldwide standard-frequency and time-signal services and explore the application of new techniques for this purpose;

2	that existing standard-frequency and time-signal services be operated in conformity with the detailed Recommendations of the ITU-R;

3	that all efforts be made to prevent or reduce the mutual interference between emissions in the allocated bands specified in the considering;

4	that the methods and results of measurements of phase instabilities over paths in bands 4 and 5 be made available to ITU-R Study Group 7;

5	that appropriate stations existing in band 5 be employed for distributing standard frequencies by precise control of their carrier frequencies as a complement to satellite systems distributing a time reference;

6	that the documentation of services in Recommendations ITU-R TF.583 and ITU-R 768 and in Chapter 2B of the ITU-R Handbook “Selection and Use of Precise Frequency and Time Systems” be taken into consideration when using existing services or planning new services.