RECOMMENDATION  457-1

USE  OF  THE  MODIFIED  JULIAN  DATE  BY  THE  STANDARD-FREQUENCY
AND  TIME-SIGNAL  SERVICES

(Question 1/7)

(1970-1974)

Rec. 457-1

The CCIR,

CONSIDERING

(a)	that for dating purposes a decimal day count is desirable in connection with the use of radio time signals and radio time codes;

(b)	that a decimal day count with reference to Universal Time, the Julian Date (JD), has long been established for dating in astronomy, chronology and related sciences;

(c)	that a decimal day count is necessary, by which the start of a day is defined at 0000 hours and not at 1200 hours as in the case of the Julian Date;

(d)	that a decimal day count is necessary, in particular in association with the time scales UTC and TAI;

(e)	that it is necessary to avoid a proliferation of different dating systems;

(f)	that a simple change from the Julian Date mentioned above to a modern decimal day count would be advantageous;

(g)	that the existing and established Julian Date, based on the start of the day being Greenwich Mean Noon, should be continued without break;

(h)	that a Modified Julian Date (MJD), which meets the requirements stated above, is already in use,

UNANIMOUSLY  RECOMMENDS

1.	that for modern timekeeping and dating requirements, wherever necessary, a decimal day count should be used; the calendar day should be counted from 0000 hours TAI, UTC or UT and be specified by a number with five significant figures;

2.	that this “Modified Julian Date” (MJD) equals the Julian Date less 2 400 000.5 and therefore has its origin, in the case of UT, at 0000 hours UT, 17 November 1858.

_________________