BS01 Solution: UHF digital Television broadcasting assignment

Prepare an electronic notice of frequency 594 MHz assigned to a digital television broadcasting station, for the modification of the Master Register for Administration of Indonesia (INS).

To prepare this notice use the “New File” functionality of TerRaNotices

- Purpose of exercise:  To create a frequency assignment notice using TerRaNotices “New File” functionality for Addition to the Master Register, for the Administration of Indonesia (INS) as the notifying administration.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select new file (TerRaNotices software can be installed from BRIFIC DVD)

- For this exercise, select the following from the New notice dialog box; Administration; and

from VHF/UHF broadcasting notices category, T02 for Television broadcasting station outside the GE06 plan scope.

- Once you get the T02 notice form shown below; select Fragment: Article 11 and Intention : Add as requested by the exercise then start to fill the required data items:

Note: that at the bottom left, the list of items to be filled are indicated. The red color boxes also indicate the missing fields. As the field is correctly filled the item disappears from the list and the color of the box also becomes normal.

- Fill in the required missing fields to complete the notice.

- Altitude of site above sea level, Effective Antenna Heights and Maximum effective antenna height can be calculated using the SRTM3 Terrain Database from Tools menu.

- Once all values are completed, Validate and save the text file containing the complete notice.

- The complete notice without error or warning can be submitted to ITU via WISFAT. (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)