BS07: Solution GE06 Plan modification

Prepare an electronic notice file of frequency 514 MHz assigned to two linked TV broadcasting stations, for the modification of the GE06D Plan.

To prepare these notices use “WIZARD” functionality of TerRaNotices, for Administration of Sweden (S) as the notifying administration.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices . This software can be found in the BRIFIC-DVD.

and Select “Wizard” from File menu.

The Wizard functionality of TerRaNotices, guides you to select the notice type to use according to the given criteria.

- Next specify the required “Plan”.

Note : the Candidate notice types at the bottom of the screen.

- Next specify the required action as “ADD” and subsection “Assignment”.

Note the candidate notice types at the bottom for the specified criteria at each stage.

- Specify the class of station.

Note: the selected type of notice by the wizard is GT1.

- Proceed to open the GT1 notice form;

Note that at the bottom left, the list of items to be filled are indicated. The red color boxes also indicate the missing fields. As the field is correctly filled the item disappears from the list.

- Additional notice can be added in the same file by choosing “Insert New Notice” from the File Menu.

Note: To add more notices in the same file, Repeat this step as many as necessary.

- Fill in the missing fields to complete the 2 notices as requested.

- Altitude of site above sea level, Effective Antenna Heights and Maximum effective antenna height can be calculated using the SRTM3 Terrain Database from Tools menu.

Note: the SRTM3 data is only available between Latitudes 60 degree North and 60 degree South.

- Once the required fields are completed, validate the notices and save file which is ready to be submitted via Wisfat (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)