BS 02 Solution: UHF digital television broadcasting assignment

Prepare an electronic notice file of frequency 474 MHz assigned to a TV broadcasting station, for its recording in the Master Register.

- Purpose of exercise:  To create a frequency assignment notice using TerRaNotices “Wizard” functionality for recording in the Master Register,

Solution

The notice type to use for the notification of a digital television broadcasting station in the UHF band outside the GE06 Regional Agreement is:  T02

- The transmission systems for digital TV are:

| Transmission System | Bandwidth | Class of emission | Description |
| --- | --- | --- | --- |
| T0 | 7 MHz | X7FXF | Digital TV (DVB-T) in VHF band with 7 MHz channel raster |
| T1 | 8 MHz | X7FXF | Digital TV (DVB-T) in UHF band with 8 MHz channel raster |
| T2 | 6 MHz | C7W-- | Digital TV, 6 MHz Bandwidth, single carrier, vestigial sideband |
| T3 | 1.536 MHz | X7WXF | Digital TV (T-DMB) Digital Multimedia Broadcasting |
| T4 | 8 MHz | X7F-- | Digital TV (DVB-H) |
| T5 | 6 MHz | X7WXF | Digital TV (ISDB-T) |
| T6 | 8 MHz | X7FXF | Digital TV (DVB-T2) in UHF band with 8 MHz channel raster |
| T7 | 6 MHz | X7FXF | Digital TV (DVB-T2), 6 MHz bandwidth |
| T8 | 8 MHz | X7F-- | Digital TV (ISDB-T) 8 MHz bandwidth |
| T9 | 6 MHz | X7F-- | Digital TV (ISDB-T), 6 MHz bandwidth |
| U0 | 6 MHz | X7FXF | Digital TV (DVB-T) in UHF band with 6 MHz channel raster |
| U1 | 6 MHz | X7WXF | Digital TV (DTMB), 6 MHz bandwidth |
| U2 | 3.900 MHz | X7WXF | Digital TV (ISDB-T), |

- The purpose of the exercise is to record in the Master Register, the relevant fragment is: Article 11

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours

Electronic notice file:

- Open TerRaNotices and Select “Wizard” from File menu.

The Wizard functionality of TerRaNotices, guides you to select the notice type to use according to the given criteria.

For this exercise, we select BRM as notifying Administration, and “Update of the Master International Frequency Register (MIFR) under Article 11 of the RR” as requested by the exercise.

- Next specify the required action as “ADD”.  Note the candidate notice types at the bottom for the specified criteria at each stage.

- The Class of station shall be selected from the “Notice’s Parameter” menu as “BT” for the Broadcasting television station. (Please refer to the PREFACE to BRFIC  which can be found within the BRIFIC DVD)

Note that the Notice type selected by the wizard for this requirement is “T02”

- Confirm the summary to generate the required Notice form.

- Then Fill all the required fields to complete the notice.

- Validate and save the text file containing the complete T02 notice.

- The complete notice without error or warning can be submitted to ITU via WISFAT.