BS 05 Solution: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of an electronic notice file which was previously prepared and saved.

- Purpose of exercise:  To validate and identify errors in Frequency assignment notice using the online validation.

Solution

- To Validate and identify errors of a notice file, we will use the online validation tool which can be accessed from http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

Electronic notice file:

- Log on to the online validation tool.

http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

- Browse the file to be validated and upload to process. For this exercise, we choose a file “BS 05_NoticeWithError.txt  “

.

- Once the process completed, we can see the following validation error messages as output;

Note that we also receive an email announcing the completion of validation.

- We can see the following error messages;

3.1 The “Necessary Bandwidth” is missing.

  - The Effective radiated power of the vertically polarized component is missing. While the Effective radiated power is given for the horizontal component which is indicated as a warning.

4.  Correction:

4.1 We can put 8000 kHz for the necessary Bandwidth,

4.2 We have a choice to correct the polarization as H to align with the effective radiated power in the horizontal direction, or to provide the effective radiated power of the vertically polarized component to align with Polarization given as Vertical.

For this exercise, the effective radiated power of horizontally polarized component is considered as Polarization is H. And the value 24.0 dbW is entered.

- Once corrected, validate and save the notice.